3GPP DTS 33.128 0.0.8a (2019-02)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Security;

Handover interface for Lawful Interception (LI)

(Release 15)

Keywords

5G, LTE, UMTS, Security, LI, Architecture

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	7

4	General	8

4.1	Introduction	8

4.2	Basic Principles for Internal Interfaces	9

4.3	Basic Principles for Handover Interfaces	10

5	Transport and Communications Protocol	10

5.1	General	10

5.2	Protocols for LI_X1 and LI_T interfaces	10

5.2.1 	General usage of ETSI TS 103 221-1	10

5.2.2 	Usage for realising LI_X1	11

5.2.3	Usage for realising LI_X1 (Management)	11

5.2.4	Usage for realising LI_T2	11

5.2.5	Usage for realising LI_T3	11

5.3	Protocols for LI_X2 and LI_X3	11

5.3.1 	General usage of ETSI TS 103 221-2	11

5.3.2 	Usage for realising LI_X2	11

5.3.3	Usage for realising LI_X3	11

5.4	Protocols for LI_HI1	12

5.4.1 	General	12

5.5	Protocols for LI_HI2 and LI_HI3	12

5.5.1 	General	12

5.5.2 	Usage for realising LI_HI2	12

5.5.3 	Usage for realising LI_HI3	12

5.6	Protocols for LI_HI4	12

5.6.1 	General	12

5.6.2 	Usage for realising LI_HI4	13

6	Network Layer Based Interception	13

6.1 	Introduction	13

6.2 	5G	13

6.2.1 	General	13

6.2.2 	LI at AMF	13

6.2.2.1	Provisioning over LI_X1	13

6.2.2.2	Generation of xIRI over LI_X2	13

6.2.2.2.1	General	13

6.2.2.2.2	Registration	14

6.2.2.2.3	Deregistration	14

6.2.2.2.4	Location Update	15

6.2.2.2.5	Start Of Interception With Registered UE	16

6.2.2.2.6	AMF Unsuccessful Procedure	16

6.2.2.3	Generation of IRI over LI_HI2	17

6.2.2.4	Identity Privacy	17

6.2.3	LI for SMF/UPF	17

6.2.3.1	Provisioning of SMF over LI_X1	17

6.2.3.2	Generation of xIRI at SMF over LI_X2	18

6.2.3.2.1	General	18

6.2.3.2.2	PDU Session Establishment	18

6.2.3.2.3	PDU Session Modification	19

6.2.3.2.4	PDU Session Release	20

6.2.3.2.5	Start of Interception with an Established PDU Session	21

6.2.3.3	Triggering of the UPF from SMF over LI_T3	22

6.2.3.4	Generation of xIRI at UPF over LI_X2	23

6.2.3.5	Generation of xCC at UPF over LI_X3	23

6.2.3.6	Generation of IRI over LI_HI2	24

6.2.3.7	Generation of CC over LI_HI3	24

6.2.4	LI at UDM for 5G	24

6.2.4.1	General description	24

6.2.5	LI at SMSF	24

6.2.5.1	Provisioning over LI_X1	24

6.2.5.2	Generation of xIRI over LI_X2	24

6.2.5.3	SMS	24

6.2.5.4	Generation of IRI over LI_HI2	25

6.2.6	LI support at NRF	25

6.3	4G	25

6.4	3G	25

7	Service Layer Based Interception	25

7.1	Introduction	25

7.2	Central Subscriber Management	26

7.2.1	General description	26

7.2.2	LI at UDM	26

7.2.2.1	General description	26

7.2.2.2	Provisioning over LI_X1	26

7.2.2.3	Generation of xIRI over LI_X2	26

7.2.2.3.1	Serving System xIRI	26

7.2.2.3.2	Subscriber Record Change xIRI	26

7.2.2.3.3	Cancel Location xIRI	26

7.2.2.3.4	Location Information Request	26

7.2.2.4	Generation of IRI over LI_HI2	26

7.2.3	LI at HSS	26

7.3	Location	26

7.3.3	Lawful Access Location Services (LALS)	26

7.3.3.1	General Description	26

7.3.3.2	Provisioning over LI_X1	27

7.3.3.2.1	Target Positioning Service	27

7.3.3.2.2	Triggered Location Service	27

7.3.3.3	Triggering over LI_X2	28

7.3.3.4	Generation of xIRI over LI_X2	28

7.3.3.5	Generation of IRI over LI_HI2	29

7.3.4	Cell Database Information Reporting	29

7.3.4.1	General Description	29

7.3.4.2	Generation of IRI over HI2	29

Annex A (normative): Structure of the Internal Interface	29

Annex B (normative): Structure of the Handover Interface	35

B.X	LI_HI4 Generic LI notification	36

Annex C (informative): Implementation Guidance	37

Annex D (informative): Drafting Rule Guidance	37

Annex Z (informative): Change history	38

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document describes protocols and procedures for Lawful Interception based on 3GPP specifications. Lawful Interception shall always be done in accordance with the applicable national or regional laws and technical regulations.

# 1	Scope

The present document describes …………………….

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.126: "Lawful Interception Requirements".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 33.127: "Lawful Interception (LI) Architecture and Functions".

[6]	ETSI TS 103 120: " Lawful Interception (LI); Interface for warrant information".

[7]	ETSI TS 103 221-1: "Lawful Interception (LI); Part 1: Internal Network Interface X1 for Lawful Interception".

[8]	ETSI TS 103 221-2: "Lawful Interception: Internal Network Interface X2/X3".

[9]	ETSI TS 102 232-1: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 1: Handover specification for IP delivery".

[10]	ETSI TS 102 232-7: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 7: Service-specific details for Mobile Services".

[11]	3GPP TS 33.501: "Security Architecture and Procedures for the 5G System".

[12]	3GPP TS 33.108: "3G security; Handover interface for Lawful Interception (LI)".

[13]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS)".

[14]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General Aspects".

[15]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[16]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[17]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[18]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[19]	3GPP TS 23.003: "Numbering, addressing and identification ".

[20]	OMA-TS-MLP: "Open Mobile Alliance; Mobile Location Protocol, Version 3.4", OMA-TS-MLP-V3-4-20150512-A.

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

Editor’s Note: Import applicable definitions from 33.126 & 33.107.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ADMF	LI Administration Function

CC	Content of Communication

CPOI	Control plane Point Of Interception

CSP	Communication Service Provider

CUPS	Control and User Plane Separation

IRI	Intercept Related Information

LALS 	Lawful Access Location Services

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

LICF	Lawful Interception Control Function

LI_CPS	Lawful Interception CC interception rules Provisioning Signalling??

LI_HI1	LI_Handover Interface 1

LI_HI2	LI_Handover Interface 2

LI_HI3	LI_Handover Interface 3

LIPF	Lawful Interception Provisioning Function

LIR	Location Immediate Request

LI_SI	Lawful Interception System Information Interface

LI_X1	Lawful Interception Internal Interface 1

LI_X2	Lawful Interception Internal Interface 2

LI_X3	Lawful Interception Internal Interface 3

LTF	Location Triggering Function

MDF	Mediation and Delivery Function

MDF2	Mediation and Delivery Function 2

MDF3	Mediation and Delivery Function 3

NPLI	Network Provided Location Information

POI	Point Of Interception

SIRF	System Information Retrieval Function

SOI	Start Of Interception

SX3LIF	Split X3 LI Interworking Function

TF	Triggering Function

xCC	X3 Communications Content.

xIRI	X2 Intercept Related Information

Editor’s Note: Only add those abbreviations which are specifically needed and rely on 21.905 where possible.

# 4	General

## 4.1	Introduction

The present document provides details of the internal and external interfaces required for a network operator, access provider and/or service provider to provide the necessary information to a Law Enforcement Agency (LEA) required to meet LI requirements. LI requirements for 3GPP networks and services are given in TS 33.126 [3].

The high-level architecture that defines the necessary interfaces is specified in TS 33.127 [5]. The generic high-level architecture is as follows:

The specification of the interfaces is split into two parts:

- Internal interfaces used between an operator’s network functions are described in clause 4.2.

- External interfaces used in communicating with a LEA are described in clause 4.3.

## 4.2	Basic principles for internal interfaces

This clause lists the internal interfaces shown in clause 4.1, indicates the protocol used to realise each interface, and gives a reference to the relevant clauses of the present document that specify how the protocol is to be used for the given interface.

Table 4.2-1: Internal interfaces and related protocols

| Interface | Description | Protocol used to realise interface | Usage |
| --- | --- | --- | --- |
| LI_SI | Used to provide system information to the LIPF from the SIRF | Out of scope of the present document |  |
| LI_X1 | Used to configure and audit Directly-provisioned POIs, TFs and MDFs. | ETSI TS 103 221-1 [7] | See clause 5.2.2 |
| LI_X1 (Management) | Used to audit Triggered POIs | ETSI TS 103 221-1 [7] | See clause 5.2.3 |
| LI_X2 | Used to pass xIRI information from IRI-POIs to the MDF2 | ETSI TS 103 221-2 [8] | See clause 5.3.2 |
| LI_X3 | Used to pass xCC information from CC-POIs to the MDF3 | ETSI TS 103 221-2 [8] | See clause 5.3.3 |
| LI_T2 | Used to pass triggering information from the IRI-TF to a Triggered IRI-POI | ETSI TS 103 221-1 [7] | See clause 5.2.4 |
| LI_T3 | Used to pass triggering information from a CC-TF to a Triggered CC-POI | ETSI TS 103 221-1 [7] | See clause 5.2.4 |
| LI_ADMF | Used to pass intercept provisioning information form the LICF to the LIPF | Out of scope of the present document |  |
| LI_MDF | Used by MDF2 and MDF3 in interactions necessary to correctly generate CC and IRI from xCC and xIRI. | Out of scope of the present document |  |

## 4.3	Basic principles for handover interfaces

This clause lists the external and handover interfaces shown in clause 4.1, indicates the protocol used to realise each interface, and gives a reference to the relevant clauses of the present document that specify how the protocol is to be used for the given interface.

Table 4.3-1: Internal interfaces and related protocols

| Interface | Description | Protocol used to realise interface | Usage |
| --- | --- | --- | --- |
| LI_HI1 | Used to send warrant and other interception request information from LEA to operator. | ETSI TS 103 120 [6] shall be supported Other methods (e.g manual exchange) may be used depending on national regulatory requirements. | See section 5.3.4 |
| LI_HI2 | Used to send IRI from the MDF2 to the LEMF | ETSI TS 102 232 - 1 [9] and ETSI TS 102 232 – 7 [10] shall be supported | See section 5.3.5 |
| LI_HI3 | Used to send CC from the MDF3 to the LEMF | ETSI TS 102 232 -1 [9] and ETSI TS 102 232 – 7 [10] shall be supported | See section 5.3.5 |
| LI_HI4 | Used to send notification information from MDF2/3 to LEMF | ETSI TS 102 232 - 1 [9] and ETSI TS 102 232 – 7 [10] shall be supported | See section 5.3.6 |

# 5	Transport and Communications Protocol

## 5.1	General

This clause describes the protocols used for each of the interfaces at a level which is agnostic of the subject service or network. Additional specific fields or behaviours are given in the relevant parts of clauses 6 and 7.

## 5.2	Protocols for LI_X1 and LI_T interfaces

### 5.2.1 	General usage of ETSI TS 103 221-1

Functions having an LI_X1, LI_T2 or LI_T3 interface shall support the use of ETSI TS 103 221-1 [7] to realise the interface.

In the event of a conflict between ETSI TS 103 221-1 [7] and the present document, the terms of the present document shall apply.

The LIPF and MDF2/3 shall maintain a mapping between internal interception identifiers (XIDs) and external interception identifiers (LIIDs), as defined by TS 103 221-1 [7] clause 5.1.2. In the case of multiple interceptions for a single target identifier, it is an implementation decision for the LIPF/TF whether multiple XIDs are used (i.e. a one-to-one mapping between XID and LIID is maintained) or whether the single XID is used and mapped to multiple LIIDs at the MDF2/3. Clauses 6 and 7 give further details for specific networks or services (e.g. minimum supported target identifier formats).

In the event that a request issued over the interface fails, or an error is reported, the LIPF should raise an alert in the appropriate LI O&M system. Further procedures (e.g. retrying a failed request) are left to CSP policy to define.

### 5.2.2 	Usage for realising LI_X1

For the purposes of realising LI_X1 between the LIPF and a POI or TF, the LIPF plays the role of the “ADMF” as defined in the ETSI TS 103 221-1 [7] reference model (clause 4.2), and the POI or TF plays the role of the “NE”.

### 5.2.3	Usage for realising LI_X1 (management)

For the purposes of realising LI_X1 between the LIPF and a triggered POI, the LIPF plays the role of the “ADMF” as defined in the ETSI TS 103 221-1 [7] reference model (clause 4.2) , and the triggered POI plays the role of the “NE”.

### 5.2.4	Usage for realising LI_T2

For the purposes of realising LI_T2 between a TF and a Triggered POI, the TF plays the role of the “ADMF” as defined in the ETSI TS 103 221-1 [7] reference model (clause 4.2), and the triggered POI plays the role of the “NE”.

### 5.2.5	Usage for realising LI_T3

For the purposes of realising LI_T3 between a TF and a Triggered POI, the TF plays the role of the “ADMF” as defined in the ETSI TS 103 221-1 [7] reference model (clause 4.2), and the triggered POI plays the role of the “NE”.

## 5.3	Protocols for LI_X2 and LI_X3

### 5.3.1	General usage of ETSI TS 103 221-2

Functions having an LI_X2 or LI_X3 interface shall support the use of ETSI TS 103 221-2 [8] to realise the interface.

In the event of a conflict between ETSI TS 103 221-2 [8] and the present document, the terms of the present document shall apply.

xIRI and xCC messages sent using ETSI TS 103 221-2 [8] shall contain the appropriate XID as received in the relevant LI_X1 provisioning message (or LI_T2/3 triggering message, as appropriate).

### 5.3.2 	Usage for realising LI_X2

The POI sending xIRI over the LI_X2 interface shall set the PDU type field to “X2 PDU”. (see ETSI TS 103 221-2 [8] clause 5.1).

The TLS transport profile (see [8] clause 6) shall be supported and used by default.

Unless otherwise specified, xIRI messages shall include the timestamp and sequence number conditional attribute fields, with the timestamp value set to the time that the event occurred.

### 5.3.3	Usage for realising LI_X3

The POI sending xCC messages over the LI_X3 interface shall set the PDU type field to “X3 PDU”. (see ETSI TS 103 221-2 [8] clause 5.1).

## 5.4	Protocols for LI_HI1

### 5.4.1 	General

Functions having an LI_HI1 interface shall support the use of ETSI TS 103 120 [6] to realise the interface.

In the event of a conflict between ETSI TS 103 120 [6] and the present document, the terms of the present document shall apply.

## 5.5	Protocols for LI_HI2 and LI_HI3

### 5.5.1	General

Functions having an LI_HI2 or LI_HI3 interface shall support the use of ETSI TS 102 232-1 [9] and ETSI TS 102 232-7 [10] to realise the interface.

In the event of a conflict between either specification and the present document, the terms of the present document shall apply.

### 5.5.2 	Usage for realising LI_HI2

The LI_HI2 messages are structured as a header and a payload. The header contains general information like LIID, timestamp, correlation information (as for example defined in ETSI TS 102 232-1 [9]). The payload contains intercept related information based on information that the MDF2 has received from sources in the network, such as the IRI-POI as described in clauses 6 and 7 of the present document. Details of the LI_HI2 messages can be found in Annex B of the present document. Messages defined as passing over the LI_HI2 interface shall be passed as the payload of the threeGPP-IRI field (see TS ETSI 102 232 -7 [10] clause 15)

Editor’s Note – ETSI TS 102 232-7 Clause 15 does not yet exist, and will either be introduced by a CR or the text above will be revised.

### 5.5.3 	Usage for realising LI_HI3

The LI_HI3 PDUs are structured as a header and a payload. The header contains general information like LIID, timestamp, correlation information (as for example defined in ETSI TS 102 232-1 [9]). The payload contains content of communication based on information that the MDF3 has received from sources in the network, such as the CC-POI as described in clauses 6 and 7 of the present document. Details of the LI_HI3 messages can be found in Annex B of the present document. Messages defined as passing over the LI_HI3 interface shall be passed as the payload of the threeGPPCC field (see ETSI TS 102 232-7 [10] clause 15).

Editor’s Note – ETSI TS 102 232-7 Clause 15 does not yet exist, and will either be introduced by a CR or the text above will be revised.

## 5.6	Protocols for LI_HI4

### 5.6.1	General

Functions having an LI_HI4 shall support the use of ETSI TS 102 232-1 [9] to realise the interface.

In the event of a conflict between ETSI TS 102 232-1 [9] and the present document, the terms of the present document shall apply.

### 5.6.2	Usage for realising LI_HI4

The LI_HI4 PDUs are structured as a header and a payload. The header contains general information like LIID, timestamp (as for example defined in ETSI TS 102 232-1 [9]). The payload contains the administrative information such as notification. Details of the LI_HI4 messages can be found in Annex B of the present document.

Where the LI_HI4 interface is present alongside an LI_HI2 or LI_HI3 interface, the LI_HI4 messages shall be transmitted along the same connection as the LI_HI2 or LI_HI3 messages. Where ETSI TS 102 232-1 [9] is used for LI_HI2 or LI_HI3, messages defined as passing over the LI_HI4 interface shall be passed as the contents of the threeGPP-HI1-Operation field.

The MDF2/3 shall support generation LI_HI4 notifications for at least the following events:

-	Activation of an interception at the MDF2/3 via LI_X1.

-	Modification of an interception at the MDF2/3 via LI_X1.

-	Deletion of an interception at the MDF2/3 via LI_X1.

# 6	Network Layer Based Interception

## 6.1	Introduction

This clause describes any remaining fields, behaviours or details necessary to implement the required LI interfaces specific 3GPP-defined network deployments not already described in clauses 4 and 5.

### 6.2	5G

### 6.2.1	General

This clause describes gives details of the LI interfaces specific to LI for 5G networks.

### 6.2.2	LI at AMF

Editor’s Note – we should specify how Correlation IDs should be set

#### 6.2.2.1	Provisioning over LI_X1

The IRI-POI present in the AMF is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI in the AMF shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

-	SUPI.

-	PEI.

-	GPSI.

#### 6.2.2.2	Generation of xIRI over LI_X2

##### 6.2.2.2.1	General

The IRI-POI present in the AMF shall emit X2 PDUs over LI_X2 for each of the events described in the following clauses.

xIRI messages pertaining to the same registration shall be given the same Correlation ID. Unless otherwise specified, the X2 “matched target identifier” conditional attribute shall be set to indicate what target identity was matched to generate the xIRI message (see ETSI TS 103 221-2 [8] clause 5.3.18).

##### 6.2.2.2.2	Registration

The IRI-POI in the AMF shall generate an XIRIAMFRegistrationMessage when the IRI-POI present in the AMF detects that a UE matching one of the target identifiers provided via LI_X1 has successfully registered to the 5GS via 3GPP NG-RAN or non-3GPP access. Accordingly, the IRI-POI in the AMF generates the XIRIAMFRegistrationMessage when one of the following events are detected:

-	AMF sends a N1: REGISTRATION ACCEPT message to the target UE and the UE 5G Mobility Management (5GMM) state within the AMF is changed to 5GMM-REGISTERED.

-	When the N1: REGISTRATION ACCEPT message is sent to the target UE that requires a confirmation from the UE (e.g. 5G GUTI is changed), AMF receives the N1: REGISTRATION COMPLETE message from the target UE and the UE 5GMM state within the AMF is 5GMM-REGISTERED.

Editor’s Note: TS 24.501 does not provide the ASN.1 definition for the referenced parameter types (e.g. NSSAI and rejected NSSAI). So, we will have to define it locally in the TS 33.128, or define as OCTET STRING and use CT spec value.

Editor’s Note – We may choose to refactor the ASN.1 into a single module later, but for now we will write each in the relevant section to make them easier to discuss. At that point we can also consider any other factorisation (e.g. grouping fields together) subject to keeping the number of OPTIONAL fields to a minimum.

Editor’s Note – Need to consider impacts of unauthenticated emergency calls and PARLOS. This editor’s note also applies to other events.

Table 6.2.2-1: Payload for XIRIAMFRegistrationMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| registrationType | Specifies the type of registration, see TS 24.501 [13] clause 9.11.3.7. This is derived from the information received from the UE in the REGISTRATION REQUEST message. | M |
| registrationResult | Specifies the result of registration, see TS 24.501 [13] clause 9.11.3.6. | M |
| sliceInformation | Provide, if available, one or more of the following: - allowed NSSAI (see TS 24.501 [13] clause 9.11.3.37) - configured NSSAI (see TS 24.501 [13] clause 9.11.3.37) - rejected NSSAI (see TS 24.501 [13] clause 9.11.3.46) This is derived from the information sent to the UE in the REGISTRATION ACCEPT message. | C |
| Supi | SUPI associated with the registration (see clause 6.2.2.4). | M |
| Suci | SUCI used in the registration. | C |
| Pei | PEI provided by the UE during the registration, if available. | C |
| Gpsi | GPSI obtained in the registration, if available as part of the subscription profile. | C |
| Guti | 5G-GUTI provided as outcome of initial registration or used in other cases, see TS 24.501 [13] clause 5.5.1.2.2. | M |
| Location | Location information determined by the network during the registration, if available. | C |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, if available. | C |

##### 6.2.2.2.3	Deregistration

The IRI-POI in the AMF shall generate an XIRIAMFDeregistrationMessage when the IRI-POI present in the AMF detects that a UE matching one of the target identifiers provided via LI_X1 has deregistered from the 5GS. Accordingly, the IRI-POI in AMF generates the XIRIAMFDeregistrationMessage when one of the following events are detected:

-	For network initiated de-registration, when the AMF receives the N1: DEREGISTRATION ACCEPT message from the target UE or when implicit deregistration timer expires; and in both cases the UE 5GMN state within the AMF is changed to 5GMM-DEREGISTERED.

-	For UE initiated de-registration, when the AMF sends the N1: DEREGISTRATION ACCEPT message to the target UE or when the AMF receives the N1: DEREGISTRATION REQUEST message from the target UE with deregistration type value of “switch off”; and in both cases the UE 5GMN state within the AMF is changed to 5GMM-DEREGISTERED.

Table 6.2.2-2: Payload for XIRIAMFDeregistrationMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| deregistrationDirection | Indicates whether the deregistration was initiated by the network or by the UE. | M |
| accessType | Indicates the access for which the deregistration is handled, see TS 24.501 [13] , clause 9.11.3.20. | M |
| Supi | SUPI associated with the deregistration (see clause 6.2.2.4), if available. | C |
| Suci | SUCI used in the deregistration, if available (see NOTE 1). | C |
| Pei | PEI used in the deregistration, if available (see NOTE 1). | C |
| Gpsi | GPSI associated to the deregistration, if available as part of the subscription profile. | C |
| Guti | 5G-GUTI used in the deregistration, if available, see TS 24.501 [13], clause 5.5.2.2.1 (see Note 1). | C |
| Cause | Indicates the 5GMM cause value for network-initiated deregistration, see TS 24.501 [13], clause 9.11.3.2. | C |
| Location | Location information determined by the network during the deregistration, if available. | C |

NOTE 1:	At least one among SUCI, PEI and GUTI shall be provided.

##### 6.2.2.2.4	Location update

The IRI-POI in the AMF shall generate an AMFLocationUpdateMessage each time the IRI-POI present in an AMF detects that the target’s UE location is updated due to target’s UE mobility or as a part of an AMF service procedure. The generation of such separate xIRI is not required if the updated UE location information is obtained as a part of a procedure producing some other xIRIs (e.g. mobility registration). In that case the location information is included into the respective xIRI message.

The UE mobility events resulting in a XIRIAMFLocationUpdateMessage generation include the “N2 Path Switch Request” (“Xn based inter NG-RAN handover” procedure described in [4], clause 4.9.1.2) and the “N2 Handover Notify” (“Inter NG-RAN node N2 based handover” procedure described in [4], clause 4.9.1.3).

Additionally, based on regulatory requirements and operator policy, the location information obtained by AMF from NG-RAN or LMF in the course of some service operation (e.g., emergency services, LCS) may generate an AMF Location Update xIRI. In the case of NG-RAN, the generation is triggered by a “Location Report” from NG-RAN induced by a “Location Report Control” from AMF, as described in [4], clause 4.10. In the case of LMF, the generation is triggered by a “Nlmf_Location_DetermineLocation Response” from LMF to AMF, as described in [4], clause 5.2.15.

NOTE :	The IRI-POI in AMF needs to monitor both the service requests and the service responses to/from the NG-RAN and LMF to properly construct the location information for xIRI Location Update messages.

Table 6.2.2-3: Payload for AMFLocationUpdateMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| Supi | SUPI associated with the location update (see clause 6.2.2.4). | M |
| Suci | SUCI associated with the location update, if available, see TS 24.501 [13]. | C |
| Pei | PEI associated with the location update, if available. | C |
| Gpsi | GPSI associated with the location update, if available as part of the subscription profile. | C |
| Guti | 5G-GUTI associated with the location update, if available, see TS 24.501 [13]. | C |
| Location | Updated location information determined by the network. | M |

##### 6.2.2.2.5	Start Of Interception with registered UE

The IRI-POI in the AMF shall generate an XIRIAMFStartOfInterceptionWithRegisteredUEMessage when the IRI-POI present in the AMF detects that interception is activated on a UE that has already been registered in the 5GS (see clause 6.2.2.4 on identity privacy). A UE is considered to be already registered to the 5GS when the 5GMM state for that UE is 5GMM-REGISTERED. Therefore, the IRI-POI present in the AMF shall generate the xIRI AMF Start Of Interception with Registered UE message when it detects that a new interception for an UE is activated (i.e. provisioned by the LIPF) and the 5G mobility management state within the AMF for that UE is 5GMM-REGISTERED.

Editor’s Note: TS 24.501 does not provide the ASN.1 definition for the referenced parameter types (e.g. NSSAI and rejected NSSAI). So, we will have to define it locally in the TS 33.128, or define as OCTET STRING and use CT spec value.

Table 6.2.2-4: Payload for XIRIAMFStartOfInterceptionWithRegisteredUEMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| registrationResult | Specifies the result of registration, see TS 24.501 [13], clause 9.11.3.6. | M |
| registrationType | Specifies the type of registration, see TS 24.501 [13] clause 9.11.3.7, if available. | C |
| sliceInformation | Provide, if available, one or more of the following: - allowed NSSAI (see TS 24.501 [13] clause 9.11.3.37). - configured NSSAI (see TS 24.501 [13] clause 9.11.3.37). - rejected NSSAI (see TS 24.501 [13] clause 9.11.3.46). This is derived from the information that was sent to the UE in the REGISTRATION ACCEPT message. IRI-POI in AMF can include this information if and only if it retained the information that it had previously sent in the REGISTRATION ACCEPT message to the UE. | C |
| supi | SUPI associated with the registration (see clause 6.2.2.4). | M |
| suci | SUCI used in the registration. | C |
| pei | PEI provided by the UE during the registration, if available. | C |
| gpsi | GPSI obtained in the registration, if available as part of the subscription profile. | C |
| guti | 5G-GUTI provided as outcome of initial registration or used in other cases, see TS 24.501 [13], clause 5.5.1.2.2. | M |
| location | Location information, if available. | C |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, if available. | C |
| timeOfRegistration | Time at which the last registration occurred, if available. This is the time stamp when the REGISTRATION ACCEPT message is sent to the UE or (when applicable) when the REGISTRATION COMPLETE is received from the UE. | C |

##### 6.2.2.2.6	AMF unsuccessful procedure

The IRI-POI in the AMF shall generate an XIRIAMFUnsuccessfulProcedure when the IRI-POI present in the AMF detects an unsuccessful procedure for a UE matching one of the target identifiers provided via LI_X1.

Unsuccessful registration shall be reported only if the target UE has been successfully authenticated.

Table 6.2.2-5: Payload for XIRIAMFUnsuccessfulProcedure

| Field name | Description | M/C/O |
| --- | --- | --- |
| failedprocedureType | Specifies the procedure which failed at the AMF. | M |
| failurecause | Provides the value of the 5GSM or 5GMM cause, see TS 24.501 [13], clauses 9.11.3.2 and 9.11.4.2. | M |
| requestedSliceInformation | Slice requested for the procedure, if available, given as a NSSAI (a list of S-NSSAI values as described in TS 24.501 [13] clause 9.11.3.37). | C |
| supi | SUPI associated with the procedure, if available (see Note). | C |
| suci | SUCI used in the procedure, if applicable and if available (see Note). | C |
| pei | PEI used in the procedure, if available (see Note). | C |
| gpsi | GPSI used in the procedure, if available (see Note). | C |
| guti | 5G-GUTI used in the procedure, if available, see TS 24.501 [13], clause 9.11.3.4 (see Note). | C |  |
| location | Location information determined during the procedure, if available. | C |

NOTE:		At least one identity shall be provided, the others shall be provided if available.

#### 6.2.2.3	Generation of IRI over LI_HI2

When an AMF xIRI message is received over LI_X2, the MDF2 shall emit an IRI message over LI_HI2 without undue delay.

The timestamp field of the psHeader structure shall be set to the time that the AMF event was observed (i.e. the timestamp field of the X2 PDU).

The payload of the xIRI messages shall be used as the payload of the IRI contents.

When additional warrants are activated on a target UE, MDF2 shall be able to generate and deliver the Start Of Interception with already registered UE message to the LEMF associated with the additional warrants without receiving a corresponding xIRI.

#### 6.2.2.4	Identity privacy

The AMF shall ensure for every registration (including re-registration) that SUPI has been provided by the UDM to the AMF and that the SUCI to SUPI mapping has been verified as defined in TS 33.501 [11]. This shall be performed regardless of whether the SUPI is a target of interception, and whether the null encryption algorithm is used for the SUCI. The AMF shall maintain the SUPI to SUCI mapping for at least the lifetime of the registration in order to allow interception on SUPI after the initial registration.

### 6.2.3	LI for SMF/UPF

Editor’s Note – we need to specify how Correlation IDs are managed.

#### 6.2.3.1	Provisioning of SMF over LI_X1

The IRI-POI and CC-TF present in the SMF are provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2. If an SMF and UPF are implemented as a single function, then this interface may be sufficient to provision both (see section 6.2.3.3).

The POI in the SMF shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

-	SUPI.

-	PEI.

-	GPSI.

Editor’s Note: Are there any significant differences between provisioning of the two functions?

Editor’s Note: Point out somehow that if the two functions are co-located or implemented together, a single provisioning message containing all the information is sufficient.

#### 6.2.3.2	Generation of xIRI at SMF over LI_X2

##### 6.2.3.2.1	General

The IRI-POI present in the SMF shall emit X2 PDUs over LI_X2 for each of the events described in the following clauses.

##### 6.2.3.2.2	PDU session establishment

The IRI-POI in the SMF shall generate an XIRISMFPDUSessionEstablishmentMessage when the IRI-POI present in the SMF detects that a PDU session has been established for the target UE. The IRI-POI present in the SMF shall generate the xIRI SMF PDU session sstablishment message for the following events:

-	For a non-roaming scenario, the SMF (or for a roaming scenario, V-SMF in the VPLMN), sends the N1 NAS message (via AMF) PDU SESSION ESTABLISHMENT ACCEPT to the UE and the 5G Session Management (5GSM) state within the SMF is changed to PDU session active (see TS 24.501 [13]).

-	For a home-routed roaming scenario, the SMF in the HPLMN (i.e. H-SMF) sends the N16: Nsmf_PDU_Session_Create response message with n1SmInfoToUe IE containing the PDU SESSION ESTABLISHMENT ACCEPT (see TS 29.502 [16]).

Editor’s Note: If UPF has two tunnels, should both tunnel IDs be reported?

Editor’s Note: Name of parameter type AMFAccessType should be common to both AMF and SMF.

Table 6.2.3-1: Payload for XIRISMFPDUSessionEstablishmentMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| supi | SUPI associated with the PDU session (e.g. as provided by the AMF in the associated Nsmf_PDU_Session_CreateSMContext service operation). Shall be present except for PEI-only unauthenticated emergency sessions. | C |
| supiUnauthenticated | Shall be present if a SUPI is present in the message, and set to “true” if the SUPI has not authenticated, or “false” if it has been authenticated. | C |
| pei | PEI associated with the PDU session if available. | C |
| gpsi | GPSI associated with the PDU session if available. | C |
| pduSessionID | PDU Session ID See clause 9.4 of TS 24.501 [13]. | M |
| gtpTunnelID | Contains the F-TEID identifying the GTP tunnel used to encapsulate the traffic, as defined in TS 29.244 [15] clause 8.2.3. Non-GTP encapsulation is for further study. | M |
| pduSessionType | Identifies selected PDU session type, see TS 24.501 [13] clause 9.11.4.11. | M |
| sNSSAIs | Slice identifiers associated with the PDU session, if available. See TS 23.003 [19] clause 28.4.2 and TS 23.501 [2] clause 5.12.2.2. | C |
| ueEndpoint | UE endpoint address(es) if available. | C |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, if available. | C |
| locationInformation | Location information provided by the AMF, if available. | C |
| dnn | Data Network Name associated with the target traffic, as defined in TS 23.003[19] [16] clause 9A and described in TS 23.501 [2] clause 4.3.2.2. | M |
| amfID | Identifier of the AMF associated with the target UE, as defined in TS 23.003 [19] clause 2.10.1 when available. | C |
| hSMFURI | URI of the Nsmf_PDUSession service of the selected H-SMF, if available. See TS 29.502 [16] clause 6.1.6.2.2. | C |
| requestType | Type of request as described in TS 24.501 [13] clause 9.11.3.47 if available. | C |
| accessType | Access type associated with the session (i.e. 3GPP or non-3GPP access) if provided by the AMF (see TS 24.501 [13] clause 9.11.3.11). | C |
| ratType | RAT Type associated with the access if provided by the AMF as part of session establishment (see TS 23.502 [4] clause 4.3.2). Values given as per TS 29.571 [17] clause 5.4.3.2. | C |
| smPDUDNRequest | Contents of the SM PDU DN Request container, if available, as described in TS 24.501 [13] clause 9.11.4.15. | C |

At least one of the SUPI, PEI or GPSI fields shall be present.

##### 6.2.3.2.3	PDU session modification

The IRI-POI in the SMF shall generate an XIRISMFPDUSessionModificationMessage when the IRI-POI present in the SMF detects that a PDU session has been modified for the target UE. The IRI-POI present in the SMF shall generate the xIRI SMF PDU session modification message for the following events:

-	For a non-roaming scenario, the SMF (or for a roaming scenario, V-SMF in the VPLMN), receives the N1 NAS message (via AMF) PDU SESSION MODIFICATION COMMAND COMPLETE from the UE and the 5G Session Management (5GSM) state within the SMF is returned to PDU session active (see TS 24.501 [13]). This applies to the following two cases:

-	UE initiated PDU session modification.

-	Network (VPLMN) initiated PDU session modification.

-	For a non-roaming scenario, the SMF (or for a roaming scenario, V-SMF in the VPLMN), sends the N1 NAS message (via AMF) PDU SESSION ESTABLISHMENT ACCEPT to the UE and the 5G Session Management (5GSM) state within the SMF remains in the PDU session active (see TS 24.501 [13]). This applies to the following case:

-	Handover from one access type to another access type happens (e.g. 3GPP to non-3GPP).

-	For a home-routed roaming scenario, the SMF in the HPLMN (i.e. H-SMF) receives the N16: Nsmf_PDU_Session_Update response message with n1SmInfoFromUe IE containing the PDU SESSION MODIFICATION COMMAND COMPLETE (see TS 29.502 [16]). This applies to the following three cases:

-	UE initiated PDU session modification.

-	Network (VPLMN) initiated PDU session modification.

-	Network (HPLMN) initiated PDU session modification.

-	For a home-routed roaming scenario, the SMF in the HPLMN (i.e. H-SMF) sends the N16: Nsmf_PDU_Session_Create response message with n1SmInfoToUe IE containing the PDU SESSION ESTABLISHMENT ACCEPT (see TS 29.502 [16]) while it had received a N16 Nsmf_PDU_Session_Create request message with an existing PDU session Id with access type being changed.  This applies to the following case:

-	Handover from one access type to another access type happens (e.g. 3GPP to non-3GPP).

Editor’s Note: The TS 23.502 infers that the SMF sends the N4: Session Modification Request after it receives the PDU Session Modification Command Complete message from the UE. The TS 23.502 flow diagram requires further analysis. If the flow is correct, then the trigger may have to be moved to the reception of N4 message from the UPF.

Table 6.2.3-2: Payload for XIRISMFPDUSessionModificationMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| supi | SUPI associated with the PDU session (e.g. as provided by the AMF in the associated Nsmf_PDU_Session_CreateSMContext service operation). Shall be present except for PEI-only unauthenticated emergency sessions. | C |
| supiUnauthenticated | Shall be present if a SUPI is present in the message, and set to “true” if the SUPI was not authenticated, or “false” if it has been authenticated. | C |
| pei | PEI associated with the PDU session if available. | C |
| gpsi | GPSI associated with the PDU session if available. | C |
| sNSSAI | Slice identifier associated with the PDU session, if available. See TS 23.003 [19] clause 28.4.2 and TS 23.501 [2] clause 5.12.2.2. | C |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, if available. | C |
| locationInformation | Location information provided by the AMF, if available. | C |
| accessType | Access type associated with the session (i.e. 3GPP or non-3GPP access) if provided by the AMF (see TS 24.501 [13] clause 9.11.3.11). | C |
| ratType | RAT type associated with the access, if available. Values given as per TS 29.571 [17] clause 5.4.3.2. | C |

##### 6.2.3.2.4	PDU session release

The IRI-POI in the SMF shall generate an XIRISMFPDUSessionReleaseMessage when the IRI-POI present in the SMF detects that a PDU session been released. The IRI-POI present in the SMF shall generate XIRISMFPDUSessionReleaseMessage for the following events:

-	For a non-roaming scenario, the SMF (or for a roaming scenario, V-SMF in the VPLMN), receives the N1 NAS message (via AMF) PDU SESSION RELEASE COMMAND COMPLETE from the UE and the 5G Session Management (5GSM) state within the SMF is changed to PDU session inactive (see TS 24.501 [13]). This applies to the following two cases:

-	UE initiated PDU session release.

-	Network initiated PDU session release.

-	For a non-roaming scenario, the SMF (or for a roaming scenario, V-SMF in the VPLMN), receives the N1 NAS message (via AMF) PDU SESSION MODIFICATION COMMAND REJECT from the UE with the cause value #43 indicating an invalid PDU Session ID and the 5G Session Management (5GSM) state within the SMF is changed to PDU session inactive (see TS 24.501 [13]). This applies to the case where the UE rejects a PDU session modification command as it finds that the indicated PDU session ID is invalid. The state is changed to PDU Session inactive implicitly within the SMF.

-	For a home-routed roaming scenario, the SMF in the HPLMN (i.e. H-SMF) receives the N16: Nsmf_PDU_Session_Update response message with n1SmInfoFromUe IE containing the PDU SESSION RELEASE COMMAND COMPLETE (see TS 29.502 [16]) from the V-SMF. This applies to the following three cases:

-	UE initiated PDU session release.

-	Network (VPLMN) initiated PDU session release.

-	Network (HPLMN) initiated PDU session release.

-	For a home-routed roaming scenario, the SMF in the HPLMN (i.e. H-SMF) receives the N16: Nsmf_PDU_Session_Update response message with n1SmInfoFromUe IE containing the PDU SESSION MODIFICATION COMMAND REJECT (see TS 29.502 [16]) from the V-SMF with the cause value #43 indicating an invalid PDU Session ID.

Editor’s Note: The TS 29.502 lists another case of H-SMF sending the N16: NotifyStatus to the V-SMF indicating that a PDU session is released in the H-SMF. Also, the TS 29.502 gives the case where V-SMF (or SMF in a non-roaming case) sending of N2: NotifySMContext Status to the AMF indicating that a PDU session is released. However, the flow in TS 23.502 implies that the message is preceded by the steps described in the above trigger. Any impact to the Notify event to the xIRI generation is required further analysis.

Table 6.2.3-3: Payload for XIRISMFPDUSessionReleaseMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| supi | SUPI associated with the PDU session. | M |
| pei | PEI associated with the PDU session if available. | C |
| gpsi | GPSI associated with the PDU session if available. | C |
| pduSessionID | PDU Session ID as assigned by the AMF. | M |
| timeOfFirstPacket | Time of first packet as reported in the usage report IE if available in the UPF deletion response (see TS 29.244 [15] clause 7.5.7.2). | C |
| timeOfLastPacket | Time of last packet as reported in the usage report IE if available (see TS 29.244 [15] clause 7.5.7.2). | C |
| uplinkVolume | Number of uplink octets, as reported in the volume report IE if available (see TS 29.244 [15] clause 7.5.7.2). | C |
| downlinkVolume | Number of downlink octets, as reporting the volume report IE if available (see TS 29.244 [15] clause 7.5.7.2). | C |
| locationInformation | Location information, if available. | O |

##### 6.2.3.2.5	Start Of Interception with an established PDU session

The IRI-POI in the SMF shall generate an XIRISMFStartOfInterceptionWithEstablishedPDUSessionMessage when the IRI-POI present in the SMF detects that a PDU session has already been established for the target UE when interception starts.

In a non-roaming scenario, the IRI-POI in the SMF (or in a roaming scenario, the IRI-POI in the V-SMF in the VPLMN) shall generate the xIRI SMF Start Of Interception with established PDU session message when it detects that a new interception for a UE is activated (i.e. provisioned by the LIPF) for the following case:

-	The 5G Session Management (5GSM) state within the SMF for that UE is 5GSM: PDU SESSION ACTIVE or 5GSM: PDU SESSION MODIFICATION PENDING.

NOTE:	The above trigger happens when the SMF (V-SMF in VPLMN) had not sent an N1 NAS message PDU SESSION RELEASE COMMAND to the UE for a PDU session and the SMF (V-SMF in the VPLMN) had previously sent an N1 NAS message PDU SESSION ESTABLISHMENT ACCEPT to that UE for the same PDU session.

In a home-routed roaming scenario, the IRI-POI in the H-SMF shall generate the xIRI Start Of Interception with established PDU session message when it detects that a new interception for a UE is activated (i.e. provisioned by the LIPF) for the following case:

-	The H-SMF had not sent an Nsmf_PDU_Session_Update Request (n1SmInfoToUe: PDU SESSION RELEASE COMMAND) to the V-SMF for a PDU session and H-SMF had previously sent an Nsmf_PDU_Session_Create response (n1SmInfoToUE: PDU SESSION ESTABLISHMENT ACCEPT) to the V-SMF for that PDU session.

The IRI-POI in the SMF shall generate the xIRI SMF Start Of Interception with established PDU session message or each of the PDU sessions (that meets the above criteria) associated with the newly identified target UEs.

Table 6.2.3-4: Payload for XIRISMFStartOfInterceptionWithEstablishedPDUSessionMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| supi | SUPI associated with the PDU session (e.g. as provided by the AMF in the associated Nsmf_PDU_Session_CreateSMContext service operation). Shall be present except for PEI-only unauthenticated emergency sessions. | C |
| supiUnauthenticated | Shall be present if a SUPI is present in the message, and set to “true” if the SUPI has not authenticated, or “false” if it has been authenticated. | C |
| pei | PEI associated with the PDU session if available. | C |
| gpsi | GPSI associated with the PDU session if available. | C |
| pduSessionID | PDU Session ID as assigned by the AMF, as defined in TS 24.007 [14] clause 11.2.3.1b. | M |
| gtpTunnelID | Contains the F-TEID identifying the tunnel used to encapsulate the traffic, as defined in TS 29.244 [15] clause 8.2.3. Non-GTP encapsulation is for further study. | M |
| pduSessionType | Identifies selected PDU session type, see TS 24.501 [13] clause 9.11.4.11. | M |
| sNSSAI | Slice identifier associated with the PDU session, if available. See TS 23.003 [19] clause 28.4.2 and TS 23.501 [2] clause 5.12.2.2. | C |
| ueEndpoint | UE endpoint address(es) if available. | C |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, if available. | C |
| locationInformation | Location information provided by the AMF at session establishment, if available. | C |
| dnn | Data Network Name associated with the target traffic, as defined in TS 23.003 [19] clause 9A and described in TS 23.501 [2] clause 4.3.2.2. | M |
| amfID | Identifier of the AMF associated with the target UE, as defined in TS 23.003 [19] clause 2.10.1. | M |
| hSMFURI | URI of the Nsmf_PDUSession service of the selected H-SMF, if available. See TS 29.502 [16] clause 6.1.6.2.2. | C |
| requestType | Type of request as described in TS 24.501 [13] clause 9.11.3.47 if available. | C |
| accessType | Access type associated with the session (i.e. 3GPP or non-3GPP access) if provided by the AMF (see TS 24.501 [13] clause 9.11.3.11). | C |
| ratType | RAT type associated with the access if provided by the AMF as part of session establishment (see TS 23.502 [4] clause 4.3.2). Values given as per TS 29.571 [17] clause 5.4.3.2. | C |
| smPDUDNRequest | Contents of the SM PDU DN request container, if available, as described in TS 24.501 [13] clause 9.11.4.15. | C |

#### 6.2.3.3	Triggering of the UPF from SMF over LI_T3

When interception of communication contents is required, the CC-TF present in the SMF sends a trigger to the CC-POI  present in the UPF over the LI_T3 interface.

When the CC-TF in the SMF detects that a PDU session has been established for a target UE, it shall send an activation message to the CC-POI in the UPF over the LI_T3 interface. The activation message shall contain the correlation identifiers that the CC-POI in the UPF shall use when emitting xCC. This can be achieved by sending an ActivateTask message as defined in ETSI TS 103 221-1 [7] clause 6.2.1 with the following details:

Table 6.2.3-5: ActivateTask message for triggering the CC-POI in the UPF

| ETSI TS 103 221-1 Field name | Value | M/C/O |
| --- | --- | --- |
| XID | Set to the same XID associated with the interception in the SMF. NOTE: It is for further study how the CC-TF and IRI-POI in the SMF agree on the XID. | M |
| TargetIdentifiers | Packet detection criteria as determined by the CC-TF in the SMF, which enables the UPF to isolate target traffic. The UPF CC-POI shall support at least the following identifier types: GTP Tunnel ID. IPv4 addressIPv6 address NOTE: This value is the target identifier for the UPF CC-POI, and may be different from the target identifier specified in the warrant. | M |
| DeliveryType | Set to “X3Only”. | M |
| ListOfDIDs | Delivery endpoints for LI_X3. These delivery endpoints shall be configured by the CC-TF in the SMF using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to first use. | M |
| CorrelationNumber | Correlation ID to assign to X3 PDUs generated by the CC-POI in the UPF. | M |

When the CC-TF in the SMF detects that a targeted PDU session has changed in a way which requires changes to the interception by the CC-POI in the UPF, the CC-TF shall modify the interception at the CC-POI in the UPF over the LI_T3 interface. This is achieved by sending an ModifyTask message as defined in ETSI TS 103 221-1[ [7] clause 6.2.2 with the following details.

Table 6.2.3-6: ModifyTask message for updating interception at the CC-POI in the UPF

| Field name | Value | M/C/O |
| --- | --- | --- |
| XID | Set to the XID associated with the interception. | M |
| TargetIdentifiers | Updated packet detection criteria as determined by the CC-TF in the SMF. NOTE: See notes on TargetIdentifiers in Table 6.2.3-5. | M |

When the CC-TF in the SMF detects that the PDU session has been released for a target UE, it shall send a deactivation message to the CC-POI in the UPF over the LI_T3 interface. When using ETSI TS 103 221-1 [7] this is achieved by sending a DeactivateTask message with the XID field set to the XID associated with the interception, as described in ETSI TS 103 221-1 [7] clause 6.2.3.

#### 6.2.3.4	Generation of xIRI at UPF over LI_X2

Editor’s Note – Someone who knows about Packet Data Header Reporting probably ought to write this…

Editor’s Note – also need a section on use of LI_T2 to trigger Packet Data Header Reporting in the UPF.

#### 6.2.3.5	Generation of xCC at UPF over LI_X3

The CC-POI present in the UPF shall emit X3 PDUs over LI_X3 for each IP packet matching the criteria specified in the Triggering message received over LI_T3 from the CC-TF in the SMF.

NOTE:	Implementers are reminded of the completeness and non-duplication requirements (see TS 33.127 [5]).

Editor’s Note: What do we do with packets that reached the UPF on N6 from the DN, but were not delivered to the UE?

Each X3 PDU shall contain the contents of the GTP-U packet given using the GTP-U payload format.

#### 6.2.3.6	Generation of IRI over LI_HI2

When an SMF xIRI PDU is received over LI_X2, the MDF2 shall send an IRI message over LI_HI2 without undue delay.

The timestamp field of the ETSI TS 102 232-1 [9] PSHeader structure shall be set to the time that the AMF event was observed (i.e. the timestamp field of the LI_X2 PDU). The LIID and CID fields shall correctly reflect the target identity and communication session to which the IRI belongs.

The threeGPPIRIContents field shall be populated with the BER-encoded PDU.

#### 6.2.3.7	Generation of CC over LI_HI3

When an UPF xCC PDU is received over LI_X3, the MDF3 shall emit a CC PDU over LI_HI3 without undue delay.

The timestamp field of the ETSI TS 102 232-1 [9] PSHeader structure shall be set to the time that the UPF observed the data (i.e. the timestamp field of the LI_X3 PDU). The LIID and CID fields shall correctly reflect the target identity and communication session to which the CC PDU belongs.

### 6.2.4	LI at UDM for 5G

#### 6.2.4.1	General description

In 5G packet core network, the UDM provides the unified data management for UE. The UDM shall have LI capabilities to generate the target UE’s service area registration related xIRI. See clause 7.2.2 for the details.

### 6.2.5	LI at SMSF

#### 6.2.5.1	Provisioning over LI_X1

The IRI-POI present in the SMSF is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI in the SMSF shall support the following Target Identifier Formats in the ETSI TS 103 221-1 [7] messages:

-	SUPI.

-	PEI.

-	GPSI.

#### 6.2.5.2	Generation of xIRI over LI_X2

The IRI-POI present in the SMSF shall send X2 PDUs over LI_X2 for each of the events listed in TS 33.127 [5] clause 6.2.5.3, each of which is described in the following clauses.

#### 6.2.5.3	SMS

The IRI-POI in the SMSF shall generate an xIRI SMS for each SMS message.

Table 6.2.5-1: Payload for XIRISMSMessage

| Field name | Description | M/C/O |
| --- | --- | --- |
| originatingSMSParty | Identity of the originating SMS party. | M |
| terminatingSMSParty | Identity of the terminating SMS party. | M |
| direction | Direction of the SMS with respect to the target. | M |
| transferStatus | Indicates whether the transfer succeeded or not. | M |
| otherMessage | In the event of a server-initiated transfer, indicates whether the server will send another SMS. May be omitted if the transfer is target-initiated. | M |
| otherNetworkFunctionAddress | Address of the other network function (SMS-GMSC/IWMSC/SMS-Router) involved in the communication of the SMS, if available. | C |
| otherNetworkFunctionType | Type of the other network function (SMS-GMSC/IWMSC/SMS-Router) involved in the communication of the SMS, if available. | C |
| location | Location information associated with the UE sending or receiving the SMS, if available. | C |
| smsTPDUData | SMS TPDU, encoded as per TS 23.040 [18] clause 9. EDITOR’S NOTE: The IRI-POI in the SMSF is required to support suppression of TP-User-Data, but the mechanism for handing this over is FFS. | M |

#### 6.2.5.4	Generation of IRI over LI_HI2

When an SMSF xIRI message is received over LI_X2, the MDF2 shall send an IRI message over LI_HI2 without undue delay.

The timestamp field of the psHeader structure shall be set to the time that the SMSF event was observed (i.e. the timestamp field of the X2 PDU). The LIID and CID fields shall correctly reflect the target identity and communication session to which the IRI belongs.

National regulations may require that the MDF2 removes information regarded as content from the smsPayload field in case of an IRI only warrant. The details of what must be removed and under what circumstances are for national regulation and outside the scope of the present document.

### 6.2.6	LI support at NRF

The SIRF present within the NRF provides SBA-related information to the LIPF over the LI_SI interface. Details for this interface are not considered in the present document and are for further study.

## 6.3	4G

The Present document does not specify details of the LI interfaces for 4G / LTE. Details for this release are specified in TS 33.108 [12].

## 6.4	3G

The Present document does not specify details of the LI interfaces for 3G / UMTS. Details for this release are specified in TS 33.108 [12].

# 7	Service Layer Based Interception

Editor’s note: Stage 3 for 33.127 section 7, describing any remaining specific fields, behaviours or details not covered in section 5

Editor’s note: Parameters in this section to have common drafting rules.

## 7.1	Introduction

This clause describes any remaining fields, behaviours or details necessary to implement the required LI interfaces for specific 3GPP-defined services not already described in clauses 4 and 5.

## 7.2	Central Subscriber Management

### 7.2.1	General description

### 7.2.2	LI at UDM

#### 7.2.2.1	General description

Editor’s Note: TBD

#### 7.2.2.2	Provisioning over LI_X1

The IRI-POI present in the UDM is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

#### 7.2.2.3	Generation of xIRI over LI_X2

#### 7.2.2.3.1	General description

The IRI-POI present in the UDM shall emit X2 PDUs over LI_X2 for each of the events listed in TS 33.127 [5] clause 7.2.2.4, each of which is described in the following clauses.

##### 7.2.2.3.2	Serving system xIRI

##### 7.2.2.3.3	Subscriber record change xIRI

##### 7.2.2.3.4	Cancel location xIRI

##### 7.2.2.3.5	LocationiInformation request

#### 7.2.2.4	Generation of IRI over LI_HI2

When an UDM xIRI message is received over LI_X2, the MDF2 shall emit an IRI message over LI_HI2 without undue delay.

The timestamp field of the psHeader structure shall be set to the time that the UDM event was observed (i.e. the timestamp field of the X2 PDU). The LIID and CID fields shall correctly reflect the target identity and communication session to which the IRI belongs.

### 7.2.3	LI at HSS

The present document does not specify details of the LI interfaces at the HSS. Details for this release are specified in TS 33.108 [12].

## 7.3	Location

Editor’s Note: Some thought required here…we need a section for LALS, but not sure about the other parts of 7.3 in 33.127?

### 7.3.3	Lawful Access Location Services (LALS)

#### 7.3.3.1	General description

The LALS architecture and functionality is specified in [5], clause 7.3.3.

#### 7.3.3.2	Provisioning over LI_X1

#### 7.3.3.2.1	Target positioning service

For the LALS target positioning service ([5], clause 7.3.3.2) the IRI-POI provided by the LI-LCS client is directly provisioned over LI_X1 by the LIPF using the LI_X1 protocol as described in clause 5.2.2 with the TaskDetailsExtensions field of the ActivateTask message specifying the type of the target positioning request, immediate vs periodic, and, in the latter case, the periodicity of the positioning requests.

Based on national regulatory requirements and CSP policy, the TaskDetailsExtensions may also include the QoS parameters (specified in OMA-TS-MLP [20]) for the use on the Le interface towards the LCS Server/GMLC. Alternatively, the QoS parameters may be statically configured in the LI-LCS client.

Table 7.3.3.2-1 shows the details of the LI_X1 ActivateTask message used for the LI-LCS client provisioning for the target positioning service.

The LI_X1 DeactivateTask shall be issued by the LIPF to terminate the target positioning service and withdraw the associated provisioning data, except for the Immediate target positioning service in which case the LI_X1 DeactivateTask is not used.

Editor’s Note: Need to consider an additional attribute for the “ActivateTask” in TS 103 221-1 to explicitly indicate to the receiver that it may deactivate the task itself, in cases like the Immediate Target Positioning. A “TaskDeactivationNotification” may also be considered to report the self-deactivations.

Table 7.3.3.2-1: ActivateTask message for LI-LCS client target positioning provisioning

| ETSI TS 103 221-1 Field name | Value | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. | M |
| TargetIdentifiers | One of the following: SUPI. PEI. GPS.I | M |
| DeliveryType | Set to “X2Only”. | M |
| ListOfDIDs | Delivery endpoints for LI_X2. These delivery endpoints are configured using the CreateDestination message as described in ETSI TS 103 221-1 [7], clause 6.3.1 prior to the task activation. | M |
| TaskDetailsExtensions/ PositioningServiceType | “Immediate” or “Periodic”. | M |
| TaskDetailsExtensions/ PositioningPeriodicity | Time interval between the positioning requests in case of Periodic positioning. | C |
| TaskDetailsExtensions/ PositioningParameters | Set of parameters (requested location type, requested response type (SYNC vs ASYNC) and timing, QoS (max location age, accuracy and QoS Class), mapData request, velocity request), as per OMA-TS-MLP [20]. | O |

Editor’s Note: the exact XSD for the “TaskDetailsExtensions” is TBD. It needs to be a common namespace for the “3GPP” “owner” of the extensions, i.e., for all TS 33.128 use cases requiring task extensions (LI_X1 for MDF, LI_T2/T3).

#### 7.3.3.2.2	Triggered location service

For the LALS triggered location service ([5], clause 7.3.3.3) the IRI-TF (LTF) is provisioned by the LIPF using the LI_X1 protocol as described in clause 5.2.2. The “TaskDetailsExtensions” parameter of the ActivateTask message in this case will carry the address of LI-LCS client to be used for the service and, optionally, the positioning parameters for use on the Le interface, similar to the target positioning provisioning.

Table 7.3.3.2-2 shows the details of the LI_X1 ActivateTask message used for the LTF provisioning for the Triggered Location service.

Table 7.3.3.2-2: ActivateTask message for LTF triggered location service provisioning

| ETSI TS 103 221-1 Field name | Value | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. | M |
| TargetIdentifiers | One of the following: SUPI. PEI. GPSI. | M |
| DeliveryType | Set to “X2Only” | M |
| ListOfDIDs | Delivery endpoints for LI-LCS Client LI_X2 messages. These delivery endpoints are configured in LTF using the CreateDestination message as described in ETSI TS 103 221-1 [7], clause 6.3.1 prior to the task activation. | M |
| TaskDetailsExtensions/ LI-LCSClientAddress | The address of the LI-LCS Client for triggering. | M |
| TaskDetailsExtensions/ PositioningParameters | Set of parameters (requested location type, requested response type (SYNC vs ASYNC) and timing, QoS (max location age, accuracy and QoS Class), mapData request, velocity request), as per OMA-TS-MLP [20]. | O |

#### 7.3.3.3	Triggering over LI_X2

An LTF, provisioned as described in clause 7.3.3.2.2, triggers the triggered IRI-POI provided by the LI-LCS client using the LI_T2 protocol as described in clause 5.2.4. The “TaskDetailsExtensions” in the LI_T2 “ActivateTask” message carries the positioning parameters mapped from the LTF provisioning over the LI_X1. The LI_T2 “ActivateTask” message header may include a correlation ID from the triggering xIRI event, if available.

Prior to issuing one or more “ActivateTask” requests towards an LI-LCS Client, the LPF shall provision the LI-LCS client with the LI_X2 destinations by using the “CreateDestination” operation(s), as per clause 5.2.2.

The LI-LCS client shall implicitly deactivate the task upon issuing the final xIRI message for the trigger. There is no DeactivateTask operation on the LI_T2 for the LI-LCS client.

The Table 7.3.3.3-1 shows the details of the LI_T2 ActivateTask message used by the LTF to trigger LI-CS client for the triggered location service.

Table 7.3.3.3-1: ActivateTask message from LTF to LI-LCS client for the triggered location service triggering

| ETSI TS 103 221-1 Field name | Value | M/C/O |
| --- | --- | --- |
| XID | The same value as in the LTF provisioning (clause 7.3.3.2.2). | M |
| TargetIdentifiers | One of the following, per LTF provisioning: SUPI. PEI. GPSI. | M |
| DeliveryType | Set to “X2Only”. | M |
| ListOfDIDs | Delivery endpoints identifiers for LI-LCS Client LI_X2 messages. These delivery endpoints are configured in LTF using the CreateDestination message as described in ETSI TS 103 221-1 [7], clause 6.3.1 prior to the task activation. | M |
| TaskDetailsExtensions/ PositioningParameters | Set of parameters (requested location type, requested response type (SYNC vs ASYNC) and timing, QoS (max location age, accuracy and QoS Class), mapData request, velocity request), as per OMA-TS-MLP [20], as specified in the LTF provisioning message over LI_X1. | C |

#### 7.3.3.4	Generation of xIRI over LI_X2

The IRI-POI provided by the LI-LCS client shall deliver the target location reports to respective MDF(s) as xIRI messages over the LI_X2 interface.

Table 7.3.3.4-1: Payload for the LALS Report xIRI message

| Field name | Description | M/C/O |
| --- | --- | --- |
| sUPI | SUPI of the target, if used for the service. | C |
| pEI | PEI of the target, if used for the service. | C |
| gPSI | GPSI of the target, if used for the service. | C |
| location | location of the target, if obtained successfully. | C |
| rawMLPResponse | The parameter contains a copy of the unparsed XML code of MLP response message, i.e. the entire XML document containing a <slia> (described in OMA-TS-MLP [20], clause 5.2.3.2.2) or a <slirep> (described in OMA-TS-MLP [20], clause 5.2.3.2.3) MLP messages. This parameter is present when the LI-LCS client cannot fully map the MLP response message into the ASN.1 LocationInformation object. | C |
| mLPErrorCode | Error code in the case of positioning failure. The error codes are the OMA MLP result identifiers defined in OMA-TS-MLP [20], Clause 5.4. | C |

The LI_X2 header (as per clause 5.3.2) of the LALS report xIRI message presented in the Table 7.3.3.4-1, shall contain the correlation ID (if provided) from a respective LI_T2 ActivationTask message.

#### 7.3.3.5	Generation of IRI over LI_HI2

The XIRILALSReportMessage payload, defined in clause 7.3.3.4, shall be used as the payload of the respective LI_H2 IRI LALS report message, no payload mediation is required.

NOTE: 	In some specific scenarios the amount of LALS reports data may overload the LI-HI2 and/or LI_X2 interfaces. To prevent the overload, a flow control for LALS Ttriggered location reports may be implemented in MDF and/or LI-LCS client, e.g. by limiting the frequency of the reports for individual targets.

### 7.3.4	Cell database information reporting

#### 7.3.4.1	General description

#### 7.3.4.2	Generation of IRI over HI2

Annex A (normative): Structure of the Internal Interface

Editor’s Note: Structure of the LI_X, LI_T & LI_SI etc interfaces similar to Annex B.

The threeGPPCCContents field shall be populated with the BER-encoded PDU as follows.

UPFPDURecord ::= OCTET STRING

The message shall contain the following payload, encoded as per clause 5.3.2.

XIRIAMFRegistrationMessage ::= SEQUENCE

{

-- See clause 6.2.2.2.2 for details of this structure

registrationType			[1] AMFRegistrationType,

registrationResult			[2]	AMFRegistrationResult,

sliceInformation			[3]	SliceInformation			OPTIONAL,

supi						[4]	SUPI,

suci						[5] SUCI 						OPTIONAL,

pei							[6] PEI							OPTIONAL,

gpsi						[7] GPSI 						OPTIONAL,

guti						[8]	GUTI,

location					[9]	LocationInformation 		OPTIONAL,

non3GPPAccessEndpoint 		[10] UEEndpointAddress 			OPTIONAL

}

AMFRegistrationType	::=	ENUMERATED

{

initial		(1),

mobility	(2),

periodic	(3),

emergency	(4)

}

AMFRegistrationResult	::= ENUMERATED

{

threeGPPAccess					(1),

nonThreeGPPAccess				(2),

threeGPPAndNonThreeGPPAccess	(3)

}

SliceInformation	::= SEQUENCE

{

allowedNSSAI					(1)	NSSAI			OPTIONAL,

configuredNSSAI					(2)	NSSAI			OPTIONAL,

rejectedNSSAI					(3)	RejectedNSSAI	OPTIONAL

}

The message shall contain the following payload, encoded as per clause 5.3.2.

XIRIAMFDeregistrationMessage ::= SEQUENCE

{

-- See clause 6.2.2.2.3 for details of this structure

deregistrationDirection	[1]	AMFDirection,

accessType				[2]	AMFAccessType,

supi					[3]	SUPI					OPTIONAL,

suci					[4]	SUCI					OPTIONAL,

pei						[5]	PEI						OPTIONAL,

gpsi					[6] GPSI					OPTIONAL,

guti					[7]	GUTI					OPTIONAL,

cause					[8]	FiveGMMCause			OPTIONAL,

location				[9] LocationInformation		OPTIONAL

}

AMFDirection	::=	ENUMERATED

{

networkInitiated 	(1),

ueInitiated			(2)

}

AMFAccessType	::=	ENUMERATED

{

threeGPPAccess					(1),

nonThreeGPPAccess				(2),

threeGPPandNonThreeGPPAccess	(3)

}

FiveGMMCause	::= INTEGER (0..255)

The XIRIAMFLocationUpdateMessage message shall contain the following payload, encoded as per clause 5.3.2.

XIRIAMFLocationUpdateMessage ::= SEQUENCE

{

-- See clause 6.2.2.2.4 for details of this structure

supi				[1]	SUPI,

suci				[2]	SUCI				OPTIONAL,

pei					[3]	PEI					OPTIONAL,

gpsi				[4] GPSI				OPTIONAL,

guti				[5]	GUTI				OPTIONAL,

location			[6] LocationInformation

}

The message shall contain the following payload, encoded as per clause 5.3.2.

XIRIAMFStartOfInterceptionWithRegisteredUEMessage ::= SEQUENCE

{

-- See clause 6.2.2.2.5 for details of this structure

registrationResult			[1]		AMFRegistrationResult,

registrationType			[2]		AMFRegistrationType			OPTIONAL,

sliceInformation	[3]		SliceInformation 	OPTIONAL,

supi						[4]		SUPI,

suci						[5] 	SUCI						OPTIONAL,

pei							[6] 	PEI						OPTIONAL,

gpsi						[7] 	GPSI						OPTIONAL,

guti						[8]		GUTI,

location					[9]		LocationInformation 		OPTIONAL,

non3GPPAccessEndpoint 		[10] 	UEEndpointAddress 			OPTIONAL,

timeOfRegistration			[11]	Timestamp 					OPTIONAL

}

AMFRegistrationResult	::= ENUMERATED

{

threeGPPAccess					(1),

nonThreeGPPaccess				(2),

threeGPPAndNonThreeGPPAccess	(3)

}

AMFRegistrationType	::=	ENUMERATED

{

initial		(1),

mobility	(2),

periodic	(3),

emergency	(4)

}

SliceInformation	::= SEQUENCE

{

allowedNSSAI					(1)	NSSAI			OPTIONAL,

configuredNSSAI					(2)	NSSAI			OPTIONAL,

rejectedNSSAI					(3)	RejectedNSSAI	OPTIONAL

}

The message shall contain the following payload, encoded as per clause 5.3.2.

XIRIAMFUnsuccessfulProcedure ::= SEQUENCE

{

-- See clause 6.2.2.2.6 for details of this structure

failedprocedureType			[1] FailedAMFProcedureType,

failurecause				[2]	AMFFailureCause,

requestedSliceInformation	[3]	NSSAI	OPTIONAL,

supi						[4]	SUPI						OPTIONAL,

suci						[5] SUCI						OPTIONAL,

pei							[6] PEI							OPTIONAL,

gpsi						[7] GPSI						OPTIONAL,

guti						[8]	GUTI						OPTIONAL,

location					[9]	LocationInformation 		OPTIONAL

}

FailedAMFProcedureType ::= ENUMERATED

{

registration            (1),

deregistration          (2),

locationupdate          (3),

sms                     (4),

pDUSessionEstablishment (5)

}

AMFFailureCause	::= CHOICE

{

fiveGMMCause 	[1]		FiveGMMCause,

fiveGSMCause	[2]		FiveGSMCause

}

FiveGMMCause	::= INTEGER (0..255)

FiveGSMCause	::= INTEGER (0..255)

The message shall contain the following payload, encoded as per clause 5.3.2.

XIRISMFPDUSessionEstablishmentMessage ::= SEQUENCE

{

-- See clause 6.2.3.2.2 for details of this structure

supi					[0]		SUPI 							OPTIONAL,

supiUnauthenticated 	[1] 	SUPIUnauthenticatedIndication	OPTIONAL,

pei						[2] 	PEI 							OPTIONAL,

gpsi					[3] 	GPSI 							OPTIONAL,

pduSessionID			[4] 	PDUSessionID,

gtpTunnelID				[5] 	FTEID,

pduSessionType			[6]		PDUSessionType,

sNSSAI					[7] 	SNSSAI OPTIONAL,

ueEndpoint				[8] 	SEQUENCE OF UEEndpointAddress	OPTIONAL,

non3GPPAccessEndpoint 	[9] 	UEEndpointAddress 				OPTIONAL,

locationInformation		[10]	Location 						OPTIONAL,

dnn						[11] 	DNN,

amfID					[12] 	AMFID							OPTIONAL,

hSMFURI					[13] 	HSMFURI							OPTIONAL,

requestType				[14] 	FiveGSMRequestType,

accessType				[15] 	AMFAccessType 					OPTIONAL,

ratType					[16] 	RATType 						OPTIONAL,

smPDUDNRequest			[17] 	SMPDUDNRequest 					OPTIONAL

}

PDUsessionType ::= ENUMERATED

{

ipv4 			(1),

ipv6 			(2),

ipv4v6 			(3),

unstructured	(4),

ethernet		(5)

}

PDUSessionID ::= INTEGER (0..255)

SNSSAI ::= SEQUENCE

{

sliceServiceType 	[0] INTEGER (0..255),

sliceDifferentiator	[1]	OCTET STRING (SIZE(3)) OPTIONAL

}

FTEID ::= SEQUENCE

{

teid		[0] INTEGER (0.. 4294967295),

ipv4Address	[1] IPv4Address OPTIONAL,

ipv6Address	[2] IPv6Address OPTIONAL

}

UEEndpointAddress ::= CHOICE

{

ipv4Address		[1] IPv4Address,

ipv6Address		[2] IPv6Address,

ethernetAddress	[3] MACAddress

}

SMPDUDNRequest ::= OCTET STRING

DNN ::= UTF8STRING

SUPIUnauthenticatedIndication ::= BOOLEAN

AMFID ::= SEQUENCE

{

amfRegionID		[1] INTEGER (0..255),

amfSetID		[2] INTEGER (0..1023),

amfPointer		[3] INTEGER (0..63)

}

HSMFURI ::= UTF8String

RATType ::= ENUMERATED

{

nr(0),

eutra(1),

wlan(2),

virtual(3)

}

FiveGSMRequestType ::= ENUMERATED

{

initialRequest(0),

existingPDUSession(1),

initialEmergencyRequest(2),

existingEmergencyPDUSession(3),

modificationRequest(4),

reserved(5)

}

The message shall contain the following payload, encoded as per clause 5.3.2.

XIRISMFPDUSessionModificationMessage ::= SEQUENCE

{

- See clause 6.2.3.2.3 for details of this structure

- supi						[0]	SUPI 							OPTIONAL,

- supiUnauthenticated		[1] SUPIUnauthenticatedIndication	OPTIONAL,

- pei						[2] PEI 							OPTIONAL,

- gpsi					[3] GPSI 							OPTIONAL,

- sNSSAI					[4] SNSSAI 							OPTIONAL,

- non3GPPAccessEndpoint 	[5] UEEndpointAddress 				OPTIONAL,

- locationInformation		[6]	Location 						OPTIONAL,

- requestType				[7] FiveGSMRequestType,

- accessType				[8] AMFAccessType 					OPTIONAL,

- ratType					[9] RATType 						OPTIONAL

}

The message shall contain the following payload, encoded as per clause 5.3.2.

XIRISMFPDUSessionReleaseMessage ::= SEQUENCE

{

- See clause 6.2.3.3.4 for details of this structure

supi				[0]	SUPI,

pei					[1] PEI 			OPTIONAL,

gpsi				[2] GPSI 			OPTIONAL,

pduSessionID		[3] PDUSessionID,

timeOfFirstPacket	[4] GeneralizedTime OPTIONAL,

timeOfLastPacket	[5] GeneralizedTime OPTIONAL,

uplinkVolume		[6] INTEGER 		OPTIONAL,

downlinkVolume		[7] INTEGER 		OPTIONAL,

locationInformation	[8]	Location 		OPTIONAL

}

The message shall contain the following payload, encoded as per clause 5.3.2.

XIRISMFStartOfInterceptionWithEstablishedPDUSessionMessage ::= SEQUENCE

{

-- See clause 6.2.3.3.5 for details of this structure

supi					[0]	SUPI 							OPTIONAL,

supiUnauthenticated 	[1] SUPIUnauthenticatedIndication	OPTIONAL,

pei						[2] PEI 							OPTIONAL,

gpsi					[3] GPSI 							OPTIONAL,

pduSessionID			[4] PDUSessionID,

gtpTunnelID				[5] FTEID,

pduSessionType			[6]	PDUSessionType,

sNSSAI					[7] SNSSAI 							OPTIONAL,

ueEndpoint				[8] SEQUENCE OF UEEndpointAddress,

non3GPPAccessEndpoint 	[9] UEEndpointAddress 				OPTIONAL,

locationInformation		[10] Location 						OPTIONAL,

dnn						[11] DNN,

amfID					[12] AMFID							OPTIONAL,

hSMFURI					[13] HSMFURI						OPTIONAL,

requestType				[14] FiveGSMRequestType,

accessType				[15] AMFAccessType 					OPTIONAL,

ratType					[16] RATType 						OPTIONAL,

smPDUDNRequest			[17] SMPDUPDNRequest				OPTIONAL

}

The XIRISMSMessage shall contain the following payload, encoded as per clause 5.3.2.

XIRISMSMessage ::= SEQUENCE

{

-- See clause 6.2.5.3 for details of this structure

originatingSMSParty		[1] SMSParty,

terminatingSMSParty		[2] SMSParty,

direction				[3] SMSDirection,

transferStatus			[4] SMSTransferStatus,

otherMessage			[5] SMSOtherMessageIndication 	OPTIONAL,

location				[6]	LocationInformation 		OPTIONAL,

otherNFAddress			[7] SMSNFAddress 				OPTIONAL,

otherNFType				[8] SMSNFType 					OPTIONAL,

smsTPDUData					[9] SMSTPDUData					OPTIONAL

}

SMSParty ::= SEQUENCE

{

supi	[0] SUPI 	OPTIONAL,

pei		[1] PEI		OPTIONAL,

gpsi	[2] GPSI	OPTIONAL

}

SMSDirection ::= ENUMERATED

{

toTarget 	(0),

fromTarget 	(1)

}

SMSTransferStatus ::=  ENUMERATED

{

transferSucceeded 	(0),

transferFailed		(1),

undefined 			(2)

}

SMSOtherMessageIndication ::= BOOLEAN

SMSNFAddress ::= CHOICE

{

ipAddress	[0] IPAddress,

e164Number	[1] E164Number
}

SMSNFType ::= ENUMERATED

{

sms-GMSC	(0),

iwMSC		(1),

sms-Router	(2)

}

SMSTPDUData ::= CHOICE

{

smsTPDU [0] SMSTPDU

}

SMSTPDU ::= OCTET STRING (SIZE(1..270))

The message shall be encoded as per clause 5.3.2, with the following payload:

XIRILALSReportMessage ::= SEQUENCE

{

sUPI						[1]	SUPI 					OPTIONAL,

pEI							[2] PEI 					OPTIONAL,

gPSI						[3] GPSI 					OPTIONAL,

location					[4]	LocationInformation		OPTIONAL,

rawMLPResponse				[5] RawMLPResponse 			OPTIONAL,

mLPErrorCode				[6] MLPErrorCode 			OPTIONAL

}

RawMLPResponse ::= UTF8String

MLPErrorCode ::= INTEGER (1..699)

Annex B (normative): Structure of the Handover Interface

Editor’s Note: Equivalent of ANNEX B in 33.108

Editor’s Note: Need to agree whether this is ASN.1 or something else

Editor’s Note: If ASN.1, Need to structure ASN.1 with a single common module structure such that parameters are not repeated unnecessarily for different technology scenarios.

Editor’s Note: As far as practical, message syntax (e.g. parameters formats etc) should be described alongside the semantics in the appropriate sections (5, 6 or 7) to relieve a developer of the burden of flicking between sections (and relieve the editor of the burden of maintaining details across these sections).

## B.X	LI_HI4 Generic LI notification

Editor’s Note: Assign clause number after LI_HI1/2/3 are inserted.

Based on clause 5.6 of the present document, this clause defines a system of management notification of LI system with the LI_HI4.

The LI_HI4 shall transport specific LI service O&M information from the CSP n to the LEMF. The individual notification parameters shall be coded using ASN.1 and the basic encoding rules (BER). The delivery of LI_HI4 shall be performed directly using the same mechanism as used for LI_HI2 and LI_HI3, in order to limit and to protect the MDF2/3 in terms of the number of interfaces to any other CSP's O&M.

If the LI_HI4 is used, it shall be used for LI management to send electronic notification to the LEMF in the following cases:

1)	after the activation of lawful interception;

2)	after the deactivation of lawful interception

3)	after the modification of an active lawful interception.

Table B.X.1: LI_HI4 notification message

| Field name | Description | M/C/O |
| --- | --- | --- |
| notificationType | Information on the type of notification: activation, deactivation or modification | M |
| deliveryInformation | Delivery Information which has been decided by the LEA in terms of delivery numbers, IP addresses for LI_HI2 and LI_HI3 | O |
| appliedTargetID | Target Identifier applied in the POI for the warrant | O |
| appliedStartTime | Start time applied to the POI for the warrant | O |
| appliedEndTime | End time applied to the POI for the warrant | O |

There shall be no control over the CSP's equipment via LI_HI4.

The individual notifications parameters shall be sent to the LEMF as soon as possible with the lowest latency at least once (if available).

The MDF2/3 will deliver the LI_HI4 notification operation to LEMF.

HI4Notification ::= SEQUENCE

{

notificationType				[1] NotificationType,

appliedTargetID					[2] TargetIdentifier			OPTIONAL,

appliedDeliveryInformation		[3] AppliedDeliveryInformation	OPTIONAL,

appliedStartTime				[4] Timestamp					OPTIONAL,

-- The applied start time of the warrant in ADMF.

appliedEndTime					[5] Timestamp					OPTIONAL

-- The applied end time of the warrant in ADMF.

}

NotificationType	::= ENUMERATED

{

activation		(1),

deactivation	(2),

modification	(3)

}

AppliedDeliveryInformation ::= SEQUENCE OF

{

hi2DeliveryIpAddress 	[1] IPAddress							OPTIONAL,

-- LI_HI2 address of the LEMF. There could be multiple LI_HI2 delivery addresses

hi3DeliveryIpAddress	[2] IPAddress							OPTIONAL,

-- LI_HI3 address of the LEMF. There could be more LI_HI3 delivery addresses

hi2DeliveryPortNumber	[3] PortNumber							OPTIONAL,

-- LI_HI2 portnumber of the LEMF. There could be multiple LI_HI2 delivery port numbers.

hi3DeliveryPortNumber	[4] PortNumber							OPTIONAL

-- LI_HI3 portnumber of the LEMF. There could be multiple LI_HI3 delivery port numbers.

}

Editor’s note: The exact meaning of notification type and the use of the optional parameters needs to be evaluated

Annex C (informative): Implementation Guidance

Editors Note: Guidance that is essential to implementors.

# Annex D (informative): Drafting Rule Guidance

Editor’s Note: Is annex needed?

Editor’s Note: In this draft we’ve tried to follow convention of camel-cased field names without punctuation (e.g. targetIdentity, not Target-Identity) and without part-capitalising acronyms (so supi, not sUPI). We can discuss this as the content develops (but suggest we do not fixate for too long on it at the expense of actually getting the details we need).

# Annex Z (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-09 |  |  | - | - | - | First Skeleton Specification | 0.0.0 |
| 2018-10 | SA3LI#70BIS |  | - | - | - | Output of SA3LI#70BIS, implementing comments on initial draft. | 0.0.1 |
| 2018-11 | SA3LI#71 |  | - | - | - | Output of SA3LI#71, implementing outcome of discussion on initial draft with corrections to make consistent with TS 33.127 v1.0.0 | 0.0.2 |
| 2018-11 | Conf Call 14th Nov |  | - | - | - | Output of conf call 14th Nov. | 0.0.3 |
| 2018-11 | Conf Call 28th Nov |  | - | - | - | Output of conf call 28th Nov. | 0.0.4 |
| 2018 -12 | Conf Call 5th Dec |  | - | - | - | Output of conf call 5th Nov, inc c3i180014/15/16/17. | 0.0.5 |
| 2018-12 | Conf Call 17th Dec |  | - | - | - | Output of conf call 17th Dec inc c3i1800018/23. Table and section numbering also revised. | 0.0.6 |
| 2019-01 | Conf Call 9th Jan |  | - | - | - | Output of conf call 9th Jan inc c3i1800021, c3i1900007/9 and editorial corrections. | 0.0.7 |
| 2019-01 | SA3-LI#72 |  | - | - | - | Output of SA3-LI#72 inc S3i190051/52/53/54/55/56/58/47/67/59/60/ 61/62/63. All ASN.1 moved to annex A from sections 6 & 7. | 0.0.8 |
| 2019-02 | Input for SA3LI#72-Bis |  |  |  |  | General check by Koen Jaspers and Carmine (Lino) Rizzo | 0.0.8a |