3GPP TSG-SA3 Meeting #77-LI-e	s3i200153

# Online, , 21st Apr 2020 - 24th Apr 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0080 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for PTC Stage 3 asn.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI16 |  |  |  |  |  | Date: |  |  | 2020-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 33.128 does not provide an LI capability for LI of Push to Talk over Cellular (PTC) type services. Such an LI capability is essential for carriers to meet their LI obligations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of stage three PTC LI events and messages for LI and corresponding asn.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No method for the CSP to deliver LI of PTC services. The CSP cannot meet their obligatory requirements for LI for PTC type services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of MODIFICATION ***

Annex A (normative): Structure of both the Internal and External Interfaces

TS33128Payloads

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) threeGPP(4) ts33128(19) r16(16) version1(1)}

DEFINITIONS IMPLICIT TAGS EXTENSIBILITY IMPLIED ::=

BEGIN

-- =============

-- Relative OIDs

-- =============

xIRIPayloadOID RELATIVE-OID ::= {threeGPP(4) ts33128(19) r16(16) version1(1) xIRI(1)}

xCCPayloadOID RELATIVE-OID ::= {threeGPP(4) ts33128(19) r16(16) version1(1) xCC(2)}

iRIPayloadOID RELATIVE-OID ::= {threeGPP(4) ts33128(19) r16(16) version1(1) iRI(3)}

cCPayloadOID RELATIVE-OID ::= {threeGPP(4) ts33128(19) r16(16) version1(1) cC(4)}

lINotificationPayloadOID RELATIVE-OID ::= {threeGPP(4) ts33128(19) r16(16) version0(0) lINotification(5)}

-- ===============

-- X2 xIRI payload

-- ===============

XIRIPayload ::= SEQUENCE

{

relativeOID         [1] RELATIVE-OID,

event               [2] XIRIEvent

}

XIRIEvent ::= CHOICE

{

-- Access and mobility related events, see clause 6.2.2

registration                                        [1] AMFRegistration,

deregistration                                      [2] AMFDeregistration,

locationUpdate                                      [3] AMFLocationUpdate,

startOfInterceptionWithRegisteredUE                 [4] AMFStartOfInterceptionWithRegisteredUE,

unsuccessfulAMProcedure                             [5] AMFUnsuccessfulProcedure,

-- PDU session-related events, see clause 6.2.3

pDUSessionEstablishment                             [6] SMFPDUSessionEstablishment,

pDUSessionModification                              [7] SMFPDUSessionModification,

pDUSessionRelease                                   [8] SMFPDUSessionRelease,

startOfInterceptionWithEstablishedPDUSession        [9] SMFStartOfInterceptionWithEstablishedPDUSession,

unsuccessfulSMProcedure                             [10] SMFUnsuccessfulProcedure,

-- Subscriber-management related events, see clause 7.2.2

servingSystemMessage                                [11] UDMServingSystemMessage,

-- SMS-related events, see clause 6.2.5

sMSMessage                                          [12] SMSMessage,

-- LALS-related events, see clause 7.3.3

lALSReport                                          [13] LALSReport,

-- PDHR/PDSR-related events, see clause 6.2.3.4.1

pDHeaderReport                                      [14] PDHeaderReport,

pDSummaryReport                                     [15] PDSummaryReport,

-- MMS-related events, see clause 7.X.2.2

mMSSend                                             [16] MMSSend,

mMSNotification                                     [17] MMSNotification,

mMSNotificationResponse                             [18] MMSNotificationResponse,

mMSRetrieval                                        [19] MMSRetrieval,

mMSRetrievalAck                                     [20] MMSRetrievalAck,

mMSForward                                          [21] MMSForward,

mMSStore                                            [22] MMSStore,

mMSUpload                                           [23] MMSUpload,

mMSDelete                                           [24] MMSDelete,

mMSDeliveryNotification                             [25] MMSDeliveryNotification,

mMSReadReply                                        [26] MMSReadReply,

mMSCancel                                           [27] MMSCancel,

mMSViewRequest                                      [28] MMSViewRequest,

mMSViewConfirm                                      [29] MMSViewConfirm,

mMBoxDescription                                    [30] MMBoxDescriptionPDU,

mMSCCPDU                                            [31] MMSCCPDU,

-- PTC-related events, see clause 7.x.2.4

pTCRegistration                                     [32] PTCRegistration,

pTCServingsystem                                    [33] PTCServingSystem,

pTCSessioninitiation                                [34] PTCSessionInitiation,

pTCSessionabandon                                   [35] PTCSessionAbandon,

pTCSessionstart                                     [36] PTCSessionStart,

pTCSessionend                                       [37] PTCSessionEnd,

pTCStartofInterception                              [38] PTCStartofInterception,

pTCPre-establishedSession                           [39] PTCPre-establishedSession,

pTCInstantPersonalAlert                             [40] PTCInstantPersonalAlert,

pTCPartyJoin                                        [41]PTCPartyJoin,

pTCPartyDrop                                        [42]PTCPartyDrop,

pTCPartyHold                                        [43]PTCPartyHold,

pTCMediaModification                                [44]PTCMediaModification,

pTCGroupAdvertisement                               [45]PTCGroupAdvertisement,

pTCFloorControl                                     [46]PTCFloorControl,

pTCTargetPresence                                   [47]PTCTargetPresence,

pTCAssociatePresence                                [48]PTCAssociatePresence,

pTCListManagement                                   [49]PTCListManagement,

pTCAccessPolicy                                     [50]PTCAccessPolicy,

pTCMediaTypeNotification                            [51]PTCMediaTypeNotification,

pTCEncryptionMessage                                [52]PTCEncryptionMessage

}

-- ==============

-- X3 xCC payload

-- ==============

-- No explicit payload required in release 15, see clause 6.2.3.5

-- ===============

-- HI2 IRI payload

-- ===============

IRIPayload ::= SEQUENCE

{

relativeOID         [1] RELATIVE-OID,

event               [2] IRIEvent,

targetIdentifiers   [3] SEQUENCE OF IRITargetIdentifier OPTIONAL

}

IRIEvent ::= CHOICE

{

-- Registration-related events, see clause 6.2.2

registration                                        [1] AMFRegistration,

deregistration                                      [2] AMFDeregistration,

locationUpdate                                      [3] AMFLocationUpdate,

startOfInterceptionWithRegisteredUE                 [4] AMFStartOfInterceptionWithRegisteredUE,

unsuccessfulRegistrationProcedure                   [5] AMFUnsuccessfulProcedure,

-- PDU session-related events, see clause 6.2.3

pDUSessionEstablishment                             [6] SMFPDUSessionEstablishment,

pDUSessionModification                              [7] SMFPDUSessionModification,

pDUSessionRelease                                   [8] SMFPDUSessionRelease,

startOfInterceptionWithEstablishedPDUSession        [9] SMFStartOfInterceptionWithEstablishedPDUSession,

unsuccessfulSessionProcedure                        [10] SMFUnsuccessfulProcedure,

-- Subscriber-management related events, see clause 7.2.2

servingSystemMessage                                [11] UDMServingSystemMessage,

-- SMS-related events, see clause 6.2.5

sMSMessage                                          [12] SMSMessage,

-- LALS-related events, see clause 7.3.3

lALSReport                                          [13] LALSReport,

-- PDHR/PDSR-related events, see clause 6.2.3.4.1

pDHeaderReport                                      [14] PDHeaderReport,

pDSummaryReport                                     [15] PDSummaryReport,

-- MDF-related events, see clause 7.3.4

mDFCellSiteReport                                   [16] MDFCellSiteReport,

-- MMS-related events, see clause 7.X.2.2

mMSSend                                             [17] MMSSend,

mMSNotification                                     [18] MMSNotification,

mMSNotificationResponse                             [19] MMSNotificationResponse,

mMSRetrieval                                        [20] MMSRetrieval,

mMSRetrievalAck                                     [21] MMSRetrievalAck,

mMSForward                                          [22] MMSForward,

mMSStore                                            [23] MMSStore,

mMSUpload                                           [24] MMSUpload,

mMSDelete                                           [25] MMSDelete,

mMSDeliveryNotification                             [26] MMSDeliveryNotification,

mMSReadReply                                        [27] MMSReadReply,

mMSCancel                                           [28] MMSCancel,

mMSViewRequest                                      [29] MMSViewRequest,

mMSViewConfirm                                      [30] MMSViewConfirm,

mMBoxDescription                                    [31] MMBoxDescriptionPDU,

mMSCCPDU                                            [32] MMSCCPDU,

-- PTC-related events, see clause 7.x.2.4

pTCRegistration                                    [33] PTCRegistration,

pTCServingSystem                                   [34] PTCServingSystem,

pTCSessionInitiation                               [35] PTCSessionInitiation,

pTCSessionAbandon                                  [36] PTCSessionAbandon,

pTCSessionStart                                    [37] PTCSessionStart,

pTCSessionEnd                                      [38] PTCSessionEnd,

pTCStartofInterception                             [39] PTCStartofInterception,

pTCPre-establishedSession                          [40] PTCPre-establishedSession,

pTCInstantPersonalAlert                            [41] PTCInstantPersonalAlert,

pTCPartyJoin                                       [42] PTCPartyJoin,

pTCPartyDrop                                       [43] PTCPartyDrop,

pTCPartyHold                                       [44] PTCPartyHold,

pTCMediaModification                               [45] PTCMediaModification,

pTCGroupAdvertisement                              [46] PTCGroupAdvertisement,

pTCFloorControl                                    [47] PTCFloorControl,

pTCTargetPresence                                  [48] PTCTargetPresence,

pTCAssociatePresence                               [49] PTCAssociatePresence,

pTCListManagement                                  [50] PTCListManagement,

pTCAccessPolicy                                    [51] PTCAccessPolicy,

pTCMediaTypeNotification                           [52] PTCMediaTypeNotification,

pTCEncryptionMessage                               [53] PTCEncryptionMessage

}

IRITargetIdentifier ::= SEQUENCE

{

identifier                                          [1] TargetIdentifier,

provenance                                          [2] TargetIdentifierProvenance OPTIONAL

}

-- ==============

-- HI3 CC payload

-- ==============

CCPayload ::= SEQUENCE

{

relativeOID         [1] RELATIVE-OID,

pDU                 [2] CCPDU

}

CCPDU ::= CHOICE

{

uPFCCPDU            [1] UPFCCPDU,

mMSCCPDU            [2] MMSCCPDU,

pTCCCPDU            [3] PTCCCPDU

}

-- ===========================

-- HI4 LI notification payload

-- ===========================

LINotificationPayload ::= SEQUENCE

{

relativeOID         [1] RELATIVE-OID,

notification        [2] LINotificationMessage

}

LINotificationMessage ::= CHOICE

{

lINotification      [1] LINotification

}

-- ==================

-- 5G AMF definitions

-- ==================

-- See clause 6.2.2.2.2 for details of this structure

AMFRegistration ::= SEQUENCE

{

registrationType            [1] AMFRegistrationType,

registrationResult          [2] AMFRegistrationResult,

slice                       [3] Slice OPTIONAL,

sUPI                        [4] SUPI,

sUCI                        [5] SUCI OPTIONAL,

pEI                         [6] PEI OPTIONAL,

gPSI                        [7] GPSI OPTIONAL,

gUTI                        [8] FiveGGUTI,

location                    [9] Location OPTIONAL,

non3GPPAccessEndpoint       [10] UEEndpointAddress OPTIONAL

}

-- See clause 6.2.2.2.3 for details of this structure

AMFDeregistration ::= SEQUENCE

{

deregistrationDirection     [1] AMFDirection,

accessType                  [2] AccessType,

sUPI                        [3] SUPI OPTIONAL,

sUCI                        [4] SUCI OPTIONAL,

pEI                         [5] PEI OPTIONAL,

gPSI                        [6] GPSI OPTIONAL,

gUTI                        [7] FiveGGUTI OPTIONAL,

cause                       [8] FiveGMMCause OPTIONAL,

location                    [9] Location OPTIONAL

}

-- See clause 6.2.2.2.4 for details of this structure

AMFLocationUpdate ::= SEQUENCE

{

sUPI                        [1] SUPI,

sUCI                        [2] SUCI OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

gUTI                        [5] FiveGGUTI OPTIONAL,

location                    [6] Location

}

-- See clause 6.2.2.2.5 for details of this structure

AMFStartOfInterceptionWithRegisteredUE ::= SEQUENCE

{

registrationResult          [1] AMFRegistrationResult,

registrationType            [2] AMFRegistrationType OPTIONAL,

slice                       [3] Slice OPTIONAL,

sUPI                        [4] SUPI,

sUCI                        [5] SUCI OPTIONAL,

pEI                         [6] PEI OPTIONAL,

gPSI                        [7] GPSI OPTIONAL,

gUTI                        [8] FiveGGUTI,

location                    [9] Location OPTIONAL,

non3GPPAccessEndpoint       [10] UEEndpointAddress OPTIONAL,

timeOfRegistration          [11] Timestamp OPTIONAL

}

-- See clause 6.2.2.2.6 for details of this structure

AMFUnsuccessfulProcedure ::= SEQUENCE

{

failedProcedureType         [1] AMFFailedProcedureType,

failureCause                [2] AMFFailureCause,

requestedSlice              [3] NSSAI OPTIONAL,

sUPI                        [4] SUPI OPTIONAL,

sUCI                        [5] SUCI OPTIONAL,

pEI                         [6] PEI OPTIONAL,

gPSI                        [7] GPSI OPTIONAL,

gUTI                        [8] FiveGGUTI OPTIONAL,

location                    [9] Location OPTIONAL

}

-- =================

-- 5G AMF parameters

-- =================

AMFID ::= SEQUENCE

{

aMFRegionID [1] AMFRegionID,

aMFSetID    [2] AMFSetID,

aMFPointer  [3] AMFPointer

}

AMFDirection ::= ENUMERATED

{

networkInitiated(1),

uEInitiated(2)

}

AMFFailedProcedureType ::= ENUMERATED

{

registration(1),

sMS(2),

pDUSessionEstablishment(3)

}

AMFFailureCause ::= CHOICE

{

fiveGMMCause        [1] FiveGMMCause,

fiveGSMCause        [2] FiveGSMCause

}

AMFPointer ::= INTEGER (0..1023)

AMFRegistrationResult ::= ENUMERATED

{

threeGPPAccess(1),

nonThreeGPPAccess(2),

threeGPPAndNonThreeGPPAccess(3)

}

AMFRegionID ::= INTEGER (0..255)

AMFRegistrationType ::= ENUMERATED

{

initial(1),

mobility(2),

periodic(3),

emergency(4)

}

AMFSetID ::= INTEGER (0..63)

-- ==================

-- 5G SMF definitions

-- ==================

-- See clause 6.2.3.2.2 for details of this structure

SMFPDUSessionEstablishment ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

pDUSessionID                [5] PDUSessionID,

gTPTunnelID                 [6] FTEID,

pDUSessionType              [7] PDUSessionType,

sNSSAI                      [8] SNSSAI OPTIONAL,

uEEndpoint                  [9] SEQUENCE OF UEEndpointAddress OPTIONAL,

non3GPPAccessEndpoint       [10] UEEndpointAddress OPTIONAL,

location                    [11] Location OPTIONAL,

dNN                         [12] DNN,

aMFID                       [13] AMFID OPTIONAL,

hSMFURI                     [14] HSMFURI OPTIONAL,

requestType                 [15] FiveGSMRequestType,

accessType                  [16] AccessType OPTIONAL,

rATType                     [17] RATType OPTIONAL,

sMPDUDNRequest              [18] SMPDUDNRequest OPTIONAL

}

-- See clause 6.2.3.2.3 for details of this structure

SMFPDUSessionModification ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

sNSSAI                      [5] SNSSAI OPTIONAL,

non3GPPAccessEndpoint       [6] UEEndpointAddress OPTIONAL,

location                    [7] Location OPTIONAL,

requestType                 [8] FiveGSMRequestType,

accessType                  [9] AccessType OPTIONAL,

rATType                     [10] RATType OPTIONAL

}

-- See clause 6.2.3.2.4 for details of this structure

SMFPDUSessionRelease ::= SEQUENCE

{

sUPI                        [1] SUPI,

pEI                         [2] PEI OPTIONAL,

gPSI                        [3] GPSI OPTIONAL,

pDUSessionID                [4] PDUSessionID,

timeOfFirstPacket           [5] Timestamp OPTIONAL,

timeOfLastPacket            [6] Timestamp OPTIONAL,

uplinkVolume                [7] INTEGER OPTIONAL,

downlinkVolume              [8] INTEGER OPTIONAL,

location                    [9] Location OPTIONAL

}

-- See clause 6.2.3.2.5 for details of this structure

SMFStartOfInterceptionWithEstablishedPDUSession ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

pDUSessionID                [5] PDUSessionID,

gTPTunnelID                 [6] FTEID,

pDUSessionType              [7] PDUSessionType,

sNSSAI                      [8] SNSSAI OPTIONAL,

uEEndpoint                  [9] SEQUENCE OF UEEndpointAddress,

non3GPPAccessEndpoint       [10] UEEndpointAddress OPTIONAL,

location                    [11] Location OPTIONAL,

dNN                         [12] DNN,

aMFID                       [13] AMFID OPTIONAL,

hSMFURI                     [14] HSMFURI OPTIONAL,

requestType                 [15] FiveGSMRequestType,

accessType                  [16] AccessType OPTIONAL,

rATType                     [17] RATType OPTIONAL,

sMPDUDNRequest              [18] SMPDUDNRequest OPTIONAL

}

-- See clause 6.2.3.2.6 for details of this structure

SMFUnsuccessfulProcedure ::= SEQUENCE

{

failedProcedureType         [1] SMFFailedProcedureType,

failureCause                [2] FiveGSMCause,

initiator                   [3] Initiator,

requestedSlice              [4] NSSAI OPTIONAL,

sUPI                        [5] SUPI OPTIONAL,

sUPIUnauthenticated         [6] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [7] PEI OPTIONAL,

gPSI                        [8] GPSI OPTIONAL,

pDUSessionID                [9] PDUSessionID OPTIONAL,

uEEndpoint                  [10] SEQUENCE OF UEEndpointAddress OPTIONAL,

non3GPPAccessEndpoint       [11] UEEndpointAddress OPTIONAL,

dNN                         [12] DNN OPTIONAL,

aMFID                       [13] AMFID OPTIONAL,

hSMFURI                     [14] HSMFURI OPTIONAL,

requestType                 [15] FiveGSMRequestType OPTIONAL,

accessType                  [16] AccessType OPTIONAL,

rATType                     [17] RATType OPTIONAL,

sMPDUDNRequest              [18] SMPDUDNRequest OPTIONAL,

location                    [19] Location OPTIONAL

}

-- =================

-- 5G SMF parameters

-- =================

SMFFailedProcedureType ::= ENUMERATED

{

pDUSessionEstablishment(1),

pDUSessionModification(2),

pDUSessionRelease(3)

}

-- =================

-- 5G UPF parameters

-- =================

UPFCCPDU ::= OCTET STRING

-- ==================

-- 5G UDM definitions

-- ==================

UDMServingSystemMessage ::= SEQUENCE

{

sUPI                        [1] SUPI,

pEI                         [2] PEI OPTIONAL,

gPSI                        [3] GPSI OPTIONAL,

gUAMI                       [4] GUAMI OPTIONAL,

gUMMEI                      [5] GUMMEI OPTIONAL,

pLMNID                      [6] PLMNID OPTIONAL,

servingSystemMethod         [7] UDMServingSystemMethod

}

-- =================

-- 5G UDM parameters

-- =================

UDMServingSystemMethod ::= ENUMERATED

{

amf3GPPAccessRegistration(0),

amfNon3GPPAccessRegistration(1),

unknown(2)

}

-- ===================

-- 5G SMSF definitions

-- ===================

-- See clause 6.2.5.3 for details of this structure

SMSMessage ::= SEQUENCE

{

originatingSMSParty         [1] SMSParty,

terminatingSMSParty         [2] SMSParty,

direction                   [3] Direction,

transferStatus              [4] SMSTransferStatus,

otherMessage                [5] SMSOtherMessageIndication OPTIONAL,

location                    [6] Location OPTIONAL,

peerNFAddress               [7] SMSNFAddress OPTIONAL,

peerNFType                  [8] SMSNFType OPTIONAL,

sMSTPDUData                 [9] SMSTPDUData OPTIONAL

}

-- ==================

-- 5G SMSF parameters

-- ==================

SMSParty ::= SEQUENCE

{

sUPI        [1] SUPI OPTIONAL,

pEI         [2] PEI OPTIONAL,

gPSI        [3] GPSI OPTIONAL

}

SMSTransferStatus ::= ENUMERATED

{

transferSucceeded(1),

transferFailed(2),

undefined(3)

}

SMSOtherMessageIndication ::= BOOLEAN

SMSNFAddress ::= CHOICE

{

iPAddress   [1] IPAddress,

e164Number  [2] E164Number

}

SMSNFType ::= ENUMERATED

{

sMSGMSC(1),

iWMSC(2),

sMSRouter(3)

}

SMSTPDUData ::= CHOICE

{

sMSTPDU [1] SMSTPDU

}

SMSTPDU ::= OCTET STRING (SIZE(1..270))

-- ===================

-- 5G MMS definitions

-- ===================

MMSSend ::= SEQUENCE

{

originatingMMSParty [1]  MMSParty,

terminatingMMSParty [2]  MMSParty,

direction           [3]  Direction,

correlationNumber   [4]  UTF8String              OPTIONAL,

cCRecipients        [5]  MMSParty                OPTIONAL,

bCCRecipients       [6]  MMSParty                OPTIONAL,

mMSVersion          [7]  UTF8String,

transactionID       [8]  UTF8String,

messageID           [9]  UTF8String,

mMSDateTime         [10] Timestamp,

messageClass        [11] UTF8String              OPTIONAL,

expiry              [12] Expiry,

desiredDeliveryTime [13] Timestamp               OPTIONAL,

priority            [14] UTF8String              OPTIONAL,

senderVisibility    [15] BOOLEAN                 OPTIONAL,

deliveryReport      [16] BOOLEAN                 OPTIONAL,

readReport          [17] BOOLEAN                 OPTIONAL,

store               [18] BOOLEAN                 OPTIONAL,

applicID            [19] UTF8String              OPTIONAL,

replyApplicID       [20] UTF8String              OPTIONAL,

contentClass        [21] UTF8String              OPTIONAL,

dRMContent          [22] BOOLEAN                 OPTIONAL,

adaptationAllowed   [23] Adaptation              OPTIONAL,

contentType         [24] UTF8String,

contentLocation     [25] UTF8String              OPTIONAL,

responseStatus      [26] BOOLEAN                 OPTIONAL,

responseStatusText  [27] UTF8String              OPTIONAL,

storeStatus         [28] BOOLEAN                 OPTIONAL,

storeStatusText     [29] UTF8String              OPTIONAL

}

MMSNotification ::= SEQUENCE

{

originatingMMSParty        [1]  MMSParty,

terminatingMMSParty        [2]  MMSParty,

direction                  [3]  Direction,

correlationNumber          [4]  UTF8String              OPTIONAL,

cCRecipients               [5]  MMSParty                OPTIONAL,

bCCRecipients              [6]  MMSParty                OPTIONAL,

mMSVersion                 [7]  UTF8String,

transactionID              [8]  UTF8String,

messageID                  [9]  UTF8String,

mMSDateTime                [10] Timestamp,

messageClass               [11] UTF8String,

expiry                     [12] Expiry,

distributionIndicator      [13] BOOLEAN                 OPTIONAL,

elementDescriptor          [14] ElementDescriptor       OPTIONAL,

manualRetrievalRecommended [15] BOOLEAN                 OPTIONAL,

retrievalModeText          [16] UTF8String              OPTIONAL,

senderVisibility           [17] BOOLEAN                 OPTIONAL,

deliveryReport             [18] BOOLEAN                 OPTIONAL,

readReport                 [19] BOOLEAN                 OPTIONAL,

applicID                   [20] UTF8String              OPTIONAL,

replyApplicID              [21] UTF8String              OPTIONAL,

auxApplicInfo              [22] UTF8String              OPTIONAL,

contentClass               [23] UTF8String              OPTIONAL,

dRMContent                 [24] BOOLEAN                 OPTIONAL,

replaceID                  [25] UTF8String              OPTIONAL,

contentLocation            [26] UTF8String              OPTIONAL

}

MMSNotificationResponse ::= SEQUENCE

{

originatingMMSParty [1] MMSParty,

terminatingMMSParty [2] MMSParty,

direction           [3] Direction,

correlationNumber   [4] UTF8String      OPTIONAL,

mMSVersion          [5] UTF8String,

transactionID       [6] UTF8String,

messageID           [7] UTF8String,

mMStatus            [8] UTF8String,

reportAllowed       [9] BOOLEAN         OPTIONAL

}

MMSRetrieval ::= SEQUENCE

{

originatingMMSParty     [1]  MMSParty,

terminatingMMSParty     [2]  MMSParty,

direction               [3]  Direction,

correlationNumber       [4]  UTF8String              OPTIONAL,

cCRecipients            [5]  MMSParty                OPTIONAL,

bCCRecipients           [6]  MMSParty                OPTIONAL,

mMSVersion              [7]  UTF8String,

transactionID           [8] UTF8String,

messageID               [9] UTF8String,

mMSDateTime             [10] Timestamp,

previouslySentBy        [11] PreviouslySentBy        OPTIONAL,

mMState                 [12] UTF8String              OPTIONAL,

messageClass            [13] UTF8String              OPTIONAL,

priority                [14] UTF8String,

deliveryReportRequested [15] BOOLEAN                 OPTIONAL,

readReportRequested     [16] BOOLEAN                 OPTIONAL,

senderVisibility        [17] BOOLEAN                 OPTIONAL,

retrieveStatus          [18] BOOLEAN                 OPTIONAL,

retrieveStatusText      [19] UTF8String              OPTIONAL,

distributionIndicator   [20] BOOLEAN                 OPTIONAL,

applicID                [21] UTF8String              OPTIONAL,

replyApplicID           [22] UTF8String              OPTIONAL,

auxApplicInfo           [23] UTF8String              OPTIONAL,

contentClass            [24] UTF8String              OPTIONAL,

dRMContent              [25] BOOLEAN                 OPTIONAL,

replaceID               [26] UTF8String              OPTIONAL,

contentType             [27] UTF8String              OPTIONAL

}

MMSRetrievalAck ::= SEQUENCE

{

originatingMMSParty [1] MMSParty,

terminatingMMSParty [2] MMSParty,

direction           [3] Direction,

mMSVersion          [4] UTF8String,

transactionID       [5] UTF8String,

messageID           [6] UTF8String,

reportAllowed       [7] BOOLEAN          OPTIONAL

}

MMSForward ::= SEQUENCE

{

originatingMMSParty   [1]  MMSParty,

terminatingMMSParty   [2]  MMSParty,

direction             [3]  Direction,

correlationNumber     [4]  UTF8String               OPTIONAL,

cCRecipients          [5]  MMSParty                 OPTIONAL,

bCCRecipients         [6]  MMSParty                 OPTIONAL,

mMSVersion            [7]  UTF8String,

transactionID         [8]  UTF8String,

messageID             [9] UTF8String,

mMSForwardDateTime    [10] Timestamp,

messageClass          [11] UTF8String              OPTIONAL,

expiry                [12] Expiry,

desiredDeliveryTime   [13] Timestamp               OPTIONAL,

priority              [14] UTF8String              OPTIONAL,

senderVisibility      [15] BOOLEAN                 OPTIONAL,

deliveryReportAllowed [16] BOOLEAN,

deliveryReport        [17] UTF8String              OPTIONAL,

store                 [18] BOOLEAN                 OPTIONAL,

mMState               [19] UTF8String              OPTIONAL,

contentLocation       [20] UTF8String,

responseStatus        [21] BOOLEAN,

responseStatusText    [22] UTF8String              OPTIONAL,

storeStatus           [23] BOOLEAN                 OPTIONAL,

storeStatusText       [24] UTF8String              OPTIONAL

}

MMSStore ::= SEQUENCE

{

originatingMMSParty [1]  MMSParty,

terminatingMMSParty [2]  MMSParty,

direction           [3]  Direction,

mMSVersion          [4]  UTF8String,

transactionID       [5]  UTF8String,

mMState             [6]  UTF8String        OPTIONAL,

mMStateFlags        [7]  UTF8String        OPTIONAL,

contentLocation     [8]  UTF8String,

storeStatus         [9]  BOOLEAN,

storeStatusText     [10] UTF8String        OPTIONAL

}

MMSUpload ::= SEQUENCE

{

originatingMMSParty [1]  MMSParty,

terminatingMMSParty [2]  MMSParty,

direction           [3]  Direction,

mMSVersion          [4]  UTF8String,

transactionID       [5]  UTF8String,

mMState             [6]  UTF8String              OPTIONAL,

mMStateFlags        [7]  UTF8String              OPTIONAL,

contentType         [8]  UTF8String,

contentLocation     [9]  UTF8String              OPTIONAL,

storeStatus         [10] BOOLEAN,

storeStatusText     [11] UTF8String              OPTIONAL,

mMBoxDescription    [12] MMBoxDescriptionPDU

}

MMSDelete ::= SEQUENCE

{

originatingMMSParty [1] MMSParty,

terminatingMMSParty [2] MMSParty,

direction           [3] Direction,

mMSVersion          [4] UTF8String,

transactionID       [5] UTF8String,

contentLocation     [6] UTF8String,

responseStatus      [7] BOOLEAN,

responseStatusText  [8] UTF8String          OPTIONAL

}

MMSDeliveryNotification ::= SEQUENCE

{

originatingMMSParty [1]  MMSParty,

terminatingMMSParty [2]  MMSParty,

direction           [3]  Direction,

mMSVersion          [4]  UTF8String,

messageID           [5]  UTF8String,

mMSDateTime         [6]  Timestamp,

mMStatus            [7]  UTF8String,

mMStatusText        [8]  UTF8String             OPTIONAL,

applicID            [9]  UTF8String             OPTIONAL,

replyApplicID       [10] UTF8String             OPTIONAL,

auxApplicInfo       [11] UTF8String             OPTIONAL

}

MMSReadReply ::= SEQUENCE

{

originatingMMSParty [1]  MMSParty,

terminatingMMSParty [2]  MMSParty,

direction           [3]  Direction,

mMSVersion          [4]  UTF8String,

messageID           [5]  UTF8String,

mMSDateTime         [6]  Timestamp,

readStatus          [7]  UTF8String,

applicID            [8]  UTF8String     OPTIONAL,

replyApplicID       [9]  UTF8String     OPTIONAL,

auxApplicInfo       [10] UTF8String     OPTIONAL

}

MMSCancel ::= SEQUENCE

{

originatingMMSParty [1] MMSParty,

terminatingMMSParty [2] MMSParty,

direction           [3] Direction,

mMSVersion          [4] UTF8String,

messageID           [5] UTF8String,

mMSDateTime         [6] Timestamp,

cancelID            [7] UTF8String,

cancelStatus        [8] UTF8String    OPTIONAL

}

MMSViewRequest ::= SEQUENCE

{

originatingMMSParty [1]  MMSParty,

terminatingMMSParty [2]  MMSParty,

direction           [3]  Direction,

mMSVersion          [4]  UTF8String,

transactionID       [5]  UTF8String,

messageID           [6]  UTF8String,

mMSDateTime         [7]  Timestamp,

mMState             [8]  UTF8String              OPTIONAL,

mMSStateFlags       [9]  UTF8String              OPTIONAL,

contentLocation     [10] UTF8String              OPTIONAL,

mMSStart            [11] INTEGER                 OPTIONAL,

mMSLimit            [12] INTEGER                 OPTIONAL,

mMSAttributes       [13] SEQUENCE OF UTF8String  OPTIONAL,

mMSTotals           [14] INTEGER                 OPTIONAL,

mMSQuotas           [15] MMSQuota                OPTIONAL

}

MMSViewConfirm ::= SEQUENCE

{

originatingMMSParty [1]  MMSParty,

terminatingMMSParty [2]  MMSParty,

direction           [3]  Direction,

mMSVersion          [4]  UTF8String,

transactionID       [5]  UTF8String,

messageID           [6]  UTF8String,

mMSDateTime         [7]  Timestamp,

mMState             [8]  UTF8String              OPTIONAL,

mMStateFlags        [9]  UTF8String              OPTIONAL,

contentLocation     [10] UTF8String             OPTIONAL,

mMSStart            [11] INTEGER                 OPTIONAL,

mMSLimit            [12] INTEGER                 OPTIONAL,

mMSAttributes       [13] SEQUENCE OF UTF8String  OPTIONAL,

mMSTotals           [14] BOOLEAN                 OPTIONAL,

mMSQuotas           [15] BOOLEAN                 OPTIONAL,

responseStatus      [16] BOOLEAN,

responseStatusText  [17] UTF8String              OPTIONAL,

mMSMessageCount     [18] INTEGER                 OPTIONAL,

contentType         [19] UTF8String,

mMBoxDescription    [20] MMBoxDescriptionPDU

}

MMBoxDescriptionPDU ::= SEQUENCE

{

correlationNumber   [1]  UTF8String               OPTIONAL,

originatingMMSParty [2]  MMSParty,

terminatingMMSParty [3]  MMSParty,

direction           [4]  Direction,

cCRecipients        [5]  MMSParty                 OPTIONAL,

bCCRecipients       [6]  MMSParty                 OPTIONAL,

messageID           [7]  UTF8String               OPTIONAL,

mMSDateTime         [8]  Timestamp,

previouslySentBy    [9]  PreviouslySentBy         OPTIONAL,

mMState             [10] UTF8String               OPTIONAL,

mMStateFlags        [11] UTF8String               OPTIONAL,

messageClass        [12] UTF8String               OPTIONAL,

priority            [13] UTF8String               OPTIONAL,

deliveryTime        [14] Timestamp                OPTIONAL,

expiry              [15] Expiry                   OPTIONAL,

deliveryReport      [16] BOOLEAN                  OPTIONAL,

readReport          [17] BOOLEAN                  OPTIONAL,

messageSize         [18] INTEGER                  OPTIONAL,

contentType         [19] UTF8String,

contentLocation     [20] UTF8String               OPTIONAL

}

-- ==================

-- 5G MMS CCPDU

-- ==================

MMSCCPDU ::= SEQUENCE

{

correlationNumber [1] UTF8String,

mMSVersion        [2] UTF8String,

transactionID     [3] UTF8String,

mMSContent        [4] OCTET STRING

}

-- ==================

-- 5G MMS parameters

-- ==================

Adaptation ::= SEQUENCE

{

allowed   [1] BOOLEAN,

overriden [2] BOOLEAN

}

ElementDescriptor ::= SEQUENCE

{

descriptor    [1] UTF8String,

contentFormat [2] UTF8String   OPTIONAL

}

Expiry ::= SEQUENCE

{

expiryPeriod [1] INTEGER,

expiryUnit   [2] TimeUnit,

periodFormat [3] PeriodFormat

}

MMSParty ::= SEQUENCE

{

mMSPartyIDs [1] SEQUENCE OF MMSPartyID,

nonLocalID  [2] NonLocalID

}

MMSPartyID ::= CHOICE

{

e164Number [1] E164Number,

email      [2] Email,

iMSI       [3] IMSI,

iMPU       [4] IMPU,

iMPI       [5] NAI,

sUPI       [6] SUPI,

gPSI       [7] GPSI

}

MMSQuota ::= SEQUENCE

{

quota     [1] INTEGER,

quotaUnit [2] QuotaUnit

}

PeriodFormat ::= ENUMERATED

{

absolute (1),

relative (2)

}

PreviouslySentBy ::= SEQUENCE

{

previouslySentByParty [1] SET OF MMSParty, --multiples identifiers for SAME sender

sequenceNumber        [2] INTEGER,

previousSendDateTime  [3] Timestamp

}

QuotaUnit ::= ENUMERATED

{

numMessages (1),

bytes       (2)

}

TimeUnit ::= ENUMERATED

{

seconds (1),

minutes (2),

hours   (3),

days    (4),

weeks   (5),

months  (6),

years   (7)

}

-- ==================

-- 5G PTC definitions

-- ==================

PTCRegistration  ::= SEQUENCE

{

registrationType            [1] INTEGER,

registrationResult          [2] INTEGER,

pTCPartyID                  [3] MCPTTID OPTIONAL,

pTCServerURI                [4] UTF8String OPTIONAL

}

PTCServingSystem  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

networkIdentifier           [2] UTF8String,

servingSystemID             [3] UTF8String

}

PTCSessionInitiation  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String OPTIONAL,

networkIdentifier           [3] UTF8String,

pTCServerURI                [4] UTF8String OPTIONAL,

pTCSessioninfo              [5] UTF8String,

pTCOriginatingID            [6] UTF8String OPTIONAL,

pTCParticipants             [7] SEQUENCE OF MCPTTID OPTIONAL,

associatePresenceStatus     [8] UTF8String OPTIONAL,

location                    [9] Location OPTIONAL,

initiationCause             [10] UTF8String,

bearerCapability            [11] UTF8String OPTIONAL,

pTCHost                     [12] MCPTTID OPTIONAL

}

PTCSessionAbandon  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String OPTIONAL,

networkIdentifier           [3] UTF8String,

pTCSessioninfo              [4] UTF8String,

location                    [5] Location OPTIONAL,

cause                       [6] UTF8String

}

PTCSessionStart  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String OPTIONAL,

networkIdentifier           [3] UTF8String,

pTCServerURI                [4] UTF8String OPTIONAL,

pTCSessioninfo              [5] UTF8String,

pTCOriginatingID            [6] UTF8String OPTIONAL,

pTCParticipants             [7] SEQUENCE OF MCPTTID OPTIONAL,

associatePresenceStatus     [8] UTF8String OPTIONAL,

location                    [9] Location OPTIONAL,

initiationCause             [10] UTF8String OPTIONAL,

pTCHost                     [11] MCPTTID OPTIONAL,

bearerCapability            [12] UTF8String OPTIONAL

}

PTCSessionEnd  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String,

networkIdentifier           [3] UTF8String,

pTCServerURI                [4] UTF8String OPTIONAL,

pTCSessioninfo              [5] UTF8String,

preEstStatus                [6] BOOLEAN OPTIONAL,

pTCOriginatingID            [7] UTF8String OPTIONAL,

pTCParticipants             [8] SEQUENCE OF MCPTTID OPTIONAL,

location                    [9] Location OPTIONAL,

cause                       [10] UTF8String

}

PTCStartofInterception  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String,

networkIdentifier           [3] UTF8String,

preEstSessionID             [4] UTF8String OPTIONAL,

pTCOriginatingID            [5] UTF8String OPTIONAL,

pTCSessioninfo              [6] UTF8String OPTIONAL,

pTCHost                     [7] MCPTTID OPTIONAL,

pTCParticipants             [8] SEQUENCE OF MCPTTID,

mediaStreamAvail            [9] BOOLEAN OPTIONAL,

bearerCapability            [10] UTF8String OPTIONAL

}

PTCPre-establishedSession  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String,

networkIdentifier           [3] UTF8String,

pTCserverURI                [4] UTF8String,

rTPSetting                  [5] UTF8String,

pTCMediaCapability          [6] UTF8String,

preEstSessionID             [7] UTF8String,

preEstStatus                [8] BOOLEAN,

mediaStreamAvail            [9] BOOLEAN OPTIONAL,

location                    [10] Location OPTIONAL,

failureCode                 [11] UTF8String OPTIONAL

}

PTCInstantPersonalAlert  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

pTCSessioninfo              [2] UTF8String,

iPAPartyID                  [3] UTF8String,

iPADirection                [4] Direction

}

PTCPartyJoin  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String,

networkIdentifier           [3] UTF8String,

iPAPartyID                  [4] UTF8String,

pTCSessioninfo              [5] UTF8String,

pTCParticipants             [6] SEQUENCE OF MCPTTID OPTIONAL,

associatePresenceStatus     [7] UTF8String OPTIONAL,

mediaStreamAvail            [8] BOOLEAN OPTIONAL,

bearerCapability            [9] UTF8String OPTIONAL

}

PTCPartyDrop  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String,

networkIdentifier           [3] UTF8String,

pTCSessioninfo              [4] UTF8String,

pTCPartyDrop                [5] UTF8String,

associatePresenceStatus     [6] UTF8String OPTIONAL

}

PTCPartyHold  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String,

networkIdentifier           [3] UTF8String,

pTCSessioninfo              [4] UTF8String,

pTCParticipants             [5] SEQUENCE OF MCPTTID OPTIONAL,

pTCHoldID                   [6] MCPTTID,

retrievelnd                 [7] BOOLEAN

}

PTCMediaModification  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String,

networkIdentifier           [3] UTF8String,

pTCSessioninfo              [4] UTF8String,

mediaStreamAvail            [5] BOOLEAN OPTIONAL,

bearerCapability            [6] UTF8String

}

PTCGroupAdvertisement  ::=SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String,

networkIdentifier           [3] UTF8String,

pTCIDList                   [4] UTF8String OPTIONAL,

pTCGroupID                  [5] UTF8String OPTIONAL,

groupAuthRule               [6] UTF8String OPTIONAL,

groupAdSender               [7] UTF8String,

pTCHost                     [8] MCPTTID OPTIONAL,

groupCharacteristics        [9] UTF8String OPTIONAL

}

PTCFloorControl  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String,

networkIdentifier           [3] UTF8String,

pTCSessioninfo              [4] UTF8String,

floorActivity               [5] UTF8String,

floorSpeakerID              [6] MCPTTID OPTIONAL,

maxTBTime                   [7] TimeUnit OPTIONAL,

queuedFloorControl          [8] BOOLEAN OPTIONAL,

queuedPosition              [9] INTEGER OPTIONAL,

talkBurstPriority           [10] UTF8String,

talkBurstreason             [11] UTF8String OPTIONAL

}

PTCTargetPresence  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String,

networkIdentifier           [3] UTF8String,

targetPresenceStatus        [4] UTF8String OPTIONAL

}

PTCAssociatePresence  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String,

networkIdentifier           [3] UTF8String,

associatePresenceStatus     [4] UTF8String OPTIONAL

}

PTCListManagement  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String,

networkIdentifier           [3] UTF8String,

targetPresenceStatus        [4] UTF8String OPTIONAL,

listManagementType          [5] UTF8String OPTIONAL,

listManagementAction        [6] UTF8String OPTIONAL,

listManagementFailure       [7] UTF8String OPTIONAL,

contactID                   [8] MCPTTID OPTIONAL,

pTCGroupID                  [9] UTF8String OPTIONAL,

pTCIDList                   [10] MCPTTID OPTIONAL,

pTCHost                     [11] MCPTTID OPTIONAL

}

PTCAccessPolicy  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String,

networkIdentifier           [3] UTF8String,

targetPresenceStatus        [4] UTF8String,

accessPolicyType            [5] UTF8String OPTIONAL,

pTCUserAccessPolicy         [6] UTF8String OPTIONAL,

groupAuthRule               [7] UTF8String OPTIONAL,

contactID                   [8] MCPTTID OPTIONAL,

pTCGroupID                  [9] UTF8String OPTIONAL,

accessPolicyFailure         [10] UTF8String OPTIONAL

}

PTCMediaTypeNotification  ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

correlation                 [2] UTF8String,

networkIdentifier           [3] UTF8String,

mediaStreamAvail            [4] BOOLEAN OPTIONAL,

pTCSessioninfo              [5] UTF8String,

mediaType                   [6] UTF8String

}

PTCEncryptionMessage ::= SEQUENCE

{

pTCPartyID                  [1] MCPTTID OPTIONAL,

pTCType                     [2] UTF8String,

eventTime                   [3] TimeUnit,

cryptoContext               [4] UTF8String OPTIONAL,

cipher                      [5] UTF8String,

key                         [6] UTF8String,

salt                        [7] UTF8String OPTIONAL,

keyEncoding                 [8] UTF8String OPTIONAL,

pTCOther                    [9] UTF8String OPTIONAL

}

-- =========

-- PTC CCPDU

-- =========

PTCCCPDU ::= SEQUENCE

{

correlationNumber [1] UTF8String,

pTCVersion	   [2] UTF8String,

transactionID	   [3] UTF8String,

pTCContent	   [4] OCTET STRING  (SIZE(0..255))

}

-- =================

-- 5G PTC parameters

-- =================

PTCPartyID  ::= CHOICE

{

mCPTTID                    [1] MCPTTID,

iMEI                       [2] IMEI,

pEI                        [3] PEI

}

MCPTTID ::= SEQUENCE

{

nAME                         [1] UTF8String,

aGENCYID                     [2] UTF8String,

aGENCYNAME                   [3] UTF8String,

aGENCYSTATE                  [4] UTF8String

}

-- ===================

-- 5G LALS definitions

-- ===================

LALSReport ::= SEQUENCE

{

sUPI                [1] SUPI OPTIONAL,

pEI                 [2] PEI OPTIONAL,

gPSI                [3] GPSI OPTIONAL,

location            [4] Location OPTIONAL

}

-- =====================

-- PDHR/PDSR definitions

-- =====================

PDHeaderReport ::= SEQUENCE

{

pDUSessionID                [1] PDUSessionID,

sourceIPAddress             [2] IPAddress,

sourcePort                  [3] PortNumber OPTIONAL,

destinationIPAddress        [4] IPAddress,

destinationPort             [5] PortNumber OPTIONAL,

nextLayerProtocol           [6] NextLayerProtocol,

iPv6flowLabel               [7] IPv6FlowLabel OPTIONAL,

direction                   [8] Direction,

packetSize                  [9] INTEGER

}

PDSummaryReport ::= SEQUENCE

{

pDUSessionID                [1] PDUSessionID,

sourceIPAddress             [2] IPAddress,

sourcePort                  [3] PortNumber OPTIONAL,

destinationIPAddress        [4] IPAddress,

destinationPort             [5] PortNumber OPTIONAL,

nextLayerProtocol           [6] NextLayerProtocol,

iPv6flowLabel               [7] IPv6FlowLabel OPTIONAL,

direction                   [8] Direction,

pDSRSummaryTrigger          [9] PDSRSummaryTrigger,

firstPacketTimestamp        [10] Timestamp,

lastPacketTimestamp         [11] Timestamp,

packetCount                 [12] INTEGER,

byteCount                   [13] INTEGER

}

-- ====================

-- PDHR/PDSR parameters

-- ====================

PDSRSummaryTrigger ::= ENUMERATED

{

timerExpiry(1),

packetCount(2),

byteCount(3)

}

-- ===========================

-- LI Notification definitions

-- ===========================

LINotification ::= SEQUENCE

{

notificationType                    [1] LINotificationType,

appliedTargetID                     [2] TargetIdentifier OPTIONAL,

appliedDeliveryInformation          [3] SEQUENCE OF LIAppliedDeliveryInformation OPTIONAL,

appliedStartTime                    [4] Timestamp OPTIONAL,

appliedEndTime                      [5] Timestamp OPTIONAL

}

-- ==========================

-- LI Notification parameters

-- ==========================

LINotificationType ::= ENUMERATED

{

activation(1),

deactivation(2),

modification(3)

}

LIAppliedDeliveryInformation ::= SEQUENCE

{

hI2DeliveryIPAddress                [1] IPAddress OPTIONAL,

hI2DeliveryPortNumber               [2] PortNumber OPTIONAL,

hI3DeliveryIPAddress                [3] IPAddress OPTIONAL,

hI3DeliveryPortNumber               [4] PortNumber OPTIONAL

}

-- ===============

-- MDF definitions

-- ===============

MDFCellSiteReport ::= SEQUENCE OF CellInformation

-- =================

-- Common Parameters

-- =================

AccessType ::= ENUMERATED

{

threeGPPAccess(1),

nonThreeGPPAccess(2),

threeGPPandNonThreeGPPAccess(3)

}

Direction ::= ENUMERATED

{

fromTarget(1),

toTarget(2)

}

DNN ::= UTF8String

E164Number ::= NumericString (SIZE(1..15))

Email ::= UTF8String

FiveGGUTI ::= SEQUENCE

{

mCC         [1] MCC,

mNC         [2] MNC,

aMFRegionID [3] AMFRegionID,

aMFSetID    [4] AMFSetID,

aMFPointer  [5] AMFPointer,

fiveGTMSI   [6] FiveGTMSI

}

FiveGMMCause ::= INTEGER (0..255)

FiveGSMRequestType ::= ENUMERATED

{

initialRequest(1),

existingPDUSession(2),

initialEmergencyRequest(3),

existingEmergencyPDUSession(4),

modificationRequest(5),

reserved(6)

}

FiveGSMCause ::= INTEGER (0..255)

FiveGTMSI ::= INTEGER (0..4294967295)

FTEID ::= SEQUENCE

{

tEID        [1] INTEGER (0.. 4294967295),

iPv4Address [2] IPv4Address OPTIONAL,

iPv6Address [3] IPv6Address OPTIONAL

}

GPSI ::= CHOICE

{

mSISDN      [1] MSISDN,

nAI         [2] NAI

}

GUAMI ::= SEQUENCE

{

aMFID       [1] AMFID,

pLMNID      [2] PLMNID

}

GUMMEI ::= SEQUENCE

{

mMEID       [1] MMEID,

mCC         [2] MCC,

mNC         [3] MNC

}

HomeNetworkPublicKeyID ::= OCTET STRING

HSMFURI ::= UTF8String

IMEI ::= NumericString (SIZE(14))

IMEISV ::= NumericString (SIZE(16))

IMPI ::= NAI

IMPU ::= CHOICE

{

sIPURI    [1] SIPURI,

tELURL    [2] TELURL

}

IMSI ::= NumericString (SIZE(6..15))

Initiator ::= ENUMERATED

{

uE(1),

network(2),

unknown(3)

}

IPAddress ::= CHOICE

{

iPv4Address [1] IPv4Address,

iPv6Address [2] IPv6Address

}

IPv4Address ::= OCTET STRING (SIZE(4))

IPv6Address ::= OCTET STRING (SIZE(16))

IPv6FlowLabel ::= INTEGER(0..1048575)

MACAddress ::= OCTET STRING (SIZE(6))

MCC ::= NumericString (SIZE(3))

MNC ::= NumericString (SIZE(2..3))

MMEID ::= SEQUENCE

{

mMEGI       [1] MMEGI,

mMEC        [2] MMEC

}

MMEC ::= NumericString

MMEGI ::= NumericString

MSISDN ::= NumericString (SIZE(1..15))

NAI ::= UTF8String

NextLayerProtocol ::= INTEGER(0..255)

NonLocalID ::= BOOLEAN

NSSAI ::= SEQUENCE OF SNSSAI

PLMNID ::= SEQUENCE

{

mCC [1] MCC,

mNC [2] MNC

}

PDUSessionID ::= INTEGER (0..255)

PDUSessionType ::= ENUMERATED

{

iPv4(1),

iPv6(2),

iPv4v6(3),

unstructured(4),

ethernet(5)

}

PEI ::= CHOICE

{

iMEI        [1] IMEI,

iMEISV      [2] IMEISV

}

PortNumber ::= INTEGER(0..65535)

ProtectionSchemeID ::= INTEGER (0..15)

RATType ::= ENUMERATED

{

nR(1),

eUTRA(2),

wLAN(3),

virtual(4)

}

RejectedNSSAI ::= SEQUENCE OF RejectedSNSSAI

RejectedSNSSAI ::= SEQUENCE

{

causeValue  [1] RejectedSliceCauseValue,

sNSSAI      [2] SNSSAI

}

RejectedSliceCauseValue ::= INTEGER (0..255)

RoutingIndicator ::= INTEGER (0..9999)

SchemeOutput ::= OCTET STRING

SIPURI ::= OCTET STRING

Slice ::= SEQUENCE

{

allowedNSSAI        [1] NSSAI OPTIONAL,

configuredNSSAI     [2] NSSAI OPTIONAL,

rejectedNSSAI       [3] RejectedNSSAI OPTIONAL

}

SMPDUDNRequest ::= OCTET STRING

SNSSAI ::= SEQUENCE

{

sliceServiceType    [1] INTEGER (0..255),

sliceDifferentiator [2] OCTET STRING (SIZE(3)) OPTIONAL

}

SUCI ::= SEQUENCE

{

mCC                         [1] MCC,

mNC                         [2] MNC,

routingIndicator            [3] RoutingIndicator,

protectionSchemeID          [4] ProtectionSchemeID,

homeNetworkPublicKeyID      [5] HomeNetworkPublicKeyID,

schemeOutput                [6] SchemeOutput

}

SUPI ::= CHOICE

{

iMSI        [1] IMSI,

nAI         [2] NAI

}

SUPIUnauthenticatedIndication ::= BOOLEAN

TargetIdentifier ::= CHOICE

{

sUPI                [1] SUPI,

iMSI                [2] IMSI,

pEI                 [3] PEI,

iMEI                [4] IMEI,

gPSI                [5] GPSI,

mISDN               [6] MSISDN,

nAI                 [7] NAI,

iPv4Address         [8] IPv4Address,

iPv6Address         [9] IPv6Address,

ethernetAddress     [10] MACAddress

}

TargetIdentifierProvenance ::= ENUMERATED

{

lEAProvided(1),

observed(2),

matchedOn(3),

other(4)

}

TELURL ::= OCTET STRING

Timestamp ::= GeneralizedTime

UEEndpointAddress ::= CHOICE

{

iPv4Address         [1] IPv4Address,

iPv6Address         [2] IPv6Address,

ethernetAddress     [3] MACAddress

}

-- ===================

-- Location parameters

-- ===================

Location ::= SEQUENCE

{

locationInfo                [1] LocationInfo OPTIONAL,

positioningInfo             [2] PositioningInfo OPTIONAL,

locationPresenceReport      [3] LocationPresenceReport OPTIONAL

}

CellSiteInformation ::= SEQUENCE

{

geographicalCoordinates     [1] GeographicalCoordinates,

azimuth                     [2] INTEGER (0..359) OPTIONAL,

operatorSpecificInformation [3] UTF8String OPTIONAL

}

-- TS 29.518 [22], clause 6.4.6.2.6

LocationInfo ::= SEQUENCE

{

userLocation                [1] UserLocation OPTIONAL,

currentLoc                  [2] BOOLEAN OPTIONAL,

geoInfo                     [3] GeographicArea OPTIONAL,

rATType                     [4] RATType OPTIONAL,

timeZone                    [5] TimeZone OPTIONAL,

additionalCellIDs           [6] SEQUENCE OF CellInformation OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.7

UserLocation ::= SEQUENCE

{

eUTRALocation               [1] EUTRALocation OPTIONAL,

nRLocation                  [2] NRLocation OPTIONAL,

n3GALocation                [3] N3GALocation OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.8

EUTRALocation ::= SEQUENCE

{

tAI                         [1] TAI,

eCGI                        [2] ECGI,

ageOfLocatonInfo            [3] INTEGER OPTIONAL,

uELocationTimestamp         [4] Timestamp OPTIONAL,

geographicalInformation     [5] UTF8String OPTIONAL,

geodeticInformation         [6] UTF8String OPTIONAL,

globalNGENbID               [7] GlobalRANNodeID OPTIONAL,

cellSiteInformation         [8] CellSiteInformation OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.9

NRLocation ::= SEQUENCE

{

tAI                         [1] TAI,

nCGI                        [2] NCGI,

ageOfLocatonInfo            [3] INTEGER OPTIONAL,

uELocationTimestamp         [4] Timestamp OPTIONAL,

geographicalInformation     [5] UTF8String OPTIONAL,

geodeticInformation         [6] UTF8String OPTIONAL,

globalGNbID                 [7] GlobalRANNodeID OPTIONAL,

cellSiteInformation         [8] CellSiteInformation OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.10

N3GALocation ::= SEQUENCE

{

tAI                         [1] TAI OPTIONAL,

n3IWFID                     [2] N3IWFIDNGAP OPTIONAL,

uEIPAddr                    [3] IPAddr OPTIONAL,

portNumber                  [4] INTEGER OPTIONAL

}

-- TS 38.413 [23], clause 9.3.2.4

IPAddr ::= SEQUENCE

{

iPv4Addr                    [1] IPv4Address OPTIONAL,

iPv6Addr                    [2] IPv6Address OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.28

GlobalRANNodeID ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

aNNodeID                    [2] CHOICE

{

n3IWFID [1] N3IWFIDSBI,

gNbID   [2] GNbID,

nGENbID [3] NGENbID

}

}

-- TS 38.413 [23], clause 9.3.1.6

GNbID ::= BIT STRING(SIZE(22..32))

-- TS 29.571 [17], clause 5.4.4.4

TAI ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

tAC                         [2] TAC

}

-- TS 29.571 [17], clause 5.4.4.5

ECGI ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

eUTRACellID                 [2] EUTRACellID

}

-- TS 29.571 [17], clause 5.4.4.6

NCGI ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

nRCellID                    [2] NRCellID

}

RANCGI ::= CHOICE

{

eCGI                        [1] ECGI,

nCGI                        [2] NCGI

}

CellInformation ::= SEQUENCE

{

rANCGI                      [1] RANCGI,

cellSiteinformation         [2] CellSiteInformation OPTIONAL,

timeOfLocation              [3] Timestamp OPTIONAL

}

-- TS 38.413 [23], clause 9.3.1.57

N3IWFIDNGAP ::= BIT STRING (SIZE(16))

-- TS 29.571 [17], clause 5.4.4.28

N3IWFIDSBI ::= UTF8String

-- TS 29.571 [17], table 5.4.2-1

TAC ::= OCTET STRING (SIZE(2..3))

-- TS 38.413 [23], clause 9.3.1.9

EUTRACellID ::= BIT STRING (SIZE(28))

-- TS 38.413 [23], clause 9.3.1.7

NRCellID ::= BIT STRING (SIZE(36))

-- TS 38.413 [23], clause 9.3.1.8

NGENbID ::= CHOICE

{

macroNGENbID                [1] BIT STRING (SIZE(20)),

shortMacroNGENbID           [2] BIT STRING (SIZE(18)),

longMacroNGENbID            [3] BIT STRING (SIZE(21))

}

-- TS 29.518 [22], clause 6.4.6.2.3

PositioningInfo ::= SEQUENCE

{

positionInfo                [1] LocationData OPTIONAL,

rawMLPResponse              [2] RawMLPResponse OPTIONAL

}

RawMLPResponse ::= CHOICE

{

-- The following parameter contains a copy of unparsed XML code of the

-- MLP response message, i.e. the entire XML document containing

-- a <slia> (described in OMA-TS-MLP-V3_5-20181211-C [20], clause 5.2.3.2.2) or

-- a <slirep> (described in OMA-TS-MLP-V3_5-20181211-C [20], clause 5.2.3.2.3) MLP message.

mLPPositionData             [1] UTF8String,

-- OMA MLP result id, defined in OMA-TS-MLP-V3_5-20181211-C [20], Clause 5.4

mLPErrorCode                [2] INTEGER (1..699)

}

-- TS 29.572 [24], clause 6.1.6.2.3

LocationData ::= SEQUENCE

{

locationEstimate            [1] GeographicArea,

accuracyFulfilmentIndicator [2] AccuracyFulfilmentIndicator OPTIONAL,

ageOfLocationEstimate       [3] AgeOfLocationEstimate OPTIONAL,

velocityEstimate            [4] VelocityEstimate OPTIONAL,

civicAddress                [5] CivicAddress OPTIONAL,

positioningDataList         [6] SET OF PositioningMethodAndUsage OPTIONAL,

gNSSPositioningDataList     [7] SET OF GNSSPositioningMethodAndUsage OPTIONAL,

eCGI                        [8] ECGI OPTIONAL,

nCGI                        [9] NCGI OPTIONAL,

altitude                    [10] Altitude OPTIONAL,

barometricPressure          [11] BarometricPressure OPTIONAL

}

-- TS 29.518 [22], clause 6.2.6.2.5

LocationPresenceReport ::= SEQUENCE

{

type                        [1] AMFEventType,

timestamp                   [2] Timestamp,

areaList                    [3] SET OF AMFEventArea OPTIONAL,

timeZone                    [4] TimeZone OPTIONAL,

accessTypes                 [5] SET OF AccessType OPTIONAL,

rMInfoList                  [6] SET OF RMInfo OPTIONAL,

cMInfoList                  [7] SET OF CMInfo OPTIONAL,

reachability                [8] UEReachability OPTIONAL,

location                    [9] UserLocation OPTIONAL,

additionalCellIDs           [10] SEQUENCE OF CellInformation OPTIONAL

}

-- TS 29.518 [22], clause 6.2.6.3.3

AMFEventType ::= ENUMERATED

{

locationReport(1),

presenceInAOIReport(2)

}

-- TS 29.518 [22], clause 6.2.6.2.16

AMFEventArea ::= SEQUENCE

{

presenceInfo                [1] PresenceInfo OPTIONAL,

lADNInfo                    [2] LADNInfo OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.27

PresenceInfo ::= SEQUENCE

{

presenceState               [1] PresenceState OPTIONAL,

trackingAreaList            [2] SET OF TAI OPTIONAL,

eCGIList                    [3] SET OF ECGI OPTIONAL,

nCGIList                    [4] SET OF NCGI OPTIONAL,

globalRANNodeIDList         [5] SET OF GlobalRANNodeID OPTIONAL

}

-- TS 29.518 [22], clause 6.2.6.2.17

LADNInfo ::= SEQUENCE

{

lADN                        [1] UTF8String,

presence                    [2] PresenceState OPTIONAL

}

-- TS 29.571 [17], clause 5.4.3.20

PresenceState ::= ENUMERATED

{

inArea(1),

outOfArea(2),

unknown(3),

inactive(4)

}

-- TS 29.518 [22], clause 6.2.6.2.8

RMInfo ::= SEQUENCE

{

rMState                     [1] RMState,

accessType                  [2] AccessType

}

-- TS 29.518 [22], clause 6.2.6.2.9

CMInfo ::= SEQUENCE

{

cMState                     [1] CMState,

accessType                  [2] AccessType

}

-- TS 29.518 [22], clause 6.2.6.3.7

UEReachability ::= ENUMERATED

{

unreachable(1),

reachable(2),

regulatoryOnly(3)

}

-- TS 29.518 [22], clause 6.2.6.3.9

RMState ::= ENUMERATED

{

registered(1),

deregistered(2)

}

-- TS 29.518 [22], clause 6.2.6.3.10

CMState ::= ENUMERATED

{

idle(1),

connected(2)

}

-- TS 29.572 [24], clause 6.1.6.2.5

GeographicArea ::= CHOICE

{

point                       [1] Point,

pointUncertaintyCircle      [2] PointUncertaintyCircle,

pointUncertaintyEllipse     [3] PointUncertaintyEllipse,

polygon                     [4] Polygon,

pointAltitude               [5] PointAltitude,

pointAltitudeUncertainty    [6] PointAltitudeUncertainty,

ellipsoidArc                [7] EllipsoidArc

}

-- TS 29.572 [24], clause 6.1.6.3.12

AccuracyFulfilmentIndicator ::= ENUMERATED

{

requestedAccuracyFulfilled(1),

requestedAccuracyNotFulfilled(2)

}

-- TS 29.572 [24], clause

VelocityEstimate ::= CHOICE

{

horVelocity                         [1] HorizontalVelocity,

horWithVertVelocity                 [2] HorizontalWithVerticalVelocity,

horVelocityWithUncertainty          [3] HorizontalVelocityWithUncertainty,

horWithVertVelocityAndUncertainty   [4] HorizontalWithVerticalVelocityAndUncertainty

}

-- TS 29.572 [24], clause 6.1.6.2.14

CivicAddress ::= SEQUENCE

{

country                             [1] UTF8String,

a1                                  [2] UTF8String OPTIONAL,

a2                                  [3] UTF8String OPTIONAL,

a3                                  [4] UTF8String OPTIONAL,

a4                                  [5] UTF8String OPTIONAL,

a5                                  [6] UTF8String OPTIONAL,

a6                                  [7] UTF8String OPTIONAL,

prd                                 [8] UTF8String OPTIONAL,

pod                                 [9] UTF8String OPTIONAL,

sts                                 [10] UTF8String OPTIONAL,

hno                                 [11] UTF8String OPTIONAL,

hns                                 [12] UTF8String OPTIONAL,

lmk                                 [13] UTF8String OPTIONAL,

loc                                 [14] UTF8String OPTIONAL,

nam                                 [15] UTF8String OPTIONAL,

pc                                  [16] UTF8String OPTIONAL,

bld                                 [17] UTF8String OPTIONAL,

unit                                [18] UTF8String OPTIONAL,

flr                                 [19] UTF8String OPTIONAL,

room                                [20] UTF8String OPTIONAL,

plc                                 [21] UTF8String OPTIONAL,

pcn                                 [22] UTF8String OPTIONAL,

pobox                               [23] UTF8String OPTIONAL,

addcode                             [24] UTF8String OPTIONAL,

seat                                [25] UTF8String OPTIONAL,

rd                                  [26] UTF8String OPTIONAL,

rdsec                               [27] UTF8String OPTIONAL,

rdbr                                [28] UTF8String OPTIONAL,

rdsubbr                             [29] UTF8String OPTIONAL

}

-- TS 29.572 [24], clause 6.1.6.2.15

PositioningMethodAndUsage ::= SEQUENCE

{

method                              [1] PositioningMethod,

mode                                [2] PositioningMode,

usage                               [3] Usage

}

-- TS 29.572 [24], clause 6.1.6.2.16

GNSSPositioningMethodAndUsage ::= SEQUENCE

{

mode                                [1] PositioningMode,

gNSS                                [2] GNSSID,

usage                               [3] Usage

}

-- TS 29.572 [24], clause 6.1.6.2.6

Point ::= SEQUENCE

{

geographicalCoordinates             [1] GeographicalCoordinates

}

-- TS 29.572 [24], clause 6.1.6.2.7

PointUncertaintyCircle ::= SEQUENCE

{

geographicalCoordinates             [1] GeographicalCoordinates,

uncertainty                         [2] Uncertainty

}

-- TS 29.572 [24], clause 6.1.6.2.8

PointUncertaintyEllipse ::= SEQUENCE

{

geographicalCoordinates             [1] GeographicalCoordinates,

uncertainty                         [2] UncertaintyEllipse,

confidence                          [3] Confidence

}

-- TS 29.572 [24], clause 6.1.6.2.9

Polygon ::= SEQUENCE

{

pointList                           [1] SET SIZE (3..15) OF GeographicalCoordinates

}

-- TS 29.572 [24], clause 6.1.6.2.10

PointAltitude ::= SEQUENCE

{

point                               [1] GeographicalCoordinates,

altitude                            [2] Altitude

}

-- TS 29.572 [24], clause 6.1.6.2.11

PointAltitudeUncertainty ::= SEQUENCE

{

point                               [1] GeographicalCoordinates,

altitude                            [2] Altitude,

uncertaintyEllipse                  [3] UncertaintyEllipse,

uncertaintyAltitude                 [4] Uncertainty,

confidence                          [5] Confidence

}

-- TS 29.572 [24], clause 6.1.6.2.12

EllipsoidArc ::= SEQUENCE

{

point                               [1] GeographicalCoordinates,

innerRadius                         [2] InnerRadius,

uncertaintyRadius                   [3] Uncertainty,

offsetAngle                         [4] Angle,

includedAngle                       [5] Angle,

confidence                          [6] Confidence

}

-- TS 29.572 [24], clause 6.1.6.2.4

GeographicalCoordinates ::= SEQUENCE

{

latitude                            [1] UTF8String,

longitude                           [2] UTF8String,

mapDatumInformation                 [3] OGCURN OPTIONAL

}

-- TS 29.572 [24], clause 6.1.6.2.22

UncertaintyEllipse ::= SEQUENCE

{

semiMajor                           [1] Uncertainty,

semiMinor                           [2] Uncertainty,

orientationMajor                    [3] Orientation

}

-- TS 29.572 [24], clause 6.1.6.2.18

HorizontalVelocity ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle

}

-- TS 29.572 [24], clause 6.1.6.2.19

HorizontalWithVerticalVelocity ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle,

vSpeed                              [3] VerticalSpeed,

vDirection                          [4] VerticalDirection

}

-- TS 29.572 [24], clause 6.1.6.2.20

HorizontalVelocityWithUncertainty ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle,

uncertainty                         [3] SpeedUncertainty

}

-- TS 29.572 [24], clause 6.1.6.2.21

HorizontalWithVerticalVelocityAndUncertainty ::= SEQUENCE

{

hspeed                              [1] HorizontalSpeed,

bearing                             [2] Angle,

vSpeed                              [3] VerticalSpeed,

vDirection                          [4] VerticalDirection,

hUncertainty                        [5] SpeedUncertainty,

vUncertainty                        [6] SpeedUncertainty

}

-- The following types are described in TS 29.572 [24], table 6.1.6.3.2-1

Altitude ::= UTF8String

Angle ::= INTEGER (0..360)

Uncertainty ::= INTEGER (0..127)

Orientation ::= INTEGER (0..180)

Confidence ::= INTEGER (0..100)

InnerRadius ::= INTEGER (0..65535)

AgeOfLocationEstimate ::= INTEGER (0..32767)

HorizontalSpeed ::= UTF8String

VerticalSpeed ::= UTF8String

SpeedUncertainty ::= UTF8String

BarometricPressure ::= INTEGER (30000..155000)

-- TS 29.572 [24], clause 6.1.6.3.13

VerticalDirection ::= ENUMERATED

{

upward(1),

downward(2)

}

-- TS 29.572 [24], clause 6.1.6.3.6

PositioningMethod ::= ENUMERATED

{

cellID(1),

eCID(2),

oTDOA(3),

barometricPresure(4),

wLAN(5),

bluetooth(6),

mBS(7)

}

-- TS 29.572 [24], clause 6.1.6.3.7

PositioningMode ::= ENUMERATED

{

uEBased(1),

uEAssisted(2),

conventional(3)

}

-- TS 29.572 [24], clause 6.1.6.3.8

GNSSID ::= ENUMERATED

{

gPS(1),

galileo(2),

sBAS(3),

modernizedGPS(4),

qZSS(5),

gLONASS(6)

}

-- TS 29.572 [24], clause 6.1.6.3.9

Usage ::= ENUMERATED

{

unsuccess(1),

successResultsNotUsed(2),

successResultsUsedToVerifyLocation(3),

successResultsUsedToGenerateLocation(4),

successMethodNotDetermined(5)

}

-- TS 29.571 [17], table 5.2.2-1

TimeZone ::= UTF8String

-- Open Geospatial Consortium URN [35]

OGCURN ::= UTF8String

END

*** End of All MODIFICATIONS ***