3GPP SA3LI#79-e-a	S3i200612

October 19-23, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Input to TR 33.929 – IMS conferencing – part V

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI16

Abstract of the contribution: This pCR provides a call flow to illustrate the LI steps when a target call to just one other conferee is reconfigured to a 2-party call.   These is a re-incarnated flow from TS 33.107.

Proposed changes

### 5.X.5	Reconfiguration from conference to two-party session

Figure 5.X-7: Conference is reconfigured to a two-party session

D3 represents the dialogue of the session between Party A (target) and the conference. D10 represents the dialogue between Party B and the conference.

The IRI/CC delivered for the conferencing (i.e., D3) uses the Correlation ID 3.

At the end of this flow, Party A (target) and Party B are connected directly (without the conference). The IRI/CC delivered for this session between Party A (target) and Party B (D3 and D10) uses the Correlation ID 3.

NOTE:	Reconfiguration may done using other ways (e.g., AS/MRFC sending a REFER to Party_A (target) to invite Party C). The sequence of steps and the Correlation ID used can be different with such a flow.