3GPP TSG-SA3 Meeting #81-LI-e-b	s3i210342

# Online, , 19th May 2021 - 21st May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0130 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of TWIF and TNGF as Non-3GPP Accesses |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI(OTD, OFCOM(CH), Nokia, Nokia Shanghai Bell) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI16 |  |  |  |  |  | Date: |  |  | 2021-05-12 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Additional types of Non-3GPP access were defined in Release 16 with the addition of the TWIF and TNGF. The current version of TS 33.127 does not support these changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of TWIF and TNGF to non-3GPP Access descriptions and diagrams. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 33.127 will not align with other documents. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.2.2.6, 6.2.2.7, 6.2.3.6, 6.2.5.6, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 33.127 CR 0129 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TS 33.127 CR 0129 (TDoc s3i210341) is the Rel 16 tdoc this document is mirrored from. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of First Change ***

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5GS	5G System

ADMF	LI Administration Function

AMF	Access and Mobility Management Function

AS	Application Server

AUSF	Authentication Server Function

BBIFF	Bearer Binding Intercept and Forward Function

BSS	Business Support System

CAG	Closed Access Group

CC	Content of Communication

CP	Control Plane

CSI	Cell Supplemental Information

CSP	Communication Service Provider

CUPS	Control and User Plane Separation

DN	Data Network

DNAI	Data Network Access Identifier

E-CSCF	Emergency – Call Session Control Function

GPSI	Generic Public Subscription Identifier

HMEE	Hardware Mediated Execution Enclave

HR	Home Routed

IBCF	Interconnection Border Control Functions

ICF	Identifier Caching Function

IEF	Identifier Event Function

IMS-AGW	IMS Access Gateway

IM-MGW	IM Media Gateway

IP	Interception Product

IQF	Identifier Query Function

IRI	Intercept Related Information

LALS	Lawful Access Location Services

LBO	Local Break Out

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

LI CA	Lawful Interception Certificate Authority

LICF	Lawful Interception Control Function

LI_HI1	Lawful Interception Handover Interface 1

LI_HI2	Lawful Interception Handover Interface 2

LI_HI3	Lawful Interception Handover Interface 3

LI_HI4	Lawful Interception Handover Interface 4

LI_HIQR	Lawful Interception Handover Interface Query Response

LIID	Lawful Interception Identifier

LIPF	Lawful Interception Provisioning Function

LIR	Location Immediate Request

LI_SI	Lawful Interception System Information Interface

LI_T1	Lawful Interception Internal Triggering Interface 1

LI_T2	Lawful Interception Internal Triggering Interface 2

LI_T3	Lawful Interception Internal Triggering Interface 3

LI_X0		Lawful Interception Internal Interface 0

LI_X1	Lawful Interception Internal Interface 1

LI_X2	Lawful Interception Internal Interface 2

LI_X3	Lawful Interception Internal Interface 3

LI_X3A	Lawful Interception Internal Interface 3 Aggregator

LI_XEM1	Lawful Interception Internal Interface Event Management Interface 1

LI_XER	Lawful Interception Internal Interface Event Record

LI_XQR	Lawful Interception Internal Interface Query Response

LMF	Location Management Function

LMISF	LI Mirror IMS State Function

LMISF-CC	LMISF for the handling of CC

LMISF-IRI	LMISF for the handling of IRI

LTF	Location Triggering Function

MA	Multi-Access

MANO	Management and Orchestration

MDF	Mediation and Delivery Function

MDF2	Mediation and Delivery Function 2

MDF3	Mediation and Delivery Function 3

MRFP	Multimedia Resource Function Processor

N9HR	N9 Home Routed

N3IWF	Non 3GPP Inter Working Function

NFV	Network Function Virtualisation

NFVI	Network Function Virtualisation Infrastructure

NFVO	Network Function Virtualisation Orchestrator

NPLI	Network Provided Location Information

NR	New Radio

NRF	Network Repository Function

NSSF	Network Slice Selection Function

OSS	Operations Support System

PAG	POI Aggregator

PCF	Policy Control Function

P-CSCF	Proxy - Call Session Control Function

PEI	Permanent Equipment Identifier

PGW	PDN Gateway

PGW-U	PDN Gateway User Plane

POI	Point Of Interception

PLMN	Public Land Mobile Network

PTC	Push to Talk over Cellular

S8HR	S8 Home Routed

SIRF	System Information Retrieval Function

S-CSCF	Serving - Call Session Control Function

SMF	Session Management Function

SMSF	SMS-Function

SUCI	Subscriber Concealed Identifier

SUPI	Subscriber Permanent Identifier

TF	Triggering Function

TNGF	Trusted Non-3GPP Gateway Function

TWIF	Trusted WLAN Interworking Function

TrGW	Transit Gateway

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UPF	User Plane Function

VNF	Virtual Network Function

VNFC	Virtual Network Function Component

xCC	LI_X3 Communications Content

xIRI	LI_X2 Intercept Related Information

*** Start of Change 2 of 5 ***

#### 6.2.2.6	Specific IRI parameters

The list of parameters in each xIRI are defined in TS 33.128 [15]. The following give a summary.

The registration xIRI shall include the following:

-	Registration type information.

-	Access type information.

-	Requested slice information.

The deregistration xIRI shall include the following:

-	UE initiated de-registration.

-	Access type information.

-	Network initiated de-registration.

The location update xIRI shall include the following:

-	Location of the target UE (see clause 7.3).

The identifier association xIRI shall include the following:

-	Subscription permanent identifier.

-	Temporary identifier association (i.e. SUCI or 5G-GUTI).

-	Association change type indication.

The start of interception with already registered UE xIRI shall include the following:

-	Access type information.

-	Requested slice information.

The unsuccessful communication attempt xIRI shall include the following:

-	Rejected type of communication attempt.

-	Access type information.

-	Failure reason.

When the access type is non-3GPP, the IP address used by the UE to reach the N3IWF, TNGF or TWIF shall be reported. The port shall also be reported if available.

#### 6.2.2.7	Network topologies

The AMF shall provide the IRI-POI functions in the following network topology cases:

-	Non-roaming case.

-	Roaming case, in VPLMN.

-	Roaming case, in HPLMN for non-3GPP access.

In a roaming case, it is possible that the target UE may use non-3GPP access with the N3IWF, TNGF or TWIF present in the HPLMN.

*** Start of Change 3 of 5 ***

The SMF shall provide the IRI-POI functions in the following network topology cases:

-	Non-roaming case.

-	Roaming case, in VPLMN.

-	Roaming case, in HPLMN.

-	Non-3GPP access case, in the PLMN where N3IWF, TNGF or TWIF resides.

When the target UE has multiple PDU sessions active, the generation and delivery of xCC for each PDU session shall be done independently, each with separate correlation information.

When a target UE's PDU session involves multiple Data Network (DN) connections (i.e. multiple connections to the same DN as described in clause A.3 of the present document), the generation and delivery of xCC shall be done in such a way that:

-	All applicable user plane packets are captured and delivered.

-	Duplicate delivery of CC is suppressed to the extent possible.

-	Each user plane packet is delivered with the associated DN Access Identifier (DNAI).

A PDU session may involve more than one UPFs. In that case, the CC-TF present in the SMF shall determine which UPF(s) is (are) more suitable to provide the CC-POI functions adhering to the above requirements. Furthermore, independent of which UPF is used to generate the xCC, the CC delivered from the MDF3 shall be correlated to the IRI messages related to the PDU session.

*** Start of Change 4 of 5 ***

The SMSF shall provide the IRI-POI functions in the following network topology cases:

-	Non-roaming case.

-	Roaming case, in VPLMN.

NOTE:	SMS message delivery over non-3GPP access with N3IWF, TNGF or TWIF in the HPLMN is considered a non-roaming case.

*** Start of Change 5 of 5 ***

# A.4	Non-3GPP access in a non-roaming scenario

## A.4.1	General

When the target UE is connected to the 5G core network via non-3GPP access, the POIs present in the following NFs of the PLMN where the N3IWF, TNGF or TWIF resides provide the LI functions:

-	AMF.

-	SMF.

-	UPF.

-	SMSF.

When the PLMN that has the N3IWF is the HPLMN, as illustrated in clause A.1, the IRI-POI present in the UDM also provide the LI functions.

When the PLMN that has N3IWF, TNGF or TWIF is different from the PLMN that provides the 3GPP access to the target UE, two different AMFs are involved in handling the target UE's registration accepts (this is not illustrated in this clause). In this case, depending on the operator policy, the SMSF present in either of the two networks may perform the routing of SMS messages to and from the target UE.

The PLMN that provides the 3GPP access can be a VPLMN and PLMN where the N3IWF, TNGF or TWIF resides can be the HPLMN. In this case, the AMF in the HPLMN provides the IRI-POI functions for non-3GPP access related registration events when the target UE is roaming. The SMSF present in the HPLMN may have to provide the IRI-POI functions for the SMS related messages routed via non-3GPP access network.

## A.4.2	Topology view

The overall network configuration for non-3GPP access in a non-roaming scenario with the LI aspects is shown in figure A.4-1. In this view, the target UE is not connected to a 3GPP access network.

The 5G core system is shown using the service-based representation (as shown in TS 23.501 [2]) with the use of point-to-point LI system.

Figure A.4-1: Network topology showing LI for non-3GPP access to 5G via N3IWF

Figure A.4-Fi1: Network topology showing LI for non-3GPP access to 5G via TNGF

Figure A.4-Fi1 shows the network topology of 5G system in a service-based representation, however, all the LI-related interfaces remain to be point-to-point.

Figure A.4-Fi2: Network topology showing LI for non-3GPP access to 5G via TWIF

Figures A.4-1, A.4-Fi1 and A.4-Fi2 show the network topology of 5G system in a service-based representation, however, all the LI-related interfaces remain to be point-to-point.

The IRI-POIs present in the AMF, UDM, SMSF and SMF deliver the xIRI to the MDF2 and CC-POI present in the UPF delivers the xCC to the MDF3. The MDF3 address to CC-POI present in UPF is provided by the CC-TF present in the SMF over LI_T3 reference point.

The LIPF present in the ADMF provisions the IRI-POIs present in the NFs with the intercept related data. The LI_X1 interfaces between the LIPF and the UPF is to monitor the user plane data.

*** End of All Changes ***