3GPP TSG-SA3 Meeting #82-LI-e-a	s3i210410

# Online, , 12th Jul 2021 - 16th Jul 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0216 | Rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RCS in TS 33.128 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2021-07-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RCS services (capability discovery, standalone messaging, chat, file transfer) cannot be intercepted |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds Stage 3 for RCS Services |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LI for RCS Services would continue to be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.X, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 33.127, CR 0135 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.126: "Lawful Interception Requirements".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 33.127: "Lawful Interception (LI) Architecture and Functions".

[6]	ETSI TS 103 120: "Lawful Interception (LI); Interface for warrant information".

[7]	ETSI TS 103 221-1: "Lawful Interception (LI); Internal Network Interfaces; Part 1: X1".

[8]	ETSI TS 103 221-2: "Lawful Interception (LI); Internal Network Interfaces; Part 2: X2/X3".

[9]	ETSI TS 102 232-1: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 1: Handover specification for IP delivery".

[10]	ETSI TS 102 232-7: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 7: Service-specific details for Mobile Services".

[11]	3GPP TS 33.501: "Security Architecture and Procedures for the 5G System".

[12]	3GPP TS 33.108: "3G security; Handover interface for Lawful Interception (LI)".

[13]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS)".

[14]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General Aspects".

[15]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[16]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[17]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[18]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[19]	3GPP TS 23.003: "Numbering, addressing and identification ".

[20]	OMA-TS-MLP-V3_5-20181211-C: "Open Mobile Alliance; Mobile Location Protocol, Candidate Version 3.5", https://www.openmobilealliance.org/release/MLS/V1_4-20181211-C/OMA-TS-MLP-V3_5-20181211-C.pdf.

[21]	3GPP TS 29.540: "5G System; SMS Services; Stage 3".

[22]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[23]	3GPP TS 38.413: "NG Application Protocol (NGAP)".

[24]	3GPP TS 29.572: "Location Management Services; Stage 3".

[25]	3GPP TS 29.503: "5G System; Unified Data Management Services".

[26]	IETF RFC 815: "IP datagram reassembly algorithms".

[27]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[28]	IETF RFC 793: "Transmission Control Protocol".

[29]	IETF RFC 768: "User Datagram Protocol".

[30]	IETF RFC 4340: "Datagram Congestion Control Protocol (DCCP)".

[31]	IETF RFC 4960: "Stream Control Transmission Protocol".

[32]	IANA (www.iana.org): Assigned Internet Protocol Numbers, "Protocol Numbers".

[33]	IETF RFC 6437: "IPv6 Flow Label Specification".

[34]	IETF RFC 791: "Internet Protocol".

[35]	Open Geospatial Consortium OGC 05-010: "URNs of definitions in ogc namespace".

[36]	3GPP TS 33.107: "3G security; Lawful interception architecture and functions".

[37]	3GPP TS 37.340: "Evolved Universal Radio Access (E-UTRA) and NR-Multi-connectivity; Stage 2".

[38]	3GPP TS 36.413: "S1 Application Protocol (S1AP)".

[39]	OMA-TS-MMS_ENC-V1_3-20110913-A: "Multimedia Messaging Service Encapsulation Protocol".

[40]	3GPP TS 23.140: "Multimedia Messaging Protocol. Functional Description. Stage 2".

[41]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[42]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[43]	IETF RFC 4566: "SDP: Session Description Protocol".

[44]	3GPP TS 24.193: "Stage 3: Access Traffic Steering, Switching and Splitting (ATSSS)".

[45]	3GPP TS 29.509: "5G System; Authentication Server Services; Stage 3".

[46]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[47]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[48]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[49]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository services for Subscription Data; Stage 3".

[50]	3GPP TS 23.401 "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[51]	3GPP TS 24.301 "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS), Stage 3".

[52]	3GPP TS 23.271 "Functional stage 2 description of Location Services (LCS)".

[53]	3GPP TS 29.172 "Evolved Packet Core (EPC) LCS Protocol (ELP) between the Gateway Mobile Location Centre (GMLC) and the Mobile Management Entity (MME); SLg interface".

[54]	3GPP TS 29.171 "LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface".

[55]	3GPP TS 24.379: "Mission Critical Push to Talk (MCPTT) call control; protocol specification".

[56]	OMA-TS-PoC-System_Description-V2_1-20110802-A: "OMA PoC System Description".

[57]	3GPP TS 29.541: "5G System; Network Exposure (NE) function services for Non-IP Data Delivery (NIDD); Stage 3".

[58]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[59]	3GPP TS 29.338: "Diameter based protocols to support Short Message Service (SMS) capable Mobile Management Entities (MMEs); Stage 3".

[60]	3GPP TS 29.337: "Diameter-based T4 interface for communications with packet data networks and applications".

[61]	3GPP TS 24.250: "Protocol for Reliable Data Service; Stage 3".

[62]	3GPP TS 29.128: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) interfaces for interworking with packet data networks and applications".

[63]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[64]	3GPP TS 29.598: "5G System; Unstructured Data Storage Services; Stage3".

[XA]	IETF RFC 4566: "SDP: Session Description Protocol ».

# First change

## 7.X	RCS

### 7.X.1	Provisioning over LI_X1

#### 7.X.1.1	General

#### For large message mode standalone messaging, 1-to-1 chat, group chat and file transfer:

-	If delivery type for the warrant is "HI2andHI3", then the IRI-POI and the CC-POI in the RCS server, the MDF2 and MDF3 shall be provisioned.

-	If delivery type for the warrant is "HI2Only" only, then the IRI-POI in the RCS server and the MDF2 shall be provisioned.

-	Delivery type "HI3Only" only is not applicable to the warrant.

#### For capability discovery and pager mode standalone messaging:

-	The delivery type for the warrant is "HI2Only" only; the IRI-POI in the RCS server and the MDF2 shall be provisioned.

#### 7.X.1.2	Provisioning of the IRI-POI and CC-POI in RCS server

The IRI-POI and CC-POI present in the RCS server are provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI in the RCS server shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used).

-	IMPU.

NOTE: 	For capability discovery, additional target identifiers are applicable: IMPI and IMEI. For file transfer, IMPI is also applicable.

### 7.X.2	LI for capability discovery

#### 7.X.2.1	Generation of xIRI at IRI-POI in RCS server over LI_X2

##### 7.X.2.1.1	General

The IRI-POI present in the RCS server shall send xIRI over LI_X2 for each of the events listed in TS 33.127 [5] clause 7.X.3, each of which is described in the following clauses.

##### 7.X.2.1.2	Registration

The IRI-POI present in the RCS server shall generate an xIRI containing an RCSRegistration record when the IRI-POI present in the RCS server detects that a RCS target matching one of the RCS identifiers, provided via LI_X1 has registered, re-registered, or de-registered for RCS services, regardless of whether it is successful or unsuccessful. Accordingly, the IRI-POI in the RCS server generates the xIRI when the following event is detected:

-	when the RCS server receives a SIP REGISTER from an RCS target.

Table 7.X-1: Payload for RCSRegistration record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| rCSServerURI | Shall include the identity of the RCS server serving the RCS target. | M |
| rCSRegistrationRequest | Identifies the type of registration request (register, re-register, or de-register). | M |
| rCSRegistrationOutcome | Identifies success or failure of the registration. | M |
| rCSTargetCapabilites | Identifies the list of RCS service capabilities supported by the target. | M |
| location | Shall include the target’s location when reporting of the target’s location information is authorized and available. | C |

##### 7.X.2.1.3	Capability discovery

The IRI-POI present in the RCS server shall generate an xIRI containing an RCSCapabilityDiscovery when the IRI-POI present in the RCS server detects that an RCS target has received RCS service capabilities for his contact(s).

Accordingly, the IRI-POI in the RCS server generates the xIRI when any of the following events is detected:

-	The RCS server returns a SIP response which response code is 200, 480, 408, 404 or 604 for a SIP OPTIONS request sent by the target. The OPTIONS request contains the capabilities of the target in the Contact header and the response includes the RCS state and RCS capabilities of a target’s contact.

-	The RCS server sends a SIP NOTIFY request to the target with the Event header set to "presence.winfo". The NOTIFY request contains the RCS state and RCS capabilities of a target’s contact.

-	The RCS server receives a SIP PUBLISH request from the target. The PUBLISH request contains the RCS capabilities of the target.

Table 7.X-2: Payload for RCSCapabilityDiscovery record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| contactIdentity | IMPUs associated with the target’s contact. | C |
| targetCapabilities | List of RCS service capabilities supported by the target. | C |
| contactCapabilities | List of RCS service capabilities supported by the contact. | C |
| rCSFlag | Indicates if the contact is an RCS user; TRUE: yes; FALSE: No | C |
| contactRCSState | Indicates the current state of the contact if the contact is an RCS user: registered, deregistered. | C |
| location | Shall include the target’s location when reporting of the target’s location information is authorized and available. | C |

### 7.X.3	LI for pager mode standalone messaging

#### 7.X.3.1	Generation of xIRI at IRI-POI in RCS server over LI_X2

##### 7.X.3.1.1	General

The IRI-POI present in the RCS server shall send xIRI over LI_X2 for each of the events listed in TS 33.127 [5] clause 7.X.4, each of which is described in the following clauses.

##### 7.X.3.1.2	Pager mode message delivery

When a pager mode standalone message is sent or received by the target, the IRI POI in the RCS Server shall generate an xIRI containing an RCSPagerModeMessageDelivery record; this xRI shall also be generated for NonLocal ID interception. Accordingly, the IRI-POI in the RCS server generates the xIRI when the following events are detected:

-	When the RCS server receives a SIP MESSAGE from the target. The SIP MESSAGE has the "Contact" header which includes +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.msg"; the SIP "Content-Type" header shall be "message/cpim".

-	When the RCS Server sends a SIP MESSAGE to the target. The SIP MESSAGE has the "Contact" header which includes +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.msg"; the "Content-Type" header shall be "message/cpim".

Table 7.X-3: Payload for RCSPagerModeMessageDelivery record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| originatingIdentity | Shall be provided to identify the destination address for the Pager Mode message. | M |
| destinationIdentities | Shall be provided to identify the destination address(es) of the Pager Mode message. | M |
| messageID | Message Identifier used to correlate the message delivery and message delivery notifications. | M |
| pagerModeMessageContent | Provided if the delivery of Pager Mode message is lawfully authorized. | C |
| direction | Shall be provided to indicate whether the pager mode message is sent or received by the target. | M |
| fileTransferFlag | Indicates if the Pager Mode message is used for file transfer. | C |
| fileURL | If the pager mode message is used for file transfer, fileURL provides the file URL | C |
| location | Shall include the target’s location when reporting of the target’s location information is authorized and available. | C |

##### 7.X.3.1.3	Pager mode message delivery notification

The IRI-POI in the RCS Server shall generate an xIRI containing an RCSPagerModeMessageDeliveryNotification record when this IRI-POI in the RCS Server delivers a SIP MESSAGE containing delivery or display notification.

Accordingly, the IRI-POI in the RCS server generates the xIRI when the following events are detected:

-	When the RCS server receives a SIP MESSAGE from the target. The SIP MESSAGE has the "Contact" header which includes +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.msg"; the SIP "Content-Type" header shall be "message/cpim"; the CPIM "Content-Type" header shall be "message/imdn+xml".

-	When the RCS Server sends a SIP MESSAGE to the target. The SIP MESSAGE has the SIP "Contact" header which includes +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.msg"; the SIP "Content-Type" header shall be "message/cpim"; the CPIM "Content-Type" header shall be "message/imdn+xml ".

deliveryOutcome : The delivery notification type indicates whether or not the Pager Mode CPM Standalone Message has been delivered to the Recipient. The delivery notification type can have the following states:

-	"delivered" to indicate successful delivery.

-	"failed" to indicate failure in delivery.

-	"forbidden" to indicate denial for the sender to receive the requested notification.

-	"error" to indicate an error in determining the fate of a Pager Mode message.

displayOutcome : The display notification type indicates whether or not the Recipient rendered the Pager Mode CPM Standalone Message to the user. The display notification type can have the following states:

-	"displayed" to indicate that the Pager Mode CPM Standalone Message has been rendered to the user.

-	"forbidden" to indicate denial, by the Pager Mode CPM Standalone Message Recipient, for the Pager Mode CPM Standalone Message Sender to receive the requested notification.

-	"error" to indicate an error in determining the fate of a Pager Mode CPM Standalone Message.

Table 7.X-4: Payload for RCSPagerModeMessageDeliveryNotification record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| originatingIdentity | Shall be provided to identify the address of the sender of the Pager Mode message delivery notification. | M |
| destinationIdentity | Shall be provided to identify the destination address for the Pager Mode message delivery notification. | M |
| messageID | Message Identifier present in the delivery notification. | M |
| deliveryOutcome | Provides the delivery outcome of the Pager Mode message: “delivered”, “failed”, “forbidden”, “error”. | C |
| displayOutcome | Provides the display outcome of the Pager Mode message: “displayed”, “forbidden”, “error”. | C |

### 7.X.4	LI for large message mode standalone messaging

#### 7.X.4.1	Generation of xIRI at IRI-POI in RCS server over LI_X2

##### 7.X.4.1.1	General

The IRI-POI present in the RCS server shall send xIRI over LI_X2 for each of the events listed in TS 33.127 [5] clause 7.X.5, each of which is described in the following clauses.

##### 7.X.4.1.2	Large message mode session establishment

The IRI-POI in the RCS server shall generate an xIRI containing an RCSLargeMessageModeSessionEstablishment record when a SIP Session is established for large message mode message delivery and possibly delivery and display notifications. Accordingly, the IRI-POI in the RCS server generates the xIRI when any of the following events is detected:

-	When the RCS server receives a SIP 200 OK from the target in response to a SIP INVITE sent to the target with presence of +g.gsma.rcs.cpm.pager-large or +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.largemsg" in the SIP "Contact" header.

-	When the RCS Server returns a SIP 200 OK to the target in response to a SIP INVITE received from the target with presence of +g.gsma.rcs.cpm.pager-large or +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.largemsg" in the SIP "Contact" header.

Table 7.X-5: Payload for RCSLargeMessageModeSessionEstablishment record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| direction | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget.". | M |
| originationIdentity | Shall identify the originating of the SIP session for Large Message Mode message delivery. | M |
| destinationIdentities | Shall identify the individual destination address(es) of the SIP session, recipient(s) of the Large Message Mode message. | M |
| bearerCapability | Shall provide when known the media characteristics information elements of the SIP session, encoded in SDP format as per RFC 4566 [XA] clause 5. | C |
| largeMessageSize | Indicates the size of the unique large message shall be sent in the SIP Session. | C |
| fileTransferFlag | Indicates if the session is established for a file transfer. | C |
| location | Shall include the target’s location when reporting of the target’s location information is authorized and available. | C |

##### 7.X.4.1.3	Large message mode unsuccessful procedure

The IRI-POI in the RCS server shall generate an xIRI containing an RCSLargeMessageModeUnsuccessfulProcedure record when a SIP Session establishment procedure for large message mode message delivery fails. Accordingly, the IRI-POI in the RCS server generates the xIRI when any of the following events is detected:

-	When the RCS server receives a SIP 4XX, SIP 5XX or SIP 6XX from the target in response to a SIP INVITE sent to the target with presence of +g.gsma.rcs.cpm.pager-large or +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.largemsg" in the SIP "Contact" header.

-	When the RCS Server returns a SIP 4XX, SIP 5XX or SIP 6XX to the target in response to a SIP INVITE received from the target with presence of +g.gsma.rcs.cpm.pager-large or +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.largemsg" in the SIP "Contact" header.

Table 7.X-6: Payload for RCSLargeMessageModeUnsuccessfulProcedure record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| direction | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget". | M |
| originationIdentity | Shall identify the originating of the SIP session for Large Message Mode message delivery. | M |
| destinationIdentities | Shall identify the individual destination address(es) of the SIP session, recipient(s) of the Large Message Mode message. | M |
| fileTransferFlag | Indicates if the session is established for a file transfer. | C |
| location | Shall include the target’s location when reporting of the target’s location information is authorized and available. | C |
| cause | Cause for failure of SIP session establishment procedure, corresponding to the SIP response code and SIP reason phrase. | M |

#### 7.X.4.2	Generation of xCC at CC-POI in RCS server over LI_X3

The CC-POI present in the RCS server shall send xCC over LI_X3 for each MSRP Message.

Each X3 PDU shall contain the contents of the user plane packet given using the MSRP payload format.

The CC-POI present in the RCS server shall set the payload format to indicate the appropriate payload type (13 for MSRP Message as described in ETSI TS 103 221-2 [8] clauses 5.4 and 5.4.14.

#### 7.X.4.3	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the RCS server, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g., additional location information).

The timestamp field of the ETSI TS 102 232-1 [9] PSHeader structure shall be set to the time at which the RCS server event was observed (i.e., the timestamp field of the xIRI).

Tables 7.X-7 shows the IRI type (see ETSI TS 102 232-1 [9] clause 5.2.10) to be used for each record type.

Table 7.X-7: IRI type for messages

| Record type | IRI Type |
| --- | --- |
| RCSLargeMessageModeSessionEstablishment | REPORT |
| RCSLargeMessageModeUnsuccessfulProcedure | REPORT |

#### 7.X.4.4	Generation of CC over LI_HI3

When the xCC is received over LI_X3, the MDF3 shall emit the CC over LI_HI3 without undue delay.

The timestamp field of the ETSI TS 102 232-1 [9] PSHeader structure shall be set to the time that the RCS server observed the data (i.e. the timestamp field of the xCC). The LIID and CID fields shall correctly reflect the target identity and communication session to which the CC belongs.

The MDF3 shall populate the threeGPP33128DefinedCC field with a BER-encoded CCPayload structure containing an MSRP Message.

### 7.X.5	LI for 1-to-1 chat

#### 7.X.5.1	Generation of xIRI at IRI-POI in RCS server over LI_X2

##### 7.X.5.1.1	General

The IRI-POI present in the RCS server shall send xIRI over LI_X2 for each of the events listed in TS 33.127 [5] clause 7.X.6, each of which is described in the following clauses.

##### 7.X.5.1.2	1-to-1 chat session establishment

The IRI-POI present in the RCS shall generate an xIRI containing an RCS1to1ChatSessionEstablishment record when the IRI-POI detects that the 1-to-1 chat session has been established.

Accordingly, the IRI-POI in the RCS server generates the xIRI when the following events are detected:

-	When the RCS server receives a SIP 200 OK from the target in response to a SIP INVITE sent to the target with presence of +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.session" in the SIP "Contact" header.

-	When the RCS Server returns a SIP 200 OK to the target in response to a SIP INVITE received from the target with presence of +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.session" in the SIP "Contact" header.

Table 7.X-8: Payload for RCS1to1ChatSessionEstablishment record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| direction | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget". | M |
| rCSServerURI | Shall include the identity of the RCS server serving the target UE. | M |
| originatingIdentity | Shall identify the originating party. | M |
| destinationIdentity | Shall identify the destination party. | M |
| sDPOffer | Shall provide the media characteristics information elements of the 1-to-1 chat session initiator, encoded in SDP format as per RFC 4566 [XA] clause 5 when known. | C |
| sDPAnswer | Shall provide the media characteristics information elements of the 1-to-1 chat session termination, encoded in SDP format as per RFC 4566 [XA] clause 5 when known. | C |
| location | Shall include the target’s location when reporting of the target’s location information is authorized and available. | C |
| fileTransferFlag | Indicates if the session is established for a file transfer. | C |
| firstMessage | Provides the first message exchanged if included in the SIP INVITE for 1-to-1 chat session establishment. | C |

##### 7.X.5.1.3	1-to-1 chat session release

The IRI-POI present in the RCS shall generate an xIRI containing an RCS1to1ChatSessionRelease record when the IRI-POI detects that the 1-to-1 chat session has been released.

Accordingly, the IRI-POI in the RCS server generates the xIRI when the following events are detected:

-	When the RCS server receives a SIP BYE from the target related to an established 1-to-1 chat session.

-	When the RCS server sends a SIP BYE to the target related to an established 1-to-1 chat session.

Table 7.X-9: Payload for RCS1to1ChatSessionRelease record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| direction | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget". | M |
| rCSServerURI | Shall include the identity of the RCS server serving the target UE. | M |
| location | Shall include the target’s location when reporting of the target’s location information is authorized and available. | C |
| cause | Shall identify the reason for the 1-to-1 chat session release: - Release of the chat session by a one of the two participants. - Chat inactivity timeout. | M |

##### 7.X.5.1.4	1-to-1 chat session unsuccessful procedure

The IRI-POI present in the RCS server shall generate an xIRI containing a RCS1to1ChatSessionUnsuccessful record when the IRI-POI present in RCS server detects that 1-to-1 chat session is not established and the request is abandoned before the 1-to-1 chat session starts. Accordingly, the IRI-POI in the RCS server generates the xIRI when the following events are detected:

-	When the RCS server receives a SIP 4XX, 5XX or 6XX from the target in response to a SIP INVITE sent to the target with presence of +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.session" in the SIP "Contact" header.

-	when the RCS Server returns a SIP 4XX, 5XX or 6XX to the target in response to a SIP INVITE received from the target with presence of +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.session" in the SIP "Contact" header.

Table 7.X-10: Payload for RCS1to1ChatUnsuccessfulProcedure record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| direction | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget". | M |
| rCSServerURI | Shall include the identity of the RCS server serving the RCS target. | M |
| originatingIdentity | Shall identify the originating party. | M |
| destinationIdentity | Shall identify the destination party of the 1-to-1 chat session. | M |
| fileTransferFlag | Indicates if the session is established for a file transfer. | C |
| location | Shall include the target’s location when reporting of the target’s location information is authorized and available. | C |
| cause | Shall identify the reason for the unsuccessful 1-to-1 chat session establishment procedure corresponding to the SIP response code and SIP reason phrase. | M |

##### 7.X.5.1.5	Start of interception with established 1-to-1 chat session

The IRI-POI present in the RCS server shall generate an xIRI containing an RCSStartOfInterceptionforEstablished1to1ChatSession record when a target has an active 1-to-1 chat session in progress. If multiple 1-to-1 chat sessions are active at the start of interception, an RCSStartOfInterceptionforEstablished1to1ChatSession record is generated for each active 1-to-1 chat session. Accordingly, the IRI-POI in the RCS server generates the xIRI when the following event is detected:

-	When the RCS server receives a SIP 3XX, 4XX, 5XX or 6XX from the target in response to a SIP INVITE sent to the target with presence of +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.session" in the SIP "Contact" header.

Table 7.X-11: Payload for RCSStartOfInterceptionWithEstablished1to1ChatSession record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| direction | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget". | M |
| rCSServerURI | Shall include the identity of the RCS server serving the target UE. | M |
| originatingIdentity | Shall identify the originating party. | M |
| destinationIdentity | Shall identify the destination party. | M |
| sdpOffer | Shall provide the media characteristics information elements of the 1-to-1 chat session initiator, encoded in SDP format as per RFC 4566 [XA] clause 5 when known. | C |
| sdpAnswer | Shall provide the media characteristics information elements of the 1-to-1 chat session termination, encoded in SDP format as per RFC 4566 [XA] clause 5 when known. | C |
| location | Shall include the target’s location when reporting of the target’s location information is authorized and available. | C |

#### 7.X.5.2	Generation of xCC at CC-POI in RCS server over LI_X3

The CC-POI present in the RCS server shall send xCC over LI_X3 for each MSRP Message.

Each X3 PDU shall contain the contents of the user plane packet given using the MSRP payload format.

The CC-POI present in the RCS server shall set the payload format to indicate the appropriate payload type (13 for MSRP Message as described in ETSI TS 103 221-2 [8] clauses 5.4 and 5.4.14.

#### 7.X.5.3	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the RCS server, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g., additional location information).

The timestamp field of the ETSI TS 102 232-1 [9] PSHeader structure shall be set to the time at which the RCS server event was observed (i.e., the timestamp field of the xIRI).

Tables 7.X-12 shows the IRI type (see ETSI TS 102 232-1 [9] clause 5.2.10) to be used for each record type.

Table 7.X-12: IRI type for messages

| Record type | IRI Type |
| --- | --- |
| RCS1to1ChatSessionEstablishment | BEGIN |
| RCS1to1ChatSessionRelease | END |
| RCSStartOfInterceptionWithEstablished1to1ChatSession | BEGIN |
| RCS1to1ChatSessionUnsuccessfulProcedure | REPORT |

#### 7.X.5.4	Generation of CC over LI_HI3

When the xCC is received over LI_X3, the MDF3 shall emit the CC over LI_HI3 without undue delay.

The timestamp field of the ETSI TS 102 232-1 [9] PSHeader structure shall be set to the time that the RCS server observed the data (i.e. the timestamp field of the xCC). The LIID and CID fields shall correctly reflect the target identity and communication session to which the CC belongs.

The MDF3 shall populate the threeGPP33128DefinedCC field with a BER-encoded CCPayload structure containing an MSRP Message.

### 7.X.6	LI for group chat

#### 7.X.6.1	Generation of xIRI at IRI-POI in RCS server over LI_X2

##### 7.X.6.1.1	General

The IRI-POI present in the RCS server shall send xIRI over LI_X2 for each of the events listed in TS 33.127 [5] clause 7.X.7, each of which is described in the following clauses.

##### 7.X.6.1.2	Group chat session establishment

The IRI-POI present in the RCS shall generate an xIRI containing an RCSGroupChatSessionEstablishment record when the IRI-POI detects that the group chat session has been established.

Accordingly, the IRI-POI in the RCS server generates the xIRI when the following events are detected:

-	When the RCS server sends a SIP 200 OK to the target in response to a SIP INVITE from the target with presence of +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.session" in the SIP "Contact" header. The target originates the group chat session.

-	When the RCS Server receives a SIP 200 OK from the target in response to a SIP INVITE sent to the target with presence of +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.session" in the SIP "Contact" header. The target receives an invitation to join the group chat session.

Table 7.X-13: Payload for RCSGroupChatSessionEstablishment record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| direction | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget". | M |
| rCSServerURI | Shall include the identity of the RCS server serving the target UE. | M |
| groupChatSessionID | Group chat session URI. | M |
| originatingIdentity | Shall identify the originating party. | M |
| destinationIdentities | Shall identify the individual participants of the group chat session, when known. | M |
| bearerCapability | Shall provide the media characteristics information elements of the RCS session, encoded in SDP format as per RFC 4566 [XA] clause 5 when known. | C |
| location | Shall include the target’s location when reporting of the target’s location information is authorized and available. | C |
| fileTransferFlag | Indicates if the session is established for a file transfer. | C |

##### 7.X.6.1.3	Group chat session release

The IRI-POI present in the RCS shall generate an xIRI containing an RCSGroupChatSessionRelease record when the IRI-POI detects that the group chat session has been released for any reason (i.e., normal or abnormal release). Accordingly, the IRI-POI in the RCS server generates the xIRI when the following events are detected:

-	When the PTC server receives a SIP BYE from the target to end the session.

-	When the PTC server receives a SIP 200 OK from the target in response to a SIP BYE.

Table 7.X-14: Payload for RCSGroupChatSessionRelease record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| direction | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget". | M |
| rCSServerURI | Shall include the identity of the RCS server serving the target UE. | M |
| groupChatSessionID | Group chat session URI. | M |
| participants | Shall identify the individual participants of the group chat session, when known. | C |
| location | Shall include the PTC target’s location when reporting of the PTC target’s location information is authorized and available. | C |
| cause | Shall identify the reason for the 1-to-1 chat session release: - Session initiator leaves session. - Number of participants less than certain value. - Chat inactivity timeout. | M |

##### 7.X.6.1.4	Group chat session unsuccessful procedure

The IRI-POI present in the RCS server shall generate an xIRI containing a RCS1to1ChatSessionUnsuccessful record when the IRI-POI present in RCS server detects that 1-to-1 chat session is not established and the request is abandoned before the 1-to-1 chat session starts.

Accordingly, the IRI-POI in the RCS server generates the xIRI when the following events are detected:

-	When the RCS server receives a SIP 3XX, 4XX, 5XX or 6XX from the target in response to a SIP INVITE sent to the target with presence of +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.session" in the SIP "Contact" header.

-	When the RCS Server returns a SIP 3XX, 4XX, 5XX or 6XX to the target in response to a SIP INVITE received from the target with presence of +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.session" in the SIP "Contact" header.

Table 7.X-15: Payload for RCSGroupChatUnsuccessfuProcedure record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| direction | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget". | M |
| rCSServerURI | Shall include the identity of the RCS server serving the RCS target. | M |
| originatingIdentity | Shall identify the originating party. | M |
| destinationIdentities | Shall identify the destination parties of the group chat session. | M |
| location | Shall include the target’s location when reporting of the target’s location information is authorized and available. | C |
| cause | Shall identify the reason for the unsuccessful group chat session establishment procedure corresponding to the SIP response code and SIP reason phrase. | M |
| fileTransferFlag | Indicates if the session is established for a file transfer. | C |

##### 7.X.6.1.5	Start of interception with established group chat session

The IRI-POI present in the RCS server shall generate an xIRI containing an RCSStartOfInterceptionforEstablishedGroupChatSession record when a target has an active group chat session in progress. If multiple group chat sessions are active at the start of interception, an RCSStartOfInterceptionforEstablishedGroupChatSession record is generated for each active group chat session. Accordingly, the IRI-POI in the RCS server generates the xIRI when the following event is detected:

-	When the RCS server detects that LI is enabled on a target with an active group chat session.

Table 7.X-16: Payload for RCSStartOfInterceptionWithEstablishedGroupChatSession record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| direction | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget". | M |
| rCSServerURI | Shall include the identity of the RCS server serving the target UE. | M |
| groupChatSessionID | Group chat session URI. | M |
| originatingIdentity | Shall identify the originating party. | M |
| participants | Shall identify the individual participants of the group chat session, when known. | M |
| bearerCapability | Shall provide the media characteristics information elements of the PTC session, encoded in SDP format as per RFC 4566 [XA] clause 5 when known. | C |
| location | Shall include the target’s location when reporting of the target’s location information is authorized and available. | C |

##### 7.X.6.1.6	Group chat session party join

The IRI-POI present in the RCS server generates an xIRI containing an RCSGroupChatSessionPartyJoin record when a participant joins an on-going chat group session.

Accordingly, the IRI-POI in the RCS server generates the xIRI when the following event is detected:

-	When the RCS server hosting the group chat session sends a SIP 200 OK in response to a SIP INVITE indicating a participant joining the group chat session.

Table 7.X-17: Payload for RCSGroupChatSessionPartyJoin record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| direction | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget". | M |
| groupChatSessionID | Group chat session URI. | M |
| partyJoin | Shall provide the identity of the participant that joins the group chat session. | M |
| participants | Shall identify the individual participants of the group chat session, when known. | C |
| bearerCapability | Shall provide the media characteristics information elements of the PTC session, encoded in SDP format as per RFC 4566 [XA] clause 5 when known. | C |

##### 7.X.6.1.7	Group chat session party drop

The IRI-POI present in the RCS server generates an xIRI containing an RCSGroupChatSessionPartyDrop record when a participant leaves the group chat session that still remains active with other participants. Accordingly, the IRI-POI in the participating PTC server generates the xIRI when the following event is detected:

-	When the RCS server hosting a group chat session sends a SIP 200 OK in response to a SIP BYE with the group chat session remaining active with other participants.

Table 7.X-18: Payload for RCSGroupChatSessionPartyDrop record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| direction | Indicates the direction of the session relative to the target: "toTarget" or "fromTarget". | M |
| groupChatSessionID | Group chat session URI. | M |
| partyDrop | Shall provide the identity of the participant that leaves the group chat session. | M |
| participants | Shall identify the individual participants of the group chat session, when known. | C |

#### 7.X.6.2	Generation of xCC at CC-POI in RCS server over LI_X3

The CC-POI present in the RCS server shall send xCC over LI_X3 for each MSRP Message.

Each X3 PDU shall contain the contents of the user plane packet given using the MSRP payload format.

The CC-POI present in the RCS server shall set the payload format to indicate the appropriate payload type (13 for MSRP Message as described in ETSI TS 103 221-2 [8] clauses 5.4 and 5.4.14.

#### 7.X.6.3	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the RCS server, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g., additional location information).

The timestamp field of the ETSI TS 102 232-1 [9] PSHeader structure shall be set to the time at which the RCS server event was observed (i.e., the timestamp field of the xIRI).

Tables 7.X-19 shows the IRI type (see ETSI TS 102 232-1 [9] clause 5.2.10) to be used for each record type.

Table 7.X-19: IRI type for messages

| Record type | IRI Type |
| --- | --- |
| RCSGroupChatSessionEstablishment | BEGIN |
| RCSGroupChatSessionRelease | END |
| RCSStartOfInterceptionWithEstablishedGroupChatSession | BEGIN |
| RCSGroupChatSessionUnsuccessfulProcedure | REPORT |
| RCSGroupChatSessionPartyJoin | CONTINUE |
| RCSGroupChatSessionPartyDrop | CONTINUE |

#### 7.X.6.4	Generation of CC over LI_HI3

When the xCC is received over LI_X3, the MDF3 shall emit the CC over LI_HI3 without undue delay.

The timestamp field of the ETSI TS 102 232-1 [9] PSHeader structure shall be set to the time that the RCS server observed the data (i.e. the timestamp field of the xCC). The LIID and CID fields shall correctly reflect the target identity and communication session to which the CC belongs.

The MDF3 shall populate the threeGPP33128DefinedCC field with a BER-encoded CCPayload structure containing an MSRP Message.

### 7.X.7	LI for file transfer

#### 7.X.7.1	Generation of xIRI at IRI-POI in RCS server over LI_X2

##### 7.X.7.1.1	File transfer

The IRI-POI in the RCS server generates any of the xIRIs defined for pager mode standalone messaging, large message mode standalone messaging, 1-to-1 chat or group chat for file transfer. The IRI-POI detects a file transfer procedure when any of the following events occurs:

-	The RCS server receives a SIP MESSAGE from the target; the SIP MESSAGE shall have the SIP "Contact" header which includes +g.3gpp.iari-ref="urn%3Aurn-7%3A3gppapplication.ims.iari.rcs.fthttp"; the CPIM "Content-Type" header field shall be "application/vnd.gsma.rcs-ft-http+xml". In this scenario, the file transfer procedure uses pager mode standalone messaging.

-	The RCS Server sends a SIP MESSAGE to the target; the SIP MESSAGE shall have the SIP "Contact" header which includes +g.3gpp.iari-ref="urn%3Aurn-7%3A3gppapplication.ims.iari.rcs.fthttp"; the "Content-Type" header field shall be "application/vnd.gsma.rcs-ft-http+xml". In this scenario, the file transfer procedure uses pager mode standalone messaging.

-	The RCS server receives a SIP 200 OK from the target as a response to a SIP INVITE sent to the target with presence of +g.3gpp.iari-ref="urn%3Aurn-7%3A3gppapplication.ims.iari.rcs.fthttp" in the SIP "Contact" header and presence in the SDP of "a=accept-wrapped-types" attribute indicating the content-type "application/vnd.gsma.rcs-ft-http+xml". In this scenario, the file transfer procedure uses large message mode standalone messaging, 1-to-1 chat or group chat.

-	The RCS Server returns a SIP 200 OK to the target as a response to a SIP INVITE received from the target with presence of +g.3gpp.iari-ref="urn%3Aurn-7%3A3gppapplication.ims.iari.rcs.fthttp" in the SIP "Contact" header and presence in the SDP of "a=accept-wrapped-types" attribute indicating the content-type "application/vnd.gsma.rcs-ft-http+xml". In this scenario, the file transfer procedure uses large message mode standalone messaging, 1-to-1 chat or group chat

For file transfer, the fileTransferFlag parameter of the xIRIs is set to TRUE.

##### 7.X.7.1.2	File upload

The IRI-POI in the RCS server shall generate an xIRI containing an RCSFileUpload record when a file is uploaded by the target to the HTTP Content Server of the RCS Server.

Accordingly, the IRI-POI in the RCS server generates the xIRI when the following event is detected:

-	When the HTTP Content Server of the RCS server returns a HTTP 200 OK in response to a HTTP POST sent by the target with presence of Content-Disposition header which value includes ": form-data; name="File"".

Table 7.X-20: Payload for RCSFileUpload record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| contentType | Content type of uploaded file. | M |
| fileContent | Content of the uploaded file. | M |
| fileName | Original file name of the uploaded file. | M |
| fileURL | HTTP URL for the uploaded file. | M |
| fileSize | Uploaded file size in bytes. | M |
| fileValidity | Validity of the file shall be specified by providing the date the file shall be removed on the server. | C |

##### 7.X.7.1.3	File download

The IRI-POI in the RCS server shall generate an xIRI containing an RCSFileDownload record when a file is downloaded by the target from the HTTP Content Server of the RCS Server.

Accordingly, the IRI-POI in the RCS server generates the xIRI when the following event is detected:

-	When the HTTP Content Server of the RCS server returns a HTTP 200 OK in response to a HTTP GET sent by the target for file download.

Table 7.X-21: Payload for RCSFileDownload record

| Field name | Description | M/C/O |
| --- | --- | --- |
| rCSIdentities | Provide RCS target identity. At least one among IMPU, IMPI, IMEI shall be provided for rCSIdentities. | M |
| rCSUploadIdentity | IMPU of the user who has uploaded the file | C |
| contentType | Content type of the downloaded file | M |
| fileContent | File content of the downloaded file | M |
| fileName | File name of downloaded file | M |
| fileURL | HTTP URL for the downloaded file | M |
| fileSize | Downloaded file size in bytes | M |

#### 7.X.7.2	Generation of xCC at CC-POI in RCS server over LI_X3

If file transfer uses large message mode standalone messaging, 1-to-1 Chat and group chat procedures, the CC-POI present in the RCS server shall send xCC over LI_X3 for each MSRP Message.

Each X3 PDU shall contain the contents of the user plane packet given using the MSRP payload format.

The CC-POI present in the RCS server shall set the payload format to indicate the appropriate payload type (13 for MSRP Message as described in ETSI TS 103 221-2 [8] clauses 5.4 and 5.4.14.

# Second change

Annex A (normative):
ASN.1 Schema for the Internal and External Interfaces

TS33128Payloads

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) threeGPP(4) ts33128(19) r17(17) version1(1)}

DEFINITIONS IMPLICIT TAGS EXTENSIBILITY IMPLIED ::=

BEGIN

-- =============

-- Relative OIDs

-- =============

tS33128PayloadsOID          RELATIVE-OID ::= {threeGPP(4) ts33128(19) r17(17) version1(1)}

xIRIPayloadOID              RELATIVE-OID ::= {tS33128PayloadsOID xIRI(1)}

xCCPayloadOID               RELATIVE-OID ::= {tS33128PayloadsOID xCC(2)}

iRIPayloadOID               RELATIVE-OID ::= {tS33128PayloadsOID iRI(3)}

cCPayloadOID                RELATIVE-OID ::= {tS33128PayloadsOID cC(4)}

lINotificationPayloadOID    RELATIVE-OID ::= {tS33128PayloadsOID lINotification(5)}

-- ===============

-- X2 xIRI payload

-- ===============

XIRIPayload ::= SEQUENCE

{

xIRIPayloadOID      [1] RELATIVE-OID,

event               [2] XIRIEvent

}

XIRIEvent ::= CHOICE

{

-- Access and mobility related events, see clause 6.2.2

registration                                        [1] AMFRegistration,

deregistration                                      [2] AMFDeregistration,

locationUpdate                                      [3] AMFLocationUpdate,

startOfInterceptionWithRegisteredUE                 [4] AMFStartOfInterceptionWithRegisteredUE,

unsuccessfulAMProcedure                             [5] AMFUnsuccessfulProcedure,

-- PDU session-related events, see clause 6.2.3

pDUSessionEstablishment                             [6] SMFPDUSessionEstablishment,

pDUSessionModification                              [7] SMFPDUSessionModification,

pDUSessionRelease                                   [8] SMFPDUSessionRelease,

startOfInterceptionWithEstablishedPDUSession        [9] SMFStartOfInterceptionWithEstablishedPDUSession,

unsuccessfulSMProcedure                             [10] SMFUnsuccessfulProcedure,

-- Subscriber-management related events, see clause 7.2.2

servingSystemMessage                                [11] UDMServingSystemMessage,

-- SMS-related events, see clause 6.2.5, see also sMSReport ([56] below)

sMSMessage                                          [12] SMSMessage,

-- LALS-related events, see clause 7.3.3

lALSReport                                          [13] LALSReport,

-- PDHR/PDSR-related events, see clause 6.2.3.4.1

pDHeaderReport                                      [14] PDHeaderReport,

pDSummaryReport                                     [15] PDSummaryReport,

-- tag 16 is reserved because there is no equivalent mDFCellSiteReport in XIRIEvent

-- MMS-related events, see clause 7.4.2

mMSSend                                             [17] MMSSend,

mMSSendByNonLocalTarget                             [18] MMSSendByNonLocalTarget,

mMSNotification                                     [19] MMSNotification,

mMSSendToNonLocalTarget                             [20] MMSSendToNonLocalTarget,

mMSNotificationResponse                             [21] MMSNotificationResponse,

mMSRetrieval                                        [22] MMSRetrieval,

mMSDeliveryAck                                      [23] MMSDeliveryAck,

mMSForward                                          [24] MMSForward,

mMSDeleteFromRelay                                  [25] MMSDeleteFromRelay,

mMSDeliveryReport                                   [26] MMSDeliveryReport,

mMSDeliveryReportNonLocalTarget                     [27] MMSDeliveryReportNonLocalTarget,

mMSReadReport                                       [28] MMSReadReport,

mMSReadReportNonLocalTarget                         [29] MMSReadReportNonLocalTarget,

mMSCancel                                           [30] MMSCancel,

mMSMBoxStore                                        [31] MMSMBoxStore,

mMSMBoxUpload                                       [32] MMSMBoxUpload,

mMSMBoxDelete                                       [33] MMSMBoxDelete,

mMSMBoxViewRequest                                  [34] MMSMBoxViewRequest,

mMSMBoxViewResponse                                 [35] MMSMBoxViewResponse,

-- PTC-related events, see clause 7.5.2

pTCRegistration                                     [36] PTCRegistration,

pTCSessionInitiation                                [37] PTCSessionInitiation,

pTCSessionAbandon                                   [38] PTCSessionAbandon,

pTCSessionStart                                     [39] PTCSessionStart,

pTCSessionEnd                                       [40] PTCSessionEnd,

pTCStartOfInterception                              [41] PTCStartOfInterception,

pTCPreEstablishedSession                            [42] PTCPreEstablishedSession,

pTCInstantPersonalAlert                             [43] PTCInstantPersonalAlert,

pTCPartyJoin                                        [44] PTCPartyJoin,

pTCPartyDrop                                        [45] PTCPartyDrop,

pTCPartyHold                                        [46] PTCPartyHold,

pTCMediaModification                                [47] PTCMediaModification,

pTCGroupAdvertisement                               [48] PTCGroupAdvertisement,

pTCFloorControl                                     [49] PTCFloorControl,

pTCTargetPresence                                   [50] PTCTargetPresence,

pTCParticipantPresence                              [51] PTCParticipantPresence,

pTCListManagement                                   [52] PTCListManagement,

pTCAccessPolicy                                     [53] PTCAccessPolicy,

-- More Subscriber-management related events, see clause 7.2.2

subscriberRecordChangeMessage                       [54] UDMSubscriberRecordChangeMessage,

cancelLocationMessage                               [55] UDMCancelLocationMessage,

-- SMS-related events continued from choice 12

sMSReport                                           [56] SMSReport,

-- MA PDU session-related events, see clause 6.2.3.2.7

sMFMAPDUSessionEstablishment                        [57] SMFMAPDUSessionEstablishment,

sMFMAPDUSessionModification                         [58] SMFMAPDUSessionModification,

sMFMAPDUSessionRelease                              [59] SMFMAPDUSessionRelease,

startOfInterceptionWithEstablishedMAPDUSession      [60] SMFStartOfInterceptionWithEstablishedMAPDUSession,

unsuccessfulMASMProcedure                           [61] SMFMAUnsuccessfulProcedure,

-- Identifier Association events, see clauses 6.2.2.2.7 and 6.3.2.2.2

aMFIdentifierAssocation                             [62] AMFIdentifierAssocation,

mMEIdentifierAssocation                             [63] MMEIdentifierAssocation,

-- PDU to MA PDU session-related events, see clause 6.2.3.2.8

sMFPDUtoMAPDUSessionModification                    [64] SMFPDUtoMAPDUSessionModification,

-- NEF services related events, see clause 7.7.2

nEFPDUSessionEstablishment                          [65] NEFPDUSessionEstablishment,

nEFPDUSessionModification                           [66] NEFPDUSessionModification,

nEFPDUSessionRelease                                [67] NEFPDUSessionRelease,

nEFUnsuccessfulProcedure                            [68] NEFUnsuccessfulProcedure,

nEFStartOfInterceptionWithEstablishedPDUSession     [69] NEFStartOfInterceptionWithEstablishedPDUSession,

nEFdeviceTrigger                                    [70] NEFDeviceTrigger,

nEFdeviceTriggerReplace                             [71] NEFDeviceTriggerReplace,

nEFdeviceTriggerCancellation                        [72] NEFDeviceTriggerCancellation,

nEFdeviceTriggerReportNotify                        [73] NEFDeviceTriggerReportNotify,

nEFMSISDNLessMOSMS                                  [74] NEFMSISDNLessMOSMS,

nEFExpectedUEBehaviourUpdate                        [75] NEFExpectedUEBehaviourUpdate,

-- SCEF services related events, see clause 7.8.2

sCEFPDNConnectionEstablishment                      [76] SCEFPDNConnectionEstablishment,

sCEFPDNConnectionUpdate                             [77] SCEFPDNConnectionUpdate,

sCEFPDNConnectionRelease                            [78] SCEFPDNConnectionRelease,

sCEFUnsuccessfulProcedure                           [79] SCEFUnsuccessfulProcedure,

sCEFStartOfInterceptionWithEstablishedPDNConnection [80] SCEFStartOfInterceptionWithEstablishedPDNConnection,

sCEFdeviceTrigger                                   [81] SCEFDeviceTrigger,

sCEFdeviceTriggerReplace                            [82] SCEFDeviceTriggerReplace,

sCEFdeviceTriggerCancellation                       [83] SCEFDeviceTriggerCancellation,

sCEFdeviceTriggerReportNotify                       [84] SCEFDeviceTriggerReportNotify,

sCEFMSISDNLessMOSMS                                 [85] SCEFMSISDNLessMOSMS,

sCEFCommunicationPatternUpdate                      [86] SCEFCommunicationPatternUpdate,

--EPS Events, see clause 6.3

--MME Events, see clause 6.3.2.2

mMEAttach                                           [87] MMEAttach,

mMEDetach                                           [88] MMEDetach,

mMELocationUpdate                                   [89] MMELocationUpdate,

mMEStartOfInterceptionWithEPSAttachedUE             [90] MMEStartOfInterceptionWithEPSAttachedUE,

mMEUnsuccessfulProcedure                            [91] MMEUnsuccessfulProcedure,

-- RCS services related events, see clause 7.X

rCSRegistration                                     [21692] RCSRegistration,

rCSCapabilityDiscovery                              [21693] RCSCapabilityDiscovery,

rCSPagerModeMessageDelivery                         [21694] RCSPagerModeMessageDelivery,

rCSPagerModeMessageDeliveryNotification             [21695] RCSPagerModeMessageDeliveryNotification,

rCSLargeMessageModeSessionEstablishment             [21696] RCSLargeMessageModeSessionEstablishment,

rCSLargeMessageModeUnsuccessfulProcedure            [21697] RCSLargeMessageModeUnsuccessfulProcedure,

rCS1to1ChatSessionEstablishment                     [21698] RCS1to1ChatSessionEstablishment,

rCS1to1ChatSessionRelease                           [21699] RCS1to1ChatSessionRelease,

rCS1to1ChatUnsuccessfulProcedure                    [216100] RCS1to1ChatUnsuccessfulProcedure,

rCSStartOfInterceptionWithEstablished1to1ChatSession [216101] RCSStartOfInterceptionWithEstablished1to1ChatSession,

rCSGroupChatSessionEstablishment                    [216102] RCSGroupChatSessionEstablishment,

rCSGroupChatSessionRelease                          [216103] RCSGroupChatSessionRelease,

rCSGroupChatUnsuccessfulProcedure                   [216104] RCSGroupChatUnsuccessfulProcedure,

rCSStartOfInterceptionWithEstablishedGroupChatSession [216105] RCSStartOfInterceptionWithEstablishedGroupChatSession,

rCSGroupChatSessionPartyJoin                        [216106] RCSGroupChatSessionPartyJoin,

rCSGroupChatSessionPartyDrop                        [216107] RCSGroupChatSessionPartyDrop,

rCSFileUpload                                       [216108] RCSFileUpload,

rCSFileDownload                                     [216109] RCSFileDownload

}

-- ==============

-- X3 xCC payload

-- ==============

-- No additional xCC payload definitions required in the present document.

-- ===============

-- HI2 IRI payload

-- ===============

IRIPayload ::= SEQUENCE

{

iRIPayloadOID       [1] RELATIVE-OID,

event               [2] IRIEvent,

targetIdentifiers   [3] SEQUENCE OF IRITargetIdentifier OPTIONAL

}

IRIEvent ::= CHOICE

{

-- Registration-related events, see clause 6.2.2

registration                                        [1] AMFRegistration,

deregistration                                      [2] AMFDeregistration,

locationUpdate                                      [3] AMFLocationUpdate,

startOfInterceptionWithRegisteredUE                 [4] AMFStartOfInterceptionWithRegisteredUE,

unsuccessfulRegistrationProcedure                   [5] AMFUnsuccessfulProcedure,

-- PDU session-related events, see clause 6.2.3

pDUSessionEstablishment                             [6] SMFPDUSessionEstablishment,

pDUSessionModification                              [7] SMFPDUSessionModification,

pDUSessionRelease                                   [8] SMFPDUSessionRelease,

startOfInterceptionWithEstablishedPDUSession        [9] SMFStartOfInterceptionWithEstablishedPDUSession,

unsuccessfulSessionProcedure                        [10] SMFUnsuccessfulProcedure,

-- Subscriber-management related events, see clause 7.2.2

servingSystemMessage                                [11] UDMServingSystemMessage,

-- SMS-related events, see clause 6.2.5, see also sMSReport ([56] below)

sMSMessage                                          [12] SMSMessage,

-- LALS-related events, see clause 7.3.3

lALSReport                                          [13] LALSReport,

-- PDHR/PDSR-related events, see clause 6.2.3.4.1

pDHeaderReport                                      [14] PDHeaderReport,

pDSummaryReport                                     [15] PDSummaryReport,

-- MDF-related events, see clause 7.3.4

mDFCellSiteReport                                   [16] MDFCellSiteReport,

-- MMS-related events, see clause 7.4.2

mMSSend                                             [17] MMSSend,

mMSSendByNonLocalTarget                             [18] MMSSendByNonLocalTarget,

mMSNotification                                     [19] MMSNotification,

mMSSendToNonLocalTarget                             [20] MMSSendToNonLocalTarget,

mMSNotificationResponse                             [21] MMSNotificationResponse,

mMSRetrieval                                        [22] MMSRetrieval,

mMSDeliveryAck                                      [23] MMSDeliveryAck,

mMSForward                                          [24] MMSForward,

mMSDeleteFromRelay                                  [25] MMSDeleteFromRelay,

mMSDeliveryReport                                   [26] MMSDeliveryReport,

mMSDeliveryReportNonLocalTarget                     [27] MMSDeliveryReportNonLocalTarget,

mMSReadReport                                       [28] MMSReadReport,

mMSReadReportNonLocalTarget                         [29] MMSReadReportNonLocalTarget,

mMSCancel                                           [30] MMSCancel,

mMSMBoxStore                                        [31] MMSMBoxStore,

mMSMBoxUpload                                       [32] MMSMBoxUpload,

mMSMBoxDelete                                       [33] MMSMBoxDelete,

mMSMBoxViewRequest                                  [34] MMSMBoxViewRequest,

mMSMBoxViewResponse                                 [35] MMSMBoxViewResponse,

-- PTC-related events, see clause 7.5.2

pTCRegistration                                     [36] PTCRegistration,

pTCSessionInitiation                                [37] PTCSessionInitiation,

pTCSessionAbandon                                   [38] PTCSessionAbandon,

pTCSessionStart                                     [39] PTCSessionStart,

pTCSessionEnd                                       [40] PTCSessionEnd,

pTCStartOfInterception                              [41] PTCStartOfInterception,

pTCPreEstablishedSession                            [42] PTCPreEstablishedSession,

pTCInstantPersonalAlert                             [43] PTCInstantPersonalAlert,

pTCPartyJoin                                        [44] PTCPartyJoin,

pTCPartyDrop                                        [45] PTCPartyDrop,

pTCPartyHold                                        [46] PTCPartyHold,

pTCMediaModification                                [47] PTCMediaModification,

pTCGroupAdvertisement                               [48] PTCGroupAdvertisement,

pTCFloorControl                                     [49] PTCFloorControl,

pTCTargetPresence                                   [50] PTCTargetPresence,

pTCParticipantPresence                              [51] PTCParticipantPresence,

pTCListManagement                                   [52] PTCListManagement,

pTCAccessPolicy                                     [53] PTCAccessPolicy,

-- More Subscriber-management related events, see clause 7.2.2

subscriberRecordChangeMessage                      [54] UDMSubscriberRecordChangeMessage,

cancelLocationMessage                              [55] UDMCancelLocationMessage,

-- SMS-related events, continued from choice 12

sMSReport                                           [56] SMSReport,

-- MA PDU session-related events, see clause 6.2.3.2.7

sMFMAPDUSessionEstablishment                        [57] SMFMAPDUSessionEstablishment,

sMFMAPDUSessionModification                         [58] SMFMAPDUSessionModification,

sMFMAPDUSessionRelease                              [59] SMFMAPDUSessionRelease,

startOfInterceptionWithEstablishedMAPDUSession      [60] SMFStartOfInterceptionWithEstablishedMAPDUSession,

unsuccessfulMASMProcedure                           [61] SMFMAUnsuccessfulProcedure,

-- Identifier Association events, see clauses 6.2.2.2.7 and 6.3.2.2.2

aMFIdentifierAssocation                            [62] AMFIdentifierAssocation,

mMEIdentifierAssocation                            [63] MMEIdentifierAssocation,

-- PDU to MA PDU session-related events, see clause 6.2.3.2.8

sMFPDUtoMAPDUSessionModification                    [64] SMFPDUtoMAPDUSessionModification,

-- NEF services related events, see clause 7.7.2,

nEFPDUSessionEstablishment                          [65] NEFPDUSessionEstablishment,

nEFPDUSessionModification                           [66] NEFPDUSessionModification,

nEFPDUSessionRelease                                [67] NEFPDUSessionRelease,

nEFUnsuccessfulProcedure                            [68] NEFUnsuccessfulProcedure,

nEFStartOfInterceptionWithEstablishedPDUSession     [69] NEFStartOfInterceptionWithEstablishedPDUSession,

nEFdeviceTrigger                                    [70] NEFDeviceTrigger,

nEFdeviceTriggerReplace                             [71] NEFDeviceTriggerReplace,

nEFdeviceTriggerCancellation                        [72] NEFDeviceTriggerCancellation,

nEFdeviceTriggerReportNotify                        [73] NEFDeviceTriggerReportNotify,

nEFMSISDNLessMOSMS                                  [74] NEFMSISDNLessMOSMS,

nEFExpectedUEBehaviourUpdate                        [75] NEFExpectedUEBehaviourUpdate,

-- SCEF services related events, see clause 7.8.2

sCEFPDNConnectionEstablishment                      [76] SCEFPDNConnectionEstablishment,

sCEFPDNConnectionUpdate                             [77] SCEFPDNConnectionUpdate,

sCEFPDNConnectionRelease                            [78] SCEFPDNConnectionRelease,

sCEFUnsuccessfulProcedure                           [79] SCEFUnsuccessfulProcedure,

sCEFStartOfInterceptionWithEstablishedPDNConnection [80] SCEFStartOfInterceptionWithEstablishedPDNConnection,

sCEFdeviceTrigger                                   [81] SCEFDeviceTrigger,

sCEFdeviceTriggerReplace                            [82] SCEFDeviceTriggerReplace,

sCEFdeviceTriggerCancellation                       [83] SCEFDeviceTriggerCancellation,

sCEFdeviceTriggerReportNotify                       [84] SCEFDeviceTriggerReportNotify,

sCEFMSISDNLessMOSMS                                 [85] SCEFMSISDNLessMOSMS,

sCEFCommunicationPatternUpdate                      [86] SCEFCommunicationPatternUpdate,

--EPS Events, see clause 6.3

--MME Events, see clause 6.3.2.2

mMEAttach                                           [87] MMEAttach,

mMEDetach                                           [88] MMEDetach,

mMELocationUpdate                                   [89] MMELocationUpdate,

mMEStartOfInterceptionWithEPSAttachedUE             [90] MMEStartOfInterceptionWithEPSAttachedUE,

mMEUnsuccessfulProcedure                            [91] MMEUnsuccessfulProcedure,

-- RCS services related events, see clause 7.X

rCSRegistration                                     [21692] RCSRegistration,

rCSCapabilityDiscovery                              [21693] RCSCapabilityDiscovery,

rCSPagerModeMessageDelivery                         [21694] RCSPagerModeMessageDelivery,

rCSPagerModeMessageDeliveryNotification             [21695] RCSPagerModeMessageDeliveryNotification,

rCSLargeMessageModeSessionEstablishment             [21696] RCSLargeMessageModeSessionEstablishment,

rCSLargeMessageModeUnsuccessfulProcedure            [21697] RCSLargeMessageModeUnsuccessfulProcedure,

rCS1to1ChatSessionEstablishment                     [21698] RCS1to1ChatSessionEstablishment,

rCS1to1ChatSessionRelease                           [21699] RCS1to1ChatSessionRelease,

rCS1to1ChatUnsuccessfulProcedure                    [216100] RCS1to1ChatUnsuccessfulProcedure,

rCSStartOfInterceptionWithEstablished1to1ChatSession [216101] RCSStartOfInterceptionWithEstablished1to1ChatSession,

rCSGroupChatSessionEstablishment                    [216102] RCSGroupChatSessionEstablishment,

rCSGroupChatSessionRelease                          [216103] RCSGroupChatSessionRelease,

rCSGroupChatUnsuccessfulProcedure                   [216104] RCSGroupChatUnsuccessfulProcedure,

rCSStartOfInterceptionWithEstablishedGroupChatSession [216105] RCSStartOfInterceptionWithEstablishedGroupChatSession,

rCSGroupChatSessionPartyJoin                        [216106] RCSGroupChatSessionPartyJoin,

rCSGroupChatSessionPartyDrop                        [216107] RCSGroupChatSessionPartyDrop,

rCSFileUpload                                       [216108] RCSFileUpload,

rCSFileDownload                                     [216109] RCSFileDownload

}

IRITargetIdentifier ::= SEQUENCE

{

identifier                                          [1] TargetIdentifier,

provenance                                          [2] TargetIdentifierProvenance OPTIONAL

}

-- ==============

-- HI3 CC payload

-- ==============

CCPayload ::= SEQUENCE

{

cCPayloadOID         [1] RELATIVE-OID,

pDU                  [2] CCPDU

}

CCPDU ::= CHOICE

{

uPFCCPDU            [1] UPFCCPDU,

extendedUPFCCPDU    [2] ExtendedUPFCCPDU,

mMSCCPDU            [3] MMSCCPDU,

nIDDCCPDU           [4] NIDDCCPDU

}

-- ===========================

-- HI4 LI notification payload

-- ===========================

LINotificationPayload ::= SEQUENCE

{

lINotificationPayloadOID         [1] RELATIVE-OID,

notification                     [2] LINotificationMessage

}

LINotificationMessage ::= CHOICE

{

lINotification      [1] LINotification

}

-- ==================

-- 5G NEF definitions

-- ==================

-- See clause 7.7.2.1.2 for details of this structure

NEFPDUSessionEstablishment ::= SEQUENCE

{

sUPI                  [1] SUPI,

gPSI                  [2] GPSI,

pDUSessionID          [3] PDUSessionID,

sNSSAI                [4] SNSSAI,

nEFID                 [5] NEFID,

dNN                   [6] DNN,

rDSSupport            [7] RDSSupport,

sMFID                 [8] SMFID,

aFID                  [9] AFID

}

-- See clause 7.7.2.1.3 for details of this structure

NEFPDUSessionModification ::= SEQUENCE

{

sUPI                         [1] SUPI,

gPSI                         [2] GPSI,

sNSSAI                       [3] SNSSAI,

initiator                    [4] Initiator,

rDSSourcePortNumber          [5] RDSPortNumber OPTIONAL,

rDSDestinationPortNumber     [6] RDSPortNumber OPTIONAL,

applicationID                [7] ApplicationID OPTIONAL,

aFID                         [8] AFID OPTIONAL,

rDSAction                    [9] RDSAction OPTIONAL,

serializationFormat          [10] SerializationFormat OPTIONAL

}

-- See clause 7.7.2.1.4 for details of this structure

NEFPDUSessionRelease ::= SEQUENCE

{

sUPI                   [1] SUPI,

gPSI                   [2] GPSI,

pDUSessionID           [3] PDUSessionID,

timeOfFirstPacket      [4] Timestamp OPTIONAL,

timeOfLastPacket       [5] Timestamp OPTIONAL,

uplinkVolume           [6] INTEGER OPTIONAL,

downlinkVolume         [7] INTEGER OPTIONAL,

releaseCause           [8] NEFReleaseCause

}

-- See clause 7.7.2.1.5 for details of this structure

NEFUnsuccessfulProcedure ::= SEQUENCE

{

failureCause                 [1] NEFFailureCause,

sUPI                         [2] SUPI,

gPSI                         [3] GPSI OPTIONAL,

pDUSessionID                 [4] PDUSessionID,

dNN                          [5] DNN OPTIONAL,

sNSSAI                       [6] SNSSAI OPTIONAL,

rDSDestinationPortNumber     [7] RDSPortNumber,

applicationID                [8] ApplicationID,

aFID                         [9] AFID

}

-- See clause 7.7.2.1.6 for details of this structure

NEFStartOfInterceptionWithEstablishedPDUSession ::= SEQUENCE

{

sUPI               [1] SUPI,

gPSI               [2] GPSI,

pDUSessionID       [3] PDUSessionID,

dNN                [4] DNN,

sNSSAI             [5] SNSSAI,

nEFID              [6] NEFID,

rDSSupport         [7] RDSSupport,

sMFID              [8] SMFID,

aFID               [9] AFID

}

-- See clause 7.7.3.1.1 for details of this structure

NEFDeviceTrigger ::= SEQUENCE

{

sUPI                  [1] SUPI,

gPSI                  [2] GPSI,

triggerId             [3] TriggerID,

aFID                  [4] AFID,

triggerPayload        [5] TriggerPayload OPTIONAL,

validityPeriod        [6] INTEGER OPTIONAL,

priorityDT            [7] PriorityDT OPTIONAL,

sourcePortId          [8] PortNumber OPTIONAL,

destinationPortId     [9] PortNumber OPTIONAL

}

-- See clause 7.7.3.1.2 for details of this structure

NEFDeviceTriggerReplace ::= SEQUENCE

{

sUPI                     [1] SUPI,

gPSI                     [2] GPSI,

triggerId                [3] TriggerID,

aFID                     [4] AFID,

triggerPayload           [5] TriggerPayload OPTIONAL,

validityPeriod           [6] INTEGER OPTIONAL,

priorityDT               [7] PriorityDT OPTIONAL,

sourcePortId             [8] PortNumber OPTIONAL,

destinationPortId        [9] PortNumber OPTIONAL

}

-- See clause 7.7.3.1.3 for details of this structure

NEFDeviceTriggerCancellation ::= SEQUENCE

{

sUPI                  [1] SUPI,

gPSI                  [2] GPSI,

triggerId             [3] TriggerID

}

-- See clause 7.7.3.1.4 for details of this structure

NEFDeviceTriggerReportNotify ::= SEQUENCE

{

sUPI                             [1] SUPI,

gPSI                             [2] GPSI,

triggerId                        [3] TriggerID,

deviceTriggerDeliveryResult      [4] DeviceTriggerDeliveryResult

}

-- See clause 7.7.4.1.1 for details of this structure

NEFMSISDNLessMOSMS ::= SEQUENCE

{

sUPI                      [1] SUPI,

gPSI                      [2] GPSI,

terminatingSMSParty       [3] AFID,

sMS                       [4] SMSTPDUData OPTIONAL,

sourcePort                [5] PortNumber OPTIONAL,

destinationPort           [6] PortNumber OPTIONAL

}

-- See clause 7.7.5.1.1 for details of this structure

NEFExpectedUEBehaviourUpdate ::= SEQUENCE

{

gPSI                                  [1] GPSI,

expectedUEMovingTrajectory            [2] SEQUENCE OF UMTLocationArea5G OPTIONAL,

stationaryIndication                  [3] StationaryIndication OPTIONAL,

communicationDurationTime             [4] INTEGER OPTIONAL,

periodicTime                          [5] INTEGER OPTIONAL,

scheduledCommunicationTime            [6] ScheduledCommunicationTime OPTIONAL,

scheduledCommunicationType            [7] ScheduledCommunicationType OPTIONAL,

batteryIndication                     [8] BatteryIndication OPTIONAL,

trafficProfile                        [9] TrafficProfile OPTIONAL,

expectedTimeAndDayOfWeekInTrajectory  [10] SEQUENCE OF UMTLocationArea5G OPTIONAL,

aFID                                  [11] AFID,

validityTime                          [12] Timestamp OPTIONAL

}

-- ==========================

-- Common SCEF/NEF parameters

-- ==========================

RDSSupport ::= BOOLEAN

RDSPortNumber ::= INTEGER (0..15)

RDSAction ::= ENUMERATED

{

reservePort(1),

releasePort(2)

}

SerializationFormat ::= ENUMERATED

{

xml(1),

json(2),

cbor(3)

}

ApplicationID ::= OCTET STRING

NIDDCCPDU ::= OCTET STRING

TriggerID ::= UTF8String

PriorityDT ::= ENUMERATED

{

noPriority(1),

priority(2)

}

TriggerPayload ::= OCTET STRING

DeviceTriggerDeliveryResult ::= ENUMERATED

{

success(1),

unknown(2),

failure(3),

triggered(4),

expired(5),

unconfirmed(6),

replaced(7),

terminate(8)

}

StationaryIndication ::= ENUMERATED

{

stationary(1),

mobile(2)

}

BatteryIndication ::= ENUMERATED

{

batteryRecharge(1),

batteryReplace(2),

batteryNoRecharge(3),

batteryNoReplace(4),

noBattery(5)

}

ScheduledCommunicationTime ::= SEQUENCE

{

days [1] SEQUENCE OF Daytime

}

UMTLocationArea5G ::= SEQUENCE

{

timeOfDay        [1] Daytime,

durationSec      [2] INTEGER,

location         [3] NRLocation

}

Daytime ::= SEQUENCE

{

daysOfWeek       [1] Day OPTIONAL,

timeOfDayStart   [2] Timestamp OPTIONAL,

timeOfDayEnd     [3] Timestamp OPTIONAL

}

Day ::= ENUMERATED

{

monday(1),

tuesday(2),

wednesday(3),

thursday(4),

friday(5),

saturday(6),

sunday(7)

}

TrafficProfile ::= ENUMERATED

{

singleTransUL(1),

singleTransDL(2),

dualTransULFirst(3),

dualTransDLFirst(4),

multiTrans(5)

}

ScheduledCommunicationType ::= ENUMERATED

{

downlinkOnly(1),

uplinkOnly(2),

bidirectional(3)

}

-- =================

-- 5G NEF parameters

-- =================

NEFFailureCause ::= ENUMERATED

{

userUnknown(1),

niddConfigurationNotAvailable(2),

contextNotFound(3),

portNotFree(4),

portNotAssociatedWithSpecifiedApplication(5)

}

NEFReleaseCause ::= ENUMERATED

{

sMFRelease(1),

dNRelease(2),

uDMRelease(3),

cHFRelease(4),

localConfigurationPolicy(5),

unknownCause(6)

}

AFID ::= UTF8String

NEFID ::= UTF8String

-- ==================

-- SCEF definitions

-- ==================

-- See clause 7.8.2.1.2 for details of this structure

SCEFPDNConnectionEstablishment ::= SEQUENCE

{

iMSI                  [1] IMSI OPTIONAL,

mSISDN                [2] MSISDN OPTIONAL,

externalIdentifier    [3] NAI OPTIONAL,

iMEI                  [4] IMEI OPTIONAL,

ePSBearerID           [5] EPSBearerID,

sCEFID                [6] SCEFID,

aPN                   [7] APN,

rDSSupport            [8] RDSSupport,

sCSASID               [9] SCSASID

}

-- See clause 7.8.2.1.3 for details of this structure

SCEFPDNConnectionUpdate ::= SEQUENCE

{

iMSI                         [1] IMSI OPTIONAL,

mSISDN                       [2] MSISDN OPTIONAL,

externalIdentifier           [3] NAI OPTIONAL,

initiator                    [4] Initiator,

rDSSourcePortNumber          [5] RDSPortNumber OPTIONAL,

rDSDestinationPortNumber     [6] RDSPortNumber OPTIONAL,

applicationID                [7] ApplicationID OPTIONAL,

sCSASID                      [8] SCSASID OPTIONAL,

rDSAction                    [9] RDSAction OPTIONAL,

serializationFormat          [10] SerializationFormat OPTIONAL

}

-- See clause 7.8.2.1.4 for details of this structure

SCEFPDNConnectionRelease ::= SEQUENCE

{

iMSI                   [1] IMSI OPTIONAL,

mSISDN                 [2] MSISDN OPTIONAL,

externalIdentifier     [3] NAI OPTIONAL,

ePSBearerID            [4] EPSBearerID,

timeOfFirstPacket      [5] Timestamp OPTIONAL,

timeOfLastPacket       [6] Timestamp OPTIONAL,

uplinkVolume           [7] INTEGER OPTIONAL,

downlinkVolume         [8] INTEGER OPTIONAL,

releaseCause           [9] SCEFReleaseCause

}

-- See clause 7.8.2.1.5 for details of this structure

SCEFUnsuccessfulProcedure ::= SEQUENCE

{

failureCause                 [1] SCEFFailureCause,

iMSI                         [2] IMSI OPTIONAL,

mSISDN                       [3] MSISDN OPTIONAL,

externalIdentifier           [4] NAI OPTIONAL,

ePSBearerID                  [5] EPSBearerID,

aPN                          [6] APN,

rDSDestinationPortNumber     [7] RDSPortNumber OPTIONAL,

applicationID                [8] ApplicationID OPTIONAL,

sCSASID                      [9] SCSASID

}

-- See clause 7.8.2.1.6 for details of this structure

SCEFStartOfInterceptionWithEstablishedPDNConnection ::= SEQUENCE

{

iMSI                  [1] IMSI OPTIONAL,

mSISDN                [2] MSISDN OPTIONAL,

externalIdentifier    [3] NAI OPTIONAL,

iMEI                  [4] IMEI OPTIONAL,

ePSBearerID           [5] EPSBearerID,

sCEFID                [6] SCEFID,

aPN                   [7] APN,

rDSSupport            [8] RDSSupport,

sCSASID               [9] SCSASID

}

-- See clause 7.8.3.1.1 for details of this structure

SCEFDeviceTrigger ::= SEQUENCE

{

iMSI                  [1] IMSI,

mSISDN                [2] MSISDN,

externalIdentifier    [3] NAI,

triggerId             [4] TriggerID,

sCSASID               [5] SCSASID OPTIONAL,

triggerPayload        [6] TriggerPayload OPTIONAL,

validityPeriod        [7] INTEGER OPTIONAL,

priorityDT            [8] PriorityDT OPTIONAL,

sourcePortId          [9] PortNumber OPTIONAL,

destinationPortId     [10] PortNumber OPTIONAL

}

-- See clause 7.8.3.1.2 for details of this structure

SCEFDeviceTriggerReplace ::= SEQUENCE

{

iMSI                     [1] IMSI OPTIONAL,

mSISDN                   [2] MSISDN OPTIONAL,

externalIdentifier       [3] NAI OPTIONAL,

triggerId                [4] TriggerID,

sCSASID                  [5] SCSASID OPTIONAL,

triggerPayload           [6] TriggerPayload OPTIONAL,

validityPeriod           [7] INTEGER OPTIONAL,

priorityDT               [8] PriorityDT OPTIONAL,

sourcePortId             [9] PortNumber OPTIONAL,

destinationPortId        [10] PortNumber OPTIONAL

}

-- See clause 7.8.3.1.3 for details of this structure

SCEFDeviceTriggerCancellation ::= SEQUENCE

{

iMSI                     [1] IMSI OPTIONAL,

mSISDN                   [2] MSISDN OPTIONAL,

externalIdentifier       [3] NAI OPTIONAL,

triggerId                [4] TriggerID

}

-- See clause 7.8.3.1.4 for details of this structure

SCEFDeviceTriggerReportNotify ::= SEQUENCE

{

iMSI                             [1] IMSI OPTIONAL,

mSISDN                           [2] MSISDN OPTIONAL,

externalIdentifier               [3] NAI OPTIONAL,

triggerId                        [4] TriggerID,

deviceTriggerDeliveryResult      [5] DeviceTriggerDeliveryResult

}

-- See clause 7.8.4.1.1 for details of this structure

SCEFMSISDNLessMOSMS ::= SEQUENCE

{

iMSI                      [1] IMSI OPTIONAL,

mSISDN                    [2] MSISDN OPTIONAL,

externalIdentifie         [3] NAI OPTIONAL,

terminatingSMSParty       [4] SCSASID,

sMS                       [5] SMSTPDUData OPTIONAL,

sourcePort                [6] PortNumber OPTIONAL,

destinationPort           [7] PortNumber OPTIONAL

}

-- See clause 7.8.5.1.1 for details of this structure

SCEFCommunicationPatternUpdate ::= SEQUENCE

{

mSISDN                                [1] MSISDN OPTIONAL,

externalIdentifier                    [2] NAI OPTIONAL,

periodicCommunicationIndicator        [3] PeriodicCommunicationIndicator OPTIONAL,

communicationDurationTime             [4] INTEGER OPTIONAL,

periodicTime                          [5] INTEGER OPTIONAL,

scheduledCommunicationTime            [6] ScheduledCommunicationTime OPTIONAL,

scheduledCommunicationType            [7] ScheduledCommunicationType OPTIONAL,

stationaryIndication                  [8] StationaryIndication OPTIONAL,

batteryIndication                     [9] BatteryIndication OPTIONAL,

trafficProfile                        [10] TrafficProfile OPTIONAL,

expectedUEMovingTrajectory            [11] SEQUENCE OF UMTLocationArea5G OPTIONAL,

sCSASID                               [13] SCSASID,

validityTime                          [14] Timestamp OPTIONAL

}

-- =================

-- SCEF parameters

-- =================

SCEFFailureCause ::= ENUMERATED

{

userUnknown(1),

niddConfigurationNotAvailable(2),

invalidEPSBearer(3),

operationNotAllowed(4),

portNotFree(5),

portNotAssociatedWithSpecifiedApplication(6)

}

SCEFReleaseCause ::= ENUMERATED

{

mMERelease(1),

dNRelease(2),

hSSRelease(3),

localConfigurationPolicy(4),

unknownCause(5)

}

SCSASID ::= UTF8String

SCEFID ::= UTF8String

PeriodicCommunicationIndicator ::= ENUMERATED

{

periodic(1),

nonPeriodic(2)

}

EPSBearerID ::= INTEGER (0..255)

APN ::= UTF8String

-- ==================

-- 5G AMF definitions

-- ==================

-- See clause 6.2.2.2.2 for details of this structure

AMFRegistration ::= SEQUENCE

{

registrationType            [1] AMFRegistrationType,

registrationResult          [2] AMFRegistrationResult,

slice                       [3] Slice OPTIONAL,

sUPI                        [4] SUPI,

sUCI                        [5] SUCI OPTIONAL,

pEI                         [6] PEI OPTIONAL,

gPSI                        [7] GPSI OPTIONAL,

gUTI                        [8] FiveGGUTI,

location                    [9] Location OPTIONAL,

non3GPPAccessEndpoint       [10] UEEndpointAddress OPTIONAL,

fiveGSTAIList               [11] TAIList OPTIONAL,

sMSOverNasIndicator         [12] SMSOverNASIndicator OPTIONAL,

oldGUTI                     [13] EPS5GGUTI OPTIONAL,

eMM5GRegStatus              [14] EMM5GMMStatus OPTIONAL

}

-- See clause 6.2.2.2.3 for details of this structure

AMFDeregistration ::= SEQUENCE

{

deregistrationDirection     [1] AMFDirection,

accessType                  [2] AccessType,

sUPI                        [3] SUPI OPTIONAL,

sUCI                        [4] SUCI OPTIONAL,

pEI                         [5] PEI OPTIONAL,

gPSI                        [6] GPSI OPTIONAL,

gUTI                        [7] FiveGGUTI OPTIONAL,

cause                       [8] FiveGMMCause OPTIONAL,

location                    [9] Location OPTIONAL,

switchOffIndicator          [10] SwitchOffIndicator OPTIONAL,

reRegRequiredIndicator      [11] ReRegRequiredIndicator OPTIONAL

}

-- See clause 6.2.2.2.4 for details of this structure

AMFLocationUpdate ::= SEQUENCE

{

sUPI                        [1] SUPI,

sUCI                        [2] SUCI OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

gUTI                        [5] FiveGGUTI OPTIONAL,

location                    [6] Location,

sMSOverNASIndicator         [7] SMSOverNASIndicator OPTIONAL,

oldGUTI                     [8] EPS5GGUTI OPTIONAL

}

-- See clause 6.2.2.2.5 for details of this structure

AMFStartOfInterceptionWithRegisteredUE ::= SEQUENCE

{

registrationResult          [1] AMFRegistrationResult,

registrationType            [2] AMFRegistrationType OPTIONAL,

slice                       [3] Slice OPTIONAL,

sUPI                        [4] SUPI,

sUCI                        [5] SUCI OPTIONAL,

pEI                         [6] PEI OPTIONAL,

gPSI                        [7] GPSI OPTIONAL,

gUTI                        [8] FiveGGUTI,

location                    [9] Location OPTIONAL,

non3GPPAccessEndpoint       [10] UEEndpointAddress OPTIONAL,

timeOfRegistration          [11] Timestamp OPTIONAL,

fiveGSTAIList               [12] TAIList OPTIONAL,

sMSOverNASIndicator         [13] SMSOverNASIndicator OPTIONAL,

oldGUTI                     [14] EPS5GGUTI OPTIONAL,

eMM5GRegStatus              [15] EMM5GMMStatus OPTIONAL

}

-- See clause 6.2.2.2.6 for details of this structure

AMFUnsuccessfulProcedure ::= SEQUENCE

{

failedProcedureType         [1] AMFFailedProcedureType,

failureCause                [2] AMFFailureCause,

requestedSlice              [3] NSSAI OPTIONAL,

sUPI                        [4] SUPI OPTIONAL,

sUCI                        [5] SUCI OPTIONAL,

pEI                         [6] PEI OPTIONAL,

gPSI                        [7] GPSI OPTIONAL,

gUTI                        [8] FiveGGUTI OPTIONAL,

location                    [9] Location OPTIONAL

}

-- =================

-- 5G AMF parameters

-- =================

AMFID ::= SEQUENCE

{

aMFRegionID [1] AMFRegionID,

aMFSetID    [2] AMFSetID,

aMFPointer  [3] AMFPointer

}

AMFDirection ::= ENUMERATED

{

networkInitiated(1),

uEInitiated(2)

}

AMFFailedProcedureType ::= ENUMERATED

{

registration(1),

sMS(2),

pDUSessionEstablishment(3)

}

AMFFailureCause ::= CHOICE

{

fiveGMMCause        [1] FiveGMMCause,

fiveGSMCause        [2] FiveGSMCause

}

AMFPointer ::= INTEGER (0..63)

AMFRegistrationResult ::= ENUMERATED

{

threeGPPAccess(1),

nonThreeGPPAccess(2),

threeGPPAndNonThreeGPPAccess(3)

}

AMFRegionID ::= INTEGER (0..255)

AMFRegistrationType ::= ENUMERATED

{

initial(1),

mobility(2),

periodic(3),

emergency(4)

}

AMFSetID ::= INTEGER (0..1023)

-- ==================

-- 5G SMF definitions

-- ==================

-- See clause 6.2.3.2.2 for details of this structure

SMFPDUSessionEstablishment ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

pDUSessionID                [5] PDUSessionID,

gTPTunnelID                 [6] FTEID,

pDUSessionType              [7] PDUSessionType,

sNSSAI                      [8] SNSSAI OPTIONAL,

uEEndpoint                  [9] SEQUENCE OF UEEndpointAddress OPTIONAL,

non3GPPAccessEndpoint       [10] UEEndpointAddress OPTIONAL,

location                    [11] Location OPTIONAL,

dNN                         [12] DNN,

aMFID                       [13] AMFID OPTIONAL,

hSMFURI                     [14] HSMFURI OPTIONAL,

requestType                 [15] FiveGSMRequestType,

accessType                  [16] AccessType OPTIONAL,

rATType                     [17] RATType OPTIONAL,

sMPDUDNRequest              [18] SMPDUDNRequest OPTIONAL,

uEEPSPDNConnection          [19] UEEPSPDNConnection OPTIONAL

}

-- See clause 6.2.3.2.3 for details of this structure

SMFPDUSessionModification ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

sNSSAI                      [5] SNSSAI OPTIONAL,

non3GPPAccessEndpoint       [6] UEEndpointAddress OPTIONAL,

location                    [7] Location OPTIONAL,

requestType                 [8] FiveGSMRequestType,

accessType                  [9] AccessType OPTIONAL,

rATType                     [10] RATType OPTIONAL,

pDUSessionID                [11] PDUSessionID OPTIONAL

}

-- See clause 6.2.3.2.4 for details of this structure

SMFPDUSessionRelease ::= SEQUENCE

{

sUPI                        [1] SUPI,

pEI                         [2] PEI OPTIONAL,

gPSI                        [3] GPSI OPTIONAL,

pDUSessionID                [4] PDUSessionID,

timeOfFirstPacket           [5] Timestamp OPTIONAL,

timeOfLastPacket            [6] Timestamp OPTIONAL,

uplinkVolume                [7] INTEGER OPTIONAL,

downlinkVolume              [8] INTEGER OPTIONAL,

location                    [9] Location OPTIONAL,

cause                       [10] SMFErrorCodes OPTIONAL

}

-- See clause 6.2.3.2.5 for details of this structure

SMFStartOfInterceptionWithEstablishedPDUSession ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

pDUSessionID                [5] PDUSessionID,

gTPTunnelID                 [6] FTEID,

pDUSessionType              [7] PDUSessionType,

sNSSAI                      [8] SNSSAI OPTIONAL,

uEEndpoint                  [9] SEQUENCE OF UEEndpointAddress,

non3GPPAccessEndpoint       [10] UEEndpointAddress OPTIONAL,

location                    [11] Location OPTIONAL,

dNN                         [12] DNN,

aMFID                       [13] AMFID OPTIONAL,

hSMFURI                     [14] HSMFURI OPTIONAL,

requestType                 [15] FiveGSMRequestType,

accessType                  [16] AccessType OPTIONAL,

rATType                     [17] RATType OPTIONAL,

sMPDUDNRequest              [18] SMPDUDNRequest OPTIONAL,

timeOfSessionEstablishment  [19] Timestamp OPTIONAL

}

-- See clause 6.2.3.2.6 for details of this structure

SMFUnsuccessfulProcedure ::= SEQUENCE

{

failedProcedureType         [1] SMFFailedProcedureType,

failureCause                [2] FiveGSMCause,

initiator                   [3] Initiator,

requestedSlice              [4] NSSAI OPTIONAL,

sUPI                        [5] SUPI OPTIONAL,

sUPIUnauthenticated         [6] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [7] PEI OPTIONAL,

gPSI                        [8] GPSI OPTIONAL,

pDUSessionID                [9] PDUSessionID OPTIONAL,

uEEndpoint                  [10] SEQUENCE OF UEEndpointAddress OPTIONAL,

non3GPPAccessEndpoint       [11] UEEndpointAddress OPTIONAL,

dNN                         [12] DNN OPTIONAL,

aMFID                       [13] AMFID OPTIONAL,

hSMFURI                     [14] HSMFURI OPTIONAL,

requestType                 [15] FiveGSMRequestType OPTIONAL,

accessType                  [16] AccessType OPTIONAL,

rATType                     [17] RATType OPTIONAL,

sMPDUDNRequest              [18] SMPDUDNRequest OPTIONAL,

location                    [19] Location OPTIONAL

}

-- See clause 6.2.3.2.8 for details of this structure

SMFPDUtoMAPDUSessionModification ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

sNSSAI                      [5] SNSSAI OPTIONAL,

non3GPPAccessEndpoint       [6] UEEndpointAddress OPTIONAL,

location                    [7] Location OPTIONAL,

requestType                 [8] FiveGSMRequestType,

accessType                  [9] AccessType OPTIONAL,

rATType                     [10] RATType OPTIONAL,

pDUSessionID                [11] PDUSessionID,

requestIndication           [12] RequestIndication,

aTSSSContainer              [13] ATSSSContainer

}

-- See clause 6.2.3.2.7.1 for details of this structure

SMFMAPDUSessionEstablishment ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

pDUSessionID                [5] PDUSessionID,

pDUSessionType              [6] PDUSessionType,

accessInfo                  [7] SEQUENCE OF AccessInfo,

sNSSAI                      [8] SNSSAI OPTIONAL,

uEEndpoint                  [9] SEQUENCE OF UEEndpointAddress OPTIONAL,

location                    [10] Location OPTIONAL,

dNN                         [11] DNN,

aMFID                       [12] AMFID OPTIONAL,

hSMFURI                     [13] HSMFURI OPTIONAL,

requestType                 [14] FiveGSMRequestType,

sMPDUDNRequest              [15] SMPDUDNRequest OPTIONAL,

servingNetwork              [16] SMFServingNetwork,

oldPDUSessionID             [17] PDUSessionID OPTIONAL,

mAUpgradeIndication         [18] SMFMAUpgradeIndication OPTIONAL,

ePSPDNCnxInfo               [19] SMFEPSPDNCnxInfo OPTIONAL,

mAAcceptedIndication        [20] SMFMAAcceptedIndication,

aTSSSContainer              [21] ATSSSContainer OPTIONAL

}

-- See clause 6.2.3.2.7.2 for details of this structure

SMFMAPDUSessionModification ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

pDUSessionID                [5] PDUSessionID,

accessInfo                  [6] SEQUENCE OF AccessInfo OPTIONAL,

sNSSAI                      [7] SNSSAI OPTIONAL,

location                    [8] Location OPTIONAL,

requestType                 [9] FiveGSMRequestType OPTIONAL,

servingNetwork              [10] SMFServingNetwork,

oldPDUSessionID             [11] PDUSessionID OPTIONAL,

mAUpgradeIndication         [12] SMFMAUpgradeIndication OPTIONAL,

ePSPDNCnxInfo               [13] SMFEPSPDNCnxInfo OPTIONAL,

mAAcceptedIndication        [14] SMFMAAcceptedIndication,

aTSSSContainer              [15] ATSSSContainer OPTIONAL

}

-- See clause 6.2.3.2.7.3 for details of this structure

SMFMAPDUSessionRelease ::= SEQUENCE

{

sUPI                        [1] SUPI,

pEI                         [2] PEI OPTIONAL,

gPSI                        [3] GPSI OPTIONAL,

pDUSessionID                [4] PDUSessionID,

timeOfFirstPacket           [5] Timestamp OPTIONAL,

timeOfLastPacket            [6] Timestamp OPTIONAL,

uplinkVolume                [7] INTEGER OPTIONAL,

downlinkVolume              [8] INTEGER OPTIONAL,

location                    [9] Location OPTIONAL,

cause                       [10] SMFErrorCodes OPTIONAL

}

-- See clause 6.2.3.2.7.4 for details of this structure

SMFStartOfInterceptionWithEstablishedMAPDUSession ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

pDUSessionID                [5] PDUSessionID,

pDUSessionType              [6] PDUSessionType,

accessInfo                  [7] SEQUENCE OF AccessInfo,

sNSSAI                      [8] SNSSAI OPTIONAL,

uEEndpoint                  [9] SEQUENCE OF UEEndpointAddress OPTIONAL,

location                    [10] Location OPTIONAL,

dNN                         [11] DNN,

aMFID                       [12] AMFID OPTIONAL,

hSMFURI                     [13] HSMFURI OPTIONAL,

requestType                 [14] FiveGSMRequestType OPTIONAL,

sMPDUDNRequest              [15] SMPDUDNRequest OPTIONAL,

servingNetwork              [16] SMFServingNetwork,

oldPDUSessionID             [17] PDUSessionID OPTIONAL,

mAUpgradeIndication         [18] SMFMAUpgradeIndication OPTIONAL,

ePSPDNCnxInfo               [19] SMFEPSPDNCnxInfo OPTIONAL,

mAAcceptedIndication        [20] SMFMAAcceptedIndication,

aTSSSContainer              [21] ATSSSContainer OPTIONAL

}

-- See clause 6.2.3.2.7.5 for details of this structure

SMFMAUnsuccessfulProcedure ::= SEQUENCE

{

failedProcedureType         [1] SMFFailedProcedureType,

failureCause                [2] FiveGSMCause,

requestedSlice              [3] NSSAI OPTIONAL,

initiator                   [4] Initiator,

sUPI                        [5] SUPI OPTIONAL,

sUPIUnauthenticated         [6] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [7] PEI OPTIONAL,

gPSI                        [8] GPSI OPTIONAL,

pDUSessionID                [9] PDUSessionID OPTIONAL,

accessInfo                  [10] SEQUENCE OF AccessInfo,

uEEndpoint                  [11] SEQUENCE OF UEEndpointAddress OPTIONAL,

location                    [12] Location OPTIONAL,

dNN                         [13] DNN OPTIONAL,

aMFID                       [14] AMFID OPTIONAL,

hSMFURI                     [15] HSMFURI OPTIONAL,

requestType                 [16] FiveGSMRequestType OPTIONAL,

sMPDUDNRequest              [17] SMPDUDNRequest OPTIONAL

}

-- =================

-- 5G SMF parameters

-- =================

SMFID ::= UTF8String

SMFFailedProcedureType ::= ENUMERATED

{

pDUSessionEstablishment(1),

pDUSessionModification(2),

pDUSessionRelease(3)

}

SMFServingNetwork ::= SEQUENCE

{

pLMNID  [1] PLMNID,

nID     [2] NID OPTIONAL

}

AccessInfo ::= SEQUENCE

{

accessType            [1] AccessType,

rATType               [2] RATType OPTIONAL,

gTPTunnelID           [3] FTEID,

non3GPPAccessEndpoint [4] UEEndpointAddress OPTIONAL,

establishmentStatus   [5] EstablishmentStatus,

aNTypeToReactivate    [6] AccessType OPTIONAL

}

-- see Clause 6.1.2 of TS 24.193[44] for the details of the ATSSS container contents.

ATSSSContainer ::= OCTET STRING

EstablishmentStatus ::= ENUMERATED

{

established(0),

released(1)

}

SMFMAUpgradeIndication ::= BOOLEAN

-- Given in YAML encoding as defined in clause 6.1.6.2.31 of TS 29.502[16]

SMFEPSPDNCnxInfo ::= UTF8String

SMFMAAcceptedIndication ::= BOOLEAN

-- see Clause 6.1.6.3.8 of TS 29.502[16] for the details of this structure.

SMFErrorCodes ::= UTF8String

-- see Clause 6.1.6.3.2 of TS 29.502[16] for details of this structure.

UEEPSPDNConnection ::= OCTET STRING

-- see Clause 6.1.6.3.6 of TS 29.502[16] for the details of this structure.

RequestIndication ::= ENUMERATED

{

uEREQPDUSESMOD(0),

uEREQPDUSESREL(1),

pDUSESMOB(2),

nWREQPDUSESAUTH(3),

nWREQPDUSESMOD(4),

nWREQPDUSESREL(5),

eBIASSIGNMENTREQ(6),

rELDUETO5GANREQUEST(7)

}

-- ==================

-- 5G UPF definitions

-- ==================

UPFCCPDU ::= OCTET STRING

-- See clause 6.2.3.8 for the details of this structure

ExtendedUPFCCPDU ::= SEQUENCE

{

payload [1] UPFCCPDUPayload,

qFI     [2] QFI OPTIONAL

}

-- =================

-- 5G UPF parameters

-- =================

UPFCCPDUPayload ::= CHOICE

{

uPFIPCC           [1] OCTET STRING,

uPFEthernetCC     [2] OCTET STRING,

uPFUnstructuredCC [3] OCTET STRING

}

QFI ::= INTEGER (0..63)

-- ==================

-- 5G UDM definitions

-- ==================

UDMServingSystemMessage ::= SEQUENCE

{

sUPI                        [1] SUPI,

pEI                         [2] PEI OPTIONAL,

gPSI                        [3] GPSI OPTIONAL,

gUAMI                       [4] GUAMI OPTIONAL,

gUMMEI                      [5] GUMMEI OPTIONAL,

pLMNID                      [6] PLMNID OPTIONAL,

servingSystemMethod         [7] UDMServingSystemMethod,

serviceID                   [8] ServiceID OPTIONAL

}

UDMSubscriberRecordChangeMessage ::= SEQUENCE

{

sUPI                           [1] SUPI OPTIONAL,

pEI                            [2] PEI OPTIONAL,

gPSI                           [3] GPSI OPTIONAL,

oldPEI                         [4] PEI OPTIONAL,

oldSUPI                        [5] SUPI OPTIONAL,

oldGPSI                        [6] GPSI OPTIONAL,

oldserviceID                   [7] ServiceID OPTIONAL,

subscriberRecordChangeMethod   [8] UDMSubscriberRecordChangeMethod,

serviceID                      [9] ServiceID OPTIONAL

}

UDMCancelLocationMessage ::= SEQUENCE

{

sUPI                        [1] SUPI,

pEI                         [2] PEI OPTIONAL,

gPSI                        [3] GPSI OPTIONAL,

gUAMI                       [4] GUAMI OPTIONAL,

pLMNID                      [5] PLMNID OPTIONAL,

cancelLocationMethod        [6] UDMCancelLocationMethod

}

-- =================

-- 5G UDM parameters

-- =================

UDMServingSystemMethod ::= ENUMERATED

{

amf3GPPAccessRegistration(0),

amfNon3GPPAccessRegistration(1),

unknown(2)

}

UDMSubscriberRecordChangeMethod ::= ENUMERATED

{

pEIChange(1),

sUPIChange(2),

gPSIChange(3),

uEDeprovisioning(4),

unknown(5),

serviceIDChange(6)

}

UDMCancelLocationMethod ::= ENUMERATED

{

aMF3GPPAccessDeregistration(1),

aMFNon3GPPAccessDeregistration(2),

uDMDeregistration(3),

unknown(4)

}

ServiceID ::= SEQUENCE

{

nSSAI                     [1] NSSAI OPTIONAL,

cAGID                     [2] SEQUENCE OF CAGID OPTIONAL

}

CAGID ::= UTF8String

-- ===================

-- 5G SMSF definitions

-- ===================

-- See clause 6.2.5.3 for details of this structure

SMSMessage ::= SEQUENCE

{

originatingSMSParty         [1] SMSParty,

terminatingSMSParty         [2] SMSParty,

direction                   [3] Direction,

linkTransferStatus          [4] SMSTransferStatus,

otherMessage                [5] SMSOtherMessageIndication OPTIONAL,

location                    [6] Location OPTIONAL,

peerNFAddress               [7] SMSNFAddress OPTIONAL,

peerNFType                  [8] SMSNFType OPTIONAL,

sMSTPDUData                 [9] SMSTPDUData OPTIONAL,

messageType                 [10] SMSMessageType OPTIONAL,

rPMessageReference          [11] SMSRPMessageReference OPTIONAL

}

SMSReport ::= SEQUENCE

{

location           [1] Location OPTIONAL,

sMSTPDUData        [2] SMSTPDUData,

messageType        [3] SMSMessageType,

rPMessageReference [4] SMSRPMessageReference

}

-- ==================

-- 5G SMSF parameters

-- ==================

SMSAddress ::= OCTET STRING(SIZE(2..12))

SMSMessageType ::= ENUMERATED

{

deliver(1),

deliverReportAck(2),

deliverReportError(3),

statusReport(4),

command(5),

submit(6),

submitReportAck(7),

submitReportError(8),

reserved(9)

}

SMSParty ::= SEQUENCE

{

sUPI        [1] SUPI OPTIONAL,

pEI         [2] PEI OPTIONAL,

gPSI        [3] GPSI OPTIONAL,

sMSAddress  [4] SMSAddress OPTIONAL

}

SMSTransferStatus ::= ENUMERATED

{

transferSucceeded(1),

transferFailed(2),

undefined(3)

}

SMSOtherMessageIndication ::= BOOLEAN

SMSNFAddress ::= CHOICE

{

iPAddress   [1] IPAddress,

e164Number  [2] E164Number

}

SMSNFType ::= ENUMERATED

{

sMSGMSC(1),

iWMSC(2),

sMSRouter(3)

}

SMSRPMessageReference ::= INTEGER (0..255)

SMSTPDUData ::= CHOICE

{

sMSTPDU [1] SMSTPDU,

truncatedSMSTPDU [2] TruncatedSMSTPDU

}

SMSTPDU ::= OCTET STRING (SIZE(1..270))

TruncatedSMSTPDU ::= OCTET STRING (SIZE(1..130))

-- ===============

-- MMS definitions

-- ===============

MMSSend ::= SEQUENCE

{

transactionID       [1]  UTF8String,

version             [2]  MMSVersion,

dateTime            [3]  Timestamp,

originatingMMSParty [4]  MMSParty,

terminatingMMSParty [5]  SEQUENCE OF MMSParty OPTIONAL,

cCRecipients        [6]  SEQUENCE OF MMSParty OPTIONAL,

bCCRecipients       [7]  SEQUENCE OF MMSParty OPTIONAL,

direction           [8]  MMSDirection,

subject             [9]  MMSSubject OPTIONAL,

messageClass        [10]  MMSMessageClass OPTIONAL,

expiry              [11] MMSExpiry,

desiredDeliveryTime [12] Timestamp OPTIONAL,

priority            [13] MMSPriority OPTIONAL,

senderVisibility    [14] BOOLEAN OPTIONAL,

deliveryReport      [15] BOOLEAN OPTIONAL,

readReport          [16] BOOLEAN OPTIONAL,

store               [17] BOOLEAN OPTIONAL,

state               [18] MMState OPTIONAL,

flags               [19] MMFlags OPTIONAL,

replyCharging       [20] MMSReplyCharging OPTIONAL,

applicID            [21] UTF8String OPTIONAL,

replyApplicID       [22] UTF8String OPTIONAL,

auxApplicInfo       [23] UTF8String OPTIONAL,

contentClass        [24] MMSContentClass OPTIONAL,

dRMContent          [25] BOOLEAN OPTIONAL,

adaptationAllowed   [26] MMSAdaptation OPTIONAL,

contentType         [27] MMSContentType,

responseStatus      [28] MMSResponseStatus,

responseStatusText  [29] UTF8String OPTIONAL,

messageID           [30] UTF8String

}

MMSSendByNonLocalTarget ::= SEQUENCE

{

version             [1]  MMSVersion,

transactionID       [2]  UTF8String,

messageID           [3]  UTF8String,

terminatingMMSParty [4]  SEQUENCE OF MMSParty,

originatingMMSParty [5]  MMSParty,

direction           [6]  MMSDirection,

contentType         [7]  MMSContentType,

messageClass        [8]  MMSMessageClass OPTIONAL,

dateTime            [9]  Timestamp,

expiry              [10] MMSExpiry OPTIONAL,

deliveryReport      [11] BOOLEAN OPTIONAL,

priority            [12] MMSPriority OPTIONAL,

senderVisibility    [13] BOOLEAN OPTIONAL,

readReport          [14] BOOLEAN OPTIONAL,

subject             [15] MMSSubject OPTIONAL,

forwardCount        [16] INTEGER OPTIONAL,

previouslySentBy    [17] MMSPreviouslySentBy OPTIONAL,

prevSentByDateTime  [18] Timestamp OPTIONAL,

applicID            [19] UTF8String OPTIONAL,

replyApplicID       [20] UTF8String OPTIONAL,

auxApplicInfo       [21] UTF8String OPTIONAL,

contentClass        [22] MMSContentClass OPTIONAL,

dRMContent          [23] BOOLEAN OPTIONAL,

adaptationAllowed   [24] MMSAdaptation OPTIONAL

}

MMSNotification ::= SEQUENCE

{

transactionID           [1]  UTF8String,

version                 [2]  MMSVersion,

originatingMMSParty     [3]  MMSParty OPTIONAL,

direction               [4]  MMSDirection,

subject                 [5]  MMSSubject OPTIONAL,

deliveryReportRequested [6]  BOOLEAN OPTIONAL,

stored                  [7]  BOOLEAN OPTIONAL,

messageClass            [8]  MMSMessageClass,

priority                [9]  MMSPriority OPTIONAL,

messageSize             [10]  INTEGER,

expiry                  [11] MMSExpiry,

replyCharging           [12] MMSReplyCharging OPTIONAL

}

MMSSendToNonLocalTarget ::= SEQUENCE

{

version             [1]  MMSVersion,

transactionID       [2]  UTF8String,

messageID           [3]  UTF8String,

terminatingMMSParty [4]  SEQUENCE OF MMSParty,

originatingMMSParty [5]  MMSParty,

direction           [6]  MMSDirection,

contentType         [7]  MMSContentType,

messageClass        [8]  MMSMessageClass OPTIONAL,

dateTime            [9]  Timestamp,

expiry              [10] MMSExpiry OPTIONAL,

deliveryReport      [11] BOOLEAN OPTIONAL,

priority            [12] MMSPriority OPTIONAL,

senderVisibility    [13] BOOLEAN OPTIONAL,

readReport          [14] BOOLEAN OPTIONAL,

subject             [15] MMSSubject OPTIONAL,

forwardCount        [16] INTEGER OPTIONAL,

previouslySentBy    [17] MMSPreviouslySentBy OPTIONAL,

prevSentByDateTime  [18] Timestamp OPTIONAL,

applicID            [19] UTF8String OPTIONAL,

replyApplicID       [20] UTF8String OPTIONAL,

auxApplicInfo       [21] UTF8String OPTIONAL,

contentClass        [22] MMSContentClass OPTIONAL,

dRMContent          [23] BOOLEAN OPTIONAL,

adaptationAllowed   [24] MMSAdaptation OPTIONAL

}

MMSNotificationResponse ::= SEQUENCE

{

transactionID [1] UTF8String,

version       [2] MMSVersion,

direction     [3] MMSDirection,

status        [4] MMStatus,

reportAllowed [5] BOOLEAN OPTIONAL

}

MMSRetrieval ::= SEQUENCE

{

transactionID       [1]  UTF8String,

version             [2]  MMSVersion,

messageID           [3]  UTF8String,

dateTime            [4]  Timestamp,

originatingMMSParty [5]  MMSParty OPTIONAL,

previouslySentBy    [6]  MMSPreviouslySentBy OPTIONAL,

prevSentByDateTime  [7]  Timestamp OPTIONAL,

terminatingMMSParty [8]  SEQUENCE OF MMSParty OPTIONAL,

cCRecipients        [9]  SEQUENCE OF MMSParty OPTIONAL,

direction           [10] MMSDirection,

subject             [11] MMSSubject OPTIONAL,

state               [12] MMState OPTIONAL,

flags               [13] MMFlags OPTIONAL,

messageClass        [14] MMSMessageClass OPTIONAL,

priority            [15] MMSPriority,

deliveryReport      [16] BOOLEAN OPTIONAL,

readReport          [17] BOOLEAN OPTIONAL,

replyCharging       [18] MMSReplyCharging OPTIONAL,

retrieveStatus      [19] MMSRetrieveStatus OPTIONAL,

retrieveStatusText  [20] UTF8String OPTIONAL,

applicID            [21] UTF8String OPTIONAL,

replyApplicID       [22] UTF8String OPTIONAL,

auxApplicInfo       [23] UTF8String OPTIONAL,

contentClass        [24] MMSContentClass OPTIONAL,

dRMContent          [25] BOOLEAN OPTIONAL,

replaceID           [26] UTF8String OPTIONAL,

contentType         [27] UTF8String OPTIONAL

}

MMSDeliveryAck ::= SEQUENCE

{

transactionID [1] UTF8String,

version       [2] MMSVersion,

reportAllowed [3] BOOLEAN OPTIONAL,

status        [4] MMStatus,

direction     [5] MMSDirection

}

MMSForward ::= SEQUENCE

{

transactionID         [1]  UTF8String,

version               [2]  MMSVersion,

dateTime              [3]  Timestamp OPTIONAL,

originatingMMSParty   [4]  MMSParty,

terminatingMMSParty   [5]  SEQUENCE OF MMSParty OPTIONAL,

cCRecipients          [6]  SEQUENCE OF MMSParty OPTIONAL,

bCCRecipients         [7]  SEQUENCE OF MMSParty OPTIONAL,

direction             [8]  MMSDirection,

expiry                [9]  MMSExpiry OPTIONAL,

desiredDeliveryTime   [10] Timestamp OPTIONAL,

deliveryReportAllowed [11] BOOLEAN OPTIONAL,

deliveryReport        [12] BOOLEAN OPTIONAL,

store                 [13] BOOLEAN OPTIONAL,

state                 [14] MMState OPTIONAL,

flags                 [15] MMFlags OPTIONAL,

contentLocationReq    [16] UTF8String,

replyCharging         [17] MMSReplyCharging OPTIONAL,

responseStatus        [18] MMSResponseStatus,

responseStatusText    [19] UTF8String  OPTIONAL,

messageID             [20] UTF8String OPTIONAL,

contentLocationConf   [21] UTF8String OPTIONAL,

storeStatus           [22] MMSStoreStatus OPTIONAL,

storeStatusText       [23] UTF8String OPTIONAL

}

MMSDeleteFromRelay ::= SEQUENCE

{

transactionID        [1] UTF8String,

version              [2] MMSVersion,

direction            [3] MMSDirection,

contentLocationReq   [4] SEQUENCE OF UTF8String,

contentLocationConf  [5] SEQUENCE OF UTF8String,

deleteResponseStatus [6] MMSDeleteResponseStatus,

deleteResponseText   [7] SEQUENCE OF UTF8String

}

MMSMBoxStore ::= SEQUENCE

{

transactionID       [1] UTF8String,

version             [2] MMSVersion,

direction           [3] MMSDirection,

contentLocationReq  [4] UTF8String,

state               [5] MMState OPTIONAL,

flags               [6] MMFlags OPTIONAL,

contentLocationConf [7] UTF8String OPTIONAL,

storeStatus         [8] MMSStoreStatus,

storeStatusText     [9] UTF8String OPTIONAL

}

MMSMBoxUpload ::= SEQUENCE

{

transactionID       [1]  UTF8String,

version             [2]  MMSVersion,

direction           [3]  MMSDirection,

state               [4]  MMState OPTIONAL,

flags               [5]  MMFlags OPTIONAL,

contentType         [6]  UTF8String,

contentLocation     [7]  UTF8String OPTIONAL,

storeStatus         [8]  MMSStoreStatus,

storeStatusText     [9]  UTF8String OPTIONAL,

mMessages           [10] SEQUENCE OF MMBoxDescription

}

MMSMBoxDelete ::= SEQUENCE

{

transactionID       [1] UTF8String,

version             [2] MMSVersion,

direction           [3] MMSDirection,

contentLocationReq  [4] SEQUENCE OF UTF8String,

contentLocationConf [5] SEQUENCE OF UTF8String OPTIONAL,

responseStatus      [6] MMSDeleteResponseStatus,

responseStatusText  [7] UTF8String OPTIONAL

}

MMSDeliveryReport ::= SEQUENCE

{

version             [1] MMSVersion,

messageID           [2] UTF8String,

terminatingMMSParty [3] SEQUENCE OF MMSParty,

mMSDateTime         [4] Timestamp,

responseStatus      [5] MMSResponseStatus,

responseStatusText  [6] UTF8String OPTIONAL,

applicID            [7] UTF8String OPTIONAL,

replyApplicID       [8] UTF8String OPTIONAL,

auxApplicInfo       [9] UTF8String OPTIONAL

}

MMSDeliveryReportNonLocalTarget ::= SEQUENCE

{

version             [1]  MMSVersion,

transactionID       [2]  UTF8String,

messageID           [3]  UTF8String,

terminatingMMSParty [4]  SEQUENCE OF MMSParty,

originatingMMSParty [5]  MMSParty,

direction           [6]  MMSDirection,

mMSDateTime         [7]  Timestamp,

forwardToOriginator [8]  BOOLEAN OPTIONAL,

status              [9]  MMStatus,

statusExtension     [10] MMStatusExtension,

statusText          [11] MMStatusText,

applicID            [12] UTF8String OPTIONAL,

replyApplicID       [13] UTF8String OPTIONAL,

auxApplicInfo       [14] UTF8String OPTIONAL

}

MMSReadReport ::= SEQUENCE

{

version             [1] MMSVersion,

messageID           [2] UTF8String,

terminatingMMSParty [3] SEQUENCE OF MMSParty,

originatingMMSParty [4] SEQUENCE OF MMSParty,

direction           [5] MMSDirection,

mMSDateTime         [6] Timestamp,

readStatus          [7] MMSReadStatus,

applicID            [8] UTF8String OPTIONAL,

replyApplicID       [9] UTF8String OPTIONAL,

auxApplicInfo       [10] UTF8String OPTIONAL

}

MMSReadReportNonLocalTarget ::= SEQUENCE

{

version             [1] MMSVersion,

transactionID       [2] UTF8String,

terminatingMMSParty [3] SEQUENCE OF MMSParty,

originatingMMSParty [4] SEQUENCE OF MMSParty,

direction           [5] MMSDirection,

messageID           [6] UTF8String,

mMSDateTime         [7] Timestamp,

readStatus          [8] MMSReadStatus,

readStatusText      [9] MMSReadStatusText OPTIONAL,

applicID            [10] UTF8String OPTIONAL,

replyApplicID       [11] UTF8String OPTIONAL,

auxApplicInfo       [12] UTF8String OPTIONAL

}

MMSCancel ::= SEQUENCE

{

transactionID [1] UTF8String,

version       [2] MMSVersion,

cancelID      [3] UTF8String,

direction     [4] MMSDirection

}

MMSMBoxViewRequest ::= SEQUENCE

{

transactionID   [1]  UTF8String,

version         [2]  MMSVersion,

contentLocation [3]  UTF8String OPTIONAL,

state           [4]  SEQUENCE OF MMState OPTIONAL,

flags           [5]  SEQUENCE OF MMFlags OPTIONAL,

start           [6]  INTEGER OPTIONAL,

limit           [7]  INTEGER OPTIONAL,

attributes      [8]  SEQUENCE OF UTF8String OPTIONAL,

totals          [9]  INTEGER OPTIONAL,

quotas          [10] MMSQuota OPTIONAL

}

MMSMBoxViewResponse ::= SEQUENCE

{

transactionID   [1]  UTF8String,

version         [2]  MMSVersion,

contentLocation [3]  UTF8String OPTIONAL,

state           [4]  SEQUENCE OF MMState OPTIONAL,

flags           [5]  SEQUENCE OF MMFlags OPTIONAL,

start           [6]  INTEGER OPTIONAL,

limit           [7]  INTEGER OPTIONAL,

attributes      [8]  SEQUENCE OF UTF8String OPTIONAL,

mMSTotals       [9]  BOOLEAN OPTIONAL,

mMSQuotas       [10] BOOLEAN OPTIONAL,

mMessages       [11] SEQUENCE OF MMBoxDescription

}

MMBoxDescription ::= SEQUENCE

{

contentLocation          [1]  UTF8String OPTIONAL,

messageID                [2]  UTF8String OPTIONAL,

state                    [3]  MMState OPTIONAL,

flags                    [4]  SEQUENCE OF MMFlags OPTIONAL,

dateTime                 [5]  Timestamp OPTIONAL,

originatingMMSParty      [6]  MMSParty OPTIONAL,

terminatingMMSParty      [7]  SEQUENCE OF MMSParty OPTIONAL,

cCRecipients             [8]  SEQUENCE OF MMSParty OPTIONAL,

bCCRecipients            [9]  SEQUENCE OF MMSParty OPTIONAL,

messageClass             [10] MMSMessageClass OPTIONAL,

subject                  [11] MMSSubject OPTIONAL,

priority                 [12] MMSPriority OPTIONAL,

deliveryTime             [13] Timestamp OPTIONAL,

readReport               [14] BOOLEAN OPTIONAL,

messageSize              [15] INTEGER OPTIONAL,

replyCharging            [16] MMSReplyCharging OPTIONAL,

previouslySentBy         [17] MMSPreviouslySentBy OPTIONAL,

previouslySentByDateTime [18] Timestamp OPTIONAL,

contentType              [19] UTF8String OPTIONAL

}

-- =========

-- MMS CCPDU

-- =========

MMSCCPDU ::= SEQUENCE

{

version    [1] MMSVersion,

transactionID [2] UTF8String,

mMSContent    [3] OCTET STRING

}

-- ==============

-- MMS parameters

-- ==============

MMSAdaptation ::= SEQUENCE

{

allowed   [1] BOOLEAN,

overriden [2] BOOLEAN

}

MMSCancelStatus ::= ENUMERATED

{

cancelRequestSuccessfullyReceived(1),

cancelRequestCorrupted(2)

}

MMSContentClass ::= ENUMERATED

{

text(1),

imageBasic(2),

imageRich(3),

videoBasic(4),

videoRich(5),

megaPixel(6),

contentBasic(7),

contentRich(8)

}

MMSContentType ::= UTF8String

MMSDeleteResponseStatus ::= ENUMERATED

{

ok(1),

errorUnspecified(2),

errorServiceDenied(3),

errorMessageFormatCorrupt(4),

errorSendingAddressUnresolved(5),

errorMessageNotFound(6),

errorNetworkProblem(7),

errorContentNotAccepted(8),

errorUnsupportedMessage(9),

errorTransientFailure(10),

errorTransientSendingAddressUnresolved(11),

errorTransientMessageNotFound(12),

errorTransientNetworkProblem(13),

errorTransientPartialSuccess(14),

errorPermanentFailure(15),

errorPermanentServiceDenied(16),

errorPermanentMessageFormatCorrupt(17),

errorPermanentSendingAddressUnresolved(18),

errorPermanentMessageNotFound(19),

errorPermanentContentNotAccepted(20),

errorPermanentReplyChargingLimitationsNotMet(21),

errorPermanentReplyChargingRequestNotAccepted(22),

errorPermanentReplyChargingForwardingDenied(23),

errorPermanentReplyChargingNotSupported(24),

errorPermanentAddressHidingNotSupported(25),

errorPermanentLackOfPrepaid(26)

}

MMSDirection ::= ENUMERATED

{

fromTarget(0),

toTarget(1)

}

MMSElementDescriptor ::= SEQUENCE

{

reference [1] UTF8String,

parameter [2] UTF8String     OPTIONAL,

value     [3] UTF8String     OPTIONAL

}

MMSExpiry ::= SEQUENCE

{

expiryPeriod [1] INTEGER,

periodFormat [2] MMSPeriodFormat

}

MMFlags ::= SEQUENCE

{

length     [1] INTEGER,

flag       [2] MMStateFlag,

flagString [3] UTF8String

}

MMSMessageClass ::= ENUMERATED

{

personal(1),

advertisement(2),

informational(3),

auto(4)

}

MMSParty ::= SEQUENCE

{

mMSPartyIDs [1] SEQUENCE OF MMSPartyID,

nonLocalID  [2] NonLocalID

}

MMSPartyID ::= CHOICE

{

e164Number   [1] E164Number,

emailAddress [2] EmailAddress,

iMSI         [3] IMSI,

iMPU         [4] IMPU,

iMPI         [5] IMPI,

sUPI         [6] SUPI,

gPSI         [7] GPSI

}

MMSPeriodFormat ::= ENUMERATED

{

absolute(1),

relative(2)

}

MMSPreviouslySent ::= SEQUENCE

{

previouslySentByParty [1] MMSParty,

sequenceNumber        [2] INTEGER,

previousSendDateTime  [3] Timestamp

}

MMSPreviouslySentBy ::= SEQUENCE OF MMSPreviouslySent

MMSPriority ::= ENUMERATED

{

low(1),

normal(2),

high(3)

}

MMSQuota ::= SEQUENCE

{

quota     [1] INTEGER,

quotaUnit [2] MMSQuotaUnit

}

MMSQuotaUnit ::= ENUMERATED

{

numMessages(1),

bytes(2)

}

MMSReadStatus ::= ENUMERATED

{

read(1),

deletedWithoutBeingRead(2)

}

MMSReadStatusText ::= UTF8String

MMSReplyCharging ::= ENUMERATED

{

requested(0),

requestedTextOnly(1),

accepted(2),

acceptedTextOnly(3)

}

MMSResponseStatus ::= ENUMERATED

{

ok(1),

errorUnspecified(2),

errorServiceDenied(3),

errorMessageFormatCorrupt(4),

errorSendingAddressUnresolved(5),

errorMessageNotFound(6),

errorNetworkProblem(7),

errorContentNotAccepted(8),

errorUnsupportedMessage(9),

errorTransientFailure(10),

errorTransientSendingAddressUnresolved(11),

errorTransientMessageNotFound(12),

errorTransientNetworkProblem(13),

errorTransientPartialSuccess(14),

errorPermanentFailure(15),

errorPermanentServiceDenied(16),

errorPermanentMessageFormatCorrupt(17),

errorPermanentSendingAddressUnresolved(18),

errorPermanentMessageNotFound(19),

errorPermanentContentNotAccepted(20),

errorPermanentReplyChargingLimitationsNotMet(21),

errorPermanentReplyChargingRequestNotAccepted(22),

errorPermanentReplyChargingForwardingDenied(23),

errorPermanentReplyChargingNotSupported(24),

errorPermanentAddressHidingNotSupported(25),

errorPermanentLackOfPrepaid(26)

}

MMSRetrieveStatus ::= ENUMERATED

{

success(1),

errorTransientFailure(2),

errorTransientMessageNotFound(3),

errorTransientNetworkProblem(4),

errorPermanentFailure(5),

errorPermanentServiceDenied(6),

errorPermanentMessageNotFound(7),

errorPermanentContentUnsupported(8)

}

MMSStoreStatus ::= ENUMERATED

{

success(1),

errorTransientFailure(2),

errorTransientNetworkProblem(3),

errorPermanentFailure(4),

errorPermanentServiceDenied(5),

errorPermanentMessageFormatCorrupt(6),

errorPermanentMessageNotFound(7),

errorMMBoxFull(8)

}

MMState ::= ENUMERATED

{

draft(1),

sent(2),

new(3),

retrieved(4),

forwarded(5)

}

MMStateFlag ::= ENUMERATED

{

add(1),

remove(2),

filter(3)

}

MMStatus ::= ENUMERATED

{

expired(1),

retrieved(2),

rejected(3),

deferred(4),

unrecognized(5),

indeterminate(6),

forwarded(7),

unreachable(8)

}

MMStatusExtension ::= ENUMERATED

{

rejectionByMMSRecipient(0),

rejectionByOtherRS(1)

}

MMStatusText ::= UTF8String

MMSSubject ::= UTF8String

MMSVersion ::= SEQUENCE

{

majorVersion [1] INTEGER,

minorVersion [2] INTEGER

}

-- ==================

-- 5G PTC definitions

-- ==================

PTCRegistration  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCServerURI                  [2] UTF8String,

pTCRegistrationRequest        [3] PTCRegistrationRequest,

pTCRegistrationOutcome        [4] PTCRegistrationOutcome

}

PTCSessionInitiation  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCServerURI                  [3] UTF8String,

pTCSessionInfo                [4] PTCSessionInfo,

pTCOriginatingID              [5] PTCTargetInformation,

pTCParticipants               [6] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCParticipantPresenceStatus  [7] MultipleParticipantPresenceStatus OPTIONAL,

location                      [8] Location OPTIONAL,

pTCBearerCapability           [9] UTF8String OPTIONAL,

pTCHost                       [10] PTCTargetInformation OPTIONAL

}

PTCSessionAbandon  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

location                      [4] Location OPTIONAL,

pTCAbandonCause               [5] INTEGER

}

PTCSessionStart  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCServerURI                  [3] UTF8String,

pTCSessionInfo                [4] PTCSessionInfo,

pTCOriginatingID              [5] PTCTargetInformation,

pTCParticipants               [6] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCParticipantPresenceStatus  [7] MultipleParticipantPresenceStatus OPTIONAL,

location                      [8] Location OPTIONAL,

pTCHost                       [9] PTCTargetInformation OPTIONAL,

pTCBearerCapability           [10] UTF8String OPTIONAL

}

PTCSessionEnd  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCServerURI                  [3] UTF8String,

pTCSessionInfo                [4] PTCSessionInfo,

pTCParticipants               [5] SEQUENCE OF PTCTargetInformation OPTIONAL,

location                      [6] Location OPTIONAL,

pTCSessionEndCause            [7] PTCSessionEndCause

}

PTCStartOfInterception  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

preEstSessionID               [3] PTCSessionInfo OPTIONAL,

pTCOriginatingID              [4] PTCTargetInformation,

pTCSessionInfo                [5] PTCSessionInfo OPTIONAL,

pTCHost                       [6] PTCTargetInformation OPTIONAL,

pTCParticipants               [7] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCMediaStreamAvail           [8] BOOLEAN OPTIONAL,

pTCBearerCapability           [9] UTF8String OPTIONAL

}

PTCPreEstablishedSession  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCServerURI                  [2] UTF8String,

rTPSetting                    [3] RTPSetting,

pTCMediaCapability            [4] UTF8String,

pTCPreEstSessionID            [5] PTCSessionInfo,

pTCPreEstStatus               [6] PTCPreEstStatus,

pTCMediaStreamAvail           [7] BOOLEAN OPTIONAL,

location                      [8] Location OPTIONAL,

pTCFailureCode                [9] PTCFailureCode OPTIONAL

}

PTCInstantPersonalAlert  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCIPAPartyID                 [2] PTCTargetInformation,

pTCIPADirection               [3] Direction

}

PTCPartyJoin  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

pTCParticipants               [4] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCParticipantPresenceStatus  [5] MultipleParticipantPresenceStatus OPTIONAL,

pTCMediaStreamAvail           [6] BOOLEAN OPTIONAL,

pTCBearerCapability           [7] UTF8String OPTIONAL

}

PTCPartyDrop  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

pTCPartyDrop                  [4] PTCTargetInformation,

pTCParticipantPresenceStatus  [5] PTCParticipantPresenceStatus OPTIONAL

}

PTCPartyHold  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

pTCParticipants               [4] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCHoldID                     [5] SEQUENCE OF PTCTargetInformation,

pTCHoldRetrieveInd            [6] BOOLEAN

}

PTCMediaModification  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

pTCMediaStreamAvail           [4] BOOLEAN OPTIONAL,

pTCBearerCapability           [5] UTF8String

}

PTCGroupAdvertisement  ::=SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCIDList                     [3] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCGroupAuthRule              [4] PTCGroupAuthRule OPTIONAL,

pTCGroupAdSender              [5] PTCTargetInformation,

pTCGroupNickname              [6] UTF8String OPTIONAL

}

PTCFloorControl  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessioninfo                [3] PTCSessionInfo,

pTCFloorActivity              [4] SEQUENCE OF PTCFloorActivity,

pTCFloorSpeakerID             [5] PTCTargetInformation OPTIONAL,

pTCMaxTBTime                  [6] INTEGER OPTIONAL,

pTCQueuedFloorControl         [7] BOOLEAN OPTIONAL,

pTCQueuedPosition             [8] INTEGER OPTIONAL,

pTCTalkBurstPriority          [9] PTCTBPriorityLevel OPTIONAL,

pTCTalkBurstReason            [10] PTCTBReasonCode OPTIONAL

}

PTCTargetPresence  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCTargetPresenceStatus       [2] PTCParticipantPresenceStatus

}

PTCParticipantPresence  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCParticipantPresenceStatus  [2] PTCParticipantPresenceStatus

}

PTCListManagement  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCListManagementType         [3] PTCListManagementType OPTIONAL,

pTCListManagementAction       [4] PTCListManagementAction OPTIONAL,

pTCListManagementFailure      [5] PTCListManagementFailure OPTIONAL,

pTCContactID                  [6] PTCTargetInformation OPTIONAL,

pTCIDList                     [7] SEQUENCE OF PTCIDList OPTIONAL,

pTCHost                       [8] PTCTargetInformation OPTIONAL

}

PTCAccessPolicy  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCAccessPolicyType           [3] PTCAccessPolicyType OPTIONAL,

pTCUserAccessPolicy           [4] PTCUserAccessPolicy OPTIONAL,

pTCGroupAuthRule              [5] PTCGroupAuthRule OPTIONAL,

pTCContactID                  [6] PTCTargetInformation OPTIONAL,

pTCAccessPolicyFailure        [7] PTCAccessPolicyFailure OPTIONAL

}

-- =================

-- 5G PTC parameters

-- =================

PTCRegistrationRequest  ::= ENUMERATED

{

register(1),

reRegister(2),

deRegister(3)

}

PTCRegistrationOutcome  ::= ENUMERATED

{

success(1),

failure(2)

}

PTCSessionEndCause  ::= ENUMERATED

{

initiaterLeavesSession(1),

definedParticipantLeaves(2),

numberOfParticipants(3),

sessionTimerExpired(4),

pTCSpeechInactive(5),

allMediaTypesInactive(6)

}

PTCTargetInformation  ::= SEQUENCE

{

identifiers                [1] SEQUENCE SIZE(1..MAX) OF PTCIdentifiers

}

PTCIdentifiers  ::= CHOICE

{

mCPTTID                    [1] UTF8String,

instanceIdentifierURN      [2] UTF8String,

pTCChatGroupID             [3] PTCChatGroupID,

iMPU                       [4] IMPU,

iMPI                       [5] IMPI

}

PTCSessionInfo  ::= SEQUENCE

{

pTCSessionURI              [1] UTF8String,

pTCSessionType             [2] PTCSessionType

}

PTCSessionType  ::= ENUMERATED

{

ondemand(1),

preEstablished(2),

adhoc(3),

prearranged(4),

groupSession(5)

}

MultipleParticipantPresenceStatus  ::= SEQUENCE OF PTCParticipantPresenceStatus

PTCParticipantPresenceStatus  ::= SEQUENCE

{

presenceID                 [1] PTCTargetInformation,

presenceType               [2] PTCPresenceType,

presenceStatus             [3] BOOLEAN

}

PTCPresenceType  ::= ENUMERATED

{

pTCClient(1),

pTCGroup(2)

}

PTCPreEstStatus  ::= ENUMERATED

{

established(1),

modified(2),

released(3)

}

RTPSetting  ::= SEQUENCE

{

iPAddress                  [1] IPAddress,

portNumber                 [2] PortNumber

}

PTCIDList  ::= SEQUENCE

{

pTCPartyID                 [1] PTCTargetInformation,

pTCChatGroupID             [2] PTCChatGroupID

}

PTCChatGroupID  ::= SEQUENCE

{

groupIdentity              [1] UTF8String

}

PTCFloorActivity  ::= ENUMERATED

{

tBCPRequest(1),

tBCPGranted(2),

tBCPDeny(3),

tBCPIdle(4),

tBCPTaken(5),

tBCPRevoke(6),

tBCPQueued(7),

tBCPRelease(8)

}

PTCTBPriorityLevel  ::= ENUMERATED

{

preEmptive(1),

highPriority(2),

normalPriority(3),

listenOnly(4)

}

PTCTBReasonCode  ::= ENUMERATED

{

noQueuingAllowed(1),

oneParticipantSession(2),

listenOnly(3),

exceededMaxDuration(4),

tBPrevented(5)

}

PTCListManagementType  ::= ENUMERATED

{

contactListManagementAttempt(1),

groupListManagementAttempt(2),

contactListManagementResult(3),

groupListManagementResult(4),

requestUnsuccessful(5)

}

PTCListManagementAction  ::= ENUMERATED

{

create(1),

modify(2),

retrieve(3),

delete(4),

notify(5)

}

PTCAccessPolicyType  ::= ENUMERATED

{

pTCUserAccessPolicyAttempt(1),

groupAuthorizationRulesAttempt(2),

pTCUserAccessPolicyQuery(3),

groupAuthorizationRulesQuery(4),

pTCUserAccessPolicyResult(5),

groupAuthorizationRulesResult(6),

requestUnsuccessful(7)

}

PTCUserAccessPolicy  ::= ENUMERATED

{

allowIncomingPTCSessionRequest(1),

blockIncomingPTCSessionRequest(2),

allowAutoAnswerMode(3),

allowOverrideManualAnswerMode(4)

}

PTCGroupAuthRule  ::= ENUMERATED

{

allowInitiatingPTCSession(1),

blockInitiatingPTCSession(2),

allowJoiningPTCSession(3),

blockJoiningPTCSession(4),

allowAddParticipants(5),

blockAddParticipants(6),

allowSubscriptionPTCSessionState(7),

blockSubscriptionPTCSessionState(8),

allowAnonymity(9),

forbidAnonymity(10)

}

PTCFailureCode  ::= ENUMERATED

{

sessionCannotBeEstablished(1),

sessionCannotBeModified(2)

}

PTCListManagementFailure  ::= ENUMERATED

{

requestUnsuccessful(1),

requestUnknown(2)

}

PTCAccessPolicyFailure  ::= ENUMERATED

{

requestUnsuccessful(1),

requestUnknown(2)

}

-- ==================

-- RCS definitions

-- ==================

-- See clause 7.X.2.1.2 for details of this structure

RCSRegistration ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

rCSServerURI                  [2] UTF8String,

rCSRegistrationRequest        [3] RCSRegistrationRequest,

rCSRegistrationOutcome        [4] RCSRegistrationOutcome,

rCSTargetCapabilities         [5] SEQUENCE OF RCSCapability,

location                      [6] Location OPTIONAL

}

-- See clause 7.X.2.1.3 for details of this structure

RCSCapabilityDiscovery ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

contactIdentity               [2] SEQUENCE OF IMPU OPTIONAL,

targetCapabilities            [3] SEQUENCE OF RCSCapability OPTIONAL,

contactCapabilities           [4] SEQUENCE OF RCSCapability OPTIONAL,

rCSFlag                       [5] BOOLEAN OPTIONAL,

contactRCSState               [6] RCSState OPTIONAL,

location                      [7] Location OPTIONAL

}

-- See clause 7.X.3.1.2 for details of this structure

RCSPagerModeMessageDelivery ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

originatingIdentity           [2] IMPU,

destinationIdentities         [3] SEQUENCE OF IMPU,

messageID                     [4] UTF8String,

pagerModeMessageContent       [5] OCTET STRING (SIZE(1..1300)) OPTIONAL,

direction                     [6] Direction,

fileTransferFlag              [7] BOOLEAN OPTIONAL,

fileURL                       [8] UTF8String OPTIONAL,

location                      [9] Location OPTIONAL

}

-- See clause 7.X.3.1.3 for details of this structure

RCSPagerModeMessageDeliveryNotification ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

originatingIdentity           [2] IMPU,

destinationIdentity           [3] IMPU,

messageID                     [4] UTF8String,

deliveryOutcome               [5] DeliveryOutcome OPTIONAL,

displayOutcome                [6] DisplayOutcome OPTIONAL

}

-- See clause 7.X.4.1.2 for details of this structure

RCSLargeMessageModeSessionEstablishment ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

direction                     [2] Direction,

originatingIdentity           [3] IMPU,

destinationIdentities         [4] SEQUENCE OF IMPU,

bearerCapability              [5] OCTET STRING OPTIONAL,

largeMessageSize              [6] INTEGER OPTIONAL,

fileTransferFlag              [7] BOOLEAN OPTIONAL,

location                      [8] Location OPTIONAL

}

-- See clause 7.X.4.1.3 for details of this structure

RCSLargeMessageModeUnsuccessfulProcedure ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

direction                     [2] Direction,

originatingIdentity           [3] IMPU,

destinationIdentities         [4] SEQUENCE OF IMPU,

fileTransferFlag              [5] BOOLEAN OPTIONAL,

location                      [6] Location OPTIONAL,

cause                         [7] UTF8String

}

-- See clause 7.X.5.1.2 for details of this structure

RCS1to1ChatSessionEstablishment ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

direction                     [2] Direction,

rCSServerURI                  [3] UTF8String,

originatingIdentity           [4] IMPU,

destinationIdentities         [5] SEQUENCE OF IMPU,

sDPOffer                      [6] OCTET STRING OPTIONAL,

sDPAnswer                     [7] OCTET STRING OPTIONAL,

location                      [8] Location OPTIONAL,

fileTransferFlag              [9] BOOLEAN OPTIONAL,

firstMessage                  [10] OCTET STRING OPTIONAL

}

-- See clause 7.X.5.1.3 for details of this structure

RCS1to1ChatSessionRelease ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

direction                     [2] Direction,

rCSServerURI                  [3] UTF8String,

location                      [4] Location OPTIONAL,

cause                         [5] Chat1to1SessionReleaseCause

}

-- See clause 7.X.5.1.4 for details of this structure

RCS1to1ChatUnsuccessfulProcedure ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

direction                     [2] Direction,

originatingIdentity           [3] IMPU,

destinationIdentity           [4] IMPU,

fileTransferFlag              [5] BOOLEAN OPTIONAL,

location                      [6] Location OPTIONAL,

cause                         [7] UTF8String

}

-- See clause 7.X.5.1.5 for details of this structure

RCSStartOfInterceptionWithEstablished1to1ChatSession ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

direction                     [2] Direction,

rCSServerURI                  [3] UTF8String,

originatingIdentity           [4] IMPU,

destinationIdentities         [5] SEQUENCE OF IMPU,

sDPOffer                      [6] OCTET STRING OPTIONAL,

sDPAnswer                     [7] OCTET STRING OPTIONAL,

location                      [8] Location OPTIONAL

}

-- See clause 7.X.6.1.2 for details of this structure

RCSGroupChatSessionEstablishment ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

direction                     [2] Direction,

rCSServerURI                  [3] UTF8String,

groupChatSessionID            [4] UTF8String,

originatingIdentity           [5] IMPU,

destinationIdentities         [6] SEQUENCE OF IMPU,

bearerCapability              [7] OCTET STRING OPTIONAL,

location                      [8] Location OPTIONAL,

fileTransferFlag              [9] BOOLEAN OPTIONAL

}

-- See clause 7.X.6.1.3 for details of this structure

RCSGroupChatSessionRelease ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

direction                     [2] Direction,

rCSServerURI                  [3] UTF8String,

groupChatSessionID            [4] UTF8String,

participants                  [5] SEQUENCE OF IMPU OPTIONAL,

location                      [6] Location OPTIONAL,

cause                         [7] GroupChatSessionReleaseCause

}

-- See clause 7.X.6.1.4 for details of this structure

RCSGroupChatUnsuccessfulProcedure ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

direction                     [2] Direction,

originatingIdentity           [3] IMPU,

destinationIdentities         [4] SEQUENCE OF IMPU,

location                      [5] Location OPTIONAL,

cause                         [6] UTF8String,

fileTransferFlag              [7] BOOLEAN OPTIONAL

}

-- See clause 7.X.6.1.5 for details of this structure

RCSStartOfInterceptionWithEstablishedGroupChatSession ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

direction                     [2] Direction,

rCSServerURI                  [3] UTF8String,

groupChatSessionID            [4] UTF8String,

originatingIdentity           [5] IMPU,

participants                  [6] SEQUENCE OF IMPU OPTIONAL,

bearerCapability              [7] OCTET STRING OPTIONAL,

location                      [8] Location OPTIONAL

}

-- See clause 7.X.6.1.6 for details of this structure

RCSGroupChatSessionPartyJoin ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

direction                     [2] Direction,

groupChatSessionID            [3] UTF8String,

partyJoin                     [4] IMPU,

participants                  [5] SEQUENCE OF IMPU OPTIONAL,

bearerCapability              [6] OCTET STRING OPTIONAL

}

-- See clause 7.X.6.1.7 for details of this structure

RCSGroupChatSessionPartyDrop ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

direction                     [2] Direction,

groupChatSessionID            [3] UTF8String,

partyDrop                     [4] IMPU,

participants                  [5] SEQUENCE OF IMPU OPTIONAL

}

-- See clause 7.X.7.1.2 for details of this structure

RCSFileUpload ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

contentType                   [2] UTF8String,

fileContent                   [3] OCTET STRING,

fileName                      [4] UTF8String,

fileURL                       [5] UTF8String,

fileSize                      [6] INTEGER,

fileValidity                  [7] Timestamp OPTIONAL

}

-- See clause 7.X.7.1.3 for details of this structure

RCSFileDownload ::= SEQUENCE

{

rCSIdentities                 [1] RCSIdentities,

rCSUploadIdentity             [2] IMPU,

contentType                   [3] UTF8String,

fileContent                   [4] OCTET STRING,

fileName                      [5] UTF8String,

fileURL                       [6] UTF8String,

fileSize                      [7] INTEGER

}

-- =================

-- RCS parameters

-- =================

RCSIdentities ::= SEQUENCE

{

publicIdentities                  [1] SEQUENCE OF IMPU,

privateIdentity                   [2] IMPI,

terminalIdentity                  [3] IMEI

}

RCSRegistrationRequest ::= ENUMERATED

{

register(1),

reRegister(2),

deRegister(3)

}

RCSRegistrationOutcome ::= ENUMERATED

{

success(1),

failure(2)

}

DeliveryOutcome ::= ENUMERATED

{

delivered(1),

failed(2),

forbidden(3),

error(4)

}

DisplayOutcome ::= ENUMERATED

{

displayed(1),

forbidden(2),

error(3)

}

RCSState ::= ENUMERATED

{

registered(1),

deregistered(2)

}

RCSCapability ::= ENUMERATED

{

pagerModeMessaging(1),

largeModeMessaging(2),

chat1to1(3),

chat1ToN(4),

fileTransferHTTP(5)

}

Chat1to1SessionReleaseCause ::= ENUMERATED

{

normal(1),

inactivityTimeout(2)

}

GroupChatSessionReleaseCause ::= ENUMERATED

{

normal(1),

lowParticipantNumber(2),

inactivityTimeout(3)

}

-- ===================

-- 5G LALS definitions

-- ===================

LALSReport ::= SEQUENCE

{

sUPI                [1] SUPI OPTIONAL,

--  pEI                 [2] PEI OPTIONAL, deprecated in Release-16, do not re-use this tag number

gPSI                [3] GPSI OPTIONAL,

location            [4] Location OPTIONAL,

iMPU                [5] IMPU OPTIONAL,

iMSI                [7] IMSI OPTIONAL,

mSISDN              [8] MSISDN OPTIONAL

}

-- =====================

-- PDHR/PDSR definitions

-- =====================

PDHeaderReport ::= SEQUENCE

{

pDUSessionID                [1] PDUSessionID,

sourceIPAddress             [2] IPAddress,

sourcePort                  [3] PortNumber OPTIONAL,

destinationIPAddress        [4] IPAddress,

destinationPort             [5] PortNumber OPTIONAL,

nextLayerProtocol           [6] NextLayerProtocol,

iPv6flowLabel               [7] IPv6FlowLabel OPTIONAL,

direction                   [8] Direction,

packetSize                  [9] INTEGER

}

PDSummaryReport ::= SEQUENCE

{

pDUSessionID                [1] PDUSessionID,

sourceIPAddress             [2] IPAddress,

sourcePort                  [3] PortNumber OPTIONAL,

destinationIPAddress        [4] IPAddress,

destinationPort             [5] PortNumber OPTIONAL,

nextLayerProtocol           [6] NextLayerProtocol,

iPv6flowLabel               [7] IPv6FlowLabel OPTIONAL,

direction                   [8] Direction,

pDSRSummaryTrigger          [9] PDSRSummaryTrigger,

firstPacketTimestamp        [10] Timestamp,

lastPacketTimestamp         [11] Timestamp,

packetCount                 [12] INTEGER,

byteCount                   [13] INTEGER

}

-- ====================

-- PDHR/PDSR parameters

-- ====================

PDSRSummaryTrigger ::= ENUMERATED

{

timerExpiry(1),

packetCount(2),

byteCount(3),

startOfFlow(4),

endOfFlow(5)

}

-- ==================================

-- Identifier Association definitions

-- ==================================

AMFIdentifierAssocation ::= SEQUENCE

{

sUPI             [1] SUPI,

sUCI             [2] SUCI OPTIONAL,

pEI              [3] PEI OPTIONAL,

gPSI             [4] GPSI OPTIONAL,

gUTI             [5] FiveGGUTI,

location         [6] Location,

fiveGSTAIList    [7] TAIList OPTIONAL

}

MMEIdentifierAssocation ::= SEQUENCE

{

iMSI        [1] IMSI,

iMEI        [2] IMEI OPTIONAL,

mSISDN      [3] MSISDN OPTIONAL,

gUTI        [4] GUTI,

location    [5] Location,

tAIList     [6] TAIList OPTIONAL

}

-- =================================

-- Identifier Association parameters

-- =================================

MMEGroupID ::= OCTET STRING (SIZE(2))

MMECode ::= OCTET STRING (SIZE(1))

TMSI ::= OCTET STRING (SIZE(4))

-- ===================

-- EPS MME definitions

-- ===================

MMEAttach ::= SEQUENCE

{

attachType       [1] EPSAttachType,

attachResult     [2] EPSAttachResult,

iMSI             [3] IMSI,

iMEI             [4] IMEI OPTIONAL,

mSISDN           [5] MSISDN OPTIONAL,

gUTI             [6] GUTI OPTIONAL,

location         [7] Location OPTIONAL,

ePSTAIList       [8] TAIList OPTIONAL,

sMSServiceStatus [9] EPSSMSServiceStatus OPTIONAL,

oldGUTI          [10] GUTI OPTIONAL,

eMM5GRegStatus   [11] EMM5GMMStatus OPTIONAL

}

MMEDetach ::= SEQUENCE

{

detachDirection    [1] MMEDirection,

detachType         [2] EPSDetachType,

iMSI               [3] IMSI,

iMEI               [4] IMEI OPTIONAL,

mSISDN             [5] MSISDN OPTIONAL,

gUTI               [6] GUTI OPTIONAL,

cause              [7] EMMCause OPTIONAL,

location           [8] Location OPTIONAL,

switchOffIndicator [9] SwitchOffIndicator OPTIONAL

}

MMELocationUpdate ::= SEQUENCE

{

iMSI             [1] IMSI,

iMEI             [2] IMEI OPTIONAL,

mSISDN           [3] MSISDN OPTIONAL,

gUTI             [4] GUTI OPTIONAL,

location         [5] Location OPTIONAL,

oldGUTI          [6] GUTI OPTIONAL,

sMSServiceStatus [7] EPSSMSServiceStatus OPTIONAL

}

MMEStartOfInterceptionWithEPSAttachedUE ::= SEQUENCE

{

attachType         [1] EPSAttachType,

attachResult       [2] EPSAttachResult,

iMSI               [3] IMSI,

iMEI               [4] IMEI OPTIONAL,

mSISDN             [5] MSISDN OPTIONAL,

gUTI               [6] GUTI OPTIONAL,

location           [7] Location OPTIONAL,

ePSTAIList         [9] TAIList OPTIONAL,

sMSServiceStatus   [10] EPSSMSServiceStatus OPTIONAL,

eMM5GRegStatus     [12] EMM5GMMStatus OPTIONAL

}

MMEUnsuccessfulProcedure ::= SEQUENCE

{

failedProcedureType [1] MMEFailedProcedureType,

failureCause        [2] MMEFailureCause,

iMSI                [3] IMSI OPTIONAL,

iMEI                [4] IMEI OPTIONAL,

mSISDN              [5] MSISDN OPTIONAL,

gUTI                [6] GUTI OPTIONAL,

location            [7] Location OPTIONAL

}

-- ==================

-- EPS MME parameters

-- ==================

EMMCause ::= INTEGER (0..255)

ESMCause ::= INTEGER (0..255)

EPSAttachType ::= ENUMERATED

{

ePSAttach(1),

combinedEPSIMSIAttach(2),

ePSRLOSAttach(3),

ePSEmergencyAttach(4),

reserved(5)

}

EPSAttachResult ::= ENUMERATED

{

ePSOnly(1),

combinedEPSIMSI(2)

}

EPSDetachType ::= ENUMERATED

{

ePSDetach(1),

iMSIDetach(2),

combinedEPSIMSIDetach(3),

reAttachRequired(4),

reAttachNotRequired(5),

reserved(6)

}

EPSSMSServiceStatus ::= ENUMERATED

{

sMSServicesNotAvailable(1),

sMSServicesNotAvailableInThisPLMN(2),

networkFailure(3),

congestion(4)

}

MMEDirection ::= ENUMERATED

{

networkInitiated(1),

uEInitiated(2)

}

MMEFailedProcedureType ::= ENUMERATED

{

attachReject(1),

authenticationReject(2),

securityModeReject(3),

serviceReject(4),

trackingAreaUpdateReject(5),

activateDedicatedEPSBearerContextReject(6),

activateDefaultEPSBearerContextReject(7),

bearerResourceAllocationReject(8),

bearerResourceModificationReject(9),

modifyEPSBearerContectReject(10),

pDNConnectivityReject(11),

pDNDisconnectReject(12)

}

MMEFailureCause ::= CHOICE

{

eMMCause [1] EMMCause,

eSMCause [2] ESMCause

}

-- ===========================

-- LI Notification definitions

-- ===========================

LINotification ::= SEQUENCE

{

notificationType                    [1] LINotificationType,

appliedTargetID                     [2] TargetIdentifier OPTIONAL,

appliedDeliveryInformation          [3] SEQUENCE OF LIAppliedDeliveryInformation OPTIONAL,

appliedStartTime                    [4] Timestamp OPTIONAL,

appliedEndTime                      [5] Timestamp OPTIONAL

}

-- ==========================

-- LI Notification parameters

-- ==========================

LINotificationType ::= ENUMERATED

{

activation(1),

deactivation(2),

modification(3)

}

LIAppliedDeliveryInformation ::= SEQUENCE

{

hI2DeliveryIPAddress                [1] IPAddress OPTIONAL,

hI2DeliveryPortNumber               [2] PortNumber OPTIONAL,

hI3DeliveryIPAddress                [3] IPAddress OPTIONAL,

hI3DeliveryPortNumber               [4] PortNumber OPTIONAL

}

-- ===============

-- MDF definitions

-- ===============

MDFCellSiteReport ::= SEQUENCE OF CellInformation

-- ==============================

-- 5G EPS Interworking Parameters

-- ==============================

EMM5GMMStatus ::= SEQUENCE

{

eMMRegStatus  [1] EMMRegStatus OPTIONAL,

fiveGMMStatus [2] FiveGMMStatus OPTIONAL

}

EPS5GGUTI ::= CHOICE

{

gUTI      [1] GUTI,

fiveGGUTI [2] FiveGGUTI

}

EMMRegStatus ::= ENUMERATED

{

uEEMMRegistered(1),

uENotEMMRegistered(2)

}

FiveGMMStatus ::= ENUMERATED

{

uE5GMMRegistered(1),

uENot5GMMRegistered(2)

}

-- =================

-- Common Parameters

-- =================

AccessType ::= ENUMERATED

{

threeGPPAccess(1),

nonThreeGPPAccess(2),

threeGPPandNonThreeGPPAccess(3)

}

Direction ::= ENUMERATED

{

fromTarget(1),

toTarget(2)

}

DNN ::= UTF8String

E164Number ::= NumericString (SIZE(1..15))

EmailAddress ::= UTF8String

FiveGGUTI ::= SEQUENCE

{

mCC         [1] MCC,

mNC         [2] MNC,

aMFRegionID [3] AMFRegionID,

aMFSetID    [4] AMFSetID,

aMFPointer  [5] AMFPointer,

fiveGTMSI   [6] FiveGTMSI

}

FiveGMMCause ::= INTEGER (0..255)

FiveGSMRequestType ::= ENUMERATED

{

initialRequest(1),

existingPDUSession(2),

initialEmergencyRequest(3),

existingEmergencyPDUSession(4),

modificationRequest(5),

reserved(6),

mAPDURequest(7)

}

FiveGSMCause ::= INTEGER (0..255)

FiveGTMSI ::= INTEGER (0..4294967295)

FTEID ::= SEQUENCE

{

tEID        [1] INTEGER (0.. 4294967295),

iPv4Address [2] IPv4Address OPTIONAL,

iPv6Address [3] IPv6Address OPTIONAL

}

GPSI ::= CHOICE

{

mSISDN      [1] MSISDN,

nAI         [2] NAI

}

GUAMI ::= SEQUENCE

{

aMFID       [1] AMFID,

pLMNID      [2] PLMNID

}

GUMMEI ::= SEQUENCE

{

mMEID       [1] MMEID,

mCC         [2] MCC,

mNC         [3] MNC

}

GUTI ::= SEQUENCE

{

mCC          [1] MCC,

mNC          [2] MNC,

mMEGroupID   [3] MMEGroupID,

mMECode      [4] MMECode,

mTMSI        [5] TMSI

}

HomeNetworkPublicKeyID ::= OCTET STRING

HSMFURI ::= UTF8String

IMEI ::= NumericString (SIZE(14))

IMEISV ::= NumericString (SIZE(16))

IMPI ::= NAI

IMPU ::= CHOICE

{

sIPURI [1] SIPURI,

tELURI [2] TELURI

}

IMSI ::= NumericString (SIZE(6..15))

Initiator ::= ENUMERATED

{

uE(1),

network(2),

unknown(3)

}

IPAddress ::= CHOICE

{

iPv4Address [1] IPv4Address,

iPv6Address [2] IPv6Address

}

IPv4Address ::= OCTET STRING (SIZE(4))

IPv6Address ::= OCTET STRING (SIZE(16))

IPv6FlowLabel ::= INTEGER(0..1048575)

MACAddress ::= OCTET STRING (SIZE(6))

MCC ::= NumericString (SIZE(3))

MNC ::= NumericString (SIZE(2..3))

MMEID ::= SEQUENCE

{

mMEGI       [1] MMEGI,

mMEC        [2] MMEC

}

MMEC ::= NumericString

MMEGI ::= NumericString

MSISDN ::= NumericString (SIZE(1..15))

NAI ::= UTF8String

NextLayerProtocol ::= INTEGER(0..255)

NonLocalID ::= ENUMERATED

{

local(1),

nonLocal(2)

}

NSSAI ::= SEQUENCE OF SNSSAI

PLMNID ::= SEQUENCE

{

mCC [1] MCC,

mNC [2] MNC

}

PDUSessionID ::= INTEGER (0..255)

PDUSessionType ::= ENUMERATED

{

iPv4(1),

iPv6(2),

iPv4v6(3),

unstructured(4),

ethernet(5)

}

PEI ::= CHOICE

{

iMEI        [1] IMEI,

iMEISV      [2] IMEISV

}

PortNumber ::= INTEGER(0..65535)

ProtectionSchemeID ::= INTEGER (0..15)

RATType ::= ENUMERATED

{

nR(1),

eUTRA(2),

wLAN(3),

virtual(4),

nBIOT(5),

wireline(6),

wirelineCable(7),

wirelineBBF(8),

lTEM(9),

nRU(10),

eUTRAU(11),

trustedN3GA(12),

trustedWLAN(13),

uTRA(14),

gERA(15)

}

RejectedNSSAI ::= SEQUENCE OF RejectedSNSSAI

RejectedSNSSAI ::= SEQUENCE

{

causeValue  [1] RejectedSliceCauseValue,

sNSSAI      [2] SNSSAI

}

RejectedSliceCauseValue ::= INTEGER (0..255)

ReRegRequiredIndicator ::= ENUMERATED

{

reRegistrationRequired(1),

reRegistrationNotRequired(2)

}

RoutingIndicator ::= INTEGER (0..9999)

SchemeOutput ::= OCTET STRING

SIPURI ::= UTF8String

Slice ::= SEQUENCE

{

allowedNSSAI        [1] NSSAI OPTIONAL,

configuredNSSAI     [2] NSSAI OPTIONAL,

rejectedNSSAI       [3] RejectedNSSAI OPTIONAL

}

SMPDUDNRequest ::= OCTET STRING

-- TS 24.501 [13], clause 9.11.3.6.1

SMSOverNASIndicator ::= ENUMERATED

{

sMSOverNASNotAllowed(1),

sMSOverNASAllowed(2)

}

SNSSAI ::= SEQUENCE

{

sliceServiceType    [1] INTEGER (0..255),

sliceDifferentiator [2] OCTET STRING (SIZE(3)) OPTIONAL

}

SUCI ::= SEQUENCE

{

mCC                         [1] MCC,

mNC                         [2] MNC,

routingIndicator            [3] RoutingIndicator,

protectionSchemeID          [4] ProtectionSchemeID,

homeNetworkPublicKeyID      [5] HomeNetworkPublicKeyID,

schemeOutput                [6] SchemeOutput

}

SUPI ::= CHOICE

{

iMSI        [1] IMSI,

nAI         [2] NAI

}

SUPIUnauthenticatedIndication ::= BOOLEAN

SwitchOffIndicator ::= ENUMERATED

{

normalDetach(1),

switchOff(2)

}

TargetIdentifier ::= CHOICE

{

sUPI                [1] SUPI,

iMSI                [2] IMSI,

pEI                 [3] PEI,

iMEI                [4] IMEI,

gPSI                [5] GPSI,

mSISDN              [6] MSISDN,

nAI                 [7] NAI,

iPv4Address         [8] IPv4Address,

iPv6Address         [9] IPv6Address,

ethernetAddress     [10] MACAddress

}

TargetIdentifierProvenance ::= ENUMERATED

{

lEAProvided(1),

observed(2),

matchedOn(3),

other(4)

}

TELURI ::= UTF8String

Timestamp ::= GeneralizedTime

UEEndpointAddress ::= CHOICE

{

iPv4Address         [1] IPv4Address,

iPv6Address         [2] IPv6Address,

ethernetAddress     [3] MACAddress

}

-- ===================

-- Location parameters

-- ===================

Location ::= SEQUENCE

{

locationInfo                [1] LocationInfo OPTIONAL,

positioningInfo             [2] PositioningInfo OPTIONAL,

locationPresenceReport      [3] LocationPresenceReport OPTIONAL,

ePSLocationInfo             [4] EPSLocationInfo OPTIONAL

}

CellSiteInformation ::= SEQUENCE

{

geographicalCoordinates     [1] GeographicalCoordinates,

azimuth                     [2] INTEGER (0..359) OPTIONAL,

operatorSpecificInformation [3] UTF8String OPTIONAL

}

-- TS 29.518 [22], clause 6.4.6.2.6

LocationInfo ::= SEQUENCE

{

userLocation                [1] UserLocation OPTIONAL,

currentLoc                  [2] BOOLEAN OPTIONAL,

geoInfo                     [3] GeographicArea OPTIONAL,

rATType                     [4] RATType OPTIONAL,

timeZone                    [5] TimeZone OPTIONAL,

additionalCellIDs           [6] SEQUENCE OF CellInformation OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.7

UserLocation ::= SEQUENCE

{

eUTRALocation               [1] EUTRALocation OPTIONAL,

nRLocation                  [2] NRLocation OPTIONAL,

n3GALocation                [3] N3GALocation OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.8

EUTRALocation ::= SEQUENCE

{

tAI                         [1] TAI,

eCGI                        [2] ECGI,

ageOfLocationInfo           [3] INTEGER OPTIONAL,

uELocationTimestamp         [4] Timestamp OPTIONAL,

geographicalInformation     [5] UTF8String OPTIONAL,

geodeticInformation         [6] UTF8String OPTIONAL,

globalNGENbID               [7] GlobalRANNodeID OPTIONAL,

cellSiteInformation         [8] CellSiteInformation OPTIONAL,

globalENbID                 [9] GlobalRANNodeID OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.9

NRLocation ::= SEQUENCE

{

tAI                         [1] TAI,

nCGI                        [2] NCGI,

ageOfLocationInfo           [3] INTEGER OPTIONAL,

uELocationTimestamp         [4] Timestamp OPTIONAL,

geographicalInformation     [5] UTF8String OPTIONAL,

geodeticInformation         [6] UTF8String OPTIONAL,

globalGNbID                 [7] GlobalRANNodeID OPTIONAL,

cellSiteInformation         [8] CellSiteInformation OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.10

N3GALocation ::= SEQUENCE

{

tAI                         [1] TAI OPTIONAL,

n3IWFID                     [2] N3IWFIDNGAP OPTIONAL,

uEIPAddr                    [3] IPAddr OPTIONAL,

portNumber                  [4] INTEGER OPTIONAL,

tNAPID                      [5] TNAPID OPTIONAL,

tWAPID                      [6] TWAPID OPTIONAL,

hFCNodeID                   [7] HFCNodeID OPTIONAL,

gLI                         [8] GLI OPTIONAL,

w5GBANLineType              [9] W5GBANLineType OPTIONAL,

gCI                         [10] GCI OPTIONAL

}

-- TS 38.413 [23], clause 9.3.2.4

IPAddr ::= SEQUENCE

{

iPv4Addr                    [1] IPv4Address OPTIONAL,

iPv6Addr                    [2] IPv6Address OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.28

GlobalRANNodeID ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

aNNodeID                    [2] ANNodeID,

nID                         [3] NID OPTIONAL

}

ANNodeID ::= CHOICE

{

n3IWFID [1] N3IWFIDSBI,

gNbID   [2] GNbID,

nGENbID [3] NGENbID,

eNbID   [4] ENbID,

wAGFID  [5] WAGFID,

tNGFID  [6] TNGFID

}

-- TS 38.413 [23], clause 9.3.1.6

GNbID ::= BIT STRING(SIZE(22..32))

-- TS 29.571 [17], clause 5.4.4.4

TAI ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

tAC                         [2] TAC,

nID                         [3] NID OPTIONAL

}

CGI ::= SEQUENCE

{

lAI    [1] LAI,

cellID [2] CellID

}

LAI ::= SEQUENCE

{

pLMNID [1] PLMNID,

lAC    [2] LAC

}

LAC ::= OCTET STRING (SIZE(2))

CellID ::= OCTET STRING (SIZE(2))

SAI ::= SEQUENCE

{

pLMNID [1] PLMNID,

lAC    [2] LAC,

sAC    [3] SAC

}

SAC ::= OCTET STRING (SIZE(2))

-- TS 29.571 [17], clause 5.4.4.5

ECGI ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

eUTRACellID                 [2] EUTRACellID,

nID                         [3] NID OPTIONAL

}

TAIList ::= SEQUENCE OF TAI

-- TS 29.571 [17], clause 5.4.4.6

NCGI ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

nRCellID                    [2] NRCellID,

nID                         [3] NID OPTIONAL

}

RANCGI ::= CHOICE

{

eCGI                        [1] ECGI,

nCGI                        [2] NCGI

}

CellInformation ::= SEQUENCE

{

rANCGI                      [1] RANCGI,

cellSiteinformation         [2] CellSiteInformation OPTIONAL,

timeOfLocation              [3] Timestamp OPTIONAL

}

-- TS 38.413 [23], clause 9.3.1.57

N3IWFIDNGAP ::= BIT STRING (SIZE(16))

-- TS 29.571 [17], clause 5.4.4.28

N3IWFIDSBI ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.28 and table 5.4.2-1

TNGFID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.28 and table 5.4.2-1

WAGFID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.62

TNAPID ::= SEQUENCE

{

sSID         [1] SSID OPTIONAL,

bSSID        [2] BSSID OPTIONAL,

civicAddress [3] CivicAddressBytes OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.64

TWAPID ::= SEQUENCE

{

sSID         [1] SSID OPTIONAL,

bSSID        [2] BSSID OPTIONAL,

civicAddress [3] CivicAddressBytes OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.62 and clause 5.4.4.64

SSID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.62 and clause 5.4.4.64

BSSID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.36 and table 5.4.2-1

HFCNodeID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.10 and table 5.4.2-1

-- Contains the original binary data i.e. value of the YAML field after base64 encoding is removed

GLI ::= OCTET STRING (SIZE(0..150))

-- TS 29.571 [17], clause 5.4.4.10 and table 5.4.2-1

GCI ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.10 and clause 5.4.3.33

W5GBANLineType ::= ENUMERATED

{

dSL(1),

pON(2)

}

-- TS 29.571 [17], table 5.4.2-1

TAC ::= OCTET STRING (SIZE(2..3))

-- TS 38.413 [23], clause 9.3.1.9

EUTRACellID ::= BIT STRING (SIZE(28))

-- TS 38.413 [23], clause 9.3.1.7

NRCellID ::= BIT STRING (SIZE(36))

-- TS 38.413 [23], clause 9.3.1.8

NGENbID ::= CHOICE

{

macroNGENbID                [1] BIT STRING (SIZE(20)),

shortMacroNGENbID           [2] BIT STRING (SIZE(18)),

longMacroNGENbID            [3] BIT STRING (SIZE(21))

}

-- TS 23.003 [19], clause 12.7.1 encoded as per TS 29.571 [17], clause 5.4.2

NID ::= UTF8String (SIZE(11))

-- TS 36.413 [38], clause 9.2.1.37

ENbID ::= CHOICE

{

macroENbID                  [1] BIT STRING (SIZE(20)),

homeENbID                   [2] BIT STRING (SIZE(28)),

shortMacroENbID             [3] BIT STRING (SIZE(18)),

longMacroENbID              [4] BIT STRING (SIZE(21))

}

-- TS 29.518 [22], clause 6.4.6.2.3

PositioningInfo ::= SEQUENCE

{

positionInfo                [1] LocationData OPTIONAL,

rawMLPResponse              [2] RawMLPResponse OPTIONAL

}

RawMLPResponse ::= CHOICE

{

-- The following parameter contains a copy of unparsed XML code of the

-- MLP response message, i.e. the entire XML document containing

-- a <slia> (described in OMA-TS-MLP-V3_5-20181211-C [20], clause 5.2.3.2.2) or

-- a <slirep> (described in OMA-TS-MLP-V3_5-20181211-C [20], clause 5.2.3.2.3) MLP message.

mLPPositionData             [1] UTF8String,

-- OMA MLP result id, defined in OMA-TS-MLP-V3_5-20181211-C [20], Clause 5.4

mLPErrorCode                [2] INTEGER (1..699)

}

-- TS 29.572 [24], clause 6.1.6.2.3

LocationData ::= SEQUENCE

{

locationEstimate            [1] GeographicArea,

accuracyFulfilmentIndicator [2] AccuracyFulfilmentIndicator OPTIONAL,

ageOfLocationEstimate       [3] AgeOfLocationEstimate OPTIONAL,

velocityEstimate            [4] VelocityEstimate OPTIONAL,

civicAddress                [5] CivicAddress OPTIONAL,

positioningDataList         [6] SET OF PositioningMethodAndUsage OPTIONAL,

gNSSPositioningDataList     [7] SET OF GNSSPositioningMethodAndUsage OPTIONAL,

eCGI                        [8] ECGI OPTIONAL,

nCGI                        [9] NCGI OPTIONAL,

altitude                    [10] Altitude OPTIONAL,

barometricPressure          [11] BarometricPressure OPTIONAL

}

-- TS 29.172 [53], table 6.2.2-2

EPSLocationInfo ::= SEQUENCE

{

locationData  [1] LocationData,

cGI           [2] CGI OPTIONAL,

sAI           [3] SAI OPTIONAL,

eSMLCCellInfo [4] ESMLCCellInfo OPTIONAL

}

-- TS 29.172 [53], clause 7.4.57

ESMLCCellInfo ::= SEQUENCE

{

eCGI          [1] ECGI,

cellPortionID [2] CellPortionID

}

-- TS 29.171 [54], clause 7.4.31

CellPortionID ::= INTEGER (0..4095)

-- TS 29.518 [22], clause 6.2.6.2.5

LocationPresenceReport ::= SEQUENCE

{

type                        [1] AMFEventType,

timestamp                   [2] Timestamp,

areaList                    [3] SET OF AMFEventArea OPTIONAL,

timeZone                    [4] TimeZone OPTIONAL,

accessTypes                 [5] SET OF AccessType OPTIONAL,

rMInfoList                  [6] SET OF RMInfo OPTIONAL,

cMInfoList                  [7] SET OF CMInfo OPTIONAL,

reachability                [8] UEReachability OPTIONAL,

location                    [9] UserLocation OPTIONAL,

additionalCellIDs           [10] SEQUENCE OF CellInformation OPTIONAL

}

-- TS 29.518 [22], clause 6.2.6.3.3

AMFEventType ::= ENUMERATED

{

locationReport(1),

presenceInAOIReport(2)

}

-- TS 29.518 [22], clause 6.2.6.2.16

AMFEventArea ::= SEQUENCE

{

presenceInfo                [1] PresenceInfo OPTIONAL,

lADNInfo                    [2] LADNInfo OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.27

PresenceInfo ::= SEQUENCE

{

presenceState               [1] PresenceState OPTIONAL,

trackingAreaList            [2] SET OF TAI OPTIONAL,

eCGIList                    [3] SET OF ECGI OPTIONAL,

nCGIList                    [4] SET OF NCGI OPTIONAL,

globalRANNodeIDList         [5] SET OF GlobalRANNodeID OPTIONAL,

globalENbIDList             [6] SET OF GlobalRANNodeID OPTIONAL

}

-- TS 29.518 [22], clause 6.2.6.2.17

LADNInfo ::= SEQUENCE

{

lADN                        [1] UTF8String,

presence                    [2] PresenceState OPTIONAL

}

-- TS 29.571 [17], clause 5.4.3.20

PresenceState ::= ENUMERATED

{

inArea(1),

outOfArea(2),

unknown(3),

inactive(4)

}

-- TS 29.518 [22], clause 6.2.6.2.8

RMInfo ::= SEQUENCE

{

rMState                     [1] RMState,

accessType                  [2] AccessType

}

-- TS 29.518 [22], clause 6.2.6.2.9

CMInfo ::= SEQUENCE

{

cMState                     [1] CMState,

accessType                  [2] AccessType

}

-- TS 29.518 [22], clause 6.2.6.3.7

UEReachability ::= ENUMERATED

{

unreachable(1),

reachable(2),

regulatoryOnly(3)

}

-- TS 29.518 [22], clause 6.2.6.3.9

RMState ::= ENUMERATED

{

registered(1),

deregistered(2)

}

-- TS 29.518 [22], clause 6.2.6.3.10

CMState ::= ENUMERATED

{

idle(1),

connected(2)

}

-- TS 29.572 [24], clause 6.1.6.2.5

GeographicArea ::= CHOICE

{

point                       [1] Point,

pointUncertaintyCircle      [2] PointUncertaintyCircle,

pointUncertaintyEllipse     [3] PointUncertaintyEllipse,

polygon                     [4] Polygon,

pointAltitude               [5] PointAltitude,

pointAltitudeUncertainty    [6] PointAltitudeUncertainty,

ellipsoidArc                [7] EllipsoidArc

}

-- TS 29.572 [24], clause 6.1.6.3.12

AccuracyFulfilmentIndicator ::= ENUMERATED

{

requestedAccuracyFulfilled(1),

requestedAccuracyNotFulfilled(2)

}

-- TS 29.572 [24], clause 6.1.6.2.17

VelocityEstimate ::= CHOICE

{

horVelocity                         [1] HorizontalVelocity,

horWithVertVelocity                 [2] HorizontalWithVerticalVelocity,

horVelocityWithUncertainty          [3] HorizontalVelocityWithUncertainty,

horWithVertVelocityAndUncertainty   [4] HorizontalWithVerticalVelocityAndUncertainty

}

-- TS 29.572 [24], clause 6.1.6.2.14

CivicAddress ::= SEQUENCE

{

country                             [1] UTF8String,

a1                                  [2] UTF8String OPTIONAL,

a2                                  [3] UTF8String OPTIONAL,

a3                                  [4] UTF8String OPTIONAL,

a4                                  [5] UTF8String OPTIONAL,

a5                                  [6] UTF8String OPTIONAL,

a6                                  [7] UTF8String OPTIONAL,

prd                                 [8] UTF8String OPTIONAL,

pod                                 [9] UTF8String OPTIONAL,

sts                                 [10] UTF8String OPTIONAL,

hno                                 [11] UTF8String OPTIONAL,

hns                                 [12] UTF8String OPTIONAL,

lmk                                 [13] UTF8String OPTIONAL,

loc                                 [14] UTF8String OPTIONAL,

nam                                 [15] UTF8String OPTIONAL,

pc                                  [16] UTF8String OPTIONAL,

bld                                 [17] UTF8String OPTIONAL,

unit                                [18] UTF8String OPTIONAL,

flr                                 [19] UTF8String OPTIONAL,

room                                [20] UTF8String OPTIONAL,

plc                                 [21] UTF8String OPTIONAL,

pcn                                 [22] UTF8String OPTIONAL,

pobox                               [23] UTF8String OPTIONAL,

addcode                             [24] UTF8String OPTIONAL,

seat                                [25] UTF8String OPTIONAL,

rd                                  [26] UTF8String OPTIONAL,

rdsec                               [27] UTF8String OPTIONAL,

rdbr                                [28] UTF8String OPTIONAL,

rdsubbr                             [29] UTF8String OPTIONAL,

prm                                 [30] UTF8String OPTIONAL,

pom                                 [31] UTF8String OPTIONAL

}

-- TS 29.571 [17], clauses 5.4.4.62 and 5.4.4.64

-- Contains the original binary data i.e. value of the YAML field after base64 encoding is removed

CivicAddressBytes ::= OCTET STRING

-- TS 29.572 [24], clause 6.1.6.2.15

PositioningMethodAndUsage ::= SEQUENCE

{

method                              [1] PositioningMethod,

mode                                [2] PositioningMode,

usage                               [3] Usage,

methodCode                          [4] MethodCode OPTIONAL

}

-- TS 29.572 [24], clause 6.1.6.2.16

GNSSPositioningMethodAndUsage ::= SEQUENCE

{

mode                                [1] PositioningMode,

gNSS                                [2] GNSSID,

usage                               [3] Usage

}

-- TS 29.572 [24], clause 6.1.6.2.6

Point ::= SEQUENCE

{

geographicalCoordinates             [1] GeographicalCoordinates

}

-- TS 29.572 [24], clause 6.1.6.2.7

PointUncertaintyCircle ::= SEQUENCE

{

geographicalCoordinates             [1] GeographicalCoordinates,

uncertainty                         [2] Uncertainty

}

-- TS 29.572 [24], clause 6.1.6.2.8

PointUncertaintyEllipse ::= SEQUENCE

{

geographicalCoordinates             [1] GeographicalCoordinates,

uncertainty                         [2] UncertaintyEllipse,

confidence                          [3] Confidence

}

-- TS 29.572 [24], clause 6.1.6.2.9

Polygon ::= SEQUENCE

{

pointList                           [1] SET SIZE (3..15) OF GeographicalCoordinates

}

-- TS 29.572 [24], clause 6.1.6.2.10

PointAltitude ::= SEQUENCE

{

point                               [1] GeographicalCoordinates,

altitude                            [2] Altitude

}

-- TS 29.572 [24], clause 6.1.6.2.11

PointAltitudeUncertainty ::= SEQUENCE

{

point                               [1] GeographicalCoordinates,

altitude                            [2] Altitude,

uncertaintyEllipse                  [3] UncertaintyEllipse,

uncertaintyAltitude                 [4] Uncertainty,

confidence                          [5] Confidence

}

-- TS 29.572 [24], clause 6.1.6.2.12

EllipsoidArc ::= SEQUENCE

{

point                               [1] GeographicalCoordinates,

innerRadius                         [2] InnerRadius,

uncertaintyRadius                   [3] Uncertainty,

offsetAngle                         [4] Angle,

includedAngle                       [5] Angle,

confidence                          [6] Confidence

}

-- TS 29.572 [24], clause 6.1.6.2.4

GeographicalCoordinates ::= SEQUENCE

{

latitude                            [1] UTF8String,

longitude                           [2] UTF8String,

mapDatumInformation                 [3] OGCURN OPTIONAL

}

-- TS 29.572 [24], clause 6.1.6.2.22

UncertaintyEllipse ::= SEQUENCE

{

semiMajor                           [1] Uncertainty,

semiMinor                           [2] Uncertainty,

orientationMajor                    [3] Orientation

}

-- TS 29.572 [24], clause 6.1.6.2.18

HorizontalVelocity ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle

}

-- TS 29.572 [24], clause 6.1.6.2.19

HorizontalWithVerticalVelocity ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle,

vSpeed                              [3] VerticalSpeed,

vDirection                          [4] VerticalDirection

}

-- TS 29.572 [24], clause 6.1.6.2.20

HorizontalVelocityWithUncertainty ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle,

uncertainty                         [3] SpeedUncertainty

}

-- TS 29.572 [24], clause 6.1.6.2.21

HorizontalWithVerticalVelocityAndUncertainty ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle,

vSpeed                              [3] VerticalSpeed,

vDirection                          [4] VerticalDirection,

hUncertainty                        [5] SpeedUncertainty,

vUncertainty                        [6] SpeedUncertainty

}

-- The following types are described in TS 29.572 [24], table 6.1.6.3.2-1

Altitude ::= UTF8String

Angle ::= INTEGER (0..360)

Uncertainty ::= INTEGER (0..127)

Orientation ::= INTEGER (0..180)

Confidence ::= INTEGER (0..100)

InnerRadius ::= INTEGER (0..65535)

AgeOfLocationEstimate ::= INTEGER (0..32767)

HorizontalSpeed ::= UTF8String

VerticalSpeed ::= UTF8String

SpeedUncertainty ::= UTF8String

BarometricPressure ::= INTEGER (30000..155000)

-- TS 29.572 [24], clause 6.1.6.3.13

VerticalDirection ::= ENUMERATED

{

upward(1),

downward(2)

}

-- TS 29.572 [24], clause 6.1.6.3.6

PositioningMethod ::= ENUMERATED

{

cellID(1),

eCID(2),

oTDOA(3),

barometricPressure(4),

wLAN(5),

bluetooth(6),

mBS(7),

motionSensor(8),

dLTDOA(9),

dLAOD(10),

multiRTT(11),

nRECID(12),

uLTDOA(13),

uLAOA(14),

networkSpecific(15)

}

-- TS 29.572 [24], clause 6.1.6.3.7

PositioningMode ::= ENUMERATED

{

uEBased(1),

uEAssisted(2),

conventional(3)

}

-- TS 29.572 [24], clause 6.1.6.3.8

GNSSID ::= ENUMERATED

{

gPS(1),

galileo(2),

sBAS(3),

modernizedGPS(4),

qZSS(5),

gLONASS(6),

bDS(7),

nAVIC(8)

}

-- TS 29.572 [24], clause 6.1.6.3.9

Usage ::= ENUMERATED

{

unsuccess(1),

successResultsNotUsed(2),

successResultsUsedToVerifyLocation(3),

successResultsUsedToGenerateLocation(4),

successMethodNotDetermined(5)

}

-- TS 29.571 [17], table 5.2.2-1

TimeZone ::= UTF8String

-- Open Geospatial Consortium URN [35]

OGCURN ::= UTF8String

-- TS 29.572 [24], clause 6.1.6.2.15

MethodCode ::= INTEGER (16..31)

END

# End of changes