3GPP SA3LI#84e-a	S3i220019

# eMeeting, 24-28 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0300 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LI_X1 (management) details |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Ericsson, NTAC) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2022-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 33.128 does not provide details on LI_X1 (management) interface between the LIPF and a triggered POI |  |  |  |  |  |  |  |  |
| Summary of change: |  | Details are proposed on which messages specified in ETSI TS 103 221-1 have to be used to implement the LI_X1 (management) interface and how they apply to a 3GPP LI architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing specification text, TS 33.128 would not have a fully specified standard solution to have full interoperability between the LIPF and a triggered CC-POI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### *** FIRST CHANGE ***

### 5.2.3	Usage for realising LI_X1 (management)

For the purposes of realising LI_X1 between the LIPF and a triggered POI, the LIPF plays the role of the “ADMF” as defined in ETSI TS 103 221-1 [7] reference model (clause 4.2), and the triggered POI plays the role of the “NE”.

The LI_X1 (management) interface is realized by means of the following set of messages specified in ETSI TS 103 221-1 [7]:

- DeactivateTask (clause 6.2.3) and DeactivateAllTasks (clause 6.2.4)

- Getting information from NE (clause 6.4)

- Reporting issue from NE (clause 6.5)

- Pings and keepalive (clause 6.6)

Note: All clauses above refer to ETSI TS 103 221-1 [7]

As specified in clauses 6.2.3.3.1 and 6.2.3.4, when triggering interception in the triggered POI, the TF shall allocate a XID for each communication session; this implies that the XID used by the TF to trigger the POI can be different from the XID used by the LIPF to provision the TF. The following is then applicable:

- Unless the XID generated by the TF is available at the LIPF by other means (e.g. LIPF received a ReportTaskIssue on a given XID by the triggered POI), in order to retrieve the XID generated by the TF before sending a GetTaskDetailsRequest message, the LIPF can send a GetAllDetailsRequest message to the triggered POI, in order to get in the response message the association between the XID allocated by the LIPF (which will be available as ProductId in the GetAllDetailsResponse message) and the XID used in the triggered POI (allocated by the TF).

- In case the LIPF sends a GetAllDetailsRequest message, the XIDs provided by the triggered POI in the GetAllDetailsResponse message shall be the ones allocated by the TF; the ProductID parameter in each TaskResponseDetails structure shall be used to include the XID generated by the LIPF and used to provision the TF.

- In case the LIPF sends a ListAllDetailsRequest message, the XIDs provided by the triggered POI in the ListAllDetailsResponse message shall be the ones allocated by the TF and so can be different from the ones allocated by the LIPF and provided to the TF. In such case XIDs allocated by the LIPF and provided to the TF will not be available in the response message.

- In case the triggered POI sends a ReportTaskIssueRequest, the message shall include the XID used at the triggered POI, i.e. the one which was allocated by the TF. As the LIPF can be not aware of the XID used at the triggered POI, upon reception of the ReportTaskIssueRequest message for a given XID, the LIPF can use the GetAllDetails to recover the XID allocated by the LIPF for the task.

The present clause assumes that the TF and the triggered POI share the same LIPF. Different scenarios are outside the scope of the present document.

### *** END OF CHANGES ***