3GPP SA3LI#84e-a	S3i220005

# eMeeting, 24-28 January 2022

3GPP TSG-SA3 Meeting #105-e 	S3-214336

e-meeting, 8 - 19 November 2021

Title:	Reply to LS on broadcast of NTN GW or gNB position

Response to:	LS S3-213818 (R1-2106332) on broadcast of NTN GW or gNB position from RAN1 and response to LS S3-213819 (S1-213211) on Reply LS on broadcast of NTN GW or gNB position from SA1

Release:	Release 17

Source:	SA3

To:	RAN1, SA1

Cc:	SA3-LI

Contact person:	Vlasios Tsiatsis

vlasios dot tsiatsis at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA3 would like to thank RAN1 for the LS on broadcast of NTN GW or gNB position (S3-213818).

SA3 would also like to thank SA1 for the Reply LS on broadcast of NTN GW or gNB position (S3-213819) for the same RAN1 LS.

SA3 would like to answer the questions included in the LS from a security perspective. For regulatory aspects SA1 has provided some information and SA3-LI may respond.

SA3 would like to remind RAN1 that broadcasting any unprotected information is subject to tampering and therefore could not be fully trusted.

From a regulatory aspect, it has to be observed that some regions may allow the publication of base stations’ locations while some regions not.  In the regions prohibiting broadcasting NTN-GW/gNB location the security and privacy concerns mentioned below apply.

RAN1 Question 1: Is there any security/regulatory aspect that needs to be taken into account if the NTN-GW/gNB position is broadcasted including any aspects related to accuracy of the position?

SA3 Answer

Broadcasting its own location information by a NTN gNB or GW will lower the barrier for attackers, because many more attackers across countries or continents can obtain the NTN-GW/gNB locations effortlessly without having to physically get close to it. The more accurate the position is, the riskier the NTN-GW/gNB’s exposure becomes.

A resourceful attacker could localize the NTN GW or the gNB using other means than the broadcast information. A gNB or NTN GW could also be in visible locations and their locations may be exposed by people on purpose or by accident. However, the situation is different from directly receiving the location information broadcast over the air interface, which is much less resource (time + cost) demanding.

RAN1 Question 2:  Is there any security/regulatory aspect that needs to be taken into account if the NTN-GW/gNB position is possible to be derived by the UE with assistance information from the network including any aspects related to accuracy of the position?

SA3 Answer

If the UE has the means to calculate the positions of NTN gNBs or GWs then the NTN gNB or GW location may not be easily accessible to a casual user but it can be accessible to a resourceful attacker. As a result the situation is not different from an attacker's point of view compared to the alternative to provide the NTN gNB or GW location information in a broadcast channel.

It can be noted that a UE in a terrestrial network may also calculate the position of the serving gNB by using assistance information from the network after deciphering, namely by means of trilateration of timing advance commands (TAC) received from the gNB. Broadcasting unprotected assistance information only lowers the barrier for an attacker. Therefore, the potential risk is comparable to that in a terrestrial network.

# 2	Actions

To TSG RAN WG1

ACTION: 	SA3 asks RAN1 to take the answers to the questions into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#106	7-11 February 2022	TBD

SA3#106-Bis	4 - 8 April 2022		TBD