3GPP SA3LI#84-e-a	S3i220027

- eMeeting, 24-28 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0305 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS LI Stage 3, Annex: Illustrations of CC-POI triggering concepts |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia, Nokia Shanghai Bell) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2022-01-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The stage 3 details of LI_T3 were not defined in the TS 33.128. The triggering interface introduces some new concepts. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An informative annex is provided with illustrative diagrams. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Triggering interface details for the CC interception won't be clarified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### *** First change ***

Annex X (informative):

Illustrations of IMS CC-POI triggering scenarios

### X.1	SDP offer in the forward message – originating session

The figure X-1 illustrates the LI_T3 trigger sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW as per the 2 options described in clause 7.12.5.X.2.1 for an IMS originating session from a target when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X-1: ActivateTask for target originated sessions (SDP offer in forward SIP)

The ActivateTask that can be sent after step 3 (according to clause 7.12.5.X.2.1) has the following:

When the media interception is performed at the core network side:

-	Target Identifiers: IP address 198.51.100.20.

-	UDP port number 10060.

-	Target Identifier Destiny: ToTarget.

When the media interception is performed at the access side:

-	Target Identifiers: IP address 198.51.100.10.

-	UDP port number 10030.

-	Target Identifier Destiny: FromTarget.

The IP Address 198.51.100.20 and the UDP port number 10060 are also present in the SDP offer sent toward the terminating end of the session.

The IP Address 198.51.100.10 and the UDP port number 10030 are also present in the SDP answer sent toward the originating end of the session (i.e. target UE).

The RTP streams received at the IP address 198.51.100.20 and UDP port 10060 are to the target (core network side media interception) and RTP streams received at the IP address 198.51.100.10 and the UDP port 10030 (access side media interception) are from the target.

### X.2	SDP offer in the response message – originating session

The figure X-2 illustrates the LI_T3 trigger sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW as per the 2 options described in clause 7.12.5.X.2.1 for an IMS originating session from a target when the SDP offer is included in a response SIP message (e.g. SIP 200 OK).

Figure X-2: ActivateTask for target originated sessions (SDP offer in response SIP)

The ActivateTask that can be sent after step 5 (according to clause 7.12.5.X.2.1) has the following:

When the media interception is performed at the core network side:

-	Target Identifiers: IP address 198.51.100.20.

-	UDP port number 10060.

-	Target Identifier Destiny: ToTarget.

When the media interception is performed at the access side:

-	Target Identifiers: IP address 198.51.100.10.

-	UDP port number 10030.

-	Target Identifier Destiny: FromTarget.

The IP Address 198.51.100.20 and the UDP port number 10060 are also present in the SDP answer sent toward the terminating end of the session.

The IP Address 198.51.100.10 and the UDP port number 10030 are also present in the SDP offer sent toward the originating end of the session (i.e. target UE).

The RTP streams received at the IP address 198.51.100.20 and UDP port 10060 (core network side media interception) are to the target and RTP streams received at the IP address 198.51.100.10 and the UDP port 10030 (access side media interception) are from the target.

### X.3	SDP offer in the forward message – terminating session

The figure X-3 illustrates the LI_T3 trigger sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW as per the 2 options described in clause 7.12.5.X.2.1 for an IMS terminating session from a target when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X-3: ActivateTask for target terminating sessions (SDP offer in forward SIP)

The ActivateTask that can be sent after step 5 (according to clause 7.12.5.X.2.1) has the following:

When the media interception is performed at the core network side:

-	Target Identifiers: IP address 203.0.113.10.

-	UDP port number 20040.

-	Target Identifier Destiny: ToTarget.

When the media interception is performed at the access side:

-	Target Identifiers: IP address 203.0.113.20.

-	UDP port number 20060.

-	Target Identifier Destiny: FromTarget.

The IP Address 203.0.113.10 and the UDP port number 20040 are also present in the SDP answer sent toward the originating end of the session.

The IP Address 203.0.113.20 and the UDP port number 20060 are also present in the SDP offer sent toward the terminating end of the session (i.e. target UE).

The RTP streams received at the IP address 203.0.113.10 and UDP port 20040 (core network side media interception) are to the target and RTP streams received at the IP address 203.0.1113.20 and the UDP port 20060 (access side media interception) are from the target.

### X.4	SDP offer in the response message – terminating session

The figure X-3 illustrates the LI_T3 trigger sent from a CC-TF present in the P-CSCF to the CC-POI present in the IMS-AGW as per the 2 options described in clause 7.12.5.X.2.1 for an IMS terminating session from a target when the SDP offer is included in a response SIP message (e.g. SIP 200 OK).

Figure X-4: ActivateTask for target terminating sessions (SDP offer in response SIP)

The ActivateTask that can be sent after step 3 (according to clause 7.12.5.X.2.1) has the following:

When the media interception is performed at the core network side:

-	Target Identifiers: IP address 203.0.113.10.

-	UDP port number 20040.

-	Target Identifier Destiny: ToTarget.

When the media interception is performed at the access side:

-	Target Identifiers: IP address 203.0.113.20.

-	UDP port number 20060.

-	Target Identifier Destiny: FromTarget.

The IP Address 203.0.113.10 and the UDP port number 20040 are also present in the SDP offer sent toward the originating end of the session.

The IP Address 203.0.113.20 and the UDP port number 20060 are also present in the SDP answer sent toward the terminating end of the session (i.e. target UE).

The RTP streams received at the IP address 203.0.113.10 and UDP port 20040 (core network side media interception) are to the target and RTP streams received at the IP address 203.0.1113.20 and the UDP port 20060 (access side media interception) are from the target.

### X.5	SDP offer in the forward message – session to target non-local ID (IP)

The figure X-5 illustrates the LI_T3 trigger sent from a CC-TF present in the IBCF to the CC-POI present in the TrGW as described in clause 7.12.5.X.2.1 for an IMS sessions to a target non-local ID (in the IP domain) when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X-5: ActivateTask for sessions to target non-local ID in IP domain (SDP offer in forward SIP)

The ActivateTask that can be sent after step 5 (according to clause 7.12.5.X.2.1) has the following:

The media interception is performed at the core network side:

-	Target Identifiers: IP address 203.0.113.10.

-	UDP port number 20040.

-	Target Identifier Destiny: ToTarget.

When the media interception is performed at the peer network side:

-	Target Identifiers: IP address 203.0.113.20.

-	UDP port number 20060.

-	Target Identifier Destiny: FromTarget.

The IP Address 203.0.113.10 and the UDP port number 20040 are also present in the SDP answer sent toward the originating end of the session.

The IP Address 203.0.113.20 and the UDP port number 20060 are also present in the SDP offer sent toward the terminating end of the session (i.e. toward the network that has the target non-local ID).

The RTP streams received at the IP address 203.0.113.10 and UDP port 20040 (core network side media interception) are to the target and RTP streams received at the IP address 203.0.1113.20 and the UDP port 20060 (peer network side media interception) are from the target.

### X.6	SDP offer in the forward message – session from target non-local ID (IP)

The figure X-6 illustrates the LI_T3 trigger sent from a CC-TF present in the IBCF to the CC-POI present in the TrGW as described in clause 7.12.5.X.2.1 for an IMS sessions from a target non-local ID (IP domain) when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X-6: ActivateTask for sessions from target non-local ID in IP domain (SDP offer in forward SIP)

The ActivateTask that can be sent after step 3 (according to clause 7.12.5.X.2.1) has the following:

The media interception is performed at the core network side:

-	Target Identifiers: IP address 198.51.100.20.

-	UDP port number 10060.

-	Target Identifier Destiny: ToTarget.

When the media interception is performed at the peer network side:

-	Target Identifiers: IP address 198.51.100.10.

-	UDP port number 10040.

-	Target Identifier Destiny: FromTarget.

The IP Address 198.51.100.20 and the UDP port number 10060 are also present in the SDP offer sent toward the terminating end of the session.

The IP Address 198.51.100.10 and the UDP port number 10040 are also present in the SDP answer sent toward the originating end of the session.

The RTP streams received at the IP address 198.51.100.20 and UDP port 10060 (core network side media interception) are to the target and RTP streams received at the IP address 198.51.100.10 and the UDP port 10040 (peer network side media interception) are from the target.

### X.7	SDP offer in the forward message – session to target non-local ID (CS)

The figure X-7 illustrates the LI_T3 trigger sent from a CC-TF present in the MGCF to the CC-POI present in the IM-MGW as described in clause 7.12.5.X.2.1 for an IMS sessions to a target non-local ID (CS domain) when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X-7: ActivateTask for sessions to target non-local ID in CS domain (SDP offer in forward SIP)

The ActivateTask that can be sent after step 5 (according to clause 7.12.5.X.2.1) has the following:

The media interception is performed at the core network side:

-	Target Identifiers: IP address 203.0.113.10.

-	UDP port number 20040.

-	Target Identifier Destiny: ToTarget.

The IP Address 203.0.113.10 and the UDP port number 20040 are also present in the SDP answer sent toward the originating end of the session.

The inter-PLMN signaling CS-based (e.g. ISUP). The details of this inter-PLMN signaling are not shown in the figure X-7.

The RTP streams received at the IP address 203.0.113.10 and UDP port 20040 are to the target.

### X.8	SDP offer in the forward message – session from target non-local ID (CS)

The figure X-8 illustrates the LI_T3 trigger sent from a CC-TF present in the MGCF to the CC-POI present in the IM-MGW as described in clause 7.12.5.X.2.1 for an IMS sessions from a target non-local ID (CS domain) when the SDP offer is included in a forward SIP message (e.g. SIP INVITE).

Figure X-8: ActivateTask for sessions from target non-local ID in CS domain (SDP offer in forward SIP)

The ActivateTask that can be sent after step 3 (according to clause 7.12.5.X.2.1) has the following:

The media interception is performed at the core network side:

-	Target Identifiers: IP address 198.51.100.10.

-	UDP port number 10060.

-	Target Identifier Destiny: ToTarget.

The IP Address 198.51.100.10 and the UDP port number 10060 are also present in the SDP offer sent toward the terminating end of the session.

The inter-PLMN signaling is CS-based (e.g. ISUP). The details of this inter-PLMN signaling are not shown in the figure X-8.

The RTP streams received at the IP address 198.51.100.10 and UDP port 10060 are to the target.

##### *** End of all changes ***