3GPP SA3LI#86-b	S3i220424

# Sophia Antipolis, France, 30 August - 2 September 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0394 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LIPF Logic Annex updates to accommodate STIR/SHAKEN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia, Nokia Shanghai Bell) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2022-08-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | LIP logic diagrams in annex G do not accommodate the STIR/SHAKEN aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Logic diagrams are modified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LIPF logic when used by the implimenters will miss the STIR/SHAKEN aspects. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.5.4.1, G.5.4.2.3, G.5.5.1, G.5.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ** First Change **

## G.5.4	LIPF logic for targets that are not non-local ID

### G.5.4.1	The flowchart

Figures G.5-3, G.5-4, G.5.5 and G.5.6 show the LIPF logic for the service type of Voice when the target is not a non-local ID.

Figure G.5-3: Top level LIPF logic for service type of Voice when target is not a non-local ID

For the delivery type of IRI + CC, the IRI-POIs and the CC-TFs are provisioned. For the delivery type of IRI, the IRI-POIs are provisioned. For the delivery type of CC, the CC-TFs are provisioned.

Figure G.5-4 shows the LIPF logic for the provisioning of IRI-POIs and figure G.5-5 shows the LIPF logic for the provisioning of CC-TFs.

Figure G.5-4: LIPF logic for delivery type of IRI for service type of Voice when target is not a non-local ID

The S-CSCF, E-CSCF, P-CSCF, IBCF, MGCF and AS (in figure G.5-4) provide IRI-POI functions under certain conditions as noted within the illustration. To prevent those IRI-POIs from providing the LI functions when not supposed to, the LIPF may have to include a parameter during the provisioning. If STIR/SHAKEN is supported in the network, the provisioning of IRI-POIs in IBCF and AS include the parameter DivPASSporTScope if the target identity is IMPU. Additional STIR/SHAKEN related provisioning is illustrated in figure G.5-7a.

Figure G.5-5: LIPF logic for delivery type of CC for service type of Voice when target is not a non-local ID

The IBCF, MGCF and AS (in figure G.5-5) provide CC-TF functions under certain conditions as noted within the illustration. To prevent those CC-TFs from triggering the CC-POIs when not supposed to, the LIPF may have to include a parameter during the provisioning.

Figure G.5-6 illustrates the LIPF logic for LALS triggering.

Figure G.5-6: LIPF logic for LALS triggering for the service type of Voice

In LALS triggering option 1, the host NF that providcs the LTF is same as the NF that provides the IRI-POI functions. And therefore, the host NFs that provide the LTF can be different based on the IMS deployment options.

The P-CSCF, IBCF and LMISF-IRI (in figure G.5-6) provide LTF under certain conditions as noted within the illustration. Under special scenario, the S-CSCF may provide the IRI-POI for emergency services (instead of E-CSCF) as specified in TS 33.127 [5]. To prevent those LTFs from providing the LI functions when not supposed to, the LIPF may have to include a parameter during the provisioning.

The figure G.5.7a illustrates the LIPF logic for additional STIR/SHAKEN related provisioning.

Figure G.5-7a: LIPF logic for additional STIR/SHAKEN related provisioning

For STIR/SHAKEN related reporting, IBCF, AS, P-CSCF (VPLMN with LBO), LMISF-IRI (VPLMN with HR) provide the IRI-POI functions. The LIPF logic shown in figure G.5-7a is additional logic required to support the STIR/SHAKEN.

As illustrated in figure G.5-4, the IRI-POIs in P-CSCF, LMISF-IRI and AS are provisioned as part IMS-based voice LI. Likewise, the IRI-POI in IBCF is also provisioned for IMS-based voice LI when the alternate option is used.

When the STIR/SHAKEN is supported in the network, the IRI-POIs in IBCF and AS are to be provisioned with DivPASSporTScope value when the target identity is IMPU.

#### G.5.4.2.3	Summary

Table G.5-1 provides the scope of NF domain that provides the IRI-POI/CC-TF/CC-POI functions for the service type of Voice with the IMS deployment option Default.

Table G.5-1: Scope of NF domain in IMS providing the LI functions with Default

| NFs with LI function |  |  | Non-roaming | Roaming with LBO |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VPLMN | HPLMN | VPLMN | HPLMN |
| HSS |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| AS (NOTE 6, NOTE 12) |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| AS (NOTE 7) |  |  | CC-TF | n/a | CC-TF | n/a | CC-TF |
| MRFP (NOTE 7) |  |  | CC-POI | n/a | CC-POI | n/a | CC-POI |
| S-CSCF (NOTE 8) |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| E-CSCF (NOTE 8) |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
| P-CSCF |  |  | n/a | IRI-POI (NOTE 1) | n/a | n/a | n/a |
| P-CSCF |  |  | CC-TF | CC-TF | n/a | CC-TF (NOTE 2) | n/a |
| IMS-AGW |  |  | CC-POI | CC-POI | n/a | CC-POI (NOTE 2) | n/a |
| MGCF (NOTE 3) |  |  | CC-TF | n/a | CC-TF | n/a | CC-TF |
| IM-MGW (NOTE 3) |  |  | CC-POI | n/a | CC-POI | n/a | CC-POI |
| IBCF (NOTE 13) |  |  | IRI-POI | IRI-POI | IRI-POI | IRI-POI | IRI-POI |
| IBCF (NOTE 4) |  |  | CC-TF | n/a | CC-TF | n/a | CC-TF |
| TrGW (NOTE 4) |  |  | CC-POI | n/a | CC-POI | n/a | CC-POI |
| LMISF-IRI (NOTE 1) |  |  | n/a | n/a | n/a | IRI-POI | n/a |
| LMISF-CC (NOTE 1) |  |  | n/a | n/a | n/a | CC-POI | n/a |
| LALS triggering | Option 1 | S-CSCF | LTF | n/a | LTF | n/a | LTF |
|  |  | E-CSCF | LTF | LTF | n/a | LTF | n/a |
|  |  | P-CSCF | n/a | LTF (NOTE 1) | n/a | n/a | n/a |
|  |  | LMISF-IRI | n/a | n/a | n/a | LTF (NOTE 1) | n/a |
|  | Option 2 | MDF2 | LTF | LTF | LTF | LTF | LTF |

Table G.5-2 provides the scope of NF domain that provides the IRI-POI/CC-TF/CC-POI functions for the service type of Voice with the IMS deployment option Alternate option.

Table G.5-2: Scope of NF domain in IMS providing the LI functions with Alternate option

| NFs with LI function |  |  | Non-roaming | Roaming with LBO |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VPLMN | HPLMN | VPLMN | HPLMN |
| HSS |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| AS (NOTE 6, NOTE 12) |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| AS (NOTE 7) |  |  | CC-TF | n/a | CC-TF | n/a | CC-TF |
| MRFP (NOTE 7) |  |  | CC-POI | n/a | CC-POI | n/a | CC-POI |
| P-CSCF |  |  | IRI-POI | IRI-POI | n/a | IRI-POI (NOTE 2) | IRI-POI |
| P-CSCF |  |  | CC-TF | CC-TF | n/a | CC-TF (NOTE 2) | CC-TF |
| IMS-AGW |  |  | CC-POI | CC-POI | n/a | CC-POI (NOTE 2) | CC-POI |
| MGCF (NOTE 3) |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| MGCF (NOTE 3) |  |  | CC-TF | n/a | CC-TF | n/a | CC-TF |
| IM-MGW (NOTE 3) |  |  | CC-POI | n/a | CC-POI | n/a | CC-POI |
| IBCF |  |  | IRI-POI (NOTE 13) | IBCF (NOTE 14) | IRI-POI (NOTE 13) | IRI-POI (NOTE 14) | IRI-POI (NOTE 13) |
| IBCF (NOTE 4) |  |  | CC-TF | n/a | CC-TF | n/a | CC-TF |
| TrGW (NOTE4) |  |  | CC-POI | n/a | CC-POI | n/a | CC-POI |
| LMISF-IRI (NOTE 1) |  |  | n/a | n/a | n/a | IRI-POI | n/a |
| LMISF-CC (NOTE 1) |  |  | n/a | n/a | n/a | CC-POI | n/a |
| LALS Triggering | Option 1 | P-CSCF | LTF | LTF | n/a | LTF (NOTE 2) | LTF |
|  |  | IBCF | n/a | n/a | LTF (NOTE 5) | n/a | n/a |
|  |  | LMISF-IRI | n/a | n/a | n/a | LTF (NOTE 1) | n/a |
|  | Option 2 | MDF2 | LTF | LTF | LTF | LTF | LTF |

NOTE 1:	For non-emergency sessions only.

NOTE 2:	For emergency sessions only.

NOTE 3:	Only when an incoming session to a target is redirected over a CS domain.

NOTE 4:	Only when target is outbound roaming or when an incoming session to a target is redirected over an IP domain, or to an outbound roaming party with LBO.

NOTE 5:	Only when the target is outbound roaming without a redirection.

NOTE 6:	When the interception of conferencing services is required

NOTE 7:	When the content interception of conferencing, or application of music/ is required.

NOTE 8:	For IMS emergency sessions in fixed networks when the S-CSCF is on the signaling path, S-CSCF may optionally (instead of E-CSCF) provide the IRI-POI functions with the default option.

NOTE 9:	The use of "n/a" in the above table implies that the LI function is not applicable to the NF for the indicated scenario.

NOTE 10:	The LIPF is not aware of the above role played by the host NFs in providing the LI functions.

NOTE 11:	MDF2, MDF3 and LI-LCS Client which are also involved in providing the LI functions are not shown in the tables above.

NOTE 12:	When STIR/SHAKEN is supported in the network.

NOTE 13:		Only when target is outbound roaming or when an incoming session to a target is redirected over an IP domain, or to an outbound roaming party with LBO, or when STIR/SHAKEN is supported in the network.

NOTE 14:	Only when the STIR/SHAKEN is supported in the network.

## G.5.5	LIPF logic for targets that are non-local ID

### G.5.5.1	The flowchart

Figures G.5-7, G.5-8 and G.5.9 show the LIPF logic for the service type of Voice when the target is a non-local ID.

Figure G.5-7: LIPF logic for service type of Voice when target is a non-local ID

For the delivery type of IRI + CC, the IRI-POIs and the CC-TFs are provisioned. For the delivery type of IRI, the IRI-POIs are provisioned. For the delivery type of CC, the CC-TFs are provisioned.

Figure G.5-8 shows the LIPF logic for the provisioning of IRI-POIs and figure G.5-9 shows the LIPF logic for the provisioning of CC-TFs.

Figure G.5-8: LIPF logic for delivery type of IRI for service type of Voice when target is a non-local ID

The P-CSCF, IBCF and MGCF (in figure G.5-8) provide IRI-POI functions under certain conditions as noted within the illustration. To prevent those IRI-POIs from providing the LI functions when not supposed to, the LIPF may have to include a parameter during the provisioning.

If STIR/SHAKEN is supported in the network, the provisioning of IRI-POI in IBCF include the parameter DivPASSporTScope. Additional STIR/SHAKEN related provisioning for target non-local ID is illustrated in figure G.5-9b.

Figure G.5-9: LIPF logic for delivery type of CC for service type of Voice when target is a non-local ID

The P-CSCF, IBCF and MGCF (in figure G.5-9) provide CC-TF functions under certain conditions as noted within the illustration. To prevent those CC-TFs from triggering the CC-POI when not supposed to, the LIPF may have to include a parameter during the provisioning.

The figure G.5.9a illustrates the LIPF logic for additional STIR/SHAKEN related provisioning for target non-local ID.

Figure G.5-9a: LIPF logic for additional STIR/SHAKEN related provisioning for target non-local ID

For STIR/SHAKEN related reporting IBCF, AS, P-CSCF (VPLMN with LBO), LMISF-IRI-POI (VPLMN with HR) provide the IRI-POI functions. The LIPF logic shown in figure G.5-9a is additional logic required to support the STIR/SHAKEN when the target is non-local ID.

As illustrated in figure G.5-8, the IRI-POIs in P-CSCF, LMISF-IRI are provisioned as part IMS-based voice LI. Likewise, the IRI-POI in IBCF is also provisioned for IMS-based voice LI when default option is used in HPLMN, or when alternate options are used in both HPLMN and VPLMN.

When the STIR/SHAKEN is supported in the network, the IRI-POIs in IBCF and AS are to be provisioned with DivPASSporTScope value.

#### G.5.5.2.2	Summary

Table G.5-3 provides the scope of NF domain that provides the IRI-POI/CC-TF/CC-POI functions for the service type of Voice with the IMS deployment option Default in HPLMN.

Table G.5-3: Scope of NF domain in IMS providing the LI functions with Default in HPLMN

| NF with LI function | Non-roaming | Roaming with LBO |  |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | VPLMN |  | HPLMN | VPLMN | HPLMN |
|  |  | Default | Alternate |  |  |  |
| P-CSCF | n/a | IRI-POI | n/a | n/a | n/a | n/a |
| P-CSCF | n/a | CC-TF | n/a | n/a | n/a | n/a |
| IMS-AGW | n/a | CC-POI | n/a | n/a | n/a | n/a |
| AS (NOTE 5) | IRI-POI | n/a | n/a | IRI-POI | n/a | IRI-POI |
| MGCF | IRI-POI | n/a | n/a | IRI-POI | n/a | IRI-POI |
| MGCF | CC-TF | n/a | n/a | CC-TF | n/a | CC-TF |
| IM-MGW | CC-POI | n/a | n/a | CC-POI | n/a | CC-POI |
| IBCF | IRI-POI | IRI-POI (NOTE 5) | IRI-POI | IRI-POI | IRI-POI (NOTE5) | IRI-POI |
| IBCF | CC-TF | n/a | CC-TF | CC-TF | n/a | CC-TF |
| TrGW | CC-POI | n/a | CC-POI | CC-POI | n/a | CC-POI |
| LMISF-IRI | n/a | n/a | n/a | n/a | IRI-POI | n/a |
| LMISF-CC | n/a | n/a | n/a | n/a | CC-POI | n/a |

Table G.5-4 provides the scope of NF domain that provides the IRI-POI/CC-TF/CC-POI functions for the service type of Voice with the IMS deployment option Alternate option in HPLMN.

Table G.5-4: Scope of NF domain in IMS providing the LI functions with Alternate option in HPLMN

| NF with LI function | Non-roaming | Roaming with LBO |  |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | VPLMN |  | HPLMN | VPLMN | HPLMN |
|  |  | Default | Alternate |  |  |  |
| P-CSCF | n/a | IRI-POI | n/a | n/a | n/a | n/a |
| P-CSCF | n/a | CC-TF | n/a | n/a | n/a | n/a |
| IMS-AGW | n/a | CC-POI | n/a | n/a | n/a | n/a |
| AS (NOTE 5) | IRI-POI | n/a | n/a | IRI-POI | n.a | IRI-POI |
| S-CSCF | IRI-POI | n/a | n/a | IRI-POI | n/a | IRI-POI |
| MGCF | CC-TF | n/a | n/a | CC-TF | n/a | CC-TF |
| IM-MGW | CC-POI | n/a | n/a | CC-POI | n/a | CC-POI |
| IBCF (NOTE 5) | IRI-POI | IRI-POI | IRI-POI | IRI-POI | IBCF | I-IRI-POI |
| IBCF | CC-TF | n/a | CC-TF | CC-TF | n/a | CC-TF |
| TrGW | CC-POI | n/a | CC-POI | CC-POI | n/a | CC-POI |
| LMISF-IRI | n/a | n/a | n/a | n/a | IRI-POI | n/a |
| LMISF-CC | n/a | n/a | n/a | n/a | CC-POI | n/a |

NOTE 1:	In tables G.5-3 and G.5-4, the use of the phrase non-roaming/roaming applies to the party communicating with the target non-local ID.

NOTE 2:	The use of "n/a" in the above table implies that the LI function is not applicable to the NF for the indicated scenario.

NOTE 3:	The LIPF is not aware of the above role played by the host NFs in providing the LI functions.

NOTE 4:	MDF2, MDF3 and LI-LCS Client which are also involved in providing the LI functions are not shown in the tables above.

NOTE 5:	Only when the STIR/SHAKEN is supported in the network.

### ** End of all Changes **