3GPP SA3LI#87b	S3i220643

# 01-04 November 2022, San Antonio (US)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0456 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LI_X1 (Management) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Verizon Switzerland, SS8) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2022-10-24 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | LI_X1 (Management) interface is not sufficiently defined in TS33.128 to allow the LIPF to assess LI states of the triggered POIs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define LI_X1 (Management) interface requirements to allow the LIPF to assess LI states of the triggered POIs and support corrective action if necessary. A second approach using the LI_X1 interface to the TF is also offered as an option. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation of LI_X1 (Management) is not standardized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### START OF CHANGE

### 5.2.3 Usage for realising LI_X1 (management)

For the purposes of realising LI_X1 (management) interface between the LIPF and a triggered POI, the LIPF plays the role of the “ADMF” as defined in ETSI TS 103 221-1 [7] reference model (clause 4.2), and the triggered POI plays the role of the “NE”.

To enable the LI ADMF/LIPF to assess the integrity of the LI states of the triggered POIs and to take or support corrective actions if necessary, two approaches can be implemented.

Approach 1:

The utilization of LI_X1 (Management) interface

The LI_X1 (management) interface is realized by means of the following set of messages specified in ETSI TS 103 221-1 [7]:

- DeactivateTask (clause 6.2.3)

- DeactivateAllTasks (clause 6.2.4)

- CreateDestination (clause 6.3.1)

- ModifyDestination (clause 6.3.2)

- RemoveDestination (clause 6.3.3)

- RemoveAllDestinations (clause 6.3.4)

- GetTaskDetails (clause 6.4.2)

- GetDestinationDetails (clause 6.4.3)

- GetNEStatus (clause 6.4.4)

- GetAllDetails (clause 6.4.5)

- ListAllDetails (clause 6.4.6)

- ReportTaskIssue (clause 6.5.2)

- ReportDestinationIssue (clause 6.5.3)

- ReportNEIssue (clause 6.5.4)

- Pings (clause 6.6.1)

- Keepalive (clause 6.6.2)

Note: As specified in clauses 6.2.3.3.1 and 6.2.3.4, when triggering interception in the triggered POI, the TF shall allocate a XID for each communication session; this implies that the XID created by the TF to trigger the POI can be different from the XID used by the LIPF to provision the TF. In such cases, the TF shall set the Product ID field in any ActivateTask or ModifyTask message issued to a triggered POI (see ETSI TS 103 221-1 [7] clause 6.2.1.2). The TF shall set the Product ID to the XID of the Task object associated with the interception at the POI in order to allow correlation of LI product at the MDF. The LIPF may need to query the LI state information in the TF to determine the XID to ProductID relationship.

If “orphaned” tasks are detected, the LIPF can, optionally, automatically send the DeactivateTask message to the triggered POI using the LI_X1 (management) interface to remove the unauthorized tasks, or raise an alert for human intervention.

If “unauthorized” destinations are detected – an indication of potential issues such as mis-provisioning or security breach, the LIPF can, optionally, automatically send the RemoveDestination message to the triggered POI using the LI_X1 (management) interface to remove them or raise an alert for human intervention.

Approach 2:

The utilization of LI_X1 interface to the TF

The TF shall support all LI ADMF messages as defined in ETSI TS 103 221-1 to allow management responsibilities such as provisioning, auditing and correcting issues (such as removing unauthorized destinations and XIDs, etc.) of its triggered POIs via LI-T2/T3.

The following tasks shall be supported:

- ActivateTask (clause 6.2.1)

- ModifyTask (clause 6.2.2)

- DeactivateTask (clause 6.2.3)

- DeactivateAllTasks (clause 6.2.4)

- CreateDestination (clause 6.3.1)

- ModifyDestination (clause 6.3.2)

- RemoveDestination (clause 6.3.3)

- RemoveAllDestinations (clause 6.3.4)

- GetTaskDetails (clause 6.4.2)

- GetDestinationDetails (clause 6.4.3)

- GetNEStatus (clause 6.4.4)

- GetAllDetails (clause 6.4.5)

- ListAllDetails (clause 6.4.6)

- ReportTaskIssue (clause 6.5.2)

- ReportDestinationIssue (clause 6.5.3)

- ReportNEIssue (clause 6.5.4)

- Pings (clause 6.6.1)

- Keepalive (clause 6.6.2)

The TF shall perform periodic (period configurable by CSP) audits of its triggered POIs to ensure all triggered tasks are still valid and active, and optionally take corrective actions if discrepancies (e.g., orphaned tasks, unauthorized destinations) are detected.

If issues (ReportTaskIssue, ReportDestinationIssue, ReportNEIssue) are reported by the NE, the TF can either correct them automatically, if enabled, or optionally report them to the LIPF via LI_X1.

END OF CHANGE