3GPP SA3LI#87b	S3i220616

# 01-04 November 2022, San Antonio (US)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0188 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on functional requirements for LI for NIDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Ericsson) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2022-10-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current version of the TS indicates in clause 7.8.2.1.3 that, in order to support LI for NIDD in 5GS, in roaming scenarios the LIPF present in the ADMF provisions the IRI-POI present in the AMF. This is not in line with other requirements which state that the V-SMF shall provide IRI-POI and CC-POI to support LI for NIDD in roaming scenarios. Moreover, in clause 7.10.1.3.2, it is stated that the PDN connection for NIDD using PtP SGi tunnel is established between UE and PGW via V-MME and V-SMF in the visited network. In this scenario the tunnel is established via V-SGW and not via V-SMF, as correcetly indicated in the remaing applicable normative text and figure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | AMF is replaced by V-SMF in the affected clause. V-SMF is replaced by V-SGW in the affected clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong functional requirements, mismatches inside the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.8.2.1.3; 7.10.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** FIRST CHANGE ***

##### 7.8.2.1.3	Target identifiers

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the V-SMF:

-	SUPI.

-	PEI.

-	GPSI.

The interception performed on the above three identities are mutually independent, even though, an xIRI may contain the information about the other identities when available.

*** NEXT CHANGE ***

##### 7.10.1.3.2	Delivery using a PtP SGi tunnel

In roaming scenario, the PDN connection for NIDD using PtP SGi tunnel is established between UE and PGW via V-MME and V-SGW in the visited network and PGW in the home network. The user traffic is exchanged with DoNAS between UE and V-MME, over S11 interface between V-MME and V-SGW, over S8 interface between V-SGW and home PGW and finally over a PtP SGi tunnel between PGW and SCS/AS (figure 7.10-4).

Figure 7.10-4: EPS Architecture for NIDD using a PtP SGi tunnel in roaming situation

*** END OF CHANGES ***