3GPP SA3LI#87b	S3i220651

# 01-04 November 2022, San Antonio (US)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0187 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on functional requirements for LI for NIDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Ericsson, Nokia, Nokia Shanghai Bell, Ministère Economie et Finances ) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current version of the TS indicates in clause 7.8.2.1.3 that, in order to support LI for NIDD in 5GS, in roaming scenarios the LIPF present in the ADMF provisions the IRI-POI present in the AMF. This is not in line with other requirements which state that the V-SMF shall provide IRI-POI and CC-POI to support LI for NIDD in roaming scenarios. Moreover, in clause 7.10.1.3.2, it is stated that the PDN connection for NIDD using PtP SGi tunnel is established between UE and PGW via V-MME and V-SMF in the visited network. In this scenario the tunnel is established via SGW and not via V-SMF, as correcetly indicated in the remaing applicable normative text and figure. Finally, in the normative text, and in figure 7.10-3, nodes/network functions which are only in the visited network are wrongly indicated (e.g. V-MME, V-SGW). |  |  |  |  |  |  |  |  |
| Summary of change: |  | AMF is replaced by V-SMF in the affected clause. V-SMF is replaced by SGW in the affected clause. V-MME and V-SGW are replaced by MME and SGW. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong functional requirements, mismatches inside the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.8.2.1.3; 7.10.3.1; 7.10.1.3.2; 7.10.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i220615 |  |  |  |  |  |  |  |  |

*** FIRST CHANGE ***

##### 7.8.2.1.3	Target identifiers

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the V-SMF:

-	SUPI.

-	PEI.

-	GPSI.

The interception performed on the above three identities are mutually independent, even though, an xIRI may contain the information about the other identities when available.

*** NEXT CHANGE ***

##### 7.10.1.3.1	Delivery using SCEF

In roaming scenario, the PDN connection for NIDD using SCEF is established between UE and SCEF via MME and IWK-SCEF in the visited network and SCEF in the home network. The user traffic is exchanged with DoNAS between UE and MME, over T6a interface between MME and IWK-SCEF, over T7a interface between IWK-SCEF and SCEF and finally over T8 interface between SCEF and SCS/AS. Figure 7.10-3 shows the architecture for delivery of NIDD using SCEF in roaming situation.

Figure 7.10-3: EPS Architecture for NIDD using SCEF in roaming situation

*** NEXT CHANGE ***

##### 7.10.1.3.2	Delivery using a PtP SGi tunnel

In roaming scenario, the PDN connection for NIDD using PtP SGi tunnel is established between UE and PGW via MME and SGW in the visited network and PGW in the home network. The user traffic is exchanged with DoNAS between UE and MME, over S11 interface between MME and SGW, over S8 interface between SGW and PGW and finally over a PtP SGi tunnel between PGW and SCS/AS (figure 7.10-4).

Figure 7.10-4: EPS Architecture for NIDD using a PtP SGi tunnel in roaming situation

*** NEXT CHANGE ***

#### 7.10.2.2	LI for NIDD using a PtP SGi tunnel

In non-roaming scenario, the PGW provides an IRI-POI and a CC-POI. Although SGW provides an IRI POI and a CC-POI for IP and Ethernet-based PDN connections, PGW terminates the PtP SGI tunnel with the SCS/AS and has the same capabilities as SCEF for NIDD.

In roaming scenario, SGW in the visited network and PGW in the home network shall provide the IRI-POI and CC-POI functions as shown in figure 6.3-2 which also concerns IRI-POI and CC-POI functions for IP and Ethernet-based PDN connections.

NOTE: 	Only home-routed mode applies for NIDD using a PtP SGi tunnel.

The LI architecture for NIDD using a PtP SGi tunnel is the same as presented in figure 6.3-2.

However, the user plane packets between UE and PGW flow through the MME and SGW as shown in figures 7.10-2 and 7.10-4.

The same xIRIs as specified in clause 6.3.3.3 for PDN connections of IP or Ethernet type and the same xCC are also considered for PDN connections for NIDD using a PtP SGi tunnel, considering unstructured payload format.

*** END OF CHANGES ***