3GPP TSG-SA3 Meeting #87-LI-e-b	s3i220659

# San Antonio, US, 01-04 Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 438 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for LI_HI1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (NTAC) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2022-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Additional details for LI_HI1 required |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of details for LI_HI1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Details for LI_HI1 will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 5.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: Merge Request: 114 Commit hash: 3ebc6f11bc2308ab0f12458b93e6a2faf545f8e6 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i220610 |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 5.4	Protocols for LI_HI1

### 5.4.1	General

Functions having an LI_HI1 interface shall support the use of ETSI TS 103 120 [6] to realise the interface.

In the event of a conflict between ETSI TS 103 120 [6] and the present document, the terms of the present document shall apply.

The representation of tasking requests shall be as specified in the present clause.

Each request to intercept a particular identifier shall be represented as an LITaskObject (see ETSI TS 103 120 [6] clause 8.2). Table 5.4.1-1 shows the minimum details required for the LITaskObject to be valid.

Table 5.4.1-1: LITaskObject details

| ETSI TS 103 120 [6] field name | Description | M/C/O |
| --- | --- | --- |
| Reference | Set to the LIID associated with the interception. | M |
| DesiredStatus | Set to "Active" to indicate that LI should commence. | M |
| TimeSpan | At a minimum, EndTime shall be set. | M |
| TargetIdentifier | See table 5.4.1-2. | M |
| DeliveryType | Set to the appropriate delivery type (IRI, CC or both). | M |
| DeliveryDetails | Shall include at least one appropriate LI delivery destination. | M |

Table 5.4.1-2: LITaskObject TargetIdentifier details

| ETSI TS 103 120 [6] field name | Description | M/C/O |
| --- | --- | --- |
| TargetIdentifierValues | Shall contain at least one valid target identifier. | M |
| ServiceType | If used, set to the appropriate service scoping dictionary value as defined in clause 5.4.2. | O |

### 5.4.2	Service scoping

Functions having an LI_HI1 interface (i.e. the ADMF) shall be able to receive the service scoping as applicable to the warrant from the LEA over the LI_HI1 interface (see clause 4.4).

Where TS 103 120 [6] is used to realise LI_HI1, and where the details in clause 5.4.1 apply, the ServiceType field of the TargetIdentifier in a given LITaskObject shall be used to identify the appropriate service scoping. For each service scoping type defined in clause 4.4.2 that is required, the appropriate dictionary entry defined in Table 5.4.2-1 below shall be included in the ServiceType field. If no service type is required to be provisioned, the ServiceType field shall be omitted.

Table 5.4.2-1: ServiceType Dictionary

| Dictionary Owner | Dictionary Name |
| --- | --- |
| 3GPP | ServiceType |
| Defined DictionaryEntries |  |
| Value | Meaning |
| Voice | Service scoping shall include the Voice service type as given in clause 4.4.2. |
| Data | Service scoping shall include the Data service type as given in clause 4.4.2. |
| Messaging | Service scoping shall include the Messaging service type as given in clause 4.4.2. |
| PTC | Service scoping shall include the Push-to-Talk service type as given in clause 4.4.2. |
| LALS | Service scoping shall include the LALS service type as given in clause 4.4.2. |
| RCS | Service scoping shall include the RCS service type as given in clause 4.4.2. |

### 5.4.3	Location acquisition

When required for location acquisition, the warrant sent over the LI_HI1 interface will specify the delivery method using task flags populated as shown in table 5.4.3-1. If the delivery method is the delivery via MDF2, the LIPF shall ensure that the MDF2 (clause 7.3.5.6.1) is provisioned and the destination endpoints in the IRI-POIs are created. Subsequently, the LAF will use this information while processing location acquisition requests received over the LI_HILA interface.

Table 5.4.3-1: LATaskFlag Dictionary for LI_HI1

| Dictionary Owner | Dictionary Name |
| --- | --- |
| 3GPP | LATaskFlag |
| Defined DictionaryEntries |  |
| Value | Meaning |
| HILADelivery | The location information shall be delivered via the LI_HILA interface. |
| HI2Delivery | The location information shall be delivered via the LI_HI2 interface. |

END OF CHANGES