# EXAMPLE CR FORM DEMONSTRATING THE PROPOSED PROCEDURES FOR UPDATING ATTACHMENTS

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0412 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Document compatibility issues with PTC and NIDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Softel Systems |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LI16 |  |  |  |  |  | Date: |  |  | 2022-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The ASN.1 tag numbers for TS33128Payloads.CCPDU fields differ in purpose between Rel-16 and Rel-17 (and newer), causing a conflict between PTC and NIDD. The types PTCCCPDU and NIDDCCPDU are both OCTET STRING, so a Rel-17 ASN.1 decoder should decode the BER structure correctly. { removed for brevity } Related CRs for Rel-17 and Rel-18 add comments to note the change so that implementors can decide to decode tag 4 appropriately based on the cCPayloadOID of the encapsulating CCPayload. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add comments to the ASN.1 describing the inter-release compatibility issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Decoders using Rel-17 or newer may misinterpret the meaning of PTC CC PDUs encoded in Rel-16 as NIDD CC PDUs without it being obvious as to why that's the case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: Merge request: !93 Commit hash: 39a6e57a85703b6922d9b13f416991fd1f500acd |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE  (attachments)

diff --git a/33128/r16/TS33128Payloads.asn b/33128/r16/TS33128Payloads.asn

index 6576c098961d79b55661c2e2f7236a7aea5b0869..05bdfbd269949531a2a9f7fe0024b7089a76c6a9 100644

--- a/33128/r16/TS33128Payloads.asn

+++ b/33128/r16/TS33128Payloads.asn

@@ -254,7 +254,15 @@ CCPDU ::= CHOICE

uPFCCPDU            [1] UPFCCPDU,

extendedUPFCCPDU    [2] ExtendedUPFCCPDU,

mMSCCPDU            [3] MMSCCPDU,

+

+    -- In Rel-17 and newer (threeGPP(4) ts33128(19) r17(17) version1(1)),

+    -- tag 4 is nIDDCCPDU and tag 5 is pTCCCPDU.

+    -- Rel-16 decoders should not decode tag 4 contents as PTCCCPDU if

+    -- r17 or newer is used in cCPayloadOID.

pTCCCPDU            [4] PTCCCPDU

+

+    -- Tag 5 is reserved for pTCCCPDU in Rel-17 and newer.

+

}

-- ===========================

END OF CHANGES