3GPP TSG-SA3 Meeting #88-LI-e-a	s3i230025

# Online,  23rd Jan 2023 - 27th Jan 2023

Title:	Overall description of AF session with QoS

Release:	18

Work Item:	LI18

Source:	SA3LI (Ministère de l’économie et des finances)

# 1	Call Flow

A 3rd party AF may request that a data session to a UE that is served by the 3rd party service provider (AF session) is set up with a specific QoS. This functionality is exposed via the NEF towards the AF.

The AF can request the network to provide QoS for the AF session based on the application and service requirements.

When the NEF receives the request from the AF to provide QoS for an AF session, the NEF on its turn transfers the request to provide QoS for an AF session to the PCF.

If the NEF gets informed about user plane events, e.g., transmission resources are released/lost, the NEF shall inform the AF about the event.

# 2	Next steps

The goal of the 2 CRs on TS 33.127 and TS 33.128, are related to the case of third party service providers represented by AFs which provide multimedia services to a target UE having a subscription with a given network operator. The external AF will invoke the “AF session with QoS” service exposed by the NEF of the network operator to request a given QoS for the multimedia session between the AF and the UE. This framework may be typically used for streaming services.

The xIRIS being considered are generated by the IRI-POI in the NEF and provide information about the AF identity, the flow description and the required QoS for that flow by the AF, and user plane events, e.g., QoS monitoring reports, usage threshold (traffic volume reaching a given threshold specified by the AF) and accumulated usage for the flow associated to that QoS.

The xIRIs are:

- The AF session with QoS provision xIRI which is generated when the IRI-POI present in the NEF detects that a request to reserve/update/revoke QoS for an AF session associated with the target UE has been received from an AF.

- The AF session with QoS notification xIRI which is generated when the IRI-POI present in the NEF detects that the NEF notifies the AF about user plane events of an AF session associated with the target UE.

Nota Bene

Same concepts are proposed in case of AS session with QoS, based on SCEF