3GPP TSG-SA3 Meeting #88-LI-e-a	s3i230041

# Online, , 23rd Jan 2023 - 27th Jan 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0473 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RATType to AMF Registration Record |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2023-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RATType is errantly missing from the AMF Registration Record. If LEAs are not authorized location, they should still have RAT Type included in LI Product. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added RAT Type to AMF Registration record. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LEAs will not receive RAT Type in the AMF Registration Record if Location is not authorized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.2.2, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: Merge Request: !141 Commit Hash: dfdbbbc5670b464376c5a805b16b15c78c2e44cb This CR is associated with a corresponding mirror CR, s3i230042 (CR0474) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

START OF FIRST CHANGE

##### 6.2.2.2.2	Registration

The IRI-POI in the AMF shall generate an xIRI containing an AMFRegistration record when the IRI-POI present in the AMF detects that a UE matching one of the target identifiers provided via LI_X1 has successfully registered to the 5GS via 3GPP NG-RAN or non-3GPP access. Accordingly, the IRI-POI in the AMF generates the xIRI when the following event is detected:

-	AMF sends a N1: REGISTRATION ACCEPT message to the target UE and the UE 5G Mobility Management (5GMM) state for the access type (3GPP NG-RAN or non-3GPP access) within the AMF is changed to 5GMM-REGISTERED.

Table 6.2.2-1: Payload for AMFRegistration record

| Field name | Description | M/C/O |
| --- | --- | --- |
| registrationType | Specifies the type of registration, see TS 24.501 [13] clause 9.11.3.7. This is derived from the information received from the UE in the REGISTRATION REQUEST message. | M |
| registrationResult | Specifies the result of registration, see TS 24.501 [13] clause 9.11.3.6. | M |
| slice | Provide, if available, one or more of the following: - allowed NSSAI (see TS 24.501 [13] clause 9.11.3.37). - configured NSSAI (see TS 24.501 [13] clause 9.11.3.37). - rejected NSSAI (see TS 24.501 [13] clause 9.11.3.46). This is derived from the information sent to the UE in the REGISTRATION ACCEPT message. | C |
| sUPI | SUPI associated with the registration (see clause 6.2.2.4). | M |
| sUCI | SUCI used in the registration, if available. | C |
| pEI | PEI provided by the UE during the registration, if available. | C |
| gPSI | GPSI obtained in the registration, if available as part of the subscription profile. | C |
| gUTI | 5G-GUTI provided as outcome of initial registration or used in other cases, see TS 24.501 [13] clause 5.5.1.2.2. | M |
| location | Location information determined by the network during the registration, if available. Encoded as a userLocation parameter (location>locationInfo>userLocation) and, when Dual Connectivity is activated, as an additionalCellIDs parameter (location>locationInfo>additionalCellIDs), see Annex A. | C |
| non3GPPAccessEndpoint | UE's local IP address used to reach the N3IWF, TNGF or TWIF, if available. IP addresses are given as 4 octets (for IPv4) or 16 octets (for IPv6) with the most significant octet first (network byte order). | C |
| fiveGSTAIList | List of tracking areas associated with the registration area within which the UE is current registered, see TS 24.501 [13] clause 9.11.3.9 (see NOTE) | C |
| sMSoverNASIndicator | Indicates whether SMS over NAS is supported. Provide, if included in registrationResult, see TS 24.501 [13] clause 9.11.3.6. | C |
| oldGUTI | GUTI or 5G-GUTI, if provided in the REGISTRATION REQUEST message, see TS 24.501 [13] clause 5.5.1.2.2. | C |
| eMM5GRegStatus | UE Status, if provided in the REGISTRATION REQUEST message, see TS 24.501 [13] clause 9.11.3.56. | C |
| nonIMEISVPEI | MACAddress or EUI-64 used as UE equipment identity if IMEI or IMEISV based PEI is not available. Provide if known, see TS 24.501 [13] clause 8.2.26.4. | C |
| mACRestIndicator | Indicates whether the non-IMEISV PEI MACAddress can be used as an equipment identifier. Required if non-IMEISVPEI is used, see TS 24.501 [13] clause 9.11.3.4. | C |
| pagingRestrictionIndicator | Indicates if paging is restricted or the type of paging allowed. Include if sent in the REGISTRATION REQUEST message. Encoded per TS 24.501 [13] clause 9.11.3.77.2, omitting the first two octets. | C |
| rATType | RAT Type shall be present if known by the AMF. RAT Type is determined by the AMF during registration. See TS 23.501 [2] clause 5.3.2.3 | C |
| NOTE: List shall be included each time there is a change to the registration area. |  |  |

END OF MAIN DOCUMENT CHANGES

START OF CHANGES TO ATTACHMET TS33128PAYLOADS.ASN

Annex A (normative):
ASN.1 Schema for the Internal and External Interfaces

TS33128Payloads

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) threeGPP(4) ts33128(19) r17(17) version6(6)}

DEFINITIONS IMPLICIT TAGS EXTENSIBILITY IMPLIED ::=

BEGIN

-- =============

-- Relative OIDs

-- =============

tS33128PayloadsOID          RELATIVE-OID ::= {threeGPP(4) ts33128(19) r17(17) version6(6)}

xIRIPayloadOID              RELATIVE-OID ::= {tS33128PayloadsOID xIRI(1)}

xCCPayloadOID               RELATIVE-OID ::= {tS33128PayloadsOID xCC(2)}

iRIPayloadOID               RELATIVE-OID ::= {tS33128PayloadsOID iRI(3)}

cCPayloadOID                RELATIVE-OID ::= {tS33128PayloadsOID cC(4)}

lINotificationPayloadOID    RELATIVE-OID ::= {tS33128PayloadsOID lINotification(5)}

-- ===============

-- X2 xIRI payload

-- ===============

XIRIPayload ::= SEQUENCE

{

xIRIPayloadOID      [1] RELATIVE-OID,

event               [2] XIRIEvent

}

XIRIEvent ::= CHOICE

{

-- Access and mobility related events, see clause 6.2.2

registration                                        [1] AMFRegistration,

deregistration                                      [2] AMFDeregistration,

locationUpdate                                      [3] AMFLocationUpdate,

startOfInterceptionWithRegisteredUE                 [4] AMFStartOfInterceptionWithRegisteredUE,

unsuccessfulAMProcedure                             [5] AMFUnsuccessfulProcedure,

-- PDU session-related events, see clause 6.2.3

pDUSessionEstablishment                             [6] SMFPDUSessionEstablishment,

pDUSessionModification                              [7] SMFPDUSessionModification,

pDUSessionRelease                                   [8] SMFPDUSessionRelease,

startOfInterceptionWithEstablishedPDUSession        [9] SMFStartOfInterceptionWithEstablishedPDUSession,

unsuccessfulSMProcedure                             [10] SMFUnsuccessfulProcedure,

-- Subscriber-management related events, see clause 7.2.2

servingSystemMessage                                [11] UDMServingSystemMessage,

-- SMS-related events, see clause 6.2.5, see also sMSReport ([56] below)

sMSMessage                                          [12] SMSMessage,

-- LALS-related events, see clause 7.3.1

lALSReport                                          [13] LALSReport,

-- PDHR/PDSR-related events, see clause 6.2.3.4.1

pDHeaderReport                                      [14] PDHeaderReport,

pDSummaryReport                                     [15] PDSummaryReport,

-- tag 16 is reserved because there is no equivalent mDFCellSiteReport in XIRIEvent

-- MMS-related events, see clause 7.4.2

mMSSend                                             [17] MMSSend,

mMSSendByNonLocalTarget                             [18] MMSSendByNonLocalTarget,

mMSNotification                                     [19] MMSNotification,

mMSSendToNonLocalTarget                             [20] MMSSendToNonLocalTarget,

mMSNotificationResponse                             [21] MMSNotificationResponse,

mMSRetrieval                                        [22] MMSRetrieval,

mMSDeliveryAck                                      [23] MMSDeliveryAck,

mMSForward                                          [24] MMSForward,

mMSDeleteFromRelay                                  [25] MMSDeleteFromRelay,

mMSDeliveryReport                                   [26] MMSDeliveryReport,

mMSDeliveryReportNonLocalTarget                     [27] MMSDeliveryReportNonLocalTarget,

mMSReadReport                                       [28] MMSReadReport,

mMSReadReportNonLocalTarget                         [29] MMSReadReportNonLocalTarget,

mMSCancel                                           [30] MMSCancel,

mMSMBoxStore                                        [31] MMSMBoxStore,

mMSMBoxUpload                                       [32] MMSMBoxUpload,

mMSMBoxDelete                                       [33] MMSMBoxDelete,

mMSMBoxViewRequest                                  [34] MMSMBoxViewRequest,

mMSMBoxViewResponse                                 [35] MMSMBoxViewResponse,

-- PTC-related events, see clause 7.5.2

pTCRegistration                                     [36] PTCRegistration,

pTCSessionInitiation                                [37] PTCSessionInitiation,

pTCSessionAbandon                                   [38] PTCSessionAbandon,

pTCSessionStart                                     [39] PTCSessionStart,

pTCSessionEnd                                       [40] PTCSessionEnd,

pTCStartOfInterception                              [41] PTCStartOfInterception,

pTCPreEstablishedSession                            [42] PTCPreEstablishedSession,

pTCInstantPersonalAlert                             [43] PTCInstantPersonalAlert,

pTCPartyJoin                                        [44] PTCPartyJoin,

pTCPartyDrop                                        [45] PTCPartyDrop,

pTCPartyHold                                        [46] PTCPartyHold,

pTCMediaModification                                [47] PTCMediaModification,

pTCGroupAdvertisement                               [48] PTCGroupAdvertisement,

pTCFloorControl                                     [49] PTCFloorControl,

pTCTargetPresence                                   [50] PTCTargetPresence,

pTCParticipantPresence                              [51] PTCParticipantPresence,

pTCListManagement                                   [52] PTCListManagement,

pTCAccessPolicy                                     [53] PTCAccessPolicy,

-- More Subscriber-management related events, see clause 7.2.2

subscriberRecordChangeMessage                       [54] UDMSubscriberRecordChangeMessage,

cancelLocationMessage                               [55] UDMCancelLocationMessage,

-- SMS-related events continued from choice 12

sMSReport                                           [56] SMSReport,

-- MA PDU session-related events, see clause 6.2.3.2.7

sMFMAPDUSessionEstablishment                        [57] SMFMAPDUSessionEstablishment,

sMFMAPDUSessionModification                         [58] SMFMAPDUSessionModification,

sMFMAPDUSessionRelease                              [59] SMFMAPDUSessionRelease,

startOfInterceptionWithEstablishedMAPDUSession      [60] SMFStartOfInterceptionWithEstablishedMAPDUSession,

unsuccessfulMASMProcedure                           [61] SMFMAUnsuccessfulProcedure,

-- Identifier Association events, see clauses 6.2.2.2.7 and 6.3.2.2.2

aMFIdentifierAssociation                            [62] AMFIdentifierAssociation,

mMEIdentifierAssociation                            [63] MMEIdentifierAssociation,

-- PDU to MA PDU session-related events, see clause 6.2.3.2.8

sMFPDUtoMAPDUSessionModification                    [64] SMFPDUtoMAPDUSessionModification,

-- NEF services related events, see clause 7.7.2

nEFPDUSessionEstablishment                          [65] NEFPDUSessionEstablishment,

nEFPDUSessionModification                           [66] NEFPDUSessionModification,

nEFPDUSessionRelease                                [67] NEFPDUSessionRelease,

nEFUnsuccessfulProcedure                            [68] NEFUnsuccessfulProcedure,

nEFStartOfInterceptionWithEstablishedPDUSession     [69] NEFStartOfInterceptionWithEstablishedPDUSession,

nEFdeviceTrigger                                    [70] NEFDeviceTrigger,

nEFdeviceTriggerReplace                             [71] NEFDeviceTriggerReplace,

nEFdeviceTriggerCancellation                        [72] NEFDeviceTriggerCancellation,

nEFdeviceTriggerReportNotify                        [73] NEFDeviceTriggerReportNotify,

nEFMSISDNLessMOSMS                                  [74] NEFMSISDNLessMOSMS,

nEFExpectedUEBehaviourUpdate                        [75] NEFExpectedUEBehaviourUpdate,

-- SCEF services related events, see clause 7.8.2

sCEFPDNConnectionEstablishment                      [76] SCEFPDNConnectionEstablishment,

sCEFPDNConnectionUpdate                             [77] SCEFPDNConnectionUpdate,

sCEFPDNConnectionRelease                            [78] SCEFPDNConnectionRelease,

sCEFUnsuccessfulProcedure                           [79] SCEFUnsuccessfulProcedure,

sCEFStartOfInterceptionWithEstablishedPDNConnection [80] SCEFStartOfInterceptionWithEstablishedPDNConnection,

sCEFdeviceTrigger                                   [81] SCEFDeviceTrigger,

sCEFdeviceTriggerReplace                            [82] SCEFDeviceTriggerReplace,

sCEFdeviceTriggerCancellation                       [83] SCEFDeviceTriggerCancellation,

sCEFdeviceTriggerReportNotify                       [84] SCEFDeviceTriggerReportNotify,

sCEFMSISDNLessMOSMS                                 [85] SCEFMSISDNLessMOSMS,

sCEFCommunicationPatternUpdate                      [86] SCEFCommunicationPatternUpdate,

-- EPS Events, see clause 6.3

-- MME Events, see clause 6.3.2.2

mMEAttach                                           [87] MMEAttach,

mMEDetach                                           [88] MMEDetach,

mMELocationUpdate                                   [89] MMELocationUpdate,

mMEStartOfInterceptionWithEPSAttachedUE             [90] MMEStartOfInterceptionWithEPSAttachedUE,

mMEUnsuccessfulProcedure                            [91] MMEUnsuccessfulProcedure,

-- AKMA key management events, see clause 7.9.1

aAnFAnchorKeyRegister                               [92] AAnFAnchorKeyRegister,

aAnFKAKMAApplicationKeyGet                          [93] AAnFKAKMAApplicationKeyGet,

aAnFStartOfInterceptWithEstablishedAKMAKeyMaterial  [94] AAnFStartOfInterceptWithEstablishedAKMAKeyMaterial,

aAnFAKMAContextRemovalRecord                        [95] AAnFAKMAContextRemovalRecord,

aFAKMAApplicationKeyRefresh                         [96] AFAKMAApplicationKeyRefresh,

aFStartOfInterceptWithEstablishedAKMAApplicationKey [97] AFStartOfInterceptWithEstablishedAKMAApplicationKey,

aFAuxiliarySecurityParameterEstablishment           [98] AFAuxiliarySecurityParameterEstablishment,

aFApplicationKeyRemoval                             [99] AFApplicationKeyRemoval,

-- HR LI Events, see clause 7.10.3.3

n9HRPDUSessionInfo                                  [100] N9HRPDUSessionInfo,

s8HRBearerInfo                                      [101] S8HRBearerInfo,

-- Separated Location Reporting, see clause 7.3.4

separatedLocationReporting                          [102] SeparatedLocationReporting,

-- STIR SHAKEN and RCD/eCNAM Events, see clause 7.11.2

sTIRSHAKENSignatureGeneration                       [103] STIRSHAKENSignatureGeneration,

sTIRSHAKENSignatureValidation                       [104] STIRSHAKENSignatureValidation,

-- IMS events, see clause 7.12.4.2

iMSMessage                                          [105] IMSMessage,

startOfInterceptionForActiveIMSSession              [106] StartOfInterceptionForActiveIMSSession,

iMSCCUnavailable                                    [107] IMSCCUnavailable,

-- UDM events, see clause 7.2.2

uDMLocationInformationResult                        [108] UDMLocationInformationResult,

uDMUEInformationResponse                            [109] UDMUEInformationResponse,

uDMUEAuthenticationResponse                         [110] UDMUEAuthenticationResponse,

-- AMF events, see 6.2.2.2.8

positioningInfoTransfer                             [111] AMFPositioningInfoTransfer,

-- MME Events, see clause 6.3.2.2.8

mMEPositioningInfoTransfer                          [112] MMEPositioningInfoTransfer,

-- Tags 113 to 131 are not used in this version of the specification

-- AMF events, see 6.2.2.2.9, continued from tag 111

aMFUEConfigurationUpdate                            [132] AMFUEConfigurationUpdate

}

-- ==============

-- X3 xCC payload

-- ==============

-- No additional xCC payload definitions required in the present document.

-- ===============

-- HI2 IRI payload

-- ===============

IRIPayload ::= SEQUENCE

{

iRIPayloadOID       [1] RELATIVE-OID,

event               [2] IRIEvent,

targetIdentifiers   [3] SEQUENCE OF IRITargetIdentifier OPTIONAL

}

IRIEvent ::= CHOICE

{

-- Registration-related events, see clause 6.2.2

registration                                        [1] AMFRegistration,

deregistration                                      [2] AMFDeregistration,

locationUpdate                                      [3] AMFLocationUpdate,

startOfInterceptionWithRegisteredUE                 [4] AMFStartOfInterceptionWithRegisteredUE,

unsuccessfulRegistrationProcedure                   [5] AMFUnsuccessfulProcedure,

-- PDU session-related events, see clause 6.2.3

pDUSessionEstablishment                             [6] SMFPDUSessionEstablishment,

pDUSessionModification                              [7] SMFPDUSessionModification,

pDUSessionRelease                                   [8] SMFPDUSessionRelease,

startOfInterceptionWithEstablishedPDUSession        [9] SMFStartOfInterceptionWithEstablishedPDUSession,

unsuccessfulSessionProcedure                        [10] SMFUnsuccessfulProcedure,

-- Subscriber-management related events, see clause 7.2.2

servingSystemMessage                                [11] UDMServingSystemMessage,

-- SMS-related events, see clause 6.2.5, see also sMSReport ([56] below)

sMSMessage                                          [12] SMSMessage,

-- LALS-related events, see clause 7.3.1

lALSReport                                          [13] LALSReport,

-- PDHR/PDSR-related events, see clause 6.2.3.4.1

pDHeaderReport                                      [14] PDHeaderReport,

pDSummaryReport                                     [15] PDSummaryReport,

-- MDF-related events, see clause 7.3.2

mDFCellSiteReport                                   [16] MDFCellSiteReport,

-- MMS-related events, see clause 7.4.2

mMSSend                                             [17] MMSSend,

mMSSendByNonLocalTarget                             [18] MMSSendByNonLocalTarget,

mMSNotification                                     [19] MMSNotification,

mMSSendToNonLocalTarget                             [20] MMSSendToNonLocalTarget,

mMSNotificationResponse                             [21] MMSNotificationResponse,

mMSRetrieval                                        [22] MMSRetrieval,

mMSDeliveryAck                                      [23] MMSDeliveryAck,

mMSForward                                          [24] MMSForward,

mMSDeleteFromRelay                                  [25] MMSDeleteFromRelay,

mMSDeliveryReport                                   [26] MMSDeliveryReport,

mMSDeliveryReportNonLocalTarget                     [27] MMSDeliveryReportNonLocalTarget,

mMSReadReport                                       [28] MMSReadReport,

mMSReadReportNonLocalTarget                         [29] MMSReadReportNonLocalTarget,

mMSCancel                                           [30] MMSCancel,

mMSMBoxStore                                        [31] MMSMBoxStore,

mMSMBoxUpload                                       [32] MMSMBoxUpload,

mMSMBoxDelete                                       [33] MMSMBoxDelete,

mMSMBoxViewRequest                                  [34] MMSMBoxViewRequest,

mMSMBoxViewResponse                                 [35] MMSMBoxViewResponse,

-- PTC-related events, see clause 7.5.2

pTCRegistration                                     [36] PTCRegistration,

pTCSessionInitiation                                [37] PTCSessionInitiation,

pTCSessionAbandon                                   [38] PTCSessionAbandon,

pTCSessionStart                                     [39] PTCSessionStart,

pTCSessionEnd                                       [40] PTCSessionEnd,

pTCStartOfInterception                              [41] PTCStartOfInterception,

pTCPreEstablishedSession                            [42] PTCPreEstablishedSession,

pTCInstantPersonalAlert                             [43] PTCInstantPersonalAlert,

pTCPartyJoin                                        [44] PTCPartyJoin,

pTCPartyDrop                                        [45] PTCPartyDrop,

pTCPartyHold                                        [46] PTCPartyHold,

pTCMediaModification                                [47] PTCMediaModification,

pTCGroupAdvertisement                               [48] PTCGroupAdvertisement,

pTCFloorControl                                     [49] PTCFloorControl,

pTCTargetPresence                                   [50] PTCTargetPresence,

pTCParticipantPresence                              [51] PTCParticipantPresence,

pTCListManagement                                   [52] PTCListManagement,

pTCAccessPolicy                                     [53] PTCAccessPolicy,

-- More Subscriber-management related events, see clause 7.2.2

subscriberRecordChangeMessage                      [54] UDMSubscriberRecordChangeMessage,

cancelLocationMessage                              [55] UDMCancelLocationMessage,

-- SMS-related events, continued from choice 12

sMSReport                                           [56] SMSReport,

-- MA PDU session-related events, see clause 6.2.3.2.7

sMFMAPDUSessionEstablishment                        [57] SMFMAPDUSessionEstablishment,

sMFMAPDUSessionModification                         [58] SMFMAPDUSessionModification,

sMFMAPDUSessionRelease                              [59] SMFMAPDUSessionRelease,

startOfInterceptionWithEstablishedMAPDUSession      [60] SMFStartOfInterceptionWithEstablishedMAPDUSession,

unsuccessfulMASMProcedure                           [61] SMFMAUnsuccessfulProcedure,

-- Identifier Association events, see clauses 6.2.2.2.7 and 6.3.2.2.2

aMFIdentifierAssociation                           [62] AMFIdentifierAssociation,

mMEIdentifierAssociation                           [63] MMEIdentifierAssociation,

-- PDU to MA PDU session-related events, see clause 6.2.3.2.8

sMFPDUtoMAPDUSessionModification                    [64] SMFPDUtoMAPDUSessionModification,

-- NEF services related events, see clause 7.7.2,

nEFPDUSessionEstablishment                          [65] NEFPDUSessionEstablishment,

nEFPDUSessionModification                           [66] NEFPDUSessionModification,

nEFPDUSessionRelease                                [67] NEFPDUSessionRelease,

nEFUnsuccessfulProcedure                            [68] NEFUnsuccessfulProcedure,

nEFStartOfInterceptionWithEstablishedPDUSession     [69] NEFStartOfInterceptionWithEstablishedPDUSession,

nEFdeviceTrigger                                    [70] NEFDeviceTrigger,

nEFdeviceTriggerReplace                             [71] NEFDeviceTriggerReplace,

nEFdeviceTriggerCancellation                        [72] NEFDeviceTriggerCancellation,

nEFdeviceTriggerReportNotify                        [73] NEFDeviceTriggerReportNotify,

nEFMSISDNLessMOSMS                                  [74] NEFMSISDNLessMOSMS,

nEFExpectedUEBehaviourUpdate                        [75] NEFExpectedUEBehaviourUpdate,

-- SCEF services related events, see clause 7.8.2

sCEFPDNConnectionEstablishment                      [76] SCEFPDNConnectionEstablishment,

sCEFPDNConnectionUpdate                             [77] SCEFPDNConnectionUpdate,

sCEFPDNConnectionRelease                            [78] SCEFPDNConnectionRelease,

sCEFUnsuccessfulProcedure                           [79] SCEFUnsuccessfulProcedure,

sCEFStartOfInterceptionWithEstablishedPDNConnection [80] SCEFStartOfInterceptionWithEstablishedPDNConnection,

sCEFdeviceTrigger                                   [81] SCEFDeviceTrigger,

sCEFdeviceTriggerReplace                            [82] SCEFDeviceTriggerReplace,

sCEFdeviceTriggerCancellation                       [83] SCEFDeviceTriggerCancellation,

sCEFdeviceTriggerReportNotify                       [84] SCEFDeviceTriggerReportNotify,

sCEFMSISDNLessMOSMS                                 [85] SCEFMSISDNLessMOSMS,

sCEFCommunicationPatternUpdate                      [86] SCEFCommunicationPatternUpdate,

-- EPS Events, see clause 6.3

-- MME Events, see clause 6.3.2.2

mMEAttach                                           [87] MMEAttach,

mMEDetach                                           [88] MMEDetach,

mMELocationUpdate                                   [89] MMELocationUpdate,

mMEStartOfInterceptionWithEPSAttachedUE             [90] MMEStartOfInterceptionWithEPSAttachedUE,

mMEUnsuccessfulProcedure                            [91] MMEUnsuccessfulProcedure,

-- AKMA key management events, see clause 7.9.1

aAnFAnchorKeyRegister                               [92] AAnFAnchorKeyRegister,

aAnFKAKMAApplicationKeyGet                          [93] AAnFKAKMAApplicationKeyGet,

aAnFStartOfInterceptWithEstablishedAKMAKeyMaterial  [94] AAnFStartOfInterceptWithEstablishedAKMAKeyMaterial,

aAnFAKMAContextRemovalRecord                        [95] AAnFAKMAContextRemovalRecord,

aFAKMAApplicationKeyRefresh                         [96] AFAKMAApplicationKeyRefresh,

aFStartOfInterceptWithEstablishedAKMAApplicationKey [97] AFStartOfInterceptWithEstablishedAKMAApplicationKey,

aFAuxiliarySecurityParameterEstablishment           [98] AFAuxiliarySecurityParameterEstablishment,

aFApplicationKeyRemoval                             [99] AFApplicationKeyRemoval,

-- tag 100 is reserved because there is no equivalent n9HRPDUSessionInfo in IRIEvent.

-- tag 101 is reserved because there is no equivalent S8HRBearerInfo in IRIEvent.

-- Separated Location Reporting, see clause 7.3.4

separatedLocationReporting                          [102] SeparatedLocationReporting,

-- STIR SHAKEN and RCD/eCNAM Events, see clause 7.11.3

sTIRSHAKENSignatureGeneration                       [103] STIRSHAKENSignatureGeneration,

sTIRSHAKENSignatureValidation                       [104] STIRSHAKENSignatureValidation,

-- IMS events, see clause 7.11.4.2

iMSMessage                                          [105] IMSMessage,

startOfInterceptionForActiveIMSSession              [106] StartOfInterceptionForActiveIMSSession,

iMSCCUnavailable                                    [107] IMSCCUnavailable,

-- UDM events, see clause 7.2.2

uDMLocationInformationResultRecord                  [108] UDMLocationInformationResult,

uDMUEInformationResponse                            [109] UDMUEInformationResponse,

uDMUEAuthenticationResponse                         [110] UDMUEAuthenticationResponse,

-- AMF events, see 6.2.2.2.8

positioningInfoTransfer                             [111] AMFPositioningInfoTransfer,

-- MME Events, see clause 6.3.2.2.8

mMEPositioningInfoTransfer                          [112] MMEPositioningInfoTransfer,

-- Tags 113 to 131 are not used in this version of the specification

-- AMF events, see 6.2.2.3, continued from tag 111

aMFUEConfigurationUpdate                            [132] AMFUEConfigurationUpdate

}

IRITargetIdentifier ::= SEQUENCE

{

identifier                                          [1] TargetIdentifier,

provenance                                          [2] TargetIdentifierProvenance OPTIONAL

}

-- ==============

-- HI3 CC payload

-- ==============

CCPayload ::= SEQUENCE

{

cCPayloadOID         [1] RELATIVE-OID,

pDU                  [2] CCPDU

}

CCPDU ::= CHOICE

{

uPFCCPDU            [1] UPFCCPDU,

extendedUPFCCPDU    [2] ExtendedUPFCCPDU,

mMSCCPDU            [3] MMSCCPDU,

-- In Rel-16 (threeGPP(4) ts33128(19) r16(16) version9(9)),

-- tag 4 is pTCCCPDU and tag 5 is not used.

-- Rel-17 or newer decoders should decode tag 4 contents as PTCCCPDU if

-- r16 is used in cCPayloadOID.

nIDDCCPDU           [4] NIDDCCPDU,

pTCCCPDU            [5] PTCCCPDU,

iMSCCPDU            [6] IMSCCPDU

}

-- ===========================

-- HI4 LI notification payload

-- ===========================

LINotificationPayload ::= SEQUENCE

{

lINotificationPayloadOID         [1] RELATIVE-OID,

notification                     [2] LINotificationMessage

}

LINotificationMessage ::= CHOICE

{

lINotification      [1] LINotification

}

-- =================

-- HR LI definitions

-- =================

N9HRPDUSessionInfo ::= SEQUENCE

{

sUPI                            [1] SUPI,

pEI                             [2] PEI OPTIONAL,

pDUSessionID                    [3] PDUSessionID,

location                        [4] Location OPTIONAL,

sNSSAI                          [5] SNSSAI OPTIONAL,

dNN                             [6] DNN OPTIONAL,

messageCause                    [7] N9HRMessageCause

}

S8HRBearerInfo ::= SEQUENCE

{

iMSI                            [1] IMSI,

iMEI                            [2] IMEI OPTIONAL,

bearerID                        [3] EPSBearerID,

linkedBearerID                  [4] EPSBearerID OPTIONAL,

location                        [5] Location OPTIONAL,

aPN                             [6] APN OPTIONAL,

sGWIPAddress                    [7] IPAddress OPTIONAL,

messageCause                    [8] S8HRMessageCause

}

-- ================

-- HR LI parameters

-- ================

N9HRMessageCause ::= ENUMERATED

{

pDUSessionEstablished(1),

pDUSessionModified(2),

pDUSessionReleased(3),

updatedLocationAvailable(4),

sMFChanged(5),

other(6),

hRLIEnabled(7)

}

S8HRMessageCause ::= ENUMERATED

{

bearerActivated(1),

bearerModified(2),

bearerDeleted(3),

pDNDisconnected(4),

updatedLocationAvailable(5),

sGWChanged(6),

other(7),

hRLIEnabled(8)

}

-- ==================

-- 5G NEF definitions

-- ==================

-- See clause 7.7.2.1.2 for details of this structure

NEFPDUSessionEstablishment ::= SEQUENCE

{

sUPI                  [1] SUPI,

gPSI                  [2] GPSI,

pDUSessionID          [3] PDUSessionID,

sNSSAI                [4] SNSSAI,

nEFID                 [5] NEFID,

dNN                   [6] DNN,

rDSSupport            [7] RDSSupport,

sMFID                 [8] SMFID,

aFID                  [9] AFID

}

-- See clause 7.7.2.1.3 for details of this structure

NEFPDUSessionModification ::= SEQUENCE

{

sUPI                         [1] SUPI,

gPSI                         [2] GPSI,

sNSSAI                       [3] SNSSAI,

initiator                    [4] Initiator,

rDSSourcePortNumber          [5] RDSPortNumber OPTIONAL,

rDSDestinationPortNumber     [6] RDSPortNumber OPTIONAL,

applicationID                [7] ApplicationID OPTIONAL,

aFID                         [8] AFID OPTIONAL,

rDSAction                    [9] RDSAction OPTIONAL,

serializationFormat          [10] SerializationFormat OPTIONAL

}

-- See clause 7.7.2.1.4 for details of this structure

NEFPDUSessionRelease ::= SEQUENCE

{

sUPI                   [1] SUPI,

gPSI                   [2] GPSI,

pDUSessionID           [3] PDUSessionID,

timeOfFirstPacket      [4] Timestamp OPTIONAL,

timeOfLastPacket       [5] Timestamp OPTIONAL,

uplinkVolume           [6] INTEGER OPTIONAL,

downlinkVolume         [7] INTEGER OPTIONAL,

releaseCause           [8] NEFReleaseCause

}

-- See clause 7.7.2.1.5 for details of this structure

NEFUnsuccessfulProcedure ::= SEQUENCE

{

failureCause                 [1] NEFFailureCause,

sUPI                         [2] SUPI,

gPSI                         [3] GPSI OPTIONAL,

pDUSessionID                 [4] PDUSessionID,

dNN                          [5] DNN OPTIONAL,

sNSSAI                       [6] SNSSAI OPTIONAL,

rDSDestinationPortNumber     [7] RDSPortNumber,

applicationID                [8] ApplicationID,

aFID                         [9] AFID

}

-- See clause 7.7.2.1.6 for details of this structure

NEFStartOfInterceptionWithEstablishedPDUSession ::= SEQUENCE

{

sUPI               [1] SUPI,

gPSI               [2] GPSI,

pDUSessionID       [3] PDUSessionID,

dNN                [4] DNN,

sNSSAI             [5] SNSSAI,

nEFID              [6] NEFID,

rDSSupport         [7] RDSSupport,

sMFID              [8] SMFID,

aFID               [9] AFID

}

-- See clause 7.7.3.1.1 for details of this structure

NEFDeviceTrigger ::= SEQUENCE

{

sUPI                  [1] SUPI,

gPSI                  [2] GPSI,

triggerId             [3] TriggerID,

aFID                  [4] AFID,

triggerPayload        [5] TriggerPayload OPTIONAL,

validityPeriod        [6] INTEGER OPTIONAL,

priorityDT            [7] PriorityDT OPTIONAL,

sourcePortId          [8] PortNumber OPTIONAL,

destinationPortId     [9] PortNumber OPTIONAL

}

-- See clause 7.7.3.1.2 for details of this structure

NEFDeviceTriggerReplace ::= SEQUENCE

{

sUPI                     [1] SUPI,

gPSI                     [2] GPSI,

triggerId                [3] TriggerID,

aFID                     [4] AFID,

triggerPayload           [5] TriggerPayload OPTIONAL,

validityPeriod           [6] INTEGER OPTIONAL,

priorityDT               [7] PriorityDT OPTIONAL,

sourcePortId             [8] PortNumber OPTIONAL,

destinationPortId        [9] PortNumber OPTIONAL

}

-- See clause 7.7.3.1.3 for details of this structure

NEFDeviceTriggerCancellation ::= SEQUENCE

{

sUPI                  [1] SUPI,

gPSI                  [2] GPSI,

triggerId             [3] TriggerID

}

-- See clause 7.7.3.1.4 for details of this structure

NEFDeviceTriggerReportNotify ::= SEQUENCE

{

sUPI                             [1] SUPI,

gPSI                             [2] GPSI,

triggerId                        [3] TriggerID,

deviceTriggerDeliveryResult      [4] DeviceTriggerDeliveryResult

}

-- See clause 7.7.4.1.1 for details of this structure

NEFMSISDNLessMOSMS ::= SEQUENCE

{

sUPI                      [1] SUPI,

gPSI                      [2] GPSI,

terminatingSMSParty       [3] AFID,

sMS                       [4] SMSTPDUData OPTIONAL,

sourcePort                [5] PortNumber OPTIONAL,

destinationPort           [6] PortNumber OPTIONAL

}

-- See clause 7.7.5.1.1 for details of this structure

NEFExpectedUEBehaviourUpdate ::= SEQUENCE

{

gPSI                                  [1] GPSI,

expectedUEMovingTrajectory            [2] SEQUENCE OF UMTLocationArea5G OPTIONAL,

stationaryIndication                  [3] StationaryIndication OPTIONAL,

communicationDurationTime             [4] INTEGER OPTIONAL,

periodicTime                          [5] INTEGER OPTIONAL,

scheduledCommunicationTime            [6] ScheduledCommunicationTime OPTIONAL,

scheduledCommunicationType            [7] ScheduledCommunicationType OPTIONAL,

batteryIndication                     [8] BatteryIndication OPTIONAL,

trafficProfile                        [9] TrafficProfile OPTIONAL,

expectedTimeAndDayOfWeekInTrajectory  [10] SEQUENCE OF UMTLocationArea5G OPTIONAL,

aFID                                  [11] AFID,

validityTime                          [12] Timestamp OPTIONAL

}

-- ==========================

-- Common SCEF/NEF parameters

-- ==========================

RDSSupport ::= BOOLEAN

RDSPortNumber ::= INTEGER (0..15)

RDSAction ::= ENUMERATED

{

reservePort(1),

releasePort(2)

}

SerializationFormat ::= ENUMERATED

{

xml(1),

json(2),

cbor(3)

}

ApplicationID ::= OCTET STRING

NIDDCCPDU ::= OCTET STRING

TriggerID ::= UTF8String

PriorityDT ::= ENUMERATED

{

noPriority(1),

priority(2)

}

TriggerPayload ::= OCTET STRING

DeviceTriggerDeliveryResult ::= ENUMERATED

{

success(1),

unknown(2),

failure(3),

triggered(4),

expired(5),

unconfirmed(6),

replaced(7),

terminate(8)

}

StationaryIndication ::= ENUMERATED

{

stationary(1),

mobile(2)

}

BatteryIndication ::= ENUMERATED

{

batteryRecharge(1),

batteryReplace(2),

batteryNoRecharge(3),

batteryNoReplace(4),

noBattery(5)

}

ScheduledCommunicationTime ::= SEQUENCE

{

days [1] SEQUENCE OF Daytime

}

UMTLocationArea5G ::= SEQUENCE

{

timeOfDay        [1] Daytime,

durationSec      [2] INTEGER,

location         [3] NRLocation

}

Daytime ::= SEQUENCE

{

daysOfWeek       [1] Day OPTIONAL,

timeOfDayStart   [2] Timestamp OPTIONAL,

timeOfDayEnd     [3] Timestamp OPTIONAL

}

Day ::= ENUMERATED

{

monday(1),

tuesday(2),

wednesday(3),

thursday(4),

friday(5),

saturday(6),

sunday(7)

}

TrafficProfile ::= ENUMERATED

{

singleTransUL(1),

singleTransDL(2),

dualTransULFirst(3),

dualTransDLFirst(4),

multiTrans(5)

}

ScheduledCommunicationType ::= ENUMERATED

{

downlinkOnly(1),

uplinkOnly(2),

bidirectional(3)

}

-- =================

-- 5G NEF parameters

-- =================

NEFFailureCause ::= ENUMERATED

{

userUnknown(1),

niddConfigurationNotAvailable(2),

contextNotFound(3),

portNotFree(4),

portNotAssociatedWithSpecifiedApplication(5)

}

NEFReleaseCause ::= ENUMERATED

{

sMFRelease(1),

dNRelease(2),

uDMRelease(3),

cHFRelease(4),

localConfigurationPolicy(5),

unknownCause(6)

}

AFID ::= UTF8String

NEFID ::= UTF8String

-- ==================

-- SCEF definitions

-- ==================

-- See clause 7.8.2.1.2 for details of this structure

SCEFPDNConnectionEstablishment ::= SEQUENCE

{

iMSI                  [1] IMSI OPTIONAL,

mSISDN                [2] MSISDN OPTIONAL,

externalIdentifier    [3] NAI OPTIONAL,

iMEI                  [4] IMEI OPTIONAL,

ePSBearerID           [5] EPSBearerID,

sCEFID                [6] SCEFID,

aPN                   [7] APN,

rDSSupport            [8] RDSSupport,

sCSASID               [9] SCSASID

}

-- See clause 7.8.2.1.3 for details of this structure

SCEFPDNConnectionUpdate ::= SEQUENCE

{

iMSI                         [1] IMSI OPTIONAL,

mSISDN                       [2] MSISDN OPTIONAL,

externalIdentifier           [3] NAI OPTIONAL,

initiator                    [4] Initiator,

rDSSourcePortNumber          [5] RDSPortNumber OPTIONAL,

rDSDestinationPortNumber     [6] RDSPortNumber OPTIONAL,

applicationID                [7] ApplicationID OPTIONAL,

sCSASID                      [8] SCSASID OPTIONAL,

rDSAction                    [9] RDSAction OPTIONAL,

serializationFormat          [10] SerializationFormat OPTIONAL

}

-- See clause 7.8.2.1.4 for details of this structure

SCEFPDNConnectionRelease ::= SEQUENCE

{

iMSI                   [1] IMSI OPTIONAL,

mSISDN                 [2] MSISDN OPTIONAL,

externalIdentifier     [3] NAI OPTIONAL,

ePSBearerID            [4] EPSBearerID,

timeOfFirstPacket      [5] Timestamp OPTIONAL,

timeOfLastPacket       [6] Timestamp OPTIONAL,

uplinkVolume           [7] INTEGER OPTIONAL,

downlinkVolume         [8] INTEGER OPTIONAL,

releaseCause           [9] SCEFReleaseCause

}

-- See clause 7.8.2.1.5 for details of this structure

SCEFUnsuccessfulProcedure ::= SEQUENCE

{

failureCause                 [1] SCEFFailureCause,

iMSI                         [2] IMSI OPTIONAL,

mSISDN                       [3] MSISDN OPTIONAL,

externalIdentifier           [4] NAI OPTIONAL,

ePSBearerID                  [5] EPSBearerID,

aPN                          [6] APN,

rDSDestinationPortNumber     [7] RDSPortNumber OPTIONAL,

applicationID                [8] ApplicationID OPTIONAL,

sCSASID                      [9] SCSASID

}

-- See clause 7.8.2.1.6 for details of this structure

SCEFStartOfInterceptionWithEstablishedPDNConnection ::= SEQUENCE

{

iMSI                  [1] IMSI OPTIONAL,

mSISDN                [2] MSISDN OPTIONAL,

externalIdentifier    [3] NAI OPTIONAL,

iMEI                  [4] IMEI OPTIONAL,

ePSBearerID           [5] EPSBearerID,

sCEFID                [6] SCEFID,

aPN                   [7] APN,

rDSSupport            [8] RDSSupport,

sCSASID               [9] SCSASID

}

-- See clause 7.8.3.1.1 for details of this structure

SCEFDeviceTrigger ::= SEQUENCE

{

iMSI                  [1] IMSI,

mSISDN                [2] MSISDN,

externalIdentifier    [3] NAI,

triggerId             [4] TriggerID,

sCSASID               [5] SCSASID OPTIONAL,

triggerPayload        [6] TriggerPayload OPTIONAL,

validityPeriod        [7] INTEGER OPTIONAL,

priorityDT            [8] PriorityDT OPTIONAL,

sourcePortId          [9] PortNumber OPTIONAL,

destinationPortId     [10] PortNumber OPTIONAL

}

-- See clause 7.8.3.1.2 for details of this structure

SCEFDeviceTriggerReplace ::= SEQUENCE

{

iMSI                     [1] IMSI OPTIONAL,

mSISDN                   [2] MSISDN OPTIONAL,

externalIdentifier       [3] NAI OPTIONAL,

triggerId                [4] TriggerID,

sCSASID                  [5] SCSASID OPTIONAL,

triggerPayload           [6] TriggerPayload OPTIONAL,

validityPeriod           [7] INTEGER OPTIONAL,

priorityDT               [8] PriorityDT OPTIONAL,

sourcePortId             [9] PortNumber OPTIONAL,

destinationPortId        [10] PortNumber OPTIONAL

}

-- See clause 7.8.3.1.3 for details of this structure

SCEFDeviceTriggerCancellation ::= SEQUENCE

{

iMSI                     [1] IMSI OPTIONAL,

mSISDN                   [2] MSISDN OPTIONAL,

externalIdentifier       [3] NAI OPTIONAL,

triggerId                [4] TriggerID

}

-- See clause 7.8.3.1.4 for details of this structure

SCEFDeviceTriggerReportNotify ::= SEQUENCE

{

iMSI                             [1] IMSI OPTIONAL,

mSISDN                           [2] MSISDN OPTIONAL,

externalIdentifier               [3] NAI OPTIONAL,

triggerId                        [4] TriggerID,

deviceTriggerDeliveryResult      [5] DeviceTriggerDeliveryResult

}

-- See clause 7.8.4.1.1 for details of this structure

SCEFMSISDNLessMOSMS ::= SEQUENCE

{

iMSI                      [1] IMSI OPTIONAL,

mSISDN                    [2] MSISDN OPTIONAL,

externalIdentifie         [3] NAI OPTIONAL,

terminatingSMSParty       [4] SCSASID,

sMS                       [5] SMSTPDUData OPTIONAL,

sourcePort                [6] PortNumber OPTIONAL,

destinationPort           [7] PortNumber OPTIONAL

}

-- See clause 7.8.5.1.1 for details of this structure

SCEFCommunicationPatternUpdate ::= SEQUENCE

{

mSISDN                                [1] MSISDN OPTIONAL,

externalIdentifier                    [2] NAI OPTIONAL,

periodicCommunicationIndicator        [3] PeriodicCommunicationIndicator OPTIONAL,

communicationDurationTime             [4] INTEGER OPTIONAL,

periodicTime                          [5] INTEGER OPTIONAL,

scheduledCommunicationTime            [6] ScheduledCommunicationTime OPTIONAL,

scheduledCommunicationType            [7] ScheduledCommunicationType OPTIONAL,

stationaryIndication                  [8] StationaryIndication OPTIONAL,

batteryIndication                     [9] BatteryIndication OPTIONAL,

trafficProfile                        [10] TrafficProfile OPTIONAL,

expectedUEMovingTrajectory            [11] SEQUENCE OF UMTLocationArea5G OPTIONAL,

sCSASID                               [13] SCSASID,

validityTime                          [14] Timestamp OPTIONAL

}

-- =================

-- SCEF parameters

-- =================

SCEFFailureCause ::= ENUMERATED

{

userUnknown(1),

niddConfigurationNotAvailable(2),

invalidEPSBearer(3),

operationNotAllowed(4),

portNotFree(5),

portNotAssociatedWithSpecifiedApplication(6)

}

SCEFReleaseCause ::= ENUMERATED

{

mMERelease(1),

dNRelease(2),

hSSRelease(3),

localConfigurationPolicy(4),

unknownCause(5)

}

SCSASID ::= UTF8String

SCEFID ::= UTF8String

PeriodicCommunicationIndicator ::= ENUMERATED

{

periodic(1),

nonPeriodic(2)

}

EPSBearerID ::= INTEGER (0..255)

APN ::= UTF8String

-- =======================

-- AKMA AAnF definitions

-- =======================

AAnFAnchorKeyRegister ::= SEQUENCE

{

aKID                  [1] NAI,

sUPI                  [2] SUPI,

kAKMA                 [3] KAKMA OPTIONAL

}

AAnFKAKMAApplicationKeyGet ::= SEQUENCE

{

type                  [1] KeyGetType,

aKID                  [2] NAI,

keyInfo               [3] AFKeyInfo

}

AAnFStartOfInterceptWithEstablishedAKMAKeyMaterial ::= SEQUENCE

{

aKID                  [1] NAI,

kAKMA                 [2] KAKMA OPTIONAL,

aFKeyList             [3] SEQUENCE OF AFKeyInfo OPTIONAL

}

AAnFAKMAContextRemovalRecord ::= SEQUENCE

{

aKID                  [1] NAI,

nFID                  [2] NFID

}

-- ======================

-- AKMA common parameters

-- ======================

FQDN ::= UTF8String

NFID ::= UTF8String

UAProtocolID ::= OCTET STRING (SIZE(5))

AKMAAFID ::= SEQUENCE

{

aFFQDN                [1] FQDN,

uaProtocolID          [2] UAProtocolID

}

UAStarParams ::= CHOICE

{

tls12                 [1] TLS12UAStarParams,

generic               [2] GenericUAStarParams

}

GenericUAStarParams ::= SEQUENCE

{

genericClientParams [1] OCTET STRING,

genericServerParams [2] OCTET STRING

}

-- ===========================================

-- Specific UaStarParmas for TLS 1.2 (RFC5246)

-- ===========================================

TLSCipherType ::= ENUMERATED

{

stream(1),

block(2),

aead(3)

}

TLSCompressionAlgorithm ::= ENUMERATED

{

null(1),

deflate(2)

}

TLSPRFAlgorithm ::= ENUMERATED

{

rfc5246(1)

}

TLSCipherSuite ::= SEQUENCE (SIZE(2)) OF INTEGER (0..255)

TLS12UAStarParams ::= SEQUENCE

{

preMasterSecret       [1] OCTET STRING (SIZE(6)) OPTIONAL,

masterSecret          [2] OCTET STRING (SIZE(6)),

pRFAlgorithm          [3] TLSPRFAlgorithm,

cipherSuite           [4] TLSCipherSuite,

cipherType            [5] TLSCipherType,

encKeyLength          [6] INTEGER (0..255),

blockLength           [7] INTEGER (0..255),

fixedIVLength         [8] INTEGER (0..255),

recordIVLength        [9] INTEGER (0..255),

macLength             [10] INTEGER (0..255),

macKeyLength          [11] INTEGER (0..255),

compressionAlgorithm  [12] TLSCompressionAlgorithm,

clientRandom          [13] OCTET STRING (SIZE(4)),

serverRandom          [14] OCTET STRING (SIZE(4)),

clientSequenceNumber  [15] INTEGER,

serverSequenceNumber  [16] INTEGER,

sessionID             [17] OCTET STRING (SIZE(0..32)),

tLSExtensions         [18] OCTET STRING (SIZE(0..65535))

}

KAF ::= OCTET STRING

KAKMA ::= OCTET STRING

-- ====================

-- AKMA AAnF parameters

-- ====================

KeyGetType ::= ENUMERATED

{

internal(1),

external(2)

}

AFKeyInfo ::= SEQUENCE

{

aFID                 [1] AKMAAFID,

kAF                  [2] KAF,

kAFExpTime           [3] KAFExpiryTime

}

-- =======================

-- AKMA AF definitions

-- =======================

AFAKMAApplicationKeyRefresh ::= SEQUENCE

{

aFID                  [1] AFID,

aKID                  [2] NAI,

kAF                   [3] KAF,

uaStarParams          [4] UAStarParams OPTIONAL

}

AFStartOfInterceptWithEstablishedAKMAApplicationKey ::= SEQUENCE

{

aFID                  [1] FQDN,

aKID                  [2] NAI,

kAFParamList          [3] SEQUENCE OF AFSecurityParams

}

AFAuxiliarySecurityParameterEstablishment ::= SEQUENCE

{

aFSecurityParams      [1] AFSecurityParams

}

AFSecurityParams ::= SEQUENCE

{

aFID                  [1] AFID,

aKID                  [2] NAI,

kAF                   [3] KAF,

uaStarParams          [4] UAStarParams

}

AFApplicationKeyRemoval ::= SEQUENCE

{

aFID                  [1] AFID,

aKID                  [2] NAI,

removalCause          [3] AFKeyRemovalCause

}

-- ===================

-- AKMA AF parameters

-- ===================

KAFParams ::= SEQUENCE

{

aKID                 [1] NAI,

kAF                  [2] KAF,

kAFExpTime           [3] KAFExpiryTime,

uaStarParams         [4] UAStarParams

}

KAFExpiryTime ::= GeneralizedTime

AFKeyRemovalCause ::= ENUMERATED

{

unknown(1),

keyExpiry(2),

applicationSpecific(3)

}

-- ==================

-- 5G AMF definitions

-- ==================

-- See clause 6.2.2.2.2 for details of this structure

AMFRegistration ::= SEQUENCE

{

registrationType            [1] AMFRegistrationType,

registrationResult          [2] AMFRegistrationResult,

slice                       [3] Slice OPTIONAL,

sUPI                        [4] SUPI,

sUCI                        [5] SUCI OPTIONAL,

pEI                         [6] PEI OPTIONAL,

gPSI                        [7] GPSI OPTIONAL,

gUTI                        [8] FiveGGUTI,

location                    [9] Location OPTIONAL,

non3GPPAccessEndpoint       [10] UEEndpointAddress OPTIONAL,

fiveGSTAIList               [11] TAIList OPTIONAL,

sMSOverNasIndicator         [12] SMSOverNASIndicator OPTIONAL,

oldGUTI                     [13] EPS5GGUTI OPTIONAL,

eMM5GRegStatus              [14] EMM5GMMStatus OPTIONAL,

nonIMEISVPEI                [15] NonIMEISVPEI OPTIONAL,

mACRestIndicator            [16] MACRestrictionIndicator OPTIONAL,

pagingRestrictionIndicator  [17] PagingRestrictionIndicator OPTIONAL,

rATType                     [18] RATType OPTIONAL

}

-- See clause 6.2.2.2.3 for details of this structure

AMFDeregistration ::= SEQUENCE

{

deregistrationDirection     [1] AMFDirection,

accessType                  [2] AccessType,

sUPI                        [3] SUPI OPTIONAL,

sUCI                        [4] SUCI OPTIONAL,

pEI                         [5] PEI OPTIONAL,

gPSI                        [6] GPSI OPTIONAL,

gUTI                        [7] FiveGGUTI OPTIONAL,

cause                       [8] FiveGMMCause OPTIONAL,

location                    [9] Location OPTIONAL,

switchOffIndicator          [10] SwitchOffIndicator OPTIONAL,

reRegRequiredIndicator      [11] ReRegRequiredIndicator OPTIONAL

}

-- See clause 6.2.2.2.4 for details of this structure

AMFLocationUpdate ::= SEQUENCE

{

sUPI                        [1] SUPI,

sUCI                        [2] SUCI OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

gUTI                        [5] FiveGGUTI OPTIONAL,

location                    [6] Location,

sMSOverNASIndicator         [7] SMSOverNASIndicator OPTIONAL,

oldGUTI                     [8] EPS5GGUTI OPTIONAL

}

-- See clause 6.2.2.2.5 for details of this structure

AMFStartOfInterceptionWithRegisteredUE ::= SEQUENCE

{

registrationResult          [1] AMFRegistrationResult,

registrationType            [2] AMFRegistrationType OPTIONAL,

slice                       [3] Slice OPTIONAL,

sUPI                        [4] SUPI,

sUCI                        [5] SUCI OPTIONAL,

pEI                         [6] PEI OPTIONAL,

gPSI                        [7] GPSI OPTIONAL,

gUTI                        [8] FiveGGUTI,

location                    [9] Location OPTIONAL,

non3GPPAccessEndpoint       [10] UEEndpointAddress OPTIONAL,

timeOfRegistration          [11] Timestamp OPTIONAL,

fiveGSTAIList               [12] TAIList OPTIONAL,

sMSOverNASIndicator         [13] SMSOverNASIndicator OPTIONAL,

oldGUTI                     [14] EPS5GGUTI OPTIONAL,

eMM5GRegStatus              [15] EMM5GMMStatus OPTIONAL

}

-- See clause 6.2.2.2.6 for details of this structure

AMFUnsuccessfulProcedure ::= SEQUENCE

{

failedProcedureType         [1] AMFFailedProcedureType,

failureCause                [2] AMFFailureCause,

requestedSlice              [3] NSSAI OPTIONAL,

sUPI                        [4] SUPI OPTIONAL,

sUCI                        [5] SUCI OPTIONAL,

pEI                         [6] PEI OPTIONAL,

gPSI                        [7] GPSI OPTIONAL,

gUTI                        [8] FiveGGUTI OPTIONAL,

location                    [9] Location OPTIONAL

}

-- See clause 6.2.2.2.8 on for details of this structure

AMFPositioningInfoTransfer ::= SEQUENCE

{

sUPI                        [1] SUPI,

sUCI                        [2] SUCI OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

gUTI                        [5] FiveGGUTI OPTIONAL,

nRPPaMessage                [6] OCTET STRING OPTIONAL,

lPPMessage                  [7] OCTET STRING OPTIONAL,

lcsCorrelationId            [8] UTF8String (SIZE(1..255))

}

--See clause 6.2.2.2.9 on for details of this structure

AMFUEConfigurationUpdate ::= SEQUENCE

{

userIdentifiers     [1] UserIdentifiers,

gUTI                [2] GUTI,

oldGUTI             [3] EPS5GGUTI OPTIONAL,

fiveGSTAIList       [4] TAIList OPTIONAL,

slice               [5] Slice OPTIONAL,

serviceAreaList     [6] ServiceAreaList OPTIONAL,

registrationResult  [7] AMFRegistrationResult OPTIONAL,

sMSOverNASIndicator [8] SMSOverNASIndicator OPTIONAL

}

-- =================

-- 5G AMF parameters

-- =================

AMFID ::= SEQUENCE

{

aMFRegionID [1] AMFRegionID,

aMFSetID    [2] AMFSetID,

aMFPointer  [3] AMFPointer

}

AMFDirection ::= ENUMERATED

{

networkInitiated(1),

uEInitiated(2)

}

AMFFailedProcedureType ::= ENUMERATED

{

registration(1),

sMS(2),

pDUSessionEstablishment(3)

}

AMFFailureCause ::= CHOICE

{

fiveGMMCause        [1] FiveGMMCause,

fiveGSMCause        [2] FiveGSMCause

}

AMFPointer ::= INTEGER (0..63)

AMFRegistrationResult ::= ENUMERATED

{

threeGPPAccess(1),

nonThreeGPPAccess(2),

threeGPPAndNonThreeGPPAccess(3)

}

AMFRegionID ::= INTEGER (0..255)

AMFRegistrationType ::= ENUMERATED

{

initial(1),

mobility(2),

periodic(3),

emergency(4),

sNPNOnboarding(5),

disasterMobility(6),

disasterInitial(7)

}

AMFSetID ::= INTEGER (0..1023)

-- TS 24.501 [13], clause 9.11.3.49

ServiceAreaList ::= OCTET STRING (SIZE(4..112))

-- ==================

-- 5G SMF definitions

-- ==================

-- See clause 6.2.3.2.2 for details of this structure

SMFPDUSessionEstablishment ::= SEQUENCE

{

sUPI                          [1] SUPI OPTIONAL,

sUPIUnauthenticated           [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                           [3] PEI OPTIONAL,

gPSI                          [4] GPSI OPTIONAL,

pDUSessionID                  [5] PDUSessionID,

gTPTunnelID                   [6] FTEID,

pDUSessionType                [7] PDUSessionType,

sNSSAI                        [8] SNSSAI OPTIONAL,

uEEndpoint                    [9] SEQUENCE OF UEEndpointAddress OPTIONAL,

non3GPPAccessEndpoint         [10] UEEndpointAddress OPTIONAL,

location                      [11] Location OPTIONAL,

dNN                           [12] DNN,

aMFID                         [13] AMFID OPTIONAL,

hSMFURI                       [14] HSMFURI OPTIONAL,

requestType                   [15] FiveGSMRequestType,

accessType                    [16] AccessType OPTIONAL,

rATType                       [17] RATType OPTIONAL,

sMPDUDNRequest                [18] SMPDUDNRequest OPTIONAL,

uEEPSPDNConnection            [19] UEEPSPDNConnection OPTIONAL,

ePS5GSComboInfo               [20] EPS5GSComboInfo OPTIONAL,

selectedDNN                   [21] DNN OPTIONAL,

servingNetwork                [22] SMFServingNetwork OPTIONAL,

oldPDUSessionID               [23] PDUSessionID OPTIONAL,

handoverState                 [24] HandoverState OPTIONAL,

gTPTunnelInfo                 [25] GTPTunnelInfo OPTIONAL,

pCCRules                      [26] PCCRuleSet OPTIONAL,

ePSPDNConnectionEstablishment [27] EPSPDNConnectionEstablishment OPTIONAL

}

-- See clause 6.2.3.2.3 for details of this structure

SMFPDUSessionModification ::= SEQUENCE

{

sUPI                        [1] SUPI OPTIONAL,

sUPIUnauthenticated         [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

sNSSAI                      [5] SNSSAI OPTIONAL,

non3GPPAccessEndpoint       [6] UEEndpointAddress OPTIONAL,

location                    [7] Location OPTIONAL,

requestType                 [8] FiveGSMRequestType,

accessType                  [9] AccessType OPTIONAL,

rATType                     [10] RATType OPTIONAL,

pDUSessionID                [11] PDUSessionID OPTIONAL,

ePS5GSComboInfo             [12] EPS5GSComboInfo OPTIONAL,

uEEndpoint                  [13] UEEndpointAddress OPTIONAL,

servingNetwork              [14] SMFServingNetwork OPTIONAL,

handoverState               [15] HandoverState OPTIONAL,

gTPTunnelInfo               [16] GTPTunnelInfo OPTIONAL,

pCCRules                    [17] PCCRuleSet OPTIONAL,

ePSPDNConnectionModification[18] EPSPDNConnectionModification OPTIONAL,

uPPathChange                [19] UPPathChange OPTIONAL,

pFDDataForApp               [20] PFDDataForApp OPTIONAL

}

-- See clause 6.2.3.2.4 for details of this structure

SMFPDUSessionRelease ::= SEQUENCE

{

sUPI                        [1] SUPI,

pEI                         [2] PEI OPTIONAL,

gPSI                        [3] GPSI OPTIONAL,

pDUSessionID                [4] PDUSessionID,

timeOfFirstPacket           [5] Timestamp OPTIONAL,

timeOfLastPacket            [6] Timestamp OPTIONAL,

uplinkVolume                [7] INTEGER OPTIONAL,

downlinkVolume              [8] INTEGER OPTIONAL,

location                    [9] Location OPTIONAL,

cause                       [10] SMFErrorCodes OPTIONAL,

ePS5GSComboInfo             [11] EPS5GSComboInfo OPTIONAL,

nGAPCause                   [12] NGAPCauseInt OPTIONAL,

fiveGMMCause                [13] FiveGMMCause OPTIONAL,

pCCRuleIDs                  [14] PCCRuleIDSet OPTIONAL,

ePSPDNConnectionRelease     [15] EPSPDNConnectionRelease OPTIONAL

}

-- See clause 6.2.3.2.5 for details of this structure

SMFStartOfInterceptionWithEstablishedPDUSession ::= SEQUENCE

{

sUPI                                               [1] SUPI OPTIONAL,

sUPIUnauthenticated                                [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                                                [3] PEI OPTIONAL,

gPSI                                               [4] GPSI OPTIONAL,

pDUSessionID                                       [5] PDUSessionID,

gTPTunnelID                                        [6] FTEID,

pDUSessionType                                     [7] PDUSessionType,

sNSSAI                                             [8] SNSSAI OPTIONAL,

uEEndpoint                                         [9] SEQUENCE OF UEEndpointAddress,

non3GPPAccessEndpoint                              [10] UEEndpointAddress OPTIONAL,

location                                           [11] Location OPTIONAL,

dNN                                                [12] DNN,

aMFID                                              [13] AMFID OPTIONAL,

hSMFURI                                            [14] HSMFURI OPTIONAL,

requestType                                        [15] FiveGSMRequestType,

accessType                                         [16] AccessType OPTIONAL,

rATType                                            [17] RATType OPTIONAL,

sMPDUDNRequest                                     [18] SMPDUDNRequest OPTIONAL,

timeOfSessionEstablishment                         [19] Timestamp OPTIONAL,

ePS5GSComboInfo                                    [20] EPS5GSComboInfo OPTIONAL,

uEEPSPDNConnection                                 [21] UEEPSPDNConnection OPTIONAL,

servingNetwork                                     [22] SMFServingNetwork OPTIONAL,

gTPTunnelInfo                                      [23] GTPTunnelInfo OPTIONAL,

pCCRules                                           [24] PCCRuleSet OPTIONAL,

ePSStartOfInterceptionWithEstablishedPDNConnection [25] EPSStartOfInterceptionWithEstablishedPDNConnection OPTIONAL,

pFDDataForApps                                     [26] PFDDataForApps OPTIONAL

}

-- See clause 6.2.3.2.6 for details of this structure

SMFUnsuccessfulProcedure ::= SEQUENCE

{

failedProcedureType         [1] SMFFailedProcedureType,

failureCause                [2] FiveGSMCause,

initiator                   [3] Initiator,

requestedSlice              [4] NSSAI OPTIONAL,

sUPI                        [5] SUPI OPTIONAL,

sUPIUnauthenticated         [6] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [7] PEI OPTIONAL,

gPSI                        [8] GPSI OPTIONAL,

pDUSessionID                [9] PDUSessionID OPTIONAL,

uEEndpoint                  [10] SEQUENCE OF UEEndpointAddress OPTIONAL,

non3GPPAccessEndpoint       [11] UEEndpointAddress OPTIONAL,

dNN                         [12] DNN OPTIONAL,

aMFID                       [13] AMFID OPTIONAL,

hSMFURI                     [14] HSMFURI OPTIONAL,

requestType                 [15] FiveGSMRequestType OPTIONAL,

accessType                  [16] AccessType OPTIONAL,

rATType                     [17] RATType OPTIONAL,

sMPDUDNRequest              [18] SMPDUDNRequest OPTIONAL,

location                    [19] Location OPTIONAL

}

-- See clause 6.2.3.2.8 for details of this structure

SMFPDUtoMAPDUSessionModification ::= SEQUENCE

{

sUPI                         [1] SUPI OPTIONAL,

sUPIUnauthenticated          [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                          [3] PEI OPTIONAL,

gPSI                         [4] GPSI OPTIONAL,

sNSSAI                       [5] SNSSAI OPTIONAL,

non3GPPAccessEndpoint        [6] UEEndpointAddress OPTIONAL,

location                     [7] Location OPTIONAL,

requestType                  [8] FiveGSMRequestType,

accessType                   [9] AccessType OPTIONAL,

rATType                      [10] RATType OPTIONAL,

pDUSessionID                 [11] PDUSessionID,

requestIndication            [12] RequestIndication,

aTSSSContainer               [13] ATSSSContainer,

uEEndpoint                   [14] UEEndpointAddress OPTIONAL,

servingNetwork               [15] SMFServingNetwork OPTIONAL,

handoverState                [16] HandoverState OPTIONAL,

gTPTunnelInfo                [17] GTPTunnelInfo OPTIONAL,

ePSPDNConnectionModification [18] EPSPDNConnectionModification OPTIONAL

}

-- See clause 6.2.3.2.7.1 for details of this structure

SMFMAPDUSessionEstablishment ::= SEQUENCE

{

sUPI                          [1] SUPI OPTIONAL,

sUPIUnauthenticated           [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                           [3] PEI OPTIONAL,

gPSI                          [4] GPSI OPTIONAL,

pDUSessionID                  [5] PDUSessionID,

pDUSessionType                [6] PDUSessionType,

accessInfo                    [7] SEQUENCE OF AccessInfo,

sNSSAI                        [8] SNSSAI OPTIONAL,

uEEndpoint                    [9] SEQUENCE OF UEEndpointAddress OPTIONAL,

location                      [10] Location OPTIONAL,

dNN                           [11] DNN,

aMFID                         [12] AMFID OPTIONAL,

hSMFURI                       [13] HSMFURI OPTIONAL,

requestType                   [14] FiveGSMRequestType,

sMPDUDNRequest                [15] SMPDUDNRequest OPTIONAL,

servingNetwork                [16] SMFServingNetwork,

oldPDUSessionID               [17] PDUSessionID OPTIONAL,

mAUpgradeIndication           [18] SMFMAUpgradeIndication OPTIONAL,

ePSPDNCnxInfo                 [19] SMFEPSPDNCnxInfo OPTIONAL,

mAAcceptedIndication          [20] SMFMAAcceptedIndication,

aTSSSContainer                [21] ATSSSContainer OPTIONAL,

uEEPSPDNConnection            [22] UEEPSPDNConnection OPTIONAL,

ePS5GSComboInfo               [23] EPS5GSComboInfo OPTIONAL,

selectedDNN                   [24] DNN OPTIONAL,

handoverState                 [25] HandoverState OPTIONAL,

pCCRules                      [26] PCCRuleSet OPTIONAL,

ePSPDNConnectionEstablishment [27] EPSPDNConnectionEstablishment OPTIONAL

}

-- See clause 6.2.3.2.7.2 for details of this structure

SMFMAPDUSessionModification ::= SEQUENCE

{

sUPI                         [1] SUPI OPTIONAL,

sUPIUnauthenticated          [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                          [3] PEI OPTIONAL,

gPSI                         [4] GPSI OPTIONAL,

pDUSessionID                 [5] PDUSessionID,

accessInfo                   [6] SEQUENCE OF AccessInfo OPTIONAL,

sNSSAI                       [7] SNSSAI OPTIONAL,

location                     [8] Location OPTIONAL,

requestType                  [9] FiveGSMRequestType OPTIONAL,

servingNetwork               [10] SMFServingNetwork,

oldPDUSessionID              [11] PDUSessionID OPTIONAL,

mAUpgradeIndication          [12] SMFMAUpgradeIndication OPTIONAL,

ePSPDNCnxInfo                [13] SMFEPSPDNCnxInfo OPTIONAL,

mAAcceptedIndication         [14] SMFMAAcceptedIndication,

aTSSSContainer               [15] ATSSSContainer OPTIONAL,

uEEPSPDNConnection           [16] UEEPSPDNConnection OPTIONAL,

ePS5GSComboInfo              [17] EPS5GSComboInfo OPTIONAL,

handoverState                [18] HandoverState OPTIONAL,

pCCRules                     [19] PCCRuleSet OPTIONAL,

uPPathChange                 [20] UPPathChange OPTIONAL,

pFDDataForApp                [21] PFDDataForApp OPTIONAL,

ePSPDNConnectionModification [22] EPSPDNConnectionModification OPTIONAL

}

-- See clause 6.2.3.2.7.3 for details of this structure

SMFMAPDUSessionRelease ::= SEQUENCE

{

sUPI                        [1] SUPI,

pEI                         [2] PEI OPTIONAL,

gPSI                        [3] GPSI OPTIONAL,

pDUSessionID                [4] PDUSessionID,

timeOfFirstPacket           [5] Timestamp OPTIONAL,

timeOfLastPacket            [6] Timestamp OPTIONAL,

uplinkVolume                [7] INTEGER OPTIONAL,

downlinkVolume              [8] INTEGER OPTIONAL,

location                    [9] Location OPTIONAL,

cause                       [10] SMFErrorCodes OPTIONAL,

nGAPCause                   [11] NGAPCauseInt OPTIONAL,

fiveGMMCause                [12] FiveGMMCause OPTIONAL,

pCCRuleIDs                  [13] PCCRuleIDSet OPTIONAL,

ePSPDNConnectionRelease     [14] EPSPDNConnectionRelease OPTIONAL

}

-- See clause 6.2.3.2.7.4 for details of this structure

SMFStartOfInterceptionWithEstablishedMAPDUSession ::= SEQUENCE

{

sUPI                                               [1] SUPI OPTIONAL,

sUPIUnauthenticated                                [2] SUPIUnauthenticatedIndication OPTIONAL,

pEI                                                [3] PEI OPTIONAL,

gPSI                                               [4] GPSI OPTIONAL,

pDUSessionID                                       [5] PDUSessionID,

pDUSessionType                                     [6] PDUSessionType,

accessInfo                                         [7] SEQUENCE OF AccessInfo,

sNSSAI                                             [8] SNSSAI OPTIONAL,

uEEndpoint                                         [9] SEQUENCE OF UEEndpointAddress OPTIONAL,

location                                           [10] Location OPTIONAL,

dNN                                                [11] DNN,

aMFID                                              [12] AMFID OPTIONAL,

hSMFURI                                            [13] HSMFURI OPTIONAL,

requestType                                        [14] FiveGSMRequestType OPTIONAL,

sMPDUDNRequest                                     [15] SMPDUDNRequest OPTIONAL,

servingNetwork                                     [16] SMFServingNetwork,

oldPDUSessionID                                    [17] PDUSessionID OPTIONAL,

mAUpgradeIndication                                [18] SMFMAUpgradeIndication OPTIONAL,

ePSPDNCnxInfo                                      [19] SMFEPSPDNCnxInfo OPTIONAL,

mAAcceptedIndication                               [20] SMFMAAcceptedIndication,

aTSSSContainer                                     [21] ATSSSContainer OPTIONAL,

ePS5GSComboInfo                                    [22] EPS5GSComboInfo OPTIONAL,

uEEPSPDNConnection                                 [23] UEEPSPDNConnection OPTIONAL,

pCCRules                                           [24] PCCRuleSet OPTIONAL,

pFDDataForApps                                     [25] PFDDataForApps OPTIONAL,

ePSStartOfInterceptionWithEstablishedPDNConnection [26] EPSStartOfInterceptionWithEstablishedPDNConnection OPTIONAL

}

-- See clause 6.2.3.2.7.5 for details of this structure

SMFMAUnsuccessfulProcedure ::= SEQUENCE

{

failedProcedureType         [1] SMFFailedProcedureType,

failureCause                [2] FiveGSMCause,

requestedSlice              [3] NSSAI OPTIONAL,

initiator                   [4] Initiator,

sUPI                        [5] SUPI OPTIONAL,

sUPIUnauthenticated         [6] SUPIUnauthenticatedIndication OPTIONAL,

pEI                         [7] PEI OPTIONAL,

gPSI                        [8] GPSI OPTIONAL,

pDUSessionID                [9] PDUSessionID OPTIONAL,

accessInfo                  [10] SEQUENCE OF AccessInfo,

uEEndpoint                  [11] SEQUENCE OF UEEndpointAddress OPTIONAL,

location                    [12] Location OPTIONAL,

dNN                         [13] DNN OPTIONAL,

aMFID                       [14] AMFID OPTIONAL,

hSMFURI                     [15] HSMFURI OPTIONAL,

requestType                 [16] FiveGSMRequestType OPTIONAL,

sMPDUDNRequest              [17] SMPDUDNRequest OPTIONAL

}

-- =================

-- 5G SMF parameters

-- =================

SMFID ::= UTF8String

SMFFailedProcedureType ::= ENUMERATED

{

pDUSessionEstablishment(1),

pDUSessionModification(2),

pDUSessionRelease(3)

}

SMFServingNetwork ::= SEQUENCE

{

pLMNID  [1] PLMNID,

nID     [2] NID OPTIONAL

}

AccessInfo ::= SEQUENCE

{

accessType            [1] AccessType,

rATType               [2] RATType OPTIONAL,

gTPTunnelID           [3] FTEID,

non3GPPAccessEndpoint [4] UEEndpointAddress OPTIONAL,

establishmentStatus   [5] EstablishmentStatus,

aNTypeToReactivate    [6] AccessType OPTIONAL,

gTPTunnelInfo         [7] GTPTunnelInfo OPTIONAL

}

-- see Clause 6.1.2 of TS 24.193[44] for the details of the ATSSS container contents.

ATSSSContainer ::= OCTET STRING

DLRANTunnelInformation ::= SEQUENCE

{

dLQOSFlowTunnelInformation                    [1] QOSFlowTunnelInformation OPTIONAL,

additionalDLQOSFlowTunnelInformation          [2] QOSFlowTunnelInformationList OPTIONAL,

redundantDLQOSFlowTunnelInformation           [3] QOSFlowTunnelInformationList OPTIONAL,

additionalredundantDLQOSFlowTunnelInformation [4] QOSFlowTunnelInformationList OPTIONAL

}

EstablishmentStatus ::= ENUMERATED

{

established(0),

released(1)

}

FiveGSGTPTunnels ::= SEQUENCE

{

uLNGUUPTunnelInformation           [1] FTEID OPTIONAL,

additionalULNGUUPTunnelInformation [2] FTEIDList OPTIONAL,

dLRANTunnelInformation             [3] DLRANTunnelInformation OPTIONAL

}

FiveQI ::= INTEGER (0..255)

HandoverState ::= ENUMERATED

{

none(1),

preparing(2),

prepared(3),

completed(4),

cancelled(5)

}

NGAPCauseInt ::= SEQUENCE

{

group [1] NGAPCauseGroupInt,

value [2] NGAPCauseValueInt

}

-- Derived as described in TS 29.571 [17] clause 5.4.4.12

NGAPCauseGroupInt ::= INTEGER

NGAPCauseValueInt ::= INTEGER

SMFMAUpgradeIndication ::= BOOLEAN

-- Given in YAML encoding as defined in clause 6.1.6.2.31 of TS 29.502[16]

SMFEPSPDNCnxInfo ::= UTF8String

SMFMAAcceptedIndication ::= BOOLEAN

-- see Clause 6.1.6.3.8 of TS 29.502[16] for the details of this structure.

SMFErrorCodes ::= UTF8String

-- see Clause 6.1.6.3.2 of TS 29.502[16] for details of this structure.

UEEPSPDNConnection ::= OCTET STRING

-- see Clause 6.1.6.3.6 of TS 29.502[16] for the details of this structure.

RequestIndication ::= ENUMERATED

{

uEREQPDUSESMOD(0),

uEREQPDUSESREL(1),

pDUSESMOB(2),

nWREQPDUSESAUTH(3),

nWREQPDUSESMOD(4),

nWREQPDUSESREL(5),

eBIASSIGNMENTREQ(6),

rELDUETO5GANREQUEST(7)

}

QOSFlowTunnelInformation ::= SEQUENCE

{

uPTunnelInformation   [1] FTEID,

associatedQOSFlowList [2] QOSFlowLists

}

QOSFlowTunnelInformationList ::= SEQUENCE OF QOSFlowTunnelInformation

QOSFlowDescription ::= OCTET STRING

QOSFlowLists ::= SEQUENCE OF QOSFlowList

QOSFlowList ::= SEQUENCE

{

qFI                      [1] QFI,

qOSRules                 [2] QOSRules OPTIONAL,

eBI                      [3] EPSBearerID OPTIONAL,

qOSFlowDescription       [4] QOSFlowDescription OPTIONAL,

qOSFlowProfile           [5] QOSFlowProfile OPTIONAL,

associatedANType         [6] AccessType OPTIONAL,

defaultQOSRuleIndication [7] BOOLEAN OPTIONAL

}

QOSFlowProfile ::= SEQUENCE

{

fiveQI [1] FiveQI

}

QOSRules ::= OCTET STRING

-- See clauses 5.6.2.6-1 and 5.6.2.9-1 of TS 29.512 [89], clause table 5.6.2.5-1 of TS 29.508 [90] for the details of this structure

PCCRule ::= SEQUENCE

{

pCCRuleID                     [1] PCCRuleID OPTIONAL,

appId                         [2] UTF8String OPTIONAL,

flowInfos                     [3] FlowInformationSet OPTIONAL,

appReloc                      [4] BOOLEAN OPTIONAL,

simConnInd                    [5] BOOLEAN OPTIONAL,

simConnTerm                   [6] INTEGER OPTIONAL,

maxAllowedUpLat               [7] INTEGER OPTIONAL,

trafficRoutes                 [8] RouteToLocationSet,

trafficSteeringPolIdDl        [9] UTF8String OPTIONAL,

trafficSteeringPolIdUl        [10] UTF8String OPTIONAL,

sourceDNAI                    [11] DNAI OPTIONAL,

targetDNAI                    [12] DNAI OPTIONAL,

dNAIChangeType                [13] DNAIChangeType OPTIONAL,

sourceUEIPAddr                [14] IPAddress OPTIONAL,

targetUEIPAddr                [15] IPAddress OPTIONAL,

sourceTrafficRouting          [16] RouteToLocation OPTIONAL,

targetTrafficRouting          [17] RouteToLocation OPTIONAL,

eASIPReplaceInfos             [18] EASIPReplaceInfos OPTIONAL

}

--See clause table 5.6.2.5-1 of TS 29.508 [90] for the details of this structure.

UPPathChange ::= SEQUENCE

{

sourceDNAI                    [1] DNAI OPTIONAL,

targetDNAI                    [2] DNAI OPTIONAL,

dNAIChangeType                [3] DNAIChangeType OPTIONAL,

sourceUEIPAddr                [4] IPAddress OPTIONAL,

targetUEIPAddr                [5] IPAddress OPTIONAL,

sourceTrafficRouting          [6] RouteToLocation OPTIONAL,

targetTrafficRouting          [7] RouteToLocation OPTIONAL,

mACAddress                    [8] MACAddress OPTIONAL

}

-- See table 5.6.2.14-1 of TS 29.512 [89]

PCCRuleID ::= UTF8String

PCCRuleSet ::= SET OF PCCRule

PCCRuleIDSet ::= SET OF PCCRuleID

FlowInformationSet ::= SET OF FlowInformation

RouteToLocationSet ::= SET OF RouteToLocation

-- See table 5.6.2.14 of TS 29.512 [89]

FlowInformation ::= SEQUENCE

{

flowDescription    [1] FlowDescription OPTIONAL,

ethFlowDescription [2] EthFlowDescription OPTIONAL,

tosTrafficClass    [3] OCTET STRING (SIZE(2)) OPTIONAL,

spi                [4] OCTET STRING (SIZE(4)) OPTIONAL,

flowLabel          [5] OCTET STRING (SIZE(3)) OPTIONAL,

flowDirection      [6] FlowDirection OPTIONAL

}

-- See table 5.6.2.14 of TS 29.512 [89]

FlowDescription ::= SEQUENCE

{

sourceIPAddress       [1] IPAddressOrRangeOrAny,

destinationIPAddress  [2] IPAddressOrRangeOrAny,

sourcePortNumber      [3] PortNumber OPTIONAL,

destinationPortNumber [4] PortNumber OPTIONAL,

protocol              [5] NextLayerProtocolOrAny

}

IPAddressOrRangeOrAny ::= CHOICE

{

iPAddress      [1] IPAddress,

ipAddressRange [2] IPMask,

anyIPAddress   [3] AnyIPAddress

}

IPMask ::= SEQUENCE

{

fromIPAddress [1] IPAddress,

toIPAddress   [2] IPAddress

}

AnyIPAddress ::= ENUMERATED

{

any(1)

}

NextLayerProtocolOrAny ::= CHOICE

{

nextLayerProtocol    [1] NextLayerProtocol,

anyNextLayerProtocol [2] AnyNextLayerProtocol

}

AnyNextLayerProtocol ::= ENUMERATED

{

ip(1)

}

-- See table 5.6.2.17-1 of TS 29.514 [91]

EthFlowDescription ::= SEQUENCE

{

destMacAddress    [1] MACAddress OPTIONAL,

ethType           [2] OCTET STRING (SIZE(2)),

fDesc             [3] FlowDescription OPTIONAL,

fDir              [4] FDir OPTIONAL,

sourceMacAddress  [5] MACAddress OPTIONAL,

vlanTags          [6] SET OF VLANTag,

srcMacAddrEnd     [7] MACAddress OPTIONAL,

destMacAddrEnd    [8] MACAddress OPTIONAL

}

-- See table 5.6.2.17-1 of TS 29.514 [91]

FDir ::= ENUMERATED

{

downlink(1)

}

-- See table 5.6.2.17-1 of TS 29.514 [91]

VLANTag ::= SEQUENCE

{

priority [1] BIT STRING (SIZE(3)),

cFI      [2] BIT STRING (SIZE(1)),

vLANID   [3] BIT STRING (SIZE(12))

}

-- See table 5.6.2.14 of TS 29.512 [89]

FlowDirection ::= ENUMERATED

{

downlinkOnly(1),

uplinkOnly(2),

dowlinkAndUplink(3)

}

-- See table 5.4.2.1 of TS 29.571 [17]

DNAIChangeType ::= ENUMERATED

{

early(1),

earlyAndLate(2),

late(3)

}

-- See table 5.6.2.15 of TS 29.571 [17]

RouteToLocation ::= SEQUENCE

{

dNAI            [1] DNAI,

routeInfo       [2] RouteInfo

}

-- See table 5.4.2.1 of TS 29.571 [17]

DNAI ::= UTF8String

-- See table 5.4.4.16 of TS 29.571 [17]

RouteInfo ::= SEQUENCE

{

iPAddressTunnelEndpoint       [1] IPAddress,

uDPPortNumberTunnelEndpoint   [2] PortNumber

}

-- See clause 4.1.4.2 of TS 29.512 [89]

EASIPReplaceInfos ::= SEQUENCE

{

sourceEASAddress [1] EASServerAddress,

targetEASAddress [2] EASServerAddress

}

-- See clause 4.1.4.2 of TS 29.512 [89]

EASServerAddress ::= SEQUENCE

{

iPAddress        [1]  IPAddress,

port             [2]  PortNumber

}

-- ================================

-- PGW-C + SMF PDNConnection Events

-- ================================

EPSPDNConnectionEstablishment ::= SEQUENCE

{

ePSSubscriberIDs                   [1] EPSSubscriberIDs,

iMSIUnauthenticated                [2] IMSIUnauthenticatedIndication OPTIONAL,

defaultBearerID                    [3] EPSBearerID,

gTPTunnelInfo                      [4] GTPTunnelInfo OPTIONAL,

pDNConnectionType                  [5] PDNConnectionType,

uEEndpoints                        [6] SEQUENCE OF UEEndpointAddress OPTIONAL,

non3GPPAccessEndpoint              [7] UEEndpointAddress OPTIONAL,

location                           [8] Location OPTIONAL,

additionalLocation                 [9] Location OPTIONAL,

aPN                                [10] APN,

requestType                        [11] EPSPDNConnectionRequestType OPTIONAL,

accessType                         [12] AccessType OPTIONAL,

rATType                            [13] RATType OPTIONAL,

protocolConfigurationOptions       [14] PDNProtocolConfigurationOptions OPTIONAL,

servingNetwork                     [15] SMFServingNetwork OPTIONAL,

sMPDUDNRequest                     [16] SMPDUDNRequest OPTIONAL,

bearerContextsCreated              [17] SEQUENCE OF EPSBearerContextCreated,

bearerContextsMarkedForRemoval     [18] SEQUENCE OF EPSBearerContextForRemoval OPTIONAL,

indicationFlags                    [19] PDNConnectionIndicationFlags OPTIONAL,

handoverIndication                 [20] PDNHandoverIndication OPTIONAL,

nBIFOMSupport                      [21] PDNNBIFOMSupport OPTIONAL,

fiveGSInterworkingInfo             [22] FiveGSInterworkingInfo OPTIONAL,

cSRMFI                             [23] CSRMFI OPTIONAL,

restorationOfPDNConnectionsSupport [24] RestorationOfPDNConnectionsSupport OPTIONAL,

pGWChangeIndication                [25] PGWChangeIndication OPTIONAL,

pGWRNSI                            [26] PGWRNSI OPTIONAL

}

EPSPDNConnectionModification ::= SEQUENCE

{

ePSSubscriberIDs                   [1] EPSSubscriberIDs,

iMSIUnauthenticated                [2] IMSIUnauthenticatedIndication OPTIONAL,

defaultBearerID                    [3] EPSBearerID,

gTPTunnelInfo                      [4] GTPTunnelInfo OPTIONAL,

pDNConnectionType                  [5] PDNConnectionType,

uEEndpoints                        [6] SEQUENCE OF UEEndpointAddress OPTIONAL,

non3GPPAccessEndpoint              [7] UEEndpointAddress OPTIONAL,

location                           [8] Location OPTIONAL,

additionalLocation                 [9] Location OPTIONAL,

aPN                                [10] APN,

requestType                        [11] EPSPDNConnectionRequestType OPTIONAL,

accessType                         [12] AccessType OPTIONAL,

rATType                            [13] RATType OPTIONAL,

protocolConfigurationOptions       [14] PDNProtocolConfigurationOptions OPTIONAL,

servingNetwork                     [15] SMFServingNetwork OPTIONAL,

sMPDUDNRequest                     [16] SMPDUDNRequest OPTIONAL,

bearerContextsCreated              [17] SEQUENCE OF EPSBearerContextCreated OPTIONAL,

bearerConcextsModified             [18] SEQUENCE OF EPSBearerContextModified,

bearerContextsMarkedForRemoval     [19] SEQUENCE OF EPSBearerContextForRemoval OPTIONAL,

bearersDeleted                     [20] SEQUENCE OF EPSBearersDeleted OPTIONAL,

indicationFlags                    [21] PDNConnectionIndicationFlags OPTIONAL,

handoverIndication                 [22] PDNHandoverIndication OPTIONAL,

nBIFOMSupport                      [23] PDNNBIFOMSupport OPTIONAL,

fiveGSInterworkingInfo             [24] FiveGSInterworkingInfo OPTIONAL,

cSRMFI                             [25] CSRMFI OPTIONAL,

restorationOfPDNConnectionsSupport [26] RestorationOfPDNConnectionsSupport OPTIONAL,

pGWChangeIndication                [27] PGWChangeIndication OPTIONAL,

pGWRNSI                            [28] PGWRNSI OPTIONAL

}

EPSPDNConnectionRelease ::= SEQUENCE

{

ePSSubscriberIDs    [1] EPSSubscriberIDs,

iMSIUnauthenticated [2] IMSIUnauthenticatedIndication OPTIONAL,

defaultBearerID     [3] EPSBearerID,

location            [4] Location OPTIONAL,

gTPTunnelInfo       [5] GTPTunnelInfo OPTIONAL,

rANNASCause         [6] EPSRANNASCause OPTIONAL,

pDNConnectionType   [7] PDNConnectionType,

indicationFlags     [8] PDNConnectionIndicationFlags OPTIONAL,

scopeIndication     [9] EPSPDNConnectionReleaseScopeIndication OPTIONAL,

bearersDeleted      [10] SEQUENCE OF EPSBearersDeleted OPTIONAL

}

EPSStartOfInterceptionWithEstablishedPDNConnection ::= SEQUENCE

{

ePSSubscriberIDs                   [1] EPSSubscriberIDs,

iMSIUnauthenticated                [2] IMSIUnauthenticatedIndication OPTIONAL,

defaultBearerID                    [3] EPSBearerID,

gTPTunnelInfo                      [4] GTPTunnelInfo OPTIONAL,

pDNConnectionType                  [5] PDNConnectionType,

uEEndpoints                        [6] SEQUENCE OF UEEndpointAddress OPTIONAL,

non3GPPAccessEndpoint              [7] UEEndpointAddress OPTIONAL,

location                           [8] Location OPTIONAL,

additionalLocation                 [9] Location OPTIONAL,

aPN                                [10] APN,

requestType                        [11] EPSPDNConnectionRequestType OPTIONAL,

accessType                         [12] AccessType OPTIONAL,

rATType                            [13] RATType OPTIONAL,

protocolConfigurationOptions       [14] PDNProtocolConfigurationOptions OPTIONAL,

servingNetwork                     [15] SMFServingNetwork OPTIONAL,

sMPDUDNRequest                     [16] SMPDUDNRequest OPTIONAL,

bearerContexts                     [17] SEQUENCE OF EPSBearerContext

}

PFDDataForApps ::= SET OF PFDDataForApp

PFDDataForApp ::= SEQUENCE

{

aPPId [1] UTF8String,

pFDs  [2] PFDs

}

PFDs ::= SET OF PFD

-- See table 5.6.2.5-1 of TS 29.551 [94]

PFD ::= SEQUENCE

{

pFDId                [1] UTF8String,

pFDFlowDescriptions  [2] PFDFlowDescriptions,

urls                 [3] PFDURLs,

domainNames          [4] DomainNames,

dnProtocol           [5] DnProtocol

}

PFDURLs ::= SET OF UTF8String

PFDFlowDescriptions ::= SET OF PFDFlowDescription

DomainNames ::= SET OF UTF8String

PFDFlowDescription ::= SEQUENCE

{

nextLayerProtocol [1] NextLayerProtocol,

serverIPAddress   [2] IPAddress,

serverPortNumber  [3] PortNumber

}

-- See table 5.14.2.2.4-1 of TS 29.122 [63]

DnProtocol ::= ENUMERATED

{

dnsQname(1),

tlsSni(2),

tlsSan(3),

tlsScn(4)

}

-- ======================

-- PGW-C + SMF Parameters

-- ======================

CSRMFI ::= BOOLEAN

EPS5GSComboInfo ::= SEQUENCE

{

ePSInterworkingIndication [1] EPSInterworkingIndication,

ePSSubscriberIDs          [2] EPSSubscriberIDs,

ePSPDNCnxInfo             [3] EPSPDNCnxInfo OPTIONAL,

ePSBearerInfo             [4] EPSBearerInfo OPTIONAL

}

EPSInterworkingIndication ::= ENUMERATED

{

none(1),

withN26(2),

withoutN26(3),

iwkNon3GPP(4)

}

EPSSubscriberIDs ::= SEQUENCE

{

iMSI   [1] IMSI OPTIONAL,

mSISDN [2] MSISDN OPTIONAL,

iMEI   [3] IMEI OPTIONAL

}

EPSPDNCnxInfo ::= SEQUENCE

{

pGWS8ControlPlaneFTEID [1] FTEID,

linkedBearerID         [2] EPSBearerID OPTIONAL

}

EPSBearerInfo ::= SEQUENCE OF EPSBearers

EPSBearers ::= SEQUENCE

{

ePSBearerID         [1] EPSBearerID,

pGWS8UserPlaneFTEID [2] FTEID,

qCI                 [3] QCI

}

EPSBearerContext ::= SEQUENCE

{

ePSBearerID     [1] EPSBearerID,

uPGTPTunnelInfo [2] GTPTunnelInfo,

bearerQOS       [3] EPSBearerQOS

}

EPSBearerContextCreated ::= SEQUENCE

{

ePSBearerID                  [1] EPSBearerID,

cause                        [2] EPSBearerCreationCauseValue,

gTPTunnelInfo                [3] GTPTunnelInfo OPTIONAL,

bearerQOS                    [4] EPSBearerQOS OPTIONAL,

protocolConfigurationOptions [5] PDNProtocolConfigurationOptions OPTIONAL

}

EPSBearerContextModified ::= SEQUENCE

{

ePSBearerID                  [1] EPSBearerID,

cause                        [2] EPSBearerModificationCauseValue,

gTPTunnelInfo                [3] GTPTunnelInfo OPTIONAL,

bearerQOS                    [4] EPSBearerQOS OPTIONAL,

protocolConfigurationOptions [5] PDNProtocolConfigurationOptions OPTIONAL

}

EPSBearersDeleted ::= SEQUENCE

{

linkedEPSBearerID            [1] EPSBearerID OPTIONAL,

ePSBearerIDs                 [2] SEQUENCE OF EPSBearerID OPTIONAL,

protocolConfigurationOptions [3] PDNProtocolConfigurationOptions OPTIONAL,

cause                        [4] EPSBearerDeletionCauseValue OPTIONAL,

deleteBearerResponse         [5] EPSDeleteBearerResponse

}

EPSDeleteBearerResponse ::= SEQUENCE

{

cause                        [1] EPSBearerDeletionCauseValue,

linkedEPSBearerID            [2] EPSBearerID OPTIONAL,

bearerContexts               [3] SEQUENCE OF EPSDeleteBearerContext OPTIONAL,

protocolConfigurationOptions [4] PDNProtocolConfigurationOptions OPTIONAL

}

EPSDeleteBearerContext ::= SEQUENCE

{

cause                        [1] EPSBearerDeletionCauseValue,

ePSBearerID                  [2] EPSBearerID,

protocolConfigurationOptions [3] PDNProtocolConfigurationOptions OPTIONAL,

rANNASCause                  [4] EPSRANNASCause OPTIONAL

}

EPSBearerContextForRemoval ::= SEQUENCE

{

ePSBearerID [1] EPSBearerID,

cause       [2] EPSBearerRemovalCauseValue

}

EPSBearerCreationCauseValue ::= INTEGER (0..255)

EPSBearerDeletionCauseValue ::= INTEGER (0..255)

EPSBearerModificationCauseValue ::= INTEGER (0..255)

EPSBearerRemovalCauseValue ::= INTEGER (0..255)

EPSBearerQOS ::= SEQUENCE

{

qCI                       [1] QCI OPTIONAL,

maximumUplinkBitRate      [2] BitrateBinKBPS OPTIONAL,

maximumDownlinkBitRate    [3] BitrateBinKBPS OPTIONAL,

guaranteedUplinkBitRate   [4] BitrateBinKBPS OPTIONAL,

guaranteedDownlinkBitRate [5] BitrateBinKBPS OPTIONAL,

priorityLevel             [6] EPSQOSPriority OPTIONAL

}

EPSRANNASCause ::= OCTET STRING

EPSQOSPriority ::= INTEGER (1..15)

BitrateBinKBPS ::= OCTET STRING

EPSGTPTunnels ::= SEQUENCE

{

controlPlaneSenderFTEID  [1] FTEID OPTIONAL,

controlPlanePGWS5S8FTEID [2] FTEID OPTIONAL,

s1UeNodeBFTEID           [3] FTEID OPTIONAL,

s5S8SGWFTEID             [4] FTEID OPTIONAL,

s5S8PGWFTEID             [5] FTEID OPTIONAL,

s2bUePDGFTEID            [6] FTEID OPTIONAL,

s2aUePDGFTEID            [7] FTEID OPTIONAL

}

EPSPDNConnectionRequestType ::= ENUMERATED

{

initialRequest(1),

handover(2),

rLOS(3),

emergency(4),

handoverOfEmergencyBearerServices(5),

reserved(6)

}

EPSPDNConnectionReleaseScopeIndication ::= BOOLEAN

FiveGSInterworkingInfo ::= SEQUENCE

{

fiveGSInterworkingIndicator  [1] FiveGSInterworkingIndicator,

fiveGSInterworkingWithoutN26 [2] FiveGSInterworkingWithoutN26,

fiveGCNotRestrictedSupport   [3] FiveGCNotRestrictedSupport

}

FiveGSInterworkingIndicator ::= BOOLEAN

FiveGSInterworkingWithoutN26 ::= BOOLEAN

FiveGCNotRestrictedSupport ::= BOOLEAN

PDNConnectionIndicationFlags ::= OCTET STRING

PDNHandoverIndication ::= BOOLEAN

PDNNBIFOMSupport ::= BOOLEAN

PDNProtocolConfigurationOptions ::= SEQUENCE

{

requestPCO   [1] PDNPCO OPTIONAL,

requestAPCO  [2] PDNPCO OPTIONAL,

requestEPCO  [3] PDNPCO OPTIONAL,

responsePCO  [4] PDNPCO OPTIONAL,

responseAPCO [5] PDNPCO OPTIONAL,

responseEPCO [6] PDNPCO OPTIONAL

}

PDNPCO ::= OCTET STRING

PGWChangeIndication ::= BOOLEAN

PGWRNSI ::= BOOLEAN

QCI ::= INTEGER (0..255)

GTPTunnelInfo ::= SEQUENCE

{

fiveGSGTPTunnels [1] FiveGSGTPTunnels OPTIONAL,

ePSGTPTunnels    [2] EPSGTPTunnels OPTIONAL

}

RestorationOfPDNConnectionsSupport ::= BOOLEAN

-- ==================

-- 5G UPF definitions

-- ==================

UPFCCPDU ::= OCTET STRING

-- See clause 6.2.3.8 for the details of this structure

ExtendedUPFCCPDU ::= SEQUENCE

{

payload [1] UPFCCPDUPayload,

qFI     [2] QFI OPTIONAL

}

-- =================

-- 5G UPF parameters

-- =================

UPFCCPDUPayload ::= CHOICE

{

uPFIPCC           [1] OCTET STRING,

uPFEthernetCC     [2] OCTET STRING,

uPFUnstructuredCC [3] OCTET STRING

}

QFI ::= INTEGER (0..63)

-- ==================

-- 5G UDM definitions

-- ==================

UDMServingSystemMessage ::= SEQUENCE

{

sUPI                        [1] SUPI,

pEI                         [2] PEI OPTIONAL,

gPSI                        [3] GPSI OPTIONAL,

gUAMI                       [4] GUAMI OPTIONAL,

gUMMEI                      [5] GUMMEI OPTIONAL,

pLMNID                      [6] PLMNID OPTIONAL,

servingSystemMethod         [7] UDMServingSystemMethod,

serviceID                   [8] ServiceID OPTIONAL,

roamingIndicator            [9] RoamingIndicator OPTIONAL

}

UDMSubscriberRecordChangeMessage ::= SEQUENCE

{

sUPI                           [1] SUPI OPTIONAL,

pEI                            [2] PEI OPTIONAL,

gPSI                           [3] GPSI OPTIONAL,

oldPEI                         [4] PEI OPTIONAL,

oldSUPI                        [5] SUPI OPTIONAL,

oldGPSI                        [6] GPSI OPTIONAL,

oldserviceID                   [7] ServiceID OPTIONAL,

subscriberRecordChangeMethod   [8] UDMSubscriberRecordChangeMethod,

serviceID                      [9] ServiceID OPTIONAL

}

UDMCancelLocationMessage ::= SEQUENCE

{

sUPI                        [1] SUPI,

pEI                         [2] PEI OPTIONAL,

gPSI                        [3] GPSI OPTIONAL,

gUAMI                       [4] GUAMI OPTIONAL,

pLMNID                      [5] PLMNID OPTIONAL,

cancelLocationMethod        [6] UDMCancelLocationMethod

}

UDMLocationInformationResult ::= SEQUENCE

{

sUPI                     [1] SUPI,

pEI                      [2] PEI OPTIONAL,

gPSI                     [3] GPSI OPTIONAL,

locationInfoRequest      [4] UDMLocationInfoRequest,

vPLMNID                  [5] PLMNID OPTIONAL,

currentLocationIndicator [6] BOOLEAN OPTIONAL,

aMFInstanceID            [7] NFID OPTIONAL,

sMSFInstanceID           [8] NFID OPTIONAL,

location                 [9] Location OPTIONAL,

rATType                  [10] RATType OPTIONAL,

problemDetails           [11] UDMProblemDetails OPTIONAL

}

UDMUEInformationResponse ::= SEQUENCE

{

sUPI                        [1] SUPI,

tADSInfo                    [2] UEContextInfo OPTIONAL,

fiveGSUserStateInfo         [3] FiveGSUserStateInfo OPTIONAL,

fiveGSRVCCInfo              [4] FiveGSRVCCInfo OPTIONAL,

problemDetails              [5] UDMProblemDetails OPTIONAL

}

UDMUEAuthenticationResponse ::= SEQUENCE

{

sUPI                        [1] SUPI,

authenticationInfoRequest   [2] UDMAuthenticationInfoRequest,

aKMAIndicator               [3] BOOLEAN OPTIONAL,

problemDetails              [4] UDMProblemDetails OPTIONAL

}

-- =================

-- 5G UDM parameters

-- =================

UDMServingSystemMethod ::= ENUMERATED

{

amf3GPPAccessRegistration(0),

amfNon3GPPAccessRegistration(1),

unknown(2)

}

UDMSubscriberRecordChangeMethod ::= ENUMERATED

{

pEIChange(1),

sUPIChange(2),

gPSIChange(3),

uEDeprovisioning(4),

unknown(5),

serviceIDChange(6)

}

UDMCancelLocationMethod ::= ENUMERATED

{

aMF3GPPAccessDeregistration(1),

aMFNon3GPPAccessDeregistration(2),

uDMDeregistration(3),

unknown(4)

}

ServiceID ::= SEQUENCE

{

nSSAI                     [1] NSSAI OPTIONAL,

cAGID                     [2] SEQUENCE OF CAGID OPTIONAL

}

CAGID ::= UTF8String

UDMAuthenticationInfoRequest ::= SEQUENCE

{

infoRequestType    [1] UDMInfoRequestType,

rGAuthCtx          [2] SEQUENCE SIZE(1..MAX) OF SubscriberIdentifier,

authType           [3] PrimaryAuthenticationType,

servingNetworkName [4] PLMNID,

aUSFInstanceID     [5] NFID OPTIONAL,

cellCAGInfo        [6] CAGID OPTIONAL,

n5GCIndicator      [7] BOOLEAN OPTIONAL

}

UDMLocationInfoRequest ::= SEQUENCE

{

requested5GSLocation     [1] BOOLEAN OPTIONAL,

requestedCurrentLocation [2] BOOLEAN OPTIONAL,

requestedRATType         [3] BOOLEAN OPTIONAL,

requestedTimeZone        [4] BOOLEAN OPTIONAL,

requestedServingNode     [5] BOOLEAN OPTIONAL

}

UDMProblemDetails ::= SEQUENCE

{

cause        [1] UDMProblemDetailsCause OPTIONAL

}

UDMProblemDetailsCause ::= CHOICE

{

uDMDefinedCause       [1] UDMDefinedCause,

otherCause            [2] UDMProblemDetailsOtherCause

}

UDMDefinedCause ::= ENUMERATED

{

userNotFound(1),

dataNotFound(2),

contextNotFound(3),

subscriptionNotFound(4),

other(5)

}

UDMInfoRequestType ::= ENUMERATED

{

hSS(1),

aUSF(2),

other(3)

}

UDMProblemDetailsOtherCause ::= SEQUENCE

{

problemDetailsType   [1] UTF8String OPTIONAL,

title                [2] UTF8String OPTIONAL,

status               [3] INTEGER OPTIONAL,

detail               [4] UTF8String OPTIONAL,

instance             [5] UTF8String OPTIONAL,

cause                [6] UTF8String OPTIONAL,

uDMInvalidParameters [7] UDMInvalidParameters,

uDMSupportedFeatures [8] UTF8String

}

UDMInvalidParameters ::= SEQUENCE

{

parameter    [1] UTF8String OPTIONAL,

reason       [2] UTF8String OPTIONAL

}

RoamingIndicator ::= BOOLEAN

-- ===================

-- 5G SMSF definitions

-- ===================

-- See clause 6.2.5.3 for details of this structure

SMSMessage ::= SEQUENCE

{

originatingSMSParty         [1] SMSParty,

terminatingSMSParty         [2] SMSParty,

direction                   [3] Direction,

linkTransferStatus          [4] SMSTransferStatus,

otherMessage                [5] SMSOtherMessageIndication OPTIONAL,

location                    [6] Location OPTIONAL,

peerNFAddress               [7] SMSNFAddress OPTIONAL,

peerNFType                  [8] SMSNFType OPTIONAL,

sMSTPDUData                 [9] SMSTPDUData OPTIONAL,

messageType                 [10] SMSMessageType OPTIONAL,

rPMessageReference          [11] SMSRPMessageReference OPTIONAL

}

SMSReport ::= SEQUENCE

{

location           [1] Location OPTIONAL,

sMSTPDUData        [2] SMSTPDUData,

messageType        [3] SMSMessageType,

rPMessageReference [4] SMSRPMessageReference

}

-- ==================

-- 5G SMSF parameters

-- ==================

SMSAddress ::= OCTET STRING(SIZE(2..12))

SMSMessageType ::= ENUMERATED

{

deliver(1),

deliverReportAck(2),

deliverReportError(3),

statusReport(4),

command(5),

submit(6),

submitReportAck(7),

submitReportError(8),

reserved(9)

}

SMSParty ::= SEQUENCE

{

sUPI        [1] SUPI OPTIONAL,

pEI         [2] PEI OPTIONAL,

gPSI        [3] GPSI OPTIONAL,

sMSAddress  [4] SMSAddress OPTIONAL

}

SMSTransferStatus ::= ENUMERATED

{

transferSucceeded(1),

transferFailed(2),

undefined(3)

}

SMSOtherMessageIndication ::= BOOLEAN

SMSNFAddress ::= CHOICE

{

iPAddress   [1] IPAddress,

e164Number  [2] E164Number

}

SMSNFType ::= ENUMERATED

{

sMSGMSC(1),

iWMSC(2),

sMSRouter(3)

}

SMSRPMessageReference ::= INTEGER (0..255)

SMSTPDUData ::= CHOICE

{

sMSTPDU [1] SMSTPDU,

truncatedSMSTPDU [2] TruncatedSMSTPDU

}

SMSTPDU ::= OCTET STRING (SIZE(1..270))

TruncatedSMSTPDU ::= OCTET STRING (SIZE(1..130))

-- ===============

-- MMS definitions

-- ===============

MMSSend ::= SEQUENCE

{

transactionID       [1]  UTF8String,

version             [2]  MMSVersion,

dateTime            [3]  Timestamp,

originatingMMSParty [4]  MMSParty,

terminatingMMSParty [5]  SEQUENCE OF MMSParty OPTIONAL,

cCRecipients        [6]  SEQUENCE OF MMSParty OPTIONAL,

bCCRecipients       [7]  SEQUENCE OF MMSParty OPTIONAL,

direction           [8]  MMSDirection,

subject             [9]  MMSSubject OPTIONAL,

messageClass        [10]  MMSMessageClass OPTIONAL,

expiry              [11] MMSExpiry,

desiredDeliveryTime [12] Timestamp OPTIONAL,

priority            [13] MMSPriority OPTIONAL,

senderVisibility    [14] BOOLEAN OPTIONAL,

deliveryReport      [15] BOOLEAN OPTIONAL,

readReport          [16] BOOLEAN OPTIONAL,

store               [17] BOOLEAN OPTIONAL,

state               [18] MMState OPTIONAL,

flags               [19] MMFlags OPTIONAL,

replyCharging       [20] MMSReplyCharging OPTIONAL,

applicID            [21] UTF8String OPTIONAL,

replyApplicID       [22] UTF8String OPTIONAL,

auxApplicInfo       [23] UTF8String OPTIONAL,

contentClass        [24] MMSContentClass OPTIONAL,

dRMContent          [25] BOOLEAN OPTIONAL,

adaptationAllowed   [26] MMSAdaptation OPTIONAL,

contentType         [27] MMSContentType,

responseStatus      [28] MMSResponseStatus,

responseStatusText  [29] UTF8String OPTIONAL,

messageID           [30] UTF8String

}

MMSSendByNonLocalTarget ::= SEQUENCE

{

version             [1]  MMSVersion,

transactionID       [2]  UTF8String,

messageID           [3]  UTF8String,

terminatingMMSParty [4]  SEQUENCE OF MMSParty,

originatingMMSParty [5]  MMSParty,

direction           [6]  MMSDirection,

contentType         [7]  MMSContentType,

messageClass        [8]  MMSMessageClass OPTIONAL,

dateTime            [9]  Timestamp,

expiry              [10] MMSExpiry OPTIONAL,

deliveryReport      [11] BOOLEAN OPTIONAL,

priority            [12] MMSPriority OPTIONAL,

senderVisibility    [13] BOOLEAN OPTIONAL,

readReport          [14] BOOLEAN OPTIONAL,

subject             [15] MMSSubject OPTIONAL,

forwardCount        [16] INTEGER OPTIONAL,

previouslySentBy    [17] MMSPreviouslySentBy OPTIONAL,

prevSentByDateTime  [18] Timestamp OPTIONAL,

applicID            [19] UTF8String OPTIONAL,

replyApplicID       [20] UTF8String OPTIONAL,

auxApplicInfo       [21] UTF8String OPTIONAL,

contentClass        [22] MMSContentClass OPTIONAL,

dRMContent          [23] BOOLEAN OPTIONAL,

adaptationAllowed   [24] MMSAdaptation OPTIONAL

}

MMSNotification ::= SEQUENCE

{

transactionID           [1]  UTF8String,

version                 [2]  MMSVersion,

originatingMMSParty     [3]  MMSParty OPTIONAL,

direction               [4]  MMSDirection,

subject                 [5]  MMSSubject OPTIONAL,

deliveryReportRequested [6]  BOOLEAN OPTIONAL,

stored                  [7]  BOOLEAN OPTIONAL,

messageClass            [8]  MMSMessageClass,

priority                [9]  MMSPriority OPTIONAL,

messageSize             [10]  INTEGER,

expiry                  [11] MMSExpiry,

replyCharging           [12] MMSReplyCharging OPTIONAL

}

MMSSendToNonLocalTarget ::= SEQUENCE

{

version             [1]  MMSVersion,

transactionID       [2]  UTF8String,

messageID           [3]  UTF8String,

terminatingMMSParty [4]  SEQUENCE OF MMSParty,

originatingMMSParty [5]  MMSParty,

direction           [6]  MMSDirection,

contentType         [7]  MMSContentType,

messageClass        [8]  MMSMessageClass OPTIONAL,

dateTime            [9]  Timestamp,

expiry              [10] MMSExpiry OPTIONAL,

deliveryReport      [11] BOOLEAN OPTIONAL,

priority            [12] MMSPriority OPTIONAL,

senderVisibility    [13] BOOLEAN OPTIONAL,

readReport          [14] BOOLEAN OPTIONAL,

subject             [15] MMSSubject OPTIONAL,

forwardCount        [16] INTEGER OPTIONAL,

previouslySentBy    [17] MMSPreviouslySentBy OPTIONAL,

prevSentByDateTime  [18] Timestamp OPTIONAL,

applicID            [19] UTF8String OPTIONAL,

replyApplicID       [20] UTF8String OPTIONAL,

auxApplicInfo       [21] UTF8String OPTIONAL,

contentClass        [22] MMSContentClass OPTIONAL,

dRMContent          [23] BOOLEAN OPTIONAL,

adaptationAllowed   [24] MMSAdaptation OPTIONAL

}

MMSNotificationResponse ::= SEQUENCE

{

transactionID [1] UTF8String,

version       [2] MMSVersion,

direction     [3] MMSDirection,

status        [4] MMStatus,

reportAllowed [5] BOOLEAN OPTIONAL

}

MMSRetrieval ::= SEQUENCE

{

transactionID       [1]  UTF8String,

version             [2]  MMSVersion,

messageID           [3]  UTF8String,

dateTime            [4]  Timestamp,

originatingMMSParty [5]  MMSParty OPTIONAL,

previouslySentBy    [6]  MMSPreviouslySentBy OPTIONAL,

prevSentByDateTime  [7]  Timestamp OPTIONAL,

terminatingMMSParty [8]  SEQUENCE OF MMSParty OPTIONAL,

cCRecipients        [9]  SEQUENCE OF MMSParty OPTIONAL,

direction           [10] MMSDirection,

subject             [11] MMSSubject OPTIONAL,

state               [12] MMState OPTIONAL,

flags               [13] MMFlags OPTIONAL,

messageClass        [14] MMSMessageClass OPTIONAL,

priority            [15] MMSPriority,

deliveryReport      [16] BOOLEAN OPTIONAL,

readReport          [17] BOOLEAN OPTIONAL,

replyCharging       [18] MMSReplyCharging OPTIONAL,

retrieveStatus      [19] MMSRetrieveStatus OPTIONAL,

retrieveStatusText  [20] UTF8String OPTIONAL,

applicID            [21] UTF8String OPTIONAL,

replyApplicID       [22] UTF8String OPTIONAL,

auxApplicInfo       [23] UTF8String OPTIONAL,

contentClass        [24] MMSContentClass OPTIONAL,

dRMContent          [25] BOOLEAN OPTIONAL,

replaceID           [26] UTF8String OPTIONAL,

contentType         [27] UTF8String OPTIONAL

}

MMSDeliveryAck ::= SEQUENCE

{

transactionID [1] UTF8String,

version       [2] MMSVersion,

reportAllowed [3] BOOLEAN OPTIONAL,

status        [4] MMStatus,

direction     [5] MMSDirection

}

MMSForward ::= SEQUENCE

{

transactionID         [1]  UTF8String,

version               [2]  MMSVersion,

dateTime              [3]  Timestamp OPTIONAL,

originatingMMSParty   [4]  MMSParty,

terminatingMMSParty   [5]  SEQUENCE OF MMSParty OPTIONAL,

cCRecipients          [6]  SEQUENCE OF MMSParty OPTIONAL,

bCCRecipients         [7]  SEQUENCE OF MMSParty OPTIONAL,

direction             [8]  MMSDirection,

expiry                [9]  MMSExpiry OPTIONAL,

desiredDeliveryTime   [10] Timestamp OPTIONAL,

deliveryReportAllowed [11] BOOLEAN OPTIONAL,

deliveryReport        [12] BOOLEAN OPTIONAL,

store                 [13] BOOLEAN OPTIONAL,

state                 [14] MMState OPTIONAL,

flags                 [15] MMFlags OPTIONAL,

contentLocationReq    [16] UTF8String,

replyCharging         [17] MMSReplyCharging OPTIONAL,

responseStatus        [18] MMSResponseStatus,

responseStatusText    [19] UTF8String  OPTIONAL,

messageID             [20] UTF8String OPTIONAL,

contentLocationConf   [21] UTF8String OPTIONAL,

storeStatus           [22] MMSStoreStatus OPTIONAL,

storeStatusText       [23] UTF8String OPTIONAL

}

MMSDeleteFromRelay ::= SEQUENCE

{

transactionID        [1] UTF8String,

version              [2] MMSVersion,

direction            [3] MMSDirection,

contentLocationReq   [4] SEQUENCE OF UTF8String,

contentLocationConf  [5] SEQUENCE OF UTF8String,

deleteResponseStatus [6] MMSDeleteResponseStatus,

deleteResponseText   [7] SEQUENCE OF UTF8String

}

MMSMBoxStore ::= SEQUENCE

{

transactionID       [1] UTF8String,

version             [2] MMSVersion,

direction           [3] MMSDirection,

contentLocationReq  [4] UTF8String,

state               [5] MMState OPTIONAL,

flags               [6] MMFlags OPTIONAL,

contentLocationConf [7] UTF8String OPTIONAL,

storeStatus         [8] MMSStoreStatus,

storeStatusText     [9] UTF8String OPTIONAL

}

MMSMBoxUpload ::= SEQUENCE

{

transactionID       [1]  UTF8String,

version             [2]  MMSVersion,

direction           [3]  MMSDirection,

state               [4]  MMState OPTIONAL,

flags               [5]  MMFlags OPTIONAL,

contentType         [6]  UTF8String,

contentLocation     [7]  UTF8String OPTIONAL,

storeStatus         [8]  MMSStoreStatus,

storeStatusText     [9]  UTF8String OPTIONAL,

mMessages           [10] SEQUENCE OF MMBoxDescription

}

MMSMBoxDelete ::= SEQUENCE

{

transactionID       [1] UTF8String,

version             [2] MMSVersion,

direction           [3] MMSDirection,

contentLocationReq  [4] SEQUENCE OF UTF8String,

contentLocationConf [5] SEQUENCE OF UTF8String OPTIONAL,

responseStatus      [6] MMSDeleteResponseStatus,

responseStatusText  [7] UTF8String OPTIONAL

}

MMSDeliveryReport ::= SEQUENCE

{

version             [1] MMSVersion,

messageID           [2] UTF8String,

terminatingMMSParty [3] SEQUENCE OF MMSParty,

mMSDateTime         [4] Timestamp,

responseStatus      [5] MMSResponseStatus,

responseStatusText  [6] UTF8String OPTIONAL,

applicID            [7] UTF8String OPTIONAL,

replyApplicID       [8] UTF8String OPTIONAL,

auxApplicInfo       [9] UTF8String OPTIONAL

}

MMSDeliveryReportNonLocalTarget ::= SEQUENCE

{

version             [1]  MMSVersion,

transactionID       [2]  UTF8String,

messageID           [3]  UTF8String,

terminatingMMSParty [4]  SEQUENCE OF MMSParty,

originatingMMSParty [5]  MMSParty,

direction           [6]  MMSDirection,

mMSDateTime         [7]  Timestamp,

forwardToOriginator [8]  BOOLEAN OPTIONAL,

status              [9]  MMStatus,

statusExtension     [10] MMStatusExtension,

statusText          [11] MMStatusText,

applicID            [12] UTF8String OPTIONAL,

replyApplicID       [13] UTF8String OPTIONAL,

auxApplicInfo       [14] UTF8String OPTIONAL

}

MMSReadReport ::= SEQUENCE

{

version             [1] MMSVersion,

messageID           [2] UTF8String,

terminatingMMSParty [3] SEQUENCE OF MMSParty,

originatingMMSParty [4] SEQUENCE OF MMSParty,

direction           [5] MMSDirection,

mMSDateTime         [6] Timestamp,

readStatus          [7] MMSReadStatus,

applicID            [8] UTF8String OPTIONAL,

replyApplicID       [9] UTF8String OPTIONAL,

auxApplicInfo       [10] UTF8String OPTIONAL

}

MMSReadReportNonLocalTarget ::= SEQUENCE

{

version             [1] MMSVersion,

transactionID       [2] UTF8String,

terminatingMMSParty [3] SEQUENCE OF MMSParty,

originatingMMSParty [4] SEQUENCE OF MMSParty,

direction           [5] MMSDirection,

messageID           [6] UTF8String,

mMSDateTime         [7] Timestamp,

readStatus          [8] MMSReadStatus,

readStatusText      [9] MMSReadStatusText OPTIONAL,

applicID            [10] UTF8String OPTIONAL,

replyApplicID       [11] UTF8String OPTIONAL,

auxApplicInfo       [12] UTF8String OPTIONAL

}

MMSCancel ::= SEQUENCE

{

transactionID [1] UTF8String,

version       [2] MMSVersion,

cancelID      [3] UTF8String,

direction     [4] MMSDirection

}

MMSMBoxViewRequest ::= SEQUENCE

{

transactionID   [1]  UTF8String,

version         [2]  MMSVersion,

contentLocation [3]  UTF8String OPTIONAL,

state           [4]  SEQUENCE OF MMState OPTIONAL,

flags           [5]  SEQUENCE OF MMFlags OPTIONAL,

start           [6]  INTEGER OPTIONAL,

limit           [7]  INTEGER OPTIONAL,

attributes      [8]  SEQUENCE OF UTF8String OPTIONAL,

totals          [9]  INTEGER OPTIONAL,

quotas          [10] MMSQuota OPTIONAL

}

MMSMBoxViewResponse ::= SEQUENCE

{

transactionID   [1]  UTF8String,

version         [2]  MMSVersion,

contentLocation [3]  UTF8String OPTIONAL,

state           [4]  SEQUENCE OF MMState OPTIONAL,

flags           [5]  SEQUENCE OF MMFlags OPTIONAL,

start           [6]  INTEGER OPTIONAL,

limit           [7]  INTEGER OPTIONAL,

attributes      [8]  SEQUENCE OF UTF8String OPTIONAL,

mMSTotals       [9]  BOOLEAN OPTIONAL,

mMSQuotas       [10] BOOLEAN OPTIONAL,

mMessages       [11] SEQUENCE OF MMBoxDescription

}

MMBoxDescription ::= SEQUENCE

{

contentLocation          [1]  UTF8String OPTIONAL,

messageID                [2]  UTF8String OPTIONAL,

state                    [3]  MMState OPTIONAL,

flags                    [4]  SEQUENCE OF MMFlags OPTIONAL,

dateTime                 [5]  Timestamp OPTIONAL,

originatingMMSParty      [6]  MMSParty OPTIONAL,

terminatingMMSParty      [7]  SEQUENCE OF MMSParty OPTIONAL,

cCRecipients             [8]  SEQUENCE OF MMSParty OPTIONAL,

bCCRecipients            [9]  SEQUENCE OF MMSParty OPTIONAL,

messageClass             [10] MMSMessageClass OPTIONAL,

subject                  [11] MMSSubject OPTIONAL,

priority                 [12] MMSPriority OPTIONAL,

deliveryTime             [13] Timestamp OPTIONAL,

readReport               [14] BOOLEAN OPTIONAL,

messageSize              [15] INTEGER OPTIONAL,

replyCharging            [16] MMSReplyCharging OPTIONAL,

previouslySentBy         [17] MMSPreviouslySentBy OPTIONAL,

previouslySentByDateTime [18] Timestamp OPTIONAL,

contentType              [19] UTF8String OPTIONAL

}

-- =========

-- MMS CCPDU

-- =========

MMSCCPDU ::= SEQUENCE

{

version    [1] MMSVersion,

transactionID [2] UTF8String,

mMSContent    [3] OCTET STRING

}

-- ==============

-- MMS parameters

-- ==============

MMSAdaptation ::= SEQUENCE

{

allowed   [1] BOOLEAN,

overriden [2] BOOLEAN

}

MMSCancelStatus ::= ENUMERATED

{

cancelRequestSuccessfullyReceived(1),

cancelRequestCorrupted(2)

}

MMSContentClass ::= ENUMERATED

{

text(1),

imageBasic(2),

imageRich(3),

videoBasic(4),

videoRich(5),

megaPixel(6),

contentBasic(7),

contentRich(8)

}

MMSContentType ::= UTF8String

MMSDeleteResponseStatus ::= ENUMERATED

{

ok(1),

errorUnspecified(2),

errorServiceDenied(3),

errorMessageFormatCorrupt(4),

errorSendingAddressUnresolved(5),

errorMessageNotFound(6),

errorNetworkProblem(7),

errorContentNotAccepted(8),

errorUnsupportedMessage(9),

errorTransientFailure(10),

errorTransientSendingAddressUnresolved(11),

errorTransientMessageNotFound(12),

errorTransientNetworkProblem(13),

errorTransientPartialSuccess(14),

errorPermanentFailure(15),

errorPermanentServiceDenied(16),

errorPermanentMessageFormatCorrupt(17),

errorPermanentSendingAddressUnresolved(18),

errorPermanentMessageNotFound(19),

errorPermanentContentNotAccepted(20),

errorPermanentReplyChargingLimitationsNotMet(21),

errorPermanentReplyChargingRequestNotAccepted(22),

errorPermanentReplyChargingForwardingDenied(23),

errorPermanentReplyChargingNotSupported(24),

errorPermanentAddressHidingNotSupported(25),

errorPermanentLackOfPrepaid(26)

}

MMSDirection ::= ENUMERATED

{

fromTarget(0),

toTarget(1)

}

MMSElementDescriptor ::= SEQUENCE

{

reference [1] UTF8String,

parameter [2] UTF8String     OPTIONAL,

value     [3] UTF8String     OPTIONAL

}

MMSExpiry ::= SEQUENCE

{

expiryPeriod [1] INTEGER,

periodFormat [2] MMSPeriodFormat

}

MMFlags ::= SEQUENCE

{

length     [1] INTEGER,

flag       [2] MMStateFlag,

flagString [3] UTF8String

}

MMSMessageClass ::= ENUMERATED

{

personal(1),

advertisement(2),

informational(3),

auto(4)

}

MMSParty ::= SEQUENCE

{

mMSPartyIDs [1] SEQUENCE OF MMSPartyID,

nonLocalID  [2] NonLocalID

}

MMSPartyID ::= CHOICE

{

e164Number   [1] E164Number,

emailAddress [2] EmailAddress,

iMSI         [3] IMSI,

iMPU         [4] IMPU,

iMPI         [5] IMPI,

sUPI         [6] SUPI,

gPSI         [7] GPSI

}

MMSPeriodFormat ::= ENUMERATED

{

absolute(1),

relative(2)

}

MMSPreviouslySent ::= SEQUENCE

{

previouslySentByParty [1] MMSParty,

sequenceNumber        [2] INTEGER,

previousSendDateTime  [3] Timestamp

}

MMSPreviouslySentBy ::= SEQUENCE OF MMSPreviouslySent

MMSPriority ::= ENUMERATED

{

low(1),

normal(2),

high(3)

}

MMSQuota ::= SEQUENCE

{

quota     [1] INTEGER,

quotaUnit [2] MMSQuotaUnit

}

MMSQuotaUnit ::= ENUMERATED

{

numMessages(1),

bytes(2)

}

MMSReadStatus ::= ENUMERATED

{

read(1),

deletedWithoutBeingRead(2)

}

MMSReadStatusText ::= UTF8String

MMSReplyCharging ::= ENUMERATED

{

requested(0),

requestedTextOnly(1),

accepted(2),

acceptedTextOnly(3)

}

MMSResponseStatus ::= ENUMERATED

{

ok(1),

errorUnspecified(2),

errorServiceDenied(3),

errorMessageFormatCorrupt(4),

errorSendingAddressUnresolved(5),

errorMessageNotFound(6),

errorNetworkProblem(7),

errorContentNotAccepted(8),

errorUnsupportedMessage(9),

errorTransientFailure(10),

errorTransientSendingAddressUnresolved(11),

errorTransientMessageNotFound(12),

errorTransientNetworkProblem(13),

errorTransientPartialSuccess(14),

errorPermanentFailure(15),

errorPermanentServiceDenied(16),

errorPermanentMessageFormatCorrupt(17),

errorPermanentSendingAddressUnresolved(18),

errorPermanentMessageNotFound(19),

errorPermanentContentNotAccepted(20),

errorPermanentReplyChargingLimitationsNotMet(21),

errorPermanentReplyChargingRequestNotAccepted(22),

errorPermanentReplyChargingForwardingDenied(23),

errorPermanentReplyChargingNotSupported(24),

errorPermanentAddressHidingNotSupported(25),

errorPermanentLackOfPrepaid(26)

}

MMSRetrieveStatus ::= ENUMERATED

{

success(1),

errorTransientFailure(2),

errorTransientMessageNotFound(3),

errorTransientNetworkProblem(4),

errorPermanentFailure(5),

errorPermanentServiceDenied(6),

errorPermanentMessageNotFound(7),

errorPermanentContentUnsupported(8)

}

MMSStoreStatus ::= ENUMERATED

{

success(1),

errorTransientFailure(2),

errorTransientNetworkProblem(3),

errorPermanentFailure(4),

errorPermanentServiceDenied(5),

errorPermanentMessageFormatCorrupt(6),

errorPermanentMessageNotFound(7),

errorMMBoxFull(8)

}

MMState ::= ENUMERATED

{

draft(1),

sent(2),

new(3),

retrieved(4),

forwarded(5)

}

MMStateFlag ::= ENUMERATED

{

add(1),

remove(2),

filter(3)

}

MMStatus ::= ENUMERATED

{

expired(1),

retrieved(2),

rejected(3),

deferred(4),

unrecognized(5),

indeterminate(6),

forwarded(7),

unreachable(8)

}

MMStatusExtension ::= ENUMERATED

{

rejectionByMMSRecipient(0),

rejectionByOtherRS(1)

}

MMStatusText ::= UTF8String

MMSSubject ::= UTF8String

MMSVersion ::= SEQUENCE

{

majorVersion [1] INTEGER,

minorVersion [2] INTEGER

}

-- ==================

-- 5G PTC definitions

-- ==================

PTCRegistration  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCServerURI                  [2] UTF8String,

pTCRegistrationRequest        [3] PTCRegistrationRequest,

pTCRegistrationOutcome        [4] PTCRegistrationOutcome

}

PTCSessionInitiation  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCServerURI                  [3] UTF8String,

pTCSessionInfo                [4] PTCSessionInfo,

pTCOriginatingID              [5] PTCTargetInformation,

pTCParticipants               [6] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCParticipantPresenceStatus  [7] MultipleParticipantPresenceStatus OPTIONAL,

location                      [8] Location OPTIONAL,

pTCBearerCapability           [9] UTF8String OPTIONAL,

pTCHost                       [10] PTCTargetInformation OPTIONAL

}

PTCSessionAbandon  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

location                      [4] Location OPTIONAL,

pTCAbandonCause               [5] INTEGER

}

PTCSessionStart  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCServerURI                  [3] UTF8String,

pTCSessionInfo                [4] PTCSessionInfo,

pTCOriginatingID              [5] PTCTargetInformation,

pTCParticipants               [6] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCParticipantPresenceStatus  [7] MultipleParticipantPresenceStatus OPTIONAL,

location                      [8] Location OPTIONAL,

pTCHost                       [9] PTCTargetInformation OPTIONAL,

pTCBearerCapability           [10] UTF8String OPTIONAL

}

PTCSessionEnd  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCServerURI                  [3] UTF8String,

pTCSessionInfo                [4] PTCSessionInfo,

pTCParticipants               [5] SEQUENCE OF PTCTargetInformation OPTIONAL,

location                      [6] Location OPTIONAL,

pTCSessionEndCause            [7] PTCSessionEndCause

}

PTCStartOfInterception  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

preEstSessionID               [3] PTCSessionInfo OPTIONAL,

pTCOriginatingID              [4] PTCTargetInformation,

pTCSessionInfo                [5] PTCSessionInfo OPTIONAL,

pTCHost                       [6] PTCTargetInformation OPTIONAL,

pTCParticipants               [7] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCMediaStreamAvail           [8] BOOLEAN OPTIONAL,

pTCBearerCapability           [9] UTF8String OPTIONAL

}

PTCPreEstablishedSession  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCServerURI                  [2] UTF8String,

rTPSetting                    [3] RTPSetting,

pTCMediaCapability            [4] UTF8String,

pTCPreEstSessionID            [5] PTCSessionInfo,

pTCPreEstStatus               [6] PTCPreEstStatus,

pTCMediaStreamAvail           [7] BOOLEAN OPTIONAL,

location                      [8] Location OPTIONAL,

pTCFailureCode                [9] PTCFailureCode OPTIONAL

}

PTCInstantPersonalAlert  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCIPAPartyID                 [2] PTCTargetInformation,

pTCIPADirection               [3] Direction

}

PTCPartyJoin  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

pTCParticipants               [4] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCParticipantPresenceStatus  [5] MultipleParticipantPresenceStatus OPTIONAL,

pTCMediaStreamAvail           [6] BOOLEAN OPTIONAL,

pTCBearerCapability           [7] UTF8String OPTIONAL

}

PTCPartyDrop  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

pTCPartyDrop                  [4] PTCTargetInformation,

pTCParticipantPresenceStatus  [5] PTCParticipantPresenceStatus OPTIONAL

}

PTCPartyHold  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

pTCParticipants               [4] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCHoldID                     [5] SEQUENCE OF PTCTargetInformation,

pTCHoldRetrieveInd            [6] BOOLEAN

}

PTCMediaModification  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessionInfo                [3] PTCSessionInfo,

pTCMediaStreamAvail           [4] BOOLEAN OPTIONAL,

pTCBearerCapability           [5] UTF8String

}

PTCGroupAdvertisement  ::=SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCIDList                     [3] SEQUENCE OF PTCTargetInformation OPTIONAL,

pTCGroupAuthRule              [4] PTCGroupAuthRule OPTIONAL,

pTCGroupAdSender              [5] PTCTargetInformation,

pTCGroupNickname              [6] UTF8String OPTIONAL

}

PTCFloorControl  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCSessioninfo                [3] PTCSessionInfo,

pTCFloorActivity              [4] SEQUENCE OF PTCFloorActivity,

pTCFloorSpeakerID             [5] PTCTargetInformation OPTIONAL,

pTCMaxTBTime                  [6] INTEGER OPTIONAL,

pTCQueuedFloorControl         [7] BOOLEAN OPTIONAL,

pTCQueuedPosition             [8] INTEGER OPTIONAL,

pTCTalkBurstPriority          [9] PTCTBPriorityLevel OPTIONAL,

pTCTalkBurstReason            [10] PTCTBReasonCode OPTIONAL

}

PTCTargetPresence  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCTargetPresenceStatus       [2] PTCParticipantPresenceStatus

}

PTCParticipantPresence  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCParticipantPresenceStatus  [2] PTCParticipantPresenceStatus

}

PTCListManagement  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCListManagementType         [3] PTCListManagementType OPTIONAL,

pTCListManagementAction       [4] PTCListManagementAction OPTIONAL,

pTCListManagementFailure      [5] PTCListManagementFailure OPTIONAL,

pTCContactID                  [6] PTCTargetInformation OPTIONAL,

pTCIDList                     [7] SEQUENCE OF PTCIDList OPTIONAL,

pTCHost                       [8] PTCTargetInformation OPTIONAL

}

PTCAccessPolicy  ::= SEQUENCE

{

pTCTargetInformation          [1] PTCTargetInformation,

pTCDirection                  [2] Direction,

pTCAccessPolicyType           [3] PTCAccessPolicyType OPTIONAL,

pTCUserAccessPolicy           [4] PTCUserAccessPolicy OPTIONAL,

pTCGroupAuthRule              [5] PTCGroupAuthRule OPTIONAL,

pTCContactID                  [6] PTCTargetInformation OPTIONAL,

pTCAccessPolicyFailure        [7] PTCAccessPolicyFailure OPTIONAL

}

-- =========

-- PTC CCPDU

-- =========

PTCCCPDU ::= OCTET STRING

-- =================

-- 5G PTC parameters

-- =================

PTCRegistrationRequest  ::= ENUMERATED

{

register(1),

reRegister(2),

deRegister(3)

}

PTCRegistrationOutcome  ::= ENUMERATED

{

success(1),

failure(2)

}

PTCSessionEndCause  ::= ENUMERATED

{

initiaterLeavesSession(1),

definedParticipantLeaves(2),

numberOfParticipants(3),

sessionTimerExpired(4),

pTCSpeechInactive(5),

allMediaTypesInactive(6)

}

PTCTargetInformation  ::= SEQUENCE

{

identifiers                [1] SEQUENCE SIZE(1..MAX) OF PTCIdentifiers

}

PTCIdentifiers  ::= CHOICE

{

mCPTTID                    [1] UTF8String,

instanceIdentifierURN      [2] UTF8String,

pTCChatGroupID             [3] PTCChatGroupID,

iMPU                       [4] IMPU,

iMPI                       [5] IMPI

}

PTCSessionInfo  ::= SEQUENCE

{

pTCSessionURI              [1] UTF8String,

pTCSessionType             [2] PTCSessionType

}

PTCSessionType  ::= ENUMERATED

{

ondemand(1),

preEstablished(2),

adhoc(3),

prearranged(4),

groupSession(5)

}

MultipleParticipantPresenceStatus  ::= SEQUENCE OF PTCParticipantPresenceStatus

PTCParticipantPresenceStatus  ::= SEQUENCE

{

presenceID                 [1] PTCTargetInformation,

presenceType               [2] PTCPresenceType,

presenceStatus             [3] BOOLEAN

}

PTCPresenceType  ::= ENUMERATED

{

pTCClient(1),

pTCGroup(2)

}

PTCPreEstStatus  ::= ENUMERATED

{

established(1),

modified(2),

released(3)

}

RTPSetting  ::= SEQUENCE

{

iPAddress                  [1] IPAddress,

portNumber                 [2] PortNumber

}

PTCIDList  ::= SEQUENCE

{

pTCPartyID                 [1] PTCTargetInformation,

pTCChatGroupID             [2] PTCChatGroupID

}

PTCChatGroupID  ::= SEQUENCE

{

groupIdentity              [1] UTF8String

}

PTCFloorActivity  ::= ENUMERATED

{

tBCPRequest(1),

tBCPGranted(2),

tBCPDeny(3),

tBCPIdle(4),

tBCPTaken(5),

tBCPRevoke(6),

tBCPQueued(7),

tBCPRelease(8)

}

PTCTBPriorityLevel  ::= ENUMERATED

{

preEmptive(1),

highPriority(2),

normalPriority(3),

listenOnly(4)

}

PTCTBReasonCode  ::= ENUMERATED

{

noQueuingAllowed(1),

oneParticipantSession(2),

listenOnly(3),

exceededMaxDuration(4),

tBPrevented(5)

}

PTCListManagementType  ::= ENUMERATED

{

contactListManagementAttempt(1),

groupListManagementAttempt(2),

contactListManagementResult(3),

groupListManagementResult(4),

requestUnsuccessful(5)

}

PTCListManagementAction  ::= ENUMERATED

{

create(1),

modify(2),

retrieve(3),

delete(4),

notify(5)

}

PTCAccessPolicyType  ::= ENUMERATED

{

pTCUserAccessPolicyAttempt(1),

groupAuthorizationRulesAttempt(2),

pTCUserAccessPolicyQuery(3),

groupAuthorizationRulesQuery(4),

pTCUserAccessPolicyResult(5),

groupAuthorizationRulesResult(6),

requestUnsuccessful(7)

}

PTCUserAccessPolicy  ::= ENUMERATED

{

allowIncomingPTCSessionRequest(1),

blockIncomingPTCSessionRequest(2),

allowAutoAnswerMode(3),

allowOverrideManualAnswerMode(4)

}

PTCGroupAuthRule  ::= ENUMERATED

{

allowInitiatingPTCSession(1),

blockInitiatingPTCSession(2),

allowJoiningPTCSession(3),

blockJoiningPTCSession(4),

allowAddParticipants(5),

blockAddParticipants(6),

allowSubscriptionPTCSessionState(7),

blockSubscriptionPTCSessionState(8),

allowAnonymity(9),

forbidAnonymity(10)

}

PTCFailureCode  ::= ENUMERATED

{

sessionCannotBeEstablished(1),

sessionCannotBeModified(2)

}

PTCListManagementFailure  ::= ENUMERATED

{

requestUnsuccessful(1),

requestUnknown(2)

}

PTCAccessPolicyFailure  ::= ENUMERATED

{

requestUnsuccessful(1),

requestUnknown(2)

}

-- ===============

-- IMS definitions

-- ===============

-- See clause 7.12.4.2.1 for details of this structure

IMSMessage ::= SEQUENCE

{

payload               [1] IMSPayload,

sessionDirection      [2] SessionDirection,

voIPRoamingIndication [3] VoIPRoamingIndication OPTIONAL,

location              [6] Location OPTIONAL

}

-- See clause 7.12.4.2.2 for details of this structure

StartOfInterceptionForActiveIMSSession ::= SEQUENCE

{

originatingId         [1] SEQUENCE OF IMPU,

terminatingId         [2] IMPU,

sDPState              [3] SEQUENCE OF OCTET STRING OPTIONAL,

diversionIdentity     [4] IMPU OPTIONAL,

voIPRoamingIndication [5] VoIPRoamingIndication OPTIONAL,

location              [7] Location OPTIONAL

}

-- See clause 7.12.4.2.3 for the details.

IMSCCUnavailable ::= SEQUENCE

{

cCUnavailableReason   [1] UTF8String,

sDPState              [2] OCTET STRING OPTIONAL

}

-- =========

-- IMS CCPDU

-- =========

IMSCCPDU ::= SEQUENCE

{

payload [1] IMSCCPDUPayload,

sDPInfo [2] OCTET STRING OPTIONAL

}

IMSCCPDUPayload ::= OCTET STRING

-- ==============

-- IMS parameters

-- ==============

IMSPayload ::= CHOICE

{

encapsulatedSIPMessage            [1] SIPMessage

}

SIPMessage ::= SEQUENCE

{

iPSourceAddress       [1] IPAddress,

iPDestinationAddress  [2] IPAddress,

sIPContent            [3] OCTET STRING

}

VoIPRoamingIndication ::= ENUMERATED

{

roamingLBO(1),

roamingS8HR(2),

roamingN9HR(3)

}

SessionDirection ::= ENUMERATED

{

fromTarget(1),

toTarget(2),

combined(3),

indeterminate(4)

}

HeaderOnlyIndication ::= BOOLEAN

-- =================================

-- STIR/SHAKEN/RCD/eCNAM definitions

-- =================================

-- See clause 7.11.2.1.2 for details of this structure

STIRSHAKENSignatureGeneration ::= SEQUENCE

{

pASSporTs                 [1] SEQUENCE OF PASSporT,

encapsulatedSIPMessage    [2] SIPMessage OPTIONAL

}

-- See clause 7.11.2.1.3 for details of this structure

STIRSHAKENSignatureValidation ::= SEQUENCE

{

pASSporTs                 [1] SEQUENCE OF PASSporT OPTIONAL,

rCDTerminalDisplayInfo    [2] RCDDisplayInfo OPTIONAL,

eCNAMTerminalDisplayInfo  [3] ECNAMDisplayInfo OPTIONAL,

sHAKENValidationResult    [4] SHAKENValidationResult,

sHAKENFailureStatusCode   [5] SHAKENFailureStatusCode OPTIONAL,

encapsulatedSIPMessage    [6] SIPMessage OPTIONAL

}

-- ================================

-- STIR/SHAKEN/RCD/eCNAM parameters

-- ================================

PASSporT ::= SEQUENCE

{

pASSporTHeader    [1] PASSporTHeader,

pASSporTPayload   [2] PASSporTPayload,

pASSporTSignature [3] OCTET STRING

}

PASSporTHeader ::= SEQUENCE

{

type          [1] JWSTokenType,

algorithm     [2] UTF8String,

ppt           [3] UTF8String OPTIONAL,

x5u           [4] UTF8String

}

JWSTokenType ::= ENUMERATED

{

passport(1)

}

PASSporTPayload ::= SEQUENCE

{

issuedAtTime    [1] GeneralizedTime,

originator      [2] STIRSHAKENOriginator,

destination     [3] STIRSHAKENDestinations,

attestation     [4] Attestation,

origId          [5] UTF8String,

diversion       [6] STIRSHAKENDestination

}

STIRSHAKENOriginator ::= CHOICE

{

telephoneNumber [1] STIRSHAKENTN,

sTIRSHAKENURI   [2] UTF8String

}

STIRSHAKENDestinations ::= SEQUENCE OF STIRSHAKENDestination

STIRSHAKENDestination ::= CHOICE

{

telephoneNumber [1] STIRSHAKENTN,

sTIRSHAKENURI   [2] UTF8String

}

STIRSHAKENTN ::= CHOICE

{

mSISDN [1] MSISDN

}

Attestation ::= ENUMERATED

{

attestationA(1),

attestationB(2),

attestationC(3)

}

SHAKENValidationResult ::= ENUMERATED

{

tNValidationPassed(1),

tNValidationFailed(2),

noTNValidation(3)

}

SHAKENFailureStatusCode ::= INTEGER

ECNAMDisplayInfo ::= SEQUENCE

{

name           [1] UTF8String,

additionalInfo [2] OCTET STRING OPTIONAL

}

RCDDisplayInfo ::= SEQUENCE

{

name [1] UTF8String,

jcd  [2] OCTET STRING OPTIONAL,

jcl  [3] OCTET STRING OPTIONAL

}

-- ===================

-- 5G LALS definitions

-- ===================

LALSReport ::= SEQUENCE

{

sUPI                [1] SUPI OPTIONAL,

--  pEI                 [2] PEI OPTIONAL, deprecated in Release-16, do not re-use this tag number

gPSI                [3] GPSI OPTIONAL,

location            [4] Location OPTIONAL,

iMPU                [5] IMPU OPTIONAL,

iMSI                [7] IMSI OPTIONAL,

mSISDN              [8] MSISDN OPTIONAL

}

-- =====================

-- PDHR/PDSR definitions

-- =====================

PDHeaderReport ::= SEQUENCE

{

pDUSessionID                [1] PDUSessionID,

sourceIPAddress             [2] IPAddress,

sourcePort                  [3] PortNumber OPTIONAL,

destinationIPAddress        [4] IPAddress,

destinationPort             [5] PortNumber OPTIONAL,

nextLayerProtocol           [6] NextLayerProtocol,

iPv6flowLabel               [7] IPv6FlowLabel OPTIONAL,

direction                   [8] Direction,

packetSize                  [9] INTEGER

}

PDSummaryReport ::= SEQUENCE

{

pDUSessionID                [1] PDUSessionID,

sourceIPAddress             [2] IPAddress,

sourcePort                  [3] PortNumber OPTIONAL,

destinationIPAddress        [4] IPAddress,

destinationPort             [5] PortNumber OPTIONAL,

nextLayerProtocol           [6] NextLayerProtocol,

iPv6flowLabel               [7] IPv6FlowLabel OPTIONAL,

direction                   [8] Direction,

pDSRSummaryTrigger          [9] PDSRSummaryTrigger,

firstPacketTimestamp        [10] Timestamp,

lastPacketTimestamp         [11] Timestamp,

packetCount                 [12] INTEGER,

byteCount                   [13] INTEGER,

perSessionTrigger           [14] BOOLEAN

}

-- ====================

-- PDHR/PDSR parameters

-- ====================

PDSRSummaryTrigger ::= ENUMERATED

{

timerExpiry(1),

packetCount(2),

byteCount(3),

startOfFlow(4),

endOfFlow(5)

}

-- ==================================

-- Identifier Association definitions

-- ==================================

AMFIdentifierAssociation ::= SEQUENCE

{

sUPI             [1] SUPI,

sUCI             [2] SUCI OPTIONAL,

pEI              [3] PEI OPTIONAL,

gPSI             [4] GPSI OPTIONAL,

gUTI             [5] FiveGGUTI,

location         [6] Location,

fiveGSTAIList    [7] TAIList OPTIONAL

}

MMEIdentifierAssociation ::= SEQUENCE

{

iMSI        [1] IMSI,

iMEI        [2] IMEI OPTIONAL,

mSISDN      [3] MSISDN OPTIONAL,

gUTI        [4] GUTI,

location    [5] Location,

tAIList     [6] TAIList OPTIONAL

}

-- =================================

-- Identifier Association parameters

-- =================================

MMEGroupID ::= OCTET STRING (SIZE(2))

MMECode ::= OCTET STRING (SIZE(1))

TMSI ::= OCTET STRING (SIZE(4))

-- ===================

-- EPS MME definitions

-- ===================

MMEAttach ::= SEQUENCE

{

attachType       [1] EPSAttachType,

attachResult     [2] EPSAttachResult,

iMSI             [3] IMSI,

iMEI             [4] IMEI OPTIONAL,

mSISDN           [5] MSISDN OPTIONAL,

gUTI             [6] GUTI OPTIONAL,

location         [7] Location OPTIONAL,

ePSTAIList       [8] TAIList OPTIONAL,

sMSServiceStatus [9] EPSSMSServiceStatus OPTIONAL,

oldGUTI          [10] GUTI OPTIONAL,

eMM5GRegStatus   [11] EMM5GMMStatus OPTIONAL

}

MMEDetach ::= SEQUENCE

{

detachDirection    [1] MMEDirection,

detachType         [2] EPSDetachType,

iMSI               [3] IMSI,

iMEI               [4] IMEI OPTIONAL,

mSISDN             [5] MSISDN OPTIONAL,

gUTI               [6] GUTI OPTIONAL,

cause              [7] EMMCause OPTIONAL,

location           [8] Location OPTIONAL,

switchOffIndicator [9] SwitchOffIndicator OPTIONAL

}

MMELocationUpdate ::= SEQUENCE

{

iMSI             [1] IMSI,

iMEI             [2] IMEI OPTIONAL,

mSISDN           [3] MSISDN OPTIONAL,

gUTI             [4] GUTI OPTIONAL,

location         [5] Location OPTIONAL,

oldGUTI          [6] GUTI OPTIONAL,

sMSServiceStatus [7] EPSSMSServiceStatus OPTIONAL

}

MMEStartOfInterceptionWithEPSAttachedUE ::= SEQUENCE

{

attachType         [1] EPSAttachType,

attachResult       [2] EPSAttachResult,

iMSI               [3] IMSI,

iMEI               [4] IMEI OPTIONAL,

mSISDN             [5] MSISDN OPTIONAL,

gUTI               [6] GUTI OPTIONAL,

location           [7] Location OPTIONAL,

ePSTAIList         [9] TAIList OPTIONAL,

sMSServiceStatus   [10] EPSSMSServiceStatus OPTIONAL,

eMM5GRegStatus     [12] EMM5GMMStatus OPTIONAL

}

MMEUnsuccessfulProcedure ::= SEQUENCE

{

failedProcedureType [1] MMEFailedProcedureType,

failureCause        [2] MMEFailureCause,

iMSI                [3] IMSI OPTIONAL,

iMEI                [4] IMEI OPTIONAL,

mSISDN              [5] MSISDN OPTIONAL,

gUTI                [6] GUTI OPTIONAL,

location            [7] Location OPTIONAL

}

-- See clause 6.3.2.2.8 for details of this structure

MMEPositioningInfoTransfer ::= SEQUENCE

{

iMSI                [1] IMSI,

iMEI                [2] IMEI OPTIONAL,

mSISDN              [3] MSISDN OPTIONAL,

gUTI                [4] GUTI OPTIONAL,

lPPaMessage         [5] OCTET STRING OPTIONAL,

lPPMessage          [6] OCTET STRING OPTIONAL,

mMELCSCorrelationId [7] OCTET STRING (SIZE(4))

}

-- ==================

-- EPS MME parameters

-- ==================

EMMCause ::= INTEGER (0..255)

ESMCause ::= INTEGER (0..255)

EPSAttachType ::= ENUMERATED

{

ePSAttach(1),

combinedEPSIMSIAttach(2),

ePSRLOSAttach(3),

ePSEmergencyAttach(4),

reserved(5)

}

EPSAttachResult ::= ENUMERATED

{

ePSOnly(1),

combinedEPSIMSI(2)

}

EPSDetachType ::= ENUMERATED

{

ePSDetach(1),

iMSIDetach(2),

combinedEPSIMSIDetach(3),

reAttachRequired(4),

reAttachNotRequired(5),

reserved(6)

}

EPSSMSServiceStatus ::= ENUMERATED

{

sMSServicesNotAvailable(1),

sMSServicesNotAvailableInThisPLMN(2),

networkFailure(3),

congestion(4)

}

MMEDirection ::= ENUMERATED

{

networkInitiated(1),

uEInitiated(2)

}

MMEFailedProcedureType ::= ENUMERATED

{

attachReject(1),

authenticationReject(2),

securityModeReject(3),

serviceReject(4),

trackingAreaUpdateReject(5),

activateDedicatedEPSBearerContextReject(6),

activateDefaultEPSBearerContextReject(7),

bearerResourceAllocationReject(8),

bearerResourceModificationReject(9),

modifyEPSBearerContectReject(10),

pDNConnectivityReject(11),

pDNDisconnectReject(12)

}

MMEFailureCause ::= CHOICE

{

eMMCause [1] EMMCause,

eSMCause [2] ESMCause

}

-- ===========================

-- LI Notification definitions

-- ===========================

LINotification ::= SEQUENCE

{

notificationType                    [1] LINotificationType,

appliedTargetID                     [2] TargetIdentifier OPTIONAL,

appliedDeliveryInformation          [3] SEQUENCE OF LIAppliedDeliveryInformation OPTIONAL,

appliedStartTime                    [4] Timestamp OPTIONAL,

appliedEndTime                      [5] Timestamp OPTIONAL

}

-- ==========================

-- LI Notification parameters

-- ==========================

LINotificationType ::= ENUMERATED

{

activation(1),

deactivation(2),

modification(3)

}

LIAppliedDeliveryInformation ::= SEQUENCE

{

hI2DeliveryIPAddress                [1] IPAddress OPTIONAL,

hI2DeliveryPortNumber               [2] PortNumber OPTIONAL,

hI3DeliveryIPAddress                [3] IPAddress OPTIONAL,

hI3DeliveryPortNumber               [4] PortNumber OPTIONAL

}

-- ===============

-- MDF definitions

-- ===============

MDFCellSiteReport ::= SEQUENCE OF CellInformation

-- ==============================

-- 5G EPS Interworking Parameters

-- ==============================

EMM5GMMStatus ::= SEQUENCE

{

eMMRegStatus  [1] EMMRegStatus OPTIONAL,

fiveGMMStatus [2] FiveGMMStatus OPTIONAL

}

EPS5GGUTI ::= CHOICE

{

gUTI      [1] GUTI,

fiveGGUTI [2] FiveGGUTI

}

EMMRegStatus ::= ENUMERATED

{

uEEMMRegistered(1),

uENotEMMRegistered(2)

}

FiveGMMStatus ::= ENUMERATED

{

uE5GMMRegistered(1),

uENot5GMMRegistered(2)

}

-- ========================================

-- Separated Location Reporting definitions

-- ========================================

SeparatedLocationReporting ::= SEQUENCE

{

sUPI                        [1] SUPI,

sUCI                        [2] SUCI OPTIONAL,

pEI                         [3] PEI OPTIONAL,

gPSI                        [4] GPSI OPTIONAL,

gUTI                        [5] FiveGGUTI OPTIONAL,

location                    [6] Location,

non3GPPAccessEndpoint       [7] UEEndpointAddress OPTIONAL,

rATType                     [8] RATType OPTIONAL

}

-- =================

-- Common Parameters

-- =================

AccessType ::= ENUMERATED

{

threeGPPAccess(1),

nonThreeGPPAccess(2),

threeGPPandNonThreeGPPAccess(3)

}

Direction ::= ENUMERATED

{

fromTarget(1),

toTarget(2)

}

DNN ::= UTF8String

E164Number ::= NumericString (SIZE(1..15))

EmailAddress ::= UTF8String

EUI64 ::= OCTET STRING (SIZE(8))

FiveGGUTI ::= SEQUENCE

{

mCC         [1] MCC,

mNC         [2] MNC,

aMFRegionID [3] AMFRegionID,

aMFSetID    [4] AMFSetID,

aMFPointer  [5] AMFPointer,

fiveGTMSI   [6] FiveGTMSI

}

FiveGSSubscriberID ::= CHOICE

{

sUPI [1] SUPI,

sUCI [2] SUCI,

pEI  [3] PEI,

gPSI [4] GPSI

}

FiveGSSubscriberIDs ::= SEQUENCE

{

fiveGSSubscriberID [1] SEQUENCE SIZE(1..MAX) OF FiveGSSubscriberID

}

FiveGMMCause ::= INTEGER (0..255)

FiveGSMRequestType ::= ENUMERATED

{

initialRequest(1),

existingPDUSession(2),

initialEmergencyRequest(3),

existingEmergencyPDUSession(4),

modificationRequest(5),

reserved(6),

mAPDURequest(7)

}

FiveGSMCause ::= INTEGER (0..255)

FiveGTMSI ::= INTEGER (0..4294967295)

FiveGSRVCCInfo ::= SEQUENCE

{

uE5GSRVCCCapability   [1] BOOLEAN,

sessionTransferNumber [2] UTF8String OPTIONAL,

correlationMSISDN     [3] MSISDN OPTIONAL

}

FiveGSUserStateInfo ::= SEQUENCE

{

fiveGSUserState [1] FiveGSUserState,

accessType      [2] AccessType

}

FiveGSUserState ::= ENUMERATED

{

deregistered(1),

registeredNotReachableForPaging(2),

registeredReachableForPaging(3),

connectedNotReachableForPaging(4),

connectedReachableForPaging(5),

notProvidedFromAMF(6)

}

FTEID ::= SEQUENCE

{

tEID        [1] INTEGER (0.. 4294967295),

iPv4Address [2] IPv4Address OPTIONAL,

iPv6Address [3] IPv6Address OPTIONAL

}

FTEIDList ::= SEQUENCE OF FTEID

GPSI ::= CHOICE

{

mSISDN      [1] MSISDN,

nAI         [2] NAI

}

GUAMI ::= SEQUENCE

{

aMFID       [1] AMFID,

pLMNID      [2] PLMNID

}

GUMMEI ::= SEQUENCE

{

mMEID       [1] MMEID,

mCC         [2] MCC,

mNC         [3] MNC

}

GUTI ::= SEQUENCE

{

mCC          [1] MCC,

mNC          [2] MNC,

mMEGroupID   [3] MMEGroupID,

mMECode      [4] MMECode,

mTMSI        [5] TMSI

}

HomeNetworkPublicKeyID ::= OCTET STRING

HSMFURI ::= UTF8String

IMEI ::= NumericString (SIZE(14))

IMEISV ::= NumericString (SIZE(16))

IMPI ::= NAI

IMPU ::= CHOICE

{

sIPURI [1] SIPURI,

tELURI [2] TELURI

}

IMSI ::= NumericString (SIZE(6..15))

IMSIUnauthenticatedIndication ::= BOOLEAN

Initiator ::= ENUMERATED

{

uE(1),

network(2),

unknown(3)

}

IPAddress ::= CHOICE

{

iPv4Address [1] IPv4Address,

iPv6Address [2] IPv6Address

}

IPv4Address ::= OCTET STRING (SIZE(4))

IPv6Address ::= OCTET STRING (SIZE(16))

IPv6FlowLabel ::= INTEGER(0..1048575)

MACAddress ::= OCTET STRING (SIZE(6))

MACRestrictionIndicator ::= ENUMERATED

{

noResrictions(1),

mACAddressNotUseableAsEquipmentIdentifier(2),

unknown(3)

}

MCC ::= NumericString (SIZE(3))

MNC ::= NumericString (SIZE(2..3))

MMEID ::= SEQUENCE

{

mMEGI       [1] MMEGI,

mMEC        [2] MMEC

}

MMEC ::= NumericString

MMEGI ::= NumericString

MSISDN ::= NumericString (SIZE(1..15))

NAI ::= UTF8String

NextLayerProtocol ::= INTEGER(0..255)

NonLocalID ::= ENUMERATED

{

local(1),

nonLocal(2)

}

NonIMEISVPEI ::= CHOICE

{

mACAddress [1] MACAddress,

eUI64      [2] EUI64

}

NSSAI ::= SEQUENCE OF SNSSAI

PagingRestrictionIndicator ::= OCTET STRING (SIZE(1..33))

PLMNID ::= SEQUENCE

{

mCC [1] MCC,

mNC [2] MNC

}

PDNConnectionType ::= ENUMERATED

{

iPv4(1),

iPv6(2),

iPv4v6(3),

nonIP(4),

ethernet(5)

}

PDUSessionID ::= INTEGER (0..255)

PDUSessionType ::= ENUMERATED

{

iPv4(1),

iPv6(2),

iPv4v6(3),

unstructured(4),

ethernet(5)

}

PEI ::= CHOICE

{

iMEI        [1] IMEI,

iMEISV      [2] IMEISV,

mACAddress  [3] MACAddress,

eUI64       [4] EUI64

}

PortNumber ::= INTEGER (0..65535)

PrimaryAuthenticationType ::= ENUMERATED

{

eAPAKAPrime(1),

fiveGAKA(2),

eAPTLS(3),

none(4),

ePSAKA(5),

eAPAKA(6),

iMSAKA(7),

gBAAKA(8),

uMTSAKA(9)

}

ProtectionSchemeID ::= INTEGER (0..15)

RATType ::= ENUMERATED

{

nR(1),

eUTRA(2),

wLAN(3),

virtual(4),

nBIOT(5),

wireline(6),

wirelineCable(7),

wirelineBBF(8),

lTEM(9),

nRU(10),

eUTRAU(11),

trustedN3GA(12),

trustedWLAN(13),

uTRA(14),

gERA(15),

nRLEO(16),

nRMEO(17),

nRGEO(18),

nROTHERSAT(19),

nRREDCAP(20)

}

RejectedNSSAI ::= SEQUENCE OF RejectedSNSSAI

RejectedSNSSAI ::= SEQUENCE

{

causeValue  [1] RejectedSliceCauseValue,

sNSSAI      [2] SNSSAI

}

RejectedSliceCauseValue ::= INTEGER (0..255)

ReRegRequiredIndicator ::= ENUMERATED

{

reRegistrationRequired(1),

reRegistrationNotRequired(2)

}

RoutingIndicator ::= INTEGER (0..9999)

SchemeOutput ::= OCTET STRING

SIPURI ::= UTF8String

Slice ::= SEQUENCE

{

allowedNSSAI        [1] NSSAI OPTIONAL,

configuredNSSAI     [2] NSSAI OPTIONAL,

rejectedNSSAI       [3] RejectedNSSAI OPTIONAL

}

SMPDUDNRequest ::= OCTET STRING

-- TS 24.501 [13], clause 9.11.3.6.1

SMSOverNASIndicator ::= ENUMERATED

{

sMSOverNASNotAllowed(1),

sMSOverNASAllowed(2)

}

SNSSAI ::= SEQUENCE

{

sliceServiceType    [1] INTEGER (0..255),

sliceDifferentiator [2] OCTET STRING (SIZE(3)) OPTIONAL

}

SubscriberIdentifier ::= CHOICE

{

sUCI   [1] SUCI,

sUPI   [2] SUPI

}

SUCI ::= SEQUENCE

{

mCC                         [1] MCC,

mNC                         [2] MNC,

routingIndicator            [3] RoutingIndicator,

protectionSchemeID          [4] ProtectionSchemeID,

homeNetworkPublicKeyID      [5] HomeNetworkPublicKeyID,

schemeOutput                [6] SchemeOutput,

routingIndicatorLength      [7] INTEGER (1..4) OPTIONAL

-- shall be included if different from the number of meaningful digits given

-- in routingIndicator

}

SUPI ::= CHOICE

{

iMSI        [1] IMSI,

nAI         [2] NAI

}

SUPIUnauthenticatedIndication ::= BOOLEAN

SwitchOffIndicator ::= ENUMERATED

{

normalDetach(1),

switchOff(2)

}

TargetIdentifier ::= CHOICE

{

sUPI                   [1] SUPI,

iMSI                   [2] IMSI,

pEI                    [3] PEI,

iMEI                   [4] IMEI,

gPSI                   [5] GPSI,

mSISDN                 [6] MSISDN,

nAI                    [7] NAI,

iPv4Address            [8] IPv4Address,

iPv6Address            [9] IPv6Address,

ethernetAddress        [10] MACAddress,

iMPU                   [11] IMPU,

iMPI                   [12] IMPI,

e164Number             [13] E164Number,

emailAddress           [14] EmailAddress,

mCPTTID                [15] UTF8String,

instanceIdentifierURN  [16] UTF8String,

pTCChatGroupID         [17] PTCChatGroupID

}

TargetIdentifierProvenance ::= ENUMERATED

{

lEAProvided(1),

observed(2),

matchedOn(3),

other(4)

}

TELURI ::= UTF8String

Timestamp ::= GeneralizedTime

UEContextInfo ::= SEQUENCE

{

supportVoPS         [1] BOOLEAN OPTIONAL,

supportVoPSNon3GPP  [2] BOOLEAN OPTIONAL,

lastActiveTime      [3] Timestamp OPTIONAL,

accessType          [4] AccessType OPTIONAL,

rATType             [5] RATType OPTIONAL

}

UEEndpointAddress ::= CHOICE

{

iPv4Address         [1] IPv4Address,

iPv6Address         [2] IPv6Address,

ethernetAddress     [3] MACAddress

}

UserIdentifiers ::= SEQUENCE

{

fiveGSSubscriberIDs [1] FiveGSSubscriberIDs OPTIONAL,

ePSSubscriberIDs    [2] EPSSubscriberIDs OPTIONAL

}

-- ===================

-- Location parameters

-- ===================

Location ::= SEQUENCE

{

locationInfo                [1] LocationInfo OPTIONAL,

positioningInfo             [2] PositioningInfo OPTIONAL,

locationPresenceReport      [3] LocationPresenceReport OPTIONAL,

ePSLocationInfo             [4] EPSLocationInfo OPTIONAL

}

CellSiteInformation ::= SEQUENCE

{

geographicalCoordinates     [1] GeographicalCoordinates,

azimuth                     [2] INTEGER (0..359) OPTIONAL,

operatorSpecificInformation [3] UTF8String OPTIONAL

}

-- TS 29.518 [22], clause 6.4.6.2.6

LocationInfo ::= SEQUENCE

{

userLocation                [1] UserLocation OPTIONAL,

currentLoc                  [2] BOOLEAN OPTIONAL,

geoInfo                     [3] GeographicArea OPTIONAL,

rATType                     [4] RATType OPTIONAL,

timeZone                    [5] TimeZone OPTIONAL,

additionalCellIDs           [6] SEQUENCE OF CellInformation OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.7

UserLocation ::= SEQUENCE

{

eUTRALocation               [1] EUTRALocation OPTIONAL,

nRLocation                  [2] NRLocation OPTIONAL,

n3GALocation                [3] N3GALocation OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.8

EUTRALocation ::= SEQUENCE

{

tAI                         [1] TAI,

eCGI                        [2] ECGI,

ageOfLocationInfo           [3] INTEGER OPTIONAL,

uELocationTimestamp         [4] Timestamp OPTIONAL,

geographicalInformation     [5] UTF8String OPTIONAL,

geodeticInformation         [6] UTF8String OPTIONAL,

globalNGENbID               [7] GlobalRANNodeID OPTIONAL,

cellSiteInformation         [8] CellSiteInformation OPTIONAL,

globalENbID                 [9] GlobalRANNodeID OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.9

NRLocation ::= SEQUENCE

{

tAI                         [1] TAI,

nCGI                        [2] NCGI,

ageOfLocationInfo           [3] INTEGER OPTIONAL,

uELocationTimestamp         [4] Timestamp OPTIONAL,

geographicalInformation     [5] UTF8String OPTIONAL,

geodeticInformation         [6] UTF8String OPTIONAL,

globalGNbID                 [7] GlobalRANNodeID OPTIONAL,

cellSiteInformation         [8] CellSiteInformation OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.10

N3GALocation ::= SEQUENCE

{

tAI                         [1] TAI OPTIONAL,

n3IWFID                     [2] N3IWFIDNGAP OPTIONAL,

uEIPAddr                    [3] IPAddr OPTIONAL,

portNumber                  [4] INTEGER OPTIONAL,

tNAPID                      [5] TNAPID OPTIONAL,

tWAPID                      [6] TWAPID OPTIONAL,

hFCNodeID                   [7] HFCNodeID OPTIONAL,

gLI                         [8] GLI OPTIONAL,

w5GBANLineType              [9] W5GBANLineType OPTIONAL,

gCI                         [10] GCI OPTIONAL,

ageOfLocationInfo           [11] INTEGER OPTIONAL,

uELocationTimestamp         [12] Timestamp OPTIONAL,

protocol                    [13] TransportProtocol OPTIONAL

}

-- TS 38.413 [23], clause 9.3.2.4

IPAddr ::= SEQUENCE

{

iPv4Addr                    [1] IPv4Address OPTIONAL,

iPv6Addr                    [2] IPv6Address OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.28

GlobalRANNodeID ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

aNNodeID                    [2] ANNodeID,

nID                         [3] NID OPTIONAL

}

ANNodeID ::= CHOICE

{

n3IWFID [1] N3IWFIDSBI,

gNbID   [2] GNbID,

nGENbID [3] NGENbID,

eNbID   [4] ENbID,

wAGFID  [5] WAGFID,

tNGFID  [6] TNGFID

}

-- TS 38.413 [23], clause 9.3.1.6

GNbID ::= BIT STRING(SIZE(22..32))

-- TS 29.571 [17], clause 5.4.4.4

TAI ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

tAC                         [2] TAC,

nID                         [3] NID OPTIONAL

}

CGI ::= SEQUENCE

{

lAI    [1] LAI,

cellID [2] CellID

}

LAI ::= SEQUENCE

{

pLMNID [1] PLMNID,

lAC    [2] LAC

}

LAC ::= OCTET STRING (SIZE(2))

CellID ::= OCTET STRING (SIZE(2))

SAI ::= SEQUENCE

{

pLMNID [1] PLMNID,

lAC    [2] LAC,

sAC    [3] SAC

}

SAC ::= OCTET STRING (SIZE(2))

-- TS 29.571 [17], clause 5.4.4.5

ECGI ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

eUTRACellID                 [2] EUTRACellID,

nID                         [3] NID OPTIONAL

}

TAIList ::= SEQUENCE OF TAI

-- TS 29.571 [17], clause 5.4.4.6

NCGI ::= SEQUENCE

{

pLMNID                      [1] PLMNID,

nRCellID                    [2] NRCellID,

nID                         [3] NID OPTIONAL

}

RANCGI ::= CHOICE

{

eCGI                        [1] ECGI,

nCGI                        [2] NCGI

}

CellInformation ::= SEQUENCE

{

rANCGI                      [1] RANCGI,

cellSiteinformation         [2] CellSiteInformation OPTIONAL,

timeOfLocation              [3] Timestamp OPTIONAL

}

-- TS 38.413 [23], clause 9.3.1.57

N3IWFIDNGAP ::= BIT STRING (SIZE(16))

-- TS 29.571 [17], clause 5.4.4.28

N3IWFIDSBI ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.28 and table 5.4.2-1

TNGFID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.28 and table 5.4.2-1

WAGFID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.62

TNAPID ::= SEQUENCE

{

sSID         [1] SSID OPTIONAL,

bSSID        [2] BSSID OPTIONAL,

civicAddress [3] CivicAddressBytes OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.64

TWAPID ::= SEQUENCE

{

sSID         [1] SSID OPTIONAL,

bSSID        [2] BSSID OPTIONAL,

civicAddress [3] CivicAddressBytes OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.62 and clause 5.4.4.64

SSID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.62 and clause 5.4.4.64

BSSID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.36 and table 5.4.2-1

HFCNodeID ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.10 and table 5.4.2-1

-- Contains the original binary data i.e. value of the YAML field after base64 encoding is removed

GLI ::= OCTET STRING (SIZE(0..150))

-- TS 29.571 [17], clause 5.4.4.10 and table 5.4.2-1

GCI ::= UTF8String

-- TS 29.571 [17], clause 5.4.4.10 and table 5.4.3.38

TransportProtocol ::= ENUMERATED

{

uDP(1),

tCP(2)

}

-- TS 29.571 [17], clause 5.4.4.10 and clause 5.4.3.33

W5GBANLineType ::= ENUMERATED

{

dSL(1),

pON(2)

}

-- TS 29.571 [17], table 5.4.2-1

TAC ::= OCTET STRING (SIZE(2..3))

-- TS 38.413 [23], clause 9.3.1.9

EUTRACellID ::= BIT STRING (SIZE(28))

-- TS 38.413 [23], clause 9.3.1.7

NRCellID ::= BIT STRING (SIZE(36))

-- TS 38.413 [23], clause 9.3.1.8

NGENbID ::= CHOICE

{

macroNGENbID                [1] BIT STRING (SIZE(20)),

shortMacroNGENbID           [2] BIT STRING (SIZE(18)),

longMacroNGENbID            [3] BIT STRING (SIZE(21))

}

-- TS 23.003 [19], clause 12.7.1 encoded as per TS 29.571 [17], clause 5.4.2

NID ::= UTF8String (SIZE(11))

-- TS 36.413 [38], clause 9.2.1.37

ENbID ::= CHOICE

{

macroENbID                  [1] BIT STRING (SIZE(20)),

homeENbID                   [2] BIT STRING (SIZE(28)),

shortMacroENbID             [3] BIT STRING (SIZE(18)),

longMacroENbID              [4] BIT STRING (SIZE(21))

}

-- TS 29.518 [22], clause 6.4.6.2.3

PositioningInfo ::= SEQUENCE

{

positionInfo                [1] LocationData OPTIONAL,

rawMLPResponse              [2] RawMLPResponse OPTIONAL

}

RawMLPResponse ::= CHOICE

{

-- The following parameter contains a copy of unparsed XML code of the

-- MLP response message, i.e. the entire XML document containing

-- a <slia> (described in OMA-TS-MLP-V3_5-20181211-C [20], clause 5.2.3.2.2) or

-- a <slirep> (described in OMA-TS-MLP-V3_5-20181211-C [20], clause 5.2.3.2.3) MLP message.

mLPPositionData             [1] UTF8String,

-- OMA MLP result id, defined in OMA-TS-MLP-V3_5-20181211-C [20], Clause 5.4

mLPErrorCode                [2] INTEGER (1..699)

}

-- TS 29.572 [24], clause 6.1.6.2.3

LocationData ::= SEQUENCE

{

locationEstimate            [1] GeographicArea,

accuracyFulfilmentIndicator [2] AccuracyFulfilmentIndicator OPTIONAL,

ageOfLocationEstimate       [3] AgeOfLocationEstimate OPTIONAL,

velocityEstimate            [4] VelocityEstimate OPTIONAL,

civicAddress                [5] CivicAddress OPTIONAL,

positioningDataList         [6] SET OF PositioningMethodAndUsage OPTIONAL,

gNSSPositioningDataList     [7] SET OF GNSSPositioningMethodAndUsage OPTIONAL,

eCGI                        [8] ECGI OPTIONAL,

nCGI                        [9] NCGI OPTIONAL,

altitude                    [10] Altitude OPTIONAL,

barometricPressure          [11] BarometricPressure OPTIONAL

}

-- TS 29.172 [53], table 6.2.2-2

EPSLocationInfo ::= SEQUENCE

{

locationData  [1] LocationData,

cGI           [2] CGI OPTIONAL,

sAI           [3] SAI OPTIONAL,

eSMLCCellInfo [4] ESMLCCellInfo OPTIONAL

}

-- TS 29.172 [53], clause 7.4.57

ESMLCCellInfo ::= SEQUENCE

{

eCGI          [1] ECGI,

cellPortionID [2] CellPortionID

}

-- TS 29.171 [54], clause 7.4.31

CellPortionID ::= INTEGER (0..4095)

-- TS 29.518 [22], clause 6.2.6.2.5

LocationPresenceReport ::= SEQUENCE

{

type                        [1] AMFEventType,

timestamp                   [2] Timestamp,

areaList                    [3] SET OF AMFEventArea OPTIONAL,

timeZone                    [4] TimeZone OPTIONAL,

accessTypes                 [5] SET OF AccessType OPTIONAL,

rMInfoList                  [6] SET OF RMInfo OPTIONAL,

cMInfoList                  [7] SET OF CMInfo OPTIONAL,

reachability                [8] UEReachability OPTIONAL,

location                    [9] UserLocation OPTIONAL,

additionalCellIDs           [10] SEQUENCE OF CellInformation OPTIONAL

}

-- TS 29.518 [22], clause 6.2.6.3.3

AMFEventType ::= ENUMERATED

{

locationReport(1),

presenceInAOIReport(2)

}

-- TS 29.518 [22], clause 6.2.6.2.16

AMFEventArea ::= SEQUENCE

{

presenceInfo                [1] PresenceInfo OPTIONAL,

lADNInfo                    [2] LADNInfo OPTIONAL

}

-- TS 29.571 [17], clause 5.4.4.27

PresenceInfo ::= SEQUENCE

{

presenceState               [1] PresenceState OPTIONAL,

trackingAreaList            [2] SET OF TAI OPTIONAL,

eCGIList                    [3] SET OF ECGI OPTIONAL,

nCGIList                    [4] SET OF NCGI OPTIONAL,

globalRANNodeIDList         [5] SET OF GlobalRANNodeID OPTIONAL,

globalENbIDList             [6] SET OF GlobalRANNodeID OPTIONAL

}

-- TS 29.518 [22], clause 6.2.6.2.17

LADNInfo ::= SEQUENCE

{

lADN                        [1] UTF8String,

presence                    [2] PresenceState OPTIONAL

}

-- TS 29.571 [17], clause 5.4.3.20

PresenceState ::= ENUMERATED

{

inArea(1),

outOfArea(2),

unknown(3),

inactive(4)

}

-- TS 29.518 [22], clause 6.2.6.2.8

RMInfo ::= SEQUENCE

{

rMState                     [1] RMState,

accessType                  [2] AccessType

}

-- TS 29.518 [22], clause 6.2.6.2.9

CMInfo ::= SEQUENCE

{

cMState                     [1] CMState,

accessType                  [2] AccessType

}

-- TS 29.518 [22], clause 6.2.6.3.7

UEReachability ::= ENUMERATED

{

unreachable(1),

reachable(2),

regulatoryOnly(3)

}

-- TS 29.518 [22], clause 6.2.6.3.9

RMState ::= ENUMERATED

{

registered(1),

deregistered(2)

}

-- TS 29.518 [22], clause 6.2.6.3.10

CMState ::= ENUMERATED

{

idle(1),

connected(2)

}

-- TS 29.572 [24], clause 6.1.6.2.5

GeographicArea ::= CHOICE

{

point                       [1] Point,

pointUncertaintyCircle      [2] PointUncertaintyCircle,

pointUncertaintyEllipse     [3] PointUncertaintyEllipse,

polygon                     [4] Polygon,

pointAltitude               [5] PointAltitude,

pointAltitudeUncertainty    [6] PointAltitudeUncertainty,

ellipsoidArc                [7] EllipsoidArc

}

-- TS 29.572 [24], clause 6.1.6.3.12

AccuracyFulfilmentIndicator ::= ENUMERATED

{

requestedAccuracyFulfilled(1),

requestedAccuracyNotFulfilled(2)

}

-- TS 29.572 [24], clause 6.1.6.2.17

VelocityEstimate ::= CHOICE

{

horVelocity                         [1] HorizontalVelocity,

horWithVertVelocity                 [2] HorizontalWithVerticalVelocity,

horVelocityWithUncertainty          [3] HorizontalVelocityWithUncertainty,

horWithVertVelocityAndUncertainty   [4] HorizontalWithVerticalVelocityAndUncertainty

}

-- TS 29.572 [24], clause 6.1.6.2.14

CivicAddress ::= SEQUENCE

{

country                             [1] UTF8String,

a1                                  [2] UTF8String OPTIONAL,

a2                                  [3] UTF8String OPTIONAL,

a3                                  [4] UTF8String OPTIONAL,

a4                                  [5] UTF8String OPTIONAL,

a5                                  [6] UTF8String OPTIONAL,

a6                                  [7] UTF8String OPTIONAL,

prd                                 [8] UTF8String OPTIONAL,

pod                                 [9] UTF8String OPTIONAL,

sts                                 [10] UTF8String OPTIONAL,

hno                                 [11] UTF8String OPTIONAL,

hns                                 [12] UTF8String OPTIONAL,

lmk                                 [13] UTF8String OPTIONAL,

loc                                 [14] UTF8String OPTIONAL,

nam                                 [15] UTF8String OPTIONAL,

pc                                  [16] UTF8String OPTIONAL,

bld                                 [17] UTF8String OPTIONAL,

unit                                [18] UTF8String OPTIONAL,

flr                                 [19] UTF8String OPTIONAL,

room                                [20] UTF8String OPTIONAL,

plc                                 [21] UTF8String OPTIONAL,

pcn                                 [22] UTF8String OPTIONAL,

pobox                               [23] UTF8String OPTIONAL,

addcode                             [24] UTF8String OPTIONAL,

seat                                [25] UTF8String OPTIONAL,

rd                                  [26] UTF8String OPTIONAL,

rdsec                               [27] UTF8String OPTIONAL,

rdbr                                [28] UTF8String OPTIONAL,

rdsubbr                             [29] UTF8String OPTIONAL,

prm                                 [30] UTF8String OPTIONAL,

pom                                 [31] UTF8String OPTIONAL

}

-- TS 29.571 [17], clauses 5.4.4.62 and 5.4.4.64

-- Contains the original binary data i.e. value of the YAML field after base64 encoding is removed

CivicAddressBytes ::= OCTET STRING

-- TS 29.572 [24], clause 6.1.6.2.15

PositioningMethodAndUsage ::= SEQUENCE

{

method                              [1] PositioningMethod,

mode                                [2] PositioningMode,

usage                               [3] Usage,

methodCode                          [4] MethodCode OPTIONAL

}

-- TS 29.572 [24], clause 6.1.6.2.16

GNSSPositioningMethodAndUsage ::= SEQUENCE

{

mode                                [1] PositioningMode,

gNSS                                [2] GNSSID,

usage                               [3] Usage

}

-- TS 29.572 [24], clause 6.1.6.2.6

Point ::= SEQUENCE

{

geographicalCoordinates             [1] GeographicalCoordinates

}

-- TS 29.572 [24], clause 6.1.6.2.7

PointUncertaintyCircle ::= SEQUENCE

{

geographicalCoordinates             [1] GeographicalCoordinates,

uncertainty                         [2] Uncertainty

}

-- TS 29.572 [24], clause 6.1.6.2.8

PointUncertaintyEllipse ::= SEQUENCE

{

geographicalCoordinates             [1] GeographicalCoordinates,

uncertainty                         [2] UncertaintyEllipse,

confidence                          [3] Confidence

}

-- TS 29.572 [24], clause 6.1.6.2.9

Polygon ::= SEQUENCE

{

pointList                           [1] SET SIZE (3..15) OF GeographicalCoordinates

}

-- TS 29.572 [24], clause 6.1.6.2.10

PointAltitude ::= SEQUENCE

{

point                               [1] GeographicalCoordinates,

altitude                            [2] Altitude

}

-- TS 29.572 [24], clause 6.1.6.2.11

PointAltitudeUncertainty ::= SEQUENCE

{

point                               [1] GeographicalCoordinates,

altitude                            [2] Altitude,

uncertaintyEllipse                  [3] UncertaintyEllipse,

uncertaintyAltitude                 [4] Uncertainty,

confidence                          [5] Confidence

}

-- TS 29.572 [24], clause 6.1.6.2.12

EllipsoidArc ::= SEQUENCE

{

point                               [1] GeographicalCoordinates,

innerRadius                         [2] InnerRadius,

uncertaintyRadius                   [3] Uncertainty,

offsetAngle                         [4] Angle,

includedAngle                       [5] Angle,

confidence                          [6] Confidence

}

-- TS 29.572 [24], clause 6.1.6.2.4

GeographicalCoordinates ::= SEQUENCE

{

latitude                            [1] UTF8String,

longitude                           [2] UTF8String,

mapDatumInformation                 [3] OGCURN OPTIONAL

}

-- TS 29.572 [24], clause 6.1.6.2.22

UncertaintyEllipse ::= SEQUENCE

{

semiMajor                           [1] Uncertainty,

semiMinor                           [2] Uncertainty,

orientationMajor                    [3] Orientation

}

-- TS 29.572 [24], clause 6.1.6.2.18

HorizontalVelocity ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle

}

-- TS 29.572 [24], clause 6.1.6.2.19

HorizontalWithVerticalVelocity ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle,

vSpeed                              [3] VerticalSpeed,

vDirection                          [4] VerticalDirection

}

-- TS 29.572 [24], clause 6.1.6.2.20

HorizontalVelocityWithUncertainty ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle,

uncertainty                         [3] SpeedUncertainty

}

-- TS 29.572 [24], clause 6.1.6.2.21

HorizontalWithVerticalVelocityAndUncertainty ::= SEQUENCE

{

hSpeed                              [1] HorizontalSpeed,

bearing                             [2] Angle,

vSpeed                              [3] VerticalSpeed,

vDirection                          [4] VerticalDirection,

hUncertainty                        [5] SpeedUncertainty,

vUncertainty                        [6] SpeedUncertainty

}

-- The following types are described in TS 29.572 [24], table 6.1.6.3.2-1

Altitude ::= UTF8String

Angle ::= INTEGER (0..360)

Uncertainty ::= INTEGER (0..127)

Orientation ::= INTEGER (0..180)

Confidence ::= INTEGER (0..100)

InnerRadius ::= INTEGER (0..327675)

AgeOfLocationEstimate ::= INTEGER (0..32767)

HorizontalSpeed ::= UTF8String

VerticalSpeed ::= UTF8String

SpeedUncertainty ::= UTF8String

BarometricPressure ::= INTEGER (30000..115000)

-- TS 29.572 [24], clause 6.1.6.3.13

VerticalDirection ::= ENUMERATED

{

upward(1),

downward(2)

}

-- TS 29.572 [24], clause 6.1.6.3.6

PositioningMethod ::= ENUMERATED

{

cellID(1),

eCID(2),

oTDOA(3),

barometricPressure(4),

wLAN(5),

bluetooth(6),

mBS(7),

motionSensor(8),

dLTDOA(9),

dLAOD(10),

multiRTT(11),

nRECID(12),

uLTDOA(13),

uLAOA(14),

networkSpecific(15)

}

-- TS 29.572 [24], clause 6.1.6.3.7

PositioningMode ::= ENUMERATED

{

uEBased(1),

uEAssisted(2),

conventional(3)

}

-- TS 29.572 [24], clause 6.1.6.3.8

GNSSID ::= ENUMERATED

{

gPS(1),

galileo(2),

sBAS(3),

modernizedGPS(4),

qZSS(5),

gLONASS(6),

bDS(7),

nAVIC(8)

}

-- TS 29.572 [24], clause 6.1.6.3.9

Usage ::= ENUMERATED

{

unsuccess(1),

successResultsNotUsed(2),

successResultsUsedToVerifyLocation(3),

successResultsUsedToGenerateLocation(4),

successMethodNotDetermined(5)

}

-- TS 29.571 [17], table 5.2.2-1

TimeZone ::= UTF8String

-- Open Geospatial Consortium URN [35]

OGCURN ::= UTF8String

-- TS 29.572 [24], clause 6.1.6.2.15

MethodCode ::= INTEGER (16..31)

END

END OF CHANGES TO ATTACHMET TS33128PAYLOADS.ASN

END OF CHANGES