SA3LI#92	 S3i240036

Seville, January 30-February 2, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	pCR to TR 33.929: STIR/SHAKEN – Part XII (IBCF Intermediate)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI18

Abstract of the contribution: This pCR adds a clause that illustrate the IMS NFs that provide POI for IMS LI and STIR/SHAKEN (IBCF Intermediate network)

BACKGROUND

The TS 33.127, TS 33.128 and TR 33.928 provide the stage 2, stage 3 and ADMF provisioning aspects of STIR/SHAKEN related lawful interception (LI) reporting. Such reporting happens when the calling party related information, redirecting party information are signed and verified for an IMS voice call.

The NFs that provide the POI functions for STIR/SHAKEN related reporting can be different from the NFs that provide the POI functions for the associated IMS sessions. Differing deployment options, call types,  the presence of Rich Call Data (RCD), the target type have an influence in determining which the NFs provide the POI functions for STIR/SHAKEN.

The set of pCRs follow the groupings as shown below:

- Introduction which also includes a background.

- Extending the background - NF selection algorithm for STIR/SHAKEN POIs.

- STIR/SHAKEN and IMS POIs.

- Reporting at the originating network domain   - there are multiple cases here.

- Reporting at the terminating network domain -  there are multiple cases here.

- Reporting in the intermediate network domain -  there are multiple cases here.

This pCR is on 1. Introduction which also includes a background and the scope. Because this is a pCR, the clause numbers are pre-selected to help the editor.

PROPOSAL

Incorporate the following to the TR 33.929.

## 4.10	STIR/SHAKEN related LI reporting

### 4.10.1	General

### 4.10.2	Background

### 4.10.3	IMS LI and STIR/SHAKEN LI

#### 4.10.3.1	General

#### 4.10.3.2	Originating IMS network – IBCF interacts with the Signing AS

#### 4.10.3.3	Terminating IMS network – IBCF interacts with the Verification AS

#### 4.10.3.4	Originating IMS network – Telephony AS interacts with the Signing AS

#### 4.10.3.5	Terminating IMS network – Telephony AS interacts with the Verification AS

#### 4.10.3.6	Intermediate IMS network

##### 4.10.3.6.1	General

##### 4.10.3.6.2	Telephony AS interacts with the Signing/Verification AS

##### 4.10.3.6.3	IBCF interacts with the Signing/Verification AS

Within the drawings, AS (B represents the Telephony AS.

Scenario 1: A calls B (target), redirected to C.

The S-CSCF (B) provides the IRI-POI functions for IMS LI with the default option, and the IBCF (Egress) provides the  IRI-POI functions for IMS LI with the alternate option.

Figure 4.10.3.6.3-1: Scenario 1 – A calls B (target), redirected to C

The left part of the diagram shows for IMS LI with default option and the right part of the diagram shows for IMS LI with alternate option.

The IBCF (Ingress) that interacts with the Verification AS provides the IRI-POI functions for the STIR/SHAKEN related LI reporting with the Validation Result for target B.

The IBCF (Egress) that interacts with the Signing AS provides the IRI-POI functions for the STIR/SHAKEN related LI reporting for target B.

Scenario 2: A (target non-local ID) calls B, redirected to C.

The IBCF (Ingress) provides the IRI-POI functions for IMS LI with the default option, and the S-CSCF (B) provides the  IRI-POI functions for IMS LI with the alternate option.

Figure 4.10.3.6.3-2: Scenario 2 – A (target local ID) calls B, redirected to C

The left part of the diagram shows for IMS LI with default option and the right part of the diagram shows for IMS LI with alternate option.

The IBCF (Ingress) that interacts with the Verification AS provides the IRI-POI functions for the STIR/SHAKEN related LI reporting with the Validation Result for target non-local ID A.

When the ReportDivPASSporTInfo provisioned for target non-local ID A is true, IBCF (Egress) that interacts with the Signing AS provides the IRI-POI functions for the STIR/SHAKEN related LI reporting for target non-local ID A.

Scenario 3: A calls B, redirected to C  (target non-local ID).

The IBCF (Egress) provides the IRI-POI functions for IMS LI with the default option, and the S-CSCF (B) provides the  IRI-POI functions for IMS LI with the alternate option.

Figure 4.10.3.6.3-3: Scenario 3 – A calls B, redirected to C (target local ID)

The left part of the diagram shows for IMS LI with default option and the right part of the diagram shows for IMS LI with alternate option.

The  IBCF (B) that interacts with the Signing AS provides the IRI-POI functions for the STIR/SHAKEN related LI reporting for target non-local ID C.

Scenario 4: A (target non-local ID) calls B, redirected to C (target non-local ID).

For target non-local ID A: The IBCF (Ingress) provides the IRI-POI functions for IMS LI with the default option, and the S-CSCF (B) provides the  IRI-POI functions for IMS LI with the alternate option.

For target non-local ID C: The IBCF (Egress) provides the IRI-POI functions for IMS LI with the default option, and the S-CSCF (B) provides the  IRI-POI functions for IMS LI with the alternate option.

Figure 4.10.3.6.3-4: Scenario 4 – A (target local ID calls B, redirected to C (target local ID)

The left part of the diagram shows for IMS LI with default option and the right part of the diagram shows for IMS LI with alternate option.

The  IBCF (Ingress) that interacts with the Verification AS provides the IRI-POI functions for the STIR/SHAKEN related LI reporting with the Validation Result for target non-local ID A.

The IBCF (Egress) that interacts with the Signing AS provides the STIR/SHAKEN related LI reporting for target non-local ID C and for target non-local ID A (when the ReportDivPASSporTInfo provisioned for target non-local ID A is true).

Scenario 5: A (target non-local ID) calls B (target), redirected to C (target non-local ID).

For target non-local ID A: The IBCF (Ingress) provides the IRI-POI functions for IMS LI with the default option, and the S-CSCF (B) provides the  IRI-POI functions for IMS LI with the alternate option.

For target B: The S-CSCF (B) provides the IRI-POI functions for IMS LI with the default option, and the IBCF (Egress) provides the  IRI-POI functions for IMS LI with the alternate option.

For target non-local ID C: The IBCF (Egress) provides the IRI-POI functions for IMS LI with the default option, and the S-CSCF (B) provides the  IRI-POI functions for IMS LI with the alternate option.

Figure 4.10.3.6.3-5: Scenario 5 – A (target local ID calls B (target) redirected to C (target local ID)

The left part of the diagram shows for IMS LI with default option and the right part of the diagram shows for IMS LI with alternate option.

The  IBCF (Ingress)  that interacts with Verification AS provides the IRI-POI functions for the STIR/SHAKEN related LI reporting with the Validation Result for target non-local ID A and target B.

The IBCF (Egress) that interacts with the Signing AS provides the STIR/SHAKEN related LI reporting for target non-local ID C, target B and target non-local ID A (when the ReportDivPASSporTInfo provisioned for target non-local ID A is true).