3GPP TSG-SA3 Meeting #92-LI	s3i240044

# Sevilla, Spain, 30th Jan 2024 - 2nd Feb 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0616 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NGAP Cause to AMF Unsuccessful Procedure Record |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2024-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, the NGAP cause is missing from the AMF unsuccesful procedure record. This is added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of trigger for error related to NGAP procedure, Add NGAP cause to table 6.2.2.2.6-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NGAP causes will not be reported. CSPs may not be able to meet LI obligations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.2.6, Attachements ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: Merge request: !245 Comitt Hash: 63096aea51cf9ea56de7c9a93f76e784c359da4a https://forge.3gpp.org/rep/sa3/li/-/merge_requests/245/diffs?commit_id=63096aea51cf9ea56de7c9a93f76e784c359da4a |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******* START OF CHANGES*******

******* START OF FIRST CHANGE*******

##### 6.2.2.2.6	AMF unsuccessful procedure

The IRI-POI in the AMF shall generate an xIRI containing an AMFUnsuccessfulProcedure record when the IRI-POI present in the AMF detects an unsuccessful procedure for a UE matching one of the target identifiers provided via LI_X1.

Accordingly, the IRI-POI in the AMF generates the xIRI when any of the following events is detected:

-	AMF sends a N1: REGISTRATION REJECT message to the target UE and the UE 5G Mobility Management (5GMM) state for the access type (3GPP NG-RAN or non-3GPP access) within the AMF is changed to 5GMM-DEREGISTERED.

-	AMF aborts a registration procedure before the UE 5G Mobility Management (5GMM) state for the access type (3GPP NG-RAN or non-3GPP access) within the AMF is changed to 5GMM-REGISTERED.

-	AMF sends a SERVICE REJECT message to the target UE including a PDU session establishment reject message type.

-	AMF aborts a UE-initiated NAS transport procedure with payload container type IE set to "SMS".

-	AMF receives a failure indication from the NG-RAN during an NGAP procedure for the target.

Unsuccessful registration shall be reported only if the target UE has been successfully authenticated.

Table 6.2.2.2.6-1: Payload for AMFUnsuccessfulProcedure record

| Field name | Description | M/C/O |
| --- | --- | --- |
| failedprocedureType | Specifies the procedure which failed at the AMF. | M |
| failureCause | Provides the value of the 5GSM or 5GMM cause, see TS 24.501 [13] clauses 9.11.3.2 and 9.11.4.2. The integer value is mapped from the second octet shown in TS 24.501 [13] clauses 9.11.3.2 and 9.11.4.2. | M |
| requestedSlice | Slice requested for the procedure, if available, given as a NSSAI (a list of S-NSSAI values as described in TS 24.501 [13] clause 9.11.3.37). | C |
| sUPI | SUPI associated with the procedure, if available (see NOTE). | C |
| sUCI | SUCI used in the procedure, if applicable and if available (see NOTE). | C |
| pEI | PEI used in the procedure, if available (see NOTE). | C |
| gPSI | GPSI used in the procedure, if available (see NOTE). | C |
| gUTI | 5G-GUTI used in the procedure, if available, see TS 24.501 [13] clause 9.11.3.4 (see NOTE). | C |
| location | Location information determined during the procedure, if available. Shall be encoded using the Location.locationInfo.userLocation parameter. If available, other parameters reportable via Location shall be included. | C |
| nGAPCause | Provides the NGAP cause associated with the unsuccessful procedure, see TS 38.413 [23] clause 9.3.1.2. Shall be derived as described in TS 29.571 [17] clause 5.4.4.12. | C |
| NOTE: At least one identity shall be provided, the others shall be provided if available. |  |  |

******* END OF FIRST CHANGE*******

***END OF MAIN DOCUMENT CHANGES***

***START OF CHANGES (ATTACHMENTS)***

START OF CHANGE 1

---a/33128/r18/TS33128Payloads.asn
+++b/33128/r18/TS33128Payloads.asn

@@ -1437,7 +1437,8 @@ AMFUnsuccessfulProcedure ::= SEQUENCE

1437	1437		    pEI                         [6] PEI OPTIONAL,

1438	1438		    gPSI                        [7] GPSI OPTIONAL,

1439	1439		    gUTI                        [8] FiveGGUTI OPTIONAL,

1440		-	    location                    [9] Location OPTIONAL

1440	+	    location                    [9] Location OPTIONAL,

1441	+	    nGAPCause                   [10] NGAPCauseInt OPTIONAL

1441	1442		}

1442	1443

1443	1444		-- See clause 6.2.2.2.8 on for details of this structure

END OF CHANGE 1

*******END OF ALL CHANGES*******