SA3LI#93	 S3i240249

Washington DC, April 16-19, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	pCR to TR 33.929: STIR/SHAKEN related LI reporting - Part-7-1 (signing not required for intra-network IMS sessions, intra-network sessions, basic scenarios)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR adds STIR/SHAKEN related LI reporting to illustrate xIRIs generated from the IRI-POIs of IMS LI and STIR/SHAKEN LI for intra-network basic sessions when signing is not required for intra-network sessions.

BACKGROUND

In this set of pCRs, the STIR/SJAKEN related LI reporting along with the IMS LI reporting are illustrated.

This pCR illustrates the STIR/SHAKEN related LI reporting basic scenarios for intra-network sessions when signing is not required for intra-network sessions.

The clause numbers are adjusted according to the recommendations of split TR 33.929 (i.e. TR 33.929-2).

PROPOSAL

Incorporate the following to the TR 33.929.

*** The following (only clause headings are shown here) are in the TR 33.929-2 and other pCRs ***

# 4 	STIR/SHAKEN related LI reporting

## 4.1	General

Text in TR 33.929-2.v001.

## 4.2	Background

Text in TR 33.929-2.v001.

## 4.3	IMS LI and STIR/SHAKEN LI

Text in TR 33.929-2.v001.

## 4.4	LI reporting details

### 4.4.1	General

### 4.4.2	General concept Overview

### 4.4.3	General principles of LI reporting

### 4.4.4	Signing required for intra-network IMS sessions

*** End of clause headings from TR 33.929-2 and other pCRs***

**** New text ****

### 4.4.5	Signing is not required for intra-network IMS sessions

#### 4.4.5.1	General

The subsequent sub-clauses illustrate the scenarios when signing is not required for intra-network IMS sessions. The IMS NFs that provide the STIR/SHAKEN related LI reporting are dependent on the call scenario and the target.

When the intra-network signing is not required, based on CSP choice either the egress IBCF or the last Telephony AS (in the signalling path within the network) would interact with the Signing AS in the originating network (or intermediate network) and either the ingress IBCF or the first Telephony AS (in the signalling path within the network) would interact with the Verification AS in the terminating network (or the intermediate network).

There are a few exceptions to the above facts:

- The Telephony AS interacts with the Signing AS if RCD is supported for the call.

- The Telephony AS that is not able to determine the whereabouts of terminating user may interact with the Signing AS even if it is not the last Telephony AS (in the signalling path within the CSP network) when the CSP choice is Telephony AS.

- The Telephony AS always interacts with the Verification AS if it sees PASSporT (s) within the incoming SIP INVITE without any Validation Results.

- The IBCF does not interact with the Signing AS if it sees PASSporT (s) without any Validation Results in the incoming SIP INVITE message, even if the CSP choice is IBCF.

The illustrations show the IMS LI reporting and STIR/SHAKEN related LI reporting as defined in TS 33.127 [3], TS 33.128 [4]. The TR 33.929-1 illustrates various scenarios for IMS LI with the default option.

The clause 4.3 of the present document indicate the conditions for an IMS NF to have the IRI-POI for the STIR/SHAKEN related reporting.

#### 4.4.5.2	Intra-network IMS sessions

##### 4.4.5.2.1	General

When the signing of intra-network IMS sessions is not required, STIR/SHAKEN related LI reporting applies to an intra-network IMS sessions only when the Telephony AS is not able to determine the whereabouts of the destination party or when RCD is supported for the call. The scenarios presented in the subsequent sub-clauses focus on the RCD aspects.

The Telephony AS that interacts with the Signing AS/Verification AS provides the LI functions for STIR/SHAKEN related reporting. It is assumed that all the PASSporTs that are validated are reported in the STIRSHAKENSignatureValdiation message to the LEMFs.

The calling party's identity, when signed, is included in the "shaken" PASSporT (with calling party's identity in the origination claim and the called party's identity in the destination claim). The redirecting party identity, when signed, is included in the "div" PASSporT (with calling party's identity in the origination claim, redirecting party's identity in the diversion claim and the redirected-to party's identity in the destination claim).

##### 4.4.5.2.2	Scenario 1: Party A (target) calls Party B

###### 4.4.5.2.2.1 	The scenario

In this scenario, Party A calls Party B. Party A is the target.

Figure 4.4.5.2.2.1-1: Party A calls Party B. Party A is the target

The clauses 4.4.5.2.2.2 and 4.4.5.2.2.3 illustrate the LI aspects for this scenario.

4.4.5.2.2.2	RCD is not supported

The illustration shown in figure 4.4.5.2.2.2-1 assumes the default option (see TS 33.127 [3]) for IMS LI and therefore, the S-CSCF of A provides the IRI-POI functions for IMS LI.

Figure 4.4.5.2.2.2-1: Party A (target) calls Party B (RCD is not supported for the call)

For target (Party A)

The IRI-POI present in the S-CSCF (of A) generates and delivers the xIRIIMSMessage to the MDF2 and the MDF2 forwards it as an IRI message IMSMessage to the LEMF.

Since signing is not required for intra-network sessions, and the Telephony AS knows that the Party B served by the same CSP as that of Party A, the Telephony AS of A does not interact with the Signing AS. Therefore, STIR/SHAKEN related LI reporting does not apply to this case.

The IRI-POI present in the S-CSCF (of A) uses the P-Asserted ID or the From headers (pointing to Party A) present in the incoming SIP INVITE to perform a target match.

4.4.5.2.2.3	RCD is supported

The illustration shown in figure 4.4.5.2.2.3-1 assumes the default option (see TS 33.127 [3]) for IMS LI and therefore, the S-CSCF of A provides the IRI-POI functions for IMS LI.

Figure 4.4.5.2.2.3-1: Party A (target) calls Party B (RCD is supported)

For target (Party A)

The IRI-POI present in the P-CSCF (of A) generates and delivers the xIRIIMSMessage to the MDF2 and the MDF2 forwards it as an IRI message IMSMessage to the LEMF.

Since RCD is supported for the call from Party A to Party B, the Telephony AS (of A) interacts with the Signing AS to authenticate/sign A’s identity and the IRI-POI present in the same Telephony AS generates and delivers the xIRISTIRSHAKENSignatureGeneration record that includes the shaken PASSporT for Party A to the MDF2. The MDF2 forwards it as an IRI message STIRSHAKENSignatureGeneration to the LEMF.

The IRI-POI present in the S-CSCF (of A) uses the P-Asserted ID or the From headers (pointing to Party A) present in the incoming SIP INVITE to perform a target match.

The IRI-POI present in the Telephony AS (of A) uses the P-Asserted ID or the From headers (pointing to Party A) present in the incoming SIP INVITE to perform a target match.

##### 4.4.5.2.3	Scenario 2: Party A calls Party B (target)

###### 4.4.5.2.3.1	The scenario

In this scenario, Party A calls Party B. Party B is the target.

Figure 4.4.5.2.3.1-1: Party A calls Party B. Party B is the target

The clauses 4.4.5.2.3.2 and 4.4.5.2.3.3 illustrate the LI aspects for this scenario.

###### 4.4.5.2.3.2	RCD is not supported

The illustration shown in figure 4.4.5.2.3.2-1 assumes the default option (see TS 33.127 [3]) for IMS LI and therefore, the S-CSCF of B provides the IRI-POI functions for IMS LI.

Figure 4.4.5.2.3.2-1: Party A calls Party B (target) - RCD is not supported

For target (Party B)

The IRI-POI present in the S-CSCF (of B) generates and delivers the xIRIIMSMessage to the MDF2 and the MDF2 forwards it as an IRI message IMSMessage to the LEMF.

Since signing is not required for intra-network sessions, and the Telephony AS of A knows that the Party B served by the same CSP as that of Party A, the Telephony AS of A does not interact with the Signing AS. Accordingly, the Telephony AS of B does not see any PASSporTs and hence, does not interact with the Verification AS. Therefore, STIR/SHAKEN related LI reporting does not apply to this case.

For the target (Party B), the IRI-POI present in the S-CSCF (of B) uses the Request URI or the To headers (pointing to Party B) present in the incoming SIP INVITE to perform a target match.

###### 4.4.5.2.3.3	RCD is supported

The illustration shown in figure 4.4.5.2.3.3-1 assumes the default option (see TS 33.127 [3]) for IMS LI and therefore, the S-CSCF of B provides the IRI-POI functions for IMS LI.

Figure 4.4.4.2.3.3-1: Party A calls Party B (target) - RCD is supported

For target (Party B)

The IRI-POI present in the S-CSCF (of B) generates and delivers the xIRIIMSMessage to the MDF2 and the MDF2 forwards it as an IRI message IMSMessage to the LEMF.

The Telephony AS (of B) interacts with the Verification AS to verify the received "shaken" PASSporT and the IRI-POI present in the same Telephony AS generates and delivers the xIRISTIRSHAKENSignatureValidation record that includes the RCD to the MDF2. The MDF2 forwards it as an IRI message STIRSHAKENSignatureValidation that includes the RCD to the LEMF.

The IRI-POI present in the S-CSCF (of B) uses the Request URI or the To headers (pointing to Party B) present in the incoming SIP INVITE to perform a target match.

The IRI-POI present in the Telephony AS (of B) uses the Request URI or the To headers (pointing to Party B) headers present in the incoming SIP INVITE to perform a target match.

**** End of new text ****