| 3GPP TR 33.929-8 V0.0.0 (2024-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Security; Lawful Interception (LI) Implementation Guidance; Part 8: LI for MMS (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Illustration of LI for MMS	8

5	Example call flows of LI for MMS	9

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The present document is part of a multi-part TR as described below:

- Part 1: LI for IMS based Services. See TR 33.929-1 [10].

- Part 2: LI for IMS based STIR/SHAKEN. See TR 33.929-2 [11].

- Part 3: LI for Messaging services. See TR 33.929-3 [12].

- Part 4: LI for Data in 5G Core. See TR 33.929-4 [13].

- Part 5: ID Association Caching. See TR 33.929-5 [14].

- Part 6: LI for IMS based RCS. See TR 33.929-6 [15].

- Part 7: LI for Location Services. See TR 33.929-7  [16].

- Part 8: LI for MMS (present document).

- Part 9: LI for PTC. See TR 33.929-9 [17].

In Part 1, the illustrations related to LI for IMS-based services focussing on the LI aspects of session-based IMS sessions which include the architecture topologies and the call flows covering basic sessions, redirected sessions, target non-local ID, conferencing, roaming (local break-out and home-routed roaming).

In Part 2, the illustrations related to LI for IMS based STIR/SHAKEN focussing on various STIR/SHAKEN related LI reporting scenarios.

In Part 3, the illustrations related LI for Messaging Services focussing on SMS over NAS and SMS over IP.

In Part 4, the illustrations for LI for Data in 5G Core focussing on the LI aspects of data interception focussing on the PDU session related events including the handover scenarios.

In Part 5, the illustrations of ID Association Caching focussing on the capabilities used to provide the temporary identity to permanent identity (and vice-versa) associations known to the CSP network. The illustrations include the architectural concepts and few examples of use-cases encountering various timing scenarios.

In Part 6, the illustrations of  LI for IMS based RCS focussing on the architecture topologies and the call flows when the RCS service is offered by the CSP or by a Third Party Provider.

In Part 7, the illustrations of LI for Location Services focussing on the conceptual overview and the flow diagrams for location reporting, LALS and Location Acquisition.

In Part 8 (present document), the illustrations of  LI for MMS focussing on the conceptual overview and the flow diagrams for MMS.

In Part 9, the illustrations of LI for PTC focussing on the conceptual overview and the flow diagrams for MCPTT and PoC, the two commonly referred to as Push to Talk over Cellular (PTC).

# Introduction

Unlike the previous generation of LI technical specifications, the latest LI technical specifications (TS 33.126 [2], TS 33.127 [3], TS 33.128 [4]) contain only the normative part of the requirements. However, the implementers may need additional information such as architectural diagrams, conceptual scenarios, flow-diagrams and additional background information to better understand the requirements in those specifications. The present document collects the relevant informative annexes from the previous generation of LI technical specifications and then expands them to include the similar level of details as for latest technologies such as 5G.

# 1	Scope

The present document provides architectural diagrams, conceptual scenarios, flow-diagrams, examples, and other background information which can be useful to implement the LI functions defined in TS 33.126 [2], TS 33.127 [3] and TS 33.128 [4].

The present document covers the LI for MMS  that include the illustrations covering the conceptual overview and the flow diagrams for MMS.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.126: "Lawful Interception Requirements".

[3]	3GPP TS 33.127: "Lawful Interception (LI) Architecture and Functions".

[4]	3GPP TS 33.128: "Lawful Interception (LI) Protocol and Procedures".

[5]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[6]	3GPP TS 23.501: "System Architecture for the 5G System".

[7]	3GPP TS 24.174: "Support of multi-device and multi-identity in the IP Multimedia Subsystem (IMS); Stage 3".

[8]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS), Stage 2".

[9]	3GPP TR 33.928: "ADMF Provisioning Logic for LI".

[10]	3GPP TR 33.929-1: "Lawful Interception Implementation Guidance; Part 1: LI for IMS-based services".

[11]	3GPP TR 33.929-2: "Lawful Interception Implementation Guidance; Part 2: LI for IMS based STIR/SHAKEN".

[12]	3GPP TR 33.929-3: "Lawful Interception Implementation Guidance; Part 3: LI for Messaging Services".

[13]	3GPP TR 33.929-4: "Lawful Interception Implementation Guidance; Part 4: LI for Data in 5G Core".

[14]	3GPP TR 33.929-5: "Lawful Interception Implementation Guidance; Part 5: ID Association Caching ".

[15]	3GPP TR 33.929-6: "Lawful Interception Implementation Guidance; Part 6: LI for IMS based RCS".

[16]	3GPP TR 33.929-7: "Lawful Interception Implementation Guidance; Part 7: Location Services".

[17]	3GPP TR 33.929-9: "Lawful Interception Implementation Guidance; Part 9: LI for PTC".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

None.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3GPP	3rd Generation Partnership Program

ADMF	Administration Function

AMF	Access and Mobility Management Function

APN	Access Pointe Name

AS	Application Server

BBIFF	Bearer Binding Intercept and Forward Function

BBIFF-C	Bearer Binding Intercept and Forward Function Control plane

BBIFF-U	Bearer Binding Intercept and Forward Function User plane

BGCF	Border Gateway Control Function

CC	Content of Communication

CSCF	Call Session Control Function

CSP	Communication Service Provider

CUPS	Control and User Plane Separation

DNN	Data Network Name

eCNAM	Enhanced Calling NAMe

EPC	Evolved Packet Core

E-CSCF	Emergency Call Session Control Function

HPLMN	Home PLMN

HSS	Home Subscriber Server

H-SMF	Home SMF

H-UPF	Home UPF

IBCF	Interrogating Border Control Function

I-CSCF	Interrogating CSCF

ID	Identity

IMPI	IP Multimedia Private Identity

IMPU	IP Multimedia Public Identity

IMS	IP Multimedia Subsystem

IMS-AGW	IMS Media Gateway

IRI	Intercept Related Information

LALS 	Lawful Access Location Services

LBO	Local Break Out

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

LICF	Lawful Interception Control Function

LI_HI1	LI_Handover Interface 1

LI_HI2	LI_Handover Interface 2

LI_HI3	LI_Handover Interface 3

LIPF	LI Provisioning Function

LI_T1	Lawful Interception Triggering Interface 1

LI_T3	Lawful Interception Triggering Interface 3

LI_X1	Lawful Interception Internal Interface 1

LI_X2	Lawful Interception Internal Interface 2

LI_X3	Lawful Interception Internal Interface 3

LMISF	LI Mirror IMS State Function

LMISF-CC	LMISF for the handling of CC

LMISF-IRI	LMISF for the handling of IRI

LTE	Long Term Evolution

MCPTT	Mission Critical Push To Talk

MDF	Mediation and Delivery Function

MDF2	Mediation and Delivery Function 2

MDF3	Mediation and Delivery Function 3

MGCF	Media Gateway Control Function

MMS	Multimedia Message Service

MRFC	Media Resource Function Controller

MRFP	Multimedia Resource Function Processor

N9HR	N9 Home Routed

NAS	Non-Access Stratum

NR	New Radio

PASSporT	Personal ASSertion Token

P-CSCF	Proxy Call Session Control Function

PDN	Packet Data Network

PEI	Permanent Equipment Identifier

PGW	PDN Gateway

PGW-C	PDN Gateway Control Plane

PGW-U	PDN Gateway User Plane

PLMN	Public Land Mobile Network

PoC	Push to talk over Cellular

POI	Point Of Interception

PTC	Push to Talk over Cellular

RCD	Rich Call Data

RCS	Rich Communication Suite

S8HR	S8 Home Routed

S-CSCF	Serving Call Session Control Function

SDP	Session Description Protocol

SHAKEN	Signature-based Handling of Asserted information using toKENs

SMF	Session Management Function

SGW	Serving Gateway

SGW-C	Serving Gateway Control Plane

SGW-U	Serving Gateway User Plane

SMS	Short Messaging Service

SUPI	Subscriber Permanent Identifier

SIP	Session Initiation Protocol

STIR	Secure Telephony Identity Revisited

TF	Triggering Function

TrGW	Transit Gateway

UDM	Unified Data Management

UE	User Equipment

UPF	User Plane Function

VPLMN	Visited PLMN

V-SMF	Visted SMF

V-UPF	Visited UPF

xCC	LI_X3 Communications Content

xIRI	LI_X2 Intercept Related Information

# 4	Illustration of LI for MMS

# 5	Example call flows of LI for MMS

Annex C (informative):
Bibliography

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-04 | SA3LI#93 |  |  |  |  | Proposed for the draft TR | 0.0.0 |