3GPP SA3LI#93	S3i240305

# Washington DC; April 16-19, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0634 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | STIR/SHAKEN – changes to accommodate the ReportDiversionPASSporTInfo - 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia, Nokia Shanghai Bell) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2024-04-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the requirement is to report the "div": PASSporTs for all the subsequent redirections when the ReportDiversionPASSporTInfo is set to "True" for the target, then an implementation that adheres to the current text would result in over-reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR modifies the text to produce a result that would report the "div" PASSporTs for all the subsequent redirections when the ReportDiversionPASSporTInfo is set to true. The conditions that define the rules are enhanced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect implementation that results in over-reporting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.11.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i240228 |  |  |  |  |  |  |  |  |

### ** First Change **

#### 7.11.2.2	Signature generation

The IRI-POI present in the Telephony AS or IBCF, shall generate an xIRI containing a STIRSHAKENSignatureGeneration record under the following conditions:

-	Telephony AS or IBCF is interacting with the SIGNING AS. Whether it is the Telephony AS or IBCF for sessions is based on network configuration and local policy of the CSP as described in clause 7.11.2.4.

-	When P-Asserted Identity or From header of SIP INVITE request received from S-CSCF is a target identity with the conditions mentioned below:

-	The identities in one or both of those headers are used to interact with the SIGNING AS.

-	The "shaken" PASSporT is not received in the SIP INVITE request from the S-CSCF.

-	The "shaken" PASSporT with those identities in the "orig" claim is received from the SIGNING AS.

-	The "shaken" PASSporT with those identities in the "orig" claim is included in the outgoing SIP INVITE.

-	When the "ReportDiversionPASSporTInfo" parameter is set to "True" in the ActivateTask with P-Asserted Identity or From header of SIP INVITE request received from S-CSCF is a target identity with the conditions mentioned below:

-	The identities in one or both of those headers are used to interact with the SIGNING AS.

-		A "div" PASSporT with those identities in the "orig" claim of "div" PASSporT is received from the SIGNING AS.

-	The "div" PASSporT with those identities in the "orig" claim is included in the outgoing SIP INVITE.

-	When Diversion header or the History Info of SIP INVITE request received from the S-CSCF includes a target identity with the conditions mentioned below:

-	The identities in one or both of those headers are used to interact with the SIGNING AS.

-	The "div" PASSporT with those identities in the "div" claim is not received in the SIP INVITE request from the S-CSCF.

-	The "div" PASSporT with those identities in the "div" claim is received from the SIGNING AS.

-	The "div" PASSporT with those identities in the "div" claim is included in the outgoing SIP INVITE.

-	The identities in P-Asserted Identity or From of SIP INVITE received from the S-CSCF are used to interact with the SIGNING AS.

-	A "div" PASSporT with the identities in P-Asserted Identity or From of SIP INVITE request received from S-CSCF are included in the "orig" claim of "div" PASSporT received from the SIGNING AS.

-	The "div" PASSporT with the identities in P-Asserted Identity or From of SIP INVITE request received from S-CSCF are included in the "orig" claim of "div" PASSporT in the outgoing SIP INVITE.

-	When the "ReportDiversionPASSporTInfo" parameter is set to "True" in the ActivateTask with Diversion or HistoryInfo header of SIP INVITE request received from S-CSCF includes the target identity with the conditions mentioned below:

-	The identities in P-Asserted Identity or From of SIP INVITE received from the S-CSCF are used to interact with the SIGNING AS.

-	A "div" PASSporT with the identities in P-Asserted Identity or From of SIP INVITE request received from S-CSCF are included in the "orig" claim of "div" PASSporT received from the SIGNING AS.

-	The "div" PASSporT with the identities in P-Asserted Identity or From of SIP INVITE request received from S-CSCF are included in the "orig" claim of "div" PASSporT in the outgoing SIP INVITE.

-	The "div" PASSporT with that identity in the "div" claim was previously received in the SIP INVITE request from the S-CSCF or from the Signing AS.

-	When Request URI of outgoing SIP INVITE is a target non-local ID and is present in the "dest" claim of "shaken" or "div" PASSporT received from the SIGNING AS and the same is included in the outgoing SIP INVITE.

-	When Telephony AS is interacting with the SIGNING AS, and when Request URI of SIP INVITE received from the S-CSCF is a target identity with the conditions mentioned below:

-	The identity is used to interact with the SIGNING AS.

-	The "div" PASSporT with that identity in the "div" claim is received from the SIGNING AS.

-	The "div" PASSporT with that identity in the "div" claim is included in the outgoing SIP INVITE.

When the target is local to the network (i.e. not a target non-local ID), the STIRSHAKENSignatureGeneration includes only the PASSporT received in the SIGING AS response with the following rules:

-

-	If REQUEST URI or To header in the SIP INVITE received from the S-CSCF is a target identity, then only the "div" PASSporT received from the SIGNING AS with those identities in the "div" claim of "div" PASSporT.

-	When the "ReportDiversionPASSporTInfo" parameter is set to "False" in the ActivateTask:

-	If P-Asserted Identity or From header in the SIP INVITE received from the S-CSCF is a target identity, then only "shaken" PASSporT received from the SIGNING AS with those identities in the "orig" claim of the "shaken" PASSporT.

-	If Diversion or HistoryInfo header in the SIP INVITE received from the S-CSCF is a target identity, then only the "div" PASSporT received from the SIGNING AS with those identities in the "div" claim of "div" PASSporT.

-	If REQUEST URI or To header in the SIP INVITE received from the S-CSCF is a target identity, then only the "div" PASSporT received from the SIGNING AS with those identities in the "div" claim of "div" PASSporT.

-	When the "ReportDiversionPASSporTInfo" parameter is set to "True" in the ActivateTask, in addition to the reporting as described above (for "ReportDiversionPASSporTInfo" parameter is set to "False"), also report the "div" PASSporTs for all the subsequent redirections that have the identity of the P-Asserted Identity or the From header in the "orig" claim of those PASSporTs.

When the target is non-local ID, STIRSHAKENSignatureGeneration includes all of the PASSporT included in the outgoing SIP message.

The following table contains parameters, with IRITargetIdentifier, generated by the IRI-POI.

Table 7.11.2.2-1: Payload for STIRSHAKENSignatureGeneration record

| Field name | Description | M/C/O |
| --- | --- | --- |
| pASSporTs | Identifies the content of the SIP Identity headers added by the originating network and transit networks This is a set of PASSporT parameter. See table 7.11.2.2-2. | M |
| encapsulatedSIPMessage | Encapsulated SIP INVITE request that includes SIP Identity header carrying the PASSporT (Outgoing SIP request) based on the structure defined in table 7.12.4.2-2 (see NOTE below). | M |
| NOTE 1: For the backward compatibility purposes the parameter is coded as OPTIONAL in the ASN.1 schema (Annex A.). NOTE 2: The same SIP message may be encapsulated in the xIRI IMSMessage as well. |  |  |

Table 7.11.22-2: Details for PASSporT parameter

| Field name | Description | M/C/O |
| --- | --- | --- |
| pASSporTHeader | PASSporT Header as defined in RFC 8224 [70] clause 4 for "shaken” PASSporT, in RFC 8946 [76] clause 3 for "div” PASSportT and in TS 24.229 [74]. See table 7.11.2.2-3. | M |
| pASSporTPayload | PASSporT Payload as defined in RFC 8224 [70] clause 4 for "shaken” PASSporT, in RFC 8946 [76] clause 3 for "div” PASSporTand in TS 24.229 [74]. See table 7.11.2.2-4. | M |
| pASSporTSignature | PASSporT Signature as defined in RFC 8224 [70] clause 4 for "shaken” PASSporT, in RFC 8946 [76] clause 3 for "div” PASSporTand in TS 24.229 [74]. | M |

Table 7.11.2.2-3: Details for pASSporTHeader parameter

| Field name | Description | M/C/O |
| --- | --- | --- |
| type | Shall be populated with the type contained in the PASSporT Header as defined in RFC 8225 [69] clause 4.1 for "shaken” PASSporT and in RFC 8946 [76] clause 3 for "div” PASSporT. | M |
| algorithm | Shall be derived from the value of the 'alg' parameter of the PASSporT Header as defined in RFC 8225 [69] clause 4.2 for "shaken” PASSporT and in RFC 8946 [76] clause 3 for “div” PASSporT. | M |
| ppt | Shall be derived from the value of the 'ppt' parameter of the PASSporT Header as defined in RFC 8225 [69] clause 8.1 for “shaken” PASSporT if the PASSporT Header contains a ppt parameter and in RFC 8946 [76] clause 3 for “div” PASSporT. | C |
| x5u | Shall be populated with the URI contained in the 'x5u' parameter of the PASSporT Header as defined in RFC 8225 [69] clause 4.3 for “shaken” PASSporT and in RFC 8946 [76] clause 3 for "div” PASSporT. | M |

Table 7.11.2.2-4: Details for pASSporTPayload parameter

| Field name | Description | M/C/O |
| --- | --- | --- |
| issuedAtTime | Shall be populated with the GenrealizedTime format timestamp converted from the NumericDate contained in the 'iat' parameter of the PASSporT Payload as defined in RFC 8225 [69] clause 5.1.1 and in RFC 8946 [76] clause 3. | M |
| originator | Shall be populated with the value of the "orig" claim of the PASSporT Payload as defined in RFC 8225 [69] clause 5.2.1 and in RFC 8946 [76] clause 3. | M |
| destination | Shall contain the list of destinations contained in the "dest" claim of the PASSporT Payload as defined in RFC 8225 [69] clause 5.2.1 and in RFC 8946 [76] clause 3. | M |
| diversion | Shall be populated with the "div" claim of the "div" PASSporT payload. For first diversion this contains the original identifier of the destination as defined in RFC 8946 [76] clause 3 for “div" PASSporT. | C |
| attestation | Indicates the attestation level as defined in RFC 8588 [71] clause 4 for the "shaken” PASSporT. The different values of attestation level are A = Full Attestation, B= Partial Attestation, C = Gateway Attestation (see NOTE 3 below). | M |
| origID | Shall be populated with the value of the origID contained in the 'origid' parameter of the PASSporT Payload as defined in RFC 8588 [71] clause 5 for the “shaken” PASSporT (see NOTE 4 below). | M |
| NOTE 3: For the backward compatibility purposes the parameter is coded as MANDATORY in the ASN.1 schema (Annex A). 'attestation' is mandatory for "shaken" PASSporT and absent for "div" PASSporT. Therefore, for div PASSporT, since "attestation" is not available, the placeholder value "Not available" shall be used. NOTE 4: For the backward compatibility purposes the parameter is coded as MANDATORY in the ASN.1 schema (Annex A). This parameter is mandatory for "shaken" PASSporT and absent for the "div" PASSporT. Therefore, for div PASSporT, since "origId" is not available, the placeholder value "Not available" shall be used. |  |  |

### ** End of all Changes **