3GPP TSG-SA3 Meeting #93-LI	s3i240271

# Washington DC, United States, 16th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0645 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of Location Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, Location to be included in LI product is derived from the User Context stored at the AMF. This CR adds the capability to report the unrefined/modified location sent by the RAN before the AMF resolves the location into 29.571 format. Primary cell of the secondary cell group is also missing from the 29.571 location format and is added here in 38.413 format. Support for IAB-MT User location and NTN specific location information was also not present in the current 29.571 location format and is added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify clause 7.3.3.2.4 to carry 38.413 location format, Add PSCell information to table in clause 7.3.3.2.5, add various location elements in 38.413 format to 7.3.3.2.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ability to report location format in 38.413 native will not be supported, as well as various location parameters will be missing in LI reporting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.2.4, 7.3.3.2.5, 7.3.3.2.6, Attachment ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: Merge request: !259 Comitt Hash: 7580a5f7531799667f15ed08deb07b66eb1192fd |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***********START OF CHANGES***************

##### 7.3.3.2.4	Type: UserLocation

The UserLocation type is derived from the data present in the UserLocation type defined in TS 29.571 [17] clause 5.4.4.7 or, alternatively, the encapsulated User Location Information IE defined in TS 38.413 [23] clause 9.3.1.16. If the NF has locations from multiple RAT types, all appropriate location fields within the userLocation parameter shall be used.

Table 7.3.3.2.4-1 contains the details for the UserLocation type.

Table 7.3.3.2.4-1: Definition of type UserLocation

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| eUTRALocation | EUTRALocation | 0..1 | Location information type derived from the data present in the EutraLocation type defined in TS 29.571 [17] clause 5.4.4.8. See clause 7.3.3.2.5 for details on this data type. This parameter shall be present if an EUTRA Location is available at the NF. | C |
| nRLocation | NRLocation | 0..1 | Location information type derived from the data present in the NrLocation type defined in TS 29.571 [17] clause 5.4.4.9. See clause 7.3.3.2.6 for details on this data type. This parameter shall be present if an NR Location is available at the NF. | C |
| n3GALocation | N3GALocation | 0..1 | Location information type derived from the data present in the N3gaLocation type defined in TS 29.571 [17] clause 5.4.4.10. See clause 7.3.3.2.7 for details on this data type. This parameter shall be present if a non-3GPP access Location is available at the NF. | C |
| uTRALocation | UTRALocation | 0..1 | Location information type derived from the data present in the UtraLocation type defined in TS 29.571 [17] clause 5.4.4.52. See clause 7.3.3.2.8 for details on this data type. This parameter shall be present if a UTRAN Access Location is available at the NF. | C |
| gERALocation | GERALocation | 0..1 | Location information type derived from the data present in the GeraLocation type defined in TS 29.571 [17] clause 5.4.4.53. See clause 7.3.3.2.9 for details on this data type. This parameter shall be present if a GERAN Access Location is available at the NF. | C |
| userLocationInformation | ExternalASNType | 1 | Provides location information of the target. The ExternalASNType.encodedASNValue.alignedPER choice shall be used when populating this type and it shall be populated with the contents of the the User Location Information IE from TS 38.413 [23] clause 9.3.1.16. | C |

##### 7.3.3.2.5	Type: EUTRALocation

The EUTRALocation type is derived from the data present in the EutraLocation type defined in TS 29.571 [17] clause 5.4.4.8 and TS 38.413 [23] clause 9.3.1.16.

Table 7.3.3.2.5-1 contains the details for the EUTRALocation type.

Table 7.3.3.2.5-1: Definition of type EUTRALocation

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| tAI | TAI | 1 | Tracking Area Identity of the target. If the TAI information is not available, the TAC of the TAI shall be set to one reserved value (e.g. 0x0000, see clause 19.4.2.3 of 3GPP TS 23.003 [19]) and the value of the ignoreTAI parameter shall be set to TRUE. | M |
| eCGI | ECGI | 1 | E-UTRA Cell Identity for the cell where the target is located. | M |
| ageOfLocationInfo | AgeOfLocation | 0..1 | The value represents the elapsed time in minutes since the last network contact of the mobile station. Shall be present if known at the NF where the POI is located. | C |
| uELocationTimestamp | Timestamp | 0..1 | The value represents the UTC time when the EUTRALocation information was acquired. Shall be present if known at the NF where the POI is located. | C |
| geographicalInformation | UTF8String | 0..1 | Shall be present if known at the NF where the POI is located. If present, this parameter shall be populated with the Hexidecimal value of the location encoded as described in TS 23.032 [104] clauses 6 and 7.3. | C |
| geodeticInformation | UTF8String | 0..1 | Shall be present if known at the NF where the POI is located. If present, this parameter shall be populated with the Hexidecimal value of the location encoded as described in ITU-T Recommendation Q.763 (1999) [105] clause 3.88. | C |
| globalNGENbID | GlobalRANNodeID | 0..1 | Indicates the global identity of the ng-eNodeB in which the UE is currently located. Shall be present if known at the NF where the POI is located. | C |
| cellSiteInformation | CellSiteInformation | 0..1 | Contains location information for the cell site being reported. Shall be present if known at the NF where the POI is located or known at the MDF. | C |
| globalENbID | GlobalRANNodeID | 0..1 | Indicates the global identity of the eNodeB in which the UE is currently located. Shall be present if known at the NF where the POI is located. | C |
| ignoreTAI | BOOLEAN | 0..1 | This flag, when present, shall indicate if the tAI shall be ignored. When present, it shall be set as follows: - TRUE: tAI shall be ignored. - FALSE: tAI shall not be ignored. | C |
| ignoreECGI | BOOLEAN | 0..1 | This flag, when present, shall indicate if the eCGI shall be ignored. When present, it shall be set as follows: - TRUE: eCGI shall be ignored. - FALSE: eCGI shall not be ignored. | C |
| pSCellInformation | NCGI | 0..1 | Indicates primary cell information of the secondary NR cell group. Shall be included if secondary cell information is present. See TS 38.413 [23] clause 9.3.1.16. | C |

##### 7.3.3.2.6	Type: NRLocation

The NRLocation type is derived from the data present in the NrLocation type defined in TS 29.571 [17] clause 5.4.4.9.

Table 7.3.3.2.6-1 contains the details for the NRLocation type.

Table 7.3.3.2.6-1: Definition of type NRLocation

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| tAI | TAI | 1 | Tracking Area Identity of the target. If the TAI information is not available, the TAC of the TAI shall be set to one reserved value (e.g. 0x0000, see clause 19.4.2.3 of 3GPP TS 23.003 [19]) and the value of the ignoreTAI parameter shall be set to TRUE. | M |
| nCGI | NCGI | 1 | NR Cell Identity for the cell where the target is located. | M |
| ageOfLocationInfo | AgeOfLocation | 0..1 | The value represents the elapsed time in minutes since the last network contact of the mobile station. Shall be present if known at the NF where the POI is located. | C |
| uELocationTimestamp | Timestamp | 0..1 | The value represents the UTC time when the NRLocation information was acquired. Shall be present if known at the NF where the POI is located. | C |
| geographicalInformation | UTF8String | 0..1 | Shall be present if known at the NF where the POI is located. If present, this parameter shall be populated with the Hexidecimal value of the location encoded as described in TS 23.032 [104] clauses 6 and 7.3. | C |
| geodeticInformation | UTF8String | 0..1 | Shall be present if known at the NF where the POI is located. If present, this parameter shall be populated with the Hexidecimal value of the location encoded as described in ITU-T Recommendation Q.763 (1999) [105] clause 3.88. | C |
| globalGNbID | GlobalRANNodeID | 0..1 | Indicates the global identity of the gNodeB in which the UE is currently located. Shall be present if known at the NF where the POI is located. | C |
| cellSiteInformation | CellSiteInformation | 0..1 | Contains location information for the cell site being reported. Shall be present if known at the NF where the POI is located or known at the MDF. | C |
| ignoreNCGI | BOOLEAN | 0..1 | This flag, when present, shall indicate if the nCGI shall be ignored. When present, it shall be set as follows: - TRUE: nCGI shall be ignored. - FALSE: nCGI shall not be ignored. | C |
| pSCellInformation | NCGI | 0..1 | Indicates primary cell information of the secondary NR cell group. Shall be included if secondary cell information is present. See TS 38.413 [23] clause 9.3.1.16. | C |
| nRNTNTAIInfo | NRNTNTAIInfo | 0..1 | Contains NR NTN TAI Information. When present the TAI shall be ignored. See TS 29.571 [17] clause 5.4.4.9 and TS 38.413 [23] clause 9.3.3.53. | C |
| iABMTUserLocation | IABMTUserLocation | 0..1 | Indicates the user location information of a mobile IAB-MT which is co-located with the UE’s serving IAB Cell. See TS 38.413 [23] clause 9.3.1.260. | C |

Table 7.3.3.2.6-2: Structure of NTNTAIInfo Parameter

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| servingPLMN | PLMNID | 1 | Indicates the PLMN currently serving the target. | M |
| tACListNRNTN | TAC | 1...MAX | Provides the available TAC List for the NTN connection of the target. | M |
| uELocationDerivedNTNTAC | TAC | 0..1 | Contains information derived from the actual UE location, if available. | C |

Table 7.3.3.2.6-3: Structure of IABMTUserLocation Parameter

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| nRCGI | NCGI | 1 | Provides the NCGI of the cell serving the mobile IAB-MT. | M |
| tAI | TAI | 0..1 | Provides the TAI supported by the cell which is serving the mobile IAB-MT. | C |

***END CHANGES TO MAIN DOCUMENT***

********START ATTCHMENT CHANGES**********

---a/33128/r18/TS33128Payloads.asn
+++b/33128/r18/TS33128Payloads.asn

@@ -6514,7 +6514,8 @@ UserLocation ::= SEQUENCE

6514	6514		    nRLocation                  [2] NRLocation OPTIONAL,

6515	6515		    n3GALocation                [3] N3GALocation OPTIONAL,

6516	6516		    uTRALocation                [4] UTRALocation OPTIONAL,

6517		-	    gERALocation                [5] GERALocation OPTIONAL

6517	+	    gERALocation                [5] GERALocation OPTIONAL,

6518	+	    userLocationInformation     [6] ExternalASNType OPTIONAL

6518	6519		}

6519	6520

6520	6521		-- TS 29.571 [17], clause 5.4.4.8

@@ -6530,7 +6531,8 @@ EUTRALocation ::= SEQUENCE

6530	6531		    cellSiteInformation         [8] CellSiteInformation OPTIONAL,

6531	6532		    globalENbID                 [9] GlobalRANNodeID OPTIONAL,

6532	6533		    ignoreTAI                   [10] BOOLEAN OPTIONAL,

6533		-	    ignoreECGI                  [11] BOOLEAN OPTIONAL

6534	+	    ignoreECGI                  [11] BOOLEAN OPTIONAL,

6535	+	    pSCellInformation           [12] NCGI OPTIONAL

6534	6536		}

6535	6537

6536	6538		-- TS 29.571 [17], clause 5.4.4.9

@@ -6544,7 +6546,10 @@ NRLocation ::= SEQUENCE

6544	6546		    geodeticInformation         [6] UTF8String OPTIONAL,

6545	6547		    globalGNbID                 [7] GlobalRANNodeID OPTIONAL,

6546	6548		    cellSiteInformation         [8] CellSiteInformation OPTIONAL,

6547		-	    ignoreNCGI                  [9] BOOLEAN OPTIONAL

6549	+	    ignoreNCGI                  [9] BOOLEAN OPTIONAL,

6550	+	    pSCellInformation           [10] NCGI OPTIONAL,

6551	+	    nRNTNTAIInfo                [11] NRNTNTAIInfo OPTIONAL,

6552	+	    iABMTUserLocation           [12] IABMTUserLocation OPTIONAL

6548	6553		}

6549	6554

6550	6555		-- TS 29.571 [17], clause 5.4.4.10

@@ -7395,6 +7400,21 @@ SIPCellularNetworkInformation ::= SEQUENCE

7395	7400		    servingPLMN          [2] PLMNID OPTIONAL

7396	7401		}

7397	7402

7403	+	-- TS 29.571 [17], clause 5.4.4.9 and

7404	+	NRNTNTAIInfo ::= SEQUENCE

7405	+	{

7406	+	    servingPLMN                   [1] PLMNID,

7407	+	    tACListNRNTN                  [2] SEQUENCE (SIZE(1..MAX)) OF TAC,

7408	+	    uELocationDerivedNTNTAC       [3] TAC OPTIONAL

7409	+	}

7410	+

7411	+	-- TS 38.413 [23], clause 9.3.1.260

7412	+	IABMTUserLocation ::= SEQUENCE

7413	+	{

7414	+	    nRCGI             [1] NCGI,

7415	+	    tAI               [2] TAI OPTIONAL

7416	+	}

7417	+

7398	7418		-- ==================================================

7399	7419		-- Structures to allow reuse of encapsulated payloads

7400	7420		-- ==================================================

********END ALL CHANGES**********