3GPP TSG-SA3 Meeting #93-LI	s3i240308

# Washington DC, United States, 16th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0631 | rev | 1 | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of parameter indicating the version of xIRI an IRI is generated from |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI15 |  |  |  |  |  | Date: |  |  | 2024-04-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, both xIRI and IRI records contain the OID of the ASN.1 module used to encode the record. In some cases, this OID is useful to determine which fields are expected to be present in a record or which events can be expected to be reported by the network. The issue is that the OID sent by the MDF in IRI is based on the version used by the MDF to encode the IRI record and does not necessarily reflect the version used at the POI to generate the xIRI. If the versions are mismatched, this may result confusion when certain fields or records that would be expected based on the MDF version are not supported by the POI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An additional parameter is added to the IRIPayload sequence and text describing its use is added to the document. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LEA may have difficulty determining what events/parameters are to be expected resulting in LI product being misinterpreted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2, 5.5.2, 5.5.3, TS33128Payloads.asn |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TDoc s3i240223 (CR 0629) is the Release 15 mirror for this CR. TDoc s3i240225 (CR 0631) is the Release 17 mirror for this CR. TDoc s3i240226 (CR 0632) is the Release 18 mirror for this CR. This CR is associated with the following changes in the Forge: Merge request: !256 Commit hash: 748bf89f5e8a7510f68d71cc0fac61331b697cd1 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i240225 |  |  |  |  |  |  |  |  |

## **** START OF FIRST CHANGE (MAIN DOCUMENT) ****

### 5.3.2	Usage for realising LI_X2

The POI sending xIRI over the LI_X2 interface shall set the PDU type field within the xIRI to "X2 PDU". (see ETSI TS 103 221-2 [8] clause 5.1).

Where a single xIRI is sent as a result of a network procedure (i.e. as result of several signaling messages exchanged between the target UE and the network), the POI sending the xIRI shall set the Payload Direction field (see ETSI TS 103 221-2 [8] clause 5.2.6) based on the initiator of the network procedure.

Unless otherwise specified by the relevant clause, the payload shall consist of a BER-encoded TS33128Payloads.XIRIPayload structure. The payload format (see ETSI TS 103 221-2 [8] clause 5.4) shall be set according to the relevant clause of the present document (the value 2 is used for TS 33128Payloads.XIRIPayload).The TLS transport profile (see ETSI TS 103 221-2 [8] clause 6) shall be supported and used by default.

Unless otherwise specified, xIRI shall include the timestamp and sequence number conditional attribute fields, with the timestamp value set to the time at which the event occurred.

Unless otherwise specified, the "Matched Target Identifier" conditional attribute shall be set to indicate what target identity was matched to generate the xIRI (see ETSI TS 103 221-2 [8] clause 5.3.18).

Unless otherwise specified, the "Other Target Identifier" conditional attribute shall be set with all other target identities present at the NF that contains the POI (see ETSI TS 103 221-2 [8] clause 5.3.19).

Unless otherwise specified, the NFID conditional attribute (see ETSI TS 103 221-2 [8] clause 5.3.7) should be set to indicate the NF that contains the POI. The NFID is defined as a unique identifier assigned to the NF by the network (e.g. FQDN) per carrier implementation and referred to in the following clauses.

Unless otherwise specified, the IPID (see ETSI TS 103 221-2 [8] clause 5.3.8) should be set to indicate the POI (within the NF) that generated the xIRI for the conditional attribute field.

Unless otherwise specified, the XIRIPayload.xIRIPayloadOID field shall be set to the value of the xIRIPayloadOID specified in the version of the ASN.1 defined in Annex A used by the IRI-POI to generate and encode the xIRI payload.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

### 5.5.2	Usage for realising LI_HI2

The IRI messages sent over LI_HI2 are structured as a header and a payload. The header contains general information like LIID, timestamp, correlation information (as for example defined in ETSI TS 102 232-1 [9]). The payload contains intercept related information based on information that the MDF2 has received from sources in the network, such as the IRI-POI as described in clauses 6 and 7 of the present document. Details of the IRI messages can be found in Annex A of the present document. Messages defined as passing over the LI_HI2 interface shall be passed as the payload of the threeGPP33128DefinedIRI field (see TS ETSI 102 232 -7 [10] clause 15).

If the LI_X2 contains the NFID conditional attribute (see ETSI TS 103 221-2 [8] clause 5.3.7), this shall be mapped into the PSHeader networkFunctionIdentifier (see ETSI TS 102 232-1 [9] clause 5.2.14 and ETSI TS 102 232-7 [10] clause 15.3).

If the LI_X2 contains the IPID conditional attribute (see ETSI TS 103 221-2 [8]), the EIPID parameter (see ETSI TS 102 232-1 [9] clause 5.2.13) shall be populated by the MDF2 with the IPID value.

Unless otherwise specified, the IRIPayload.relativeOID field shall be set to the value of the iRIPayloadOID specified in the version of the ASN.1 defined in Annex A used by the MDF2 to generate and encode the IRI payload.

If the IRI is generated from an xIRIPayload received over LI_X2, if the release and version of the xIRIPayload.relativeOID is different from the release and version of the IRIPayload.relativeOID the IRIPayload.mediatedFromIndicator.xIRIRelativeOID choice shall be used and set to the value of the xIRIPayload.relativeOID of the xIRI message used to generate the IRI message.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

### 5.5.3	Usage for realising LI_HI3

The CC sent over LI_HI3 is structured as a header and a payload. The header contains general information like LIID, timestamp, correlation information (as for example defined in ETSI TS 102 232-1 [9]). The payload contains content of communication based on information that the MDF3 has received from sources in the network, such as the CC-POI as described in clauses 6 and 7 of the present document. Details of the CC can be found in Annex A of the present document. CC defined as passing over the LI_HI3 interface shall be passed as the payload of the threeGPP33128DefinedCC field (see ETSI TS 102 232-7 [10] clause 15).

If the LI_X3 contains the NFID conditional attribute (see ETSI TS 103 221-2 [8] clause 5.3.7), this shall be mapped into the PSHeader networkFunctionIdentifier (see ETSI Ts 102 232-1 [9] clause 5.2.14 and ETSI TS 102 232-7 [10] clause 15.3).

If the LI_X3 contains the IPID conditional attribute (see ETSI TS 103 221-2 [8]), the EIPID parameter (see ETSI TS 102 232-1 [9] clause 5.2.13) shall be populated by the MDF3 with the IPID value.

Unless otherwise specified, the CCPayload. cCPayloadOID field shall be set to the value of the cCPayloadOID specified in the version of the ASN.1 defined in Annex A used by the MDF3 to generate and encode the CC payload.

NOTE:	ETSI TS 102 232-1 [9] specifies in clause 6.4 a transport layer based on TCP. However, based on agreement between network operator and LEA, in scenarios where it may not be possible to achieve the necessary LI data rates based on the transport layer based on single TCP connection, alternative profiles may be considered (e.g. based on UDP, multi path TCP or other protocols). Any alternative profile needs to ensure that LI reliability, security and completeness requirements as specified in TS 33.126 [3] are met.

## **** END OF CHANGES (MAIN DOCUMENT) ***

## **** START OF FIRST CHANGE (ATTACHMENTS) ****

---a/33128/r17/TS33128Payloads.asn
+++b/33128/r17/TS33128Payloads.asn

@@ -210,9 +210,10 @@ XIRIEvent ::= CHOICE

210	210

211	211		IRIPayload ::= SEQUENCE

212	212		{

213		-	    iRIPayloadOID       [1] RELATIVE-OID,

214		-	    event               [2] IRIEvent,

215		-	    targetIdentifiers   [3] SEQUENCE OF IRITargetIdentifier OPTIONAL

213	+	    iRIPayloadOID         [1] RELATIVE-OID,

214	+	    event                 [2] IRIEvent,

215	+	    targetIdentifiers     [3] SEQUENCE OF IRITargetIdentifier OPTIONAL,

216	+	    mediatedFromIndicator [4] MediatedFromIndicator OPTIONAL

216	217		}

217	218

218	219		IRIEvent ::= CHOICE

@@ -392,6 +393,11 @@ IRITargetIdentifier ::= SEQUENCE

392	393		    provenance                                          [2] TargetIdentifierProvenance OPTIONAL

393	394		}

394	395

396	+	MediatedFromIndicator ::= CHOICE

397	+	{

398	+	    xIRIRelativeOID   [1] RELATIVE-OID

399	+	}

400	+

395	401		-- ==============

396	402		-- HI3 CC payload

397	403		-- ==============

## **** END OF ALL CHANGES ***