3GPP TSG-SA3 Meeting #93-LI	s3i240321

# Washington DC, United States, 16th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0645 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of Location Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US, Ofcom (CH)) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2024-04-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, Location to be included in LI product is derived from the User Context stored at the AMF. This CR adds the capability to report IAB-MT User location (in TS 38.413 format) and NTN specific location information (in TS29.571 format) which were not present in the current TS 29.571 based location structure |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add reporting capabilty for NTN specific locaiton information and IAB-MT User locaiton. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Various location reporting parameters will not be supported. CSPs may not be able to meet obligations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.2.6, Attachment ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: https://forge.3gpp.org/rep/sa3/li/-/merge_requests/259/diffs?commit_id=90a131905e069e6dcd13b67ce327f0e8e1ac4620 Merge request: !259 Comitt Hash: 90a131905e069e6dcd13b67ce327f0e8e1ac4620 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i240271 |  |  |  |  |  |  |  |  |

***********START OF CHANGES***************

***********START OF FIRST CHANGE*************

##### 7.3.3.2.6	Type: NRLocation

The NRLocation type is derived from the data present in the NrLocation type defined in TS 29.571 [17] clause 5.4.4.9.

Table 7.3.3.2.6-1 contains the details for the NRLocation type.

Table 7.3.3.2.6-1: Definition of type NRLocation

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| tAI | TAI | 1 | Tracking Area Identity of the target. If the TAI information is not available, the TAC of the TAI shall be set to one reserved value (e.g. 0x0000, see clause 19.4.2.3 of 3GPP TS 23.003 [19]) and the value of the ignoreTAI parameter shall be set to TRUE. | M |
| nCGI | NCGI | 1 | NR Cell Identity for the cell where the target is located. | M |
| ageOfLocationInfo | AgeOfLocation | 0..1 | The value represents the elapsed time in minutes since the last network contact of the mobile station. Shall be present if known at the NF where the POI is located. | C |
| uELocationTimestamp | Timestamp | 0..1 | The value represents the UTC time when the NRLocation information was acquired. Shall be present if known at the NF where the POI is located. | C |
| geographicalInformation | UTF8String | 0..1 | Shall be present if known at the NF where the POI is located. If present, this parameter shall be populated with the Hexidecimal value of the location encoded as described in TS 23.032 [104] clauses 6 and 7.3. | C |
| geodeticInformation | UTF8String | 0..1 | Shall be present if known at the NF where the POI is located. If present, this parameter shall be populated with the Hexidecimal value of the location encoded as described in ITU-T Recommendation Q.763 (1999) [105] clause 3.88. | C |
| globalGNbID | GlobalRANNodeID | 0..1 | Indicates the global identity of the gNodeB in which the UE is currently located. Shall be present if known at the NF where the POI is located. | C |
| cellSiteInformation | CellSiteInformation | 0..1 | Contains location information for the cell site being reported. Shall be present if known at the NF where the POI is located or known at the MDF. | C |
| ignoreNCGI | BOOLEAN | 0..1 | This flag, when present, shall indicate if the nCGI shall be ignored. When present, it shall be set as follows: - TRUE: nCGI shall be ignored. - FALSE: nCGI shall not be ignored. | C |
| nRNTNTAIInfo | NRNTNTAIInfo | 0..1 | Contains NR NTN TAI Information. When present the TAI is set to the reserved value and shall be ignored by the receiver. See TS 29.571 [17] clause 5.4.4.9 and TS 38.413 [23] clause 9.3.3.53. | C |
| iABMTUserLocation | IABMTUserLocation | 0..1 | Indicates the user location information of a mobile IAB-MT which is co-located with the UE’s serving IAB Cell. See TS 38.413 [23] clause 9.3.1.260. | C |

Table 7.3.3.2.6-2: Structure of NTNTAIInfo Parameter

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| servingPLMN | PLMNID | 1 | Indicates the PLMN currently serving the target. | M |
| tACListNRNTN | TAC | 1..MAX | Provides the available TAC List for the NTN connection of the target. | M |
| uELocationDerivedNTNTAC | TAC | 0..1 | Contains information derived from the actual UE location, if available. | C |

Table 7.3.3.2.6-3: Structure of IABMTUserLocation Parameter

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| nRCGI | NCGI | 1 | Provides the NCGI of the cell serving the mobile IAB-MT. | M |
| tAI | TAI | 0..1 | Provides the TAI supported by the cell which is serving the mobile IAB-MT. | C |

***END CHANGES TO MAIN DOCUMENT***

********START ATTCHMENT CHANGES**********

START OF CHANGE 1

---a/33128/r18/TS33128Payloads.asn
+++b/33128/r18/TS33128Payloads.asn

@@ -6544,7 +6544,9 @@ NRLocation ::= SEQUENCE

6544	6544		    geodeticInformation         [6] UTF8String OPTIONAL,

6545	6545		    globalGNbID                 [7] GlobalRANNodeID OPTIONAL,

6546	6546		    cellSiteInformation         [8] CellSiteInformation OPTIONAL,

6547		-	    ignoreNCGI                  [9] BOOLEAN OPTIONAL

6547	+	    ignoreNCGI                  [9] BOOLEAN OPTIONAL,

6548	+	    nRNTNTAIInfo                [10] NRNTNTAIInfo OPTIONAL,

6549	+	    iABMTUserLocation           [11] IABMTUserLocation OPTIONAL

6548	6550		}

6549	6551

6550	6552		-- TS 29.571 [17], clause 5.4.4.10

@@ -7395,6 +7397,21 @@ SIPCellularNetworkInformation ::= SEQUENCE

7395	7397		    servingPLMN          [2] PLMNID OPTIONAL

7396	7398		}

7397	7399

7400	+	-- TS 29.571 [17], clause 5.4.4.9 and TS 38.413 [23] clause 9.3.3.53

7401	+	NRNTNTAIInfo ::= SEQUENCE

7402	+	{

7403	+	    servingPLMN                   [1] PLMNID,

7404	+	    tACListNRNTN                  [2] SEQUENCE (SIZE(1..MAX)) OF TAC,

7405	+	    uELocationDerivedNTNTAC       [3] TAC OPTIONAL

7406	+	}

7407	+

7408	+	-- TS 38.413 [23], clause 9.3.1.260

7409	+	IABMTUserLocation ::= SEQUENCE

7410	+	{

7411	+	    nRCGI             [1] NCGI,

7412	+	    tAI               [2] TAI OPTIONAL

7413	+	}

7414	+

7398	7415		-- ==================================================

7399	7416		-- Structures to allow reuse of encapsulated payloads

7400	7417		-- ==================================================

END OF CHANGE 1

********END ALL CHANGES**********