3GPP TSG-SA3 Meeting #93-LI	s3i240324

# Washington DC, United States, 16th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0654 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to ASN.1 references based on new drafting rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2024-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The drafting rules were updated to provide a definitive way to write references to the ASN.1 and specific ASN.1 parameters. Several of the existing references in the document have not been updated to use the newly specified convention. While making these corrections, text was modified to take advantage of the reference format in order to provide clearer explanations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correctly formats ASN.1 references. Modifies text to provide additional clarity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ASN.1 references will not align with the drafting rules. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.3, 6.2.2A.2.3, 6.2.3.7, 6.2.3.8, 6.3.2.3.2, 7.3.3.2.2, 7.4.4.1, 7.7.2.3, 7.7.2.4, 7.8.2.3, 7.8.2.4, 7.11.3, 7.12.4.2.1, 7.12.7.1, 7.13.3.6.1.2, 7.14.2.11, 7.15.3, 7.16.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i240286 |  |  |  |  |  |  |  |  |

## **** START OF FIRST CHANGE (MAIN DOCUMENT) ****

#### 6.2.2.3	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in AMF, the MDF2 shall generate the corresponding IRI message and deliver over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received in the xIRI over LI_X2. This record may be enriched with any additional information available at the MDF (e.g. additional location information).

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp shall be set to the time at which the AMF event was observed (i.e. the timestamp field of the X2 PDU).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIType  parameter (see ETSI TS 102 232-1 [9] clause 5.2.10) shall be included and coded according to table 6.2.2-7.

Table 6.2.2-7: IRI type for IRI messages

| IRI message | IRI type |
| --- | --- |
| AMFRegistration | REPORT |
| AMFDeregistration | REPORT |
| AMFLocationUpdate | REPORT |
| AMFStartOfInterceptionWithRegisteredUE | REPORT |
| AMFUnsuccessfulProcedure | REPORT |
| AMFIdentifierAssociation | REPORT |
| AMFPositioningInfoTransfer | REPORT |
| AMFRANHandoverCommand | REPORT |
| AMFRANHandoverRequest | REPORT |
| AMFUEConfigurationUpdate | REPORT |
| AMFRANTraceReport | REPORT |
| AMFUEPolicyTransfer | REPORT |
| AMFUEServiceAccept | REPORT |

These IRI messages shall omit the CIN (see ETSI TS 102 232-1 [9] clause 5.2.4).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIContents.threeGPP33128DefinedIRI field of the LI_HI2 message shall be populated with the BER-encoded IRIPayload as described in in ETSI TS 102 232-7 [10] clause 15.

When an additional warrant is activated on a target UE and the LIPF uses the same XID for the additional warrant, the MDF2 shall be able to generate and deliver the IRI message containing the AMFStartOfInterceptionWithRegisteredUE record to the LEMF associated with the additional warrant without receiving a corresponding xIRI. The payload of the AMFStartOfInterceptionWithRegisteredUE record is specified in table 6.2.2.2.5-1.

If the MDF2 did not receive a previous AMFStartOfInterceptionWithRegisteredUE.timeOfRegistration parameter for the same registration from the IRI-POI, the MDF2 shall set the value of the AMFStartOfInterceptionWithRegisteredUE.timeOfRegistration field to the time provided in the timestamp previously received in the header of the related AMFRegistration xIRI.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

##### 6.2.2A.2.3	Transmission to the ICF

When activated (see clause 5.2.7), the IEF shall establish a TLS connection to the ICF as given over LI_XEM1. If the IEF fails to establish a TLS connection, it shall report an error over LI_XEM1 using the error reporting mechanisms described in ETSI TS 103 221-1 [7] and attempt to reconnect after a configurable period of time.

When a record has been generated as described in clause 6.2.2A.2.2, the IEF shall encode the IEFAssociationRecord or IEFDeassociationRecord as a BER-encoded@TS33128IdentityAssociation.IEFMessage , and transmit it to the ICF over the established TLS connection.

The IEF may transmit a keepalive request using the keepalive record defined in Annex F. Upon receiving a keepalive request, the ICF shall respond with a keepaliveResponse record containing the same sequence number used in the request. The circumstances under which the IEF transmits keepalive requests is out of scope of the present document.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 6.2.3.7	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the SMF or the IRI-POI in the UPF, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g. additional location information).

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time at which the SMF event was observed (i.e. the timestamp field of the xIRI).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIType parameter (see ETSI TS 102 232-1 [9] clause 5.2.10) shall be included and coded according to table 6.2.3-14.

Table 6.2.3-14: IRI type for IRI messages

| Record type | IRI Type |
| --- | --- |
| SMFPDUSessionEstablishment | BEGIN |
| SMFPDUSessionRelease | END |
| SMFPDUSessionModification | CONTINUE |
| SMFStartOfInterceptionWithEstablishedPDUSession | BEGIN |
| SMFUnsuccessfulProcedure | REPORT |
| SMFMAPDUSessionEstablishment | BEGIN |
| SMFMAPDUSessionRelease | END |
| SMFMAPDUSessionModification | CONTINUE |
| SMFStartOfInterceptionWithEstablishedMAPDUSession | BEGIN |
| SMFMAUnsuccessfulProcedure | REPORT |
| SMFPDUtoMAPDUSessionModification | CONTINUE |
| PDHeaderReport | REPORT |
| PDSummaryReport | REPORT |

IRI messages associated with the same PDU Session shall be assigned the same CIN (see ETSI TS 102 232-1 [9] clause 5.2.4).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIContents.threeGPP33128DefinedIRI field (see ETSI TS 102 232-7 [10] clause 15) of the LI_HI2 message shall be populated with the BER-encoded IRIPayload.

When an additional warrant is activated on a target UE and the LIPF uses the same XID for the additional warrant, the MDF2 shall be able to generate and deliver the IRI message containing the SMFStartOfInterceptionWithEstablishedPDUSession record and the SMFStartOfInterceptionWithEstablishedMAPDUSession record to the LEMF associated with the additional warrant without receiving a corresponding xIRI. The payload of the SMFStartOfInterceptionWithEstablishedPDUSession record is specified in table 6.2.3-4, while the payload of the SMFStartOfInterceptionWithEstablishedMAPDUSession record is specified in table 6.2.3-9. The MDF2 shall generate and deliver the IRI message containing the SMFStartOfInterceptionWithEstablishedPDUSession record for each of the established PDU sessions to the LEMF associated with the new warrant. The MDF2 shall generate and deliver the IRI message containing the SMFStartOfInterceptionWithEstablishedMAPDUSession record for each of the established MA PDU sessions to the LEMF associated with the new warrant.

If the MDF2 did not receive a previous SMFStartOfInterceptionWithEstablishedPDUSession.timeOfSessionEstablishment or SMFStartOfInterceptionWithEstablishedMAPDUSession.timeOfSessionEstablishment for the same session from the IRI-POI, , the MDF2 shall set the value of the SMFStartOfInterceptionWithEstablishedPDUSession.timeOfSessionEstablishment or SMFStartOfInterceptionWithEstablishedMAPDUSession.timeOfSessionEstablishment to the time provided in the timestamp previously received in the header of the related SMFPDUSessionEstablishment or SMFMAPDUSessionEstablishment xIRI.

When the delivery of packet header information is authorised and approach 2 described in clause 6.2.3.9.1 is used, the MDF2 shall generate the IRI message and send it over LI_HI2 without undue delay when xCC is received over LI_MDF from the MDF3. The MDF2 shall generate packet header information reporting as described in clause 6.2.3.5.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 6.2.3.8	Generation of CC over LI_HI3

When the xCC is received over LI_X3, the MDF3 shall emit the CC over LI_HI3 without undue delay.

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStampstructure shall be set to the time that the UPF observed the data (i.e. the timestamp field of the xCC). The LIID and CID fields shall correctly reflect the target identity and communication session to which the CC belongs.

The MDF3 shall populate the @LI-PS-PDU.payload.cCPayloadSequence.cCContents.threeGPP33128DefinedCC field (see clause 5.5.3 of the present document) with a BER-encoded CCPayload structure containing either:

1. 	The uPFCCPDU field containing the GTP-U packet received over LI_X3. It shall only be used if the content of the GTP-U packet is an IPv4 or IPv6 packet.

2. 	The extendedUPFCCPDU field as described in table 6.2.3-15.

The MDF3 shall support delivery using either option.

Table 6.2.3-15: ExtendedUPFCCPDU structure

| Field name | Description | M/C/O |
| --- | --- | --- |
| payload | Payload of the GTP-U packet without GTP-U encapsulation. Content shall be supplied according to table 6.2.3-16. | M |
| qFI | Shall be populated with the QoS Flow Identifier value from the GTP-U header extension (see TS 38.415 [41] clause 5.5.3.3) if present over LI_X3. | C |

Table 6.2.3-16: UPFCCPDUPayload structure

| Field name | Description |
| --- | --- |
| uPFIPCC | Contains an IPv4 or IPv6 packet |
| uPFEthernetCC | Contains an Ethernet frame |
| uPFUnstructuredCC | Contains an unstructured packet |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

##### 6.3.2.3.2	Option A

The IRI message the MDF2 generates shall contain a copy of the relevant record received in the xIRI over LI_X2 and provide it over LI_HI2 without undue delay.

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time at which the MME event was observed (i.e. the timestamp field of the X2 PDU).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIType (see ETSI TS 102 232-1 [9] clause 5.2.10) shall be included and coded according to table 6.3.2-8.

Table 6.3.2-8: IRI type for IRI messages

| IRI message | IRI type |
| --- | --- |
| MMEAttach | REPORT |
| MMEDetach | REPORT |
| MMELocationUpdate | REPORT |
| MMEStartOfInterceptionWithEPSAttachedUE | REPORT |
| MMEUnsuccessfulProcedure | REPORT |
| MMEIdentifierAssociation | REPORT |
| MMEPositioningInfoTransfer | REPORT |
| EPSRANHandoverCommand | REPORT |
| EPSRANHandoverRequest | REPORT |
| MMERANTraceReport | REPORT |
| MMEUEServiceAccept | REPORT |

These IRI messages shall omit the CIN (see ETSI TS 102 232-1 [9] clause 5.2.4).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIContents.threeGPP33128DefinedIRI field of the LI_HI2 message shall be populated with the BER-encoded  IRIPayload as described in in ETSI TS 102 232-7 [10] clause 15.

When an additional warrant is activated on a target UE and the LIPF uses the same XID for the additional warrant, the MDF2 shall be able to generate and deliver the IRI message containing the MMEStartOfInterceptionWithEPSAttachedUE record to the LEMF associated with the additional warrant without receiving a corresponding xIRI. The payload of the MMEStartOfInterceptionWithEPSAttachedUE record is specified in table 6.3.2-6.

For records related to SMS over NAS in EPS, the process detailed in clause 6.3.2.3.3 shall be used.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

##### 7.3.3.2.2	Type: Location

Table 7.3.3.2.2-1 contains the details for the Location type.

Table 7.3.3.2.2-1: Definition of type Location

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| locationInfo | LocationInfo | 0..1 | Location information type derived from the data present in the ProvideLocInfo structure defined in TS 29.518 [22] clause 6.4.6.2.6. This parameter shall be used any time information from the ProvideLocInfo structure needs to be reported. This parameter shall also be used whenever information from the UserLocation type needs to be reported. | C |
| positioningInfo | PositioningInfo | 0..1 | Location information type derived from the data present in the ProvidePosInfo structure defined in TS 29.518 [22] clause 6.4.6.2.3. This parameter shall be used any time information from LCS operations needs to be reported from the 5GC. This structure may also be used any time information from the ProvidePosInfo structure needs to be reported. | C |
| locationPresenceReport | LocationPresenceReport | 0..1 | Location information type derived from the data present in the AMFEventReport structure defined in TS 29.518 [22] clause 6.2.6.2.5. This parameter shall be used any time location information from Namf_EventExposure Service operations needs to be reported. | C |
| fourGPositioningInfo | FourGPositioningInfo | 0..1 | Location information type derived from the data present in the Location-Report-Answer structure defined in TS 29.172 [53] clause 7.3.2 and the Provide Subscriber Location Answer defined in TS 29.172 [53] Table 6.2.2.2.3-1. This parameter shall be used any time information from LCS operations needs to be reported from the EPC. This structure may also be used any time information from the Provide-Location-Answer structure needs to be reported. | C |
| fourGLocationInfo | FourGLocationInfo | 0..1 | Location information type derived from the data present in the EPS-Location-Info structure defined in TS 29.272 [106] clause 7.3.111 and the User Location Information structure defined in TS 29.274 [87] clause 8.21. This parameter shall be used any time information from the EPSLocationInformation structure needs to be reported. This parameter shall also be used whenever information from the EPSUserLocation type needs to be reported. | C |
| iMSLocation | IMSLocation | 0..1 | Location information type derived from the data present in IMS SIP P-Access-Network-Info, Cellular-Network-Info and Geolocation headers. This parameter shall be used any time location information needs to be reported from IMS. This parameter shall only be used to report the target’s location. | C |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.4.4.1	Generation of IRI over LI_HI2

When an IRI-POI in the MMS Proxy-Relay generated xIRI is received over LI_X2, the MDF2 shall send an xIRI over LI_HI2 without undue delay. The xIRI shall contain a copy of the record received over LI_X2. The record may be enriched by other information available at the MDF (e.g. additional location information).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIContents.threeGPP33128DefinedIRI field of the LI_HI2 message shall be populated with the BER-encoded IRIPayload  as described in ETSI TS 102 232-7 [10] clause 15.

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time that the MMS event was observed (i.e. the timestamp field of the xIRI). The LIID and CID fields shall correctly reflect the target identity and communication session to which the IRI belongs.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.7.2.3	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the NEF, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g. additional location information).

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp shall be set to the time at which the NEF event was observed (i.e. the timestamp field of the xIRI).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIType parameter (see ETSI TS 102 232-1 [9] clause 5.2.10) shall be included and coded according to table 7.7.2-6.

Table 7.7.2-6: IRI type for IRI messages

| Record type | IRI Type |
| --- | --- |
| NEFPDUSessionEstablishment | BEGIN |
| NEFPDUSessionRelease | END |
| NEFPDUSessionModification | CONTINUE |
| NEFStartOfInterceptionWithEstablishedPDUSession | BEGIN |
| NEFUnsuccessfulProcedure | REPORT or CONTINUE |

IRI messages associated with the same PDU Session shall be assigned the same CIN (see ETSI TS 102 232-1 [9] clause 5.2.4).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIContents.threeGPP33128DefinedIRI field of the LI_HI2 message shall be populated with the BER-encoded IRIPayload as described in ETSI TS 102 232-7 [10] clause 15.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.7.2.4	Generation of CC over LI_HI3

When xCC is received over LI_X3 from the CC-POI in the NEF, the MDF3 shall populate the ETSI TS 102 232-1 [9] @LI-PS-PDU.payload.cCPayloadSequence.cCContents.threeGPP33128DefinedCC field with a CCPayload.pDU.nIDDCCPDU and send it over LI_HI3 interface to LEMF without undue delay.

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time that the NEF observed the data (i.e. the timestamp field of the xCC). The LIID and CID fields shall correctly reflect the target identity and communication session to which the CC belongs.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.8.2.3	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the SCEF/IWK-SCEF, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g. additional location information).

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time at which the SCEF event was observed (i.e. the timestamp field of the xIRI).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIType parameter (see ETSI TS 102 232-1 [9] clause 5.2.10) shall be included and coded according to table 7.8.2-6.

Table 7.8.2-6: IRI type for IRI messages

| Record type | IRI Type |
| --- | --- |
| SCEFPDNConnectionEstablishment | BEGIN |
| SCEFPDNConnectionRelease | END |
| SCEFPDNConnectionUpdate | CONTINUE |
| SCEFStartOfInterceptionWithEstablishedPDNConnection | BEGIN |
| SCEFUnsuccessfulProcedure | REPORT |

IRI messages associated with the same PDU Session shall be assigned the same CIN (see ETSI TS 102 232-1 [9] clause 5.2.4).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIContents.threeGPP33128DefinedIRI field of the LI_HI2 message shall be populated with the BER-encoded IRIPayload as described in ETSI TS 102 232-7 [10] clause 15.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.8.2.4	Generation of CC over LI_HI3

When xCC is received over LI_X3 from the CC-POI in the SCEF, the MDF3 shall populate the ETSI TS 102 232-1 [9] @LI-PS-PDU.payload.cCPayloadSequence.cCContents.threeGPP33128DefinedCC field with a CCPayload.pDU.nIDDCCPDU and send it over LI_HI3 interface to LEMF without undue delay.

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time that the SCEF observed the data (i.e. the timestamp field of the xCC). The LIID and CID fields shall correctly reflect the target identity and communication session to which the CC belongs.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

### 7.11.3	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from an IRI-POI, the MDF2 shall generate the corresponding IRI message and deliver over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received in the xIRI over LI_X2.

The MDF2 shall be able to remove information regarded as content from RCD or eCNAM parameters in the case of an IRI-only warrant. The details of what needs to be removed and under what circumstances are outside the scope of the present document.

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time present in the timestamp field of the xIRI.

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIContents.threeGPP33128DefinedIRI field of the LI_HI2 message shall be populated with the BER-encoded IRIPayload as described in ETSI TS 102 232-7 [10] clause 15.

The STIRSHAKENSignatureGeneration and STIRSHAKENSignatureValidation IRI messages shall have the same CIN as in the other IRI messages delivered for the IMS session (see ETSI TS 102 232-1 [9] clause 5.2.4).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIType parameter (see ETSI TS 102 232-1 [9] clause 5.2.10) shall be included and coded according to table 7.11.3-1.

Table 7.11.3-1: IRI type for IRI messages

| Record type | IRI Type |
| --- | --- |
| STIRSHAKENSignatureGeneration | REPORT |
| STIRSHAKENSignatureValidation | REPORT |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

##### 7.12.4.2.1	IMS Message

For an intercepted IMS based communication (see clause 7.12.2.8), the IRI-POI present in the IMS Signalling Function shall generate the xIRI IMSMessage from the SIP message used to handle that IMS based communication. All SIP messages use the same xIRI record as shown in table 7.12.4.2-1.

Table 7.12.4.2-1: Payload for IMSMessage record

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| payload | IMSPayload | 1 | One of the following payload types (other payload types may be added in future versions of the specification): - encapsulatedSIPMessage shall be chosen when the SIP message does not contain any unauthorised information. - modifiedSIPMessage shall be chosen when the SIP message contains information that is not authorised for reporting. | M |
| sessionDirection | SessionDirection | 1 | Indicates the direction of the SIP session: fromTarget, toTarget, combined (if target calls him/herself) or indeterminate if the direction cannot be determined reliable (see NOTE). | M |
| voIPRoamingIndication | VoIPRoamingIndication | 0..1 | Indicates whether the roaming mode is inbound LBO, S8HR or N9HR when the target is in roaming situation. | C |
| location | Location | 0..1 | Location with timestamp, if available. Shall include all location information for the target UE available at the NF where the POI is located encoded as a Location.iMSLocation parameter. | C |
| accessNetworkInformation | SEQUENCE OF SIPAccessNetworkInformation | 0..MAX | Provides non-location related access network information. Shall be present if available at the NF where the POI is located. One instance of SIPAccessNetworkInformation shall be used for each P-Access-Network-Information header. | C |
| cellularNetworkInformation | SEQUENCE OF SIPCellularNetworkInformation | 0..MAX | Provides non-location related cellular network information. Shall be present if available at the NF where the POI is located. One instance of SIPCellularNetworkInformation shall be used for each Cellular-Network-Info header. | C |
| NOTE: When an incoming call to a target is redirected to another user, the sessionDirection field shall be set to toTarget. When an incoming call from a target non-local ID to an IMS user is redirected to, the sessionDirection field shall be set to fromTarget. |  |  |  |  |

Table 7.12.4.2-2: Void

The IRI-POI present in the IMS Signalling Function generating an xIRI containing an IMSMessage record shall set:

-	The Payload Direction field in the PDU header to the direction of the signalling message carried in the IRI payload (see ETSI TS 103 221-2 [8] clause 5.2.6). If the signalling message was sent from the target, the Direction Value "3" (sent from the target) shall be used, if the signalling message was sent to the target, the Direction Value "2" (sent to the target) shall be used; if the direction could not be determined reliably, the Direction Value "1" (not known to the POI) shall be used. If the SIP message is sent from and to the target, the Direction Value "4" (more than one direction) shall be used. For the SIP messages generated by the network, the Direction Value "5" (not applicable) shall be used.

-	The conditional source IPv4 address or source IPv6 address field in the PDU header to the source IP address of the intercepted SIP message (see ETSI TS 103 221-2 [8] clause 5.3). It shall contain the source address of the packet from the 32-bit "Source Address" field in IPv4, as defined in IETF RFC 791 [34], or from the 128-bit "Source Address" field in IPv6, as defined in IETF RFC 2460 [27].

-	The conditional destination IPv4 address or destination IPv6 address field in the PDU header to the destination IP address of the intercepted SIP message (see ETSI TS 103 221-2 [8] clause 5.3). It shall contain the destination address of the packet from the 32-bit "Source Address" field in IPv4, as defined in IETF RFC 791 [34], or from the 128-bit "Source Address" field in IPv6, as defined in IETF RFC 2460 [27].

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.12.7.1	General

When an xIRI is received over LI_X2 from the IRI-POI, the MDF2 shall send the IRI message over LI_HI2 according to clause 5.5.2 of the present document without undue delay.

The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF2 (e.g. additional location information).

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time present in the timestamp field of the xIRI.

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIContents.threeGPP33128DefinedIRI field of the LI_HI2 message shall be populated with the BER-encoded IRIPayload as described in ETSI TS 102 232-7 [10] clause 15.

IRI messages associated with the same IMS session shall have the same CIN (see ETSI TS 102 232-1 [9] clause 5.2.4).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIType (see ETSI TS 102 232-1 [9] clause 5.2.10) shall be included and coded according to table 7.12.7.1-1.

Table 7.12.7.1-1: IRI type for IRI messages

| Record type | IRI Type |
| --- | --- |
| IMSMessage | REPORT |
| StartOfInterceptionForActiveIMSSession | REPORT |
| IMSCCUnavailable | REPORT |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

###### 7.13.3.6.1.2	StartOfInterceptForRegisteredRCSUser record

Table 7.13.3.6.1.2-1: Payload for StartOfInterceptForRegisteredRCSUser record

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| rCSTargetIdentities | SEQUENCE OF RCSIdentity | 1..MAX | RCS target identities. All identities associated to the target known at the POI shall be included. | M |
| rCSRegistrationInformation | SEQUENCE OF RCSRegistrationInformation | 1..MAX | Shall contain the registration information known at the RCS Server. If more than one set of registration information is present, the POI shall populate one instance of the RCSRegistrationInformation type per set of information. | M |
| userOnline | BOOLEAN | 1 | Shall be set to TRUE if at least one endpoint is currently online for the user. Shall be set to FALSE if no endpoints are currently online for the user. | M |
| location | Location | 0..1 | Shall include the location of the primary endpoint registered to the target RCS user when reporting of the target’s location information is authorized and available. | C |
| additionalInstanceLocation | SEQUENCE OF AdditonalInstanceLocation | 0..MAX | Shall include locations of any additional endpoints registered to the target RCS user when reporting of the target’s location information is authorized and available. | C |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 7.14.2.11	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the EES, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g. additional location information).

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time at which the EES event was observed (i.e. the timestamp field of the xIRI).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIType parameter (see ETSI TS 102 232-1 [9] clause 5.2.10) shall be included and coded according to table 7.14.2-19.

Table 7.14.2-19: IRI type for IRI messages

| Record type | IRI Type |
| --- | --- |
| EESEECRegistration | REPORT |
| EESEASDiscovery | REPORT |
| EESEASDiscoverySubscription | REPORT |
| EESEASDiscoveryNotification | REPORT |
| EESAppContextRelocation | REPORT |
| EESACRSubscription | REPORT |
| EESACRNotification | REPORT |
| EESEECContextRelocation | REPORT |
| EESStartOfInterceptionWithRegisteredEEC | REPORT |

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIContents.threeGPP33128DefinedIRI field of the LI_HI2 message shall be populated with the BER-encoded IRIPayload as described in ETSI TS 102 232-7 [10] clause 15.

When an additional warrant is activated on a target UE and the LIPF uses the same XID for the additional warrant, the MDF2 shall be able to generate and deliver the IRI message containing the EESStartOfInterceptionWithRegisteredEEC record to the LEMF associated with the additional warrant without receiving a corresponding xIRI. The payload of the EESStartOfInterceptionWithRegisteredEEC record is specified in table 7.14.2-18. The MDF2 shall generate and deliver the IRI message containing the EESStartOfInterceptionWithRegistered record for each of the registrations to the LEMF associated with the new warrant.

MDF2 delivers the IRI to the LEMF with GPSI as the target identity if and only if GPSI is present in the xIRI.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

### 7.15.3	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the 5GMS AF, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g. additional location information).

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time at which the 5GMS AF event was observed (i.e. the timestamp field of the xIRI).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIType parameter shall be included and coded according to table 7.14.2-19 (see ETSI TS 102 232-1 [9] clause 5.2.10).

Table 7.15.3-1: IRI type for IRI messages

| Record type | IRI Type |
| --- | --- |
| FiveGMSAFServiceAccessInformation | REPORT |
| FiveGMSAFConsumptionReporting | REPORT |
| FiveGMSAFDynamicPolicyInvocation | REPORT |
| FiveGMSAFMetricsReporting | REPORT |
| FiveGMSAFNetworkAssistance | REPORT |
| FiveGMSAFUnsuccessfulProcedure | REPORT |
| FiveGMSAFStartOfInterceptionWithAlreadyConfiguredUE | REPORT |

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIContents.threeGPP33128DefinedIRI field of the LI_HI2 message shall be populated with the BER-encoded IRIPayload as described in ETSI TS 102 232-7 [10] clause 15.

MDF2 delivers the IRI to the LEMF with GPSI as the target identity if and only if GPSI is present in the xIRI.

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

### 7.16.3	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the NWDAF/RE-NWDAF, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g. additional location information).

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time at which the NWDAF/RE-NWDAF event was observed (i.e. the timestamp field of the xIRI).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIType parameter shall be included and coded according to table 7.14.2-19 (see ETSI TS 102 232-1 [9] clause 5.2.10).

Table 7.16.3-1: IRI type for IRI messages

| Record type | IRI Type |
| --- | --- |
| NWDAFEventsSubscription | REPORT |
| NWDAFEventsNotification | REPORT |
| NWDAFAnalyticsInfoQuery | REPORT |
| NWDAFRoamingAnalyticsSubscription | REPORT |
| NWDAFRoamingAnalyticsNotification | REPORT |

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIContents.threeGPP33128DefinedIRI field of the LI_HI2 message shall be populated with the BER-encoded IRIPayload as described in ETSI TS 102 232-7 [10] clause 15.

## **** END OF ALL CHANGES ***