SA3LI

Amsterdam; July 9-12, 2024

CONTRIBUTION

TITLE:	Discussion: S2bHR LI

SOURCE*:	Nokia

_______________________________

ABSTRACT

The document is for discussion to evaluate on whether there is any need to support the S2bHR LI and, if so, agree on any actions to be taken.

_______________________________

Background

3GPP SA3 LI specifications have defined the details of LI functions to be deployed in the VPLMN for voice-services with Home-Routed (HR) roaming architecture. The said functionalities were named after the interfaces used between the packet core NF in the VPLMN and the HPLMN, i.e. S8HR and N9HR.

In EPS,  the S8 represents the interface between SGW (in VPLMN) and PGW (in HPLMN). In 5GS, the N9 represents the interface between the V-UPF (in the VPLMN) and the H-UPF (in the HPLMN).

In both S8HR and N9HR, the LI in the VPLMN is provided by performing the deep packet inspection and look into the SIP messages present in those packets and when the SIP identifiers included in those SIP messages point to an inbound roaming target, the lawful interception would happen for CII and, if authorized, the CC.  LI specific functions referred to as LMISF (LMISF-IRI and LMISF-CC) were introduced as a part of S8HR LI and N9HR LI, to provide such deep packet inspection and when applicable, the required LI functions. Another logical function referred to as BBIFF (BBIFF-C and BBIFF-U) was introduced and was placed in the SGW (EPS) and UPF (5GS) to forward the packets that pass the EPS bearers/5GS QoS Flows to the LMISF. The same LMISF is used for both S8HR and N9HR.

The below figures (taken from TR 33.929-1) show three cases:

- N9HR.

- S8HR.

- S8UHR.

S8UHR is often referred to as S8HR. A fourth case that is missing in the 3GPP SA3 LI specifications is the point of this discussion paper.

|  | N9HR |
| --- | --- |
|  | S8HR |
|  | S8UHR |

Forth case (discussion)

As shown in the above figures, when an inbound roaming target is on EPS-based VPLMN, the BBIFF/BBIFF-U present in SGW/SGW-U would forward the user plane packets from the EPS bearers to the LMISF. Likewise, when an inbound roaming target is on 5GS-based VPLMN, the BBIFF-U present in V-UPF would forward the user packets from the 5GS QoS Flows to the LMISF.

In all of the above cases, the inbound roaming target is accessing the network via 3GPP access (i.e. LTE or NR). However, the case that is not considered in TS 33.127/TS 33.128 is the case where an inbound roaming UE accessing  network via non-3GPP access (e.g. Wi-Fi). When the 3GPP system happens to be the EPS, the user plane packets would pass via the ePDG to the PGW (in HPLMN) without hitting the SGW present in the VPLMN.

For the analysis purpose, a few diagrams from 3GPP TS 23.501 are referenced below:

UE in VPLMN and with 3GPP access, available today - shown for a recap with the similar styled figures

| UE via LTE in VPLMN with HR. S8HR LI based LI in VPLMN. | UE via NR in VPLMN with HR. N9HR LI based LI in VPLMN. |
| --- | --- |

UE in VPLMN with non-3GPP access

| UE via non-3GPP access (in a 5GS) in VPLMN with HR. N9HR HR LI based LI in VPLMN can be used. | UE via non-3GPP access (in an EPS) in VPLMN with HR. New HR LI will have to be defined for LI in VPLMN (S2bHR). |
| --- | --- |

Another angle:

On 5G NR, UPF can provide the BBIFF. On non-3GPP access, the ePDG shall provide the BBIFF.

Needs a detailed study

#### .10.5.2	Single IPsec SA per S2b bearer Scenario

This scenario assumes both UE, and ePDG support the establishment of a separate IPsec SA per S2b bearer, while the main IPSec SA is intended for the default bearer.

Figure 4.10.5.2-1: Single IPsec SA per S2b bearer

The SWu interface between the UE and the ePDG is identical for the GTP and PMIP variants of S2b. The UE establishes a separate SWu instance (i.e. a separate IPsec SA per [9]) for each PDN connection.

One default S2b bearer is established on the S2b interface when the UE connects to a PDN, and that remains established throughout the lifetime of the PDN connection to provide the UE with always-on IP connectivity to that PDN. Additional dedicated S2b bearers may be established on S2b for the same PDN connection depending on operator policy. The PGW establishes dedicated S2b bearers on S2b for the same PDN connection based on PCC decisions as specified in TS 23.203 [19].

The ePDG releases the SWu instance, including all IP Sec SAs associated with the SWu instance, where applicable, when the default S2b bearer of the associated PDN connection is released.

The S2b bearer is realized by the following elements:

-	A GTP tunnel on S2b transports the packets of an S2b bearer between the ePDG and a PDN GW;

-	The ePDG stores the mapping between IPSec SA and the corresponding S2b bearer;

-	The PDN GW stores the mapping between downlink packet filters and an S2b bearer.

The ePDG shall establish a SA per S2b bearer as per clause 7.10. The default EPC bearer maps to the initial SA. The ePDG shall maintain a 1 to 1 mapping between an S2b bearer and an IPsec SA.

Additionally, for these UEs, TFTs and bearer QoS information is conveyed from the ePDG to the UE in IKE v2 signalling associated with the corresponding SA at EPC bearer creation and EPC bearer modification. The bearer QoS information includes information regarding the QoS characteristics of the bearer (i.e. QCI, GBR and MBR).

The IKEv2 traffic selectors TSi and TSr, defined in [9], shall not be used to route packets to the IPSec SA.

In support for the UE connectivity with the PDN:

-	A SWu instance (i.e. an IKEv2 SA with one or more IPsec SA) transports the packets of all S2b bearer(s) for the same PDN Connection between the UE and the ePDG.

The PDN GW routes downlink packets to the different bearers based on the downlink packet filters in in the TFTs assigned to the S2b bearers in the PDN connection, in the same way as the PDN GW does on GTP-based S5/S8 bearers (see TS 23.401 [4] clause 4.7.2.2).

The ePDG routes uplink packets to the different bearers based on the incoming child SA and the corresponding S2b bearer.

The UE routes uplink packets to the SAs associated with the different S2b bearers based on the uplink packet filters in the TFTs assigned to the S2b bearers in the PDN connection, in the same way as a UE does for uplink traffic under 3GPP access. If no match is found, the uplink data packet shall be sent via the SA that does not have any uplink packet filter assigned. If all IPsec SAs have been assigned an uplink packet filter, the UE shall discard the uplink data packet.

To support QoS differentiation,

-	for downlink packets, if the ePDG sets the DSCP, the ePDG shall use the QCI, and optionally the ARP, of the S2b bearer via which the downlink packet was received to derive the DSCP value for downlink packets. If the ePDG sets the DSCP field, it may also set it in the outer IP header of the ESP packet.

NOTE 1:	The ePDG maintains the mapping of DSCP values used on S2b and the outer ESP tunnel to the UE.

NOTE 2:	Based on operator policy and/or regional/national regulations, the ePDG can apply a different DSCP value to the outer ESP tunnel packet than the DSCP value of the inner IP packet.

-	for uplink packets, the UE shall either use the DSCP value that it received from the ePDG, or the QCI in dedicated bearer QoS information to set the DSCP value for uplink packets. The mapping of QoS class to DSCP could be configurable in the UE and as an example can use the recommended mappings specified in 3GPP (mapping between standardized QCIs and Release 99 QoS parameter value in TS 23.401 [4]), IEEE Std. 802.11-2012 [64], and the Wi-Fi WMM-multimedia certification profile [86]. The UE may also use information included in dedicated QoS information for local aspects of admission control (e.g. application traffic shaping), however this is out of scope for this document.

S2bHR

If S2bHR is to be realized using the concepts of S8HR/N9HR, then the figure below would represent the S2bHR LI. The diagram shows, by implementing the BBIFF functions in  ePDG,  how the LMISF and the rest of the LI functions can provide the LI for inbound roamers that use non-3GPP access network in an EPS.

As with the case of S8HR and N9HR, S2bHR LI can be realized if and only if the encryption is disabled for the user plane packets so as to have those packets in clear at the ePDG.

When the inbound roaming UE uses non-3GPP access via 5GS, the N9HR should work fine as the user plane packets pass through the UPF as shown in the figure A.4-1 (shown below) of TS 33.127:

The concepts of S8HR/N9HR LI include two phases. The phase 1 applies to all inbound roaming UEs and phase 2 applies to incoming roaming targets. With BBIFF being enabled for phase1, the user plane packets from default bearer/QoS Flow of all inbound roaming UEs are delivered to the LMISF. When the concepts of S8HR/N9HR LI are extended to support S2bHR LI, one would think BBIFF present in the ePDG had to deliver the user plane packets (from default bearer) of all inbound roaming UEs to the LMISF.  This part needs to be studied and evaluated.

Other aspects to be considered before any conclusion:

There is a case of S2a mentioned in TS 23.401, figure 4.2.3-1:

- For this case there is no NF in the VPLMN that can provide the LI.

S2b is not always between VPLMN and HPLMN ( TS 23.401, figure 4.2.3-2):

- In this case, S8HR LI can be used. So, BBIFF in ePDG shall not provide the phase 1 for S2b.

There is S2c is mentioned in 23.402, figure 4.2.3-3:

Further reading is necessary as it is not clear what is this S2c interface.

PROPOSAL

Discuss and agree on the action to be taken in SA3 LI if it is a requirement to support LI in the VPLMN with VoWiFi?