SA3#96-LI	 S3i250010

Sophia Antipolis; January 27-31, 2024

Source:	Nokia, Rogers Communications

Title:	pCR to TR 33.929-5: Changes to support GPSI

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR updates the TR 33.929-5 to include the support of GPSI in the IEF events and HIQR responses.

BACKGROUND

The ID Association Caching capability, in its initial days, was allowing the LEAs to request and get the SUPI associated with a 5G-GUTI (or SUCI) and vice versa. Later, the capability was enhanced to include the PEI (when known), and recently, to include the GPSI (when known).

The TR 33.929-5, however, does not show the inclusion of GPSI. The changes are proposed to include the GPSI related changes to the TR.

PROPOSAL

Incorporate the following changes to TR 33.929-5.

***** First Change ****

## 4.2	IDs

ID Association Caching references the following IDs:

Non-permanent IDs:

-	SUCI.

-	5G-GUTI.

-	5G-S-TMSI.

Permanent IDs:

-	SUPI.

-	PEI.

-	GPSI.

5G-S-TMSI is always part of the 5G-GUTI as shown in figure 4.2-1 below.

Figure 4.2-1: Structure of 5G-GUTI and 5G-S-TMSI

The LEAs may include the SUCI or the 5G-S-TMSI or the 5G-GUTI in the Request to retrieve the associated SUPI and PEI. As a different scenario, the LEAs may instead include the SUPI in the Request to retrieve the associated 5G-GUTI.

***** Next Change ****

### 4.3.2	Caching

Whenever the AMF assigns a new 5G-GUTI to a SUPI, the IEF present within the AMF caches 5G-GUTI to SUPI association (referred to as ID association) at the ICF.

The SUCI (when known), GPSI (when known) and the PEI (when known) may also be tagged along with the 5G-GUTI to SUPI association. The Cell-Id and TA-ID are also cached. The Association Start Time (see clause 4.3.4.9) is also cached.

For caching the ID association, the IEF present in the AMF sends the IEF Association Record message to the ICF over the LI_XER interface as defined in TS 33.128 [4].

The ICF, upon receiving the IEF Association Record message, caches the new ID association with the all the information that it receives within the message. If the previously cached ID association for that SUPI contains the PEI, then the ICF transfers that PEI to the new ID association unless the new ID association includes the SUCI. This is because a new ID association for the same SUPI not including a SUCI is presumed to be for the same PEI. However, when a new ID association for the same SUPI that includes the SUCI is received, it is possible that the PEI may have changed and was not reported.

If the previously cached ID association for that SUPI contains the GPSI, then the ICF transfers that GPSI to the new ID association unless the new ID association includes the GPSI.

IEF present in the AMF also sends an IEF Association Record message when it detects that the GPSI is changed. The ICF, upon receiving the IEF Association Record message, caches the new ID association with the all the information it receives within the message.

***** Next Change ****

#### 4.3.5.3	ID association caching aspects

##### 4.3.5.3.1	Overview

The following information are mandatory for an IEF Association Record message.

| 5G-GUTI | SUPI | Time-stamp | TAI | N-CGI | N-CGI Time |
| --- | --- | --- | --- | --- | --- |

The Time-stamp present in the IEF Association Record is the Association Start Time for this 5G-GUTI and the Association End Time for the 5G-GUTI in the previous ID association cached in the ICF for the same SUPI.

The following information, if available, should be included in an IEF Association Record message:

| SUCI | PEI | GPSI | 5G TAI List |
| --- | --- | --- | --- |

The following information are mandatory for an IEF Disassociation Record message:

| 5G-GUTI | SUPI | Time-stamp | N-CGI | N-CGI Time |
| --- | --- | --- | --- | --- |

The Time-stamp present in the IEF Dissociation Record is the Association End Time for this ID association cached in the ICF for this 5G-GUTI.

##### 4.3.5.3.2	Illustration 1

Figure 4.3.5.3.2-1 illustrates one example of ID association caching aspects within the ICF.

Figure 4.3.5.3.2-1: Caching and de-caching principles

Figure 4.3.5.3.2-1 shows 5 scenarios of the ICF handling of the cached ID associations. Note that in scenario 4, the IEF Association Record has the SUCI but not the PEI. As a result, the subsequent ID associations (scenario 5) will not have the PEI.

1.	A 5G-GUTI is assigned to the UE. The IEF sends an IEF Association Record message that contains the SUCI, 5G-GUTI, SUPI, PEI, GPSI, TAI, N-CGI, Time-stamp of Tg and N-CGI-Time. The ICF caches the information as the ID association with Tg as the Association Start Time.

2.	A new 5G-GUTI is assigned to the UE. The IEF sends an IEF Association Record message that contains the 5G-GUTI, SUPI, TAI, N-CGI, Time-stamp of Tp and N-CGI-Time. The ICF caches the information as the ID association with Tp as the Association Start Time. The ICF stores the Tp as the Association End Time in the previous ID association and begins the de-caching process for that ID association. The ICF transfers the PEI and GPSI from the previous ID association to the new ID association.

3.	The 5G-GUTI is de-assigned for the UE. The IEF sends an IEF Disassociation Record message that contains the 5G-GUTI, SUPI, TAI, N-CGI, Time-stamp of Te and N-CGI-Time. The ICF begins the de-caching process for that ID association with the storing of Te as the Association End Time within the ID association record.

4.	A 5G-GUTI is assigned to the UE. The IEF sends an IEF Association Record message that contains the SUCI, 5G-GUTI, SUPI, TAI, N-CGI, Time-stamp of Ty and N-CGI-Time. The ICF caches the information as the ID association with Ty as the Association Start Time. In this example, neither PEI nor GPSI is part of the ID association.

5.	A new 5G-GUTI is assigned to the UE. The IEF sends an IEF Association Record message that contains the 5G-GUTI, SUPI, TAI, N-CGI, Time-stamp of Tw and N-CGI-Time. The ICF caches the information as the ID association with Tw as the Association Start Time. The ICF stores the Tw as the Association End Time in the previous ID association and begins the de-caching process for that ID association. In this example, since the previous ID association had neither PEI nor GPSI, the new ID association will also have neither PEI nor GPSI.

##### 4.3.5.3.3	Illustration 2

Figure 4.3.5.3.3-1 illustrates another example of the ID association caching aspects within the ICF.

Figure 4.3.5.3.3-1: Caching and de-caching principles

Figure 4.3.5.3.3-1 shows 4 scenarios of the ICF handling of the cached ID associations. Note that in scenario 3, the IEF Association Record has the SUCI and the PEI. Therefore, the subsequent ID associations (scenario 4) will have the PEI.

1.	A 5G-GUTI is assigned to the UE. The IEF sends an IEF Association Record message that contains the SUCI, 5G-GUTI, SUPI, PEI, GPSI, TAI, N-CGI, Time-stamp of Tg and N-CGI-Time. The ICF caches the information as the ID association with Tg as the Association Start Time.

2.	A new 5G-GUTI is assigned to the UE. The IEF sends an IEF Association Record message that contains the 5G-GUTI, SUPI, TAI, N-CGI, Time-stamp of Tp and N-CGI-Time. The ICF caches the information as the ID association with Tp as the Association Start Time. The ICF stores the Tp as the Association End Time in the previous ID association and begins the de-caching process for that ID association. The ICF transfers the PEI and GPSI from the previous ID association to the new ID association.

3.	A new 5G-GUTI is assigned to the UE. The IEF sends an IEF Association Record message that contains the SUCI, 5G-GUTI, SUPI, PEI, TAI, N-CGI, Time-stamp of Ty and N-CGI-Time. The ICF caches the information as the ID association with Ty as the Association Start Time. Since the new ID association has a SUCI, the PEI is not transferred from the previous ID association. However, the PEI (which may be same as the one the previous ID associations, or can be different) received in the IEF Association Record message is cached in the ICF. The ICF transfers the GPSI from the previous ID association to the new ID association.

4.	A new 5G-GUTI is assigned to the UE. The IEF sends an IEF Association Record message that contains the 5G-GUTI, SUPI, TAI, N-CGI, Time-stamp of Tw and N-CGI-Time. The ICF caches the information as the ID association with Tw as the Association Start Time. The ICF stores the Tw as the Association End Time in the previous ID association and begins the de-caching process for that ID association. The ICF transfers the PEI and GPSI from the previous ID association (step 3) to the new ID association.

### 4.3.6	ID association caching and de-caching scenarios

#### 4.3.6.1	Caching scenarios

A new 5G-GUTI may be assigned to a SUPI when the UE registers with the 5GC or sends a new service request. The multiple possible scenarios are depicted in figure 4.3.6.1-1 below.

Figure 4.3.6.1-1: Caching scenarios

Figure 4.3.6.1-1 shows 4 scenarios of ID association caching:

Scenario 1: 5G-GUTI in the REGISTRATION ACCEPT to the UE (e.g. power-on scenario):

a)	When the UE initially powers on and sends a REGISTRATION REQUEST message to the AMF, the REGISTRATION REQUEST may contain the SUCI or the AMF may request the UE for the same using Identity Request/Response procedures.

b)	The AMF then performs the authentication/security checks for the SUCI with the AUSF/UDM and gets the SUPI associated with the SUCI from the AUSF/UDM.

c)	The AMF gets the PEI from the UE using Identity Request/Response procedures.

d)	After registering with the UDM, the AMF assigns a 5G-GUTI to the UE and sends the same to the UE in the REGISTRATION ACCEPT message. Before that, the AMF may get the Access Mobility (AM) subscriber Data from the UDM which would include the GPSI and then may subscribe to AM Subscriber Data change notification to the UDM. The illustration shows these two steps.

e)	The IEF present in the AMF detects the assignment of a new 5G-GUTI and sends the IEF Association Record message to the ICF that includes the 5G-GUTI, SUCI, SUPI, PEI, GPSI, Time-stamp = Tg, N-CGI, TAI, N-CGI Time (see clause 4.3.5.3) to the ICF.

f)	The ICF caches the received information as an ID association for the SUPI.

g)	The UE acknowledges the receipt of the 5G-GUTI to the AMF using the REGISTRATION COMPLETE message. At this, the assignment of the 5G-GUTI is successful.

Scenario 2: 5G-GUTI in CONFIGURATION UPDATE COMMAND to the UE (Paging):

a)	The AMF sends the Paging to the UE and the UE responds with a Service Request. The Service Request is expected to include the 5G-S-TMSI in clear.

b)	At the conclusion of this Service Request handling, the AMF assigns a new 5G-GUTI to the UE and sends the same in the CONFIGURATION UPDATE COMMAND message.

c)	The IEF present in the AMF detects the assignment of a new 5G-GUTI and sends the IEF Association Record message to the ICF that includes the 5G-GUTI, SUPI, Time-stamp = Tp, N-CGI, TAI, N-CGI Time (see clause 4.3.5.3) to the ICF.

f)	The ICF caches the received information as ID association for the SUPI. The ICF begins the de-caching process for the previous ID association.

g)	The UE acknowledges the receipt of the 5G-GUTI to the AMF using the CONFIGURATION UPDATE COMPLETE message. At this, the assignment of the new 5G-GUTI is successful.

Scenario 3: 5G-GUTI in CONFIGURATION UPDATE COMMAND to the UE (Service Request):

a)	The AMF receives a Service Request from the UE. The Service Request is expected to include the 5G-S-TMSI in clear.

b)	At the conclusion of this Service Request handling, the AMF assigns a new 5G-GUTI to the UE and sends the same in the CONFIGURATION UPDATE COMMAND message.

c)	The IEF present in the AMF detects the assignment of a new 5G-GUTI and sends the IEF Association Record message to the ICF that includes the 5G-GUTI, SUPI, Time-stamp = Ty, N-CGI, TAI, N-CGI Time (see clause 4.3.5.3) to the ICF.

f)	The ICF caches the received information as ID association for the SUPI. The ICF begins the de-caching process for the previous ID association.

g)	The UE acknowledges the receipt of 5G-GUTI to the AMF using the CONFIGURATION UPDATE COMPLETE message. At this, the assignment of new 5G-GUTI is successful.

Scenario 4: 5G-GUTI in REGISTRATION ACCEPT to the UE (e.g. periodic registration requests):

a)	The AMF may also receive a subsequent REGISTRATION REQUEST message from the UE. The REGISTRATION REQUEST may have the 5G-GUTI in clear.

b)	At the conclusion of this REGISTRATION REQUEST handling, the AMF assigns a new 5G-GUTI to the UE and sends the same in the REGISTRATION ACCEPT message.

c)	The IEF present in the AMF detects the assignment of a new 5G-GUTI and sends the IEF Association Record message to the ICF that includes the 5G-GUTI, SUPI, Time-stamp = Tw, N-CGI, TAI, N-CGI Time (see clause 4.3.5.3) to the ICF.

f)	The ICF caches the received information as ID association for the SUPI. The ICF begins the de-caching process for the previous ID association.

g)	The UE acknowledges the receipt of 5G-GUTI to the AMF using the REGISTRATION COMPLETE message. At this, the assignment of new 5G-GUTI is successful.

Scenario 5: GPSI is changed for the SUPI:

a)	The AMF receives the AM Subscriber Data change notification from the UDM about the GPSI change.

b)		The IEF present in the AMF detects the data change and sends the IEF Association Record message to the ICF that includes the 5G-GUTI, SUPI, GPSI, Time-stamp = Tx, N-CGI, TAI, N-CGI Time (see clause 4.3.5.3) to the ICF.

c)	The ICF caches the received information as ID association for the SUPI. The ICF begins the de-caching process for the previous ID association.

#### 4.3.6.2	De-caching scenarios

As explained in clause 4.3.3, there are two ways how an ID association in the ICF can be de-cached. The first method based on the caching of a new ID association is illustrated as a part of the caching scenarios in clause 4.3.6.1. This clause illustrates the second method where the IEF sends an IEF Disassociation Record message. Multiple possible scenarios in reference to this are depicted in figure 4.3.6.2-1 below.

Figure 4.3.6.2-1: De-caching scenarios

Figure 4.3.6.2-1 shows 3 scenarios of ID association de-caching:

Scenario 1: UE deregisters from the network (e.g. graceful power down):

a)	When the UE deregisters from the network, it sends a DEREGISTRATION REQUEST message to the AMF. The DEREGISTRATION REQUEST message contains the 5G-GUTI in clear.

b)	The AMF disassociates the 5G-GUTI from the SUPI and sends a DEREGISTRATION ACCEPT message to the UE.

c)	The IEF present in the AMF detects the removal of the 5G-GUTI for the SUPI and sends the IEF Disassociation Record message to the ICF that includes the 5G-GUTI, SUPI, Time-stamp = Te, N-CGI, N-CGI Time (see clause 4.3.5.3) to the ICF.

d)	The ICF begins the de-caching process for the cached ID association.

Scenario 2: Network de-registers the UE:

a)	The AMF when it determines to deregister a UE, removes the 5G-GUTI assigned for the SUPI and sends a DEREGISTRATION REQUEST to the UE.

b)	The IEF present in the AMF detects the removal of the 5G-GUTI for the SUPI and sends the IEF Disassociation Record message to the ICF that includes the 5G-GUTI, SUPI, Time-stamp = Te, N-CGI, N-CGI Time (see clause 4.3.5.3) to the ICF.

c)	The ICF begins the de-caching process for the cached ID association.

d)	The UE acknowledges the DEREGISTRATION REQUEST with a DEREGISTRATION ACCEPT message. At this time, the deregistration of the UE is successful.

Scenario 3: Network removes the 5G-GUTI (e.g. battery removal):

a)	When the network detects that the UE is no longer reachable, it may remove the associated 5G-GUTI for the SUPI.

b)	The IEF present in the AMF detects the removal of the 5G-GUTI for the SUPI and sends the IEF Disassociation Record message to the ICF that includes the 5G-GUTI, SUPI, Time-stamp = Te, N-CGI, N-CGI Time (see clause 4.3.5.3) to the ICF.

c)	The ICF begins the de-caching process for the cached ID association.

#### 4.3.6.3	ID association and unsuccessful 5G-GUTI assignments

This is an error scenario where the 5G-GUTI assignment is unsuccessful. It can happen when the UE fails to receive a message that carries the new 5G-GUTI or the AMF fails to receive the acknowledgement from the UE. The two scenarios are illustrated below in figure 4.3.6.3-1.

Figure 4.3.6.3-1: ID association and unsuccessful 5G-GUTI assignments

Figure 4.3.6.3-1 shows 2 scenarios of unsuccessful 5G-GUTI association and the ID Association Caching.

Scenario 1: CONFIGURATION UPDATE COMMAND does not reach the UE:

a)	The AMF assigns a 5G-GUTI to the UE and sends the same to the UE in the REGISTRATION ACCEPT message.

b)	The IEF present in the AMF detects the assignment of a new 5G-GUTI and sends the IEF Association Record message to the ICF that includes the 5G-GUTI, SUCI, SUPI, PEI, GPSI, Time-stamp = Tg, N-CGI, TAI, N-CGI Time to the ICF.

c)	The ICF caches the received information as ID association for the SUPI.

d)	The UE acknowledges the receipt of the 5G-GUTI to the AMF using the REGISTRATION COMPLETE message. At this, the assignment of the 5G-GUTI is successful.

e)	The AMF sends the Paging to the UE and the UE responds with a Service Request. The Service Request is expected to include the 5G-S-TMSI in clear.

f)	At the conclusion of this Service Request handling, the AMF assigns a new 5G-GUTI to the UE and sends the same in the CONFIGURATION UPDATE COMMAND message.

g)	The IEF present in the AMF detects the assignment of a new 5G-GUTI and sends the IEF Association Record message to the ICF that includes the 5G-GUTI, SUPI, Time-stamp = Tp, N-CGI, TAI, N-CGI Time to the ICF.

h)	ICF caches the received information as ID association for the SUPI. ICF begins the de-caching process for the previous ID association.

i)	The CONFIGURATION UPDATE COMMAND does not reach the UE. The AMF, upon failing to receive the expected CONFIGURATION UPDATE COMPLETE message, keeps both the previous 5G-GUTI (first choice) and the new 5G-GUTI (second choice) as still being associated to the SUPI.

Within the ICF, the previous ID association will be deleted after the de-caching period. In this scenario, the active ID association in the ICF contains the second choice 5G-GUTI that the AMF is associating to the SUPI. However, that 5G-GUTI does not match to the 5G-GUTI (which is the first choice 5G-GUTI that AMF has) that the UE has.

Scenario 2: CONFIGURATION UPDATE COMPLETE does not reach the AMF:

a)	The AMF sends the Paging to the UE using the first choice 5G-GUTI and the UE responds with a Service Request. The Service Request is expected to include the 5G-S-TMSI in clear.

b)	At the conclusion of this Service Request handling, the AMF assigns a new 5G-GUTI to the UE and sends the same in the CONFIGURATION UPDATE COMMAND message.

c)	The IEF present in the AMF detects the assignment of a new 5G-GUTI and sends the IEF Association Record message to the ICF that includes the 5G-GUTI, SUPI, Time-stamp = Ty, N-CGI, TAI, N-CGI Time to the ICF. The ICF transfers the PEI, GPSI from the previous record to the new record.

d)	The ICF caches the received information as an ID association for the SUPI. The ICF begins the de-caching process for the previous ID association.

f)	The UE acknowledges the receipt of 5G-GUTI to the AMF using the CONFIGURATION UPDATE COMPLETE message. However, CONFIGURATION UPDATE COMPLETE message does not reach the AMF. The AMF keeps both the previous 5G-GUTI (first choice) and the new 5G-GUTI (second choice) as still being associated to the SUPI.

Within the ICF, the previous ID association will be deleted after the de-caching period. In this scenario, the active ID association in the ICF does not contain the 5G-GUTI that the AMF is associating as the first choice 5G-GUTI to the SUPI. However, the active ID association in the ICF does correspond to the 5G-GUTI that has been accepted from the UE (which is the second choice 5G-GUTI in AMF).

Continuing the scenario 2:

h)	The AMF sends the Paging to the UE using the second choice 5G-GUTI and the UE responds with a Service Request. The Service Request is expected to include the 5G-S-TMSI in clear.

i)	At the conclusion of this Service Request handling, the AMF assigns a new 5G-GUTI to the UE and sends the same in the CONFIGURATION UPDATE COMMAND message.

j)	The IEF present in the AMF detects the assignment of a new 5G-GUTI and sends the IEF Association Record message to the ICF that includes the 5G-GUTI, SUPI, Time-stamp = Tw, N-CGI, TAI, N-CGI Time to the ICF.

k)	The ICF caches the received information as the ID association for the SUPI. The ICF begins the de-caching process for the previous ID association. The ICF transfers the PEI, GPSI from the previous record to the new record.

l)	The UE acknowledges the receipt of the 5G-GUTI to the AMF using the CONFIGURATION UPDATE COMPLETE message. At this, the assignment of 5G-GUTI is successful.

## 4.4	Retrieval principles

### 4.4.1	Overview

The second part of ID Association Caching is the retrieval of cached ID associations by the LEAs for a specific UE. The retrieval process is handled using the request and response procedures between the LEAs and the CSP.

A LEA can submit six types of requests to the CSP:

1.	Request for the SUPI associated with a SUCI.

2.	Request for the SUPI associated with a 5G-S-TMSI.

3.	Request for the SUPI associated with a 5G-GUTI.

4.	Request for the 5G-GUTI associated with a SUPI.

5.	Request for ongoing updates of new 5G-GUTI assigned to a SUPI.

6.	Request to stop the above ongoing updates.

The first three are also referred to the retrieval of a permanent ID from the non-permanent ID and the next two are referred as a retrieval of non-permanent ID from a permanent ID.

As shown in figure 4.1-1, the LEA submits a request over the LI_HIQR interface to the IQF, which in turn forwards that request to the ICF over the LI_XQR interface. The response to such a request involves the ICF retrieving the cached ID association and returning the same to the LEA via the IQF.

The ICF retrieves the cached ID association known at the time it receives the request unless the LEA specifies a particular time of interest to retrieve the ID association. The particular time which may be included in some of the requests is referred to as Observed Time. In the list shown above, the LEA includes the Observed Time in the first 4 of the requests.

The ICF within the CSP uses the Observed Time after applying a short time-window to overcome any timing issues between the LEA clock and the CSP clock to retrieve the cached ID association. The Observed Time is compared against the Association Start Time stored in the cached ID association.

### 4.4.2	LI_HIQR

The LI_HIQR is an interface that carries requests and responses between the LEA and the IQF.

On the LI_HIQR interface, the following names are used to for the messages exchanged between the LEA and the IQF:

-	LI_HIQR Request (LEA to IQF).

-	LI_HIQR Response (IQF to LEA).

-	LI_HIQR Updates (IQF to LEA).

The LI_HIQR Request shall always carry the Observed Time field when the ID included in the message is the 5G-S-TMSI or the 5G-GUTI or the SUPI except for the case of ongoing updates. The TS 33.128 [4] defines the details of the LI_HIQR Request, LI_HIQR Response and LI_HIQR Updates. The same is summarized in the table below.

Table 4.4.2-1: LI_HIQR Request possibilities

| Message Name | Request Details |  |  |  |
| --- | --- | --- | --- | --- |
|  | Request Type | Observed Time | Request Values |  |
| LI_HIQR Request | Identity Association | Present | SUCI | N-CGI, TAC |
|  |  | Present | 5G-S-TMSI | N-CGI, TAC |
|  |  | Present | 5G-GUTI | N-CGI, TAC |
|  |  | Present | SUPI | N/A |
|  | Ongoing Identity Association | Not present | SUPI | N/A |

In table 4.4.2-1, the Request Type of Identity Association with Request Values of SUCI, 5G-S-TMSI, and 5G-GUTI may also be viewed as a request for a non-permanent ID to a permanent ID. The Request Type of Identity Association with Request Values of SUPI and the Ongoing Identity Association may also be viewed as a request for a permanent ID to a non-permanent ID.

Table 4.4.2-2: LI_HIQR Response/Updates possibilities

| Message Name |  | Identity Response Details |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Identity Association Record (one or more instance, NOTE 1) |  |  |  |  |  |  |  |  |
|  | SUPI | 5G-GUTI | SUCI | PEI, GPSI | Association |  | List of TA |
|  |  |  |  |  |  |  | Start Time |  | End Time |  |
| LI_HIQR Response |  | Present |  | Present | If found | If found | Present |  | When applicable | If available |
| LI_HIQR Updates |  | Present |  | Present | If found | If found | Present |  | When applicable | If available |

NOTE 1:	LI_HIQR Updates will have at most one Identity Association Record. The LI_HIQR Response may have more than one Identity Association Records.

In table 4.4.2-2, the Association Start Time included in the LI_HIQR Response and the LI_HIQR Updates reflects the time at which the 5G-GUTI is assigned to the SUPI in the AMF. The Association End Time is included in the LI_HIQR Response and the LI_HIQR Updates, when the ID association reported in the Identity Association Record is marked for deletion after the de-caching period within the ICF.

NOTE 2:	The Association Start Time for the SUCI to SUPI retrieval case is also the time at which the 5G-GUTI is assigned in the AMF.

On the LI_HIQR interface, every message is acknowledged by the receiving function. For example: the LI_HIQR Request will have an acknowledgement. Likewise, the LI_HIQR Response will have an acknowledgement. In other words, the LI_HIQR Response is not really an acknowledgement to a LI_HIQR Request at a protocol level.

### 4.4.3	LI_XQR

The LI_XQR is the interface that carries requests and responses between the IQF and the ICF.

On the LI_XQR interface, the following names are used for the messages exchanged between the IQF and the ICF:

-	Identity Association Request (IQF to ICF).

-	Identity Association Response (ICF to IQF).

-	Activate Association Update (IQF to ICF).

-	Activate Association Update Acknowledgement (ICF to IQF).

-	Identity Association Update (ICF to IQF).

-	Identity Association Update Acknowledgement (IQF to ICF).

-	Deactivate Association Update (IQF to ICF).

-	Deactivate Association Update Acknowledgement (ICF to IQF).

TS 33.128 [4] defines the details of the above listed messages.

Table 4.4.3-1 provides a mapped LI_HIQR message for each of the above:

Table 4.4.3-1: Mapping between LI_HIQR and LI_XQR

| LI_XQR |  | LI_HIQR |  |  |
| --- | --- | --- | --- | --- |
| Message Name | Direction | Message Name | Request Type | Direction |
| Identity Association Request | IQF to ICF | LI_HIQR Request | Identity Association | LEA to IQF |
| Identity Association Response | ICF to IQF | LI_HIQR Response | N/A | IQF to LEA |
| Activate Association Update | IQF to ICF | LI_HIQR Request | Ongoing Identity Association | LEA to IQF |
| Activate Association Update Acknowledgement | ICF to IQF | LI_HIQR Response | N/A | IQF to LEA |
| Identity Association Update | ICF to IQF | LI_HIQR Updates | N/A | IQF to LEA |
| Identity Association Update Acknowledgement | IQF to ICF | N/A | N/A | N/A |
| Deactivate Association Update | IQF to ICF | LI_HIQR Request | Ongoing Identity Association | LEA to IQF |
| Deactivate Association Update Acknowledgement | ICF to IQF | N/A | N/A | N/A |

As shown in table 4.4.3-1, the Identity Association Update Acknowledgement is an acknowledgement from the IQF to the ICF for the receipt of Identity Association Update Message. This is not seen on the LI_HIQR interface. Also, the Deactivate Association Update Acknowledgement message, which is a response from ICF to IQF for indicating successfully stopping on the further reporting of new 5G-GUTI assignments, is not relayed to the LEA over LI_HIQR.

Table 4.4.3-2 provides the details of LI_XQR Identity Association Request possibilities.

Table 4.4.3-2: LI_XQR Identity Association Request possibilities

| Message Name | Request Details |  |  |  |
| --- | --- | --- | --- | --- |
|  | Request Type | Observed Time | Request Values |  |
| Identity Association Request | Identity Association | Present | SUCI | N-CGI, TAC |
|  |  | Present | 5G-S-TMSI | N-CGI, TAC |
|  |  | Present | 5G-GUTI | N-CGI, TAC |
|  |  | Present | SUPI | N/A |

Table 4.4.3-3 shows the LI_XQR Ongoing Identity Association possibilities.

Table 4.4.3-3: LI_XQR Ongoing Identity Association possibilities

| Message Name | SUPI | LD Task Object (LI_HIQR) |
| --- | --- | --- |
| Activate Association Update | Present | Awaiting Disclosure |
| Deactivate Association Update | Present | Disclosed |

The LD Task Object shown in the table 4.4.3-3 is not part of the LI_XQR messages. The IQF uses that information received in the LI_HIQR Request to generate the two messages over LI_XQR when the Request Type in the LI_HIQR Request is Ongoing Identity Association.

Table 4.4.3-4 provides the details of LI_XQR Response possibilities.

Table 4.4.3-4: LI_XQR Response possibilities

| Message Name | Identity Response Details |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Identity Association Record (one or more instance) |  |  |  |  |  |  |
|  | SUPI | 5G-GUTI | SUCI | PE, /GPSI | Association |  | List of TA |
|  |  |  |  |  | Start Time | End Time |  |
| Identity Association Response | Present | Present | If found | If found | Present | When applicable | If available |
| Activate Association Update Acknowledgement | Present | Present | If found | If found | Present | N/A | If available |
| Identity Association Update | Present | Present | If found | If found | Present | When applicable | If available |

The Identity Association Update Acknowledgement and the Deactivate Association Update Acknowledgement message are not seen on the LI_HIQR interface and therefore, the details of the same are not shown in table 4.4.3-4.

The Request Type is not included in the Activate Association Update or Deactivate Association Update messages. The LD Task Object Desired Status will have "Awaiting Disclosure" in all requests except for the Deactivate Association Update, in which case, it will be "Disclosed".

***** Next Change ****

## 4.5	Flow diagrams

### 4.5.1	General

The purpose of this clause is to show end-to-end flow diagrams depicting the scenarios of ID Association Caching with the scope of showing various caching scenarios combined with the retrieval scenarios.

For the simplicity of the drawings, all the flow-diagrams assume that the LEA clock and the CSP clock are aligned. The ICF may still have the short time window, described and illustrated in clause 4.4.

### 4.5.2	SUCI to SUPI retrieval

#### 4.5.2.1	General

As illustrated in the caching principles, the ID association is cached in the ICF whenever a 5G-GUTI is assigned to the SUPI.

The association of a SUPI to a SUCI may happen in the AMF before a 5G-GUTI is assigned. Therefore, it is possible that the Observed Time provided by the LEA Request can be before the Association Start Time of the first cached ID association. In that case the response will not have any ID Association Records.

The LEA may submit a request with a later time, for example, the time of the 5G-GUTI assignment observation. In that case, the Identity Association Response may carry the Identity Association Records.

#### 4.5.2.2	Illustration 1

Figure 4.5.2.2-1 illustrates the two scenarios of the SUCI to SUPI retrieval handling case with the Observed Time happenings before the Association Start Time of the first cached ID association.

Figure 4.5.2.2-1: SUCI to SUPI retrieval – illustration 1

Figure 4.5.2.2-1 illustrates two scenarios:

Scenario 1:

-	The Identity Association Request is received at the ICF before the ICF had the ID association caching for the SUCI. Accordingly, the Identity Association Response will have no Identity Association Records.

Scenario 2:

-	The Identity Association Request is received at the ICF while the ICF has the ID association caching for the SUCI. However, the Observed Time is before the Association Start Time Tg in the cached ID association. Accordingly, the Identity Association Response will have no Identity Association Records.

#### 4.5.2.3	Illustration 2

Figure 4.5.2.3-1 illustrates the two scenarios of SUCI to SUPI retrieval handling case with the Observed Time happenings after the Association Start Time of the first cached ID association.

Figure 4.5.2.3-1: SUCI to SUPI retrieval – illustration 2

Figure 4.5.2.3-1 illustrates two scenarios:

Scenario 1:

-	The Identity Association Request is received at the ICF while the ICF has the ID association caching for the SUCI. The Observed Time T1 in the Identity Association Request is after the Association Start Time Tg in the cached ID association. Accordingly, the Identity Association Response will have the Identity Association Record that has the Association Start Time of Tg.

Scenario 2:

-	The Identity Association Request is received at the ICF after the ID association that had the SUCI was deleted from the ICF. Therefore, the Identity Association Response will have no Identity Association Records.

### 4.5.3	5G-S-TMSI to SUPI retrieval

#### 4.5.3.1	General

As illustrated in the caching principles, the ID association is cached in the ICF whenever a 5G-GUTI is assigned to the SUPI.

The LEAs may observe the 5G-S-TMSI just before the next 5G-GUTI is assigned to that SUPI. To counter the time-delay in LEA submitting the request, a de-caching period is used within the ICF before deleting the previous ID association whenever a new ID association is cached.

If the LEA request with a 5G-S-TMSI is received at the ICF after the corresponding cached ID association is deleted, the response will have no Identity Association Records.

#### 4.5.3.2	Illustration 1

Figure 4.5.3.2-1 illustrates a flow-diagram that shows the 5G-S-TMSI to SUPI retrieval handling. The flow has three scenarios based on the time when the Request is sent and the Observed Time.

Figure 4.5.3.2-1: 5G-S-TMSI to SUPI retrieval – illustration 1

Scenario 1:

-	The Identity Association Request is received at the ICF with an Observed Time of T2. When the Identity Association Request is received, the 5G-GUTI with that 5G-S-TMSI is still active as per the ID association cached in the ICF.

-	The Identity Association Response will have the Identity Association Record that has the Association Start Time of Tg.

Scenario 2:

-	The Identity Association Request is received at the ICF with an Observed Time of T2.

-	When the Identity Association Request is received, the ID association that has the 5G-S-TMSI is in the de-caching phase and a new 5G-GUTI is assigned to the SUPI according to the cached ID association.

-	The Identity Association Response will have the Identity Association Record that has the Association Start Time of Tg and Association End Time of Tp.

Scenario 3:

-	The Identity Association Request is received at the ICF with an Observed Time of T3.

-	When the Identity Association Request is received, the ID association that has the 5G-S-TMSI is in the de-caching phase and a new 5G-GUTI is assigned to the SUPI according to the cached ID association.

-	The Identity Association Response will have the Identity Association Record that has the Association Start Time of Tp and Association End Time of Ty.

#### 4.5.3.3	Illustration 2

Figure 4.5.3.3-1 illustrates a flow-diagram that shows the 5G-S-TMSI to SUPI retrieval handling when the two 5G-GUTIs are presumed to be associated to the SUPI (only one valid).

The flow has three scenarios based on the time the Observed Time.

Figure 4.5.3.3-1: 5G-S-TMSI to SUPI retrieval – illustration 2

Scenario A:

-	The assignment of a 5G-GUTI that has the Association Start Time of Tp (in ICF) was not successful. The CONFIGURATION UPDATE COMMAND did not reach the UE.

Scenario B:

-	The assignment of a 5G-GUTI that has the Association Start Time of Ty (in ICF) was not successful. The CONFIGURATION UPDATE COMPLETE did not reach the AMF.

Scenario 1:

-	The Identity Association Request is received at the ICF with an Observed Time of T1. When the Identity Association Request is received, the ID association that has the 5G-S-TMSI is already deleted from the ICF.

-	Therefore, the Identity Association Response will have no Identity Association Records.

Scenario 2:

-	The Identity Association Request is received at the ICF with an Observed Time of T2. When the Identity Association Request is received, the ID association that has the 5G-S-TMSI is already deleted from the ICF.

-	Therefore, the Identity Association Response will have no Identity Association Records.

Scenario 3:

-	The Identity Association Request is received at the ICF with an Observed Time of T3. When the Identity Association Request is received, the ID association that has the 5G-S-TMSI is in the de-caching phase and a new 5G-GUTI is assigned to the SUPI according to the cached ID association.

-	The Identity Association Response will have the Identity Association Record that has the Association Start Time of Ty and Association End Time of Tb.

### 4.5.4	SUPI to 5G-GUTI retrieval

#### 4.5.4.1	General

This clause illustrates the retrieval scenarios where the LI_HIQR Request has the SUPI with a Request Type of Identity Association Request.

The LEA in the LI_HIQR Request includes an Observed Time which is used within the ICF to find the active 5G-GUTI at that time.

#### 4.5.4.2	Illustration 1

Figure 4.5.4.2-1 illustrates two scenarios based on the time at which the ICF receives the Identity Association Request. No active 5G-GUTI to SUPI association is found in the ICF at the Observed Time.

Figure 4.5.4.2-1: SUPI to 5G-GUTI retrieval – illustration 1

Scenario 1:

The Identity Association Request (with the SUPI in the Request Details) is received at the ICF with an Observed Time of T1.

The ICF has an ID association for that SUPI with an active 5G-GUTI, however, the Association Start Time Tg of that ID association is after the Observed Time of T1.

Accordingly, the Identity Association Response will have no Identity Association Records.

Scenario 2:

The Identity Association Request (with the SUPI in the Request Details) is received at the ICF with an Observed Time of T2.

The ICF has an ID association for that SUPI with an active 5G-GUTI, however, the Association Start Time Tp of that ID association is after the Observed Time of T2.

The ID association with a 5G-GUTI which was active at the Observed Time T2 was already deleted when the Identity Association Request is received at the ICF.

Accordingly, the Identity Association Response will have no Identity Association Records.

#### 4.5.4.3	Illustration 2

Figure 4.5.4.3-1 illustrates three scenarios based on the Observed Time and the time at which the ICF receives the Identity Association Request.

Figure 4.5.4.3-1: SUPI to 5G-GUTI retrieval – illustration 2

Scenario 1:

-	The Identity Association Request (with the SUPI in the Request Details) is received at the ICF with an Observed Time of T1.

-	The ICF has an ID association for that SUPI with an active 5G-GUTI, the Association Start Time Tg of that ID association is before the Observed Time of T1.

-	Accordingly, the Identity Association Response will have the Identity Association Record with the 5G-GUTI (including SUCI, PEI, SUPI) with an Association Start Time of Tg.

Scenario 2:

-	The Identity Association Request (with the SUPI in the Request Details) is received at the ICF with an Observed Time of T1.

-	The ICF has an ID association for that SUPI with an active 5G-GUTI, however, the Association Start Time Tp of that ID association is after the Observed Time of T1.

-	The ID association with a 5G-GUTI which was active at the Observed Time T1 is in de-caching phase within the ICF when the ICF receives the Identity Association Request.

-	Accordingly, the Identity Association Response will have the Identity Association Record with the 5G-GUTI (including SUPI, SUCI, PEI) with an Association Start Time of Tg and Association End Time of Tp.

Scenario 3:

-	The Identity Association Request (with the SUPI in the Request Details) is received at the ICF with an Observed Time of T2.

-	The ICF has an ID association for that SUPI with an active 5G-GUTI, however, the Association Start Time Ty of that ID association is after the Observed Time of T2.

-	The ID association with a 5G-GUTI which was active at the Observed Time T2 is in the de-caching phase within the ICF when the ICF receives the Identity Association Request.

-	Accordingly, the Identity Association Response will have the Identity Association Record with the 5G-GUTI (including SUPI and PEI) with an Association Start Time of Tp and Association End Time of Ty.

#### 4.5.4.4	Illustration 3

Figure 4.5.4.4-1 illustrates a scenario that shows the impact of a Time-Window used within the ICF for a SUPI to 5G-GUTI retrieval case.

Figure 4.5.4.4-1: SUPI to 5G-GUTI retrieval – impact of Time-Window

The Identity Association Request (with SUPI in the Request Details) is received at the ICF with an Observed Time of T1.

The ICF has implemented a Time-Window concept to adjust the time-lag due to the differences between the LEA clock and the CSP clock.

The ICF has an ID association for that SUPI with an active 5G-GUTI, the Association Start Time Ty is before the Observed Time T1, if the Time-Window is applied. Without the Time-Window, the Association Start Time Ty would have been concluded to be after the Observed Time T2.

There was another ID association with a 5G-GUTI that was active at the Observed Time T2 but is in a de-caching phase within the ICF when the ICF receives the Identity Association Request.

Since two ID associations are considered to be active within the Time-Window of Observed Time T1, the ICF includes two Identity Association Records:

1)	5G-GUTI (also including the SUPI and the PEI) along with the Association Start Time of Tp and Association End Time of Ty.

2)	5G-GUTI (also including the SUPI and the PEI) along with the Association Start Time of Ty.

#### 4.5.4.5	Illustration 4

Figure 4.5.4.5-1 illustrates a flow-diagram that shows the SUPI to 5G-GUTI retrieval handling when the two 5G-GUTIs are presumed to be associated to the SUPI (only one valid).

The flow has three scenarios based on the time the Observed Time.

Figure 4.5.4.5-1: SUPI to 5G-GUTI retrieval - example 4

In this illustration, the record in ICF does has neither PEI nor GPSI.

Scenario A:

-	The assignment of a 5G-GUTI that has the Association Start Time of Tp (in ICF) was not successful. The CONFIGURATION UPDATE COMMAND did not reach the UE.

Scenario B:

-	The assignment of a 5G-GUTI that has the Association Start Time of Ty (in ICF) was not successful. The CONFIGURATION UPDATE COMPLETE did not reach the AMF.

Scenario 1:

-	The Identity Association Request is received at the ICF with an Observed Time of T1.

-	The ICF has an ID association for that SUPI with an active 5G-GUTI and the Association Start Time Tg of that ID association is before the Observed Time of T1.

-	Accordingly, the Identity Association Response will have the Identity Association Record with the 5G-GUTI (including SUPI) with an Association Start Time of Tg.

Scenario 2:

-	The Identity Association Request is received at the ICF with an Observed Time of T2.

-	The ICF has an ID association for that SUPI with an active 5G-GUTI and the Association Start Time Tp of that ID association is before the Observed Time of T2.

-	Accordingly, the Identity Association Response will have the Identity Association Record with the 5G-GUTI (including SUPI) with an Association Start Time of Tp.

NOTE:	Since the associated CONFIGURATION UPDATE COMMAND (Scenario A) never reached the UE, the 5G-GUTI included in the Identity Association Record is not the real one. But, from an ICF perspective, it is the real one.

Scenario 3

-	The Identity Association Request is received at the ICF with an Observed Time of T3.

-	The ICF has an ID association for that SUPI with an active 5G-GUTI and the Association Start Time Ty of that ID association is before the Observed Time of T3.

-	Accordingly, the Identity Association Response will have the Identity Association Record with the 5G-GUTI (including SUPI) with an Association Start Time of Ty.

NOTE:	Since the associated CONFIGURATION UPDATE COMPLETE (Scenario B) never reached the AMF, the AMF would first try to page the UE with a different 5G-GUTI. However, when that fails, the AMF would page the UE with the 5G-GUTI included in the Identity Association Record.

### 4.5.5	Ongoing identity association scenarios

#### 4.5.5.1	General

This clause illustrates the end to end flow diagrams for ongoing identity association scenarios.

The LEA may send an LI_HIQR Request to activate the ongoing identity association or deactivate a previously active ongoing identity association.

#### 4.5.5.2	Illustration 1

Figure 4.5.5.2-1 shows one example of ongoing identity association handling.

Figure 4.5.5.2-1: Ongoing identity association handling – illustration 1

In this illustration, the LEA sends an LI_HIQR Request for an ongoing identity association for a SUPI. The IQF sends an Activate Association Update message to the ICF.

The ICF finds that an active ID association is cached and includes that in the Identity Association Record of the Activate Associate Update Acknowledgement message to the IQF. The Identity Association Record will have the currently active 5G-GUTI (including the SUPI, SUCI and PEI) with an Association Start Time of Tg. The IQF forwards the received information to the LEA in the LI_HIQR Response message.

Then, a new 5G-GUTI is assigned to the UE at the AMF which is cached in the ICF with an Association Start Time of Tp. The ICF sends the Identity Association Update message to the IQF. The Identity Association Record will have the new 5G-GUTI (including the SUPI, PEI, GPSI) with an Association Start Time of Tp. The IQF forwards the received information to the LEA in the LI_HIQR Updates message. The IQF would acknowledge the receipt of the Identity Association Update with an Identity Association Update Acknowledgement message.

Then, a new 5G-GUTI is assigned to the UE at the AMF which is cached in the ICF with an Association Start Time of Ty. The ICF sends the Identity Association Update message to the IQF. The Identity Association Record will have the new 5G-GUTI (including the SUPI, PEI, GPSI) with an Association Start Time of Ty. The IQF forwards the received information to the LEA in the LI_HIQR Updates message. The IQF would acknowledge the receipt of Identity Association Update with an Identity Association Update Acknowledgement message.

#### 4.5.5.3	Illustration 2

Figure 4.5.5.3-1 shows another example of ongoing identity association handling. In this example, the ICF has two ID associations, and the illustration shows that the ICF would respond with the currently active 5G-GUTI.

Figure 4.5.5.3-1: Ongoing identity association handling – illustration 2

In this illustration, the LEA sends an LI_HIQR Request for an ongoing identity association for a SUPI. The IQF sends an Activate Association Update message to the ICF.

The ICF finds that an ID association with the Association Start Time of Tg is marked for deletion after the de-caching period. The ICF also finds an active ID association cached and includes that in the Identity Association Record in the Activate Associate Update Acknowledgement message to the IQF. The Identity Association Record in that way will have the currently active the 5G-GUTI (also includes the SUPI,  PEI, GPSI with an Association Start Time of Tp. The IQF forwards the received information to the LEA in the LI_HIQR Response message.

Then, a new 5G-GUTI is assigned to the UE at the AMF which is cached in the ICF with an Association Start Time of Ty. The ICF sends the Identity Association Update message to the IQF. The Identity Association Record will have the new 5G-GUTI (including the SUPI, PEI, GPSI) with an Association Start Time of Ty. The IQF forwards the received information to the LEA in the LI_HIQR Updates message. The IQF would acknowledge the receipt of the Identity Association Update with an Identity Association Update Acknowledgement message.

#### 4.5.5.4	Illustration 3

Figure 4.5.5.4-1 shows another example of ongoing identity association handling. In this example, the UE deregisters from the network while an ongoing identity association is active in the ICF. Subsequently, the UE registers back to the network.

Figure 4.5.5.4-1: Ongoing identity association handling – illustration 3

In this example, the LEA sends an LI_HIQR Request for an ongoing identity association for a SUPI. The IQF sends an Activate Association Update message to the ICF.

The ICF finds that an active ID association is cached and includes that in the Identity Association Record in the Activate Associate Update Acknowledgement message to the IQF. The Identity Association Record will have the currently active 5G-GUTI (including the SUPI, SUCI and PEI) with an Association Start Time of Tg. The IQF forwards the received information to the LEA in the LI_HIQR Response message.

Then, a new 5G-GUTI is assigned to the UE at the AMF which is cached in the ICF with an Association Start Time of Tp. The ICF sends the Identity Association Update message to the IQF. The Identity Association Record will have the new 5G-GUTI (including the SUPI, PEI, GPSI) with an Association Start Time of Tp. The IQF forwards the received information to the LEA in the LI_HIQR Updates message. The IQF would acknowledge the receipt of the Identity Association Update with an Identity Association Update Acknowledgement message.

Then, the UE deregisters from the network. The 5G-GUTI is removed for the SUPI in the AMF. The IEF sends an IEF Disassociation Record message to the ICF. The ICF begins the de-caching process for the corresponding ID association. The ICF also sends the Identity Association Update Message to the IQF with an Identity Association Record containing the 5G-GUTI (including the SUPI and the PEI, GPSI) along with the Association End Time of Te. The IQF forwards the received information to the LEA in the LI_HIQR Updates message. The IQF would acknowledge the receipt of Identity Association Update with an Identity Association Update Acknowledgement message.

Later the UE registers to the network again. After the new 5G-GUTI is assigned, the IEF sends the IEF Association Record with the new 5G-GUTI, SUCI, SUPI, PEI to the ICF. The ICF caches the ID association with an Association Start Time of Ty. The ICF also sends the Identity Association Update Message to the IQF with an Identity Association Record containing the 5G-GUTI (including the SUPI, SUCI and the PEI) along with the Association Start Time of Ty. The IQF forwards the received information to the LEA in the LI_HIQR Updates message. The IQF would acknowledge the receipt of Identity Association Update with an Identity Association Update Acknowledgement message.

#### 4.5.5.5	Illustration 4

Figure 4.5.5.5-1 shows another example of ongoing identity association handling. In this example, the LEA deactivates the ongoing identity association.

Figure 4.5.5.5-1: Ongoing identity association handling – illustration 4

In this example, the LEA sends an LI_HIQR Request for an ongoing identity association for a SUPI. The IQF sends an Activate Association Update message to the ICF.

The ICF finds that an active ID association is cached and includes that in the Identity Association Record in the Activate Associate Update Acknowledgement message to the IQF. The Identity Association Record will have the currently active 5G-GUTI (including the SUPI, SUCI and PEI) with an Association Start Time of Tg. The IQF forwards the received information to the LEA in the LI_HIQR Response message.

Then, a new 5G-GUTI is assigned to the UE at the AMF which is cached in the ICF with an Association Start Time of Tp. The ICF sends the Identity Association Update message to the IQF. The Identity Association Record will have the new 5G-GUTI (including the SUPI, PEI) with an Association Start Time of Tp. The IQF forwards the received information to the LEA in the LI_HIQR Updates message. The IQF would acknowledge the receipt of the Identity Association Update with an Identity Association Update Acknowledgement message.

Then, the LEA sends the LI_HIQR Request to deactivate the ongoing identity association for the SUPI. The IQF sends the Deactivate Association Update message to the ICF. The ICF acknowledges the receipt of Deactivate Association Update message with a Deactivate Association Update Acknowledgement message to the ICF. The ICF does not forward any message to the LEA.

Later, a new 5G-GUTI is assigned to the UE at the AMF which is cached in the ICF with an Association Start Time of Ty. The ICF does not send the Identity Association Update message to the IQF because the ongoing identity association was previously deactivated by the LEA.

#### 4.5.5.6	Illustration 5

Figure 4.5.5.6-1 shows another example of ongoing identity association handling. In this example, the LEA deactivates the ongoing identity association while the UE was not registered to the network.

Figure 4.5.5.6-1: Ongoing identity association handling – illustration 5

In this example, the LEA sends an LI_HIQR Request for an ongoing identity association for a SUPI. The IQF sends an Activate Association Update message to the ICF.

The ICF finds that an active ID association is cached and includes that in the Identity Association Record in the Activate Associate Update Acknowledgement message to the IQF. The Identity Association Record will have the currently active 5G-GUTI (including the SUPI, SUCI and PEI) with an Association Start Time of Tg. The IQF forwards the received information to the LEA in the LI_HIQR Response message.

Then, a new 5G-GUTI is assigned to the UE at the AMF which is cached in the ICF with an Association Start Time of Tp. The ICF sends the Identity Association Update message to the IQF. The Identity Association Record will have the new 5G-GUTI (including the SUPI, PEI) with an Association Start Time of Tp. The IQF forwards the received information to the LEA in the LI_HIQR Updates message. The IQF would acknowledge the receipt of the Identity Association Update with an Identity Association Update Acknowledgement message.

Then, the UE deregisters from the network. The 5G-GUTI is removed for the SUPI in the AMF. The IEF sends an IEF Disassociation Record message to the ICF. The ICF begins the de-caching process for the corresponding ID association. The ICF also sends the Identity Association Update Message to the IQF with an Identity Association Record containing the 5G-GUTI (including the SUPI and the PEI) along with the Association End Time of Te. The IQF forwards the received information to the LEA in the LI_HIQR Updates message. The IQF would acknowledge the receipt of the Identity Association Update with an Identity Association Update Acknowledgement message.

Then, LEA sends the LI_HIQR Request to deactivate the ongoing identity association for the SUPI. The IQF sends the Deactivate Association Update message to the ICF. The ICF acknowledges the receipt of Deactivate Association Update message with a Deactivate Association Update Acknowledgement message to the ICF. The ICF does not forward any message to the LEA.

#### 4.5.5.7	Illustration 6

Figure 4.5.5.7-1 shows another example of ongoing identity association handling. In this example, a second LEA activates the ongoing identity association and the first the LEA deactivates the ongoing identity association.

Figure 4.5.5.7-1: Ongoing identity association handling – illustration 6

In this example, the LEA-1 sends an LI_HIQR Request for an ongoing identity association for a SUPI. The IQF sends an Activate Association Update message to the ICF. In the illustration, the ICF records have neither the PEI nor the GPSI.

The ICF finds that an active ID association is cached and includes that in the Identity Association Record in the Activate Associate Update Acknowledgement message to the IQF. The Identity Association Record will have the currently active 5G-GUTI (including the SUPI, SUCI and PEI) with an Association Start Time of Tg. The IQF forwards the received information to the LEA in the LI_HIQR Response message.

Then, a new 5G-GUTI is assigned to the UE at the AMF which is cached in the ICF with an Association Start Time of Tp. The ICF sends the Identity Association Update message to the IQF. The Identity Association Record will have the new the 5G-GUTI (including the SUPI, PEI) with an Association Start Time of Tp. The IQF forwards the received information to the LEA in the LI_HIQR Updates message. The IQF would acknowledge the receipt of the Identity Association Update with an Identity Association Update Acknowledgement message.

The LEA-2 sends an LI_HIQR Request for an ongoing identity association for a SUPI. The IQF sends an Activate Association Update message to the ICF.

The ICF finds that an active ID association is cached and includes that in the Identity Association Record in the Activate Associate Update Acknowledgement message to the IQF. The Identity Association Record will have the currently active the 5G-GUTI (including the SUPI, SUCI and PEI) with an Association Start Time of Tp. The IQF forwards the received information to the LEA in the LI_HIQR Response message.

Then, a new 5G-GUTI is assigned to the UE at the AMF which is cached in the ICF with an Association Start Time of Ty. The ICF sends the Identity Association Update message to the IQF. The Identity Association Record will have the new 5G-GUTI (including the SUPI, PEI) with an Association Start Time of Ty. The IQF forwards the received information to the LEA-1 in the LI_HIQR Updates message. The IQF would acknowledge the receipt of the Identity Association Update with an Identity Association Update Acknowledgement message.

The ICF sends another Identity Association Update message to the IQF. The Identity Association Record will have the new 5G-GUTI (including the SUPI, PEI) with an Association Start Time of Ty. The IQF forwards the received information to the LEA-2 in the LI_HIQR Updates message. The IQF would acknowledge the receipt of the Identity Association Update with an Identity Association Update Acknowledgement message.

Then, LEA-1 sends an LI_HIQR Request to deactivate the ongoing identity association for the SUPI. The IQF sends the Deactivate Association Update message to the ICF. The ICF acknowledges the receipt of Deactivate Association Update message with a Deactivate Association Update Acknowledgement message to the ICF. The ICF does not forward any messages to the LEA.

Later, a new 5G-GUTI is assigned to the UE at the AMF which is cached in the ICF with an Association Start Time of Tb. The ICF sends another Identity Association Update message to the IQF. The Identity Association Record will have the new 5G-GUTI (including the SUPI, PEI) with an Association Start Time of Tb. The IQF forwards the received information to the LEA-2 in the LI_HIQR Updates message. The IQF would acknowledge the receipt of the Identity Association Update with an Identity Association Update Acknowledgement message.

***** End of the Changes ****