SA3#96-LI	 S3i250011

Sophia Antipolis; January 27-31, 2024

Source:	Nokia, Rogers Wireless

Title:	pCR to TR 33.929-4: duplicate text and flows

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR updates the TR 33.929-5 to remove the duplicate text and diagrams

BACKGROUND

Few of the diagrams and text are improperly placed in clause clauses and they are duplicates.

PROPOSAL

Incorporate the following changes to TR 33.929-5

**** First Change ****

###### 5.2.2.4.3.4	Network (HPLMN) initiated PDU session release

The flow-diagram in figure 5.2.2.4.3.4-1 illustrates a high-level view of xIRI generation from the IRI-POI present in the H-SMF for a VPLMN-initiated PDU session release. In this illustration, the LI does not apply to the VPLMN. In this case, the LI is on an outbound roaming UE.

Figure 5.2.2.4.3.4-1: High-level flow diagram for PDU session release (network-initiated - HPLMN)

As defined in TS 33.128 [4], the trigger conditions for the generation of xIRI in the H-SMF is on receiving the Hsmf_ PDUSession_Update Response message that includes the NAS message PDU SESSION RELEASE COMPLETE in the n1smInfoFromUe field and even though the 5GSM state in the H-SMF is also changed to PDU Session Active, the trigger is not dependent on that.

While drawing this flow, the TS 23.502 [7], figure 4.3.4.3-1 is referred to. The session management related messages and parameter (i.e. between the V-SMF and H-SMF) are defined in TS 29.502 [10].

The flow-diagram in figure 5.2.3.3.1.3-1 illustrates a detailed view of PDU session modification for access type change for a non-roaming scenario. The flow illustrates the xIRI generation from the IRI-POI present in the SMF, LI_T3 triggering from the CC-TF present in the SMF and the xCC generation from the CC-POI present in the UPF.

**** End of First Change ****

**** Retained (duplicates) ****

###### 5.2.3.3.1.3	PDU session modification due to change in access type

The flow-diagram in figure 5.2.3.3.1.3-1 illustrates a detailed view of PDU session modification for access type change for a non-roaming scenario. The flow illustrates the xIRI generation from the IRI-POI present in the SMF, LI_T3 triggering from the CC-TF present in the SMF and the xCC generation from the CC-POI present in the UPF.

In this illustration, the case of access type change from non-3GPP access to 3GPP access is considered.

Figure 5.2.3.3.1.3-1: Detailed flow diagram for PDU session modification (non-roaming) – access type change

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is SMF sending the NAS message PDU SESSION ESTABLISHMENT ACCEPT to the target UE and the 5GSM state changing into PDU Session Active.

-	The LI_T3 Modify Task trigger is sent from CC-TF present in the SMF to the CC-POI present in the UPF when the SMF sends the N4: PFCP Session Modification Request to the UPF.

-	The UPF generates the xCC based on the LI_T3 trigger received from the CC-TF. The xCC is generated from the user-plane packets that traverse through the UPF on the PDU session.

While drawing this flow, the TS 23.502 [7], figure 4.9.2.1-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

##### 5.2.3.3.2	Home-routed roaming scenario (VPLMN)

###### 5.2.3.3.2.1	Network-initiated PDU session modification

The flow-diagram in figure 5.2.3.3.2.1-1 illustrates a detailed view of PDU session modification steps for a home-routed roaming scenario. The flow illustrates the xIRI generation from the IRI-POI present in the V-SMF, LI_T3 triggering from the CC-TF present in the V-SMF and the xCC generation from the CC-POI present in the V-UPF.

In this illustration, the case of V-SMF-initiated session modification is considered.

In this illustration, the LI does not apply to the HPLMN. In this case, the LI is on an inbound roaming UE.

Figure 5.2.3.3.2.1-1: Detailed flow diagram for PDU session modification (VPLMN) – network initiated

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is V-SMF receiving the NAS message PDU SESSION MODIFICATION COMPLETE from the target UE and the 5GSM state changing into PDU Session Active.

-	The LI_T3 Modify Task trigger may be sent from CC-TF present in the V-SMF to the CC-POI present in the V-UPF when the V-SMF sends the N4: PFCP Session Modification Request to the V-UPF. If the PDU session modification does not alter the CC-interception being done at the CC-POI in the UPF, the LI_T3 Modify Task may not be sent.

-	The V-UPF generates the xCC based on the LI_T3 trigger received from the CC-TF. The xCC is generated from the user-plane packets that traverse through the V-UPF on the PDU session.

While drawing this flow, the TS 23.502 [7], figure 4.3.3.3-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

###### 5.2.3.3.2.2	PDU session modification due to change in access type

The flow-diagram in figure 5.2.3.3.2.2-1 illustrates a detailed view of PDU session modification for access type change in a home-routed roaming scenario. The flow illustrates the xIRI generation from the IRI-POI present in the V-SMF, LI_T3 triggering from the CC-TF present in the V-SMF and the xCC generation from the CC-POI present in the V-UPF.

In this illustration, the case of access type change from non-3GPP access to 3GPP access is considered.

In this illustration, the LI does not apply to the HPLMN. In this case, the LI is on an inbound roaming UE.

Figure 5.2.3.3.2.2-1: Detailed flow diagram for PDU session modification (VPLMN) – access type change

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is V-SMF sending the NAS message PDU SESSION ESTABLISHMENT ACCEPT to the target UE and the 5GSM state changing into PDU Session Active.

-	The LI_T3 Modify Task trigger is sent from CC-TF present in the V-SMF to the CC-POI present in the V-UPF when the V-SMF sends the N4: PFCP Session Modification Request to the V-UPF.

-	The V-UPF generates the xCC based on the LI_T3 trigger received from the CC-TF. The xCC is generated from the user-plane packets that traverse through the V-UPF on the PDU session.

While drawing this flow, the TS 23.502 [7], figure 4.9.2.3.1-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

##### 5.2.3.3.3	Home-routed roaming scenario (HPLMN)

###### 5.2.3.3.3.1	Network-initiated PDU session modification

The flow-diagram in figure 5.2.3.3.3.1-1 illustrates a detailed view of PDU session modification steps for a home-routed roaming scenario. The flow illustrates the xIRI generation from the IRI-POI present in the H-SMF, LI_T3 triggering from the CC-TF present in the H-SMF and the xCC generation from the CC-POI present in the H-UPF.

In this illustration, the case of H-SMF-initiated session modification is considered.

In this illustration, the LI does not apply to the VPLMN. In this case, the LI is on an outbound roaming UE.

Figure 5.2.3.3.3.1-1: Detailed flow diagram for PDU session modification (HPLMN) – network initiated

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is H-SMF receiving the Nsmf_PDU_Session_Update Response from the V-SMF with N1SmInfoFromUe containing the message PDU SESSION MODIFICATION COMPLETE.

-	The LI_T3 Modify Task trigger is sent from CC-TF present in the H-SMF to the CC-POI present in the H-UPF when the H-SMF sends the N4: PFCP Session Modification Request to the H-UPF. For network-initiated modification, it is quite possible that the CC-interception is impacted (e.g. new QoS flow) and therefore, this is a good example where LI_T3: Modify Task is sent.

-	The H-UPF generates the xCC based on the LI_T3 trigger received from the CC-TF. The xCC is generated from the user-plane packets that traverse through the H-UPF on the PDU session.

While drawing this flow, the TS 23.502 [7], figure 4.3.3.3-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

###### 5.2.3.3.3.2	PDU session modification due to change in access type (N3IWF in the same VPLMN)

The flow-diagram in figure 5.2.3.3.3.2-1 illustrates a detailed view of PDU session modification for access type change in a home-routed roaming scenario. The flow illustrates the xIRI generation from the IRI-POI present in the H-SMF, LI_T3 triggering from the CC-TF present in the H-SMF and the xCC generation from the CC-POI present in the H-UPF.

In this illustration, the case of access type change from non-3GPP access to 3GPP access is considered. And the N3IWF is in the same VPLMN. In this illustration, the LI does not apply to the VPLMN. In this case, the LI is on an outbound roaming UE.

Figure 5.2.3.3.3.2-1: Detailed flow diagram for PDU session modification (HPLMN) – access type change (N3IWF in the same VPLMN)

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is H-SMF sending the Nsmf_PDU_Session_Update Response to the V-SMF with N1SmInfoToUe containing the message PDU SESSION ESTABLISHMENT ACCEPT.

-	The LI_T3 Modify Task trigger is sent from CC-TF present in the H-SMF to the CC-POI present in the H-UPF when the H-SMF sends the N4: PFCP Session Modification Request to the H-UPF.

-	The H-UPF generates the xCC based on the LI_T3 trigger received from the CC-TF. The xCC is generated from the user-plane packets that traverse through the H-UPF on the PDU session.

While drawing this flow, the TS 23.502 [7], figure 4.9.2.3.1-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

###### 5.2.3.3.3.3	PDU session modification due to change in access type (N3IWF in the HPLMN)

The flow-diagram in figure 5.2.3.3.3.3-1 illustrates a detailed view of PDU session modification for access type change in a home-routed roaming scenario. The flow illustrates the xIRI generation from the IRI-POI present in the H-SMF, LI_T3 triggering from the CC-TF present in the H-SMF and the xCC generation from the CC-POI present in the H-UPF.

In this illustration, the case of access type change from non-3GPP access to 3GPP access is considered. And the N3IWF is in the HPLMN. In this illustration, the LI does not apply to the VPLMN. In this case, the LI is on an outbound roaming UE.

Figure 5.2.3.3.3.3-1: Detailed flow diagram for PDU session modification (HPLMN) – access type change (N3IWF in the HPLMN)

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is H-SMF sending the Nsmf_PDU_Session_Create Response to the V-SMF with N1SmInfoToUe containing the message PDU SESSION ESTABLISHMENT ACCEPT.

-	The LI_T3 Modify Task trigger is sent from CC-TF present in the H-SMF to the CC-POI present in the H-UPF when the H-SMF sends the N4: PFCP Session Modification Request to the H-UPF.

-	The H-UPF generates the xCC based on the LI_T3 trigger received from the CC-TF. The xCC is generated from the user-plane packets that traverse through the H-UPF on the PDU session.

While drawing this flow, the TS 23.502 [7], figure 4.9.2.3.2-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

#### 5.2.3.4	PDU session release

##### 5.2.3.4.1	General

The PDU session release may be initiated by the UE or the network. In the latter case, PDU session release may be initiated by the PCF, SMF, AMF or the AN.

The PDU session may also be released without any NAS messages to the UE or also without any N2 message exchanges with the AN.

##### 5.2.3.4.2	Non-roaming scenario

###### 5.2.3.4.2.1	UE initiated PDU session release

The flow-diagram in figure 5.2.3.4.2.1-1 illustrates a detailed view of PDU session release steps for a non-roaming scenario. The flow illustrates the xIRI generation from the IRI-POI present in the SMF, LI_T3 triggering from the CC-TF present in the SMF and the xCC generation from the CC-POI present in the UPF.

In this illustration, the case of UE initiated PDU session release is considered.

Figure 5.2.3.4.2.1-1: Detailed flow diagram for PDU session release (non-roaming) – UE-initiated

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is SMF receiving the NAS message PDU SESSION RELEASE COMPLETE from the target UE and the 5GSM state changing into PDU Session Inactive.

-	The LI_T3 Deactivate Task trigger may be sent from CC-TF present in the SMF to the CC-POI present in the UPF when the SMF sends the N4: PFCP Session Deletion Request to the UPF.

-	The CC-POI in UPF stops generating the xCC upon receiving the LI_T3: Deactivate Task received from the CC-TF.

While drawing this flow, the TS 23.502 [7], figure 4.3.4.2-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

###### 5.2.3.4.2.2	Network initiated PDU session release

The flow-diagram in figure 5.2.3.4.2.2-1 illustrates a detailed view of PDU session release steps for a non-roaming scenario. The flow illustrates the xIRI generation from the IRI-POI present in the SMF, LI_T3 triggering from the CC-TF present in the SMF and the xCC generation from the CC-POI present in the UPF.

In this illustration, the case of SMF initiated PDU session release is considered.

Figure 5.2.3.4.2.2-1: Detailed flow diagram for PDU session release (non-roaming) – network-initiated

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is SMF receiving the NAS message PDU SESSION RELEASE COMPLETE from the target UE and the 5GSM state changing into PDU Session Inactive.

-	The LI_T3 Deactivate Task trigger is sent from CC-TF present in the SMF to the CC-POI present in the UPF when the SMF sends the N4: PFCP Deletion Request to the UPF.

-	The CC-POI in UPF stops generating the xCC upon receiving the LI_T3: Deactivate Task received from the CC-TF.

While drawing this flow, the TS 23.502 [7], figure 4.3.4.2-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

###### 5.2.3.4.2.3	Network initiated PDU session release without any N1 or N2 message transfer

The flow-diagram in figure 5.2.3.4.2.3-1 illustrates a detailed view of PDU session release steps for a non-roaming scenario without the transfer of any N1 (i.e. NAS) or N2 messages. In the illustration, the release procedure is initiated by the AMF.

The flow illustrates the xIRI generation from the IRI-POI present in the SMF, LI_T3 triggering from the CC-TF present in the SMF and the xCC generation from the CC-POI present in the VPF.

Figure 5.2.3.4.2.3-1: Detailed flow diagram for PDU session release without any N1 or N2 message transfer

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is SMF sending the Nsmf_PDUSession_ReleaseSMContext Response to the AMF and the 5GSM state changing into PDU Session Inactive.

-	The LI_T3 DeactivateTask trigger is sent from CC-TF present in the SMF to the CC-POI present in the UPF when the SMF sends the N4: PFCP Session Delete Request to the UPF.

-	The CC-POI in UPF stops generating the xCC upon receiving the LI_T3: Deactivate Task received from the CC-TF.

While drawing this flow, the TS 23.502 [7], figure 4.3.4.2-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

##### 5.2.3.4.3	Home-routed roaming scenario (VPLMN)

###### 5.2.3.4.3.1	UE initiated PDU session release

The flow-diagram in figure 5.2.3.4.3.1-1 illustrates a detailed view of PDU session release steps for a home-routed roaming scenario. The flow illustrates the xIRI generation from the IRI-POI present in the V-SMF, LI_T3 triggering from the CC-TF present in the V-SMF and the xCC generation from the CC-POI present in the V-UPF.

In this illustration, the case of UE initiated PDU session release is considered.

In this illustration, the LI does not apply to the HPLMN. In this case, the LI is on an inbound roaming UE.

Figure 5.2.3.4.3.1-1: Detailed flow diagram for PDU session release (VPLMN) – UE initiated

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is V-SMF receiving the NAS message PDU SESSION RELEASE COMPLETE from the target UE and the 5GSM state changing into PDU Session Inactive.

-	The LI_T3 Modify Task trigger is sent from CC-TF present in the V-SMF to the CC-POI present in the V-UPF when the V-SMF sends the N4: PFCP Session Modification Request to the V-UPF. For network-initiated modification, it is quite possible that the CC-interception is impacted (e.g. new QoS flow) and therefore, this is a good example where LI_T3: Modify Task is sent.

-	The CC-POI in V-UPF stops generating the xCC upon receiving the LI_T3: Deactivate Task received from the CC-TF.

While drawing this flow, the TS 23.502 [7], figure 4.3.3.3-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

###### 5.2.3.4.3.2	Network (HPLMN) initiated PDU session release

The flow-diagram in figure 5.2.3.4.3.2-1 illustrates a detailed view of PDU session release steps for a home-routed roaming scenario. The flow illustrates the xIRI generation from the IRI-POI present in the V-SMF, LI_T3 triggering from the CC-TF present in the V-SMF and the xCC generation from the CC-POI present in the V-UPF.

In this illustration, the case of Network (HPLMN) initiated PDU session release is considered.

In this illustration, the LI does not apply to the HPLMN. In this case, the LI is on an inbound roaming UE.

Figure 5.2.3.4.3.2-1: Detailed flow diagram for PDU session release (VPLMN) – Network (HPLMN) initiated

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is V-SMF receiving the NAS message PDU SESSION MODIFICATION COMPLETE from the target UE and the 5GSM state changing into PDU Session Inactive.

-	The LI_T3 Modify Task trigger is sent from CC-TF present in the V-SMF to the CC-POI present in the V-UPF when the V-SMF sends the N4: PFCP Session Modification Request to the V-UPF. For network-initiated modification, it is quite possible that the CC-interception is impacted (e.g. new QoS flow) and therefore, this is a good example where LI_T3: Modify Task is sent.

-	The CC-POI in V-UPF stops generating the xCC upon receiving the LI_T3: Deactivate Task received from the CC-TF.

While drawing this flow, the TS 23.502 [7], figure 4.3.3.3-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

###### 5.2.3.4.3.3	Network initiated PDU session release without any N1 or N2 message transfer

The flow-diagram in figure 5.2.3.4.3.3-1 illustrates a detailed view of PDU session release steps for a home-routed roaming scenario without the transfer of any N1 (i.e. NAS) or N2 messages. In the illustration, the release procedure is initiated by the AMF.

The flow illustrates the xIRI generation from the IRI-POI present in the V-SMF, LI_T3 triggering from the CC-TF present in the V-SMF and the xCC generation from the CC-POI present in the V-UPF.

In this illustration, the LI does not apply to the HPLMN. In this case, the LI is on an inbound roaming UE.

Figure 5.2.3.4.3.3-1: Detailed flow diagram for PDU session release without any N1 or N2 message transfer (VPLMN)

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is the V-SMF sending the Nsmf_PDUSession_ReleaseSMContext Response to the AMF and the 5GSM state changing into PDU Session Inactive.

-	The LI_T3 DeactivateTask trigger is sent from CC-TF present in the V-SMF to the CC-POI present in the V-UPF when the V-SMF sends the N4: PFCP Session Delete Request to the V-UPF.

-	The CC-POI in V-UPF stops generating the xCC upon receiving the LI_T3: Deactivate Task received from the CC-TF.

While drawing this flow, the TS 23.502 [7], figure 4.3.4.3-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

##### 5.2.3.4.4	Home-routed roaming scenario (HPLMN)

###### 5.2.3.4.4.1	UE-initiated PDU session release

The flow-diagram in figure 5.2.3.4.4.1-1 illustrates a detailed view of PDU session release steps for a home-routed roaming scenario. The flow illustrates the xIRI generation from the IRI-POI present in the H-SMF, LI_T3 triggering from the CC-TF present in the H-SMF and the xCC generation from the CC-POI present in the H-UPF.

In this illustration, the case of UE-initiated session release is considered.

In this illustration, the LI does not apply to the VPLMN. In this case, the LI is on an outbound roaming UE.

Figure 5.2.3.4.4.1-1: Detailed flow diagram for PDU session release (HPLMN) – UE initiated

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is H-SMF receiving the Nsmf_PDU_Session_Update Response from the V-SMF with N1SmInfoFromUe containing the message PDU SESSION RELEASE COMPLETE.

-	The LI_T3 Deactivate Task trigger is sent from CC-TF present in the H-SMF to the CC-POI present in the H-UPF when the H-SMF sends the N4: PFCP Deletion Request to the H-UPF.

-	The CC-POI in H-UPF stops generating the xCC upon receiving the LI_T3: Deactivate Task received from the CC-TF

While drawing this flow, the TS 23.502 [7], figure 4.3.3.3-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

###### 5.2.3.4.4.2	Network (HPLMN) initiated PDU session release

The flow-diagram in figure 5.2.3.4.4.2-1 illustrates a detailed view of PDU session release steps for a home-routed roaming scenario. The flow illustrates the xIRI generation from the IRI-POI present in the H-SMF, LI_T3 triggering from the CC-TF present in the H-SMF and the xCC generation from the CC-POI present in the H-UPF.

In this illustration, the case of network (HPLMN) initiated session release is considered.

In this illustration, the LI does not apply to the VPLMN. In this case, the LI is on an outbound roaming UE.

Figure 5.2.3.4.4.2-1: Detailed flow diagram for PDU session release (HPLMN) – network (HPLMN) initiated

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is H-SMF receiving the Nsmf_PDU_Session_Update Response from the V-SMF with N1SmInfoFromUe containing the message PDU SESSION RELEASE COMPLETE.

-	The LI_T3 Deactivate Task trigger is sent from CC-TF present in the H-SMF to the CC-POI present in the H-UPF when the H-SMF sends the N4: PFCP Deletion Request to the H-UPF.

-	The CC-POI in H-UPF stops generating the xCC upon receiving the based the LI_T3: Deactivate Task received from the CC-TF.

While drawing this flow, the TS 23.502 [7], figure 4.3.3.3-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

###### 5.2.3.4.4.3	Network initiated PDU session release without any N1 or N2 message transfer

The flow-diagram in figure 5.2.3.4.4.3-1 illustrates a detailed view of PDU session release steps for a home-routed roaming scenario without the transfer of any N1 (i.e. NAS) or N2 messages. In the illustration, the release procedure is initiated by the AMF present in the VPLMN.

The flow illustrates the xIRI generation from the IRI-POI present in the H-SMF, LI_T3 triggering from the CC-TF present in the H-SMF and the xCC generation from the CC-POI present in the H-UPF.

In this illustration, the LI does not apply to the VPLMN. In this case, the LI is on an outbound roaming UE.

Figure 5.2.3.4.4.3-1: Detailed flow diagram for PDU session release without any N1 or N2 message transfer (HPLMN)

As defined in TS 33.128 [4]:

-	The trigger conditions for the generation of xIRI is the H-SMF sending the Nsmf_PDUSession_Release Response to the V-SMF (in VPLMN).

-	The LI_T3 DeactivateTask trigger is sent from CC-TF present in the H-SMF to the CC-POI present in the H-UPF when the H-SMF sends the N4: PFCP Session Delete Request to the H-UPF.

-	The CC-POI in H-UPF stops generating the xCC upon receiving the LI_T3: Deactivate Task from the CC-TF.

While drawing this flow, the TS 23.502 [7], figure 4.3.4.3-1 is referred to. The PFCP session related messages are defined in TS 29.244 [9].

**** End of Retained (duplicates) ****