3GPP SA3LI#96	s3i250016

# 28-31 January 2025, Sophia Antipolis (France)

Title:	Presentation on LI of Pro Se Communications via 5G

Release:	Rel-19

Work Item:	LI19

Source:	Ministére de l’économie et des finances, France

- Overall description

Proximity based Services (ProSe) are services that can be provided by the 5GS based on UEs being in proximity to each other.

The 5GS enablers for ProSe include the following functions:

- 5G ProSe Direct Discovery: 5G ProSe-enabled UEs are able to discover services offered by neighbouring ProSe-enabled UEs.

- 5G ProSe Direct Communication: 5G ProSe-enabled UEs are able to directly communicate to each other's without requiring or going through a 5G gNB.

- 5G ProSe UE-to-Network Relay: A 5G ProSe-enabled UE may act as a relay to extend the reach of gNBs.

The contributions are related to:

- LI for 5G Prose Direct Discovery

- LI for 5G ProSe Remote UE communication via a 5G ProSe UE-to-Network Relay

- LI for 5G ProSe Direct Discovery

5G ProSe Direct Discovery provides mechanisms for devices to discover other devices in close proximity. Functions and capabilities introduced by ProSe can be used by any application running on top of a ProSe-enabled UE.

In case of direct discovery, the two possible discovery models are:

-	Model A ("I am here"): In this model, the UE announces its presence to other UE(s) who are interested in reading and/or processing the messages. The UE that announces certain information that could be used by other UE(s) in proximity is called the "announcing" UE. The UE that has the permission to discover and the interest to read and/or process messages from an "announcing" UE in proximity is called the "monitoring" UE.

-	Model B ("who is there" and/or "are you there"): In this model, the UE tries to discover other UE(s) by sending a request containing certain information. The UE that transmits a request containing certain information about what it is interested to discover is called the "discoverer" UE. The UE that receives and processes the request message and responds with some information related to the discoverer's request is called the "discoveree" UE.

The ProSe Direct Discovery feature is defined in an "open" mode and in a "restricted" mode. For the open mode, no permission is needed from the UE that is to be discovered, while the restricted mode requires permission from the UE that is being discovered. Use cases for open discovery may include discovering some public machine type devices (e.g., a vending machine located close to a location) for which no permission is needed from the UE (within the vending machine) that is being discovered. Restricted discovery may be the most commonly used mechanism for devices used by humans to avoid privacy concerns (e.g., discovering a user walking close to a given location will probably require explicit permission from the user).

5G DDNMF (5G Direct Discovery Name Management Function) is the function handling network related actions required for dynamic 5G ProSe Direct Discovery in Model A and Model B in open or restricted mode.

A 5G ProSe-enabled UE only interacts with the DDNMF in its HPLMN to get ProSe service authorizations using the PC3a interface. PC3a relies on 5GC user plane for transport (i.e., an "over IP" interface). The 5G DDNMF is used to authorise 5G ProSe Direct Discovery request, and perform allocation of ProSe Application Codes / ProSe Restricted Codes corresponding to ProSe Application Identities used for 5G ProSe Direct Discovery

The 5G DDNMF in the HPLMN may interact with the 5G DDNMF in a VPLMN or Local PLMN in order to manage the 5G ProSe Direct Discovery service. The local PLMN is a PLMN in which the "monitoring" UE is authorized by the HPLMN to use radio resources to engage in ProSe Direct Discovery. The Local PLMN is not the same as the serving PLMN (HPLMN or VPLMN) of the UE. Final authorization of the UE is always performed by the 5G DDNMF in the HPLMN.

The 5G DDNMF in the HPLMN merges authorization information from home, serving and local PLMNs. The DDNMF in the Local PLMN, VPLMN, or HPLMN may revoke the authorization at any time. The 5G DDNMF in the HPLMN is notified when authorization is revoked by the Local PLMN or the VPLMN. The 5G DDNMF in the HPLMN notifies the UE about changes to the authorization information.

ProSe Direct Discovery can be a stand-alone service (offered independent of ProSe direct communication), providing information about discovered UE(s) to certain applications running in the discoverer UE that are permitted to use this information, for example, “find a taxi nearby” (in these cases the discovered UE can be a device in the taxi). Additionally, depending on the information obtained, ProSe Direct Discovery can be used for subsequent actions, for example, to initiate ProSe Direct Communication, i.e., using NR radio signals via the 5G ProSe-enabled radio interface without going via the network.

This contribution considers an IRI-POI in the 5G DDNMF to intercept all the different signaling flows related to 5G ProSe Direct Discovery exchanged between a target 5G ProSe-enabled UE and the 5G DDNMF and defines the corresponding xIRIs. The xIRIS are defined at Stage 2 in the document s3i250018 and at Stage 3 in the document s3i250020.

- LI for 5G ProSe Remote UE communication via a 5G ProSe UE-to-Network Relay

The UE-to-Network Relay entity provides the relaying functionality to support connectivity to the network for 5G ProSe Remote UEs.

A 5G ProSe UE-to-Network Relay registers to the network (if not already registered). 5G ProSe UE-to-Network Relay establishes a PDU Session(s) or modifies an existing PDU Session(s) in order to provide relay traffic towards 5G ProSe Remote UE(s). PDU Session(s) supporting 5G ProSe UE-to-Network Relay shall only be used for 5G ProSe Remote UE(s) relay traffic.

New xIRIs are generated by the IRI-POI in the SMF to identify that a 5G ProSe Remote target UE is connected to or disconnected from a 5G ProSe UE-to-network relay. The xIRIs are defined at Stage 2 in the document s3i250017 and at Stage 3 in the document s3i250019.