3GPP TSG-SA3 Meeting #96-LI	s3i250019

# Sophia-Antipolis, France, 28th Jan 2025 - 31st Jan 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0704 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SMF enhancement for LI for 5G ProSe Communication via 5G ProSe UE-to-Network Relay - Stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2025-01-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Absence of LI for 5G ProSe Communication via 5G ProSe UE-to-Network Relay - Stage 3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | SMF enhancement for LI for 5G ProSe Communication via 5G ProSe UE-to-Network Relay - Stage 3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LI for 5G ProSe Communication via 5G ProSe UE-to-Network Relay - Stage 3 would still be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.1; 6.2.3.2.X ; 6.2.3.2.Y; 6.2.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | * | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | * | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | * | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Schema changes for this CR can be found on the Forge: Merge Request: https://forge.3gpp.org/rep/sa3/li/-/merge_requests/307 Commit Hash: https://forge.3gpp.org/rep/sa3/li/-/commit/9e7a33910dce675ec9b4b3fd41b9021b368abfb6 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

##### 6.2.3.2.1	General

The IRI-POI present in the SMF shall send the xIRIs over LI_X2 for each of the events listed in TS 33.127 [5] clause 6.2.3.3, the details of which are described in the following clauses. In the case where the SMF is part of a combined SMF+PGW-C, the details of the events are specified in clause 6.3.3.2. The IRI-POI present in the SMF shall also send a SeparatedLocationReporting xIRI (as described in clause 7.3.4.1) when the IRI-POI provisioned in the H-SMF detects that the V-SMF has sent location data via the HsmfUpdateData service operation to the H-SMF that does not otherwise trigger an existing SMF record type.

As specified in TS 23.501 [2] clause 5.6.1, a PDU session may support either a single-access PDU Connectivity Service (referred to as a single-access PDU Session) or a multi-access PDU Connectivity Service (referred to as a	Multi-Access PDU (MA PDU) session).

The details of the messages for single-access PDU sessions are provided below in clauses 6.2.3.2.2, 6.2.3.2.3, 6.2.3.2.4, 6.2.3.2.5 and 6.2.3.2.6.

The details of the messages for multi-access PDU sessions are provided below in clauses 6.2.3.2.7 and 6.2.3.2.8.

The details of the messages for 5G ProSe remote UE connected to or disconnected from a 5G ProSe layer-3 UE-to-network relay are provided below in clause 6.2.3.2.X and 6.2.3.2.Y.

END OF FIRST CHANGE

START OF SECOND CHANGE

##### 6.2.3.2.X	Remote UE report

The IRI-POI in the SMF shall generate an xIRI containing an SMFRemoteUEReport record when the IRI-POI present in the SMF detects that a target 5G ProSe remote UE has connected to a 5G ProSe layer-3 UE-to-network relay UE or disconnected from a 5G ProSe layer-3 UE-to-network relay UE. The IRI-POI present in the SMF shall generate the xIRI for the following events:

-	For a non-roaming scenario, the SMF (or for a roaming scenario, V-SMF in the VPLMN for HR or SMF in the VPLMN for LBO) returns a Nsmf_PDUSession_UpdateSMContext Response in response to a Nsmf_PDUSession_UpdateSMContext Request received from the AMF including the n1SmMsg parameter containing the N1 NAS message Remote UE Report related to a target 5G ProSe remote UE.

Table 6.2.3.2.X-1: Payload for SMFRemoteUEReport record

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| sUPI | SUPI | 1 | Identifies the SUPI of the target 5G ProSe remote UE connected to or disconnected from a 5G ProSe layer-3 UE-to-network relay. | M |
| gPSI | GPSI | 0..1 | Identifies the GPSI of the target 5G ProSe remote UE connected to or disconnected from a 5G ProSe layer-3 UE-to-network relay. | C |
| pEI | PEI | 0..1 | Identifies the PEI of the target 5G ProSe remote UE connected to or disconnected from a 5G ProSe layer-3 UE-to-network relay. | C |
| proSeRelaySUPI | SUPI | 1 | Identifies the SUPI of the 5G ProSe layer-3 UE-to-network relay the target UE is connected to or disconnected from. | M |
| proSeRelayGPSI | GPSI | 0..1 | Identifies the GPSI of 5G ProSe layer-3 UE-to-network relay the target 5G ProSe remote UE is connected to or disconnected from. | C |
| proSeRelayPEI | PEI | 0..1 | Identifies the PEI of the 5G ProSe layer-3 UE-to-network relay the target 5G ProSe remote UE is connected to or disconnected from. | C |
| pDUSessionID | PDUSessionID | 1 | PDU session of the 5G ProSe layer-3 UE-to-network relay associated with the target 5G ProSe remote UE connected to the 5G ProSe layer-3 UE-to-network relay UE or disconnected from the 5G ProSe layer-3 UE-to-network relay UE. | M |
| remoteUEContextConnected | RemoteUEContextList | 0..1 | Provides newly connected target 5G ProSe remote UE information. Shall contain the target remote UE context list octets if sent in the Remote UE Report message. Defined in TS 24.501 [13] clause 9.11.4.29, omitting the first three octets. | C |
| remoteUEContextDisconnected | RemoteUEContextList | 0..1 | Provides disconnected target 5G ProSe remote UE information. Shall contain the target remote UE context list octets if sent in the Remote UE Report message. Defined in TS 24.501 [13] clause 9.11.4.29, omitting the first three octets. | C |
| Location | Location | 0..1 | Location of the target 5G ProSe remote UE if available. Shall be encoded using the Location.locationInfo.userLocation parameter. If available, other parameters reportable via Location shall be included. | C |

##### 6.2.3.2.Y	Start of interception with already connected remote UE

The IRI-POI in the SMF shall generate an xIRI containing an SMFStartOfInterceptionWithAlreadyConnectedRemoteUE record when the IRI-POI present in the SMF detects that a target 5G ProSe remote UE has already been connected to a 5G ProSe layer-3 UE-to-network relay UE when interception starts.

Table 6.2.3.2.Y-1: Payload for SMFStateOfInterceptionWithAlreadyConnectedRemoteUE record

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| sUPI | SUPI | 1 | Identifies the SUPI of the target 5G ProSe remote UE connected to or disconnected from a 5G ProSe layer-3 UE-to-network relay. | M |
| gPSI | GPSI | 0..1 | Identifies the GPSI of the target 5G ProSe remote UE connected to or disconnected from a 5G ProSe layer-3 UE-to-network relay. | C |
| pEI | PEI | 0..1 | Identifies the PEI of the target 5G ProSe remote UE connected to or disconnected from a 5G ProSe layer-3 UE-to-network relay. | C |
| proSeRelaySUPI | SUPI | 1 | Identifies the SUPI of the 5G ProSe layer-3 UE-to-network relay the target UE is connected to or disconnected from. | M |
| proSeRelayGPSI | GPSI | 0..1 | Identifies the GPSI of 5G ProSe layer-3 UE-to-network relay the target 5G ProSe remote UE is connected to or disconnected from. | C |
| proSeRelayPEI | PEI | 0..1 | Identifies the PEI of the 5G ProSe layer-3 UE-to-network relay the target 5G ProSe remote UE is connected to or disconnected from. | C |
| pDUSessionID | PDUSessionID | 1 | PDU session of the 5G ProSe layer-3 UE-to-network relay associated with the target 5G ProSe remote UE connected to the 5G ProSe layer-3 UE-to-network relay UE or disconnected from the 5G ProSe layer-3 UE-to-network relay UE. | M |
| remoteUEContextConnected | RemoteUEContextList | 0..1 | Provides newly connected target 5G ProSe remote UE information. Shall contain the target remote UE context list octets if sent in the Remote UE Report message. Defined in TS 24.501 [13] clause 9.11.4.29, omitting the first three octets. | C |
| Location | Location | 0..1 | Location of the target 5G ProSe remote UE if available. Shall be encoded using the Location.locationInfo.userLocation parameter. If available, other parameters reportable via Location shall be included. | C |

END OF SECOND CHANGE

START OF THIRD CHANGE

#### 6.2.3.7	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the SMF or the IRI-POI in the UPF, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g. additional location information).

The ETSI TS 102 232-1 [9] @LI-PS-PDU.pSHeader.timeStamp field shall be set to the time at which the SMF event was observed (i.e. the timestamp field of the xIRI).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIType parameter (see ETSI TS 102 232-1 [9] clause 5.2.10) shall be included and coded according to table 6.2.3.7-1.

Table 6.2.3.7-1: IRI type for IRI messages

| Record type | IRI Type |
| --- | --- |
| SMFPDUSessionEstablishment | BEGIN |
| SMFPDUSessionRelease | END |
| SMFPDUSessionModification | CONTINUE |
| SMFStartOfInterceptionWithEstablishedPDUSession | BEGIN |
| SMFUnsuccessfulProcedure | REPORT |
| SMFMAPDUSessionEstablishment | BEGIN |
| SMFMAPDUSessionRelease | END |
| SMFMAPDUSessionModification | CONTINUE |
| SMFStartOfInterceptionWithEstablishedMAPDUSession | BEGIN |
| SMFMAUnsuccessfulProcedure | REPORT |
| SMFPDUtoMAPDUSessionModification | CONTINUE |
| PDHeaderReport | REPORT |
| PDSummaryReport | REPORT |
| SMFRemoteUEReport | REPORT |
| SMFStartOfInterceptionWithAlreadyConnectedRemoteUE | REPORT |

IRI messages associated with the same PDU Session shall be assigned the same CIN (see ETSI TS 102 232-1 [9] clause 5.2.4).

The @LI-PS-PDU.payload.iRIPayloadSequence.iRIContents.threeGPP33128DefinedIRI field (see ETSI TS 102 232-7 [10] clause 15) of the LI_HI2 message shall be populated with the BER-encoded IRIPayload.

When an additional warrant is activated on a target UE and the LIPF uses the same XID for the additional warrant, the MDF2 shall be able to generate and deliver the IRI message containing the SMFStartOfInterceptionWithEstablishedPDUSession record and the SMFStartOfInterceptionWithEstablishedMAPDUSession record to the LEMF associated with the additional warrant without receiving a corresponding xIRI. The payload of the SMFStartOfInterceptionWithEstablishedPDUSession record is specified in table 6.2.3.2.5-1, while the payload of the SMFStartOfInterceptionWithEstablishedMAPDUSession record is specified in table 6.2.3.4-1. The MDF2 shall generate and deliver the IRI message containing the SMFStartOfInterceptionWithEstablishedPDUSession record for each of the established PDU sessions to the LEMF associated with the new warrant. The MDF2 shall generate and deliver the IRI message containing the SMFStartOfInterceptionWithEstablishedMAPDUSession record for each of the established MA PDU sessions to the LEMF associated with the new warrant.

If the MDF2 did not receive a previous SMFStartOfInterceptionWithEstablishedPDUSession.timeOfSessionEstablishment or SMFStartOfInterceptionWithEstablishedMAPDUSession.timeOfSessionEstablishment for the same session from the IRI-POI, , the MDF2 shall set the value of the SMFStartOfInterceptionWithEstablishedPDUSession.timeOfSessionEstablishment or SMFStartOfInterceptionWithEstablishedMAPDUSession.timeOfSessionEstablishment to the time provided in the timestamp previously received in the header of the related SMFPDUSessionEstablishment or SMFMAPDUSessionEstablishment xIRI.

When the delivery of packet header information is authorised and approach 2 described in clause 6.2.3.9.1 is used, the MDF2 shall generate the IRI message and send it over LI_HI2 without undue delay when xCC is received over LI_MDF from the MDF3. The MDF2 shall generate packet header information reporting as described in clause 6.2.3.5.

END OF THIRD CHANGE

START OF CHANGE 1

---a/33128/r19/TS33128Payloads.asn
+++b/33128/r19/TS33128Payloads.asn

@@ -304,7 +304,11 @@ XIRIEvent ::= CHOICE

304	304		    iMSHSSSubscriberRecordChange                        [174] IMSHSSSubscriberRecordChange,

305	305

306	306		    -- AMF events, see clause 6.2.2.2.14, continued from tag 147

307		-	    aMFUEContextUpdate                                  [175] AMFUEContextUpdate

307	+	    aMFUEContextUpdate                                  [175] AMFUEContextUpdate,

308	+

309	+	    -- SMF events, see clause 6.2.3.2 continued from tag 61

310	+	    sMFRemoteUEReport                                   [176] SMFRemoteUEReport,

311	+	    sMFStartOfInterceptionWithAlreadyConnectedRemoteUE  [177] SMFStartOfInterceptionWithAlreadyConnectedRemoteUE

308	312		}

309	313

310	314		-- ==============

@@ -592,7 +596,11 @@ IRIEvent ::= CHOICE

592	596		    iMSHSSSubscriberRecordChange                        [174] IMSHSSSubscriberRecordChange,

593	597

594	598		    -- AMF events, see clause 6.2.2.3, continued from tag 147

595		-	    aMFUEContextUpdate                                  [175] AMFUEContextUpdate

599	+	    aMFUEContextUpdate                                  [175] AMFUEContextUpdate,

600	+

601	+	    -- SMF events, see clause 6.2.3.7 continued from tag 61

602	+	    sMFRemoteUEReport                                   [176] SMFRemoteUEReport,

603	+	    sMFStartOfInterceptionWithAlreadyConnectedRemoteUE  [177] SMFStartOfInterceptionWithAlreadyConnectedRemoteUE

596	604		}

597	605

598	606		IRITargetIdentifier ::= SEQUENCE

@@ -2609,6 +2617,32 @@ SMFMAUnsuccessfulProcedure ::= SEQUENCE

2609	2617		    sMPDUDNRequest              [17] SMPDUDNRequest OPTIONAL

2610	2618		}

2611	2619

2620	+	SMFRemoteUEReport ::= SEQUENCE

2621	+	{

2622	+	    sUPI                        [1] SUPI,

2623	+	    gPSI                        [2] GPSI,

2624	+	    pEI                         [3] PEI,

2625	+	    proSeRelaySUPI              [4] SUPI,

2626	+	    proSeRelayGPSI              [5] GPSI,

2627	+	    proSeRelayPEI               [6] PEI,

2628	+	    pDUSessionID                [7] PDUSessionID,

2629	+	    remoteUEContextConnected    [8] RemoteUEContextList,

2630	+	    remoteUEContextDisconnected [9] RemoteUEContextList,

2631	+	    location                    [10] Location

2632	+	}

2633	+

2634	+	SMFStartOfInterceptionWithAlreadyConnectedRemoteUE ::= SEQUENCE

2635	+	{

2636	+	    sUPI                        [1] SUPI,

2637	+	    gPSI                        [2] GPSI,

2638	+	    pEI                         [3] PEI,

2639	+	    proSeRelaySUPI              [4] SUPI,

2640	+	    proSeRelayGPSI              [5] GPSI,

2641	+	    proSeRelayPEI               [6] PEI,

2642	+	    pDUSessionID                [7] PDUSessionID,

2643	+	    remoteUEContextConnected    [8] RemoteUEContextList,

2644	+	    location                    [9] Location

2645	+	}

2612	2646

2613	2647		-- =================

2614	2648		-- 5G SMF parameters

@@ -2918,6 +2952,92 @@ EASServerAddress ::= SEQUENCE

2918	2952		-- See table 5.4.2.1 of TS 29.571 [17]

2919	2953		GEOSatelliteID ::= UTF8String

2920	2954

2955	+	RemoteUEContextList ::= SEQUENCE

2956	+	{

2957	+	    numberOfRemoteUEContexts [1] INTEGER,

2958	+	    remoteUEContexts         [2] SET OF RemoteUEContext

2959	+	}

2960	+

2961	+	RemoteUEContext ::= SEQUENCE

2962	+	{

2963	+	    remoteUEIDFormat       [1] RemoteUEIDFormat,

2964	+	    remoteUEIDType         [2] RemoteUEIDType,

2965	+	    remoteUEID             [3] RemoteUEID,

2966	+	    uDPPortRangeIndicator  [4] BOOLEAN,

2967	+	    tCPPortRangeIndicator  [5] BOOLEAN,

2968	+	    protocolUsedByRemoteUE [6] ProtocolUsedByRemoteUE,

2969	+	    addressInformation     [7] AddressInformation OPTIONAL,

2970	+	    hPLMNID                [8] PLMNID OPTIONAL

2971	+	}

2972	+

2973	+	ProtocolUsedByRemoteUE ::= ENUMERATED

2974	+	{

2975	+	    noIPInfo(1),

2976	+	    iPv4(2),

2977	+	    iPv6(3),

2978	+	    unstructured(4),

2979	+	    ethernet(5)

2980	+	}

2981	+

2982	+	RemoteUEIDFormat ::= ENUMERATED

2983	+	{

2984	+	    nAI(1),

2985	+	    sixtyFourBitString(2)

2986	+	}

2987	+

2988	+	RemoteUEIDType ::= ENUMERATED

2989	+	{

2990	+	    uPPRUKID(1),

2991	+	    cPPRUKID(2),

2992	+	    iMEI(3),

2993	+	    iMEISV(4)

2994	+	}

2995	+

2996	+	RemoteUEID ::= CHOICE

2997	+	{

2998	+	    uPPRUKIDNAI         [1] NAI,

2999	+	    uPPRUKID64BitString [2] BIT STRING (SIZE(64)),

3000	+	    cPPRUKIDNAI         [3] NAI,

3001	+	    cPPRUKID64BitString [4] BIT STRING (SIZE(64)),

3002	+	    iMEI                [5] IMEI,

3003	+	    iMEISV              [6] IMEISV

3004	+	}

3005	+

3006	+	AddressInformation ::= CHOICE

3007	+	{

3008	+	    iPv4Address                [1] IPv4Address,

3009	+	    iPv4AddressUDPPortRange    [2] IPv4AddressUDPPortRange,

3010	+	    iPv4AddressTCPPortRange    [3] IPv4AddressTCPPortRange,

3011	+	    iPv4AddressUDPTCPPortRange [4] IPv4AddressUDPTCPPortRange,

3012	+	    iPv6Address                [5] IPv6Address,

3013	+	    ethernetAddress            [6] MACAddress

3014	+	}

3015	+

3016	+	IPv4AddressUDPPortRange ::= SEQUENCE

3017	+	{

3018	+	    iPv4Address         [1] IPv4Address,

3019	+	    uDPPortRange        [2] PortRange

3020	+	}

3021	+

3022	+	IPv4AddressTCPPortRange ::= SEQUENCE

3023	+	{

3024	+	    iPv4Address         [1] IPv4Address,

3025	+	    tCPPortRange        [2] PortRange

3026	+	}

3027	+

3028	+	IPv4AddressUDPTCPPortRange ::= SEQUENCE

3029	+	{

3030	+	    iPv4Address         [1] IPv4Address,

3031	+	    uDPPortRange        [2] PortRange,

3032	+	    tCPPortRange        [2] PortRange

3033	+	}

3034	+

3035	+	PortRange ::= SEQUENCE

3036	+	{

3037	+	    portStart [1] INTEGER (0..65535),

3038	+	    portEnd   [2] INTEGER (0..65535)

3039	+	}

3040	+

2921	3041		-- ================================

2922	3042		-- PGW-C + SMF PDNConnection Events

2923	3043		-- ================================

END OF CHANGE 1

END OF LAST CHANGE