3GPP SA3LI#94	s3i250045

# 27-31 January 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0276 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 logging / audit requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (NTAC) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As discussed in s3i240484 and s3i250042, additional stage 2 text is required to properly explain how the the security and audit requirements in TS 33.126 clauses 6.5 and 6.6 are met. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds stage 2 descriptions of which interception lifecycle events are required to be logged and how. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security and audit requirements in TS 33.126 cannot be reliably met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2, 5.3.3, 5.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

### 5.3.2	Point of Interception (POI)

#### 5.3.2.1	General

The Point of Interception (POI) detects the target communication, derives the intercept related information or communications content from the target communications and delivers the POI output as xIRI to the MDF2 or as xCC to the MDF3. The output of a POI is determined by the type of the NF associated with the POI. A POI may be embedded within a Network Function (NF) or separate from a NF with which it is associated.

Multiple POIs may have to be involved in executing a warrant.

#### 5.3.2.2	Directly provisioned and triggered POIs

POIs are divided into two categories:

-	Directly provisioned POIs are provisioned by the LIPF.

-	Triggered POIs are triggered by a Triggering Function (TF) (see clause 5.3.3).

The directly provisioned POIs detect the target's communications that need to be intercepted, and then derive the intercept related information or communication contents from that target communications depending on the POI type (see clause 5.3.2.3). The triggered POIs detect the target communications based on the trigger received from an associated Triggering Function and then derives the intercept related information or communication contents of target communications depending on the POI type (see clause 5.3.2.3).

#### 5.3.2.3	IRI-POIs and CC-POIs

POIs are divided into two types for each category based on the type of data they send to the MDF (see clause 5.3.4):

-	IRI-POI delivers xIRI to the MDF2.

-	CC-POI delivers xCC to the MDF3.

Both IRI-POIs and CC-POIs are either directly provisioned or triggered (see clause 5.3.2.2).

In the present document, an xIRI is identified with the event that has caused its generation within the IRI-POI.

#### 5.3.2.4	Failure handling

In case a network procedure involving the target UE and requiring the generation of an xIRI fails, the IRI-POI shall be able to report the failure reason available from the involved network protocol.

#### 5.3.2.X	Logging and audit

The POI shall support securely creating and storing log records of at least the following events:

-	Provisioning of a new interception task by either the LIPF or a TF

-	Alteration of an existing interception task by either the LIPF or a TF

-	Deprovisioning of an existing interception task by either the LIPF or a TF

-	Transmission of any error messages back to either the LIPF or TF

Any logs that are created shall comply with the security requirements in TS 33.126 [1] clause 6.5 and 6.6; particular attention should be given to ensuring that logs cannot be accessed or modified by unauthorised actors.

Which events are logged and the retention period of any logs is a matter for CSP deployment.

START OF CHANGE 2

### 5.3.3	Triggering Function

The Triggering Function (TF) is provisioned by the LIPF and is responsible for managing the interception state of triggered functions in response to network and service events matching the criteria provisioned by the LIPF. The Triggering Function detects the target communications and sends a trigger to the associated triggered function, and deactivates interception at the associated triggered function when required.

As a part of this triggering, the Triggering Function shall send all necessary interception rules (i.e. rules that allow the POIs to detect the target communications), forwarding rules (i.e. MDF2, MDF3 address), target identity, and the correlation information.

A Triggering Function may interact with other POIs to obtain correlation information. Details of this interface are not specified by the present document.

The Triggering Function is responsible for ensuring that the correct interception state is maintained at the associated triggered function, including retrieving information about the current interception state from the triggered function, and reporting this to the LIPF. Subject to operator policy, the Triggering Function may take corrective action when a discrepancy is discovered.

The Triggering Function that triggers CC-POI is referred to as a CC-TF and the Triggering Function that triggers an IRI-POI is referred to as IRI-TF.

Triggering Functions shall support logging of the same lifecycle events as for POIs, as described clause 5.3.2.X. In addition, Triggering Functions shall support securely creating and storing log records of the following additional events:

Provisioning of a new interception task by the TF at a Triggered POI

-	Alteration of an existing interception task by the TF at a Triggered POI

-	Deprovisioning of an existing interception task by the TF at a Triggered POI

-	Reception of error messages by the TF from a Triggered POI

-	Transmission of any error messages back to the LIPF

All other requirements in 5.3.2.X shall apply to Triggering Functions.

START OF CHANGE 3

### 5.3.4	Mediation and Delivery Function (MDF)

The Mediation and Delivery Function (MDF) delivers the Interception Product to the Law Enforcement Monitoring Facility (LEMF).

Two variations of MDF are defined: MDF2 and MDF3.

MDF2 generates the IRI messages from the xIRI and sends them to one or more LEMFs. The MDF3 generates the CC from the xCC and delivers it to one or more intercepting LEMFs. An overview of this is shown in figure 5.3.4-1 below.

Figure 5.3.4-1: MDF2 and MDF3

The MDF2 and MDF3 are provisioned by the LIPF with the intercept information necessary to deliver the IRI and/or CC to one or more LEMFs.

The LI_MDF interface between MDF2 and MDF3 (shown in figure 5.3.4-1) allows the MDF3 and MDF2 to exchange information between the two.

The MDF2 and MDF3 shall support securely creating and storing log records of at least the following events:

-	Provisioning of a new mediation task by the LIPF

-	Alteration of an existing mediation task by the LIPF

-	Deprovisioning of an existing mediation task by the LIPF

-	Transmission of any error messages back to the LIPF

Any logs that are created shall comply with the security requirements in TS 33.126 [1] clause 6.5 and 6.6; particular attention should be given to ensuring that logs cannot be accessed or modified by unauthorised actors.

Which events are logged and the retention period of any logs is a matter for CSP implementation.

END OF CHANGES