# Roaming Case (LI) – scenario 1

| Scenario 1 | Scenario 2 |
| --- | --- |
| - UE-1 (local PLMN) gets the code from DDNMF (local PLMN). xIRI if UE-1 is a target in Local PLMN. - UE-1 announces to the V-UE with the code. - V-UE requests the DDNMF (HPLMN) for authorization (w/ the code). - DDNMF (HPLMN) interacts with the DDNMF (local PLMN) to confirm. xIRI from Local PLMN if UE-1 is a target. - DDNMF (HPLMN) responds to the V-UE with the authorization success. xIRI if V-UE is a target in HPLMN. - V-UE and UE-1 has ProSe connection. | - UE-2 (VPLMN) gets the code from DDNMF (VPLMN). xIRI if UE-2 is a target in Local PLMN (shown asVPLMN). - UE-2 announces to the V-UE with the code. - V-UE requests the DDNMF (HPLMN) for authorization (w/ the code). - DDNMF (HPLMN) interacts with the DDNMF (VPLMN) to confirm. xIRI if UE-2 is a target?? xIRI if V-UE is a target in VPLMN. If UE-2 is a target in Local PLMN (shown as VPLMN). - DDNMF (HPLMN) responds to the V-UE with the authorization success. xIRI if V-UE is a target in HPLMN. - V-UE and UE-2 has ProSe connection. |

# Roaming Case (LI) – scenario 2

Scenario 1

- V-UE (VPLMN) gets the code from DDNMF (HPLMN). xIRI if V-UE is a target in HPLMN. - V-UE announces to the UE-1 with the code. - UE-1 requests the DDNMF (Local PLMN) for authorization (w/ the code). - DDNMF (Local PLMN) interacts with the DDNMF HPLMN if V-UE is a target. - DDNMF (Local PLMN) responds to the UE-1 with the authorization success. xIRI if UE-1 is a target in Local PLMN.. - V-UE and UE-1 has ProSe connection.

# Non-roaming Case

| Scenario 1 | Scenario 2 |
| --- | --- |
| - UE-1 (local PLMN) gets the code. xIRI if UE-1 is a target in Local PLMN. - UE-1 announces to the H-UE with the code. - H-UE requests the DDNMF (HPLMN) for authorization (w/ the code). - DDNMF (HPLMN) interacts with the DDNMF (local PLMN) to confirm. xIRI if UE-1 is a target in Local PLMN. - DDNMF (HPLMN) responds to the H-UE with the authorization success. xIRI if H-UE is a target in HPLMN. - H-UE and UE-1 has ProSe connection. | - UE-2 (HPLMN) gets the code from DDNMF (HPLMN). xIRI if UE-2 is a target in Local PLMN (shown as HPLMN). - UE-2 announces to the H-UE with the code. - H-UE requests the DDNMF (HPLMN) for authorization (w/ the code). - DDNMF (HPLMN) responds to the H-UE with the authorization success. xIRI if H-UE is a target in HPLMN. - V-UE and UE-2 has ProSe connection. |