Source:	Dolby Laboratories, Inc.

Title:	On test material creation for P.800 based IVAS selection tests

Document for:	Discussion

Agenda Item:	1.3

# Introduction

The Audio SWG call on Dec 16, 2022, discussed and agreed steps for critical test material collection for Mushra tests. These steps are included in square brackets in the latest draft of the IVAS selection test plan [1]. For P.800 tests, the test material generation will be different since these tests are voice centric and involve naïve listeners. This requires that the test items are available in the native language of the listeners. Contribution [2] discussed this topic in detail and concluded that there are at least three approaches for test material generation, which are a) in-situ approach, b) simplified in-situ approach and c) model-based approach. Aspects of these approaches are also discussed in [3].

In this contribution, the source recites the relevant portions of [2] and offers further details of the model-based approach. The purpose is to give the group a better understanding of the usefulness of that approach.

# Test material generation for speech transmission quality tests with naïve listeners (from [2] with minor adaptations)

Speech transmission quality tests with naïve listeners typically require that the tests are carried out in the native language of the listeners. This is the main reason why the preparation of such tests has a different workflow. Here are the well-established essential steps:

- Requirement definition of the test items (typically as part of test plan)

SA4 to set requirements, e.g. in terms of addressed IVAS use case(s), audio formats, audio content, etc.

- Listening lab action 1

The listening lab generates/collects a set of raw speech materials (clean speech sentences in the native language of the listeners) meeting the test plan requirements and provides it to the processing lab.

- Processing lab action

The processing lab processes the raw speech materials (potentially along with other raw materials (e.g. background noise) obtained according to a material collection process similar to the process used for Mushra test material collection) through all experimental conditions.

- Listening lab action

The listening lab receives the processed test material and starts testing.

It is notable that this workflow has been applied in most 3GPP or ITU-T speech codec standardization efforts. However, whether and how it can also be adopted for IVAS standardization requires a deeper look. One important aspect in earlier efforts was that listening lab action 1 required very little efforts for a listening lab. A lab could either use pre-recorded sentences from earlier exercises or from a commercial speech database, or at maximum do own recordings of clean speech items. The latter is relatively easy following the guidelines of the ITU-T handbook [4] and ITU-T recommendation P.800 [5].

For IVAS, however, the situation is less straight forward. Speech material items to be coded with an IVAS candidate codec (and references) will not be representable by a single mono signal but will rather be immersive or at least stereo signals. To obtain such items, the following approaches are conceivable:

- In-situ approach

The listening labs would do all stereo/immersive capture by themselves, following a detailed test specification. Assuming for instance a listening test with conditions reflecting a use case with audio capture in a conference room, the labs would firstly have to find/setup a conference room according to test plan requirements, then placing talkers (humans) at the designated positions in that room and letting them speak the relevant sentences while doing spatial sound recordings of the audio at designated microphone positions, with well-specified microphones. If the test should involve an immersive sound background with, e.g., environmental sounds or music, the labs might additionally be required to make the corresponding arrangements.

This example makes clear that while such in-situ recordings may allow obtaining realistic original test materials, this approach is generally a significant effort which may in many cases be impractical, too costly, and may go far beyond the capabilities of a listening lab both in terms of expertise and the needed equipment. Repeatability and reproducibility would be a major problem since it would cause severe practical problems to repeat an equivalent recording at a different site, for instance if the equivalent material would be needed by another listening lab running the same experiment in another language. The reason is the practical difficulty to specify/document all relevant details of the capture environment.

- Simplified in-situ approach

A simplification of the in-situ approach would be to have a particular processing lab that would convert mono sentences received from the different listening labs to stereo/immersive items by placing a loudspeaker at the designated talker positions in the specified rooms, playing back the sentences while doing the specified stereo/immersive recordings. However, this would remain a significant effort and good repeatability/reproducibility might still be questionable because of the practical difficulty to specify/document all details of the capture environment. Also, SA4 would have very limited possibilities to control the correctness of the processing.

- Model-based approach

The model-based approach in contrast aims at obtaining stereo/immersive audio samples starting from mono input samples and a mathematical model of the relevant acoustical environment. This approach allows listening labs to use their own mono speech material as in past exercises and moves all signal processing from the mono speech sample to the IVAS candidate codec input item to the preprocessing stage of the experimental processing.

The essential steps of this model-based approach are the following:

- Start from single speech sentence samples (e.g. Harvard sentences) recorded in dry and clean environment obtained from the respective listening lab, following suitable guidelines of e.g. [4], [5].

- Have stereo/spatial room impulse responses (SRIR) respective

    - the room/environment,

    - the relevant talker positions,

    - the relevant stereo/spatial microphone positions and orientations.

- Filter (convolve) sentence samples with SRIR, then do loudness normalization.

- Combine with another stereo/spatially filtered sentence to obtain a sentence pair.

- Add stereo/spatial noise respective the room/environment with predetermined loudness level.

It is to be noted that the required SRIR would have to be obtained well in advance, e.g., from volunteering organizations. These SRIRs would have to meet certain requirements defined in the test plan. Having them well in advance would SA4 give good possibilities to verify their correctness and their meeting of the requirements.

Comparing the three approaches, the source believes that the in-situ approach may generate the most realistic stereo/immersive original test material. Unfortunately, it is also most impractical. The remaining simplified in-situ and the model-based approach may introduce a certain lack of realism mainly resulting from the fact that a human speaker will be replaced by a loudspeaker. Both lead to similar results in terms of realism if SRIRs can be recorded with high quality. The source has good experience with SRIR recordings and thus believes that once the SRIRs are available, the model-based approach is preferable due to practical advantages as well as good repeatability/reproducibility and controllability.

# Example: SRIR recording in conference room

Figure 1 below depicts a conference room in which the source carried out SRIR recordings. The conference room is of medium-large size.

The circles with numbers show the 6 different sound source positions while the microphone symbol containing the ‘Z’ indicates the location of the used Zoom H3-VR FOA microphone.

The room contains a few curtains to avoid too strong reverb, however, it can still be characterized as somewhat reverberant.

Figure 1: Room map of conference room

The table below gives details of the relative position of the source locations to the microphone location. Note that in all cases, the vertical coordinate of the source and the microphone locations was identical (1m).

Table 1: Relative positions of sound sourced to microphone

The SRIR measured for these positions were truncated (in time) to contain just the relevant portions and to avoid maintaining portions that would otherwise contribute to an increased noise floor. It may be noted that noise may be an issue when recording spatial room impulse responses. However, the effect of such a noise floor is typically mitigated when combining the filtered clean speech samples with realistic immersive ambient noises.

In the work by the source, the SRIRs were generated in B-format domain. Consequently, they are directly applicable for filtering of a mono input source to generate corresponding B-formatted FOA audio. An example of the set of 4 impulse responses for position 1 is shown in the following figure.

Figure 2: Spatial Room Impulse responses for position 1

To give interested parties the possibility to study the described model-based approach further, the source provides attached to this contribution the set SRIRs for the 6 positions.

# Conclusion

This contribution discussed the topic of test material creation for P.800 based IVAS codec selection tests. Practical details of the model-based approach relying on filtering of mono sentences with SRIR are provided. The source invites interested parties to investigate that approach allowing the group to come to a timely decision if it should be adopted in the IVAS codec selection tests.

It is further proposed to discuss whether the general workflow of the well-established essential steps (beginning of section 2) should be added to the IVAS codec selection test plan.

# References

[1]	Tdoc S4aA230005: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.0.5.1, Editor (VoiceAge)

[2]	Tdoc S4-210564: Workflows for the creation of IVAS test materials, Dolby Laboratories, Inc.

[3]	Tdoc S4aA230008: On sound material generation and lab responsibilities, Nokia Corporation, Dolby Laboratories, Inc.

[4]	ITU-T Handbook of subjective testing practical procedures

[5]	ITU–T Recommendation P.800 (1996): Methods for subjective determination of transmission quality