Source:	Orange

Title:	Follow-up on profiles for JBM behaviour evaluation

## Document for:	Discussion

## Agenda Item:	1.5

- Introduction

In this Tdoc we clarify one table in [1] and described planned experiments on JBM testing in the scope of the eUET work item [2].

- Clarification on data collected in drive tests

For each quality class defined in [1] the following statistics (taken from first three column of Table 1 in [1]) were obtained:

| Class | % of cases with packet loss (on active speech) | Average loss rate |
| --- | --- | --- |
| Perfect | 19.7% | 0.6% |
| Near-Perfect | 80.2% | 1% |
| Average | 79.2% | 3% |
| Bad | 100% | 25.1% |

This table indicates that 19.7% of cases measured in class ‘Perfect’ had packet losses (on active speech). When there are packet losses, the average loss rate is 0.6% (on 19.7% cases).

We repeat below Table 2 from [1]:

Table 2: Statistics on burst lengths

| Class | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | >10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Perfect | 1.468 | 0.145 | 0.010 | 0.004 | 0.001 | 0 | 0 | >0.001 | 0 | 0 | >0.001 |
| Near-Perfect | 1.547 | 0.301 | 0.026 | 0.011 | 0.006 | 0.006 | 0.002 | 0.001 | 0 | 0.002 | >0.001 |
| Avg. | 1.679 | 0.594 | 0.089 | 0.067 | 0.018 | 0.009 | 0.015 | 0.017 | 0.006 | 0.018 | 0.012 |
| Bad | 10.974 | 3.105 | 0.952 | 0.319 | 0.135 | 0.083 | 0.070 | 0.096 | 0.096 | 0.061 | 0.886 |

For instance in the row corresponding to ‘Perfect’, when there are packet losses we lose on average 1,468 packet (single loss) and 0,145 packet pairs, etc. One may find the overall number of lost packets by summing : 1,468 * 1 + 0,145 * 2 + 0,010 * 3 + … which would give an overall average of 1.52 lost packets.

In any case the data presented in [1] show that in real networks one can observe all types of impairments (loss, order inversion, duplication and delay). For packet losses the burst length was found to be as shown in Table 2 of [1], with isolated losses as well as consecutive packets being lost.

Remarks:

- On packet inversions, similar statistics on packet permutations were obtained, and it was observed that the only situation is to have packet n+1 arriving before packet n (and never packet n+2, n+3, etc. arriving before packet n).

- Similarly, on duplications, we could see only the case of one duplication, there were no cases with two, three or more duplications.

- Planned experiments

We already reported in [1] test results. A methodology for VoLTE realistic profile construction is detailed in [1] (with three sub-profiles: Profiles 1, 2 , 3) and is used to analyze JBM behaviour. Two test cases were reported (with MOS-LQO histograms):

- Simulation based on the reference EVS decoder (with rtpdump support)

- Measurements using an example DUT with the PCAP player feature of Rhode&Schwarz CMW500 equipment (with an RTP stream pre-recorded with packet impairments according to the given profiles).

We are planning to compare these results with measurements conducted with HEAD acoustics LabCore. The test setup is based on TCN files. There might be limitations with the underlying netem functionality to be discussed, for instance it may not be possible to have duplicated or inverted packets.

## References

- S4-221445, On profiles for JBM behaviour evaluation, Source: Orange

- SP-220610, New WID on Enhancements to UE Testing, Source: Orange, HEAD acoustics GmbH, Fraunhofer IIS, ROHDE & SCHWARZ, Amazon