SA4-e (AH) Audio SWG post 122-e						    S4aA230031

17th March 2023

Source:	Fraunhofer IIS

Title:	On Preprocessing Steps

Document for:	Discussion and Agreement

Agenda Item:	1.3

- Introduction

The IVAS Processing Plan [1] is not yet finalized. This contribution focuses on the preprocessing part and questions the need for the high-pass filtering in the pre-processing for all test setups.

- Discussion

The IVAS Processing Plan v0.8.0 currently lists the following processing steps for the pre-processing in Section 4.2:

“

## Pre-processing

### Pre-processing stages

Figure 1: Common pre-processing for input audio

### High-pass filtering

Editor’s note: TBD

### Level adjustment

Figure 2: Level adjustment using BS.1770

“

In the course of  IVAS-7a, Sections 4.4 – 4.9, the identical pre-processing is assumed to be used for all input formats and presumably across all operating points.

The source would like to share some concerns on these assumptions:

- The high-pass filtering was initially introduced to simulate the send mask characteristics of UEs. This assumes that the content fed to the IVAS codec is purely generated via UE audio pick-up, where these masks are used. However, generic immersive audio content may also consist of produced content, where such masks are in general not foreseen. The source sees that especially at the “Generic immersive audio use case scenario” [2] such produced content plays a significant role in practice and is also considered as such for the selection testing. By forcing the high-pass filter for the selection testing on all content, the test results might not reflect the actual usage of the codec in the field.

- The processing plan also covers multi-channel audio input. An important component of multi-channel audio input is the LFE channel, which typically covers frequencies < 120Hz. Applying the high-pass filter to this LFE content will remove a significant part of the content and severely impact the scene. Currently, the testing of multi-channel audio input is only foreseen in the Generic immersive audio use case scenario using BS.1534 (MUSHRA).

- Proposed Changes for Inclusion in IVAS-7a

The source proposes to implement the following changes in IVAS-7a:

- Make the use of the high-pass filter in the pre-processing optional:

  - High-pass filter should be included for the experiments designed to evaluate the Immersive conversation use case scenario using P.SUPPL800

  - High-pass filter should be skipped for the experiments designed to evaluate the Generic immersive audio use case scenario using BS.1534 (MUSHRA)

- Conclusion

The source proposes to implement the principle as outlined in Section 3 of this document in IVAS-7a.

References:

[1] S4-230367 - IVAS Permanent Document IVAS-7a: Processing Plan for Selection Phase v0.8.0

[2] S4-230367 - IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase v0.7.0