Source:	Huawei Technologies Co., Ltd

Title:	Proposed test plan for IVAS selection experiment - BS1534-7a&7b for MASA

Document for:	Discussion and Agreement

Agenda Item:	1.4

# 1. Introduction

This document describes the BS.1534 [1] MUSHRA testing plan of MASA inputs for IVAS Codec selection test experiments.

# 2. Overview

Based on the document IVAS Test Plan for Selection Phase [2], there are two MUSHRA tests for MASA inputs, BS1534-7a and BS1534-7b.

| Exp | Input format | Source material | Listening environment | Bitrates kbps | FER/jitter | DTX | Headtracking |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS1534-7a | MASA | Generic Audio | Headphones |  | ≤ x% | N |  |
| BS1534-7b | MASA | Generic Audio | Headphones |  | ≤ x% | N |  |

# 3. Proposals for experiment conditions

- It is proposed to focus on stereo-MASA in IVAS Codec selection.

- P800-8 and P800-9 cover the bitrates from 13.2 kbps to 96 kbps for MASA testing. The source materials are limited to speech and speech with background noise. Generic Audio is planned to be tested in BS1534-7a and BS1534-7b. The bitrate should be considered covering all related range.

- BS1534-7a focuses on lower bitrates (13.2 kbps to 96 kbps) and BS1534-7b focuses on higher bitrates (128 kbps to 512 kbps).

- Each of BS1534-7a and BS1534-7b selects one bitrate for IVAS FER tests.

# 4. Proposals for specific experiments

## 4.1 BS1534-7a (MASA – Generic Audio, Binaural, lower bitrates)

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 2 | IVAS candidate operated at 64, 80 kbps |
|  | 1 | IVAS candidate operated at 48 kbps with DTX off at 3% or 5% FER |
| Codec references |  |  |
| Codec references | 1 | Multi-mono EVS operated at 4*24.4 kbps with DTX off at 0% FER |
|  | 1 | Multi-mono EVS with unquantized metadata operated at 2*24.4 kbps with DTX off at 0% FER |
|  | 1 | Multi-mono EVS operated at 4*16.4 kbps with DTX off at 3% or 5% FER |
| Other references |  |  |
| Reference / Direct | 1 | Direct MASA signal(s), nominal input level |
| Hidden Reference | 1 | Direct MASA signal(s), nominal input level |
| LP7 anchor | 1 | 7kHz lowpass filtered direct MASA signal(s), nominal level |
| Common Conditions |  |  |
| Test item generation | 1 | According to material collection procedure for IVAS selection MUSHRA tests |
| Binaural rendering | 1 | MASA to binaural (external) rendering according to [tbd] |
| Audio sampling frequency / bandwidth | 1 | 48 kHz / FB |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | Adjusted by listener |
| Listeners | [12-16] | Experienced Listeners |
| Randomizations | [12-16] | Individual per listener |
| Rating Scale | 1 | Continuous MUSHRA scale: 0-100 [ITU-R BS.1534] |
| Listening System | 1 | High-quality headphones, diotic presentation |
| Listening Environment | 1 | No noise |

## 4.2 BS1534-7b (MASA – Generic Audio, Binaural, higher bitrates)

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 2 | IVAS candidate operated at 160, 256 kbps with DTX off at 0% FER |
|  | 1 | IVAS candidate operated at 96 kbps with DTX off at 3% FER |
| Codec references |  |  |
| Codec references | 2 | Multi-mono EVS operated at 4*48, 4*96 kbps with DTX off at 0% FER |
|  | 2 | Multi-mono EVS with unquantized metadata operated at 2*48, 2*96 kbps with DTX off at 0% FER |
|  | 1 | Multi-mono EVS operated at 4*32 kbps with DTX off at 3% FER |
| Other references |  |  |
| Reference / Direct | 1 | Direct MASA signal(s), nominal input level |
| Hidden Reference | 1 | Direct MASA signal(s), nominal input level |
| LP7 anchor | 1 | 7kHz lowpass filtered direct MASA signal(s), nominal level |
| Common Conditions |  | (see BS1534-7a for full list) |

# 5. Summary

We recommend to take into account the proposed test method for MASA in the IVAS Codec selection Test plan.

# References

[1]	ITU-R BS.1534-3 (10/2015) – Method for the subjective assessment of intermediate quality level of audio systems

[2]	SA4-230453: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.0.7.4