Source:	Mesaqin.com s.r.o (Ltd.), FORCE Technology, HEAD acoustics GmbH, Macquarie University

Title:	On details of speech samples to be delivered by LLs

Document for:	Agreement

# Introduction

In the Test Plan S4aA230063-IVAS-8a_TestPlan_v.0.8.3, the following details are proposed for discussion:

“[The recording SNR should be no worse than [60] dB, the leading and trailing inactivity portions should be shorter than [20] ms. The reverberation time RT60 should be less than [200] ms. The length of the sentences should typically correspond to the length of traditional Harvard sentences [29].]”

The sources would like to propose modifications to better align the test conditions with available materials, resources and procedures.

# Proposal

Sources will provide speech samples that confirm to P.800 requirements. Therefore, the following wording is proposed:

The recording SNR should be in accordance with P.800 at least 40dB but preferably 50dB or higher. The leading and trailing inactivity portions will be minimum [500] ms. The overall sample length should be [4]s. This comprises leading and trailing silent sequences and the speech sentence duration. The reverberation time RT60 should be in accordance to P.800 less than 500ms, preferably below 200ms. The length of the sentences should typically correspond to the length of traditional Harvard sentences [29].

The sources believe the fixed sample duration is essential for proper sample automized processing and that the originally mentioned minimized trailing sequence (suggested up to 20ms) refers to the generation of simulated immersive material via the processing scripts equipped by VAD.