Source:	Editor1

Title:	IVAS-7a: Processing plan for selection phase

Version:	v.0.9.1

Agenda Item:	1.4

Document History:

| Version | Date | Detail of changes |
| --- | --- | --- |
| 0.1.0 | 25 Oct 2019 | Initial version |
| 0.1.1 | 22 Jan 2020 | Implementation of S4-200119 according to agreements in EVS SWG |
| 0.2.0 | 5 Feb 2021 | Changed scope to selection phase |
| 0.3.0 | 23 Feb 2022 | Implementation of S4-220164, S4-220165, S4-220166 with agreed amendments of the EVS SWG at SA4#117-e. |
| 0.4.0 | 13 Apr 2022 | Implementation of S4-220432 |
| 0.5.0 | 13 Apr 2022 | Agreement of clause 3.5-3.7 |
| 0.6.0 | 16 May 2022 | Implementation of S4-220733 |
| 0.7.0 | 17 Nov 2022 | Implementation of S4-221464 |
| 0.8.0 | 23 Feb 2023 | Agreements based on S4-230221, S4-230239, S4-230261, S4-230262 implemented at SA4#122. |
| 0.8.1 | 3 Apr 2023 | Agreements based on S4aA220031. |
| 0.9.0 | 20 Apr 2023 | Implementation of S4-230579. |
| 0.9.1 | 3 May 2023 | Agreements based on S4aA230058, S4aA230059, S4aA230060. Editorial updates. |

CONTENTS

3.1	Acronyms	4

3.2	Definitions	4

3.3	Experiments	5

3.4	Listening lab designators	5

3.5	Bitstream format	6

3.6	Audio format	6

3.7	Audio track designators	6

3.8	Audio track configurations	7

3.9	Preamble definition	8

3.10	File naming	8

3.10.1	Generic definitions	8

3.10.2	Naming examples of the input files and processed output files	9

3.10.3	File naming error patterns	10

3.10.4	File naming jitter profiles	11

3.10.5	File naming jitter derived error patterns	11

3.10.6	File naming rate switching profiles	11

4.1	General considerations for processing	11

4.1.1	Source material requirements	11

4.1.2	Concatenated sequences processing	11

4.1.3	Frame error application	12

4.1.4	Concatenation setup	12

4.2	Generation of test samples	13

4.2.1	P.800 speech samples	13

4.3	Pre-processing	13

4.3.1	Pre-processing stages	13

4.3.2	High-pass filtering	13

4.3.3	Level adjustment	14

4.4	Processing for anchor conditions	14

4.4.1	MNRU/ESDRU	14

4.5	Processing for stereo inputs	14

4.5.1	Reference conditions	14

4.5.2	CuT conditions	15

4.6	Processing for binaural audio inputs	15

4.6.1	Pre-processing	15

4.6.2	Reference conditions	15

4.6.3	CuT conditions	15

4.7	Processing for multi-channel audio inputs	15

Reference conditions	15

4.7.1	CuT conditions	16

4.8	Processing for scene-based audio inputs	16

4.8.1	Reference conditions	16

4.8.2	CuT conditions	16

4.8.3	Reference codec conditions	17

4.9	Processing for metadata-assisted spatial audio (MASA) inputs	17

4.9.1	Reference conditions	17

4.9.2	CuT conditions	18

4.10	Processing for object-based audio inputs	18

4.10.1	Reference conditions	18

4.10.2	CuT conditions	19

5.1	Pre- and post-processing operations	19

5.1.1	General delay compensation for the STL filter tool	19

5.1.2	Filtering operations	19

5.1.3	Level adjustment	19

5.1.4	Scaling by Factor F	21

5.1.5	Low/High level De-/Scaling	21

5.1.6	Summation of speech and noise files	21

5.1.7	File concatenation	21

5.1.8	Audio format conversion	22

5.1.9	Sampling Rate changes (resampling)	22

5.1.10	Windowing and segmentation	23

5.1.11	Bit conversion	23

5.1.12	Interleave audio files	24

5.1.13	De-interleave audio files	24

5.1.14	Rendering to Binaural (HRIRs)	24

5.1.15	Binaural Room (BRIRs)	25

5.1.16	Loudspeaker	26

5.2	Processing	27

5.2.1	MNRU reference conditions	27

5.2.2	ESDRU reference conditions	27

5.2.3	Reference codecs for Narrowband conditions	27

5.2.4	Reference codecs for Wideband conditions	28

5.2.5	Reference codecs for super-wideband conditions	29

5.2.6	Delay compensation for reference conditions	30

5.2.7	EVS operation (non-JBM case)	30

5.2.8	EVS operation (JBM case)	30

5.2.9	CuT operation (non-JBM case)	31

5.2.10	CuT operation (JBM case)	31

5.3	Encoder and decoder CuT executable requirements	31

5.4	Error Insertion (EID)	31

5.4.1	Frame error tool	31

5.4.2	Pattern generation	32

5.4.3	Derive EPFs from jitter profiles	32

5.4.4	Rate switching profile generation	32

5.4.5	Network simulator for packet jitter generation	33

5.4.6	Cutting Tool for JBM tests	33

A.1	Delay compensation for filter operations and reference codecs	34

A.2	Binaries used	35

A.2.1	ITU-T STL processing tools	35

A.2.2	AMR Error insertion device	35

A.2.3	Reference codecs	36

A.2.4	Other tools	38

B.1	Databases for objective evaluation	41

B.2	Active Frame Ratio (AFR)	41

B.3	Gain measuring of active frames and inactive frames	42

B.4	JBM performance evaluation	42

B.5	Bitrate evaluation	42

C.1	Delay and error profile offset	43

C.2	Seeds for rate switching	43

C.3	Initialization of gen-patt tool	43

C.4	Random noise file selection	44

[	44

C.4.1	Overview	44

C.4.2	Background noise file offset	44

D.1.1	Delay Jitter	45

D.1.2	Error burstiness	45

# Introduction

This document defines how audio material must be prepared for the selection tests of the 3GPP IVAS codec. It should be read in conjunction with its associated IVAS selection phase test plan [1].

Editor’s Note: The processing modules need to be reviewed for processing beyond mono.

Editor’s Note: The following main remaining work has been identified:

- The permanent document lacks the definition of the following items/processing steps:

  - Pre-processing:

    - Filtering operation: Definition of filters (HP50_SHQ filters were used in EVS but are not part of STL2022) -> filters are available, just need to decide

  - Post-processing:

    - Clarification whether additional post-processing steps are needed, e.g. additional level/loudness adjustment -> post normalization still discussed, VoiceAge aims to bring input

  - Processing for test item generation for the immersive conversation use-case -> major missing item, see comments below

    - Stereo/binaural -> only SWB impulse responses available (ITU-T)

    - SBA impulse responses -> examples from Dolby, HEAD Acoustics

    - Objects -> pretty straight forward

    - MASA -> using ambisonics impulse responses as a backup plan (both FOA and HOA2 would be beneficial)

  - Collection of data base for objective Performance Requirements -> major missing item

    - Would parties be able to contribute? Under what conditions?

      - Legal framework might be needed

  - Collection of background material -> major missing item

  - Generation of Speech+Background material

    - Decide on e.g., SNR

  - Random selection of test material for Generic Audio, mixed/music? -> unclear whether material from labs could be used, or random selection from a pool of collected material would be needed

  - Processing for objective Performance Requirements -> missing, minor issue

  - Assess whether the specified version of G.191 STL (07/22) is appropriate or whether an updated version is required

- A set of corresponding processing scripts which implements the agreed processing steps -> major missing item, a proposed way forward was agreed in the Audio SWG but there are some hurdles.

# Responsibilities

The host lab (HL) [TBD] is responsible for the processing of the experiments while the cross-check lab (CL) [TBD] verifies the correctness of the processing.

The listening tests will be conducted by the listening labs (LL): TBD.

Editor’s Note: We need to define who will develop processing scripts. For EVS, one set of processing scripts was developed by the host lab, and another set by proponent companies to be used by the cross-check lab.

# Definitions and formats

## Acronyms

CL	Cross-check Laboratory

CuT	Codec under Test

DCR	Degradation Category Rating

DTX	Discontinuous transmission

ESDRU	Energy-based Spatial Distortion Reference Unit

EVS	Enhanced Voice Services

FB	Fullband

FE	Frame Erasure

FOA	First-Order Ambisonics

GAL	Global Analysis Laboratory

HL	Host Laboratory

HOA3	Higher-Order Ambisonics, 3rd order

IVAS	Immersive Voice and Audio Services

LKFS	Loudness, K-weighted, relative to Full Scale

LL	Listening Laboratory

MASA	Metadata-Assisted Spatial Audio

MNRU	Modulated Noise Reference Unit

MUSHRA	Multi Stimulus test with Hidden Reference and Anchor

NB	Narrowband

SWB	Super-wideband

WB	Wideband

## Definitions

Anchor condition	Controlled impairment, e.g., using MNRU, ESDRU, low-pass filtered signal

CuT condition	Codec under test processed for testing

Reference condition 	Quality target for comparative tests, sometimes also called Direct condition

Reference codec condition	Quality comparison to reference codec, e.g. EVS, for evaluation of performance requirements

## Experiments

The selection test plan‎ (IVAS-8a) [1] describes the listening experiments. For the processing of each experiment, a two-letter experiment designator shall be used for the file naming convention. An overview of all experiments is presented in Table 1.

Table 1: Overview of experiments

| Experiment Designator |  | Experiment Name (IVAS-8a) |  | Input format |  | Source Material |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| p01 |  | P800-1 |  | Stereo |  | Clean speech |  |
| p02 | P800-2 | Stereo | Speech+Background |
| p03 | P800-3 | Stereo | Mixed & Music |
| p04 | P800-4 | FOA | Clean speech |
| p05 | P800-5 | FOA | Speech+Background |
| p06 | P800-6 | 1 Object | Clean speech |
| p07 | P800-7 | 2 Objects | Clean speech |
| p08 | P800-8 | MASA | Clean speech |
| p09 | P800-9 | MASA | Speech+Background |
| m1a | BS1534-1a | Stereo | Generic Audio |
| m1b | BS1534-1b | Stereo | Generic Audio |
| m2a | BS1534-2a | 5.1 | Generic Audio |
| m2b | BS1534-2b | 5.1 | Generic Audio |
| m3a | BS1534-3a | 7.1.4 | Generic Audio |
| m3b | BS1534-3b | 7.1.4 | Generic Audio |
| m4a | BS1534-4a | FOA | Generic Audio |
| m4b | BS1534-4b | FOA | Generic Audio |
| m5a | BS1534-5a | HOA3 | Generic Audio |
| m5b | BS1534-5b | HOA3 | Generic Audio |
| m6a | BS1534-6a | Objects | Generic Audio |
| m6b | BS1534-6b | Objects | Generic Audio |
| m7a | BS1534-7a | MASA | Generic Audio |
| m7b | BS1534-7b | MASA | Generic Audio |

## Listening lab designators

The following table lists the responsible listening labs (LL) with their dedicated LL designator to be used for the file naming convention.

Table 2: List of listening lab designators

| LL designator l | Listening Lab Company |
| --- | --- |
| a | Force Technology |
| b | Head Acoustics/ IKS |
| c | MQ University |
| d | Mesaqin.com |

## Bitstream format

The candidate codec shall be based on ITU-T G.192 [2] format as a common bit-stream interface.

## Audio format

The format of input/output audio files is WAVE, 16-bit little endian format. For multi-track audio, the audio tracks are ordered according to Table 5.

## Audio track designators

Audio tracks are designated according to Table 3 with further specification according to Table 4.

Table 3: Track designators

| Track designator | Definition |  |
| --- | --- | --- |
| n | Index: 1, 2, … |
| M | Mono channel |  |
| L | Left channel |  |
| R | Right channel |  |
| CH_Aa_Ee | Channel at nominal azimuth angle a and nominal elevation angle e |  |
| LFE | LFE channel |  |

Table 4: Channel specification

| Specification variable | Definition |
| --- | --- |
| a | Nominal azimuth angle in range: ±180 degrees Represented by three-digit number always including a sign (padded from the left with zeros if necessary). Positive values indicate positions left of the frontal direction. The sign assigned to azimuth value zero shall be +. |
| e | Nominal elevation angle in range: ± 90 degrees Represented by two-digit number always including a sign (padded from the left with zeros if necessary). Positive values indicate positions above the horizontal plane. The sign assigned to elevation value zero shall be +. |

## Audio track configurations

Input/output audio shall follow configurations as specified in Table 5. Ambisonics components follow the ACN ordering where ACNindex = n2+n+m for real-valued spherical harmonics components of order n and degree m=0, 1, … , where m=[-n,…,n].

Table 5: Audio track configurations

| Audio format (designator) | Number of tracks | Index | Configuration (incl. ordering) | Azimuth Range | Elevation Range |
| --- | --- | --- | --- | --- | --- |
| Mono (M) | 1 | 1 | M | - | - |
| Stereo (ST) | 2 | 1,2 | L, R | - | - |
| Binaural (BIN) | 2 | 1,2 | L, R | - | - |
| Multi-channel 5.1 (MC51) | 6 | 1 | CH_A+030_E+00 | +30 | 0 |
|  |  | 2 | CH_A-030_E+00 | -30 | 0 |
|  |  | 3 | CH_A+000_E+00 | 0 | 0 |
|  |  | 4 | LFE | - | - |
|  |  | 5 | CH_A+110_E+00 | +100 … +120 | 0 … +15 |
|  |  | 6 | CH_A-110_E+00 | -100 … -120 | 0 ... +15 |
| Multi-channel 7.1 (MC71) | 8 | 1 | CH_A+030_E+00 | +30 ... +45 | 0 |
|  |  | 2 | CH_A-030_E+00 | -30 … -45 | 0 |
|  |  | 3 | CH_A+000_E+00 | 0 | 0 |
|  |  | 4 | LFE | - | - |
|  |  | 5 | CH_A+110_E+00 | +85 … +110 | 0 |
|  |  | 6 | CH_A-110_E+00 | -85 … -110 | 0 |
|  |  | 7 | CH_A+135_E+00 | +120 … +150 | 0 |
|  |  | 8 | CH_A-135_E+00 | -120 … -150 | 0 |
| Multi-channel 5.1+4 (MC514) | 10 | 1 | CH_A+030_E+00 | +30 | 0 |
|  |  | 2 | CH_A-030_E+00 | -30 | 0 |
|  |  | 3 | CH_A+000_E+00 | 0 | 0 |
|  |  | 4 | LFE | - | - |
|  |  | 5 | CH_A+110_E+00 | +100 … +120 | 0 … +15 |
|  |  | 6 | CH_A-110_E+00 | -100 … -120 | 0 … +15 |
|  |  | 7 | CH_A+030_E+35 | +30 … +45 | +30 … +55 |
|  |  | 8 | CH_A-030_E+35 | -30 … -45 | +30 … +55 |
|  |  | 9 | CH_A+110_E+35 | +100 … +135 | +30 … +55 |
|  |  | 10 | CH_A-110_E+35 | -100 … -135 | +30 … +55 |
| Multi-channel 7.1+4 (MC714) | 12 | 1 | CH_A+030_E+00 | +30 … +45 | 0 |
|  |  | 2 | CH_A-030_E+00 | -30 … -45 | 0 |
|  |  | 3 | CH_A+000_E+00 | 0 | 0 |
|  |  | 4 | LFE | - | - |
|  |  | 5 | CH_A+135_E+00 | +120 … +150 | 0 |
|  |  | 6 | CH_A-135_E+00 | -120 … -150 | 0 |
|  |  | 7 | CH_A+090_E+00 | +85 … +110 | 0 |
|  |  | 8 | CH_A-090_E+00 | -85 … -110 | 0 |
|  |  | 9 | CH_A+030_E+35 | +30 … +45 | +30 … +55 |
|  |  | 10 | CH_A-030_E+35 | -30 … -45 | +30 … +55 |
|  |  | 11 | CH_A+135_E+35 | +100 … +150 | +30 … +55 |
|  |  | 12 | CH_A-135_E+35 | -100 … -150 | +30 … +55 |
| FOA (SBA1) | 4 | 1…4 | Ambisonics components with ACNindex 0,1,2,3 | - | - |
| HOAO* (SBAO) | O+12 | 1…O+12 | Ambisonics components with ACNindex 0,1, 2,… O+12-1 | - | - |
| Mono objects (OBA) | 1…4 | 1…4 | Object(s) with ID 1…4 | - | - |
| Metadata-assisted spatial audio, mono (MASA1) | 1 | 1 | M | - | - |
| Metadata-assisted spatial audio, stereo (MASA2) | 2 | 1,2 | L, R | - | - |

*O = Ambisonics order

## Preamble definition

A preamble shall be low-level noise obtained as a digital silence of 16-bit values with amplitudes between +4 and -4. Uncorrelated noise shall be used across audio tracks.

## File naming

### Generic definitions

The following generic filename convention is used for input/output audio files:

inputtrfs		Input to a processing module (or sequence of modules)

outputtrfs		Output from a processing module (or sequence of modules)

Table 6 specifies suffixes to be used whenever needed for differentiation and may otherwise be left out.

Table 6: File name suffixes

| Suffix | Specification |
| --- | --- |
| tr | Track designator according to Table 3 or track index according to Table 5 |
| fs | Sampling frequency in kHz being .8k, .16k, .32k or .48k |

### Naming examples of the input files and processed output files

The following sections describe the file naming conventions where all letters in blue indicate variables in the filename while black letters indicate constants.

#### P.SUPPL800 Input content files

The filenames of the input content samples are represented by:

leeeayszz.wav

The filenames of the accompanying metadata files (applicable to metadata-assisted spatial audio, object-based audio) are represented by:

leeeayszz.met for metadata-assisted spatial audio

leeeayszz.csv for object-based audio

where:

- l stands for the listening lab designator (a through d according to Table 2)

- eee stands for the experiment designator, e.g. p01 (see Table 1)

- a stands audio, and y is the per experiment category according to IVAS-8a

- s stands for sample and zz is the sample number; 01, 02, 03, 04, 05, 06, 07, where 07 is the preliminary

#### BS.1534 Input content files

The filenames of the input content samples are represented by:

leeeayszz.wav

The filenames of the accompanying metadata files (applicable to metadata-assisted spatial audio, object-based audio) are represented by:

leeeayszz.met for metadata-assisted spatial audio

leeeayszz.csv for object-based audio

where:

- l stands for the listening lab designator (a through d according to Table 2)

- eee stands for the experiment designator, e.g. m1a (see Table 1)

- a stands audio, and y is set to 1

s stands for sample and zz is the sample number; 01 … 12

[

#### Noise files

Editor’s Note: Details on noise-files missing in IVAS-8a.

The filenames of the input noise samples are represented by:

nx.wav

where:

- n stands for noise[, and x is c for car noise, s for street noise and o for office noise].

]

#### Processed P.SUPPL800 content files

The filenames of the processed content samples are represented by:

leeeayszz.cnn.wav

The filenames of the accompanying metadata files (applicable to metadata-assisted spatial audio, object-based audio) are represented by:

leeeayszz.cnn.met for metadata-assisted spatial audio

leeeayszz.cnn.csv for object-based audio

where:

- l stands for the listening lab designator (a through d according to Table 2)

- eee stands for the experiment designator, e.g. p01 (see Table 1)

- a stands audio, and y is the per experiment category according to IVAS-8a

- s stands for sample and zz is the sample number; 01, 02, 03, 04, 05, 06, 07, where 07 is the preliminary

- c stands for the condition with the number nn = 01, 02, .. as specified in IVAS-8a

#### 3.10.2.5 Processed BS.1534-4 content files

The filenames of the processed content samples are represented by:

leeeayszz.cnn.wav

The filenames of the accompanying metadata files (applicable to metadata-assisted spatial audio, object-based audio) are represented by:

leeeayszz.cnn.met for metadata-assisted spatial audio

leeeayszz.cnn.csv for object-based audio

where:

- l stands for the listening lab designator (a through d according to Table 2)

- eee stands for the experiment designator, e.g. m1a (see Table 1)

- a stands audio, and y is set to 1

- s stands for sample and zz is the sample number; 01 … 12

- c stands for the condition with the number nn = 01, 02, .. as specified in IVAS-8a

### File naming error patterns

Editor’s Note: Details on frame loss rates missing in IVAS-8a.

Frame erasure files will have the name “eeefrr.g192”, where:

- eee stands for the experiment designator, e.g. p01 (see Table 1)

- f stands for frame loss pattern file

- rr stands for the frame loss rate in per cent[, i.e. 03, 05, 06, or 10].

### File naming jitter profiles

The MTSI jitter profiles have the file names “dly_error_profile_x.dat” where x is the profile number, i.e., [1,2,3,4,5,6,7,8,9,10].

### File naming jitter derived error patterns

The error patterns which are derived from the MTSI jitter profiles have the file names “f_profile_x.g192”, where:

- f stands for frame loss pattern file

- profile_x stands for the number of the MTSI jitter profile, see 3.10.4.

[

Editor’s Note: Unclear, whether needed for selection - currently, there is no test proposed which includes rate switching.

### File naming rate switching profiles

Rate switching profiles will have the name “patterns\eeersx”, where:

- eee stands for the experiment designator, e.g. p01 (see Table 1)

- rs stands for rate switching file

- x stands for the sequence in the experiment, i.e. 1, 2, etc.

]

# Processing stages

This section defines, in the form of diagrams, the processing stages that shall be supported by the IVAS candidate codec under test.

Editor’s note: The scope of the IVAS codec selection is not yet agreed. Agreed processing stages which are not yet agreed to be in scope for the IVAS codec selection are kept within brackets.

## General considerations for processing

Editor’s Note: Section 4.1 is TBD, currently based on EVS processing plans.

[

### Source material requirements

The input source material shall follow the format specification in section 3.6.

### Concatenated sequences processing

In all Experiments, the pre-processed material will be processed in concatenated files comprising a preamble followed by the samples being evaluated. The preamble concatenation will be 10 seconds long.

Audio files are concatenated after level adjustment. The concatenated files need to be 20 ms block aligned per audio track for further processing. Therefore, the concatenated sequence needs to be padded at the end with the first samples of the preamble to reach an integer number of 20 ms blocks for each audio track.

### Frame error application

In frame erasure conditions, the erasures shall affect the same segments of speech signal for the CuT and reference codecs. This shall be done by compensating for all encoder-side delays (or for all encoder-side delays of the core layer in case of embedded codecs). The CuT encoder and decoder executables shall compensate the delay of the output files. The delay of reference codecs shall be compensated prior to the processing by the reference encoder, as specified in 5.2.6.

Note: No delay compensation is applied for jitter related delay.

Note: Exact delay compensation might not be possible.

]

### Concatenation setup

Speech and music files are concatenated after level adjustment. If the format includes metadata (metadata-assisted spatial audio, object-based audio), same concatenation is applied to the corresponding metadata-files.

Concatenation for P.SUPPL800 is carried out according to Table 7 below where all samples are concatenated from left to right and line by line. The concatenation order is therefore a1s1, a3s1, a2s1, a4s1, a5s1, a6s1, a2s2, a4s2, etc. [The final concatenated speech file results in one 336 seconds long sequence (10 sec preamble + 6x7x8 sec speech/music file).] For mixed content and music, the total length may vary. For conditions that include background noise, the speech files must be concatenated prior to the addition of the background noise which will be provided in 300s long files. (Editor’s note: If individual background noises per category are defined, the exact concatenation procedure is FFS). The starting sample (offset) of the noise file is chosen randomly as described in Annex C.4.2.

Table 7: Concatenation order of P.SUPPL800 files

| Sample | Item sequence in concatenated files |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | a1 | a3 | a2 | a4 | a5 | a6 |
| 2 | a2 | a4 | a1 | a5 | a3 | a6 |
| 3 | a1 | a6 | a3 | a5 | a2 | a4 |
| 4 | a2 | a5 | a6 | a1 | a3 | a4 |
| 5 | a3 | a2 | a1 | a6 | a5 | a4 |
| 6 | a5 | a1 | a2 | a6 | a4 | a3 |
| 7 | a6 | a2 | a1 | a5 | a3 | a4 |

For all BS.1534 Tests, the files are concatenated in ascending order (by sample number).

For all Experiments, the processed concatenated files should be divided into separate speech/music samples. This must be performed before final up-sampling stage, and a cosine (Hanning) window of duration 100ms must be applied to the start and end of the separated files. The procedure for splitting and windowing concatenated files is described in 5.1.10.

For the experiments where IVAS is tested in the context of JBM testing, 0.2s of pure digital silence will be added before and after each audio sample during concatenation.

[

## Generation of test samples

### P.800 speech samples

Except for the experiment p03, p06 and p07, the test samples for the P.800 tests are created by filtering clean mono sentences with room impulse responses generating the stereo/spatial speech samples. For some experiments the stereo/spatial speech samples are mixed with stereo/spatial background samples.

The main steps for sample creation prior to mixing with background sounds are:

- Convolve a raw mono clean speech sentence of a first talker with a set of room impulse responses respective the position of that talker relative to a capture point to get a stereo or spatial representation of that sentence.

- Level normalize first convolved sentence according to test plan requirements using the BS.1770 tool.

- Convolve a raw mono clean speech sentence of a second talker with a set of room impulse responses respective the position of that talker relative to a capture point to get a stereo or spatial representation of that sentence.

- Level normalize second convolved sentence according to test plan requirements using the BS.1770 tool.

- Superpose both level normalized sentences with defined time offset. The superposition of the level normalized sentences with defined time offset is done as follows:

  - The first sentence is inserted in an empty buffer after a leading silence period (zeros) of X ms.

  - The second sentence is added to that buffer with a time offset of Y ms.

    - Y is calculated as: Y = X + length(raw mono sentence 1) + GAP, wherein ‘GAP’ is the duration of the silence period between the two sentences as specified in the test plan. In case of overtalk, this parameter may be negative.

  - Finally, a trailing silence period of Z ms (zeros) is appended.

Note: This procedure creates a two-sentence stereo or spatialized test item of length 
X + length(raw mono sentence 1) + GAP + length(filtered sentence 2) + Z.

Editor’s Note: The parameters X, and Z would need to be defined. One suggestion, based on experiences from P.SUPPL800 tests of spatial voice, was X=200 ms and Z=1000 ms.

Editor’s Note: The generated two-sentence test sample contains leading and trailing portions of artificial silence. To produce valid test vectors even for tests with clean speech, it is recommended to add low-level noise that would represent recording noise that will be present in all practical systems.

]

[

## Pre-processing

### Pre-processing stages

Figure 1: Common pre-processing for input audio. High-pass filtering may be bypassed for certain experiments.

### High-pass filtering

Editor’s note: The use of a high-pass filter in the processing may vary depending on the experiment. In addition, the details of such high-pass filter, e.g. cut-off frequency, is for further discussion.

### Level adjustment

Figure 2: Level adjustment using BS.1770

## Processing for anchor conditions

### MNRU/ESDRU

Note: ESDRU is only available for stereo/binaural audio.

Figure 3: Processing for MNRU/ESDRU conditions. Rendering is bypassed when audio format is already stereo/binaural/7.1+4.

]

[

## Processing for stereo inputs

### Reference conditions

Level normalization for stereo is assuming (virtual) loudspeakers at ±30 deg azimuth for both loudspeaker and binaural rendering.

Figure 4: Processing for stereo reference conditions.

### CuT conditions

Figure 5: Processing for stereo CuT conditions. Error insertion / Network simulation is bypassed for error-free/non-JBM conditions.

]

[

## Processing for binaural audio inputs

### Pre-processing

Editor’s Note: TBD, likely similar to stereo pre-processing.

### Reference conditions

Editor’s Note: Resampling step of the pre-processing may depend on the experiment.

Figure 6: Processing for binaural audio reference conditions.

### CuT conditions

Figure 7: Processing for binaural audio CuT conditions. Error insertion / Network simulation is bypassed for error-free/non-JBM conditions.

]

[

## Processing for multi-channel audio inputs

### Reference conditions

Editor’s Note: Resampling step of the pre-processing may depend on the experiment.

Figure 8: Processing for multi-channel audio reference conditions

### CuT conditions

Figure 9: Processing for multi-channel audio CuT conditions

## Processing for scene-based audio inputs

### Reference conditions

Editor’s Note: Resampling step of the pre-processing may depend on the experiment.

Figure 10: Processing for scene-based audio reference conditions

### CuT conditions

Figure 11: Processing for scene-based audio CuT conditions

### Reference codec conditions

Figure 12: Processing for MASA reference codec conditions based on multi-EVS and UQ metadata

Figure 13: Processing for MASA reference codec conditions based on multi-EVS FOA. All-zero Z component is added in the cases this component is not encoded.

## Processing for metadata-assisted spatial audio (MASA) inputs

Editor’s Note: Generation of 48 kHz input signals using IVAS MASA C Reference Software is missing.

### Reference conditions

Editor’s Note: Resampling step of the pre-processing may depend on the experiment.

Figure 14: Processing for MASA reference conditions

### CuT conditions

Figure 15: Processing for MASA CuT conditions

## Processing for object-based audio inputs

### Reference conditions

Editor’s Note: Resampling step of the pre-processing may depend on the experiment.

Figure 16: Processing for object-based audio reference conditions

### CuT conditions

Figure 17: Processing for object-based audio CuT conditions

]

# Processing Modules

This section specifies the operation of pre- and post-processing modules being utilized by the processing stages.

## Pre- and post-processing operations

[

### General delay compensation for the STL filter tool

All filtering steps include a delay compensation step. For preparing the delay compensation, samples of the preamble are added to the end of the input file before applying the filter step. After completion of the filtering step, the samples are to be removed from the beginning of the filtered file.

### Filtering operations

#### HP50_48KHZ filtering

To produce a 50Hz high pass filtered 48kHz sampling file use:

filter.exe HP50_48KHZ input.48k output.48k 960

#### MSIN filtering

To produce an MSIN high pass filtered 16kHz sampling file use:

filter.exe MSIN input.16k output.16k 320

]

### Level adjustment

[

#### P.56 active speech level adjustment

To normalize the P.56 ASL of an 8-kHz sampling file to -26 dBov, use:

sv56demo.exe -lev -26 -sf 8000 input.8k output.8k 160

To normalize the P.56 ASL of a 16-kHz sampled file to -26 dBov, use:

sv56demo.exe -lev -26 -sf 16000 input.16k output.16k 320

To normalize the P.56 ASL level of a 32-kHz sampled file to -26 dBov, use:

sv56demo.exe -lev -26 -sf 32000 input.32k output.32k 640

To normalize the P.56 ASL level of a 48-kHz sampled file to -26 dBov, use:

sv56demo.exe -lev -26 -sf 48000 input.48k output.48k 960

#### RMS Level adjustment

To normalize the RMS level of an 8-kHz sampled file, use:

sv56demo.exe –rms –lev xx –sf 8000 input.8k output.8k 160

To normalize the RMS level of a 16-kHz sampled file, use:

sv56demo.exe –rms –lev xx –sf 16000 input.16k output.16k 320

To normalize the RMS level of a 32-kHz sampled file, use:

sv56demo.exe –rms –lev xx –sf 32000 input.32k output.32k 640

To normalize the RMS level of a 48-kHz sampled file, use:

sv56demo.exe –rms -lev xx -sf 48000 input.48k output.48k 960

where xx is the desired level.

#### RMS Level measuring

To measure the RMS level of a MSIN filtered noise file, use:

actlev.exe –rms –blk 160 –sf 8000 input.8k

and extract the dBov value of the long-term energy (RMS).

]

#### BS.1770 Level adjustment

To normalize the level of a 48 kHz sampled file, use:

bs1770demo.exe –nchan N –lev L -conf xxxx input.48k output.48k

where N is the number of channels, L the target level in LKFS (default: -26), and xxxx a configuration string with one value per channel specifying channel weighting according to:

x = ’1’ 	loudspeaker position within |elevation| < 30 deg, 60 deg <= |azimuth| <= 120 deg, 
(weighted by 1.41)

x = ’L’ 	LFE channel (weighted by 0)

x = ’0’ 	otherwise (weighted by 1)

To normalize stereo or binaural to -26 LKFS:

bs1770demo.exe –nchan 2 –lev -26 -conf 00 input.48k output.48k

To normalize 7.1+4 multi-channel configuration, as specified in Table 5, to -26 LKFS:

bs1770demo.exe –nchan 12 –lev -26 -conf 000L00110000 input.48k output.48k

#### BS.1770 Level measuring

To measure the level of a 48 kHz sampled file, use:

bs1770demo.exe –nchan N -conf xxxx input.48k

where N is the number of channels, and xxxx a configuration string with one value per channel specifying channel weighting according to:

x = ’1’ 	loudspeaker position within |elevation| < 30 deg, 60 deg <= |azimuth| <= 120 deg, 
(weighted by 1.41)

x = ’L’ 	LFE channel (weighted by 0)

x = ’0’ 	otherwise (weighted by 1)

[

### Scaling by Factor F

To scale a noise signal at the sampling rate 8 kHz, 16 kHz, 32 kHz or 48 kHz by the Factor F use:

scaldemo.exe -dB -gain F -bits 16 -round -nopremask -blk BBB input output

where BBB is 160 for 8 kHz sampled files, 320 for 16 kHz sampled files, 640 for 32 kHz sampled files and 960 for 48 kHz sampled files. The Factor F is to be determined as TBD.

### Low/High level De-/Scaling

In case the speech level shall be -26 dBov, this processing step shall be bypassed. Otherwise, the following gain needs to be applied to the signal:

| Speech level | Scaling | Descaling |
| --- | --- | --- |
| -16 dBov | G=10 | G=-10 |
| -36 dBov | G=-10 | G=10 |

To apply the scaling operation with the gain G, use:

scaldemo.exe -dB -gain G -bits 16 -round –nopremask -blk BBB input output

where BBB is 160 for 8 kHz sampled files, 320 for 16 kHz sampled files, 640 for 32 kHz sampled files and 960 for 48 kHz sampled files.

### Summation of speech and noise files

#### Summation of a speech and a noise file

To add files in the same sampling frequency (8, 16 or 32 kHz), e.g., to produce a 32 kHz speech file mixed with noise file called output.32k by summing a 32 kHz speech file called input.32k and a 32 kHz noise file called noise.32k, use:

oper.exe –size 0 1 input.32k + 1 noise.32k 0 output.32k BBB

where BBB is 160 for 8 kHz sampled files, 320 for 16 kHz sampled files and 640 for 32 kHz sampled files.

### File concatenation

To concatenate files, the concat command is used:

concat.exe –undo undo_concat.txt file1 [file2 file3 …] catfile

Where file1, file2, … are the files to be concatenated and catfile is the concatenated file. The undo_concat.txt contains the parameters for segmentation.

In case additional silence sections needs to be inserted before concatenation, use the following for all input files

concat.exe silence fileX silence tmp_file

copy tmp_file fileX

where the silence file contains 0.2 sec of digital silence and fileX is one of the input files.

### Audio format conversion

#### PCM to WAV conversion

To convert a 32kHz sampled PCM file to a WAV file for AMR-WB+, use:

CopyAudio.exe -F WAVE-NOEX -P integer16,0,32000,native,1,default -I “” input.32k output_32k.wav

#### WAV to PCM conversion

To convert a WAV file to a 32kHz sampled PCM file for AMR-WB+, use:

CopyAudio.exe -F noheader -D integer16 input_32k.wav output.32k

### Sampling Rate changes (resampling)

#### Rate-change from 48- to 8-kHz sampling

To produce an 8-kHz sampling file from a 48-kHz sampling file, use:

filter.exe –down SHQ3 input.48k tmp.16k 960

filter.exe –down SHQ2 tmp.16k output.8k 320

#### Rate-change from 48- to 16-kHz sampling

To produce a 16-kHz sampling file from a 48-kHz sampling file, use:

filter.exe –down SHQ3 input.48k output.16k 960

#### Rate-change from 48- to 32-kHz sampling

To produce a 32 kHz sampling file from a 48 kHz sampling file, use:

filter.exe –up SHQ2 input.48k tmp.96k 960

filter.exe –down SHQ3 tmp.96k output.32k 1920

#### Rate-change from 8- to 48-kHz sampling

To produce a 48 kHz sampling file from an 8 kHz sampling file, use:

filter.exe –up SHQ2 input.8k tmp.16k 160

filter.exe –up SHQ3 tmp.16k output.48k 320

#### Rate-change from 16- to 48-kHz sampling

To produce a 48 kHz sampling file from a 16 kHz sampling file, use:

filter.exe –up SHQ3 input.16k output.48k 320

#### Rate-change from 32- to 48-kHz sampling

To produce a 48 kHz sampling file from a 32 kHz sampling file, use:

filter.exe –up SHQ3 input.32k tmp.96k 640

filter.exe –down SHQ2 tmp.96k output.48k 1920

#### Rate-change from 32- to 16-kHz sampling

To produce a 16 kHz sampling file from a 32 kHz sampling file, use:

filter.exe –down SHQ2 input.32k output.16k 640

#### Rate-change from 16- to 32-kHz sampling

To produce a 32 kHz sampling file from a 16 kHz sampling file, use:

filter.exe –up SHQ2 input.16k output.32k 320

#### Rate-change from 16- to 8-kHz sampling

To produce an 8 kHz sampling file from a 16 kHz sampling file, use:

filter.exe –down SHQ2 input.16k output.8k 320

### Windowing and segmentation

#### Segmentation for NB conditions

To extract an m sample long file beginning at sample s from an 8 kHz single channel concatenated file, use:

astrip.exe –sample –smooth –wlen 800 –start s -n m input.8k output.8k

#### Segmentation for WB conditions

To extract an m sample long file beginning at sample s from a 16 kHz single channel concatenated file, use:

astrip.exe –sample –smooth –wlen 1600 –start s -n m input.16k output.16k

#### Segmentation for SWB conditions

To extract an m sample long file beginning at sample s from a 32 kHz single channel concatenated file, use:

astrip.exe –sample –smooth –wlen 3200 –start s -n m input.32k output.32k

#### Initial windowing

To apply the initial windowing of a 48 kHz input speech or music file, use:

astrip.exe –sample –smooth –wlen 4800 –start s -n m input.48k output.48k

### Bit conversion

#### Conversion from 16 to 13 bits

To convert an 8kHz narrowband signal from a 16-bit representation to a 13-bit representation including rounding, use:

scaldemo.exe -lin -gain 1 -bits 13 -round -nopremask -blk 160 input.8k output.8k

#### Conversion from 16 to 14 bits

To convert a 16kHz wideband signal from a 16-bit representation to a 14-bit representation including rounding, use:

scaldemo.exe -lin -gain 1 -bits 14 -round -nopremask -blk 320 input.16k output.16k

### Interleave audio files

#### Interleave stereo files

To interleave stereo files, use:

stereoop.exe -interleave inputL inputR output

### De-interleave audio files

#### De-interleave stereo files

To de-interleave stereo files, use:

stereoop.exe -split input outputL outputR

]

### Rendering to Binaural (HRIRs)

#### Input format Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), surround + height (5.1+4 and 7.1+4)

For surround (5.1, 7.1) and surround + height (5.1+4, 7.1+4) input formats, direct convolution using the Default HRIR set as specified in [5], Annex B.1 is used. The subset of the HRIRs corresponding to the loudspeaker positions is selected.

Binaural rendering of stereo is implemented as a pass-through. Mono input is first rendered to stereo and then considered as binaural.

Example commandline (5.1 Input Format):

python3 -m ivas_processing_scripts.audiotools -if 5_1 -of BINAURAL
-i path/to/input_file -o path/to/output_file

#### Input format Binaural audio

For binaural audio as input and output format, no extra rendering is applied (pass-through).

#### Input format Scene-based audio (Ambisonics): FOA, HOA2 and HOA3

A computationally efficient and therefore preferred signal domain for binaural rendering of scene-based audio is the spherical harmonics domain. This relies on the HRIRs transformed from time to spherical harmonics domain in which the Ambisonics component signals can directly be convolved with the transformed HRIRs. There are various techniques for the transformation that all rely on least square optimization techniques under certain optimization constraints. During the IVAS codec Public Collaboration, several candidate techniques were studied in terms of their quality performance to keep a good balance between spatial resolution of especially higher order Ambisonics and low degree of timbral artifacts. Based on that study, the sources suggest adopting sets of transformed HRIRs that have been obtained from the default HRIRs (See Annex B of Pdoc IVAS-4) using a technique developed by one of them (Dolby). A specification of the transformation method will be made available as part of the IVAS codec deliverables.

Specifically, the sets of HRIRs in spherical harmonics domain are unique for each Ambisonics order. Their length is 128 samples. The transformed HRIRs will be made available along with the reference renderer.

Example commandline (HOA3 input format):

python3 -m ivas_processing_scripts.audiotools -if HOA3 -of BINAURAL
-i path/to/input_file -o path/to/output_file

#### Input format Metadata-assisted spatial audio

The rendering of Metadata-assisted spatial audio (MASA) is performed using the masaRenderer as provided in [10]. The masaRenderer supports rendering to binaural audio using the Default HRIR ser as specified in [5], Annex B.1. In the scripts, a simple wrapper around masaRenderer is provided.

Example commandline (MASA 1 transport channel):

python3 -m ivas_processing_scripts.audiotools -if MASA1 -of BINAURAL
-i path/to/input_file -o path/to/output_file –im metadata.met

#### Input format Object-based audio

For object based audio as an input format, rendering supporting the minimal set of object metadata as defined in [5] Annex C using the Default HRIR ([5], Annex B.1) is supported. Object position by means of azimuth and elevation on a 20ms frame basis are taken into account. A convolution of object audio with interpolated HRIRs corresponding to the object position is performed. The interpolation is performed as subsequently described:

- Interpolation is performed according to the following:

  - Frame-wise dynamic search of suitable triangle on sphere

  - Triangle has to contain target position and has to be as small as possible  proximity search starting with nearest three points on sphere (great circle distance) and subsequently increasing search distance if triangle is not suitable

  - Test if target position lies in current triangle is done by checking the signs of the spherical Barycentric coordinates [8]

  - When suitable triangle is found, compute the spherical barycentric coordinates and apply these weights to the respective HRIRs at the triangle vertices (similar to the bilinear interpolation in [9])

  - A crossfade is performed between frames

  - The interpolated HRIR is used for convolution

Example commandline (2 objects):

python3 -m ivas_processing_scripts.audiotools -if ISM2 -of BINAURAL
-i path/to/input_file -o path/to/output_file –im meta_1.csv meta_2.csv

### Binaural Room (BRIRs)

#### Input format Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), surround + height (5.1+4 and 7.1+4)

For surround (5.1, 7.1) and surround + height (5.1+4, 7.1+4) input formats, direct convolution using the Default BRIR set as specified in [5], Annex B.2 is used. The subset of the BRIRs corresponding to the loudspeaker positions is selected.

Binaural room rendering of stereo is implemented as a pass-through. Mono input is first rendered to stereo and then considered as binaural room.

Example commandline (5.1 Input Format):

python3 -m ivas_processing_scripts.audiotools -if 5_1 -of BINAURAL_ROOM
-i path/to/input_file -o path/to/output_file

#### Input format Binaural audio

For binaural audio as input and output format, no extra rendering is applied (pass-through).

#### Input format Scene-based audio (Ambisonics): FOA, HOA2 and HOA3

For scene based audio, binaural room rendering using the sparse BRIR set as specified in [5], Annex B.2 is performed in two steps:

- (Pre-) rendering from Ambisonics to 7.1+4 loudspeaker layout

- Binaural room rendereding from 7.1+4 with BRIRs (see above)

Example commandline (HOA3 Input Format):

python3 -m ivas_processing_scripts.audiotools -if HOA3 -of BINAURAL_ROOM
-i path/to/input_file -o path/to/output_file

#### Input format Metadata-assisted spatial audio

For Metadata-assisted spatial audio, binaural room rendering using the sparse BRIR set as specified in [5], Annex B.2 is performed in two steps:

- (Pre-) rendering to 7.1+4 using the masaRenderer as defined in the MASA reference software [10].

- Further rendered from 7.1+4 with BRIRs (see above)

Example commandline (MASA 1 transport channel):

python3 -m ivas_processing_scripts.audiotools -if MASA1 -of BINAURAL_ROOM
-i path/to/input_file -o path/to/output_file –im metadata.met

#### Input format Object-based audio

For Object-based audio, binaural room rendering using the sparse BRIR set as specified in [5], Annex B.2 is performed in two steps

- (Pre-) rendering from Object based audio to 7.1+4 loudspeaker layout by EFAP panning

- Further rendered from 7.1+4 with BRIRs (see above)

Example commandline (2 objects):

python3 -m ivas_processing_scripts.audiotools -if ISM2 -of BINAURAL_ROOM
-i path/to/input_file -o path/to/output_file –im meta_1.csv meta_2.csv

### Loudspeaker

#### Input format Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), surround + height (5.1+4 and 7.1+4)

The channel-based audio input formats mono (1.0 - CICP1), stereo (2.0 – CICP2), surround (5.1 – CICP6 and 7.1 – CICP12), surround + height (5.1+4 – CICP16 and 7.1+4 – CICP19) are expected to be routed directly to the corresponding loudspeaker format. No additional rendering is expected.

#### Input format Binaural audio

Playback of binaural audio on loudspeaker is currently not foreseen in the selection tests and thus currently not supported.

#### Input format Scene-based audio (Ambisonics): FOA, HOA2 and HOA3

Loudspeaker rendering for scene-based audio uses the all-round Ambisonic panning and decoding (ALLRAD) [7]. ALLRAD is implemented with EFAP [6] in intensity panning mode (a.k.a EFIP) as the panning method after subjective listening showed benefits for intensity panning over amplitude panning and EFAP over VBAP. A t-design of order 11 was chosen according to the recommendation to use a design of order  2N+1. The following output configurations are supported: Surround (5.1 – CICP6 and 7.1 – CICP12), surround + height (5.1+4 – CICP16 and 7.1+4 – CICP19).

Example commandline (HOA3 input format):

python3 -m ivas_processing_scripts.audiotools -if HOA3 -of 7_1_4
-i path/to/input_file -o path/to/output_file

#### Input format Metadata-assisted spatial audio

The rendering of Metadata-assisted spatial audio (MASA) is performed using the masaRenderer as provided in [10]. The masaRenderer supports rendering to 5.1 (CICP6) and 7.1.4 (CICP19) loudspeaker format. In the scripts, a simple wrapper around masaRenderer is provided.

Example commandline (MASA 1 transport channel):

python3 -m ivas_processing_scripts.audiotools -if MASA1 -of 7_1_4
-i path/to/input_file -o path/to/output_file –im metadata.met

#### Input format Object-based audio

Object-based audio is rendered using EFAP [6] to loudspeakers. EFAP was chosen due to benefits shown in subjective tests for sparse loudspeaker layouts. The use of polygons as opposed to triangles helps avoid inconsistencies especially when panning a virtual object e.g., in the rear trapezoid formed by the middle and upper rear speakers of most loudspeaker configurations where VBAP performs an arbitrary triangulation of the trapezoid.

The following output configurations are supported: Surround (5.1 – CICP6 and 7.1 – CICP12), surround + height (5.1+4 – CICP16 and 7.1+4 – CICP19).

Example commandline (2 objects):

python3 -m ivas_processing_scripts.audiotools -if ISM2 -of 7_1_4
-i path/to/input_file -o path/to/output_file –im meta_1.csv meta_2.csv

## Processing

This section describes the main processing modules utilized by the processing stages.

[

### MNRU reference conditions

To generate an MNRU reference at XXX dB for a file input.8k, use:

mnrudemo.exe –Q XXX input.8k output.8k 160

To generate an MNRU reference at XXX dB for a file input.16k, use:

mnrudemo.exe –Q XXX input.16k output.16k 320

To generate a P.50 MNRU reference at XXX dB for a file input.48k, use:

p50mnru.exe input.48k output.48k XXX M

Note that the P.50 MNRU processing for SWB conditions requires rate-change steps between 32kHz and 48kHz.

]

### ESDRU reference conditions

To generate an ESDRU reference at distortion level alpha for a file input, use:

esdru.exe -e_step 0.5 -seed 1 -sf fs alpha input output

where fs is the sample frequency, default 48 kHz, where alpha=[0.0,…,1.0] and alpha=0.0 corresponds to the highest distortion level.

[

### Reference codecs for Narrowband conditions

#### AMR

To process a file input.8k through the AMR codec at XXX kbit/s, use:

amr_cod_vad2.exe [-dtx] BBB input.8k bitstream

amr_dec.exe bitstream output.8k

where BBB is the bitrate mode corresponding to XXX as given in the following table.

| XXX [kbit/s] | 7.4 | 7.95 | 10.2 | 12.2 |
| --- | --- | --- | --- | --- |
| BBB | MR74 | MR795 | MR102 | MR122 |

Note that AMR operates at 20 ms frame length and has encoder and decoder delay of 5 ms and 0 ms, respectively

#### G.711

To process a file input.8k through the G.711codec with 20 ms blocks, use:

g711demo.exe LAW lili input.8k output.8k 160

where LAW is A for A-law and u for u-law. Note that G.711 does not have algorithmic delay.

#### G.718 narrowband mode

To process a file input.8k through the G.718 codec in narrowband mode at XXX bit/s, use:

g718_enc.exe [-dtx 8] –maxBR XXX 8 input.8k bitstream

g718_dec.exe -disabled_NG –maxBR XXX 8 bitstream output.8k

where XXX is either 8000 or 12000. Note that G.718 operates at 20 ms frame length and has an algorithmic delay of 33.875 ms, when operating on narrowband input and in low-delay mode.

### Reference codecs for Wideband conditions

#### AMR-WB

To process a file input.16k through the AMR-WB codec at XXX kbit/s, use ITU-T G.722.2 as:

amrwb_cod.exe [–dtx] –itu BBB input.16k bitstream

amrwb_dec.exe -itu bitstream output.16k

where BBB is the bitrate mode corresponding to XXX as given in the following table.

| XXX [kbit/s] | 8.85 | 12.65 | 14.25 | 15.85 | 18.25 | 19.85 | 23.05 | 23.85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BBB | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

Note that AMR-WB operates at 20 ms frame length and has encoder and decoder delay of 5 ms and 0.9375 ms, respectively.

#### AMR-WB encoder - G.718 IO decoder

To process a file input.16k through the AMR-WB encoder and the G.718 IO decoder at XXX bit/s, use ITU-T G.722.2 as:

amrwb_cod.exe [–dtx] –itu BBB input.16k bitstream

where BBB is described in 5.2.4.1. For decoding call

g718_dec.exe -disabled_NG -IO_G722_2 –maxBR YYY 16 bitstream output.16k

where YYY is the bitrate mode corresponding to BBB as given in the following table.

| BBB | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| YYY | 8850 | 12650 | 14250 | 15850 | 18250 | 19850 | 23050 | 23850 |

Note that AMR-WB encoder operates at 20 ms frame length and has a delay of 5 ms. G.718 IO decoder operates at 20 ms frame length and has a delay of 1.9375 ms when operating on wideband input and in low-delay mode.

#### G.718

To process a file input.16k through the G.718 codec at XXX bit/s, use:

g718_enc.exe –maxBR XXX 16 input.16k bitstream

g718_dec.exe -disabled_NG –maxBR XXX 16 bitstream output.16k

where XXX is 32000.

Note that G.718 operates at 20 ms frame length and has an overall algorithmic delay of 42.875 ms, when operating on wideband input.

#### G.722

To process a file input.16k through the G.722 codec at XXX kbit/s using 20 ms block size, use:

encg722.exe –fsize 320 –mode XXX input.16k bitstream

decg722.exe -fsize 320 –mode XXX bitstream output.16k

where XXX is either 56 or 64. Note that G.722 operates at 20 ms frame length and has an overall algorithmic delay of 1.625 ms.

#### G.722.1

To process a file input.16k through the G.722.1 codec at XXX bit/s, use:

g7221_enc.exe 1 input.16k bitstream XXX 7000

g7221_dec.exe 1 bitstream output.16k XXX 7000

where XXX is one of (24000, 32000). Note that G.722.1 operates at 20 ms frame length and has an overall algorithmic delay of 40 ms.

#### G.711.1

To process a file input.16k through the G.711.1 codec at 80 bit/s, use:

g7111_enc.exe –mode 3 A input.16k bitstream

g7111_dec.exe A –mode 3 bitstream output.16k

Note that G.711.1 operates at 20 ms frame length and has an overall algorithmic delay of 11.875 ms.

### Reference codecs for super-wideband conditions

#### AMR-WB+

To process a file input_32.wav through the AMR-WB+ codec at XXX kbit/s, use:

amrwbplus_cod.exe –rate XXX –ff raw –if input_32.wav –of bitstream

amrwbplus_dec.exe –ff raw –if bitstream –fs 32000 –mono –of output_32.wav

where XXX is one of 9.75, 12, 16. Note that AMR-WB+ has an overall algorithmic delay of approx. 72/68 ms at 9.75/12kbps.

#### G.718 Annex B

To process a file input.32k through the G.718 Annex B codec at 36 kbit/s, use:

g718_enc.exe –maxBR 36000 32 input.32k bitstream

g718_dec.exe –disabled_NG 32 bitstream output.32k

Note that G.718 Annex B operates at 20 ms frame length and has an overall algorithmic delay of 49.625 ms.

#### G.719

To process a file input.48k through the G.719 codec at XXX bit/s, use:

g719_enc.exe –r XXX –i input.48k -o bitstream

g719_dec.exe –i bitstream -o output.48k

where XXX is one of 32000, 48000, 56000, 64000, 80000, 96000, 128000. Note that G.719 operates at 20 ms frame length and has an overall algorithmic delay of 40 ms. G.719 operates at 48 kHz which implies resampling steps from and to 32 kHz.

#### G.722.1 Annex C

To process a file input.32k through the G.722.1 Annex C codec at XXX bit/s, use:

g7221_enc.exe 1 input.32k bitstream XXX 14000

g7221_dec.exe 1 bitstream output.32k XXX 14000

where XXX is one of 24000, 32000 or 48000. Note that G.722.1 Annex C operates at 20 ms frame length and has an overall algorithmic delay of 40 ms.

### Delay compensation for reference conditions

Details on the delay compensation implementation can be found in Annex A.1.

### EVS operation (non-JBM case)

To process a file input.8k through the EVS codec at XXX bit/s, use:

EVS_cod.exe [-dtx] XXX/SWF 8 input.8k bitstream

EVS_dec.exe 8 bitstream output.8k

where XXX is one of 5900, 7200, 8000, 9600 or 13200. For rate switching operation, XXX is replaced by a switching file (SWF).

To process a file input.16k through the EVS codec at XXX bit/s, use:

EVS_cod.exe [-dtx] XXX/SWF 16 input.16k bitstream

EVS_dec.exe 16 bitstream output.16k

where XXX is one of 5900, 7200, 8000, 9600, 13200, 16400, 24400, 32000, 48000, 64000, 96000 for testing non-IO modes or 6600, 8850, 12650, 14250, 15850, 18250, 19850, 23050, 23850 for testing AMR-WB IO modes. For rate switching operation, XXX is replaced by a switching file (SWF).

To process a file input.32k through the EVS codec at XXX bit/s, use:

EVS_cod.exe [-dtx] XXX/SWF 32 input.32k bitstream

EVS_dec.exe 32 bitstream output.32k

where XXX is one of 13200, 16400, 24400, 32000, 48000, 64000, 96000, 128000. For rate switching operation, XXX is replaced by a switching file (SWF).

The switching file consists of XXX values indicating the bit rate for each frame in bit/s. These values are stored in binary format using 4 bytes per value.

Note: EVS encoder and decoder provide delay compensated output files

### EVS operation (JBM case)

To process a file input.8k through the EVS codec at XXX bit/s, use:

EVS_cod.exe [-dtx] XXX 8 input.8k bitstream

EVS_dec.exe –Tracefile tracefile_dec –VOIP 8 netsimoutput output.8k

where XXX is 9600.

To process a file input.16k through the EVS codec at XXX bit/s, use:

EVS_cod.exe [-dtx] XXX 16 input.16k bitstream

EVS_dec.exe –Tracefile tracefile_dec –VOIP 16 netsimoutput output.16k

where XXX is 13200.

To process a file input.32k through the EVS codec at XXX bit/s, use:

EVS_cod.exe [-dtx] XXX 32 input.32k bitstream

EVS_dec.exe –Tracefile tracefile_dec –VOIP 32 netsimoutput output.32k

where XXX is one of 13200 or 24400. The tracefile_dec is the tracefile written by the decoder and netsimoutput is the RTP+G.192 input file written by the network simulator.

### CuT operation (non-JBM case)

Editor’s Note: TBD

### CuT operation (JBM case)

Editor’s Note: TBD

## Encoder and decoder CuT executable requirements

The executables shall be compatible to WIN32.

## Error Insertion (EID)

For the conditions where random frame erasures are desired, frame erasure patterns are applied to the bitstream using tools from the [ITU-T STL2009 library].

### Frame error tool

For the AMR codec, the following processing shall be used:

eid-amr.exe amrbsin ep.g192 amrbsout

where:

amrbsin is the AMR input bit stream

ep.g192 is the error pattern file

amrbsout is the AMR output bit stream

For the G.711 A/µ-law, the following processing shall be used:

eid-int –ep g192 –factor 2 ep.g192 ep10.g192

g711iplc.exe ep10.g192 input.8k output.8k

where:

ep.g192 is the error pattern file assuming 20ms frames and ep10.g192 is the error pattern file for the g711iplc tool where all entries are doubled to take the 10ms frame grid of the g711iplc tool into account.

input.8k is the G.711 decoded output file

output.8k is the G.711 decoded output file with packet loss concealment

For all other reference codecs and CuTs, the following processing shall be used:

eid-xor.exe –vbr –fer g192bsin ep.g192 g192bsout

where:

g192bsin is the input bit stream

ep.g192 is the error pattern file

g192bsout is the output bit stream

### Pattern generation

Before generating error patterns, the sta file of the gen-patt tool needs to be initialized as specified in Annex C.3.

The error patterns used are generated using the gen-patt tool as follows:

gen-patt.exe -tailstat -fer -g192 -gamma 0 -rate XXX -tol 0.001      -reset -n LENGTH -start 501 ep.g192

where XXX is the required erasure rate, i.e. 0.03 for 3% and 0.06 for 6% FER.

Different error patterns should be generated for each experiment. For all speech experiments the parameter LENGTH is exactly 12500,  assuming a 10 sec preamble and 6 talker with 5 samples of 8 sec length each. For the mixed/music experiments, this parameter needs to be adapted to the length of the concatenated sequence.

For the AMR-WB encoder conditions with enabled DTX, the error pattern needs to be repeated due to the repetition of the input sequence. To generate an EPF with repetition, use:

concate.exe ep.g192 ep.g192 ep.repeated.g192

### Derive EPFs from jitter profiles

To derive an error pattern file from one of the delay and error profiles 1, 2, or 3 given in the MTSI specification, use:

dlyerr_2_errpat.exe –d 200 –f 1 –w –s YYY –i dly_err_profile_XXX.dat -o f_profile_XXX.g192

where XXX is one of 1, 2, or 3, and YYY is the random offset into the profile.

To derive an error pattern file from one of the delay and error profiles 4 or 6 given in the MTSI specification, use:

dlyerr_2_errpat.exe –l 1 –f 1 –w –s YYY –i dly_err_profile_XXX.dat -o f_profile_XXX.g192

where XXX is one of 4, or 6, and YYY is the random offset into the profile.

To derive an error pattern file from one of the delay and error profile 5 given in the MTSI specification, use:

dlyerr_2_errpat.exe –l 1 –f 2 –w –s YYY –i dly_err_profile_5.dat -o f_profile_5.g192

where YYY is the random offset into the profile.

### Rate switching profile generation

To generate rate switching profiles for CuT conditions, use

gen-rate-profile.exe –layers B1,B2,…,Bx SWF 10 B1 LENGTH SEEDx

where B1,B2,…,Bx are all bit rates starting from the lowest one (B1) up to the highest one (Bx). The following list shows all possible rates for the CuT:

- no AMR-WB IO: 7200, 8000, 9600, 13200,  24400, 32000, 48000, 64000, 96000, 128000

- AMR-WB IO: 6600, 8850, 12650, 14250, 15850, 18250, 19850, 23050, 23850

SWF is the file name and SEEDx is a random seed, see C.2. For mixed/music experiments, the parameter LENGTH needs to be adapted to the length of the concatenated sequence. For speech experiments, the parameter LENGTH is exactly 12500.

To generate for instance the rate switching profile for a non-IO CuT condition from 9.6kbps to 64kbps, use

gen-rate-profile.exe –layers 9600,13200,16400,24400,32000,48000,64000 SWF 10 9600 LENGTH SEEDx

To generate a rate switching profiles consisting only of AMR-WB IO modes, use the Bx rates listed under AMR-WB IO. Mixing the non-IO and IO rates is required in case WB non-IO / IO switching is tested, e.g.to process a switched CuT non-IO / IO condition from 8.85kbps to 13.2kbps, use

gen-rate-profile.exe –layers 8850,9600,12650,13200 SWF 10 8850 LENGTH SEEDx

Note: Case B) AMR-WB encoder side rate switching is not supported so far

### Network simulator for packet jitter generation

To simulate packet jitter for CuT conditions, use

network_simulator.exe dly_error_profile_x.dat evsbitstream netsimoutput tracefile_sim nFramesPerPacket [offset]

where

- dly_error_profile_x.dat is the MTSI delay and error profile number x

- evsbitstream is the G.192 input bitstream file name

- netsimoutput is the RTP+G.192 output file name

- tracefile_sim is the trace output file name of the network simulator

- nFramesPerPacket is the number of frames per packet (1, 2)

- offset is the shift/offset in delay and error profile in frames (default: 0)

### Cutting Tool for JBM tests

To trim parameters for the file segmentation after de-jittering, use

jbmtrim.exe –file Fs tracefile_dec undo_concat.txt undo_concat_trim.txt

where

- file is to enable the undo_concat.txt support

- Fs is the operating sampling rate, i.e. 8000, 16000 or 32000

- tracefile_dec is the trace output file of the CuT decoder

- undo_concat.txt is the file name where parameters for segmentation are stored

- undo_concat_trim.txt is the file name containing the corrected parameters for segmentation using the astrip tool

]

# References

Pdoc IVAS-8a: “Test plan for selection phase”

Recommendation ITU-T G.192: “A common digital parallel interface for speech standardization activities”, March 1993

Recommendation ITU-T G.191 “Software tools for speech and audio coding standardization”, March 2010

TS 26.114, “IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction”

Pdoc IVAS-4: “IVAS Design Constraints”

C. Borß, "A Polygon-Based Panning Method for 3D Loudspeaker Setups," Paper 9106, (2014 October.)

F. Zotter, and M. Frank, "All-Round Ambisonic Panning and Decoding," J. Audio Eng. Soc., vol. 60, no. 10, pp. 807-820, (2012 October.)

T. Langer, A. Belyaev and H. Seidel, "Spherical Barycentric Coordinates," In Proceedings of the fourth Eurographics symposium on Geometry processing (SGP '06). Eurographics Association, Goslar, DEU, 81–88, (2006)

F.P. Freeland, L.W.P. Biscainho and P.S.R. Diniz, "Interpolation of Head-Related Transfer Functions (HRTFS): A multi-source approach." 2004 12th European Signal Processing Conference. Vienna, 2004, pp. 1761–1764.

S4-230221 - Processing updates for IVAS MASA C Reference Software

[

External Resources

Delay compensation for filter operations and reference codecs

The processing steps are delay-compensated in order to apply error insertion on the same parts of the audio signal and to be able to extract the original length and offset for each audio sample used in the tests.

The delay compensation is initialized by concatenating the file to be filtered and the first 960 samples of the preamble. After processing, the delay of the processing operation is compensated, and the original file length is restored.

To compensate the delay for filter operations and reference conditions for encoder and decoder in the common scripts, use:

astrip.exe –sample –start S+1 –n FILELENGTH input output

where FILELENGTH denotes the size in samples and the value for S for each filtering operation is given in Table 7 and for the reference conditions in Table 8.

Table 8: Delay compensation values for filter operations

| Filter operation | Value for delay compensation after filtering operation |
| --- | --- |
| -up SHQ2 | 436 |
| -up SHQ3 | 436 |
| -down SHQ2 | 218 |
| -down SHQ3 | 145 |
| HP50_48KHZ | 839 |
| MSIN | 92 |

Table 9: Delay compensation values for reference codecs

| Codec | Value for encoder delay compensation | Value for decoder delay compensation |
| --- | --- | --- |
| AMR | 40 | 0 |
| AMR-WB | 80 | 15 |
| AMR-WB encoder – EVS IO decoder (Case B) | 80 | 0 |
| AMR-WB encoder - G.718 IO decoder | 80 | 0 |
| AMR-WB+ | - | 2314 (9.75kbps) 2187 (12kbps) 2187 (16kbps) |
| EVS | 0 | 0 |
| EVS IO encoder – AMR-WB decoder (Case A) | 0 | 15 |
| G.711 (A and µlaw) including the g711iplc tool | 0 | 0 |
| G.711.1 | 175 | 15 |
| G.718 | 0 | 0 |
| G.718 Annex B | 0 | 0 |
| G.718 narrow band | 0 | 0 |
| G.719 | 480 | Add 480 samples to beginning of decoded output |
| G.722 | 11 | 11 |
| G.722.1 | 160 | 160 |
| G.722.1 Annex C | 320 | 320 |

Binaries used

All binaries are compiled and tested under Win32 platforms. The following section documents the origin and the compilation of the binaries.

Editor’s Note: Usage rights of tools need review and updated rights may be needed for use in IVAS standardization.

ITU-T STL processing tools

| Source | - ITU-T G.191 - S4-120344 “Filter masks for EVS testing” |
| --- | --- |
| URL | - https://github.com/openitu/STL - https://www.itu.int/rec/T-REC-G.191-202207-I Editor’s note: G.191 URL to be updated once new version is available. - http://ftp.3gpp.org/tsg_sa/WG4_CODEC/TSGS4_68/Docs/S4-120344.zip |
| Version / Release | - G.191 / STL2023 |
| Description | Software tools for speech and audio coding standardization |
| Comments | G.191 filter tool patched with S4-120344 to enable support for HP50 and SHQ filter |
| Executables | oper, astrip, concat, sv56demo, filter, scaldemo, actlev, eid-xor, gen-patt, mnrudemo, gen-rate-profile, eid-int, stereoop, bs1770demo, esdru, p50fbmnru, wmc_tool |
| Status | Available |

AMR Error insertion device

| Source | Orange: S4-120998 |
| --- | --- |
| URL | http://ftp.3gpp.org/tsg_sa/WG4_CODEC/TSGS4_70/Docs/S4-120998.zip |
| Version / Release | - |
| Description | Error insertion device for AMR bit streams |
| Comments |  |
| Executables | eid-amr |
| Status | Available |

Reference codecs

AMR

| Source | 3GPP TS 26.073: ANSI-C code for the Adaptive Multi Rate (AMR) speech codec |
| --- | --- |
| URL | http://www.3gpp.org/ftp/Specs/archive/26_series/26.073/26073-a00.zip |
| Version / Release | 3GPP Rel. 10; Software version 5.1.0 March 26, 2003 |
| Description | AMR narrow band fix point encoder and decoder software |
| Comments | Compiled with VAD version 1 and 2 |
| Executables | amr_cod_vad1.exe, amr_cod_vad2.exe, amr_dec.exe |
| Status | Available |

G.711

| Source | ITU-T G.191 |
| --- | --- |
| URL | http://www.itu.int/rec/T-REC-G.191-201003-I |
| Version / Release | Version v3.3 of 02.Feb. 2010 |
| Description | G.711 codec |
| Comments |  |
| Executables | g711demo.exe, g711iplc.exe |
| Status | Available |

AMR-WB

| Source | 3GPP TS 26.173: ANSI-C code for the Adaptive Multi-Rate - Wideband (AMR-WB) speech codec |
| --- | --- |
| URL | http://www.3gpp.org/ftp/Specs/archive/26_series/26.173/26173-a00.zip |
| Version / Release | 3GPP Rel. 10; AMR Wideband Codec 3GPP TS26.190 / ITU-T G.722.2, Aug 25, 2003. Software version 7.0.0. |
| Description | AMR-WB fixed point encoder and decoder software |
| Comments |  |
| Executables | amrwb_cod.exe, amrwb_dec.exe |
| Status | Available |

G.722

| Source | ITU-T G.191 |
| --- | --- |
| URL | http://www.itu.int/rec/T-REC-G.191-201003-I |
| Version / Release | COPYRIGHT CNET LANNION A TSS/CMC Date 24/Aug/90 COPYRIGHT Ericsson AB. Date 22/May/06 COPYRIGHT France Telecom R&D Date 23/Aug/06 |
| Description | G.722 encoder and decoder software |
| Comments |  |
| Executables | encg722.exe, decg722.exe |
| Status | Available |

G.711.1

| Source | G.711.1: Wideband embedded extension for ITU-T G.711 pulse code modulation |
| --- | --- |
| URL | http://www.itu.int/rec/T-REC-G.711.1-201209-I/en |
| Version / Release | V2.00 / (09 / 2012) |
| Description | G.711.1 encoder and decoder software |
| Comments | Sub folder: G.711.1_MB-Fix_v2.00 |
| Executables | g7111_enc.exe, g7111_dec.exe |
| Status | Available |

G.722.1 (Annex C)

| Source | G.722.1 : Low-complexity coding at 24 and 32 kbit/s for hands-free operation in systems with low frame loss |
| --- | --- |
| URL | http://www.itu.int/rec/T-REC-G.722.1-200505-I/en |
| Version / Release | Release 2.1 (05/2005); Source folder: Fixed-200505-Rel.2.1 |
| Description | G.722.1 (Annex C) fixed point encoder and decoder software |
| Comments | Includes WB and SWB operations |
| Executables | g7221_enc.exe, g7221_dec.exe |
| Status | Available |

AMR-WB+

| Source | 3GPP TS 26.273: ANSI-C code for the fixed-point Extended Adaptive Multi-Rate - Wideband (AMR-WB+) speech codec |
| --- | --- |
| URL | http://www.3gpp.org/ftp/Specs/archive/26_series/26.273/26273-a00.zip |
| Version / Release | 3GPP Rel. 10; |
| Description | AMR-WBplus fixed point encoder and decoder software |
| Comments | er-libisomedia.dll required |
| Executables | amrwbplus_cod.exe, amrwbplus_dec.exe |
| Status | Available |

G.718 (Annex B)

| Source | G.718 : Frame error robust narrow-band and wideband embedded variable bit-rate coding of speech and audio from 8-32 kbit/s |
| --- | --- |
| URL | http://www.itu.int/rec/T-REC-G.718-201101-I!Cor3/en |
| Version / Release | Corrigendum 3; Approved 2011-01; Version v1.6; Software version v1.0; Source folder: G.718_V1.6_Part3_swb_mono_AnnexB_26072010 |
| Description | G.718 (Annex B) fixed point encoder and decoder software |
| Comments | Includes NB, WB and SWB codec |
| Executables | g718_enc.exe, g718_dec.exe |
| Status | Available |

G.719

| Source | G.719 : Low-complexity, full-band audio coding for high-quality, conversational applications |
| --- | --- |
| URL | http://www.itu.int/rec/T-REC-G.719-200806-I/en |
| Version / Release | Approved in 2008-06 including G.719 (2008) Amend.1 and G.719 (2008) Amend.2.; v1.0a (2008-04-16); Source folder: Fix-point-200806-Release-1.0a |
| Description | G.719 fixed point encoder and decoder software |
| Comments |  |
| Executables | g719_enc.exe, g719_dec.exe |
| Status | Available |

EVS

| Source | TBD, [3GPP TS 26.442/TS 26.443/TS 26.452: Codec for Enhanced Voice Services (EVS)] |
| --- | --- |
| URL | TBD |
| Version / Release | [Latest version available, TBD] |
| Description | EVS encoder and decoder software |
| Comments |  |
| Executables | EVS_cod.exe, EVS_dec.exe |
| Status | Available |

Other tools

Audio format converter

| Source | McGill University, Telecommunications & Signal Processing Laboratory, Audio File Programs and Routines |
| --- | --- |
| URL | https://www-mmsp.ece.mcgill.ca/Documents/Downloads/AFsp/AFsp-v10r4a.tar.gz |
| Version / Release | Release v10r4; Software version of CopyAudio: v6r0 2003-05-08 |
| Description | CopyAudio tool from the AFsp |
| Comments |  |
| Executables | CopyAudio.exe |
| Status | Available |

Network simulator

| Source | Fraunhofer |
| --- | --- |
| URL | http://ftp.3gpp.org/tsg_sa/WG4_CODEC/TSGS4_76/Docs/S4-131277.zip |
| Version / Release | - |
| Description | Jitter Simulator |
| Comments |  |
| Executables | network_simulator.exe |
| Status | Available |

Jitter profile to EPF converter

| Source | Fraunhofer: S4-121077 |
| --- | --- |
| URL | http://ftp.3gpp.org/tsg_sa/WG4_CODEC/TSGS4_70/Docs/S4-121077.zip |
| Version / Release | - |
| Description | Converts MTSI jitter profiles to error pattern for reference codecs |
| Comments |  |
| Executables | dlyerr_2_errpat.exe |
| Status | Available |

JBM trim tool

| Source | Fraunhofer: AHEVS-181 |
| --- | --- |
| URL | http://ftp.3gpp.org/tsg_sa/WG4_CODEC/Ad-hoc_EVS/Docs/AHEVS-181.zip |
| Version / Release | - |
| Description | Tool for trim parameters for segmentation of samples |
| Comments |  |
| Executables | jbmtrim.exe |
| Status | Available |

AFR measure tool

| Source | Qualcomm, Fraunhofer |
| --- | --- |
| URL | Latest Executable: http://ftp.3gpp.org/tsg_sa/WG4_CODEC/TSGS4_76/Docs/S4-131277.zip Documentation: http://ftp.3gpp.org/tsg_sa/WG4_CODEC/TSGS4_75/Docs/S4-131020.zip |
| Version / Release | - |
| Description | Tool for measuring active frame ratio, bit rate and for identification of active frames |
| Comments |  |
| Executables | afr_br.exe |
| Status | Available |

Jitter buffer verification tool

| Source | Fraunhofer: AHEVS-235 |
| --- | --- |
| URL | http://ftp.3gpp.org/tsg_sa/WG4_CODEC/ Ad-hoc_EVS/ Docs/AHEVS-235.zip |
| Version / Release | V4.0 |
| Description | Tool for checking the objective performance requirements of the jitter buffer manager |
| Comments |  |
| Executables | checkObjectiveJbmPR.exe |
| Status | Available |

Gain check tool

| Source | VoiceAge: AHEVS-231 |
| --- | --- |
| URL | http://ftp.3gpp.org/tsg_sa/WG4_CODEC/ Ad-hoc_EVS/Docs/AHEVS-231.zip |
| Version / Release | V3.0 |
| Description | Tool for verification of amplification of active and attenuation of background noise |
| Comments |  |
| Executables | gain_chk_v2.exe |
| Status | Available |

Randomization tool

| Source | S4-121078 |
| --- | --- |
| URL | http://ftp.3gpp.org/tsg_sa/WG4_CODEC/TSGS4_70/Docs/S4-121078.zip |
| Version / Release | - |
| Description | Tool for providing all randomizations depending on a master seed |
| Comments |  |
| Executables | random.exe |
| Status | Available |

MD5 checksum tool

| Source | GNU utilities for Win32 |
| --- | --- |
| URL | http://unxutils.sourceforge.net |
| Version / Release | 2.1 |
| Description | Tool for crosschecking common script result via MD5 hashes |
| Comments |  |
| Executables | md5sum.exe |
| Status | Available |

IVAS MASA C Reference Software

| Source | Nokia, Orange: S4-230221 |
| --- | --- |
| URL | https://www.3gpp.org/ftp/TSG_SA/WG4_CODEC/TSGS4_122_Athens/Docs/S4-230221.zip |
| Version / Release | - |
| Description | masaAnalyzer is the tool that takes in input signal and processes it into transport signals and metadata specified in MASA format. masaRenderer is the tool that takes in the created MASA format signals and renders it to various listenable output configurations. |
| Comments |  |
| Executables | masaAnalyzer.exe, masaRenderer.exe |
| Status | Available |

Objective evaluation

Databases for objective evaluation

For evaluation of any objective performance, the common database used in selection testing will be available subject to an appropriate legal framework. The database will consist of the following parts:

Table 10: Available databases for objective evaluation

| Database | Description | File name |
| --- | --- | --- |
| 1 |  |  |

Active Frame Ratio (AFR)

To measure the active frame ratio, use

afr_br [OPTIONS] G192FILE

where the OPTIONS are:

-a              		needs to be enabled when the bit stream is in AMR format instead of G.192

-s SID_SIZE     	specifies one valid SID_SIZE in bits (up to five)

-f SID_FILE     	specifies a SID_FILE with valid SID frame sizes (up to five) as one line per size

-p PREAMBLE     	is the number of PREAMBLE frames (each 20ms) to skip

-b BR_IN_FILE   	input file BR_IN_FILE which contains the bit rates as external input

-v VAD_IN_FILE 	input file VAD_IN_FILE which contains VAD decisions as external input

-V VAD_OUT_FILE 	output VAD_OUT_FILE file which contains VAD decisions based on SID sizes

-o RESULT_FILE 	output file RESULT_FILE which contains the textual result

-O RESULT_BR_F 	output file RESULT_BR_F which contains the bit rate mismatches

The AFR is measured separately on all noisy speech databases as defined in Section B.1.

Gain measuring of active frames and inactive frames

Editor’s Note: To be updated based on design constraint on “Output gain tolerance”, see IVAS-4 [5].

To identify the active signal parts of the objective database, the AMR-WB VAD is to be used. Use

amrwb_cod.exe –dtx –itu 2 EVSDB_clean_speech.16k cleanspeech_bs

to generate the AMR-WB bit stream for the clean speech database. The delay compensation for the AMR-WB needs to be taken into account. To extract the information on the active frames, use

afr_br.exe –s 35 –p 500 –V VAD_f_cleanspeech cleanspeech_bs

where VAD_f_cleanspeech is the file name where VAD flags per frame are stored.

Process the databases by the required CuT conditions. Before starting to measure, strip the 10 sec preamble off the original database and the processed database.

To measure the amplification level of active signals, use the following command line:

gain_chk -i original_DB -o processed_DB -t results_f -r Fs 
[–v VAD_f_cleanspeech]

where

- original_DB is one of the databases defined in B.1.

- processed_DB is the original_DB processed with one CuT condition

- results_f is the results file to be reported for all DB files and CuT conditions

- Fs is the sampling frequency

- VAD_f_cleanspeech is the VAD file to be generated on the clean speech database. This file is required for the evaluation of clean speech and noisy speech. For mixed content and music, this option needs to be disabled.

The results contain two parts, one for the active frames and one for the inactive frames.

JBM performance evaluation

To evaluate the performance of the EVS JBM solution, it is recommended to use

checkObjectiveJbmPR.exe -v dly_error_profile_x.dat nFramesPerPacket tracefile_sim tracefile_dec

where

- dly_error_profile_x.dat is the MTSI delay and error profile number x

- nFramesPerPacket is the number of frames per packet (1, 2)

- tracefile_sim is the trace output file name of the network simulator

- tracefile_dec is the trace output file of the CuT decoder

Bitrate evaluation

To evaluate the bitrate of the CuT, it is recommended to use the AFR tool as specified in Section B.2. This tool reports on the bit rate of a bitstream. The preamble shall be excluded from calculation.

Randomization scripts

The randomization scripts require one master seed, which will be provided by the HL. The master seed (MASTER_SEED) determines all parameters for noise files, EPFs, jitter and rate switching.

In addition to the master seed, a pre-run number P is chosen per experiment and lab, which is used in combination with the master seed to determine error patterns and noise file offsets.

Table 11: Definition of Pre-run numbers P

| Experiment | LL a Pre-run number P | LL b Pre-run number P | LL c Pre-run number P | LL d Pre-run number P |
| --- | --- | --- | --- | --- |
| P800-1 | 101 | 102 | 103 | 104 |
| P800-2 | 105 | 106 | 107 | 108 |
| P800-3 | 109 | 110 | 111 | 112 |
| P800-4 | 113 | 114 | 115 | 116 |
| P800-5 | 117 | 118 | 119 | 120 |
| P800-6 | 121 | 122 | 123 | 124 |
| P800-7 | 125 | 126 | 127 | 128 |
| P800-8 | 129 | 130 | 131 | 132 |
| P800-9 | 133 | 134 | 135 | 136 |
| BS1534-1a | 137 | 138 | 139 | 140 |
| BS1534-1b | 141 | 142 | 143 | 144 |
| BS1534-2a | 145 | 146 | 147 | 148 |
| BS1534-2b | 149 | 150 | 151 | 152 |
| BS1534-3a | 153 | 154 | 155 | 156 |
| BS1534-3b | 157 | 158 | 159 | 160 |
| BS1534-4a | 161 | 162 | 163 | 164 |
| BS1534-4b | 165 | 166 | 167 | 168 |
| BS1534-5a | 169 | 170 | 171 | 172 |
| BS1534-5b | 173 | 174 | 175 | 176 |
| BS1534-6a | 177 | 178 | 179 | 180 |
| BS1534-6b | 181 | 182 | 183 | 184 |
| BS1534-7a | 185 | 186 | 187 | 188 |
| BS1534-7b | 189 | 190 | 191 | 192 |

Delay and error profile offset

As the JBM profiles can be looped, the range should be the entire profile, i.e. 7500 or 8000 entries.

To randomly select an JBM offset for profiles 1-6, use:

random -n 1 -s MASTER_SEED –d PROFILE_NUMBER –r 0 7499

where PROFILE_NUMBER is the number of the profile for profiles 1-6.

To randomly select an JBM offset for profiles 7-10, use:

random -n 1 -s MASTER_SEED –d PROFILE_NUMBER –r 0 7999

where PROFILE_NUMBER is the number of the profile for profiles 7-10.

Seeds for rate switching

The seeds to generating the rate switching profiles (Section 5.4.4) can be obtained by

- SEEDx = MASTER_SEED

Initialization of gen-patt tool

In order to initialize the gen-patt tool, an sta file needs to written. Therefore, two templates are provided:

Template for 6% frame error rate:

EID

BER           = 0.060000

GAMMA         = 0.000000

RAN-seed      = 0x176d71ac

Current State = G

GOOD->GOOD    = 0.880000

GOOD->BAD     = 1.000000

BAD ->GOOD    = 0.880000

BAD ->BAD     = 1.000000

Template for 3% frame error rate:

EID

BER           = 0.030000

GAMMA         = 0.000000

RAN-seed      = 0xab5d4825

Current State = G

GOOD->GOOD    = 0.940000

GOOD->BAD     = 1.000000

BAD ->GOOD    = 0.940000

BAD ->BAD     = 1.000000

where the RAN-seed needs to be replaced in HEX format. To generate these numbers, use

random -n 1 -s MASTER_SEED –d P –r 0 99999999

where P is the pre-run number according to Table 11.

by the following numbers:

| Experiment | EPF for 3% | EPF for 6% | EPF for 10% |
| --- | --- | --- | --- |

Random noise file selection

[

Overview

Editor’s note: Exact procedure to be decided.

Background noise file offset

To randomly select an offset for the background noise, use:

random.exe -n 1 -s MASTER_SEED –d P –r 1 2400000

where the resulting value determines the offset to be used for the dedicated background noise file. The value Q depends on the lab and the experiment as outlined in [TBD].

]

Additional Delay and Loss Profiles for Characterization

A number of VoLTE RTP packet logs under different network conditions were analyzed and the set of delay loss profiles dlp7, dlp8, dlp9 and dlp10 was derived to reflect the properties of the analyzed logs. The VoLTE logs were captured in different operator networks across different regions (South Korea and US). While different VoLTE network operating parameters affect the channel error characteristics and therefore the profile generation, it is noted that certain parameters such as the delay jitter, error burstiness, and jitter temporal evolution have more bearing on the VoLTE error profile generation. These VoLTE parameters are described below in the process of dlp7 through dlp10 generation.

Delay Jitter

The delay jitter is a characteristic of the VoLTE network under consideration. Jitter of the arrived packets depends on factors such as network buffering characteristics, scheduler delays, network load, fading characteristics and mobility of the UE. The logs were derived to reflect on the typical characteristics of VoLTE networks by extracting the delay jitter from four different real world VoLTE logs under different network conditions. In particular, typical characteristics such as random jitter, ramp up and ramp down of jitter and jitter under increased network loss were included in the four delay loss profiles.

The delay jitter histograms in Figure 23 show the different delay jitter distributions of the four delay loss profiles derived from VoLTE logs under challenging radio conditions. Figure 24 shows the first 1000 elements of the proposed profiles.

Figure 18: Delay histograms for delay loss profiles 7, 8, 9, and 10

Error burstiness

The error burstiness varies greatly with different channel conditions. The typical error burstiness of VoLTE logs under impaired channel conditions has been analyzed and is reflected in the four delay loss profiles from VoLTE logs under challenging radio conditions; the most frequent burst length varies for different profiles and that is 1, 2 or 3 in most cases.

Figure 19: Delay variation for delay loss profiles 7, 8, 9, and 10

The graphs show only first 1000 entries in each delay loss profile due to space limitations. Note that the “-1” entries represent network losses.

]