Source:	Fraunhofer IIS

Title:	Harmonization and Consolidation of the IVAS-8a Test Plan Annex E Experiments

Document for:	Discussion & Agreement

Agenda Item:	1.4

- Background

Currently, in the IVAS-8a Test Plan [1], Annex E, where all the P.SUPPL800 Experiments to be performed for selection are described, the formatting and information given for each experiment are varying. This contribution aims to harmonize all experiment descriptions to the same format. Furthermore, it is attempted to consolidate Experiment P800-3 where there are 2 descriptions and unify some parameters that have already been discussed in previous meetings e.g. Input frequency mask or FER.

Issues detected with this exercise:

In P800-4 Option 1 of the existing Table “Factors and conditions (Exp P800-4: clean speech)” was used to populate Table E.4.3, however for the CuT bitrate 16.4kbps a reference bitrate of 3 x 7.2kbps EVS (planar) as defined in IVAS-3 Performance Requirements [2] for this input format is missing.

In able E.8.3: Test conditions for Experiment P800-8 (MASA clean speech), there may be obsolete reference conditions 4x13.2kbps (with and without FER), since the CuT conditions are not being compared to this bitrate according to [2].

Note: Each experiment should be thoroughly reviewed for possible errors.

- Proposal

P.SUPPL800 Experiments

Experiment P800-1: Stereo Clean Speech Test

[

Experiment setup

Tables E.1.1 to E.1.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

Table E.1.1: Factors and conditions for Experiment P800-1

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Operating modes | [6] | 13.2, 16.4, 24.4, 32, 48 [24.4 kbps with DTX on at 0% FER/ 13.2 kbps with DTX on at 5% FER] |
| DTX | 0 | DTX off |
| Input level | 1 | -26 LKFS |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, 5% [1 delay/jitter profile] |
| Codec references |  |  |
| Codec references | 7 | EVS dual mono |
| Input level DTX | 1 0 | -26 LKFS DTX off |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, 5% [1 delay/jitter profile] |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| P.50 MNRU ESDRU | 4 3 | Q=16, 20, 24, 28 dB α = 0.1, 0.4, 0.7 |
| Input frequency mask | 1 | HP50 |
| Common Conditions |  |  |
| Test item generation: pre-processing incl. spatialization | 1 | Model-based relying on convolution of raw mono clean speech sentences with Room Impulse Responses respective to various talker positions relative to a capture point as described in the ITU-T Reverberation Tool [27] |
| Audio sampling frequency/bandwidth | 1 | 32 kHz/maximum available audio bandwidth up to SWB |
| Kind of samples | 1 | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 | 6 Different environments and talker interactions |
| Number of samples | 7 | 6 + 1 (preliminaries) samples per category |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR |
| Listening System | 1 | Headphones, in accordance with clause 4.6 |
| Listening Environment | 1 | No room noise, in accordance with clause 4.6 |
| Languages | 2 | [tbd] |

Table E.1.2: Preliminaries for Experiment P800-1

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | EVS | 2x13.2 | [5%, profile] |
| 2 | c02 |  | MNRU Q=28 dB | - | - |
| 3 | c06 |  | ESDRU α = 0.7 | - | - |
| 4 | c13 |  | EVS | 2x16.4 | No errors |
| 5 | c08 |  | ESDRU α = 0.1 | - | - |
| 6 | c16 |  | EVS | 2x7.2 | [5%, profile] |
| 7 | c04 |  | MNRU Q=20 dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c15 |  | EVS | 2x32 | No errors |
| 10 | c07 |  | ESDRU α = 0.4 | - | - |
| 11 | c05 |  | MNRU Q=16 dB | - | - |
| 12 | c21 |  | EVS | 2x24.4 | [5%, profile] |

Table E.1.3: Test conditions for Experiment P800-1,
clean speech under clean and impaired channel conditions [including delay/jitter profiles]

| Label | Condition | Bitrate [kbps] | FER/Profile | ToR |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=28 dB | - | - |  |
| c03 | MNRU Q=24 dB | - | - |  |
| c04 | MNRU Q=20 dB | - | - |  |
| c05 | MNRU Q=16 dB | - | - |  |
| c06 | ESDRU α=0.7 | - | - |  |
| c07 | ESDRU α=0.4 | - | - |  |
| c08 | ESDRU α=0.1 | - | - |  |
| c09 | EVS | 2x7.2 | No errors |  |
| c10 | EVS | 2x8 | No errors |  |
| c11 | EVS | 2x9.6 | No errors |  |
| c12 | EVS | 2x13.2 | No errors |  |
| c13 | EVS | 2x16.4 | No errors |  |
| c14 | EVS | 2x24.4 | No errors |  |
| c15 | EVS | 2x32 | No errors |  |
| c16 | EVS | 2x7.2 | [5%, profile] |  |
| c17 | EVS | 2x8 | [5%, profile] |  |
| c18 | EVS | 2x9.6 | [5%, profile] |  |
| c19 | EVS | 2x13.2 | [5%, profile] |  |
| c20 | EVS | 2x16.4 | [5%, profile] |  |
| c21 | EVS | 2x24.4 | [5%, profile] |  |
| c22 | EVS | 2x32 | [5%, profile] |  |
| c23 | CuT | 13.2 | No errors | NWT c10 OR BT c09 |
| c24 | CuT | 16.4 | No errors | NWT c11 OR BT c10 |
| c25 | CuT | 24.4 | No errors | NWT c12 OR BT c11 |
| c26 | CuT | 32 | No errors | NWT c13 OR BT c12 |
| c27 | CuT | 48 | No errors | NWT c15 OR BT c14 |
| c28 | CuT | 13.2 | [5%, profile] | NWT c17 OR BT c16 |
| c29 | CuT | 16.4 | [5%, profile] | NWT c18 OR BT c17 |
| c30 | CuT | 24.4 | [5%, profile] | NWT c19 OR BT c18 |
| c31 | CuT | 32 | [5%, profile] | NWT c20 OR BT c19 |
| c32 | CuT | 48 | [5%, profile] | NWT c22 OR BT c21 |
| c33 | CuT | 24.4 DTX on | No errors | NWT c12 OR BT c11 |
| c34 | CuT | 13.2 DTX on | 5% | NWT c17 OR BT c16 |

Content type categories and scene definitions (Exp P800-1: clean speech)

| Category | Room(1 | Reverb | Microphone Setup | Level [dB] | Overtalk [s](2 | Bandwidth( | Talker positions(3 | Talker selection by panel(4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | small | anechoic | A-B (100cm) | -26 | 1 | Max [available up to SWB] | Random | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | large | anechoic | A-B (150cm) | -26 | -1 | max [available up to SWB] | Random | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | small | anechoic | M-S | -26 | -1 | max [available up to SWB] | Random | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | small | echoic | A-B (100cm) | -26 | -1 | max [available up to SWB] | Random | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | large | echoic | A-B (150cm) | -26 | 1 | max [available up to SWB] | Random | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | large | echoic | Binaural | -26 | -1 | max [available up to SWB] | Random | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

Notes:

(1 The specific room characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences.

(2 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(3 The talker positions are part of the scene definition of the different categories. They should be chosen in a random way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different random selections should be made for the different listener panels. The details will be specified in the IVAS processing plan IVAS-7a.

(4 All sentences by the 6 talkers shall be unique.

Editor’s note: Reference conditions (following IVAS-3) missing for the CuT conditions in square brackets.

Editor’s note: Given the limited number of samples, deterministic positions might better cover the intended scenarios that “Random” positions.

Editor’s note: FER proposal need to be aligned with Section 5.

]

Experiment P800-2: Stereo Speech and Background Test

[

Experiment setup

Tables E.2.1 to E.2.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

Table E.2.1: Factors and conditions for Experiment P800-2

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Operating modes | 5 | 13.2, 16.4, 24.4, 32, 48 |
| DTX | 2 | DTX off, DTX on |
| Input level | 1 | -26 LKFS |
| Input frequency mask | 1 | HP50 |
| Noise | 1 | 15dB |
| Error Conditions | 0 | No errors |
| Codec references |  |  |
| Codec references | 7 | EVS dual mono |
| Input level DTX | 1 2 | -26 LKFS DTX off, DTX on |
| Input frequency mask | 1 | HP50 |
| Noise | 1 | 15dB |
| Error Conditions | 0 | No errors |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| P.50 MNRU ESDRU | 4 3 | Q=12, 17, 23, 28 dB α = 0.1, 0.4, 0.7 |
| Input frequency mask | 1 | HP50 |
| Common Conditions |  |  |
| Test item generation: pre-processing incl. spatialization | 1 | Model-based relying on convolution of raw mono clean speech sentences with Room Impulse Responses respective to various talker positions relative to a capture point as described in the ITU-T Reverberation Tool [27] |
| Audio sampling frequency/bandwidth | 1 | 32 kHz/maximum available audio bandwidth up to SWB |
| Kind of samples | 1 | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 | 6 Different environments with different background types (car,office, street) and talker interactions |
| Number of samples | 7 | 6 + 1 (preliminaries) samples per category |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR |
| Listening System | 1 | Headphones, in accordance with clause 4.6 |
| Listening Environment | 1 | No room noise, in accordance with clause 4.6 |
| Languages | 2 | [tbd] |

Table E.2.2: Preliminaries for Experiment P800-2

| Trial # | Label | Sample | Condition | Bitrate | DTX |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | EVS | 2x13.2 | on |
| 2 | c02 |  | MNRU Q=28 dB | - | - |
| 3 | c06 |  | ESDRU α = 0.7 | - | - |
| 4 | c13 |  | EVS | 2x16.4 | off |
| 5 | c08 |  | ESDRU α = 0.1 | - | - |
| 6 | c16 |  | EVS | 2x7.2 | on |
| 7 | c04 |  | MNRU Q=20 dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c15 |  | EVS | 2x32 | off |
| 10 | c07 |  | ESDRU α = 0.4 | - | - |
| 11 | c05 |  | MNRU Q=16 dB | - | - |
| 12 | c21 |  | EVS | 2x24.4 | on |

Table E.2.3: Test conditions for Experiment P800-2,
clean speech with background noise under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX | ToR |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=28 dB | - | - |  |
| c03 | MNRU Q=24 dB | - | - |  |
| c04 | MNRU Q=20 dB | - | - |  |
| c05 | MNRU Q=16 dB | - | - |  |
| c06 | ESDRU α=0.7 | - | - |  |
| c07 | ESDRU α=0.4 | - | - |  |
| c08 | ESDRU α=0.1 | - | - |  |
| c09 | EVS | 2x7.2 | off |  |
| c10 | EVS | 2x8 | off |  |
| c11 | EVS | 2x9.6 | off |  |
| c12 | EVS | 2x13.2 | off |  |
| c13 | EVS | 2x16.4 | off |  |
| c14 | EVS | 2x24.4 | off |  |
| c15 | EVS | 2x32 | off |  |
| c16 | EVS | 2x7.2 | on |  |
| c17 | EVS | 2x8 | on |  |
| c18 | EVS | 2x9.6 | on |  |
| c19 | EVS | 2x13.2 | on |  |
| c20 | EVS | 2x16.4 | on |  |
| c21 | EVS | 2x24.4 | on |  |
| c22 | EVS | 2x32 | on |  |
| c23 | CuT | 13.2 | off | NWT c10 OR BT c09 |
| c24 | CuT | 16.4 | off | NWT c11 OR BT c10 |
| c25 | CuT | 24.4 | off | NWT c12 OR BT c11 |
| c26 | CuT | 32 | off | NWT c13 OR BT c12 |
| c27 | CuT | 48 | off | NWT c15 OR BT c14 |
| c28 | CuT | 13.2 | on | NWT c17 OR BT c16 |
| c29 | CuT | 16.4 | on | NWT c18 OR BT c17 |
| c30 | CuT | 24.4 | on | NWT c19 OR BT c18 |
| c31 | CuT | 32 | on | NWT c20 OR BT c19 |
| c32 | CuT | 48 | on | NWT c22 OR BT c21 |

Content type categories and scene definitions (Exp P800-2: noisy speech)

| Category | Room(1 | Reverb | Microphone Setup | Level [dB] | Background | SNR In dB | Overtalk [s](2 | Bandwidth( | Talker positions(3 | Talker selection by panel(4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | small | anechoic | A-B (100cm) | -26 | car | 15 | 1 | Max [available up to SWB] | Random | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | large | anechoic | A-B (150cm) | -26 | office | 15 | -1 | max [available up to SWB] | Random | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | small | anechoic | M-S | -26 | street | 15 | -1 | max [available up to SWB] | Random | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | small | echoic | A-B (100cm) | -26 | street | 15 | -1 | max [available up to SWB] | Random | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | large | echoic | A-B (150cm) | -26 | office | 15 | 1 | max [available up to SWB] | Random | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | large | echoic | Binaural | -26 | car | 15 | -1 | max [available up to SWB] | Random | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

Notes:

(1 The specific room characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences.

(2 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(3 The talker positions are part of the scene definition of the different categories. They should be chosen in a random way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different random selections should be made for the different listener panels. The details will be specified in the IVAS processing plan IVAS-7a.

(5 All sentences by the 6 talkers shall be unique.

Editor’s note: Given the limited number of samples, deterministic positions might better cover the intended scenarios that “Random” positions.

]

Experiment P800-3: Stereo Mixed and Music Test

[

The purpose of this experiment is to evaluate the performances of the IVAS candidate algorithm with respect to EVS reference conditions in FB mixed content and music for clean and impaired channel conditions [including delay/jitter].

Experiment setup

Tables E.3.1 to E.3.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

Table E.3.1: Factors and conditions for Experiment P800-3

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Operating modes | [6] | 13.2, 16.4, 24.4, 32, 48 [24.4 kbps with DTX on at 0% FER/ 13.2 kbps with DTX on at 5% FER] |
| DTX | 0 | DTX off |
| Input level | 1 | -26 LKFS |
| Input frequency mask | 0 | 20KBP |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, [5% random frame erasures / 1 delay/jitter profile] |
| Codec references |  |  |
| Codec references | 7 | EVS dual mono |
| Input level DTX | 1 0 | -26 LKFS DTX off |
| Input frequency mask | 0 | 20KBP |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, [5% random frame erasures / 1 delay/jitter profile] |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| P.50 MNRU ESDRU | 4 3 | Q = 12, 17, 23, 28 dB α = 0.1, 0.4, 0.7 |
| Input frequency mask | 0 | 20KBP |
| Common Conditions |  |  |
| Audio sampling frequency/bandwidth | 1 | 48 kHz/maximum available audio bandwidth up to FB |
| Number of categories | 6 | 3 mixed content and 3 music (See clause 4.5.3) |
| Number of samples | 7 | 6 + 1 (preliminaries) samples per category |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR |
| Listening System | 1 | Headphones, in accordance with clause 4.6 |
| Listening Environment | 1 | No room noise, in accordance with clause 4.6 |

Table E.3.2: Preliminaries for Experiment P800-3

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | EVS | 2x13.2 | [5%, profile] |  |
| 2 | c02 |  | MNRU Q=28 dB | - | - |  |
| 3 | c06 |  | ESDRU α = 0.7 | - | - |  |
| 4 | c13 |  | EVS | 2x16.4 | No errors |  |
| 5 | c08 |  | ESDRU α = 0 | - | - |  |
| 6 | c16 |  | EVS | 2x7.2 | [5%, profile] |  |
| 7 | c04 |  | MNRU Q=23 dB | - | - |  |
| 8 | c01 |  | Reference | - | - |  |
| 9 | c15 |  | EVS | 2x32 | No errors |  |
| 10 | c07 |  | ESDRU α = 0.35 | - | - |  |
| 11 | c05 |  | MNRU Q=12 dB | - | - |  |
| 12 | c21 |  | EVS | 2x24.4 | [5%, profile] |  |

Table E.3.3: Test conditions for Experiment P800-3,
mixed contents and music under clean and impaired channel conditions [including delay/jitter profiles]

| Label | Condition | Bitrate [kbps] | FER/Profile | ToR |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=28 dB | - | - |  |
| c03 | MNRU Q=23 dB | - | - |  |
| c04 | MNRU Q=17 dB | - | - |  |
| c05 | MNRU Q=12 dB | - | - |  |
| c06 | ESDRU α=0.7 | - | - |  |
| c07 | ESDRU α=0.4 | - | - |  |
| c08 | ESDRU α=0.1 | - | - |  |
| c09 | EVS | 2x7.2 | No errors |  |
| c10 | EVS | 2x8 | No errors |  |
| c11 | EVS | 2x9.6 | No errors |  |
| c12 | EVS | 2x13.2 | No errors |  |
| c13 | EVS | 2x16.4 | No errors |  |
| c14 | EVS | 2x24.4 | No errors |  |
| c15 | EVS | 2x32 | No errors |  |
| c16 | EVS | 2x7.2 | [5%, profile] |  |
| c17 | EVS | 2x8 | [5%, profile] |  |
| c18 | EVS | 2x9.6 | [5%, profile] |  |
| c19 | EVS | 2x13.2 | [5%, profile] |  |
| c20 | EVS | 2x16.4 | [5%, profile] |  |
| c21 | EVS | 2x24.4 | [5%, profile] |  |
| c22 | EVS | 2x32 | [5%, profile] |  |
| c23 | CuT | 13.2 | No errors | NWT c10 OR BT c09 |
| c24 | CuT | 16.4 | No errors | NWT c11 OR BT c10 |
| c25 | CuT | 24.4 | No errors | NWT c12 OR BT c11 |
| c26 | CuT | 32 | No errors | NWT c13 OR BT c12 |
| c27 | CuT | 48 | No errors | NWT c15 OR BT c14 |
| c28 | CuT | 13.2 | [5%, profile] | NWT c17 OR BT c16 |
| c29 | CuT | 16.4 | [5%, profile] | NWT c18 OR BT c17 |
| c30 | CuT | 24.4 | [5%, profile] | NWT c19 OR BT c18 |
| c31 | CuT | 32 | [5%, profile] | NWT c20 OR BT c19 |
| c32 | CuT | 48 | [5%, profile] | NWT c22 OR BT c21 |
| c33 | CuT | 24.4 DTX on | No errors | NWT c12 OR BT c11 |
| c34 | CuT | 13.2 DTX on | 5% | NWT c17 OR BT c16 |

]

Experiment P800-4: FOA Clean Speech Test

[

Experiment setup

Tables E.4.1 to E.4.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

Table E.4.1: Factors and conditions for Experiment P800-4 (Clean Speech)

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Operating modes | 7 | Option1: IVAS candidate operated at [16.4, 24.4, 32, 48, 64, 80 and 96 kbps] Option2: IVAS candidate operated at [13.2, 16.4, 24.4, 32, 48, 64 and 80 kbps] |
| DTX | 0 | DTX off |
| Input level | 1 | -26 LKFS |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, 5% [1 delay/jitter profile] |
| Codec references |  |  |
| Codec references | 7 | Multi-mono EVS |
| Input level DTX | 1 0 | -26 LKFS DTX off |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, 5% [1 delay/jitter profile] |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| P.50 MNRU ESDRU | 4 3 | Q=[32, 27, 22, 17] dB α = [0.8, 0.675, 0.55] |
| Input frequency mask | 1 | HP50 |
| Common Conditions |  |  |
| Test item generation: pre-processing incl. spatialization | 1 | Model-based relying on convolution of raw mono clean speech sentences convolved with (FOA) Spatial Room Impulse Responses respective various talker positions relative to a capture point and spatial (FOA) ambient noise mixing. |
| Binaural renderer | 1 | FOA to binaural (external) rendering according to [tbd] |
| Audio sampling frequency/bandwidth | 1 | 48 kHz/maximum available audio bandwidth |
| Kind of samples | 1 | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 | 6 Different environments and talker interactions |
| Number of samples | 7 | 6 + 1 (preliminaries) samples per category |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR |
| Listening System | 1 | Headphones, in accordance with clause 4.6 |
| Listening Environment | 1 | No room noise, in accordance with clause 4.6 |
| Languages | 2 | [tbd] |

Table E.4.2: Preliminaries for Experiment P800-4

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | EVS | 4x13.2 | [5%, profile] |
| 2 | c02 |  | MNRU Q=32 dB | - | - |
| 3 | c06 |  | ESDRU α = 0.8 | - | - |
| 4 | c13 |  | EVS | 4x16.4 | No errors |
| 5 | c08 |  | ESDRU α = 0.55 | - | - |
| 6 | c16 |  | EVS | 4x7.2 | [5%, profile] |
| 7 | c04 |  | MNRU Q=22 dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c15 |  | EVS | 4x32 | No errors |
| 10 | c07 |  | ESDRU α = 0.675 | - | - |
| 11 | c05 |  | MNRU Q=17 dB | - | - |
| 12 | c21 |  | EVS | 4x24.4 | [5%, profile] |

Table E.4.3: Test conditions for Experiment P800-4,
clean speech under clean and impaired channel conditions [including delay/jitter profiles]

| Label | Condition | Bitrate [kbps] | FER/Profile | ToR |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=32 dB | - | - |  |
| c03 | MNRU Q=27 dB | - | - |  |
| c04 | MNRU Q=22 dB | - | - |  |
| c05 | MNRU Q=17 dB | - | - |  |
| c06 | ESDRU α=0.8 | - | - |  |
| c07 | ESDRU α=0.675 | - | - |  |
| c08 | ESDRU α=0.55 | - | - |  |
| c09 | EVS | 4x7.2 | No errors |  |
| c10 | EVS | 4x8 | No errors |  |
| c11 | EVS | 4x9.6 | No errors |  |
| c12 | EVS | 4x13.2 | No errors |  |
| c13 | EVS | 4x16.4 | No errors |  |
| c14 | EVS | 4x24.4 | No errors |  |
| c15 | EVS | 4x32 | No errors |  |
| c16 | EVS | 4x7.2 | [5%, profile] |  |
| c17 | EVS | 4x8 | [5%, profile] |  |
| c18 | EVS | 4x9.6 | [5%, profile] |  |
| c19 | EVS | 4x13.2 | [5%, profile] |  |
| c20 | EVS | 4x16.4 | [5%, profile] |  |
| c21 | EVS | 4x24.4 | [5%, profile] |  |
| c22 | EVS | 4x32 | [5%, profile] |  |
| c23 | CuT | 16.4 | No errors |  |
| c24 | CuT | 24.4 | No errors |  |
| c25 | CuT | 32 | No errors |  |
| c26 | CuT | 48 | No errors |  |
| c27 | CuT | 64 | No errors |  |
| c28 | CuT | 80 | No errors |  |
| c29 | CuT | 96 | No errors |  |
| c30 | CuT | 16.4 | [5%, profile] |  |
| c31 | CuT | 24.4 | [5%, profile] |  |
| c32 | CuT | 32 | [5%, profile] |  |
| c33 | CuT | 48 | [5%, profile] |  |
| c34 | CuT | 64 | [5%, profile] |  |
| c35 | CuT | 80 | [5%, profile] |  |
| c36 | CuT | 96 | [5%, profile] |  |

Content type categories and scene definitions (Exp P800-4: clean speech)

| Category | Room(1 | Reverb | Level [dB] | Background(2 | Snr [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | [dry room] | low | -26 | [low_office] | 50 | 1 | Max | According to IVAS-7a | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | [dry room] | low | -26 | [low_office] | 50 | -1 | Max | According to IVAS-7a | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | [small conf room] | medium | -26 | [low_office] | 50 | 1 | Max | According to IVAS-7a | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | [small conf room] | medium | -26 | [low_office] | 50 | -1 | Max | According to IVAS-7a | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | [open office space] | high | -26 | [low_office] | 50 | 1 | Max | According to IVAS-7a | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | [open office space] | high | -26 | [low_office] | 50 | -1 | Max | According to IVAS-7a | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

Notes:

(1 The specific room characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8a. In the present contribution, these are just examples.

(2 Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8a. In the present contribution, ‘low_office’ is just an example of a potential background with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels. The details will be specified in the IVAS processing plan IVAS-7a.

(5 All sentences by the 6 talkers shall be unique.

Editor’s note: HD 650 are open-back headphones which may cause issue for P.SUPPL800 listening, depending how acoustically isolated are the listening booths. An alternative that was mentioned were  Beyer Dynamic DT 770 Pro.

Editor’s note: FER proposal need to be aligned with Section 5.

Editor’s note: The 50 dB Snr needs clarification.

Editor’s note: The current working assumption is to specify the scenarios in IVAS-8a rather than in IVAS-7a.

]

Experiment P800-5: FOA Speech and background Test

[

Table E.5.1: Factors and conditions for Experiment P800-5

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Operating modes | 8 | 13.2, 16.4, 24.4, 32, 48, 64, 80, 96 |
| DTX | 2 | DTX off, DTX on |
| Input level | 1 | -26 LKFS |
| Input frequency mask | 1 | HP50 |
| Noise | 1 | 15dB |
| Error Conditions | 0 | No errors |
| Codec references |  |  |
| Codec references | 8 | Multi-mono EVS |
| Input level DTX | 1 2 | -26 LKFS DTX off, DTX on |
| Input frequency mask | 1 | HP50 |
| Noise | 1 | 15dB |
| Error Conditions | 0 | No errors |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| P.50 MNRU ESDRU | 4 3 | Q=[32, 27, 22, 17] dB α =[ 0.8, 0.675, 0.55] |
| Input frequency mask | 1 | HP50 |
| Common Conditions |  |  |
| Test item generation: pre-processing incl. spatialization | 1 | Model-based relying on convolution of raw mono clean speech sentences convolved with (FOA) Spatial Room Impulse Responses respective various talker positions relative to a capture point and spatial (FOA) ambient noise mixing |
| Binaural renderer | 1 | FOA to binaural (external) rendering according to [tbd] |
| Audio sampling frequency/bandwidth | 1 | 48 kHz/maximum available audio bandwidth |
| Kind of samples | 1 | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 | 6 Different environments with different background types (car,office, street) and talker interactions |
| Number of samples | 7 | 6 + 1 (preliminaries) samples per category |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR |
| Listening System | 1 | Headphones, in accordance with clause 4.6 |
| Listening Environment | 1 | No room noise, in accordance with clause 4.6 |
| Languages | 2 | [tbd] |

Table E.5.2: Preliminaries for Experiment P800-5

| Trial # | Label | Sample | Condition | Bitrate | DTX |
| --- | --- | --- | --- | --- | --- |
| 1 | c20 |  | EVS | 4x13.2 | on |
| 2 | c02 |  | MNRU Q=28 dB | - | - |
| 3 | c06 |  | ESDRU α = 0.7 | - | - |
| 4 | c14 |  | EVS | 4x16.4 | off |
| 5 | c08 |  | ESDRU α = 0.1 | - | - |
| 6 | c17 |  | EVS | 4x7.2 | on |
| 7 | c04 |  | MNRU Q=20 dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c16 |  | EVS | 4x32 | off |
| 10 | c07 |  | ESDRU α = 0.4 | - | - |
| 11 | c05 |  | MNRU Q=16 dB | - | - |
| 12 | c22 |  | EVS | 4x24.4 | on |

Table E.5.3: Test conditions for Experiment P800-5,
clean speech with background noise under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX | ToR |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=28 dB | - | - |  |
| c03 | MNRU Q=24 dB | - | - |  |
| c04 | MNRU Q=20 dB | - | - |  |
| c05 | MNRU Q=16 dB | - | - |  |
| c06 | ESDRU α=0.7 | - | - |  |
| c07 | ESDRU α=0.4 | - | - |  |
| c08 | ESDRU α=0.1 | - | - |  |
| c09 | EVS | 3x7.2 | off |  |
| c10 | EVS | 4x7.2 | off |  |
| c11 | EVS | 4x8 | off |  |
| c12 | EVS | 4x9.6 | off |  |
| c13 | EVS | 4x13.2 | off |  |
| c14 | EVS | 4x16.4 | off |  |
| c15 | EVS | 4x24.4 | off |  |
| c16 | EVS | 4x32 | off |  |
| c17 | EVS | 4x7.2 | on |  |
| c18 | EVS | 4x8 | on |  |
| c19 | EVS | 4x9.6 | on |  |
| c20 | EVS | 4x13.2 | on |  |
| c21 | EVS | 4x16.4 | on |  |
| c22 | EVS | 4x24.4 | on |  |
| c23 | CuT | 13.2 | off | NWT c09 |
| c24 | CuT | 16.4 | off | NWT c09 |
| c25 | CuT | 24.4 | off | NWT c10 |
| c26 | CuT | 32 | off | NWT c12 or BT c11 |
| c27 | CuT | 48 | off | NWT c14 or BT c13 |
| c28 | CuT | 64 | off | NWT c15 or BT c14 |
| c29 | CuT | 80 | off | NWT c15 or BT c14 |
| c30 | CuT | 96 | off | NWT c16 or BT c15 |
| c28 | CuT | 16.4 | on |  |
| c29 | CuT | 24.4 | on |  |
| c30 | CuT | 32 | on |  |
| c31 | CuT | 48 | on |  |
| c32 | CuT | 64 | on |  |
| c33 | CuT | 80 | on |  |

Content type categories and scene definitions (Exp P800-5: speech and background)

| Category | Room(1 | Reverb | Level [dB] | Background(2 | Snr [dB] | Overtalk [s](3 | Bandwidth(3 | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | [car] | low | -26 | [car noise] | 15 | -1 | Max | According to IVAS-7a | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | [open air] | low | -26 | [Street] | 15 | -1 | Max | According to IVAS-7a | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | [mid size room] | medium | -26 | [Music (club)] | 15 | -1 | Max | According to IVAS-7a | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | [mid size room] | medium | -26 | [shopping mall] | 15 | -1 | Max | According to IVAS-7a | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | [open office space] | high | -26 | [office noise] | 15 | -1 | Max | According to IVAS-7a | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | [open office space] | high | -26 | [cafeteria] | 15 | -1 | Max | According to IVAS-7a | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

Notes:

(1 The specific room characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8a. In the present contribution, these are just examples.

(2 Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8a. In the present contribution, the given examples represent a meaningful choice exposing the codec to realistic background sound scenarios.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels. The details will be specified in the IVAS processing plan IVAS-7a.

(5 All sentences by the 6 talkers shall be unique.

Editor’s note: HD 650 are open-back headphones which may cause issue for P.SUPPL800 listening, depending how acoustically isolated are the listening booths. An alternative that was mentioned were  Beyer Dynamic DT 770 Pro.

Editor’s note:Types of background need to be aligned with Section 4.5.2.

Editor’s note: The current working assumption is to specify the scenarios in IVAS-8a rather than in IVAS-7a.

]

Experiment P800-6: 1-Object Clean Speech Test

[

The purpose of this experiment is to evaluate the performances of the IVAS candidate for encoding, decoding, and rendering of a single object with metadata. The metadata determines the position of the object around the listener in a 3-dimensional space as described in the Scene description section below. The audio is rendered binaurally via headphones.

Database

48 kHz sampled speech.

P.800 sentence pairs, 8 s long.

- Note: P.800 sentence pairs following [13] are assumed in this experiment, including the leading and trailing silence of minimum of 0.5 s. The metadata corresponds to the whole duration of 8 s. This means that for moving objects, only a part of the trajectory corresponds to active speech.

Artificially created spatial samples following the Scene descriptions below.

Level adjusted as specified in IVAS-7a.

Descriptions of different scenes for metadata creation

Talker sitting at a table (elevation 0°), at different azimuths:

| Panel | Talker | Azimuth |
| --- | --- | --- |
| 1 | G1 | 0° |
| 2 | G6 | 60° |
| 3 | G5 | 120° |
| 4 | G4 | 180° |
| 5 | G3 | 240° |
| 6 | G2 | 300° |

Standing talker (elevation 35°), at different azimuths:

| Panel | Talker | Azimuth |
| --- | --- | --- |
| 1 | G2 | 120° |
| 2 | G1 | 180° |
| 3 | G6 | 240° |
| 4 | G5 | 300° |
| 5 | G4 | 0° |
| 6 | G3 | 60° |

Smaller talker (child) walking around a table in the positive sense (counter clockwise), elevation 0°. Azimuth varies continuously for the sentence pair to cover the whole circle starting at:

| Panel | Talker | Starting Azimuth |
| --- | --- | --- |
| 1 | G3 | 0° |
| 2 | G2 | 60° |
| 3 | G1 | 120° |
| 4 | G6 | 180° |
| 5 | G5 | 240° |
| 6 | G3 | 300° |

Adult talker walking around a table in the negative sense (clockwise), elevation 35°. Azimuth varies continuously for the sentence pair to cover the whole circle starting at:

| Panel | Talker | Starting Azimuth |
| --- | --- | --- |
| 1 | G4 | 0° |
| 2 | G3 | 60° |
| 3 | G2 | 120° |
| 4 | G1 | 180° |
| 5 | G6 | 240° |
| 6 | G5 | 300° |

Elevation displacement: Elevation varies continuously for the sentence pair to cover the interval of -90° to 90°. Azimuth is constant for a sentence pair, but different for different for each sentence pair:

| Panel | Talker | Azimuth |
| --- | --- | --- |
| 1 | G5 | 240° |
| 2 | G4 | 300° |
| 3 | G3 | 0° |
| 4 | G2 | 60° |
| 5 | G1 | 120° |
| 6 | G6 | 180° |

Azimuth and elevation displacement. Azimuth and elevation vary continuously. Azimuth varies in positive sense (counter clockwise) to cover the range of 180°, with different starting azimuth for each sentence pair. Elevation varies in negative sense, from 35° to -35°:

| Panel | Talker | Starting Azimuth |
| --- | --- | --- |
| 1 | G6 | 60° |
| 2 | G5 | 120° |
| 3 | G4 | 180° |
| 4 | G3 | 240° |
| 5 | G2 | 300° |
| 6 | G1 | 0° |

Allocation of scenes to talkers to each listening panel

The following table assumes that test Categories correspond to different talkers. Each of the sentences uttered by a certain talker is encoded using different scene. To balance the test, in addition to listeners of each panel listening to all talkers (Categories), and all scenes are also covered in each panel. An example of allocation of sentences to each panel is given in Table below:

Table E.6.1: Allocation of scenes to talkers to each listening panel

| Panel | G1 (M1) | G2 (F1) | G3 (M2) | G4 (F2) | G5 (M3) | G6 (F3) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | S1 (Sc a) | S1 (Sc b) | S1 (Sc c) | S1 (Sc d) | S1 (Sc e) | S1 (Sc f) |
| 2 | S2 (Sc b) | S2 (Sc c) | S2 (Sc d) | S2 (Sc e) | S2 (Sc f) | S2 (Sc a) |
| 3 | S3 (Sc c) | S3 (Sc d) | S3 (Sc e) | S3 (Sc f) | S3 (Sc a) | S3 (Sc b) |
| 4 | S4 (Sc d) | S4 (Sc e) | S4 (Sc f) | S4 (Sc a) | S4 (Sc b) | S4 (Sc c) |
| 5 | S5 (Sc e) | S5 (Sc f) | S5 (Sc a) | S5 (Sc b) | S5 (Sc c) | S5 (Sc d) |
| 6 | S6 (Sc f) | S6 (Sc a) | S6 (Sc b) | S6 (Sc c) | S6 (Sc d) | S6 (Sc e) |

Experiment setup

Tables E.6.2 to E.6.4 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

The test Categories correspond to different talkers.

Table E.6.2: Factors and conditions for Experiment P800-6

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Bitrates | 3 | 13.2, 16.4, 24.4 and 32 kbps |
| DTX | 2 | DTX ON and OFF (32 kbps DTX OFF only) |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, 3% FERs |
| Codec references |  |  |
| Codec references | 1 | EVS |
| Bitrates | 3 | 13.2, 16.4, 24.4 and 32 kbps |
| DTX | 2 | DTX ON and OFF (32 kbps DTX OFF only) |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, 3% FERs |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| MNRU | 5 | Q= 15, 23, 31, 39, 47 dB |
| ESDRU | 4 | 0.1, 0.3, 0.5, 0.7 |
| Input frequency mask | 1 | HP50 |
| Common Conditions |  |  |
| Number of talkers (categories) | 6 | 3 male and 3 female |
| Number of speech samples | 7 | 6 + 1 (preliminaries) sentence pairs per talker |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations (panels) | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | P.SUPPL800 |
| Listening System | 1 | Headphones |

Table E.6.3: Preliminaries for Experiment P800-6

| Trial | Label | Sample | Condition | Bitrate [kb/s] | DTX | FER [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | EVS | 24.4 | Off | 3 |
| 2 |  |  | MNRU 31 | - | - | - |
| 3 |  |  | EVS | 13.2 | On | 0 |
| 4 |  |  | ESDRU 0.5 | - | - | - |
| 5 |  |  | EVS | 32 | Off | 0 |
| 6 |  |  | MNRU 15 | - | - | - |
| 7 |  |  | EVS | 16.4 | Off | 0 |
| 8 |  |  | Direct | - | - | - |
| 9 |  |  | EVS | 13.2 | Off | 3 |
| 10 |  |  | MNRU 47 | - | - | - |
| 11 |  |  | EVS | 24.4 | On | 0 |
| 12 |  |  | ESDRU 0.1 | - | - | - |

Table E.6.4 Test conditions for Experiment P800-6:

| Label | Condition | Bitrate [kb/s] | DTX | FER [%] | Ref condition | ToR |
| --- | --- | --- | --- | --- | --- | --- |
| C01 | Direct |  | - | - | - | - |
| C02 | MNRU 15 |  | - | - | - | - |
| C03 | MNRU 23 |  | - | - | - | - |
| C04 | MNRU 31 |  | - | - | - | - |
| C05 | MNRU 39 |  | - | - | - | - |
| C06 | MNRU 47 |  | - | - | - | - |
| C07 | ESDRU 0.1 |  | - | - | - | - |
| C08 | ESDRU 0.3 |  | - | - | - | - |
| C09 | ESDRU 0.5 |  | - | - | - | - |
| C10 | ESDRU 0.7 |  | - | - | - | - |
| C11 | EVS | 13.2 | Off | 0 | - | - |
| C12 | EVS | 16.4 | Off | 0 | - | - |
| C13 | EVS | 24.4 | Off | 0 | - | - |
| C14 | EVS | 32 | Off | 0 | - | - |
| C15 | EVS | 13.2 | Off | 3 | - | - |
| C16 | EVS | 16.4 | Off | 3 | - | - |
| C17 | EVS | 24.4 | Off | 3 | - | - |
| C18 | EVS | 13.2 | On | 0 | - | - |
| C19 | EVS | 16.4 | On | 0 | - | - |
| C20 | EVS | 24.4 | On | 0 | - | - |
| C21 | CuT | 13.2 | Off | 0 | C11 | NWT |
| C22 | CuT | 16.4 | Off | 0 | C12 | NWT |
| C23 | CuT | 24.4 | Off | 0 | C13 | NWT |
| C24 | CuT | 32 | Off | 0 | C14 | NWT |
| C25 | CuT | 13.2 | Off | 3 | C15 | NWT |
| C26 | CuT | 16.4 | Off | 3 | C16 | NWT |
| C27 | CuT | 24.4 | Off | 3 | C17 | NWT |
| C28 | CuT | 13.2 | On | 0 | C18 | NWT |
| C29 | CuT | 16.4 | On | 0 | C19 | NWT |
| C30 | CuT | 24.4 | On | 0 | C20 | NWT |

]

Experiment P800-7: 2-Objects Clean Speech Test

[

The purpose of this experiment is to evaluate the performances of the IVAS candidate for encoding, decoding, and rendering of two simultaneous objects with metadata. The metadata determines the position of the objects around the listener in a 3-dimensional space as described in the Scene description section below. The audio is rendered binaurally via headphones.

Database

48 kHz sampling rate

The listening database consists of artificially created spatial audio samples from monophonic clean speech recordings where always 1 female and 1 male talker are combined in conversation-like scenarios following the Scene descriptions below.

A leading and trailing silence is assumed for each artificially created spatial audio sample. The metadata corresponds to the whole duration of the sample. This means that for moving objects, only a part of the trajectory corresponds to active speech.

In half of the samples, the 2nd talker’s utterance follows the 1st talker’s utterance simulating natural conversation. In the 2nd half of the samples, the situation is similar, but the utterances partially overlap. The targeted overlap should be approximately [30] %. Non-overlapping sentence pairs are used for Scenes a., c., and e. as described below. Overlapping sentence pairs are used for Scenes b., d., and f.

The length of the created spatial audio samples is 6 s.

Level adjusted as specified in IVAS-7a (Processing Plan) [3].

Descriptions of different scenes for metadata creation

- Two talkers sitting at a table (elevation 0°), at different azimuths. To increase positional variation, both the absolute azimuths and the difference of the azimuths of both talkers vary for each sentence pair:

| Azimuth of 1st talker | Azimuth of 2nd talker |
| --- | --- |
| 0° | 50° |
| 10° | 110° |
| 20° | 170° |
| 30° | 230° |
| 40° | 290° |
| 50° | 350° |

Two standing talkers (elevation 35°), at different azimuths. To increase positional variation, both the absolute azimuths and the difference of the azimuths of both talkers vary for each sentence pair:

| Azimuth of 1st talker | Azimuth of 2nd talker |
| --- | --- |
| 0° | 50° |
| 10° | 110° |
| 20° | 170° |
| 30° | 230° |
| 40° | 290° |
| 50° | 350° |

One talker sitting at a table (elevation 0°), second talker standing beside the table (elevation 45°). Non-overlapping utterances:

| Azimuth of 1st talker | Azimuth of 2nd talker |
| --- | --- |
| 1st talker | 2nd talker |
| 0° | 50° |
| 10° | 110° |
| 20° | 170° |
| 30° | 230° |
| 40° | 290° |

One talker sitting at a table (elevation 0°), second talker walking around the table (elevation 45°). The azimuth of the 2nd talker varies continually, positive sense is counter clockwise:

| Azimuth of 1st talker | Azimuth of 2nd talker |
| --- | --- |
| 50° | 180° : +1° : 120° |
| 100° | 130° : +1° : 70° |
| 150° | 80° : +1° : 20° |
| 200° | 30° : -1° : -270° (90°) |
| 250° | -20° : -1° : -320° (40°) |
| 300° | -70° : -1° : -10° |

Two talkers walking side-by-side around the table (elevation 45°). The azimuth is the same for both talkers and varies continually:

Azimuth of the talkers

180° : +1° : 120°

130° : +1° : 70°

80° : +1° : 20°

30° : -1° : -270° (90°)

-20° : -1° : -320° (40°)

-70° : -1° : -10°

Two talkers walking around the table in opposite directions (elevation 30°), starting at the same position. Azimuths of both talkers vary continually:

| Azimuth of 1st talker | Azimuth of 2nd talker |
| --- | --- |
| 180° : +1° : 120° | 180° : -1° : -120° |
| 240° (-120°) : +1° : 180° | 240° (-120°) : -1° : -60° |
| 300° (-60°) : +1° : 240° | 300° (-60°) : -1° : 0° |
| 0° +1° : 300° | 0° : -1° : 60° |
| 60° : +1° : 0° | 60° : -1° : 120° |
| 120° : +1° : 60° | 120° : -1° : 180° |

Allocation of scenes to talkers to each listening panel

The following table assumes that test Categories correspond to different talker pairs. Each of the sentence pairs uttered by a certain talker pair is encoded using a different scene. To balance the test, listeners of each panel listen to all talker pairs, and all scenes are covered in each panel.

Table E.7.1: Allocation of scenes and talkers to each listening panel

| Panel / Categories | G1 (M1+F1) | G2 (M2+F2) | G3 (M3+F3) | G4 (F1+M2) | G5 F2+M3() | G6 (F3+M1) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | S1 (Sc a) | S1 (Sc b) | S1 (Sc c) | S1 (Sc d) | S1 (Sc e) | S1 (Sc f) |
| 2 | S2 (Sc b) | S2 (Sc c) | S2 (Sc d) | S2 (Sc e) | S2 (Sc f) | S2 (Sc a) |
| 3 | S3 (Sc c) | S3 (Sc d) | S3 (Sc e) | S3 (Sc f) | S3 (Sc a) | S3 (Sc b) |
| 4 | S4 (Sc d) | S4 (Sc e) | S4 (Sc f) | S4 (Sc a) | S4 (Sc b) | S4 (Sc c) |
| 5 | S5 (Sc e) | S5 (Sc f) | S5 (Sc a) | S5 (Sc b) | S5 (Sc c) | S5 (Sc d) |
| 6 | S6 (Sc f) | S6 (Sc a) | S6 (Sc b) | S6 (Sc c) | S6 (Sc d) | S6 (Sc e) |

Experiment setup

Tables E.7.2 to E.7.4 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

The test Categories correspond to different talkers.

Table E.7.2: Factors and conditions for Experiment P800-7

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Bitrates | 4 | 16.4, 24.4, 32, and 48 kbps |
| DTX | 2 | DTX ON and OFF |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, 3% FERs |
| Codec references |  |  |
| Codec references | 1 | EVS |
| Bitrates | 4 | 2 x 8.0, 2 x 13.2, 2 x 16.4, 2 x 24.4 kbps |
| DTX | 2 | DTX ON and OFF |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, 3% FERs |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| MNRU | 5 | Q= 15, 23, 31, 39, 47 dB |
| ESDRU | 4 | 0.1, 0.3, 0.5, 0.7 |
| Input frequency mask | 1 | HP50 |
| Common Conditions |  |  |
| Number of talkers (categories) | 6 | 3 male and 3 female |
| Number of speech samples | 7 | 6 + 1 (preliminaries) sentence pairs per talker. |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations (panels) | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | P.SUPPL800 |
| Listening System | 1 | Headphones |

Table E.7.3: Preliminaries for Experiment P800-7

| Trial | Label | Sample | Condition | Bitrate [kb/s] | DTX | FER [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | EVS | 2 x 16.4 | Off | 3 |
| 2 |  |  | MNRU 31 | - | - | - |
| 3 |  |  | EVS | 2 x 8.0 | On | 0 |
| 4 |  |  | ESDRU 0.5 | - | - | - |
| 5 |  |  | EVS | 2 x 24.4 | Off | 0 |
| 6 |  |  | MNRU 15 | - | - | - |
| 7 |  |  | EVS | 2 x 13.2 | Off | 0 |
| 8 |  |  | Direct | - | - | - |
| 9 |  |  | EVS | 2 x 8.0 | Off | 3 |
| 10 |  |  | MNRU 47 | - | - | - |
| 11 |  |  | EVS | 2 x 16.4 | On | 0 |
| 12 |  |  | ESDRU 0.1 | - | - | - |

Table E.7.4 Test conditions for Experiment P800-7:

| Label | Condition | Bitrate [kb/s] | DTX | FER [%] | Ref condition | ToR |
| --- | --- | --- | --- | --- | --- | --- |
| C01 | Direct | - | - | - | - | - |
| C02 | MNRU 15 | - | - | - | - | - |
| C03 | MNRU 23 | - | - | - | - | - |
| C04 | MNRU 31 | - | - | - | - | - |
| C05 | MNRU 39 | - | - | - | - | - |
| C06 | MNRU 47 | - | - | - | - | - |
| C07 | ESDRU 0.1 | - | - | - | - | - |
| C08 | ESDRU 0.3 | - | - | - | - | - |
| C09 | ESDRU 0.5 | - | - | - | - | - |
| C10 | ESDRU 0.7 | - | - | - | - | - |
| C11 | EVS | 2 x 8.0 | Off | 0 | - | - |
| C12 | EVS | 2 x 13.2 | Off | 0 | - | - |
| C13 | EVS | 2 x 16.4 | Off | 0 | - | - |
| C14 | EVS | 2 x 24.4 | Off | 0 | - | - |
| C15 | EVS | 2 x 8.0 | Off | 3 | - | - |
| C16 | EVS | 2 x 13.2 | Off | 3 | - | - |
| C17 | EVS | 2 x 16.4 | Off | 3 | - | - |
| C18 | EVS | 2 x 24.4 | Off | 3 | - | - |
| C19 | EVS | 2 x 8.0 | On | 0 | - | - |
| C20 | EVS | 2 x 13.2 | On | 0 | - | - |
| C21 | EVS | 2 x 16.4 | On | 0 | - | - |
| C22 | EVS | 2 x 24.4 | On | 0 |  |  |
| C23 | CuT | 16.4 | Off | 0 | C11 | NWT |
| C24 | CuT | 24.4 | Off | 0 | C12 | NWT |
| C25 | CuT | 32 | Off | 0 | C13 | NWT |
| C26 | CuT | 48 | Off | 0 | C14 | NWT |
| C27 | CuT | 16.4 | Off | 3 | C15 | NWT |
| C28 | CuT | 24.4 | Off | 3 | C16 | NWT |
| C29 | CuT | 32 | Off | 3 | C17 | NWT |
| C30 | CuT | 48 | Off | 3 | C18 | NWT |
| C31 | CuT | 16.4 | On | 0 | C19 | NWT |
| C32 | CuT | 24.4 | On | 0 | C20 | NWT |
| C33 | CuT | 32 | On | 0 | C21 | NWT |
| C34 | CuT | 48 | On | 0 | C22 | NWT |

]

Experiment P800-8: MASA Clean Speech Test

[

It is proposed to focus on stereo-MASA inputs in IVAS Codec selection tests.

Tables E.8.1 to E.8.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

Table E.8.1: Factors and conditions for Experiment P800-8 (Clean Speech)

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Operating modes | 7 | 13.2, 16.4, 24.4, 32, 48, 64, 80 |
| DTX | 0 | DTX off |
| Input level | 1 | -26 LKFS |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, 5% [1 delay/jitter profile] |
| Codec references |  |  |
| Codec references | 7+5 | Multi-mono EVS + Dual-mono EVS with unquantized metadata |
| Input level DTX | 1 0 | -26 LKFS DTX off |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, 5% [1 delay/jitter profile] |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| P.50 MNRU ESDRU | 4 3 | Q=[32, 27, 22, 17] dB α = [0.8, 0.675, 0.55] |
| Input frequency mask | 1 | HP50 |
| Common Conditions |  |  |
| Test item generation: pre-processing incl. spatialization | 1 | Model-based generation according to [tbd] |
| Binaural renderer | 1 | IVAS MASA C Reference Software binaural rendering [28] |
| Audio sampling frequency/bandwidth | 1 | 48 kHz/maximum available audio bandwidth |
| Kind of samples | 1 | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 | 6 Different environments and talker interactions |
| Number of samples | 7 | 6 + 1 (preliminaries) samples per category |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR |
| Listening System | 1 | Headphones, in accordance with clause 4.6 |
| Listening Environment | 1 | No room noise, in accordance with clause 4.6 |
| Languages | 2 | [tbd] |

Table E.8.2: Preliminaries for Experiment P800-8

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c25 |  | EVS | 4x13.2 | [5%, profile] |
| 2 | c02 |  | MNRU Q=32 dB | - | - |
| 3 | c06 |  | ESDRU α = 0.8 | - | - |
| 4 | c14 |  | EVS | 4x16.4 | No errors |
| 5 | c08 |  | ESDRU α = 0.55 | - | - |
| 6 | c21 |  | EVS | 4x7.2 | [5%, profile] |
| 7 | c04 |  | MNRU Q=22 dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c15 |  | EVS | 4x24.4 | No errors |
| 10 | c07 |  | ESDRU α = 0.675 | - | - |
| 11 | c05 |  | MNRU Q=17 dB | - | - |
| 12 | c21 |  | EVS | 4x24.4 | [5%, profile] |

Table E.8.3: Test conditions for Experiment P800-8,
clean speech under clean and impaired channel conditions [including delay/jitter profiles]

| Label | Condition | Bitrate [kbps] | FER/Profile | ToR |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=32 dB | - | - |  |
| c03 | MNRU Q=27 dB | - | - |  |
| c04 | MNRU Q=22 dB | - | - |  |
| c05 | MNRU Q=17 dB | - | - |  |
| c06 | ESDRU α=0.8 | - | - |  |
| c07 | ESDRU α=0.675 | - | - |  |
| c08 | ESDRU α=0.55 | - | - |  |
| c09 | EVS | 3x7.2 | No errors |  |
| c10 | EVS | 4x7.2 | No errors |  |
| c11 | EVS | 4x8 | No errors |  |
| c12 | EVS | 4x9.6 | No errors |  |
| c13 | EVS | 4x13.2 | No errors |  |
| c14 | EVS | 4x16.4 | No errors |  |
| c15 | EVS | 4x24.4 | No errors |  |
| c16 | EVS(+unquantized metdata) | 2x7.2 | No errors |  |
| c17 | EVS(+unquantized metdata) | 2x8 | No errors |  |
| c18 | EVS(+unquantized metdata) | 2x9.6 | No errors |  |
| c19 | EVS(+unquantized metdata) | 2x16.4 | No errors |  |
| c20 | EVS(+unquantized metdata) | 2x24.4 | No errors |  |
| c21 | EVS | 3x7.2 | [5%, profile] |  |
| c22 | EVS | 4x7.2 | [5%, profile] |  |
| c23 | EVS | 4x8 | [5%, profile] |  |
| c24 | EVS | 4x9.6 | [5%, profile] |  |
| c25 | EVS | 4x13.2 | [5%, profile] |  |
| c26 | EVS | 4x16.4 | [5%, profile] |  |
| c27 | EVS | 4x24.4 | [5%, profile] |  |
| c28 | CuT | 13.2 | No errors | NWT c09 OR NWT c16 |
| c29 | CuT | 16.4 | No errors | NWT c10 OR NWT c16 |
| c30 | CuT | 24.4 | No errors | NWT c11 OR NWT c17 |
| c31 | CuT | 32 | No errors | NWT c12 OR NWT c18 |
| c32 | CuT | 48 | No errors | NWT c14 OR NWT c19 |
| c33 | CuT | 64 | No errors | NWT c15 OR NWT c20 |
| c34 | CuT | 80 | No errors | NWT c15 OR NWT c20 |
| c35 | CuT | 13.2 | [5%, profile] | NWT c21 |
| c36 | CuT | 16.4 | [5%, profile] | NWT c22 |
| c37 | CuT | 24.4 | [5%, profile] | NWT c23 |
| c38 | CuT | 32 | [5%, profile] | NWT c24 |
| c39 | CuT | 48 | [5%, profile] | NWT c26 |
| c40 | CuT | 64 | [5%, profile] | NWT c27 |

Editor’s note: Rating scale should rather refer to the Annex D.

]

Experiment P800-9: MASA Speech and Background Test

[

It is proposed to focus on stereo-MASA inputs in IVAS Codec selection tests.

Tables E.8.1 to E.8.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

Table E.9.1: Factors and conditions for Experiment P800- 9 (Clean Speech)

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Operating modes | 7 | 13.2, 16.4, 24.4, 32, 48, 64, 80 |
| DTX | 2 | DTX off, DTX on |
| Input level | 1 | -26 LKFS |
| Input frequency mask | 1 | HP50 |
| Noise | 1 | 15dB |
| Error Conditions | 0 | No errors |
| Codec references |  |  |
| Codec references | 6+3 | Multi-mono EVS + Dual-mono with unquantized metadata |
| Input level DTX | 1 2 | -26 LKFS DTX off, DTX on |
| Input frequency mask | 1 | HP50 |
| Noise | 1 | 15dB |
| Error Conditions | 2 | No errors |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| P.50 MNRU ESDRU | 4 3 | Q=[32, 27, 22, 17] dB α = [0.8, 0.675, 0.55] |
| Input frequency mask | 1 | HP50 |
| Common Conditions |  |  |
| Test item generation: pre-processing incl. spatialization | 1 | Model-based generation according to [tbd] |
| Binaural renderer | 1 | IVAS MASA C Reference Software binaural rendering [28] |
| Audio sampling frequency/bandwidth | 1 | 48 kHz/maximum available audio bandwidth |
| Kind of samples | 1 | Sentence pair uttered by different talkers and genders (3 male and 3 female) |
| Number of categories | 6 | 6 Different environments and talker interactions |
| Number of samples | 7 | 6 + 1 (preliminaries) samples per category |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR |
| Listening System | 1 | Headphones, in accordance with clause 4.6 |
| Listening Environment | 1 | No room noise, in accordance with clause 4.6 |
| Languages | 2 | [tbd] |

Table E.9.2: Preliminaries for Experiment P800-8

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c19 |  | EVS | 4x13.2 | [5%, profile] |
| 2 | c02 |  | MNRU Q=32 dB | - | - |
| 3 | c06 |  | ESDRU α = 0.8 | - | - |
| 4 | c13 |  | EVS | 4x16.4 | No errors |
| 5 | c08 |  | ESDRU α = 0.55 | - | - |
| 6 | c16 |  | EVS | 4x7.2 | [5%, profile] |
| 7 | c04 |  | MNRU Q=22 dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c15 |  | EVS | 4x32 | No errors |
| 10 | c07 |  | ESDRU α = 0.675 | - | - |
| 11 | c05 |  | MNRU Q=17 dB | - | - |
| 12 | c21 |  | EVS | 4x24.4 | [5%, profile] |

Table E.9.3: Test conditions for Experiment P800-9,
speech and background under clean channel conditions

| Label | Condition | Bitrate [kbps] | DTX | ToR |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=32 dB | - | - |  |
| c03 | MNRU Q=27 dB | - | - |  |
| c04 | MNRU Q=22 dB | - | - |  |
| c05 | MNRU Q=17 dB | - | - |  |
| c06 | ESDRU α=0.8 | - | - |  |
| c07 | ESDRU α=0.675 | - | - |  |
| c08 | ESDRU α=0.55 | - | - |  |
| c09 | EVS | 3x7.2 | off |  |
| c10 | EVS | 4x7.2 | off |  |
| c11 | EVS | 4x8 | off |  |
| c12 | EVS | 4x9.6 | off |  |
| c13 | EVS | 4x16.4 | off |  |
| c14 | EVS | 4x24.4 | off |  |
| c15 | EVS(+unquantized metdata) | 2x7.2 | on |  |
| c16 | EVS(+unquantized metdata) | 2x9.6 | on |  |
| c17 | EVS(+unquantized metdata) | 2x16.4 | on |  |
| c18 | EVS | 3x7.2 | on |  |
| c19 | EVS | 4x7.2 | on |  |
| c20 | EVS | 4x8 | on |  |
| c21 | EVS | 4x9.6 | on |  |
| c22 | EVS | 4x16.4 | on |  |
| c23 | EVS | 4x24.4 | on |  |
| c24 | CuT | 13.2 | off | NWT c09 |  |
| c25 | CuT | 16.4 | off | NWT c10 |  |
| c26 | CuT | 24.4 | off | NWT c11 |  |
| c27 | CuT | 32 | off | NWT c12 |  |
| c28 | CuT | 48 | off | NWT c13 |  |
| c29 | CuT | 64 | off | NWT c14 |  |
| c30 | CuT | 80 | off | NWT c14 |  |
| c31 | CuT | 13.2 | on | NWT c18 or NWT c15 |  |
| c32 | CuT | 16.4 | on | NWT c19 or NWT c15 |  |
| c33 | CuT | 24.4 | on | NWT c23 |  |
| c34 | CuT | 32 | on | NWT c24 or NWT c16 |  |
| c35 | CuT | 48 | on | NWT c26 or NWT c17 |  |
| c36 | CuT | 64 | on | NWT c27 |  |

Editor’s note: Rating scale should rather refer to the Annex D.

]

- References

S4aA230063 IVAS-8a: Test Plan for selection phase v.0.8.3

S4-230368 IVAS-3: Performance Requirements v.1.0.0