Source:	Fraunhofer IIS, Ericsson LM

Title:	Proposed Additional Updates to IVAS Processing Plan

Document for:	Discussion and Agreement

Agenda Item:	1.4

- Introduction

In this contribution, the sources propose the following updates to the IVAS processing plan [1]:

- Incorporating the proposed 20KBP filtering option (as proposed to IVAS-8a)

- Proposing the necessary processing steps with respect to background noise loudness adjustment and mixing

The proposed changes are presented section by section.

- Proposed Changes

The following changes are proposed:

Start change 1

## 4.2.2 General processing for background noise

The offset of the noise file, in accordance with C.4.2, shall be removed before any processing starts.

Figure 1: Random trimming of background noise.

The resulting 48 kHz trimmed noise file is further on also referenced as “48 kHz input background”.

End change  1

Start change 2

## 4.3	Pre-processing

### 4.3.1	Pre-processing stages

### 4.3.1.1	  Filtering

Filtering is performed using either

- HP50_KHZ high pass filter at 48 kHz

- 20KBP band pass filter at 48 kHz

The test plan [1] specifies which filter to use dependent on the experiment.

### 4.3.1.2	Level adjustment

Figure 2: Level adjustment using BS.1770

End change 2

Start change 3

### 4.3.2 Pre-processing for Clean speech, Music and Mixed content, Generic audio

#### Figure X1: Pre-processing for Clean speech, Music and Mixed content, Generic audio.

#### 4.3.3 Pre-processing for Speech with background

Figure X2: Pre-processing for speech with background.

For the 48 kHz input speech, first a high pass filtering using the HP50_48KHZ filter is performed. The level adjustment is done as described in Section 4.3.1.2 using BS.1770 targeting the playout level. Subsequently, the filtered and loudness corrected files are concatenated.

For the 48 kHz input background, the level adjustment is done as described in Section 4.3.1.2 using BS.1770 with gating disabled. The target loudness is adjusted according to (playout loudness – target SNR). Subsequently, both signals are mixed.

End change 3

Start change 4

### 5.1.2 Filtering operations

#### 5.1.2.1 HP50_48KHZ filtering

To produce a 50Hz high pass filtered 48kHz sampling file use:

filter.exe HP50_48KHZ input.48k output.48k 960

#### 5.1.2.2 20KBP filtering

To produce an 20KBP band pass filtered 48kHz sampling file use:

filter.exe 20KBP input.48k output.48k 960

End change 4

Start change 5

#### 5.1.3.4 BS.1770 Level adjustment

To normalize the level of a 48 kHz sampled file, use:

bs1770demo.exe –nchan N –lev L -conf xxxx input.48k output.48k

where N is the number of channels, L the target level in LKFS (default: -26), and xxxx a configuration string with one value per channel specifying channel weighting according to:

x = ’1’ 	loudspeaker position within |elevation| < 30 deg, 60 deg <= |azimuth| <= 120 deg, 
(weighted by 1.41)

x = ’L’ 	LFE channel (weighted by 0)

x = ’0’ 	otherwise (weighted by 1)

To normalize stereo or binaural to -26 LKFS:

bs1770demo.exe –nchan 2 –lev -26 -conf 00 input.48k output.48k

To normalize 7.1+4 multi-channel configuration, as specified in Table 5, to -26 LKFS:

bs1770demo.exe –nchan 12 –lev -26 -conf 000L00110000 input.48k output.48k

To normalize the level of a 48 kHz sampled background noise file, in addition the option “-rms” shall be used.

#### 5.1.3.5 BS.1770 Level measuring

To measure the level of a 48 kHz sampled file, use:

bs1770demo.exe –nchan N -conf xxxx input.48k

where N is the number of channels, and xxxx a configuration string with one value per channel specifying channel weighting according to:

x = ’1’ 	loudspeaker position within |elevation| < 30 deg, 60 deg <= |azimuth| <= 120 deg, 
(weighted by 1.41)

x = ’L’ 	LFE channel (weighted by 0)

x = ’0’ 	otherwise (weighted by 1)

To normalize the level of a 48 kHz sampled background noise file, in addition the option “-rms” shall be used.

End change 5

Start change 6

ITU-T STL processing tools

| Source | - ITU-T G.191 - S4-120344 “Filter masks for EVS testing” |
| --- | --- |
| URL | - https://github.com/openitu/STL - https://www.itu.int/rec/T-REC-G.191-202303-I - http://ftp.3gpp.org/tsg_sa/WG4_CODEC/TSGS4_68/Docs/S4-120344.zip - https://github.com/ErikNorvell-Ericsson/STL.git |
| Version / Release | - G.191 / STL2023 |
| Description | Software tools for speech and audio coding standardization |
| Comments | - G.191 filter tool patched with S4-120344 to enable support for HP50 and SHQ filter - bs1770demo is patched for support of “rms” option, see https://github.com/ErikNorvell-Ericsson/STL.git |
| Executables | oper, astrip, concat, sv56demo, filter, scaldemo, actlev, eid-xor, gen-patt, mnrudemo, gen-rate-profile, eid-int, stereoop, bs1770demo, esdru, p50fbmnru, wmc_tool |
| Status | Available |

End change 6

- Conclusion

It is proposed to agree on the proposals outlined in Section 2 of this document and include the changes into IVAS-7a.

References:

[1] S4aA230065 - IVAS-7a: Processing plan for selection phase, v0.9.1