SA4-e (AH) Audio SWG post 124						    S4aA230076

12th June 2023

Source:	Fraunhofer IIS

Title:	Proposed Corrections to IVAS Processing Plan

Document for:	Discussion and Agreement

Agenda Item:	1.4

- Introduction

After careful review of IVAS-7a, a number of smaller issues and inconsistencies has been found. The source proposes to correct these, in order to avoid ambiguities for the IVAS selection tests.

These issues can be summarized as follows:

- Object based audio metadata file format not defined

- Naming in 4.1.4 doesn’t match naming convention in 3.10.2, Text in 4.1.4 doesn’t match Table 7.

- Proposed to remove sentence on resampling for MNRU conditions in 5.2.1, since this is not relevant for the selection testing (input always in 48 kHz)

- Clarification of Annex A.1 (delay compensation) and adaption towards necessary delay values, to match the proposed implementation of the processing scripts.

- Proposed Changes

The following changes are proposed:

Start change 1

### 3.11 Object Based Audio Metadata File Format

The metadata files for object based audio (per audio object) are files consistening of comma-separated values (CSV). Each line corresponds to 20ms audio (per individual object) at the renderer and consists of:

- Azimuth (floating-point, range [-180°;180°[)

- Elevation (floating-point, range [-90°;90°])

End change 1

Start change 2

### 4.1.4 Concatenation setup

The samples are concatenated after level adjustment. If the format includes metadata (object-based audio), the same concatenation is applied to the corresponding metadata-files.

The concatenation for P.SUPPL800 involves concatenation of clean speech samples followed by superposition with noise. For the clean-speech P.SUPPL800 tests, the noise may represent very low-level recording noise. In the P.SUPPL800 tests with background noise, the noise used depends on the audio content category specified in the respective test plan.

A consequence of using different background noises for different audio content categories is that the concatenation (and subsequent processing) can only be done for the subset of samples of the same category. Since the test plan defines 6 categories, 6 subsets of clean speech samples are concatenated followed by superimposing the noise of that category. To align the processing for the clean-speech tests, even in that case, the concatenation is done in subsets corresponding to the 6 categories.

The concatenation for P.SUPPL800 is illustrated in Table 7 below where all samples belonging to a category are concatenated from left to right and line by line. Different lines represent different categories. The concatenation comprises firstly the low-level noise preamble in accordance with clause 3.9. Then the samples for the actual evaluation and one sample for the preliminary (listener familiarization) test follow. The sample order is therefore for a given category y: ays01, ays02, ays03, ays04, ays05, ays06, ays07. The total length of the final concatenated clean speech sample of a category (name eeeay_cl) may vary depending on the length of the samples being concatenated.

Table 7: Concatenation order of P.SUPPL800 files

| Concatenated sample | Category | Item sequence in concatenated files |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Preamble |  | Samples for evaluation |  |  |  |  |  | Preliminary |
| eeea1_cl |  | 1 |  | low-level noise |  | a1s01 | a1s02 | a1s03 | a1s04 | a1s05 | a1s06 | a1s07 |
| eeea2_cl |  | 2 |  | low-level noise |  | a2s01 | a2s02 | a2s03 | a2s04 | a2s05 | a2s06 | a2s07 |
| eeea3_cl |  | 3 |  | low-level noise |  | a3s01 | a3s02 | a3s03 | a3s04 | a3s05 | a3s06 | a3s07 |
| eeea4_cl |  | 4 |  | low-level noise |  | a4s01 | a4s02 | a4s03 | a4s04 | a4s05 | a4s06 | a4s07 |
| eeea5_cl |  | 5 |  | low-level noise |  | a5s01 | a5s02 | a5s03 | a5s04 | a5s05 | a5s06 | a5s07 |
| eeea6_cl |  | 6 |  | low-level noise |  | a6s01 | a6s02 | a6s03 | a6s04 | a6s05 | a6s06 | a6s07 |

For all experiments, after the main processing, the processed concatenated samples for evaluation are divided into separate samples. A half Hann window of 100 ms tapering shall be applied to the start and end of the separated samples. The procedure for splitting and windowing concatenated files is described in clause 5.1.7.

Note: Metadata-assisted spatial audio inputs are created from concatenated FOA/HOA2 audio input files.

End change 2

Start change 3

### 5.2.1 MNRU conditions

To generate a P.50 FB MNRU anchor at XXX dB for a file input.48k, use:

p50fbmnru.exe input.48k output.48k XXX M

End change 3

Start change 4

Delay compensation for filter operations and reference codecs

The processing steps are delay-compensated in order to apply error insertion on the same parts of the audio signal and to be able to extract the original length and offset for each audio sample used in the tests.

The delay compensation is initialized by concatenating 1) the file to be filtered and 2) D samples of deterministic preamble noise according to Section 3.9. D corresponds to the delay compensation after filter value in Table 8. After processing, the delay of the processing operation is compensated, and the original file length is restored.

To compensate the delay for filter operations and reference conditions for encoder and decoder the delay compensation values  in Table 8 and Table 9 shall be used.

Table 8: Delay compensation values for filter operations

| Filter operation | Value for delay compensation after filtering operation |
| --- | --- |
| -up SHQ2 | 436 |
| -up SHQ3 | 436 |
| -down SHQ2 | 218 |
| -down SHQ3 | 145 |
| HP50_48KHZ | 839 |
| 20KBP | 200 |

Table 9: Delay compensation values for reference codecs

| Codec | Value for encoder delay compensation | Value for decoder delay compensation |
| --- | --- | --- |
| EVS | 0 | 0 |

End change 4

- Conclusion

It is proposed to agree on the proposals outlined in Section 2 of this document and include the changes into IVAS-7a.

References:

[1] S4-231088 - IVAS-7a: Processing plan for selection phase, v1.0.0