| 3GPP TS 26.250 V0.0.1 (2023-08) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Codec for immersive voice and audio services; [DRAFT] General overview (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Examples for styles	8

4.1	Heading styles	8

4.2	Other common styles	8

"TSG <Name>" on the front page	9

Page setup parameters	9

Proforma copyright release text block	11

X.1	The right to copy	11

Abstract Test Suite (ATS) text block	12

Y	Abstract Test Suite (ATS)	12

Y.1	Introduction	12

Y.2	The TTCN Graphical form (TTCN.GR)	12

Y.3	The TTCN Machine Processable form (TTCN.MP)	12

Annex <A> (normative): <Normative annex for a Technical Specification>	13

Annex <B> (informative): <Informative annex for a Technical Specification>	14

B.1	Heading levels in an annex	14

Annex <C>: <Informative annex title for a Technical Report>	15

Annex <D> (informative): Bibliography	16

Annex <E> (informative): Index	17

Annex <X> (informative): Change history	18

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document is an introduction to the audio processing parts and auxiliary functions of the codec for Immersive Voice and Audio Services (IVAS codec). A general overview of the audio processing and auxiliary functions is given, with reference to the documents where each function is specified in detail.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.441: "Codec for Enhanced Voice Services (EVS); General Overview".

[3]	3GPP TS 26.253: "Codec for immersive voice and audio services (IVAS); Detailed Algorithmic Description incl. RTP payload format and SDP parameter definitions".

[4]	3GPP TS 26.254: "Codec for immersive voice and audio services (IVAS); Rendering".

[5]	3GPP TS 26.251: "Codec for immersive voice and audio services (IVAS); ANSI C code (fixed-point)".

[6]	3GPP TS 26.258: "Codec for immersive voice and audio services (IVAS); ANSI C code (floating point)".

[7]	3GPP TS 26.252: "Codec for immersive voice and audio services (IVAS); Test Sequences".

[8]	3GPP TS 26.255: "Codec for immersive voice and audio services (IVAS); Error concealment of lost packets".

[9]	3GPP TS 26.256: "Codec for immersive voice and audio services (IVAS); Jitter Buffer Management".

[10]	3GPP TR 26.997: "Codec for immersive voice and audio services (IVAS); Performance Characterization".

[11]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[12]	3GPP TS 26.131: "Terminal acoustic characteristics for telephony; Requirements".

[13]	3GPP TS 26.261: "Terminal audio quality performance requirements for immersive audio services".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	General

The codec for Immersive Voice and Audio Services is part of a framework comprising besides encoder and decoder, renderer and a number of auxiliary functions associated with the support of stereo and immersive audio formats.

The IVAS codec is an extension of the 3GPP Enhanced Voice Services (EVS) codec; it provides full and bit exact EVS codec functionality for mono speech/audio signal input.

On top of that the IVAS codec is optimized for encoding and decoding of stereo and immersive audio formats such as multi-channel audio (5.1, 5.1.2, 5.1.4, 7.1, 7.1.4 setups), scene-based audio (Ambisonics up to order 3), meta-data assisted spatial audio (MASA), object-based audio (Independent Stream with Metadata, ISM up to 4 ISMs).

The codec features VAD/DTX/CNG for rate efficient stereo and immersive conversational voice transmissions, an error concealment mechanism to combat the effects of transmission errors and lost packets. Jitter buffer management is also provided.

The IVAS codec operates on 20 ms audio frames. It is capable of switching its bit rate upon command instantly at any frame boundaries.

A reference configuration where the various send side processing functions are identified is given in Figure 1. A corresponding reference configuration identifying various receive side processing functions is given in Figure 2. In the figures, the relevant specifications for each function are also indicated.

In Figures 1 & 2, the UE Send and Receive Audio processing are included, to show the complete speech path between the audio input/output in the User Equipment (UE) and a possible digital interface in network. The detailed specification of the audio parts is not within the scope of the present document. These aspects are only considered to the extent to highlight that the function of the audio parts and the operation of the IVAS codec are closely dependent on each other.

Figure 1: Overview of audio processing functions - Transmit Side

Figure 2: Overview of audio processing functions - Receive Side

Interfaces:

1: Audio input channels (16 bit linear PCM, sampled at 8 (only EVS), 16, 32, or 48 kHz)

2: Metadata associated with input audio

3: Encoded audio frames (50 frames/s), number of bits depending on IVAS codec mode

4: Encoded SIlence Descriptor (SID) frames

5: RTP Payload packets

6: Lost Frame Indicator (BFI)

7: Renderer config data

8: Head-tracker pose information

9: Audio output channels (16 bit linear PCM, sampled at 8 (only EVS), 16, 32, or 48 kHz)

10: Metadata associated with output audio

# 5	Transcoding functions

An algorithmic description of the IVAS codec is provided in [3].

As shown in Figures 1 & 2, the audio encoder takes its input, and can produce an output at the decoder/renderer, in various audio output formats. Input and output audio signals consist of one or multiple constituent channels of the respective audio format and in some cases metadata. The constituent channels are in the form of 16-bit uniform Pulse Code Modulated (PCM) signals at sampling frequencies of 8 kHz (only EVS interoperable coding), 16 kHz, 32 kHz or 48 kHz. The audio may typically originate from and terminate within the audio part of the UE or from the network side.

The detailed mapping between blocks of input audio to encoded blocks (in which the number of bits depends on the presently used codec mode) and from these to output blocks of reconstructed audio is described in [3]. The supported bit rates of the EVS interoperable coding are provided in [2]. Stereo and immersive audio coding is offered at the following discrete bit rates [kbps]: 13,2, 16,4, 24,4, 32, 48, 64, 80, 128, 160, 192, 256, 384, 512, with supported bit rate ranges and supported source-controlled operation (DTX) listed in Table 1.

Table 1: Ranges of supported source codec bit rates for stereo and immersive audio coding modes of the IVAS codec

| Input audio format | Range of supported bit rates (kbps) | Source Controlled Operation Available |
| --- | --- | --- |
| Stereo, binaural audio | 13,2 – 256 | Yes, up to 256 kbps |
| Scene-based audio (Ambisonics, FOA, HOA2, HOA3) | 13,2 – 512 | Yes, up to 80 kbps |
| Metadata assisted spatial audio (MASA) | 13,2 – 512 | Yes, up to 512 kbps |
| Object-based audio (ISM)(1 | 13,2 – 512(2 | Yes, up to 512 kbps |
| Multi-channel audio | 13,2 – 512 | No |

Notes:

(1 Combined input audio format combining scene-based audio with ISM (OSBA) is supported. Metadata assisted spatial audio with ISM (OMASA) is also supported.

(2 Highest bitrate 128 kbps for 1 ISM, 256 kbps for 2 ISMs, 384 kbps for 3 ISMs resp. 512 kbps for 4 ISMs.

There are 3 functionality levels of IVAS codec operations (including rendering) with increasing complexity/memory requirements.

An IVAS codec implementation of functionality level 1 shall support […]

An IVAS codec implementation of functionality level 2 shall support level 1 functionality and in addition […]

An IVAS codec implementation of functionality level 3 shall support the full set of IVAS codec features and functionalities, i.e., level 2 functionality and further […].

An IVAS audio codec capable UE shall support at least functionality level 1.

# 5	Rendering

IVAS rendering is the process to generate the IVAS audio output in the same or a different audio format than the input format, whereby in some cases, such as stereo-to-stereo, there is no particular rendering processing other than the decoding. The IVAS decoder provides integrated binaural rendering functionality for headphone reproduction including head-tracking and integrated rendering for loudspeaker reproduction. The IVAS renderer also supports head-tracked binaural rendering in a split rendering fashion moving the bulk of the processing load for IVAS decoding and rendering to a (more) capable node/UE while offloading the final rendering device. There is also the possibility to feed the IVAS decoder output to a customized external renderer while bypassing the integrated renderer.

IVAS rendering can also be operated stand-alone, i.e., without prior IVAS encoding/decoding of the input audio signal.

IVAS rendering is described in [3] and [4].

# 6	C-code

The C-code of the IVAS codec including VAD/DTX/CNG functionality, rendering, error concealment of lost frames, Jitter Buffer Manager (JBM) are described in [5] for fixed point arithmetic operation and are described in [6] for floating point arithmetic operation.

The C-code is mandatory.

# 7	Test vectors

A set of digital test sequences is specified in [7], thus enabling the verification of compliance, i.e. bit-exactness, to a high degree of confidence.

The IVAS encoder, decoder and renderer (see Figures 1 and 2) are defined in bit exact arithmetic. Consequently, they shall react on being presented with a given input sequence always with the corresponding bit exact output sequence, provided that the internal state variables are also always exactly in the same state at the beginning of the test.

The input test sequences provided shall produce the corresponding output test sequences, provided that the codec is operated from reset state.

# 8	Discontinuous transmission (DTX)

The discontinuous transmission (DTX) functionality of the IVAS codec including voice activity detection and comfort noise generation is defined in [3]. DTX functionality is supported for IVAS operation modes, i.e., audio formats and bit rates, that are especially optimized for efficient stereo and immersive conversational voice transmissions (see Table 1).

During a normal telephone conversation, the participants alternate so that, on the average, each direction of transmission is occupied about 50 % of the time. Source controlled operation is a mode of operation where the encoder encodes speech frames containing only background noise with a lower bit rate than normally used for encoding speech. A network may adapt its transmission scheme to take advantage of the varying bit rate. This may be done for the following two purposes:

1)	In the UE, battery life will be prolonged, or a smaller battery could be used for a given operational duration.

2)	The average required bit rate is reduced, leading to a more efficient transmission with decreased load and hence increased capacity.

The following functions are provided by the IVAS codec for the source-controlled rate operation:

-	a Voice Activity Detector (VAD) or more accurately Sound Activity Detector (SAD) on the TX side;

-	evaluation of the background acoustic noise on the TX side, in order to transmit characteristic parameters to the RX side;

-	generation of comfort noise on the RX side during periods when no normal speech frames are received.

The transmission of comfort noise information to the RX side is achieved by means of a Silence Descriptor (SID) frame, which is sent at regular intervals.

The bit rate of the non-EVS IVAS SID frames is 5.2 kbps.

# 11	Error concealment of lost frames

The IVAS codec error concealment of erroneous or lost frames is described in [3] and [8].

Frames may be erroneous due to transmission errors or frames may be lost or delayed due to packet loss in a transport network.

In order to mask the effect of erroneous/lost frames, the decoder shall be informed about such frames, which shall initiate error concealment actions leading to generation of substitution frames for the decoded/rendered audio output.

# 12	Frame structure

The IVAS codec frame structure is described in [3].

# 13	RTP Payload Format

The IVAS coder RTP Payload Format for media handling and interaction is described in [3].

# 14	Jitter Buffer Management

The IVAS codec Jitter Buffer Management is described in [9].

# 15	Performance characterization

The IVAS codec performance characterization is described in [10].

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 09-2023 | SA#101 |  |  |  |  | Presented to TSG SA#101 for information | 1.0.0 |