Source:	VoiceAge Corporation

Title:	Further considerations for the IVAS Characterization test

Agenda Item:	1.4

Document for:	Discussion and Agreement

Introduction

At the last SA4 meeting in Chicago, contribution [1] presented an example of experiments that can be accommodated during the IVAS Characterization test, given the budget left for the Characterization subjective testing, and some preliminary assumptions about testing capacity by volunteering companies, IVAS codec configurations to be tested, and subjective test methodologies.

The example table of [1] with allocation of experiments was included in the IVAS Characterization Test Plan (IVAS-8b) [2] as a basis for further work. It was then discussed and modified during an editing session of the Chicago SA4 meeting.

In particular, the following two aspects were discussed extensively:

1) whether, and to what extent to replicate the experiments in two Listening Laboratories (LLs),

2) the methodology and the extent of formal subjective validation of the IVAS fixed-point implementation.

The goal of this contribution is to advance the discussion on these aspects, considering several alternatives, and presenting example experiment setups for selected options.

Question of replication of the Characterization experiments

During the Selection test, all experiments were replicated twice in two different LLs, generally with good correlation of the results in both LLs. In [3] it was thus suggested that replicating each experiment twice might not be needed, and that the listening resources might be better spent on larger coverage of different IVAS configurations. This applied in particular for MUSHRA tests using expert, or at least experimented, listeners. The assumption of no replication of experiments was also used in drafting the table of allocation of experiments of [1].

In the discussions during the Chicago SA4 meeting, this assumption was questioned. The main argument was that in case of unexpected results for some experiments, in absence of a x-checking LL it will not be clear whether an issue is related to the codec or to an issue in running the experiment. As a solution to this situation, it was proposed that a potential subsequent verification phase could be scheduled to verify experiments with unexpected results.

Nevertheless, the question whether replicating the experiments during the IVAS Characterization phase, and if so to what extent, remained open.

Subjective methodology for fixed-point validation

The example table of allocation of experiments in [1] simply used the same methodologies as used in the IVAS Selection - P.SUPPL800 for experiments assessing lower IVAS bitrates and lower quality, and MUSHRA to assess higher bitrates and higher quality.

During the discussion following presentation of [1] and during the editing session, the number of P.SUPPL800 experiments to validate the fixed-point was reduced significantly. In addition, the Experiment to test 1 object could potentially be removed too, as any deficiency there should also appear in experiments evaluating IVAS for higher number of objects.

Though the appropriateness of the MUSHRA test for fixed-point validation was questioned, the MUSHRA experiments were not reviewed for lack of time.

Besides P.SUPPL800 and MUSHRA, alternative methodologies for subjective validation of the fixed-point code were also mentioned, namely:

[REF]-A-B – the advantage of an A-B test is its high resolution to assess whether one of the alternatives is better, worse, or equal to the other, as it compares directly both variants for each sample. On the other hand, it is not a standardized methodology, and it is not guaranteed that its time efficiency would be better than e.g. for a DCR test.

BS.1285 [4] – it is a standardized methodology, similar to a P.800 DCR test in the sense that CuT is compared to an original reference, and impairment is judged. In some previous experiments run by Dynastat, BS.1285 was found more accurate compared to P.800 ACR, even when using less listeners. The source is however not aware of any direct comparison to P.800 DCR. Timewise, each BS.1285 trial requires at least one more listening (REF-A-B type test) compared to a DCR trial, and one more vote, as the listeners are first asked to identify which of A or B equals to REF. So using BS.1285 instead of P.800 DCR is unlikely to be more efficient timewise.

Another alternative is to use P.SUPPL800 methodology for all subjective validations of the fixed-point code. If we consider P.SUPPL800 DCR-type test, a possible experiment setup is outlined in Tables 1 and 2, giving an idea what could be tested.

Assuming all combinations of float/fixed-point encoder and float/fixed-point decoder need to be covered, 7 different configurations of the IVAS codec can be tested in one P.SUPPL800 experiment. The range of bitrates for different input formats of the IVAS codec varies from 11 (ISM) to 14 (SBA,…). This means that if we want to cover all bitrates, we need two P.SUPPL800 experiments per input format. Alternatively, it can be decided that only a subset of 7 bitrates needs to be subjectively tested for each input format. Yet another alternative that would allow to limit the fixed-point P.SUPPL800 tests to 1 experiment per input format is to skip subjective tests of floating-point encoder / fixed-point decoder, and fixed-point encoder / floating-point decoder. This suggestion was also considered during the Chicago SA4 meeting.

Table 1: Example Stereo experiment - test conditions

| Label | Condition | Bitrate [kbps] | DTX | FER/Profile | ToR |
| --- | --- | --- | --- | --- | --- |
| c01 | Reference | - | - | - | - |
| c02 | MNRU Q=28 dB | - | - | - | - |
| c03 | MNRU Q=24 dB | - | - | - | - |
| c04 | MNRU Q=20 dB | - | - | - | - |
| c05 | MNRU Q=16 dB | - | - | - | - |
| c06 | ESDRU α=0.7 | - | - | - | - |
| c07 | ESDRU α=0.4 | - | - | - | - |
| c08 | ESDRU α=0.1 | - | - | - | - |
| c09 | CuT flt-flt | 13.2 | On | 5% | - |
| c10 | CuT flt-flt | 16.4 | On | No errors | - |
| c11 | CuT flt-flt | 24.4 | Off | 5% | - |
| c12 | CuT flt-flt | 32 | Off | No errors | - |
| c13 | CuT flt-flt | 48 | Off | No errors | - |
| c14 | CuT flt-flt | 64 | Off | No errors | - |
| c15 | CuT flt-flt | 80 | Off | No errors | - |
| c16 | CuT flt-fxd | 13.2 | On | 5% | NWT c09 |
| c17 | CuT flt-fxd | 16.4 | On | No errors | NWT c10 |
| c18 | CuT flt-fxd | 24.4 | Off | 5% | NWT c11 |
| c19 | CuT flt-fxd | 32 | Off | No errors | NWT c12 |
| c20 | CuT flt-fxd | 48 | Off | No errors | NWT c13 |
| c21 | CuT flt-fxd | 64 | Off | No errors | NWT c14 |
| c22 | CuT flt-fxd | 80 | Off | No errors | NWT c15 |
| c23 | CuT fxd-flt | 13.2 | On | 5% | NWT c09 |
| c24 | CuT fxd-flt | 16.4 | On | No errors | NWT c10 |
| c25 | CuT fxd-flt | 24.4 | Off | 5% | NWT c11 |
| c26 | CuT fxd-flt | 32 | Off | No errors | NWT c12 |
| c27 | CuT fxd-flt | 48 | Off | No errors | NWT c13 |
| c28 | CuT fxd-flt | 64 | Off | No errors | NWT c14 |
| c29 | CuT fxd-flt | 80 | Off | No errors | NWT c15 |
| c30 | CuT fxd-fxd | 13.2 | On | 5% | NWT c09 |
| c31 | CuT fxd-fxd | 16.4 | On | No errors | NWT c10 |
| c32 | CuT fxd-fxd | 24.4 | Off | 5% | NWT c11 |
| c33 | CuT fxd-fxd | 32 | Off | No errors | NWT c12 |
| c34 | CuT fxd-fxd | 48 | Off | No errors | NWT c13 |
| c35 | CuT fxd-fxd | 64 | Off | No errors | NWT c14 |
| c36 | CuT fxd-fxd | 80 | Off | No errors | NWT c15 |

- Table 2: Example Stereo experiment - content type categories and scene definitions

| Category | Room | Reverb | Microphone Setup | Input Level | Material | Background | Overtalk [s](2 | Talker positions(3 | Talker selection by panel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | small | anechoic | A-B (100cm) | Nominal | Speech | 45 dB idle noise | 1 | 1-7 5-3 2-6 4-1 3-4 7-2 | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | large | anechoic | A-B (150cm) | Low (-10 dB) | Music | - | -1 | 5-11 1-6 3-7 5-8 9-7 10-9 | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | small | anechoic | M-S | High (+10 dB) | Mixed content | - | -1 | 1-7 5-3 2-6 4-1 3-4 7-2 | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | small | echoic | A-B (100cm) | Nominal | Speech | 15 dB car noise | 1 | 1-7 5-3 2-6 4-1 3-4 7-2 | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | large | echoic | A-B (150cm) | Low (-10 dB) | Speech | 15 dB street noise | 1 | 2-8 9-4 6-10 11-8 10-12 12-1 | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | small | echoic | Binaural | High (+10 dB) | Speech | 15 dB office noise | -1 | 1-7 5-3 2-6 4-1 3-4 7-2 | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

Note: Maximum available bandwidth is up to SWB.

(2 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(3 The talker positions are part of the scene definition of the different categories. They correspond to the talker positions as depicted in Figures 14.3 and 14.5 of [5] for the large and the small room, respectively.

For illustration, an example of the experiment allocation table, under the assumptions that all fixed-point experiments are P.SUPPL800 and that only one P.SUPPL800 experiment is run for a particular input format, is given in Table 3.

Table 3: Allocation of experiments

| Exp | Objectif | Input format | Source material | Listening environment | Language | LL 1 | LL 2 | Pricing [Euros] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P800-1 | Fixed-point | Stereo |  | Headphones |  |  |  |  |
| P800-2 | Fixed-point | FOA |  | Headphones |  |  |  |  |
| P800-3 | Fixed-point | HOA2 |  | Headphones |  |  |  |  |
| P800-4 | Fixed-point | HOA3 |  | Headphones |  |  |  |  |
| P800-5 | Fixed-point | 2 Objects |  | Headphones |  |  |  |  |
| P800-6 | Fixed-point | 4 Objects |  | Headphones |  |  |  |  |
| P800-7 | Fixed-point | Stereo MASA |  | Headphones |  |  |  |  |
| P800-8 | Fixed-point | 5.1 |  | Headphones |  |  |  |  |
| P800-9 | Fixed-point | 7.1.4 |  | Headphones |  |  |  |  |
| P800-10 |  | monoMASA |  | Headphones |  |  |  |  |
| P800-11 |  | monoMASA |  | Headphones |  |  |  |  |
| P800-12 |  | OSBA |  | Headphones |  |  |  |  |
| P800-13 |  | OMASA |  | Headphones |  |  |  |  |
| P800-14 | JBM/FER/Tan | Stereo |  | Headphones |  |  |  |  |
| P800-15 | JBM/FER/Tan | 2 Objects |  | Headphones |  |  |  |  |
| P800-16 | Stereo dmix | Stereo |  | Headphones |  |  |  |  |
| P800-17 |  |  |  |  |  |  |  |  |
| P800-18 |  |  |  |  |  |  |  |  |
| BS1534-1 |  | OSBA | Generic audio | Headphones |  |  |  |  |
| BS1534-2 |  | OSBA | Generic audio | 7.1+4 |  |  |  |  |
| BS1534-3 |  | OMASA | Generic audio | Headphones |  |  |  |  |
| BS1534-4 |  | OMASA | Generic audio | Headphones |  |  |  |  |
| BS1534-5 |  | 5.1+2 | Generic audio | 5.1+2 |  |  |  |  |
| BS1534-6 |  | 5.1+2 | Generic audio | 5.1+2 |  |  |  |  |
| BS1534-7 |  | 5.1+4 | Generic audio | 5.1+4 |  |  |  |  |
| BS1534-8 |  | 5.1+4 | Generic audio | 5.1+4 |  |  |  |  |
| BS1534-9 |  | 7.1 | Generic audio | 7.1 |  |  |  |  |
| BS1534-10 |  | 7.1 | Generic audio | 7.1 |  |  |  |  |
| BS1534-11 | Renderer |  | Generic audio |  |  |  |  |  |
| BS1534-12 | Renderer |  | Generic audio |  |  |  |  |  |
| BS1534-13 | Renderer |  | Generic audio |  |  |  |  |  |
| BS1534-14 | Renderer |  | Generic audio |  |  |  |  |  |
| BS1534-15 | Renderer |  | Generic audio |  |  |  |  |  |
| BS1534-16 | Renderer |  | Generic audio |  |  |  |  |  |
| BS1534-17 |  |  |  |  |  |  |  |  |
| BS1534-18 |  |  |  |  |  |  |  |  |
| BS1534-19 |  |  |  |  |  |  |  |  |
| BS1534-20 |  |  |  |  |  |  |  |  |
| BS1534-21 |  |  |  |  |  |  |  |  |
| BS1534-22 |  |  |  |  |  |  |  |  |
| BS1534-23 |  |  |  |  |  |  |  |  |
| BS1534-24 |  |  |  |  |  |  |  |  |
| BS1534-25 |  |  |  |  |  |  |  |  |
| BS1534-26 |  |  |  |  |  |  |  |  |
| BS1534-27 |  |  |  |  |  |  |  |  |
| BS1534-28 |  |  |  |  |  |  |  |  |
| BS1534-29 |  |  |  |  |  |  |  |  |
| BS1534-30 |  |  |  |  |  |  |  |  |
| ACR | 16, 32, 48 kHz |  |  |  |  |  |  |  |
|  |  | OSBA, OMASA | Speech+bckg |  |  |  |  |  |
|  | JBM/FER/Tan | FOA/MASA | Clean speech | Headphones |  |  |  |  |
|  | Room acoustics |  |  |  |  |  |  |  |
|  | 6 DoF, directivity |  |  |  |  |  |  |  |
| Total price |  |  |  |  |  |  |  | 192000 |

Proposals

- Experiments to be run twice only if there are free slots for P.SUPPL800 experiments once all IVAS configurations not tested in Selection are covered. In case of any unexpected results, the concerned experiments can be rerun at a verification phase.

- Remove the 1-Object experiment form subjective fixed-point code evaluation.

References

S4-231857 - Example experiment allocation for IVAS Characterization phase.

S4-232029 - IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, v.0.1.0.

S4-231510 - Organization of IVAS Characterization Phase.

Recommendation ITU-R  BS.1285-1 (05/2023) - Pre-selection methods for the subjective assessment
of small impairments in audio systems.

Recommendation ITU-T P.191 (03/2023): Software tools for speech and audio coding standardization.