Source:	Nokia Corporation

Title:	Proposal for IVAS Codec characterization test plan – mono MASA

Document for:	Discussion

Agenda Item:	1.4 – IVAS_Codec

1. Introduction

IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase [1], discuss specifics of IVAS Codec characterization testing.

In this document the source proposes specific test conditions for two P.800Suppl DCR experiments covering mono MASA operation. Many mono MASA operation points with respective requirements as well as some fixed / floating point inter-operation points are proposed.

2. Overview

In the current work in progress IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase [1], mono MASA has been reserved two P.800Suppl DCR tests. In order to progress the work, the source proposes specific test material and conditions for two P.SUPPL800 tests: mono MASA in clean speech and mono MASA with speech and background noise.

The present input contribution makes the first proposals for these two experiments.

3. Discussion

Some observations and considerations are made relating, e.g., to size of each experiment. In particular, the P.SUPPL800 DCR experiments can have 36 test conditions.

In P.SUPLL800 experiments, there is not sufficient room to test all conditions:

  - It is proposed to test all conditions with either IVAS internal binaural renderer or with IVAS_rend functionality for references.

  - Fixed point version of the IVAS and EVS codec should be used, if not otherwise specified by the condition.

  - It is proposed to focus only on mono MASA operation, since it was not tested at all in selection testing.

  - It is proposed to test IVAS mono MASA conditions only up to 80 kbps, except for bitrate switching conditions.

  - Bitrate switching has not been tested. It is proposed that some conditions are used to test reasonable bitrate ranges. Bitrate switching rate could correspond to FER erasure rate. E.g. 5% FER rate means that on average every 20th frame could have frame-erasure / bitrate switching.

  - In clean speech test some FER conditions are selected to give an insight to frame erasure concealment functionality of the IVAS codec with mono MASA operation.

  - In noisy speech test DTX is enabled to some bitrates to give an insight to DTX performance of the IVAS codec with mono MASA operation.

  - Some conditions in both tests are reserved to test floating / fixed point interoperability.

  - Mixed bandwidth IO has not been included to earlier testes. Some conditions can be tested with 16 kHz IO.

  - Not all performance requirements are tested, to give more space to characterize DTX, PLC and floating/fixed point interoperation.

  - Towards the lower bitrates, codec references based on multi-EVS coding of Ambisonics inputs are preferred over codec references based on unquantized metadata as the latter can be considered to define an upper limit of quality at the higher bitrates (where EVS becomes transparent).

  - It is proposed to limit number of MNRUs to 3 and number of ESDRUs to 3.

  - In IVAS selection testing phase it was found out that MNRU Q=20 is too low value for clean speech, also the highest value Q=32 seems to be a bit too low. Thus, we propose to use Q=25, Q=30 and Q=35 for clean speech

  - Similarly for noisy speech MNRU values Q=17 and Q=21 provide too low quality. Also, the highest value Q=29 has too low quality. We propose to use Q=23, Q=28 and Q=33 for noisy speech.

  - Based on selection testing phase results ESDRU scale of 0.8, 0.6 and 0.4 seems to be quite reasonable for both tests.

  - We propose to use the same listening test material as was used for P800-8 and P800-9 during the Selection testing phase [3].

4. Proposals for specific experiments

4.1 P.SUPPL800 mono MASA experiment – Clean speech

It is proposed for mono-MASA inputs to have following conditions for clean speech test.

Table 1 Overview of the conditions

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 7 | IVAS operated at 13.2, 16.4, 24.4, 32, 48, 64, 80 kbps with DTX off at 0% FER |
|  | 3 | IVAS operated at 16.4, 32, 64 kbps with DTX off at 5% FER |
|  | 3 | IVAS in frame rate switching at low (13.2 – 32 kbit/s), medium (24.4- - 64 kbit/s) and high (48 – 128 kbit/s) bitrate range. Bitrate change happens every ~20th frame to random bitrate within range. |
|  | 6 | IVAS fixed point / floating point interoperability conditions |
| Codec references |  |  |
| Codec references | 6 | Multi-mono EVS operated (via FoA) at 3*7.2, 4*7.2, 4*8, 4*9.6, 4*16.4, 4*24.4 kbps with DTX off at 0% FER |
|  | 3 | mono EVS with unquantized metadata operated at 9.6, 16.4, and 48 kbps with DTX off at 0% FER |
| Other references |  |  |
| Direct | 2 | Floating and fixed point IVAS_rend. Nominal input level |
| P.50 MNRU (applied to MASA transport streams) | 3 | Q = 35, 30, 25 dB (all: nominal input level) |
| ESDRU [ITU-T P.811] | 3 | α = 0.8, 0.6, 0.4 (output loudness set to nominal level) |
| Common Conditions |  |  |
| Test item generation | 1 | Model-based generation according to processing scripts |
| Binaural rendering | 1 | IVAS codec internal binaural renderer and for references IVAS external renderer (IVAS_rend) |
| Audio sampling frequency / bandwidth | 1 | 48 kHz / maximum available audio bandwidth (WB, SWB, FB) |
| Content types / categories | 6 | Spatial scenes as in selection test P800-8 |
| Number of talkers | 6 | Sentence pairs uttered by different talkers (one each of 3 male and 3 female talkers) |
| Number of speech samples | 7 | 6 for tests + 1 for preliminaries per category |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR with instructions according to [P Suppl. 29] |
| Languages | 1 | [tbd] |
| Listening System | 1 | High-quality headphones, diotic presentation |
| Listening Environment | 1 | No noise |

Table 2 Sample categories

| Category | Environment (impulse responses, idle noise background) | Level [dB] | Overtalk [s] | Talker positions | Talker selection by panel |
| --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_MASA (FOA) | -26 | Same as in selection P800-8 | Same as in selection P800-8 | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_2_MASA (HOA2) | -26 | Same as in selection P800-8 | Same as in selection P800-8 | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | room_3_MASA (FOA) | -26 | Same as in selection P800-8 | Same as in selection P800-8 | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_4_MASA (HOA2) | -26 | Same as in selection P800-8 | Same as in selection P800-8 | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | room_5_MASA (FOA) | -26 | Same as in selection P800-8 | Same as in selection P800-8 | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | room_6_MASA (HOA2) | -26 | Same as in selection P800-8 | Same as in selection P800-8 | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

Table 3 Test conditions for P.SUPPL800 mono MASA in clean speech under clean and impaired channel conditions

| Label | Condition | Bitrate [kbps] | FER/Profile | ToR |
| --- | --- | --- | --- | --- |
| c01 | Reference Float | - | - |  |
| c02 | Reference Fixed | - | - |  |
| c03 | MNRU Q=35 dB | - | - |  |
| c04 | MNRU Q=30 dB | - | - |  |
| c05 | MNRU Q=25 dB | - | - |  |
| c06 | ESDRU α=0.8 | - | - |  |
| c07 | ESDRU α=0.6 | - | - |  |
| c08 | ESDRU α=0.4 | - | - |  |
| c09 | IVAS | 80 | No errors | NWT c18 OR NWT c12 |
| c10 | EVS FoA path | 4x7.2 | No errors |  |
| c11 | EVS FoA path | 4x9.6 | No errors |  |
| c12 | EVS FoA path | 4x24.4 | No errors |  |
| c13 | EVS FoA path | 3x7.2 | No errors |  |
| c14 | EVS FoA path | 4x8 | No errors |  |
| c15 | EVS FoA path | 4x16.4 | No errors |  |
| c16 | EVS (+unquantized metadata) | 9.6 | No errors |  |
| c17 | EVS (+unquantized metadata) | 16.4 | No errors |  |
| c18 | EVS (+unquantized metadata) | 48 | No errors |  |
| c19 | IVAS 1TC MASA | 13.2 | No errors | NWT c13 |
| c20 | IVAS 1TC MASA | 24.4 | No errors | NWT c14 |
| c21 | IVAS 1TC MASA | 48 | No errors | NWT c15 |
| c22 | IVAS 1TC MASA | 16.4 | No errors | NWT c16 OR NWT c10 |
| c23 | IVAS 1TC MASA | 32 | No errors | NWT c17 OR NWT c11 |
| c24 | IVAS 1TC MASA | 64 | No errors | NWT c18 OR NWT c12 |
| [c25 | IVAS 1TC MASA | 13.2 - 32 | Bitrate switching | - |
| c26 | IVAS 1TC MASA | 24.4 – 64 | Bitrate switching | - |
| c27 | IVAS 1TC MASA | 48 – 96 | Bitrate switching | - |
| c28 | IVAS 1TC MASA | 32 | FER 5% | - |
| c29 | IVAS 1TC MASA | 48 | FER 5% | - |
| c30 | IVAS 1TC MASA | 64 | FER 5% | - |
| c31 | IVAS 1TC MASA flt / fxd | 13.2 – 32 | Bitrate switching | NWT c25 |
| c32 | IVAS 1TC MASA fxd / flt | 24.4 – 64 | Bitrate switching | NWT c26 |
| c33 | IVAS 1TC MASA flt / fxd | 48 – 96 | Bitrate switching | NWT c27 |
| c34 | IVAS 1TC MASA fxd / flt | 32 | FER 5% | NWT c28 |
| c35 | IVAS 1TC MASA flt / fxd | 48 | FER 5% | NWT c29 |
| C36 | IVAS 1TC MASA fxd / flt | 64 | FER 5% | NWT c30] |

Note: Conditions 25-36 need further discussion.

4.2 P.SUPPL800 mono MASA experiment – Speech + Background

It is proposed for mono-MASA inputs to have following conditions for speech + background test.

Table 4 Overview of test conditions

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 7 | IVAS operated at 13.2, 16.4, 24.4, 32, 48, 64, 80 kbps with DTX off at 0% FER |
|  | 3 | IVAS operated at 16.4, 32, 64 kbps with DTX on at 0% FER |
|  | 3 | IVAS with 16 kHz IO bandwidth at 13.2, 24.4 and 48 kbps |
|  | 6 | IVAS fixed point / floating point interoperability conditions |
| Codec references |  |  |
| Codec references | 6 | Multi-mono EVS operated (via FoA) at 3*7.2, 4*7.2, 4*8, 4*9.6, 4*16.4, 4*24.4 kbps with DTX off at 0% FER |
|  | 3 | mono EVS with unquantized metadata operated at 9.6, 16.4, and 48 kbps with DTX off at 0% FER |
| Other references |  |  |
| Direct | 2 | Floating and fixed point IVAS_rend. Nominal input level |
| P.50 MNRU (applied to MASA transport streams) | 3 | Q = 33, 28, 23 dB (all: nominal input level) |
| ESDRU [ITU-T P.811] | 3 | α = 0.8, 0.6, 0.4 (output loudness set to nominal level) |
| Common Conditions |  |  |
| Test item generation | 1 | Model-based generation according to processing scripts |
| Binaural rendering | 1 | IVAS codec internal binaural renderer and for references IVAS external renderer (IVAS_rend) |
| Audio sampling frequency / bandwidth | 2 | 48 kHz / maximum available audio bandwidth (WB, SWB, FB) and 16 kHz with WB bandwidth |
| Content types / categories | 6 | Spatial scenes as in selection test P800-9 |
| Number of talkers | 6 | Sentence pairs uttered by different talkers (one each of 3 male and 3 female talkers) |
| Number of speech samples | 7 | 6 for tests + 1 for preliminaries per category |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR with instructions according to [P Suppl. 29] |
| Languages | 1 | [tbd] |
| Listening System | 1 | High-quality headphones, diotic presentation |
| Listening Environment | 1 | No noise |

Table 5 Sample Categories

| Category | Environment (Both impulse responses and background noise) | Level [dB] | Background | SNR [dB] | Overtalk [s] | Talker positions | Talker selection by panel |
| --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | car_1_MASA (HOA2) | -26 | car_1_bg_MASA | 10 | Same as in selection P800-9 | Same as in selection P800-9 | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | car_2_MASA (FOA) | -26 | car_2_bg_MASA | 10 | Same as in selection P800-9 | Same as in selection P800-9 | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | outdoors_1_MASA (FOA) | -26 | outdoors_1_bg_MASA | 15 | Same as in selection P800-9 | Same as in selection P800-9 | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | outdoors_2_MASA (HOA2) | -26 | outdoors_2_bg_MASA | 15 | Same as in selection P800-9 | Same as in selection P800-9 | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | indoors_1_MASA (HOA2) | -26 | indoors_1_bg_MASA | 15 | Same as in selection P800-9 | Same as in selection P800-9 | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | indoors_2_MASA (FOA) | -26 | indoors_2_bg_MASA | 15 | Same as in selection P800-9 | Same as in selection P800-9 | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

Table 6 Test conditions for P.SUPPL800 mono MASA, speech and background under clean channel conditions with and without DTX

| Label | Condition | Bitrate [kbps] | FER/Profile/DTX | ToR |
| --- | --- | --- | --- | --- |
| c01 | Reference Float | - | - |  |
| c02 | Reference Fixed | - | - |  |
| c03 | MNRU Q=33 dB | - | - |  |
| c04 | MNRU Q=28 dB | - | - |  |
| c05 | MNRU Q=23 dB | - | - |  |
| c06 | ESDRU α=0.8 | - | - |  |
| c07 | ESDRU α=0.6 | - | - |  |
| c08 | ESDRU α=0.4 | - | - |  |
| c09 | IVAS | 80 | No errors | NWT c18 OR NWT c12 |
| c10 | EVS FoA path | 4x7.2 | No errors |  |
| c11 | EVS FoA path | 4x9.6 | No errors |  |
| c12 | EVS FoA path | 4x24.4 | No errors |  |
| c13 | EVS FoA path | 3x7.2 | No errors |  |
| c14 | EVS FoA path | 4x8 | No errors |  |
| c15 | EVS FoA path | 4x16.4 | No errors |  |
| c16 | EVS (+unquantized metadata) | 9.6 | No errors |  |
| c17 | EVS (+unquantized metadata) | 16.4 | No errors |  |
| c18 | EVS (+unquantized metadata) | 48 | No errors |  |
| c19 | IVAS 1TC MASA | 13.2 | No errors | NWT c13 |
| c20 | IVAS 1TC MASA | 24.4 | No errors | NWT c14 |
| c21 | IVAS 1TC MASA | 48 | No errors | NWT c15 |
| c22 | IVAS 1TC MASA | 16.4 | No errors | NWT c16 OR NWT c10 |
| c23 | IVAS 1TC MASA | 32 | No errors | NWT c17 OR NWT c11 |
| c24 | IVAS 1TC MASA | 64 | No errors | NWT c18 OR NWT c12 |
| [c25 | IVAS 1TC MASA | 13.2 | WB IO | - |
| c26 | IVAS 1TC MASA | 24.4 | WB IO | - |
| c27 | IVAS 1TC MASA | 48 | WB IO | - |
| c28 | IVAS 1TC MASA | 32 | DTX | - |
| c29 | IVAS 1TC MASA | 48 | DTX | - |
| c30 | IVAS 1TC MASA | 64 | DTX | - |
| c31 | IVAS 1TC MASA fxd / flt | 13.2 | WB IO | NWT c25 |
| c32 | IVAS 1TC MASA flt / fxd | 24.4 | WB IO | NWT c26 |
| c33 | IVAS 1TC MASA fxd / flt | 48 | WB IO | NWT c27 |
| c34 | IVAS 1TC MASA flt / fxd | 32 | DTX | NWT c28 |
| c35 | IVAS 1TC MASA fxd / flt | 48 | DTX | NWT c29 |
| c36 | IVAS 1TC MASA flt / fxd | 64 | DTX | NWT c30] |

Note: Conditions 25-36 need further discussion.

5. Summary

We have presented some IVAS characterization test plan specifics and options for discussion in clause 4 of this input. The proposal covers two P.SUPPL800 DCR experiments for IVAS mono MASA operation.

References

[1]	S4-232029-IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase, Version v.0.1.0

[2]	ITU-T P.800 – Use cases, P Suppl. 29 (01/2023)

[3]	S4aA-230086: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.1.1.0