# 1	Scope

The present document is applicable to any terminal capable of supporting narrowband, wideband, super-wideband or fullband telephony, either as a stand-alone service or as the telephony component of a multimedia service. The present document specifies requirements and test methods to verify correct implementations of the RTP payload format for 3GPP codecs in UE. The focus is on conversational services in LTE, NR and WLAN terminals when used to provide narrowband, wideband, super-wideband or fullband telephony.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	3GPP TS 26.132: "Speech and video telephony terminal acoustic test specification".

[x2]	3GPP TS 26.139: "Real-time Transport Protocol (RTP) / RTP Control Protocol (RTCP) verification procedures".

[x3]	3GPP TS 34.229-1: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[x4]	3GPP TS 34.229-2: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) specification".

[x5]	3GPP TS 34.229-3: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 3: Abstract test suite (ATS)".

[x6]	3GPP TS 34.229-4: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 4: Enabler for IP multimedia applications testing".

[x7]	3GPP TS 34.229-5: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 5: Protocol conformance specification using 5G System (5GS)".

[x8]	IETF RFC 4867: "RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs".

[x9] 	3GPP TS 26.445: "Codec for Enhanced Voice Services (EVS);Detailed algorithmic description".

[x10]		GSMA IR.92:  "IMS Profile for Voice and SMS".

[x11]		GSMA NG.114: " IMS Profile for Voice, Video and Messaging over 5GS ".

[x12]		3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Incoming CMR : CMR in RTP packet received by the DUT

Outgoing CMR : CMR in RTP packet sent by the DUT

Sending: Link from DUT to test simulator

Receiving: Link from test simulator to DUT

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

5GC 		5G Core Network

AMR		Adaptive Multi Rate

DUT		Device Under Test

EPS		Evolved Packet System

EVS 		Enhanced Voice Services

HATS 	Head and Torso Simulator

HATS		Head and Torso Simulator

IMS 		IP Multimedia Subsystem

LTE 		Long Term Evolution

MS 		Mobile Station

RTCP		Real Time Transport Control Protocol

SS 		System Simulator

TX 		Transmission

UE 		User Equipment

UMTS		Universal Mobile Telecommunications System

Vo5G		5G capable DUT supporting either voice over NR or EPS Fallback

WLAN 	Wireless Local Area Network

NR 				New Radio

# 4	Interfaces

## 4.1	General

By default, electrical interfaces should be used for testing. Acoustic interfaces may be used as an alternative. The actual interfaces used in sending and receiving shall be documented.

Test cases may be implemented either by injecting specific packet information (including CMR requests) in a live call or assuming a PCAP player scenario. To define the test setup, one approach is to reuse the test setup defined in TS 26.132 [x1] (defining acoustical and electrical interfaces) where only LTE, WLAN and NR apply.

If no acoustic measurement is required, the test setup in TS 26.139 [x2] can be reused for RTP /RTCP “data injection” with either active or passive test instrument.

## 4.1	Acoustic interfaces

See [x1] for the definition of acoustic interfaces.

## 4.2	Electrical interfaces

See clause 5.2 in [x1].

# 5	Test setup

Similar to clause 5.1 in [x2]:

1.	The "system under test" is the device (DUT). or software to be tested

2.	The "test instrument" is the equipment used:

- to place an IMS call (includes all volte, VoNR, WLAN capabilities) including configurable SIP capabilities with configurable SDP parameters

- to observe data collection (SDP/RTP/RTCP output from the system under test):  extract, calculate, and store information to be able to check the pass criteria for all the test described in the present document.

3.	The "data injection" is the device or equipment used to generate RTP/RTCP data sent to the system under test.

NOTE1: ‘data injection’ may be collocated or integrated with the test instrument.

NOTE2: data collection’ ‘SDP/RTP  / RTCP receiver) may be performed either by “test instrument” or collocated with “data injection” equipment

## 5.1	Setup for terminals

See clause 5.2 in [x1] for the setup definition (Handset UE, Headset UE, Desktop-mounted hands-free UE, Hand-held hands-free UE, Softphone UE). If no acoustical measurement is conducted, constraints on device positioning could be alleviated here.

## 5.2	Setup of the electrical interfaces of test equipment

See clause 5.2 in [x1].

## 5.3	Accuracy of test equipment

See clause 5.3 in [x1].

## 5.4	Test signals

See clause 5.4 in [x1].

For send tests, the only requirement on test setup is that a predefined active test signal is encoded (at nominal level) to produce an expected packet stream; there is no specific reason to mandate one interface type as long as this requirement is fulfilled.

## 5.5	Environmental conditions

See clause 6 in [x1].

For LTE, WLAN, and NR connections, an RF shielded room [should/shall] be one way to achieve these requirements on block error rate and jitter. Otherwise, care should be taken with potential sources of radio interference and their impact.

We would propose to extend this note into a requirement (shall or should) for connections covering WLAN as well as LTE and NR.

Whatever the used interface , it is assumed that the DUT will receive ‘nominal’ signal levels.

## 5.6	System simulator conditions

Test applicability / result may depend on the SIM card / eSIM configuration.

Depending on MNC/MCC, device could be set in test mode or load some specific parameters.

Unless otherwise stated, a new call should be established for each test case.

# 6	RTP Payload Format Conformance for AMR

## 6.1	Applicability

The requirements and test methods in this clause shall apply when UE is used to provide narrowband telephony, either as a stand-alone service, or as part of a multimedia service.

## 6.2	SDP tests

### 6.2.1	MO call

### 6.2.1	MT call

## 6.3	RTP tests

## 6.3.1		Test cases in sending

### 6.3.1.1		Q-bit verification

Purpose:

Check DUT always set Q-bit to 1 in the RTP payload (If set to 0, it indicates that the corresponding frame is severely damaged, see [x8]

#### Requirement:

The Q-bit shall always bit set to 1 in the RTP payload.

NOTE:	The Q-bit is the frame quality indicator [x8]. If set to 0, it indicates that the corresponding frame is severely damaged, and the receiver should set the RX_TYPE to either SPEECH_BAD or SID_BAD depending on the frame type (FT).]

#### Test method:

Preconditions: 	1) Setup MT call with AMR codec at predefined mode (e.g.,  7 for 12.2 kbit/s)

2) The DUT is set to send and receive RTP packets during the test session, e.g. pre-configured or set as send/receive  through test instrument signalling.

Test procedure: 	Observe the DUT output based on RTP packet payload at IP level.

Stop condition: 	Ten or more seconds have passed after first RTP packet was sent from data injection.

Pass criterion: 	All RTP packets are sent with Q-bit set to 0.

### 6.3.1.2		Handling of CMR (open offer)

#### Requirement:

[The AMR bit rate (mode) in sending shall be according to the CMR inserted in receiving after [X] ms.]

#### Test method:

[Editor’s note: test setup to be fully defined first, the principle of the test method could be to setup MT call(s) with AMR at predefined modes and analyse captured PCAP files at the IP level. ]

### 6.3.1.3		Handling of CMR (restricted offer)

#### Requirement

[The AMR bit rate (mode) in sending shall be according to the CMR inserted in receiving after [X] ms when this CMR is within the negotiated codec set.

A CMR outside the negotiated codec set shall be ignored, i.e. the AMR bit rate (mode) in sending shall not be changed after receiving a CMR outside the restricted offer and inserted in receiving.]

#### Test method:

[Editor’s note: test setup to be fully defined first, the principle of the test method could be to setup MT call(s) with AMR at predefined modes and analyse captured PCAP files at the IP level. ]

#### 6.3.1.4	SID update periodicity

##### [Requirement]

[The DUT shall respect the SID update rules described in [x9]

If SID frames are not sent in the required interval, some core network equipments may think that RTP traffic is suspended and so release the call (false communications cutting detection).

##### [Test method]

Establish a MT or MO call and any mode of AMR is negotiated

After X ms, force silence input test signal to DUT and analyse the RTP sending frames intervals.

Pass criteria:

SID frames shall be sent as following:

- First SID frame 20ms after the last speech frame : SID_FIRST

- Second SID frame (first SID UPDATE): 60 ms after the first SID

- Following SID UPDATE : every 160 ms

### 6.3.2	Test cases in receiving

### tbd

## 6.4	RTCP tests

tbd

# 7	RTP Payload Format Conformance for AMR-WB

[Editor’s note: structure to be updated similar to the AMR example in clause 6]

## 7.1	Applicability

[The requirements and test methods in this clause shall apply when UE is used to provide wideband telephony, either as a stand-alone service, or as part of a multimedia service.]

## 7.2	SDP tests

### 7.2.1	MO call

### 7.2.2	MT call

## 7.3	RTP tests

## 7.3.1	[Test cases in sending]

### 7.3.1.1	[Q-bit verification]

#### [Requirement]

[The Q-bit shall always bit set to 1 in the RTP payload.

NOTE:	The Q-bit is the frame quality indicator [RFC 4867]. If set to 0, it indicates that the corresponding frame is severely damaged, and the receiver should set the RX_TYPE to either SPEECH_BAD or SID_BAD depending on the frame type (FT).]

#### [Test method]

[Editor’s note: test setup to be fully defined first, the principle of the test method could be to setup MT call(s) with AMR-WB at predefined modes and analyse captured PCAP files at the IP level. ]

### 7.3.1.2	[Handling of CMR (open offer)]

#### [Requirement]

[The AMR-WB bit rate (mode) in sending shall be according to the CMR inserted in receiving after [X] ms.]

#### [Test method]

[Editor’s note: test setup to be fully defined first, the principle of the test method could be to setup MT call(s) with AMR-WB at predefined modes and analyse captured PCAP files at the IP level. ]

### 7.2.1.3	[Handling of CMR (restricted offer)]

#### [Requirement]

[The AMR-WB bit rate (mode) in sending shall be according to the CMR inserted in receiving after [X] ms when this CMR is within the negotiated codec set.

A CMR outside the negotiated codec set shall be ignored, i.e. the AMR-WB bit rate (mode) in sending shall not be changed after receiving a CMR outside the restricted offer and inserted in receiving.]

#### [Test method]

[Editor’s note: test setup to be fully defined first, the principle of the test method could be to setup MT call(s) with AMR-WB at predefined modes and analyse captured PCAP files at the IP level. ]

#### 7.3.1.4	SID update periodicity

### 7.2.1.4	[RTCP bandwidth verification]

[Editor’s note: one option could be to refer to TS 26.139 and require that UE comply with this specification.

#### [Requirement]

[RTCP conformance is defined in 3GPP TS 26.139.

Requirement to be defined]

#### [Test method]

[Editor’s note: test setup to be fully defined first, the principle of the test method could be to setup MT call(s) with AMR-WB at predefined modes and analyse captured PCAP files at the IP level. ]

## 7.3.2	[Test cases in receiving]

tbd

## 7.4	RTCP tests

tbd

# 8	RTP Payload Format Conformance for EVS

[Editor’s note: structure to be updated similar to the AMR example in clause 6]

## 8.1	Applicability

[The requirements and test methods in this clause shall apply when UE is used to provide narrowband, wideband, super-wideband or fullband telephony, either as a stand-alone service, or as part of a multimedia service.]

## 8.2	SDP tests

### 8.2.1	MO call

Purpose: Check SDP offer is compliant respectively with IR.92 [x10] for “LTE only DUT” (meaning not Vo5G capable device) and NG.114 [x11] for Vo5G capable UE

Test procedure:

1) DUT place an MO call

2) Observe the DUT SDP offer at IP level and check the 2 preferred payload types offers.

Pass criteria:

Case A: For non Vo5G capable device, check compliancy to IR.92 [x10] section 2.4.3.3

If EVS is proposed for super-wideband calls, it shall include as least one of the following EVS configurations:

- EVS Configuration A1: br=5.9-13.2; bw=nb-swb.

- EVS Configuration A2: br=5.9-24.4; bw=nb-swb.

- EVS Configuration B0: br=13.2; bw=swb.

- EVS Configuration B1: br=9.6-13.2; bw=swb.

- EVS Configuration B2: br=9.6-24.4; bw=swb

Case B: Vo5G capable device, check compliancy to NG.114 [x11] section 3.2.2.3

At least one of the 5 configurations (supporting super-wideband) of Case A shall be proposed.

### 8.2.1	MT call

Purpose: For MT call including an EVS offer as preferred payload type, check SDP answer is compliant with clause 6.2.2.3 of [x12] by testing several SDP offers. see list in Annex A.

Preconditions: 	none

Test procedure:

1) DUT receive an MT call with EVS offer.

2) Observe the DUT SDP answer in SIP Session in Progress packet payload at IP level and check the payload type answer.

Pass criteria:

DUT shall accept the first proposed EVS payload type.

No parameter shall be added in the answer.

### Some parameters value may be modified (depending on the real capabilities of the DUT, e.g., bw, br…).

## 8.3	RTP tests

## 8.3.1		Test cases in sending

#### 8.3.1.1	Q-bit verification

#### [Requirement:

[The Q-bit shall always bit set to 1 in the RTP payload when EVS-IO and header-full modes are negotiated.

NOTE:	The Q-bit is the frame quality indicator [x8]. If set to 0, it indicates that the corresponding frame is severely damaged, and the receiver should set the RX_TYPE to either SPEECH_BAD or SID_BAD depending on the frame type (FT).]

#### Test method:

[Editor’s note: test setup to be fully defined first, the principle of the test method could be to setup MT call(s) with AMR-WB at predefined modes and analyse captured PCAP files at the IP level. ]

### 8.3.1.2		Handling of CMR

#### Requirement:

[The EVS Primary bit rate (mode) in sending shall be according to the EVS Primary CMR inserted in receiving after [X] ms.

The EVS-IO bit rate (mode) in sending shall be according to the EVS-IO CMR inserted in receiving after [X] ms when the call is initiated with AMR-WB IO.

Is it expected when receiving an acceptable CMR - meaning whose value is allowed in the negotiated SDP that the DUT switches encoding with the bandwidth requested in the CMR  and at the max bit rate as requested by the CMR pattern. Nevertheless, encoding with lower bit rate can be granted.

Editor’s note: the list of tests can be extended and conformance tests may be formulated in a table ]

#### Test method:

[Editor’s note: test setup to be fully defined first, the principle of the test method could be to setup MT call(s) with EVS at predefined modes and analyse captured PCAP files at the IP level. ]

#### 8.3.1.3	SID update periodicity

##### Requirement:

[The DUT shall respect the SID Update rules described in [x9] section 5.6.1.1

##### Test method:

Establish a MT or MO call in where discontinuous transmission is enable (dtx  =1 or absent) and with an EVS offer

After X ms, force silence input test signal to DUT and analyse the RTP sending frames intervals.

Pass criteria:

SID frames shall be sent:

- Either at a fixed rate: in such case interval shall be 20 ms multiple in range [60ms;2s]

- or at adaptative rate: in such case interval shall be 20 ms multiple in range [160ms;1s]

### 8.3.2	Test cases in receiving

### tbd

## 8.4	RTCP tests

### 8.4.1	General

[Editor’s note: one option could be to refer to TS 26.139. If the DUT is compliant with this specification, the RTCP tests defined in this clause may be skipped, otherwise the clause would apply]

### 8.4.2	[RTCP bandwidth verification]

[Editor’s note: one option could be to refer to TS 26.139 and require that UE comply with this specification. ]

#### Requirement:

[RTCP conformance is defined in 3GPP TS 26.139.

Requirement to be defined]

#### Test method:

[Editor’s note: test setup to be fully defined first, the principle of the test method could be to setup MT call(s) with AMR-WB at predefined modes and analyse captured PCAP files at the IP level. ]

# Annex <A> (normative):

[RTP payload tools]

Editor’s note: Additional tools such as direct decoding of RTP payload or RTP payload dissectors may also be specified (see eUET WID).

# Annex <B> (normative):

[EVS SDP answer compliance]

This annex lists the EVS SDP offers to configure the system simulator for EVS SDP answer compliancy tests.

| SDP tested MT Offer |  |  | SDP MT expected answer |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Identifier | alias name | First payload type | br | bw | mode-set | evs-mode-switch | cmr | ch-aw-recv | dtx | ch-aw-recv | Additional comments |
| 1 | IO 23.85 + B2 mode-set WB | a=fmtp:98 evs-mode-switch=1;bw=swb;br=9.6-24.4;max-red=220;mode-set=0,1,2,8 |  |  |  |  |  |  |  |  |  |
| 2 | B2 mod DTX Off | a=fmtp:96 evs-mode-switch=0;bw=swb;br=9.6-24.4;dtx=0;ch-aw-recv=-1;max-red=220 |  |  |  |  |  |  |  |  |  |
| 3 | SWB 24.4 | a=fmtp:106 br=9.6-128;bw=swb;mode-change-capability=2;cmr=0;dtx=1;ch-aw-recv=3;max-red=0 | 9.6-24.4 |  |  |  |  |  |  |  |  |
| 4 | A2 mod br | a=fmtp:96 evs-mode-switch=0;bw=nb-swb;br=5.9-24.4;max-red=220 |  |  |  |  |  |  |  |  |  |
| 5 | A2 mod CMR | a=fmtp:96 evs-mode-switch=0;bw=nb-swb;br=9.6-24.4;cmr=-1;max-red=220 |  |  |  |  |  |  |  |  |  |
| 6 | IO 12.65 mod | a=fmtp:98 evs-mode-switch=1;bw=nb-fb;br=5.9-128;max-red=220;mode-set=0,1,2 | 5.9-24.4 | nb-swb |  |  |  |  |  |  |  |
| 7 | NB-WB 13.2 mod | a=fmtp:96 evs-mode-switch=0;bw=nb-wb;br=5.9-13.2;ch-aw-recv=2;max-red=220;mode-set=0,1,2 |  |  |  |  |  |  |  |  |  |
| 8 | NB-WB 24.4 CMR Off | a=fmtp:96 evs-mode-switch=0;bw=nb-wb;br=5.9-24.4;cmr=-1;max-red=220 |  |  |  |  |  |  |  |  |  |
| 9 | WB 24.4 | a=fmtp:96 evs-mode-switch=0;bw=wb;br=9.6-24.4;max-red=220 |  |  |  |  |  |  |  |  |  |
| 10 | NB-SWB 13.2 | a=: fmtp:96 evs-mode-switch=0;bw=nb-swb;br=13.2;max-red=220 |  |  |  |  |  |  |  |  |  |
| 11 |  | NB-SWB 13.2 |  | a=: fmtp:96 evs-mode-switch=0;bw=swb;br=13.2;max-red=220 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Annex <C> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | SA4#120-e | S4-221028 |  |  |  | Initial version | 0.0.1 |
| 2022-08 | SA4#120-e | S4-221189 |  |  |  | Inclusion of pCR in S4-221029 in bracket | 0.1.0 |