Source:	Nokia Corporation

Title:	Characterization of Combined formats

Document for:	Discussion

Agenda Item:	1.4 – IVAS_Codec

# 1. Introduction

IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase [1], discuss specifics of IVAS Codec characterization testing.

In this document the source proposes test material and conditions for P.800Suppl DCR experiment covering oMASA. All oMASA operation points, some FER conditions, some separate IVAS MASA + ISM coding, as well as some fixed / floating point inter-operation are proposed.

# 2. Overview

In the current work in progress IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase [1], both oMASA and oSBA has been reserved one P.800Suppl DCR test respectively. To progress the work, the source proposes specific test material and conditions that can be used to characterize these combined formats.

# 3. Discussion

Some observations and considerations are made relating, e.g., to size of experiment. In particular, the P.SUPPL800 DCR experiments can have 36 test conditions.

- It is proposed to test all conditions with either IVAS internal binaural renderer or with IVAS_rend functionality for Direct reference.

- Fixed point version of the IVAS codec should be used, if not otherwise specified by the condition.

- It is proposed to test all IVAS oMASA bitrates up to 512 kbps

- Some FER conditions are selected to give an insight to frame erasure concealment functionality of the IVAS codec with oMASA operation.

- DTX is not supported in oMASA and thus not tested.

- Some conditions in are reserved to test floating / fixed point interoperability.

- We propose to test the combined formats using the full range of number of objects (ISMs) 1 - 4.

  - These are distributed to separate sample categories. Thus, the maximum number of oMASA operation points can be covered in a single listening test.

    - All bitrates

    - All 1-4 ISM combinations

- For IVAS oMASA DCR test all objects are speech. MUSHRA tests may cover generic audio objects.

- Objects can be static or moving.

  - It is proposed that inside each sample category there are samples with all static objects and samples where one or more object(s) are moving.

  - Some of the object paths can likely be reused from ISM testing, but it is expected that some extra object paths have to be specifically constructed for this test.

- Sample overlapping

  - For single object scenario there is naturally no overlap.

  - For two object scenario we propose to test it with and without overlap.

  - For 3 and 4 objects scenarios two or more objects are always overlapping. This reduces the total length of the listening test sample to fit under 8 seconds.

- Background environment signals

  - Same background environment signals as used for IVAS selection testing of FOA and MASA can be used.

  - In addition some new samples can be provided, e.g. background environment files with some music playing in the environmental recording.

  - The level of the background noise is TBD and requires some expert listening, but SNR of 10dB between talkers and background noise is a good starting point.

  - For oMASA the background noise can be in FOA, HOA or Eigenmike format.

- There are no performance requirements, but comparison to separate MASA + ISM IVAS coding are relevant comparison points.

- Towards the lower bitrates, there are no possible separate MASA + ISM operation points available.

- IVAS ISM coding requires at minimum 24.4 kbit/s for 3-4 objects further decreasing the lower bitrate references.

# 4. Proposals for specific experiments

## 4.1 P.SUPPL800 oMASA

It is proposed for oMASA inputs to have following conditions for the IVAS characterization listening test.

Table 1 Overview of test conditions

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 15 | IVAS oMASA operated at all bitrates 13.2- 512 kbps with DTX off at 0% FER |
|  | 3 | IVAS oMASA operated at 32, 64, 128 kbps with 5% FER |
|  | 4 | IVAS oMASA fixed point / floating point interoperability conditions at 32, 64, 128 and 256 kbps |
| Codec references |  |  |
| Codec references | 6 | IVAS operation in two separate instances (MASA + ISM) |
| Other references |  |  |
| Direct | 1 | Fixed point IVAS_rend. Nominal input level |
| P.50 MNRU (applied to MASA transport streams) | 4 | Q = 34, 30, 26, 22 dB (all: nominal input level) |
| ESDRU [ITU-T P.811] | 4 | α = 0.8, 0.6, 0.4, 0.2 (output loudness set to nominal level) |
| Common Conditions |  |  |
| Test item generation | 1 | Model-based generation according to processing scripts. |
| Binaural rendering | 1 | IVAS codec internal binaural renderer and for references IVAS external renderer (IVAS_rend) |
| Audio sampling frequency / bandwidth | 1 | 48 kHz / maximum available audio bandwidth (WB, SWB, FB) |
| Content types / categories | 6 | Scenes as described in Table2 |
| Number of talkers | 6 | ISM signals are generated so that different objects are uttered by different talkers (one each of 3 male and 3 female talkers). All talkers are tried to be used equal amount. |
| Number of speech samples | 7 | 6 for tests + 1 for preliminaries per category |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Background signal (MASA) input loudness | 1 | -36 LKFS ([ITU-R BS.1770-4]) |
| ISM speech input loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR with instructions according to [P Suppl. 29] |
| Languages | 1 | [tbd] |
| Listening System | 1 | High-quality headphones, diotic presentation |
| Listening Environment | 1 | No noise |

Table 2 Sample Categories

| Category | Number of objects | Speech Level [dB] | Background signal type | Background Level | Overtalk [s] | Talker positions |
| --- | --- | --- | --- | --- | --- | --- |
| cat 1 | 1 | -26 | Indoors 1 | [-36] | No overtalk | 2 fixed, 4 with movement |
| cat 2 | 2 | -26 | Indoors 2 | [-36] | Overtalk | 2 fixed, 4 with movement* |
| cat 3 | 3 | -26 | Outdoors 1 | [-36] | Overtalk | 2 fixed, 4 with movement* |
| cat 4 | 4 | -26 | Outdoors 2 | [-36] | Overtalk | 2 fixed, 4 with movement* |
| cat 5 | 2 | -26 | Background with music 1 | [-36] | No overtalk | 2 fixed, 4 with movement* |
| cat 6 | 3 | -26 | Background with music 2 | [-36] | Overtalk | 2 fixed, 4 with movement* |

*for 2 samples one ISM is moving, for the last 2 samples two or more objects are moving. For practice sample one ISM is moving.

Table 3 Test conditions for P.SUPPL800 oMASA, speech and background environments.

| Label | Condition | Bitrate [kbps] | FER/float | Notes |
| --- | --- | --- | --- | --- |
| c01 | Reference IVAS_rend | - | - |  |
| c02 | MNRU Q=34 dB | - | - |  |
| c03 | MNRU Q=30 dB | - | - |  |
| c04 | MNRU Q=26 dB | - | - |  |
| c05 | MNRU Q=22 dB | - | - |  |
| c06 | ESDRU α=0.8 | - | - |  |
| c07 | ESDRU α=0.6 | - | - |  |
| c08 | ESDRU α=0.4 | - | - |  |
| c09 | ESDRU α=0.2 | - | - |  |
| c10 | OMASA 2TC | 32 | Float |  |
| c11 | OMASA 2TC | 64 | Float |  |
| c12 | OMASA 2TC | 128 | Float |  |
| c13 | OMASA 2TC | 256 | Float |  |
| c14 | OMASA 2TC | 32 | 5% FER |  |
| c15 | OMASA 2TC | 64 | 5% FER |  |
| c16 | OMASA 2TC | 128 | 5% FER |  |
| c17 | OMASA 2TC | 13,2 | No errors |  |
| c18 | OMASA 2TC | 16,4 | No errors |  |
| c19 | OMASA 2TC | 24,4 | No errors |  |
| c20 | OMASA 2TC | 32 | No errors |  |
| c21 | OMASA 2TC | 80 | No errors |  |
| c22 | OMASA 2TC | 160 | No errors |  |
| c23 | OMASA 2TC | 384 | No errors |  |
| c24 | OMASA 2TC | 512 | No errors |  |
| c25 | OMASA 2TC | 48 | No errors | C31 |
| c26 | OMASA 2TC | 64 | No errors | C32 |
| c27 | OMASA 2TC | 96 | No errors | C33 |
| c28 | OMASA 2TC | 128 | No errors | C34 |
| c29 | OMASA 2TC | 192 | No errors | C35 |
| c30 | OMASA 2TC | 256 | No errors | C36 |
| c31 | ISM + MASA | 24,4 + 24,4 | No errors |  |
| c32 | ISM + MASA | 32 + 32 | No errors |  |
| c33 | ISM + MASA | 48 + 48 | No errors |  |
| c34 | ISM + MASA | 64 + 64 | No errors |  |
| c35 | ISM + MASA | 96 + 96 | No errors |  |
| c36 | ISM + MASA | 128 + 128 | No errors |  |

# 5. Summary

We have presented some IVAS characterization test plan specifics and options for discussion in clause 4 of this input. The specific proposal covers P.SUPPL800 DCR experiment for IVAS oMASA operation.

# References

[1]	S4aA230127-IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase, Version v.0.1.2

[2]	ITU-T P.800 – Use cases, P Suppl. 29 (01/2023)

[3]	S4aA-230086: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.1.1.0