Title:		        Updates to clauses 4 to 7 of TR 26.933

Source:                   Panasonic Holdings Corporation

Document for:       Discussion & Agreement

Agenda item:         1.6.

1. Background

The source has reviewed the latest draft of TR 26.933 [1] and noticed inconsistencies in the text. Due to limited time between the Sep 2nd meeting and 129th e-meeting, the source has made a best effort to propose text refinement. Furthermore, some findings require additional investigation, while a few necessitate inputs from the original source.

2. Proposal

Source would like to propose the following for clause 4 to 7.

- Clause 4 Size and structure evolution of UEs

- Sub-clause “4.2 Headphones”: reviews only TWS earbuds, mostly referring to wireless in-ear headphones, but sub clause 7.2 microphone integration of headphones, discuss design details of on-ear, over-ear, in-ear headphones.

Source believes that inconsistency between 4.2 and 7.2 sub-clause should be reviewed and addressed to ensure clarity and consistency.

Proposal:

- Rename the TWS earbuds to TWS in-ear headphones or include a clarification on the TWS earbuds and in-ear headphones definition.

- Data on other types of headphones, such as on-ear, over-ear wireless headphones, is not available in the sub-clause 4.2. The source proposes further investigation to cover on-ear and over-ear wireless headphones.

- Sub-clause “4.4 Laptops”: Figures in other sub-clauses of clause 4 indicate the possible presence of microphone positions, but Figure 4.4-1 in sub-clause 4.4 lacks the detail.

Proposal:

- The source requests that the original content contributor revise the inconsistency.

- Sub-clause “4.5 Watches”: discusses various watch sizes and shapes, as well as the calling feature. To better reflect the content, we recommend renaming the title.

Proposal:

- Rename clause 4.5 title to Smartwatch

- Rename clause 7.5 title to Microphone integration in Smartwatch

- Sub-clause “4.5 Watches”: Inconsistency in sub-clause text was noticed,

Proposal:

Include following revision

The screen size and shape of smartwatch models boast round (or circular) and/or rectangular displays that resemble traditional watch displays.

The dimensions of these displays vary:

- Circular displays have a diameter of approximately 4.7 cm and a depth ranging from 10.9 to 13 cm (see Annex A.5).

Figure 4.5-1 circular smartwatch

- For rectangular displays have lengths varying from 4 cm to 5.7 cm, widths ranging from 3.4 cm to 4.57 cm, and heights ranging from 1.07 cm to 1.49 cm (see Annex A.5).

Figure 4.5-2 rectangular smartwatch front view

Figure 4.5-3 rectangular smartwatch side view

Furthermore, modern smartwatches are equipped with connectivity, with many modern models offering Bluetooth, Wi-Fi and even cellular connectivity. This allows for greater independence from a paired smartphone and enables features like voice calls.

- Sub-clause “4.5 Watches”: Figures in other sub-clauses of clause 4 indicate the possible presence of microphone positions, but Figure 4.5-1 in sub-clause 4.5 lacks the detail of microphone.

Proposal:

- The source requests that the original figures contributor review and resolve the inconsistency.

- Sub-clause “4.6 XR devices”: following text was discussed in the sub-clause.

XR devices come in a wide range of sizes. Some are substantial and weighty, akin to headband-style headsets, while others are lightweight, resembling ordinary glasses. The larger variants often adopt a headband-style design, whereas the smaller ones are designed to the appearance of regular eyewear, Therefore, the average size of the investigated model is approximately 157.48 mm*130.92mm*41.22mm on width, length, and height respectively (see Annex A.6).

- Source believes that clarity on the definition of "headband style headsets" and "regular eyewear", is needed.

Proposal:

- XR devices vary significantly in size, ranging from large, substantial head-mounted displays (HMDs) with a headband-style design to small, lightweight devices resembling standard prescription glasses or sunglasses. The larger devices typically feature a headband-style design, while the smaller ones are designed to resemble conventional eyeglasses. The average dimension of the investigated model is approximately 157.48 mm*130.92mm*41.22mm in width, length, and height respectively (see Annex A.6).  Additionally, the placement of the microphones on these devices can be flexible, with some devices positioning them near the ears for optimal audio capture.

- Sub-clause “4.6 XR devices”:  The text in the figure 4.6-1 is difficult to read due to its small font size.

Proposal:

Source request that font size be increased and/or the figure be enlarged for better readability.

- “Clause 5 Microphone used in audio capture”

- Sub-clause “5.1 Introduction”: following text is available in the TR

The function of a microphone is to convert sound pressure signal to analog electrical signal in a circuit.

4 types of microphones popular in the market are described in this section. These microphones have unique advantages in UE's immersive audio capture. They are classified to dynamic microphone, condenser microphone, MEMS and contact microphone.

- The source believes that the scope of 5.1 was limited to the contents in clause 5.2, but it requires further revision to encompass the remaining sub-clauses (5.3, 5.4, etc.).

- The text mentions that “These microphones” have unique advantages in UE's immersive audio capture. Which specific types of microphones are being referred to here?

- Furthermore, is it accurate to say that UEs such as headphones, mobile phones and others, employ dynamic, condenser, or contact microphone for immersive audio capture?

Proposal:

- Revise the introduction sub-clause to include the remaining sub-clauses, especially sub-clauses 5.3, 5.4

- The source is unable to provide an alternative text proposal, as the intention of the original contribution is unclear. The source request that the original contributor resolve the ambiguity in 5.1

- Sub-clause “5.4 Binaural acoustic simulation”: following text was discussed in the sub-clause

According to the principle of binaural signal, the typical solution involves placing microphones on each ear of the user or on a model of a human head or ears to capture the binaural cues. This model could be a head with a torso to simulate all influences, including those from the ear, head and reflections from the torso. Alternatively, it could be a single head or simply a model with a pair of ears.

- Source believes that binaural capture was discussed in 6.2, need for sub-clause 5.4 in clause 5 should be discussed.

- Additionally, text in 5.4 has some ambiguity.

Proposal

- Need for clause 5.4 should be revisited.

- Alternatively, we propose reworking sub-clause 5.4 and revisiting its title to better align with the content

- Sub-clause “5.5 Conclusion”

Conclusion focuses solely on MEMS microphones. This conclusion text was originally proposed by the source, source believes further edits is required to include other microphone types.

Proposal:

Include following revised text

The microphone types discussed in the above clauses (e.g., dynamic, condenser) are better suited for live performances and studio recordings. Due to their size and structure, these microphones are not compatible with the compact user equipment (UEs) discussed in this TR. In contrast, MEMS microphones are an ideal choice for compact UEs. Notably, digital MEMS microphones integrate preamps, ADC, and clocks, enabling direct digital signal output and making audio capture significantly more convenient. This innovation is particularly beneficial for small-sized devices, considering their size and consistent performance. Moreover, these miniature microphones can be used to generate immersive audio when combined with signal processing.

- Clause 6 Audio Capture Format

In this clause various audio capture formats were reviewed, with Clause 6.3 summarizing stereo and spatial capture. However, it is unclear how these capture methods apply to each user equipment (UE) type. Specifically, for stereo capture, spaced stereo is the only feasible option for the UEs discussed in this TR. To provide clearer guidance, it would be beneficial to identify suitable capture methods for each UE type.

Proposal:

Source, requests revisit the sub-clauses to identify or map the suitable capture methods for each UE type for clarity.

- Clause 7: Microphone integration in UEs

- Clause 4 Size and structure evolution of UEs, discuss following UEs size and structure details

  - Mobile phones

  - Headphones

  - Tablets

  - Laptops

  - Watches

  - XR devices

  - Cars

We noticed that in the clause 7, microphone integration in UEs was limited to following UEs

  - Microphone integration in mobile phones

  - Microphone integration in headphones

  - Microphone integration in tablets

  - Microphone integration in laptops

  - Microphone integration in watches

  - Summary

  - Microphone integration and frequency response

- Source noticed that no information was captured in clause 7 for XR devices and Cars. Reasons for excluding some devices is unknown.

Proposal

- Data collection on XR devices and Cars is needed to address the inconsistencies.

- The tables in Sub-clauses 7.1 and 7.2 currently include a separate column for 'Polar Pattern'. However, considering the title change from 'Microphone Design' to 'Microphone Integration in UE', source believes that this information may no longer be needed.

Proposal:

To maintain consistency with other tables in the clause, it is suggested that the 'Polar Pattern' column be removed from the table in sub-clauses 7.1 and 7.2.

3. Conclusion

The source has submitted alternative text proposals to resolve ambiguity for certain clauses and recommends agreement for inclusion in the Technical Report (TR). Furthermore, some findings have revealed gaps in the TR, and the source proposes to conduct further investigation to address these gaps and enhance the completeness of the TR.

4 References

- S4-241787: TR 26.933 v1.1.0 – Study on Diverse Audio Capturing system