3GPP TSG-SA4 Meeting #110-e	S4-201004

# Electronic, Online, 19th–28th August 2020	resubmission of S4-AHIA28

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.512 | CR | – | rev | – | Current version: | 1.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Informative annex on Content Hosting Configuration examples |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification). |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Provide the reader with some worked examples illustrating how the Content Hosting Configuration is intended to be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Worked example of Content Hosting Configuration for pull-based content ingest. - Worked example of Content Hosting Configuration for push-based content ingest.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 design for Content Hosting Configuration is more difficult for an implementer to comprehend. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-AHIA28. |  |  |  |  |  |  |  |  |

FIRST CHANGE

# Annex A (informative)	Content Hosting Configuration examples

## A.1	Pull-based content ingest example

### A.1.0	Overview

1.	The 5GMSd Client on the UE requests a media resource via M4d.

2.	The 5GMSd AS determines that it doesn't have a cached copy of the requested media resource.

3.	The 5GMSd AS transforms the M4d request URL into a request to the 5GMSd Application Provider’s origin server via M2d.

### A.1.1	Desired URL mapping

In the example shown in table A.1.1-1 below, media resources are exposed at M4d from a default canonical domain 5gmsd-as.mno.net determined by the 5GMSd System operator, and also from a custom domain name alias mno-cdn.5gmsd-ap.com that has been configured by the 5GMSd Application Provider.

Table A.1.1-1: Example URL mapping for pull-based ingest

| M4d request from 5GMSd Client | Mapped M2d request to origin server on 5GMSd AS cache miss |
| --- | --- |
| https://5gmsd-as.mno.net/m4d/provisioning-session9876/‌asset123456/video1/segment1000.mp4 | https://origin.5gmsd-ap.com/‌media/‌asset123456/video1/segment1000.mp4 |
| https://mno-cdn.5gmsd-ap.com/m4d/provisioning-session9876/‌asset123456/video1/segment1000.mp4 |  |
| https://5gmsd-as.mno.net/m4d/provisioning-session9876/‌asset123456/video2/segment1000.mp4 | https://origin.5gmsd-ap.com/‌media/‌asset123456/video2/segment1000.mp4 |
| https://mno-cdn.5gmsd-ap.com/m4d/provisioning-session9876/‌asset123456/video2/segment1000.mp4 |  |
| https://5gmsd-as.mno.net/m4d/provisioning-session9876/‌asset123456/audio1/segment1000.mp4 | https://origin.5gmsd-ap.com/‌media/‌asset123456/audio1/segment1000.mp4 |
| https://mno-cdn.5gmsd-ap.com/m4d/provisioning-session9876/‌asset123456/audio1/segment1000.mp4 |  |

### A.1.2	Content Hosting Configuration

Table A.1.2-1 below shows the relevant Content Hosting Configuration parameters needed to achieve the example mapping described in table A.1.1-1 above.

Table A.1.2-1: Content Hosting Configuration properties relevant to pull-based ingest

| Property | Example value | Set by |
| --- | --- | --- |
| IngestConfiguration |  |  |
| protocol | urn:3gpp:5gms:content-protocol:http-pull-ingest | 5GMSd Application Provider |
| pull | true |  |
| entryPoint | https://origin.5gmsd-ap.com/ |  |
| path | (Not used) | (Not applicable) |
| DistributionConfiguration |  |  |
| canonicalDomainName | 5gmsd-as.mno.net | 5GMSd AF |
| domainNameAlias | mno-cdn.5gmsd-ap.com | 5GMSd Application Provider |
| PathRewriteRules[0].requestPathPattern | ^/m4d/provisioning-session[^/]+/ |  |
| PathRewriteRules[0].mappedPath | /media/ |  |
| NOTE: The 5GMSd Application Provider needs prior knowledge of the path structure exposed at M4d in order to supply the requestPathPattern regular expression. In this example, the Provisioning Session identifier is included in the M4d distribution path as a discriminator (c.f. “Content Provider code” concept in a commercial CDN). |  |  |

## A.2	Push-based content ingest example

### A.2.0	Overview

1.	The 5GMSd Application Provider uploads content to the 5GMSd AS via M2d.

2.	The 5GMSd AS rewrites the M2d upload URL to an M4d downlink URL that is exposed to the 5GMSd Client on the UE.

### A.2.1	Desired URL mapping

In the example shown in table A.2.1-1, media resources are pushed into the 5GMSd AS at M2d by the 5GMSd Application Provider and exposed to the 5GMSd Client at M4d using the canonical name of the 5GMSd AF 5gmsd-as.mno.net and an additional domain name alias mno-cdn.5gmsd-ap.com configured by the 5GMSd Application Provider.

Table A.2.1-1: Example URL mapping for pull-based ingest

| M2d ingest URL pushed to 5GMSd AS | M4d URL exposed to 5GMSd Client |
| --- | --- |
| https://5gmsd-as.mno.net/m2d/provisioning-session9876/‌asset123456/video1/segment1000.mp4 | https://5gmsd-as.mno.net/m4d/provisioning-session9876/‌asset123456/video1/segment1000.mp4 |
|  | https://mno-cdn.5gmsd-ap.com/m4d/provisioning-session9876/‌asset123456/video1/segment1000.mp4 |
| https://5gmsd-as.mno.net/m2d/provisioning-session9876/‌asset123456/video2/segment1000.mp4 | https://5gmsd-as.mno.net/m4d/provisioning-session9876/‌asset123456/video2/segment1000.mp4 |
|  | https://mno-cdn.5gmsd-ap.com/m4d/provisioning-session9876/‌asset123456/video2/segment1000.mp4 |
| https://5gmsd-as.mno.net/m2d/provisioning-session9876/‌asset123456/audio1/segment1000.mp4 | https://5gmsd-as.mno.net/m4d/provisioning-session9876/‌asset123456/audio1/segment1000.mp4 |
|  | https://mno-cdn.5gmsd-ap.com/m4d/provisioning-session9876/‌asset123456/audio1/segment1000.mp4 |

### A.2.2	Content Hosting Configuration

Table A.2.2-1 below shows the relevant Content Hosting Configuration parameters needed to achieve the example mapping described in table A.2.1-1 above.

Table A.2.2-1: Content Hosting Configuration properties relevant to push-based ingest

| Property | Example value | Set by |
| --- | --- | --- |
| IngestConfiguration |  |  |
| protocol | urn:3gpp:5gms:content-protocol:dash-if-ingest | 5GMSd Application Provider (first M1d request) |
| pull | false |  |
| entryPoint | https://5gmsd-as.mno.net/ | 5GMSd AF (first M1d response) |
| path | /m2d/provisioning-session9876/ |  |
| DistributionConfiguration |  |  |
| canonicalDomainName | 5gmsd-as.mno.net |  |
| domainNameAlias | mno-cdn.5gmsd-ap.com | 5GMSd Application Provider (second M1d request) |
| PathRewriteRules[0].requestPathPattern | ^/m2d/provisioning-session9876/ |  |
| PathRewriteRules[0].mappedPath | /m4d/provisioning-session9876/ |  |
| NOTE1: The 5GMSd Application Provider needs knowledge of the M2d ingest path in order to set requestPathPattern. This requires a two-phase transaction when provisioning the Content Hosting Configuration at M1d. In the first request to create a Content Hosting Configuration at M1d, the 5GMSd Application Provider specifies the protocol and pull properties. In response, the 5GMSd AF sets the entryPoint and path. Then, in a second request at M1d, the 5GMSd Application Provider modifies the Content Hosting Configuration to add the necessary path rewrite rule. NOTE2: The 5GMSd Application Provider needs knowledge of the path structure exposed at M4d in order to supply the mappedPath. In this example, the Provisioning Session identifier is included in the M4d path as a discriminator (c.f. “Content Provider code” concept in a commercial CDN). |  |  |

# Annex B (Informative):	AF QoS Mapping

### B.1	Overview

This annex defines the mapping from a media streaming description into Dynamic Policy Instance properties for different types of media streaming description.

### B.2	DASH mapping

Editor’s Note: This clause should describe the mapping from DASH to Dynamic Policy Instances.

END OF CHANGES