3GPP TSG-SA4 MBS SWG Telco on 5GMS3 Maintenance 	S4aI201067

# 01 Oct 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | -- | rev | -- | Current version: | 2.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modifications to M1 API in Support of Metrics Reporting by AF to Application Provider |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, BBC, Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The M1 Metrics Reporting Provisioning API is currently missing support for the generation of metrics reports by the 5GMS AF for subsequent delivery to the 5GMS Application Provider. Such functionality has been identified and described in the 5GMS stage 2 specification, TS 26.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of clause 7.8 (M1 Metrics Reporting Provisioning API) to support metrics report generation and delivery by the 5GMS AF to the 5GMS Application Provider, in accordance to rules and location information added to the MetricsReportingConfiguration resource. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inability for QoE metrics reporting function from the 5GMS AF to the external 5GMS Application Provider, which has interest/need for obtaining such information. Misalignment with such enabled functionality in TS 26.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 4.3.9 and its sub-clauses, sub-clauses of 7.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st CHANGE: New clause 4.3.9

### 4.3.9	Metrics Reporting Configuration procedures

#### 4.3.9.1	General

##### These procedures are used by the 5GMSd Application Provider to activate and to configure QoE metrics reporting. This clause defines the basic procedures. More details are provided in clause 7.8.

##### A given instance of a Metrics Reporting Configuration is identified by the metricsReportingConfigurationId property of the MetricsReportingConfiguration resource, The parameters of that resource, as described in clause 7.3.8.1, pertain to metrics collection and reporting by the Media Session Handler to the 5GMS AF and, optionally, to metrics reporting by the 5GMS AF to an external network entity, e.g. the 5GMS Application Provider.

#### 4.3.9.2	Create Metrics Reporting Configuration

This procedure is used by the 5GMSd Application Provider to activate metrics reporting for a particular Provisioning Session. The 5GMSd Application Provider shall use the HTTP POST method to activate the metrics reporting procedure and optionally to transmit the Metrics Reporting Configuration to the 5GMSd AF. Upon successful operation, the 5GMSd AF shall respond with a 201 (Created) response message and the resource URL for the Metrics Reporting Configuration shall be returned in the Location header field. If the procedure is unsuccessful, the 5GMSd AF shall provide a response code as defined in clause 6.3.

This procedure may be used on multiple occasions to provision different Metrics Reporting Configurations in the scope of a particular Provisioning Session. Each such configuration is represented by a different value of metricsReportingConfigurationId.

#### 4.3.9.3	Read Metrics Reporting Configuration

This procedure is used by the 5GMSd Application Provider to obtain the current Metrics Reporting Configuration parameter values from the 5GMSd AF. The 5GMSd Application Provider uses the GET method for this purpose. If successful, the 5GMSd AF shall respond with a 200 (OK) and the requested MetricsReportingConfiguration resource (see clause 7.8.3.1) shall be returned in the body of the HTTP response message. If the procedure is unsuccessful, the 5GMSd AF shall provide a response code as defined in clause 6.3.

#### 4.3.9.4	Update Metrics Reporting Configuration

The update operation is invoked by the 5GMSd Application Provider to supply or modify the current Metrics Reporting Configuration parameter values. All available parameters may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) reflecting the successful update operation. If the procedure is unsuccessful, the 5GMSd AF shall provide a response code as defined in clause 6.3.

#### 4.3.9.5	Delete Metrics Reporting Configuration

This operation is used by the 5GMSd Application Provider to terminate the related metrics reporting procedure corresponding to a particular Metrics Reporting Configuration. The HTTP DELETE method shall be used for this purpose. As a result, the 5GMSd AF should release any associated resources and delete any corresponding configurations.

##### If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) response message. If the procedure is unsuccessful, the 5GMSd AF shall provide a response code as defined in clause 6.3.

END OF 1st CHANGE

2nd CHANGE: Modification to sub-clauses of 7.8

## 7.8	Metrics Reporting Provisioning API

### 7.8.1	Overview

The Metrics Reporting Provisioning API allows an 5GMS System operator or a 5GMSd Application Provider to configure the Metrics Collection and Reporting procedure for a particular Provisioning Session at interface M1d.

### 7.8.2	Resource structure

The Metrics Reporting Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1d/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.8.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column of the table shall be appended to the URL base path.

Table 7.8.2-1: Metrics Reporting Configuration resource

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create a metrics reporting configuration | metrics-reporting-configuration | POST | Create and optionally provide a configuration; returns the {metricsReportingConfigurationId}. |
| Read metrics reporting configuration | metrics-reporting-configuration/ {metricsReportingConfigurationId} | GET | Read the values of an existing configuration. |
| Update metrics reporting configuration |  | PUT, PATCH | Provide a modification of, or a replacement to, an existing configuration. |
| Delete metrics configuration |  | DELETE | Delete a configuration, disables reporting. |

### 7.8.3	Data model

#### 7.8.3.1 MetricsReportingConfiguration resource

The data model for the MetricsReportingConfiguration resource is specified in Table 7.8.3-1 below:

Table 7.8.3.1-1: Definition of MetricsReportingConfiguration resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| metricsReportingConfigurationId | String | 1..1 | An identifier for this Metrics Reporting Configuration that is unique within the scope of the enclosing Provisioning Session. |
| scheme | URI String | 0..1 | The scheme associated with this metrics configuration. A scheme may be associated with 3GPP or with a non-3GPP entity. If not specified, the 3GPP metrics scheme urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10 from TS 26.247 shall apply. |
| MediaSessionHandlerReporting | Object | 1..1 | Parameters used by the Media Session Handler when reporting metrics to the 5GMS AF in relation to this metrics reporting configuration. |
| dataNetworkName | String | 0..1 | The Data Network Name (DNN) which shall be used when sending metrics reports. If not specified, the default DNN shall be used. |
| reportingInterval | DurationSec | 0..1 | The time interval between successive metrics reports. If not specified, a single final report shall be sent after the streaming session has ended. |
| samplePercentage | Percentage | 1..1 | The proportion of streaming sessions for which metrics shall be reported. If not specified, reports shall be sent for all sessions. |
| urlFilters | Array(String) | 1..N | A list of content URL patterns for which metrics shall be reported. If not specified, reporting shall be done for all URLs. |
| metrics | Array(String) | 1..N | A list of metrics which shall be collected and reported. For the 3GPP scheme urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10 the listed metrics shall correspond to one or more of the metrics as specified in clauses 10.3 and 10.4, respectively, of TS 26.247 [7], and the quality reporting scheme and quality reporting protocol as defined in clauses 10.5 and 10.6, respectively, of [7] shall be used. If not specified, the complete (or default if applicable) set of metrics associated with the specified scheme shall be collected and reported. |
| ApplicationProviderReporting | Object | 0..1 | Processing rules and server location(s) of the 5GMS Application Provider associated with this metrics reporting configuration. |
| rules | Array (String) | 1..N | Set of one or more processing rules to be perfomed by the 5GMS AF on the metrics reporting information it receives from the Media Session Handler, in deriving output metrics reports for subsequent delivery to the endpoint(s) identified by location. Each rule is formatted as a string whose syntax and semantics are defined by the scheme of the associated metrics reporting configuration. Depending on the associated metrics scheme, these rules may be equivalent to, similar to, or altogether different from the semantics of the MediaSessionHandler properties above which govern metrics reporting at M5. |
| locations | Array (URL String) | 1..N | List of one or more destination endpoint addresses to which post-processed metrics reports shall be sent by the 5GMS AF. |

[Editor’s note: Additional details such as the syntax of the Application Provider defined metrics processing rules, 5GMS AF behavior in handling of rules that it does not support, etc. are FFS and should be further documented.]

END OF 2nd CHANGE