3GPP SA4 MBS SWG - 5GMS3 Maintenance	S4aI201069

# Telco  - 1st October 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR |  | rev |  | Current version: | 2.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Consumption reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Enensys Technology, BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | F |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Alignement with Stage 2 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In M1d, align the specitication with Stage 2 - In M5d, take into account of the UE mobility by moving the location information into the ConsumptionReportingUnit - Change the term “LocationType” for “LocationIdentifierType” to avoid confusion with GeoFencing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 6.4.2	Simple data types

Table 6.4.2-1 below specifies common simple data types used within the 5GMS APIs, including a short description of each. In cases where types from other specifications are reused, a reference is provided.

Table 6.4.2-1: Simple data types

| Type name | Type definition | Description | Reference |
| --- | --- | --- | --- |
| Percentage |  | number |  | A percentage expressed as a floating point value between 0.0 and 100.0 (inclusive). |  |  |  |
| DurationSec |  | integer |  | An unsigned integer identifying a period of time expressed in units of seconds. |  | TS 29.122 [12] table 5.2.1.3.2-2 |  |
| DateTime |  | string |  | An absolute date and time expressed using the OpenAPI date-time string format. |  | TS 29.122 [12] table 5.2.1.3.2-2 |  |
| LocationIdentifierType |  | integer |  | Identify the type of location used. CGI, ECGI and NCGI shall be represented by the values 0, 1 and 2, respectively. |  | TS 23.003 [7] |  |

Next Change

#### 7.7.3.1	ConsumptionReportingConfiguration resource

The data model for the ConsumptionReportingConfiguration resource is specified in Table 7.7.3.1-1.

Table 7.7.3.1-1: ConsumptionReportingConfiguration resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| reportingInterval | DurationSec | 0..1 | Identifies the interval between two consecutive consumption reports. The value shall be greater than zero. If absent, a single final report shall be sent immediately after the streaming session has ended. |
| samplePercentage | Percentage | 0..1 | The proportion of clients that shall report media consumption, expressed as a floating point value between 0.0 and 100.0. If not specified, all clients shall send consumption reports. |
| locationReporting | boolean | 1..1 | Stipulates whether the Media Session Handler is required to provide location data to the 5GMSd AF in consumption reporting messages (in case of MNO or trusted third parties). |

Next Change

### 11.3.3	Report format

#### 11.3.3.1	ConsumptionReport format

This type represents a consumption report data. This structure is used by the Media Session Handler to report the consumption.

Table 11.3.3.1-1: Definition of ConsumptionReport format

| Attribute name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| mediaPlayerEntry | string | 1..1 | Identifies the Media player entry. In the case of DASH, the media player entry pointer shall be the URL of the MPD. |
| reportingClientId | string | 1..1 | Identifies the identifier of the UE that consumes data. The client ID may be an MSISDN. |
| consumptionReportingUnits | Array(Consumption‌Reporting‌Unit) | 1..N | An array of consumption reporting units. |

#### 11.3.3.2	ConsumptionReportingUnit type

This type represents a single consumption reporting unit.

Table 11.3.3.2-1: Definition of type ConsumptionReportingUnit

| Attribute name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| mediaConsumed | string | 1..1 | Identifies the media consumed. In the case of DASH, the value of the Representation@id attribute shall be quoted. |
| startTime | DateTime | 1..1 | The time when this consumption reporting unit started. |
| duration | DurationSec | 1..1 | The duration of this consumption reporting unit. |
| locationIdentifierType | LocationIdentifierType | 0..1 | Identifies the UE location type if location reporting is enabled (only for trusted AF). |
| location | string | 0..1 | Identifies the UE location where the consumption media if location reporting is enabled (only for trusted AF). |